Table of Contents

Introduction	3

Getting Help	3

NOSA template	3

NOSA template	3

Metamodels	3

NOSA Version 0.5	4

NOSA Version 0.5	4

Viewstyles	4

Object Types	4

Relationship Types	4

Sub Metamodels	5

Variable classification	7

Variable classification	7

Object Types	7

Category	8

Term	9

Topic	10

Variable	11

NOSA View Style	12

Actual Measurement	13

Sensor Type	18

Sensing Element	19

Processing Element	22

Platform/Station	23

Phenomenon Requirement	27

Organization	29

Observing System	32

Observation Control System	41

Mobile Location Data	42

Required Measurement	44

Location	48

Group	51

Fixed Location Data	52

Environmental Product	54

Environmental Phenomenon	55

Environmental Parameter	57

Data Handling System	59

Characterization requirement	62

Platform/Station Type	63

Message	64

Applies to	68

Transmits	69

Supplied by	70

Use	71

Area of Operations	72

Has contact person	73

Provides info to	74

Provides data to	75

Funds	76

User	77

Primary stakeholder	78

Owns	79

Situated on	80

Contains	81

Provides data for	82

Measures	83

Supports	84

Operates	85

Characterizes	86

Drives	87

Reported to	88

Located at	89

Secondary stakeholder	90

Downstream user	91

Acquires	92

Provides data directly to	93

Is controlled by	94

Part of	95

Is type of	96

Leads to	97

Takes	98

Sent to	99

Met by	100

Not Found	101

# Introduction

## Getting Help

# NOSA template

## NOSA template

http://xml.noaa.gov/xml/templates/nosa/nosa_template.kmd#oid1

Initial NOSA template.

## Metamodels

The following list of sub topics displays all the metamodels available from the NOSA template template.

NOSA Version 0.5

## NOSA Version 0.5

### NOSA Version 0.5

http://xml.noaa.gov/xml/metamodels/nosa_metamodel.kmd#oid1

The NOAA Observing System Architecture metamodel.

### Viewstyles

The following list of sub topics displays all the viewstyles available from the NOSA Version 0.5 metamodel.

NOSA View Style

### Object Types

The following list of sub topics displays all the object types available from the NOSA Version 0.5 metamodel.

Actual Measurement

Sensor Type

Sensing Element

Processing Element

Platform/Station

Phenomenon Requirement

Organization

Observing System

Observation Control System

Mobile Location Data

Required Measurement

Location

Group

Fixed Location Data

Environmental Product

Environmental Phenomenon

Environmental Parameter

Data Handling System

Characterization requirement

Platform/Station Type

Message

### Relationship Types

The following list of sub topics displays all the relationship types available from the NOSA Version 0.5 metamodel.

Applies to

Transmits

Supplied by

Use

Area of Operations

Has contact person

Provides info to

Provides data to

Funds

User

Primary stakeholder

Owns

Situated on

Contains

Provides data for

Measures

Supports

Operates

Characterizes

Drives

Reported to

Located at

Secondary stakeholder

Downstream user

Acquires

Provides data directly to

Is controlled by

Part of

Is type of

Leads to

Takes

Sent to

Met by

### Sub Metamodels

The following list of sub topics displays all the sub metamodels available from the NOSA Version 0.5 metamodel.

Variable classification

### Variable classification

#### Variable classification

http://xml.noaa.gov/xml/metamodels/nosa_metamodel.kmd#oid3

#### Object Types

The following list of sub topics displays all the object types available from the Variable classification metamodel.

Category

Term

Topic

Variable

##### Category

http://xml.noaa.gov/xml/object_types/category.kmd#oid1

Default Symbol

Basetype

Modeling Object with name and description properties

Properties

Inherited properties

Modeling Object with name and description properties : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Modeling Object with name and description properties : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

##### Term

http://xml.noaa.gov/xml/object_types/term.kmd#oid1

Default Symbol

Basetype

Modeling Object with name and description properties

Properties

Inherited properties

Modeling Object with name and description properties : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Modeling Object with name and description properties : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

##### Topic

http://xml.noaa.gov/xml/object_types/topic.kmd#oid1

Default Symbol

Basetype

Modeling Object with name and description properties

Properties

Inherited properties

Modeling Object with name and description properties : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Modeling Object with name and description properties : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

##### Variable

http://xml.noaa.gov/xml/object_types/variable.kmd#oid1

Default Symbol

Basetype

Modeling Object with name and description properties

Properties

Standard

Property Name: standard

Property Type: string [Standard type: ANSI C++ string (short)]

Standard defining this parameter, e.g. GCMD.

Inherited properties

Modeling Object with name and description properties : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Modeling Object with name and description properties : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

#### NOSA View Style

http://xml.noaa.gov/xml/viewstyles/nosa_viewstyle.kmd#oid1

Type Views

| Object Type | Open Symbol | Closed Symbol |
| --- | --- | --- |
| Container |  |  |

#### Actual Measurement

http://xml.noaa.gov/xml/object_types/actual_measurement.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Has contact person | Person |
|  | Measures | Environmental Parameter |
|  | Supports | Goal |
|  | Characterizes | Environmental Phenomenon |
|  | Takes | Sensing Element |
|  | Met by | Required Measurement |

Properties

Actual Measurement Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name of this Environmental Parameter.

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Provide a short description of this measurement.

Measurement -> Algorithm

Property Name: measurement.algorithm

Property Type: text [Standard type: ANSI C++ string (long)]

Describe at a high level how this environmental parameter is derived. (E.g., Apparent Temperature is calculated using Air Temperature, Humidity, and Wind Speed.)

Measurement -> Measurement Unit

Property Name: measurement.unit

Property Type: Unit []

Specify the units this parameter is normally expressed in.

Measurement -> Minimum Measurement Value

Property Name: measurement.min

Property Type: float [Standard type: 64 bit floating point]

Specify the lower measurement bound (minimum) for this parameter expressed in the units selected in the Measurement Unit property.

Measurement -> Maximum Measurement Value

Property Name: measurement.max

Property Type: float [Standard type: 64 bit floating point]

Specify the upper measurement bound (maximum) for this parameter expressed in the units selected in the Measurement Unit property.

Measurement -> Representative Measurement Accuracy

Property Name: measurement.accuracy

Property Type: float [Standard type: 64 bit floating point]

For the given sensing element specify representative measurement accuracy. Measurement Accuracy represents the systematic error as specified by the difference between a measured or derived parameter and its true value in the absence of random errors.

Measurement -> Representative Measurement Precision

Property Name: measurement.precision

Property Type: float [Standard type: 64 bit floating point]

For the given sensing element specify representative measurement precision. Measurement Precision is the uncertainty in a measured or derived parameter due only to random errors, specified as the standard deviation of a parameter.

Measurement -> Representative Measurement Uncertainty

Property Name: measurement.uncertainty

Property Type: float [Standard type: 64 bit floating point]

For the given sensing element specify representative measurement uncertainty. Measurement Uncertainty is the root-mean-square of the measurement errors for an estimated parameter.

Environmental Parameter Timeline

Property Name: timeline

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the measurement timeline for this Environmental Parameter including sampling, integrating, and reporting process. Address latency and persistence if known.

Timeline Unit

Property Name: timelineUnit

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the timeline unit (e.g. seconds, minutes)

Reporting Frequency

Property Name: reportingFrequency

Property Type: float [Standard type: 64 bit floating point]

Provide the shortets Reporting Frequency (e.g. 10 minutes) for this Environmental Parameter.

Sample Frequency

Property Name: sampleFrequency

Property Type: float [Standard type: 64 bit floating point]

If different from Reporting Frequency, provide the shortest Sample Frequency for this Environmental Parameter.

Sampling Duration

Property Name: samplingDuration

Property Type: float [Standard type: 64 bit floating point]

Provide the shortest Sample Duration for this Environmental Parameter.

Measurement Stability

Property Name: measurementStability

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the Environmental Parameter stability addressing both the time needed to average over to ensure the averaging out of noise, but short enough so that meaningful analysis/trends/results are derived.

Measurement Extent

Property Name: measurementExtent

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the measurement extent (e.g. temperature at a single point, fish in a given area, soil moisture for a given cylinder with depth, or aerosol extinction as a function of sensing depth).

Mapping Uncertainty

Property Name: mappingUncertainty

Property Type: text [Standard type: ANSI C++ string (long)]

Specify necessary mapping (location and extent) accuracy for this envrionmental parameter.

Other Key Parameters

Property Name: otherKeyParameters

Property Type: text [Standard type: ANSI C++ string (long)]

Please define other key parameter properties your program uses that were not requested above that describe the measurement fidelity. Please review the remote sensing section for coverage related properties.

General Coverage Description

Property Name: coverage

Property Type: text [Standard type: ANSI C++ string (long)]

Point of Contact

Property Name: pointOfContact

Property Type: string [Standard type: ANSI C++ string (short)]

Use the Contact Person of relationship to establish a link to the contact person.

GIS Coverage Data

Property Name: gisCoverageData

Property Type: text [Standard type: ANSI C++ string (long)]

Driving Requirements

Property Name: drivingRequirements

Property Type: text [Standard type: ANSI C++ string (long)]

Key Questions

Property Name: keyQuestions

Property Type: string [Standard type: ANSI C++ string (short)]

Interested Community

Property Name: interestedCommunity

Property Type: string [Standard type: ANSI C++ string (short)]

Strategic Goals

Property Name: strategicGoals

Property Type: string [Standard type: ANSI C++ string (short)]

Use the Supports relationship to establish a link to one of the 7 strategic goals.

Other Needs

Property Name: otherNeeds

Property Type: text [Standard type: ANSI C++ string (long)]

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Methods

getContactPerson

http://xml.noaa.gov/xml/object_types/actual_measurement.kmd#oid11

getGoal

http://xml.noaa.gov/xml/object_types/actual_measurement.kmd#oid13

#### Sensor Type

http://xml.noaa.gov/xml/object_types/sensor_type.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Is type of | Sensing Element |

Properties

Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

#### Sensing Element

http://xml.noaa.gov/xml/object_types/sensing_element.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Supplied by | Organization |
|  | Contains | Observing System |
|  | Provides data for | Processing Element |
|  | Is type of | Sensor Type |
|  | Takes | Actual Measurement |

Properties

Sensing Element Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name of the Sensing Element

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Acronym

Property Name: acronym

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the acronym for this Sensing Element.

Identifier

Property Name: identifier

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the Sensing Element identifier, if applicable. If not enter NA. An example would be ASOS F420 wind sensor.

Sensor Type

Property Name: sensorType

Property Type: string [Standard type: ANSI C++ string (short)]

Indicate the type of sensor.

Measured Environmental Parameters

Property Name: environmentalParameter

Property Type: string [Standard type: ANSI C++ string (short)]

Identify the Environmental Parameters measured by this Sensing Element. This value is transferred from the name property of the associated Environmental Parameter(s)

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Provide a short description of this Sensing Element. Include its key characteristics, major modes of operation, upgrade plans, etc.

Reference Material 1

Property Name: refMaterial1

Property Type: url [Standard type: URL as xlink]

Enter a URL to a file containing reference material. Check 'Relative' if the URL is relative to the model location.

Reference Material 2

Property Name: refMaterial2

Property Type: url [Standard type: URL as xlink]

Enter a URL to a file containing reference material. Check 'Relative' if the URL is relative to the model location.

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Methods

getEnvironmentalParameter

http://xml.noaa.gov/xml/object_types/sensing_element.kmd#oid21

Open Reference Material 1

http://xml.noaa.gov/xml/object_types/sensing_element.kmd#oid10

Open Reference Material 2

http://xml.noaa.gov/xml/object_types/sensing_element.kmd#oid11

getSensorType

http://xml.noaa.gov/xml/object_types/sensing_element.kmd#oid23

#### Processing Element

http://xml.noaa.gov/xml/object_types/processing_element.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Transmits | Message |
|  | Provides data to | Data Handling System |
|  | Contains | Observing System |
|  | Provides data for | Sensing Element |
|  | Provides data directly to | Organization |

Properties

Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name of the Processing Element

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description for the Processing Element

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

#### Platform/Station

http://xml.noaa.gov/xml/object_types/platform_station.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Owns | Organization |
|  | Situated on | Observing System |
|  | Operates | Organization |
|  | Located at | Mobile Location Data |
|  | Located at | Fixed Location Data |
|  | Is type of | Platform/Station Type |
|  | Owner type | Organization |

Properties

Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name of the Platform or Station

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Acronym

Property Name: acronym

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the acronym that this Platform or Station is known by

Identifier

Property Name: identifier

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the Platform or Stations identifier, if any. For example, P3 is the identifier for the Orion aircraft.

Type

Property Name: type

Property Type: string [Standard type: ANSI C++ string (short)]

Identify the type of Platform or Station

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a short description of this Platform or Station. Describe how it is used in relation to the Observing System

Owner Type -> Commercial

Property Name: ownerType.commercial

Property Type: Boolean [Standard type: Boolean enum type]

Owner Type -> Foreign

Property Name: ownerType.foreign

Property Type: Boolean [Standard type: Boolean enum type]

Owner Type -> Military

Property Name: ownerType.military

Property Type: Boolean [Standard type: Boolean enum type]

Owner Type -> NOAA

Property Name: ownerType.noaa

Property Type: Boolean [Standard type: Boolean enum type]

Owner Type -> Other USG agency

Property Name: ownerType.usg

Property Type: Boolean [Standard type: Boolean enum type]

Owner Type -> University

Property Name: ownerType.university

Property Type: Boolean [Standard type: Boolean enum type]

Owner

Property Name: owner

Property Type: string [Standard type: ANSI C++ string (short)]

This value is transferred from the name property of the associated owner Organizations.

Operator

Property Name: operator

Property Type: string [Standard type: ANSI C++ string (short)]

This value is transferred from the name property of the associated operator Organizations.

Webpage

Property Name: webPage

Property Type: url [Standard type: URL as xlink]

Identify the URL for a webpage that describes this Platform or Station

Operational Regime

Property Name: regime

Property Type: Operational Regime []

Identify where the platform primarily operates

Spatial Coordinates (Mobile)

Property Name: spatialCoordinates

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the locations where the platform operates in terms of the boundaries of its travel. The transit region can be described in terms of geographic feature (eg, North Atlantic), bounds (eg, between 35 and 45 N Lat), corners (eg, NW corner at 35N/45E, etc), track (eg, starts at point X and ends at point Y), or satellite ephemeris data.

Temporal Characteristics (Mobile)

Property Name: temporalCharacteristics

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the characteristics of the mobile platform in terms of velocity profiles, transit times, revisit times, and so on.

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Methods

Open Webpage

http://xml.noaa.gov/xml/object_types/platform_station.kmd#oid10

getOrganizationOperator

http://xml.noaa.gov/xml/object_types/platform_station.kmd#oid25

getOrganizationOwner

http://xml.noaa.gov/xml/object_types/platform_station.kmd#oid23

getPlatformStationType

http://xml.noaa.gov/xml/object_types/platform_station.kmd#oid27

#### Phenomenon Requirement

http://xml.noaa.gov/xml/object_types/phenomenon_requirement.kmd#oid1

Default Symbol

Basetype

Requirement

Properties

Source Document

Property Name: sourceDocument

Property Type: url [Standard type: URL as xlink]

Enter the URL for the source document for this requirement, if one exists. Check 'Relative' if the URL is relative to the model location.

Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Inherited properties

General Requirement : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

General Requirement : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

General Requirement : No Label

Property Name: priority

Property Type: Priority []

Priority of the requirement.

General Requirement : No Label

Property Name: Color

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Methods

Open Source Document

http://xml.noaa.gov/xml/object_types/phenomenon_requirement.kmd#oid10

#### Organization

http://xml.noaa.gov/xml/object_types/organization.kmd#oid1

Default Symbol

Basetype

Organization

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Supplied by | Sensing Element |
|  | Has contact person | Person |
|  | User | Observing System |
|  | Primary stakeholder | Observing System |
|  | Drives | Environmental Phenomenon |
|  | Drives | Environmental Parameter |
|  | Secondary stakeholder | Observing System |
|  | Downstream user | Observing System |

Properties

DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Organization Code

Property Name: orgCode

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the Organization Code.

Mail Code

Property Name: mailCode

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the Mail Code.

Inherited properties

Organization : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Organization : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

Organization : Phone no.

Property Name: phoneNr

Property Type: PhoneNr [Phone Nr]

Telephone number used to contact the organization

Organization : Fax no.

Property Name: faxNr

Property Type: PhoneNr [Phone Nr]

Fax number

Organization : Country

Property Name: country

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Organization : X-400

Property Name: x400

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Organization : E-mail

Property Name: email

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Organization : City

Property Name: city

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Organization : Address 2 (Po. Box)

Property Name: address2

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Organization : URL

Property Name: url

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Organization : Manager

Property Name: manager

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

The name of the person responsible for the organization. This property will automatically be filled in if you connect a Person object with the Is Manager For relationships.

Organization : ZIP

Property Name: zip

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Zip code part of the address

Organization : Address 1

Property Name: address1

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Organization : Homepage

Property Name: homepage

Property Type: string [Standard type: ANSI C++ string (short)]

Homepage URL

Organization : Int/Ext Classification

Property Name: type

Property Type: Int Ext Classification []

Organization : Image

Property Name: image

Property Type: string [Standard type: ANSI C++ string (short)]

Criteria

Find Owned Observing Systems

http://xml.noaa.gov/xml/criteria/criteria.kmd#oid4

Find Primary Stakeholder Of Organization

http://xml.noaa.gov/xml/criteria/criteria.kmd#oid1

#### Observing System

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Area of Operations | Location |
|  | Funds | Organization |
|  | Funds | Observing System |
|  | Funds | Observing System |
|  | User | Organization |
|  | User | Group |
|  | Primary stakeholder | Organization |
|  | Primary stakeholder | Group |
|  | Owns | Organization |
|  | Situated on | Platform/Station |
|  | Contains | Sensing Element |
|  | Contains | Processing Element |
|  | Supports | Goal |
|  | Supports | Organization |
|  | Operates | Organization |
|  | Secondary stakeholder | Organization |
|  | Secondary stakeholder | Group |
|  | Downstream user | Organization |
|  | Downstream user | Group |
|  | Acquires | Organization |
|  | Is controlled by | Observation Control System |
|  | Part of | Observing System |
|  | Part of | Observing System |

Properties

System Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name of this observing system.

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Acronym

Property Name: acronym

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the acronym that this system is known by.

Identifier

Property Name: identifier

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the systems identifier, if any. For example, WSR-88D is the identifier for NEXRAD.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the system.

Primary Mission Need Statement

Property Name: primaryMissionNeed

Property Type: text [Standard type: ANSI C++ string (long)]

Provide a short (5-20 words) description of the mission supported by this system. (This should not be a repeat of the strategic goals).

Secondary Mission Need Statement

Property Name: secondaryMissionNeed

Property Type: text [Standard type: ANSI C++ string (long)]

Provide a short (5-20 words) description of any secondary missions supported by this system.

System Purpose

Property Name: systemPurpose

Property Type: text [Standard type: ANSI C++ string (long)]

Provide a short (10-20 words) description of what the system is used for. (This is a lower level than the system's mission(s) above.)

Life Cycle Phase

Property Name: lifeCyclePhase

Property Type: Life Cycle Phase []

Select the term that best describes the state of this system.

Key Characteristics

Property Name: keyCharacteristics

Property Type: text [Standard type: ANSI C++ string (long)]

Describe key characteristics (components, size, system types, etc.). Identify key performance parameters.

Major Modes of Operation

Property Name: modesOfOperation

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the typical ways in which this system is operated. Identify the normal modes and any out-of-normal modes.

Growth Plans

Property Name: growthPlans

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the plans and desires for upgrade and/or expansion of the system.

System Picture URL

Property Name: systemPicture

Property Type: url [Standard type: URL as xlink]

Attach a file containing a graphic or picture of the system. Check 'Relative' if the URL is relative to the model location.

Block Diagram URL

Property Name: blockDiagram

Property Type: url [Standard type: URL as xlink]

Attach a file containing a block diagram of the system. Check 'Relative' if the URL is relative to the model location.

Schedule URL

Property Name: schedule

Property Type: url [Standard type: URL as xlink]

Attach a file containing acquisition and/or operations schedules for this system. Check 'Relative' if the URL is relative to the model location.

Webpage URL

Property Name: referenceUrl

Property Type: url [Standard type: URL as xlink]

If one exists, enter the URL for the systems web site. The entire URL needs to be specified, i.e. http://www.refdesk.com/.

Key Performance Parameters

Property Name: keyPerformanceParameters

Property Type: text [Standard type: ANSI C++ string (long)]

List the most important system parameters directly related to the primary mission.

System Type

Property Name: systemType

Property Type: System Type [Type of system]

Identify type of system.

Primary Stakeholders

Property Name: primaryStakeholders

Property Type: string [Standard type: ANSI C++ string (short)]

List the mosts important groups or organizations impacted by this system. This value is transferred from the name property of the associated primary stakeholder Organizations.

Secondary Stakeholders

Property Name: secondaryStakeholders

Property Type: string [Standard type: ANSI C++ string (short)]

List other groups or organizations that are impacted by this system. This value is transferred from the name property of the associated secondary stakeholder Organizations.

Users of System Data

Property Name: users

Property Type: string [Standard type: ANSI C++ string (short)]

List groups or organizations that use the data directly from this system. This value is transferred from the name property of the associated user Organizations.

Downstream Users

Property Name: downstreamUsers

Property Type: string [Standard type: ANSI C++ string (short)]

List the groups or organizations that make use of data products derived from the direct system output. This value is transferred from the name property of the associated downstream user Organizations.

Observational Scope -> Local

Property Name: observationalScope.local

Property Type: Boolean [Standard type: Boolean enum type]

Observational Scope -> National

Property Name: observationalScope.national

Property Type: Boolean [Standard type: Boolean enum type]

Observational Scope -> Regional

Property Name: observationalScope.regional

Property Type: Boolean [Standard type: Boolean enum type]

Observational Scope -> Global

Property Name: observationalScope.global

Property Type: Boolean [Standard type: Boolean enum type]

Observational Scope -> Space

Property Name: observationalScope.space

Property Type: Boolean [Standard type: Boolean enum type]

Decisionmaking -> Local

Property Name: decisionMaking.local

Property Type: Boolean [Standard type: Boolean enum type]

Decisionmaking -> National

Property Name: decisionMaking.national

Property Type: Boolean [Standard type: Boolean enum type]

Decisionmaking -> Regional

Property Name: decisionMaking.regional

Property Type: Boolean [Standard type: Boolean enum type]

Decisionmaking -> Global

Property Name: decisionMaking.global

Property Type: Boolean [Standard type: Boolean enum type]

Quantity Deployed (Current)

Property Name: qtyDeployed

Property Type: integer [Standard type: 32 bit signed integer]

How many Observing Systems are currently deployed?

Quantity Deployed (Programmed)

Property Name: qtyPlannedDeployed

Property Type: integer [Standard type: 32 bit signed integer]

How many additional Observing Systems will be deployed to the 03/04 budget plan?

Quantity Needed

Property Name: qtyNeeded

Property Type: integer [Standard type: 32 bit signed integer]

How many Observing Systems total are needed to meet the requirements of users?

Funding - Organizations

Property Name: funding1

Property Type: string [Standard type: ANSI C++ string (short)]

List all organizations that contribute funds to this observing system. This value is transferred from the name property of the associated funding Organizations.

Funding - Higher Level Systems

Property Name: funding2

Property Type: string [Standard type: ANSI C++ string (short)]

List any higher level systems that contribute to the funding of this observing system. This value is transferred from the name property of the associated funding Higher Systems.

Acquiring Organization

Property Name: acquiringOrganization

Property Type: string [Standard type: ANSI C++ string (short)]

List the name of the organization responsible for acquisition of this observing syhstem. This value is transferred from the name property of the associated acquiring Organization(s).

Acquisition Authority

Property Name: acquisitionAuthority

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the authority under which this system is being acquired, for example, a congressional mandate, or support for a NOAA strategic goal.

Initial Acquisition Cost

Property Name: initialAcquisitionCost

Property Type: integer [Standard type: 32 bit signed integer]

Acquisition Cost Type

Property Name: costType

Property Type: Cost Type []

Specify if this cost is per unit, or total.

Subsequent Upgrade Cost

Property Name: subsequentUpgradeCost

Property Type: integer [Standard type: 32 bit signed integer]

OandM Costs -> Total

Property Name: omCosts.total

Property Type: integer [Standard type: 32 bit signed integer]

Enter the total cost, or the cost for each subelement.

OandM Costs -> Equipment

Property Name: omCosts.equipment

Property Type: integer [Standard type: 32 bit signed integer]

OandM Costs -> Personnel

Property Name: omCosts.personnel

Property Type: integer [Standard type: 32 bit signed integer]

OandM Costs -> Facilities

Property Name: omCosts.facilities

Property Type: integer [Standard type: 32 bit signed integer]

OandM Costs -> Data

Property Name: omCosts.data

Property Type: integer [Standard type: 32 bit signed integer]

OandM Costs -> Services

Property Name: omCosts.services

Property Type: integer [Standard type: 32 bit signed integer]

OandM Costs -> Other

Property Name: omCosts.other

Property Type: integer [Standard type: 32 bit signed integer]

Enter the sum total of O&M costs not covered by the previous properties. Describe the nature of these costs in the comment area for this property.

Schedule -> IOC

Property Name: scheduleDate.ioc

Property Type: Date [Date represented as YYYYMMDD]

Enter the date that this system achieved IOC (Initial Operating Capability)

Schedule -> FOC

Property Name: scheduleDate.foc

Property Type: Date [Date represented as YYYYMMDD]

Enter the date that this system achieved (or will achieve) FOC (Full Operating Capability)

Schedule -> EOFC

Property Name: scheduleDate.eofc

Property Type: Date [Date represented as YYYYMMDD]

Enter the date (or projected date) at which this system no longer perform at full specified capabilty, known as the EOFC (End of Full Capability)

Schedule -> EOL

Property Name: scheduleDate.eol

Property Type: Date [Date represented as YYYYMMDD]

Enter the projected date of End of Life (EOL) for this system

Capital Asset Plan

Property Name: capitalAssetPlan

Property Type: url [Standard type: URL as xlink]

Attach a file containing the Capital Asset Plan for this system if one exists. Check 'Relative' if the URL is relative to the model location.

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Methods

getOrganizationAcquires

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid33

getOrganizationDownstreamUser

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid31

Open Reference URL

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid10

Open System Picture

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid11

Open Block Diagram

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid12

Open Schedule

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid13

getOrganizationFunds

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid23

getHigherSystem

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid21

getOrganizationPrimaryStakeholder

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid25

getOrganizationSecondaryStakeholder

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid27

getOrganizationUser

http://xml.noaa.gov/xml/object_types/observing_system.kmd#oid29

#### Observation Control System

http://xml.noaa.gov/xml/object_types/observation_control_system.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Use | Business Process |
|  | Is controlled by | Observing System |

Properties

Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

#### Mobile Location Data

http://xml.noaa.gov/xml/object_types/location.kmd#oid1

Default Symbol

Basetype

Location Data

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Located at | Platform/Station |

Properties

Location Matrix data

Property Name: locations

Property Type: Mobile Locations []

Fixed location coordinates for each Station or Platform.

Inherited properties

Location Data : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Location Data : Location Matrix file reference

Property Name: locationMatrix

Property Type: url [Standard type: URL as xlink]

Enter the URL of a file that specifies the location data of each Station or Platform. Locations should be specified in terms of the appropriate location data.

Location Data : Webpage URL

Property Name: webPage

Property Type: url [Standard type: URL as xlink]

If one exists, enter the URL for the locations web site. The entire URL needs to be specified, i.e. http://www.refdesk.com/. Check 'Relative' if the URL is relative to the model location.

Location Data : Picture URL

Property Name: picture

Property Type: url [Standard type: URL as xlink]

If one exists, enter the URL for the locations picture. The entire URL needs to be specified, i.e. http://www.refdesk.com/pictures/image.jpg. Check 'Relative' if the URL is relative to the model location.

Inherited properties

Location : Street

Property Name: street

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Street address

Location : Zip Code

Property Name: zipCode

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Zip code

Location : Post Box

Property Name: postBox

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Postal Box address section

Location : City

Property Name: city

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

City, Town, or equivalent

Inherited properties

Location Type : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Location Type : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

#### Required Measurement

http://xml.noaa.gov/xml/object_types/measurement_requirement.kmd#oid1

Default Symbol

Basetype

Requirement

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Applies to | Environmental Parameter |
|  | Supports | Goal |
|  | Met by | Actual Measurement |

Properties

Measurement -> Algorithm

Property Name: measurement.algorithm

Property Type: text [Standard type: ANSI C++ string (long)]

Describe at a high level how this environmental parameter is derived. (E.g., Apparent Temperature is calculated using Air Temperature, Humidity, and Wind Speed.)

Measurement -> Measurement Unit

Property Name: measurement.unit

Property Type: Unit []

Specify the units this parameter is normally expressed in.

Measurement -> Minimum Measurement Value

Property Name: measurement.min

Property Type: float [Standard type: 64 bit floating point]

Specify the lower measurement bound (minimum) for this parameter expressed in the units selected in the Measurement Unit property.

Measurement -> Maximum Measurement Value

Property Name: measurement.max

Property Type: float [Standard type: 64 bit floating point]

Specify the upper measurement bound (maximum) for this parameter expressed in the units selected in the Measurement Unit property.

Measurement -> Representative Measurement Accuracy

Property Name: measurement.accuracy

Property Type: float [Standard type: 64 bit floating point]

For the given sensing element specify representative measurement accuracy. Measurement Accuracy represents the systematic error as specified by the difference between a measured or derived parameter and its true value in the absence of random errors.

Measurement -> Representative Measurement Precision

Property Name: measurement.precision

Property Type: float [Standard type: 64 bit floating point]

For the given sensing element specify representative measurement precision. Measurement Precision is the uncertainty in a measured or derived parameter due only to random errors, specified as the standard deviation of a parameter.

Measurement -> Representative Measurement Uncertainty

Property Name: measurement.uncertainty

Property Type: float [Standard type: 64 bit floating point]

For the given sensing element specify representative measurement uncertainty. Measurement Uncertainty is the root-mean-square of the measurement errors for an estimated parameter.

Environmental Parameter Timeline

Property Name: timeline

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the measurement timeline for this Environmental Parameter including sampling, integrating, and reporting process. Address latency and persistence if known.

Timeline Unit

Property Name: timelineUnit

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the timeline unit (e.g. seconds, minutes)

Reporting Frequency

Property Name: reportingFrequency

Property Type: float [Standard type: 64 bit floating point]

Provide the shortets Reporting Frequency (e.g. 10 minutes) for this Environmental Parameter.

Sample Frequency

Property Name: sampleFrequency

Property Type: float [Standard type: 64 bit floating point]

If different from Reporting Frequency, provide the shortest Sample Frequency for this Environmental Parameter.

Sampling Duration

Property Name: samplingDuration

Property Type: float [Standard type: 64 bit floating point]

Provide the shortest Sample Duration for this Environmental Parameter.

Measurement Stability

Property Name: measurementStability

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the Environmental Parameter stability addressing both the time needed to average over to ensure the averaging out of noise, but short enough so that meaningful analysis/trends/results are derived.

Measurement Extent

Property Name: measurementExtent

Property Type: text [Standard type: ANSI C++ string (long)]

Describe the measurement extent (e.g. temperature at a single point, fish in a given area, soil moisture for a given cylinder with depth, or aerosol extinction as a function of sensing depth).

Mapping Uncertainty

Property Name: mappingUncertainty

Property Type: text [Standard type: ANSI C++ string (long)]

Specify necessary mapping (location and extent) accuracy for this envrionmental parameter.

Other Key Parameters

Property Name: otherKeyParameters

Property Type: text [Standard type: ANSI C++ string (long)]

Please define other key parameter properties your program uses that were not requested above that describe the measurement fidelity. Please review the remote sensing section for coverage related properties.

General Coverage Description

Property Name: coverage

Property Type: text [Standard type: ANSI C++ string (long)]

GIS Coverage Data

Property Name: gisCoverageData

Property Type: text [Standard type: ANSI C++ string (long)]

Driving Requirements

Property Name: drivingRequirements

Property Type: text [Standard type: ANSI C++ string (long)]

Key Questions

Property Name: keyQuestions

Property Type: string [Standard type: ANSI C++ string (short)]

Interested Community

Property Name: interestedCommunity

Property Type: string [Standard type: ANSI C++ string (short)]

Strategic Goals

Property Name: strategicGoals

Property Type: string [Standard type: ANSI C++ string (short)]

Other Needs

Property Name: otherNeeds

Property Type: text [Standard type: ANSI C++ string (long)]

Inherited properties

General Requirement : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

General Requirement : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

General Requirement : No Label

Property Name: priority

Property Type: Priority []

Priority of the requirement.

General Requirement : No Label

Property Name: Color

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Methods

getGoal

http://xml.noaa.gov/xml/object_types/measurement_requirement.kmd#oid13

#### Location

http://xml.noaa.gov/xml/object_types/location.kmd#oid9

Default Symbol

Basetype

Location

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Area of Operations | Observing System |

Properties

Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Location Code

Property Name: locationCode

Property Type: string [Standard type: ANSI C++ string (short)]

Address 1

Property Name: address1

Property Type: string [Standard type: ANSI C++ string (short)]

Address 2

Property Name: address2

Property Type: string [Standard type: ANSI C++ string (short)]

City

Property Name: city

Property Type: string [Standard type: ANSI C++ string (short)]

State/Province

Property Name: stateProvince

Property Type: string [Standard type: ANSI C++ string (short)]

Postal Code

Property Name: postalCode

Property Type: string [Standard type: ANSI C++ string (short)]

Country

Property Name: country

Property Type: string [Standard type: ANSI C++ string (short)]

Phone No.

Property Name: phoneNr

Property Type: string [Standard type: ANSI C++ string (short)]

Inherited properties

Location : Street

Property Name: street

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Street address

Location : Zip Code

Property Name: zipCode

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Zip code

Location : Post Box

Property Name: postBox

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Postal Box address section

Location : City

Property Name: city

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

City, Town, or equivalent

Inherited properties

Location Type : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Location Type : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

#### Group

http://xml.noaa.gov/xml/object_types/group.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | User | Observing System |
|  | Primary stakeholder | Observing System |
|  | Secondary stakeholder | Observing System |
|  | Downstream user | Observing System |

Properties

Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

#### Fixed Location Data

http://xml.noaa.gov/xml/object_types/location.kmd#oid2

Default Symbol

Basetype

Location Data

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Located at | Platform/Station |

Properties

Location Matrix data

Property Name: locations

Property Type: Fixed Locations []

Fixed location coordinates for each Station or Platform.

Inherited properties

Location Data : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Location Data : Location Matrix file reference

Property Name: locationMatrix

Property Type: url [Standard type: URL as xlink]

Enter the URL of a file that specifies the location data of each Station or Platform. Locations should be specified in terms of the appropriate location data.

Location Data : Webpage URL

Property Name: webPage

Property Type: url [Standard type: URL as xlink]

If one exists, enter the URL for the locations web site. The entire URL needs to be specified, i.e. http://www.refdesk.com/. Check 'Relative' if the URL is relative to the model location.

Location Data : Picture URL

Property Name: picture

Property Type: url [Standard type: URL as xlink]

If one exists, enter the URL for the locations picture. The entire URL needs to be specified, i.e. http://www.refdesk.com/pictures/image.jpg. Check 'Relative' if the URL is relative to the model location.

Inherited properties

Location : Street

Property Name: street

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Street address

Location : Zip Code

Property Name: zipCode

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Zip code

Location : Post Box

Property Name: postBox

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Postal Box address section

Location : City

Property Name: city

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

City, Town, or equivalent

Inherited properties

Location Type : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Location Type : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

#### Environmental Product

http://xml.noaa.gov/xml/object_types/environmental_product.kmd#oid1

Default Symbol

Basetype

Product Item

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Reported to | Environmental Phenomenon |

Properties

DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Inherited properties

General Product : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

General Product : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

#### Environmental Phenomenon

http://xml.noaa.gov/xml/object_types/environmental_phenomenon.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Characterizes | Data Handling System |
|  | Characterizes | Actual Measurement |
|  | Characterizes | Environmental Parameter |
|  | Drives | Organization |
|  | Drives | Objective |
|  | Drives | Goal |
|  | Reported to | Environmental Product |
|  | Leads to | Characterization requirement |

Properties

Phenomenon Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Acronym

Property Name: acronym

Property Type: string [Standard type: ANSI C++ string (short)]

Identifier

Property Name: identifier

Property Type: string [Standard type: ANSI C++ string (short)]

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Environmental Products

Property Name: environmentalProducts

Property Type: string [Standard type: ANSI C++ string (short)]

Identify the environmental products where this phenomenon is typically reported. This value is transferred from the name property of the associated Environmental Product(s)

Driving Requirements

Property Name: drivingRequirements

Property Type: string [Standard type: ANSI C++ string (short)]

If known, the driving requirements measuring this phenomenon will contribute to addressing. Provide the source, source documentation, and general description of the requirement. This value is transferred from the name property of the associated Phenomenon Requirement(s).

Key Questions

Property Name: keyQuestions

Property Type: text [Standard type: ANSI C++ string (long)]

If known, list key questions measuring this phenomenon will contribute to addressing.

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Methods

getDrivingRequirements

http://xml.noaa.gov/xml/object_types/environmental_phenomenon.kmd#oid23

getEnvironmentalProducts

http://xml.noaa.gov/xml/object_types/environmental_phenomenon.kmd#oid21

#### Environmental Parameter

http://xml.noaa.gov/xml/object_types/environmental_parameter.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Applies to | Required Measurement |
|  | Measures | Actual Measurement |
|  | Characterizes | Environmental Phenomenon |
|  | Drives | Organization |
|  | Drives | Objective |
|  | Drives | Goal |

Properties

Environmental Parameter Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name of this Environmental Parameter.

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Provide a short description of this parameter.

Associated Environmental Phenomena

Property Name: environmentalPhenomena

Property Type: string [Standard type: ANSI C++ string (short)]

Identify all the Environmental Phenomena that this environmental parameter is used to characterize. This value is transferred from the name property of the associated Environmental Phenomena

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

Criteria

Find Observing Systems

http://xml.noaa.gov/xml/criteria/criteria.kmd#oid3

Methods

getEnvironmentalPhenomenon

http://xml.noaa.gov/xml/object_types/environmental_parameter.kmd#oid21

#### Data Handling System

http://xml.noaa.gov/xml/object_types/data_handling_system.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Use | Business Process |
|  | Provides info to | Organization |
|  | Provides data to | Processing Element |
|  | Characterizes | Environmental Phenomenon |
|  | Sent to | Message |

Properties

System Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name of the Data Handling System.

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Acronym

Property Name: acronym

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the acronym of the Data Handling System.

Identifier

Property Name: identifier

Property Type: string [Standard type: ANSI C++ string (short)]

Enter the identifier of the Data Handling System.

System Type

Property Name: systemType

Property Type: string [Standard type: ANSI C++ string (short)]

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description for the Data Handling System.

Models

Property Name: models

Property Type: text [Standard type: ANSI C++ string (long)]

Are any of the collected observations used in operationsl models? If so, describe what is used, how it is used, and what models are used.

Data Collection

Property Name: dataCollection

Property Type: text [Standard type: ANSI C++ string (long)]

How is data collected and/or ingested?

Data Request

Property Name: dataRequest

Property Type: text [Standard type: ANSI C++ string (long)]

How is data requested by users?

Product Generation

Property Name: productGeneration

Property Type: text [Standard type: ANSI C++ string (long)]

How and how often are products generated?

Data Archival

Property Name: dataArchival

Property Type: text [Standard type: ANSI C++ string (long)]

How is data archived, and for how long is it archived?

Data Dissemination

Property Name: dataDissemination

Property Type: text [Standard type: ANSI C++ string (long)]

How is data disseminated and made accessible to users?

Metadata

Property Name: metadata

Property Type: text [Standard type: ANSI C++ string (long)]

What metadata is needed by models, report generators, archival systems, users, etc?

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

#### Characterization requirement

http://xml.noaa.gov/xml/object_types/characterization_requirement.kmd#oid1

Default Symbol

Basetype

Requirement

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Leads to | Environmental Phenomenon |

Properties

Inherited properties

General Requirement : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

General Requirement : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

General Requirement : No Label

Property Name: priority

Property Type: Priority []

Priority of the requirement.

General Requirement : No Label

Property Name: Color

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

#### Platform/Station Type

http://xml.noaa.gov/xml/object_types/platform_station_type.kmd#oid1

Default Symbol

Basetype

NOSA Abstract Basetype

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Is type of | Platform/Station |

Properties

Platform/Station Type Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter the name for the Platform/Station Type.

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description for the Platform/Station Type.

Inherited properties

NOSA Abstract Basetype : DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Unique identifier of this object. Used for the Database Interface import.

#### Message

http://xml.noaa.gov/xml/object_types/message.kmd#oid1

Default Symbol

Basetype

Message

Connections with other Object Types

| Direction | Relationship Type | Object Type |
| --- | --- | --- |
|  | Transmits | Processing Element |
|  | Sent to | Data Handling System |

Properties

DBIF Identifier

Property Name: id

Property Type: string [Standard type: ANSI C++ string (short)]

Media Formats - Hardcopy -> Imagery/Photographs

Property Name: mediaFormatsHardcopy.imageryPhotographs

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Hardcopy -> Maps

Property Name: mediaFormatsHardcopy.maps

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Hardcopy -> Microfiche

Property Name: mediaFormatsHardcopy.microfiche

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Hardcopy -> Printer report

Property Name: mediaFormatsHardcopy.printerReport

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Hardcopy -> Reports

Property Name: mediaFormatsHardcopy.reports

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Digital -> Compact Discs

Property Name: mediaFormatsDigital.compactDiscs

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Digital -> Databases

Property Name: mediaFormatsDigital.databases

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Digital -> Files

Property Name: mediaFormatsDigital.files

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Digital -> Spreadsheets

Property Name: mediaFormatsDigital.spreadsheets

Property Type: Boolean [Standard type: Boolean enum type]

Media Formats - Digital -> Voice

Property Name: mediaFormatsDigital.voice

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> ASCII

Property Name: dataFormat.ascii

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> CDF

Property Name: dataFormat.cdf

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> HDF

Property Name: dataFormat.hdf

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> HTML and FTP protocols

Property Name: dataFormat.htmlFtp

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> Level-1B

Property Name: dataFormat.level1b

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> NetCDF

Property Name: dataFormat.netCdf

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> Voice

Property Name: dataFormat.voice

Property Type: Boolean [Standard type: Boolean enum type]

Data Format -> XLS

Property Name: dataFormat.xls

Property Type: Boolean [Standard type: Boolean enum type]

Metadata

Property Name: metadata

Property Type: text [Standard type: ANSI C++ string (long)]

Data Processing -> Derived Products

Property Name: dataProcessing.derivedProducts

Property Type: Boolean [Standard type: Boolean enum type]

Data Processing -> Raw Observations

Property Name: dataProcessing.rawObservations

Property Type: Boolean [Standard type: Boolean enum type]

Data Processing -> Reviewed Observations

Property Name: dataProcessing.reviewedObservations

Property Type: Boolean [Standard type: Boolean enum type]

Data Relay - Method

Property Name: dataRelayMethod

Property Type: text [Standard type: ANSI C++ string (long)]

Data Relay - Timing

Property Name: dataRelayTiming

Property Type: string [Standard type: ANSI C++ string (short)]

Data Access -> Customer Order via phone/fax

Property Name: dataAccess.customerOrder

Property Type: Boolean [Standard type: Boolean enum type]

Data Access -> Dedicated Connections

Property Name: dataAccess.dedicatedConnection

Property Type: Boolean [Standard type: Boolean enum type]

Data Access -> FTP Site

Property Name: dataAccess.ftpSite

Property Type: Boolean [Standard type: Boolean enum type]

Data Access -> Web Pages

Property Name: dataAccess.webPages

Property Type: Boolean [Standard type: Boolean enum type]

Data Retention - Local

Property Name: dataRetentionLocal

Property Type: string [Standard type: ANSI C++ string (short)]

Data Retention - Period

Property Name: dataRetentionPeriod

Property Type: string [Standard type: ANSI C++ string (short)]

Data Retention - Latency

Property Name: dataRetentionLatency

Property Type: string [Standard type: ANSI C++ string (short)]

Inherited properties

Message : Name

Property Name: name

Property Type: Name with Rich Text [Standard type: ANSI C++ string (short)]

Enter a short name for the object

Note that this property is also the label property. The value for the label property can be displayed on a tooltip (when the preference is set in Metis) and is usually displayed on the symbol. The value of this property is also displayed for the object in the Object and View trees.

Message : Description

Property Name: description

Property Type: Description with Rich Text [Standard type: ANSI C++ string (long)]

Enter a description of the object

#### Applies to

http://xml.noaa.gov/xml/relationship_types/applies_to.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Required Measurement | Environmental Parameter |

#### Transmits

http://xml.noaa.gov/xml/relationship_types/transmits.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Processing Element | Message |

#### Supplied by

http://xml.noaa.gov/xml/relationship_types/supplied_by.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Sensing Element | Organization |

#### Use

http://xml.noaa.gov/xml/relationship_types/use.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Business Process | Data Handling System |
| Business Process | Observation Control System |

#### Area of Operations

http://xml.noaa.gov/xml/relationship_types/area_of_operations.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Observing System | Location |

#### Has contact person

http://xml.noaa.gov/xml/relationship_types/has_contact_person.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Actual Measurement | Person |
| Organization | Person |

#### Provides info to

http://xml.noaa.gov/xml/relationship_types/provides_info_to.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Data Handling System | Organization |

#### Provides data to

http://xml.noaa.gov/xml/relationship_types/provides_data_to.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Processing Element | Data Handling System |

#### Funds

http://xml.noaa.gov/xml/relationship_types/funds.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Observing System |
| Observing System | Observing System |

#### User

http://xml.noaa.gov/xml/relationship_types/user.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Observing System |
| Group | Observing System |

#### Primary stakeholder

http://xml.noaa.gov/xml/relationship_types/primary_stakeholder.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Observing System |
| Group | Observing System |

#### Owns

http://xml.noaa.gov/xml/relationship_types/owns_owned-by.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Platform/Station |
| Organization | Observing System |

#### Situated on

http://xml.noaa.gov/xml/relationship_types/situated-on.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Observing System | Platform/Station |

#### Contains

http://xml.noaa.gov/xml/relationship_types/contains.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Observing System | Sensing Element |
| Observing System | Processing Element |

#### Provides data for

http://xml.noaa.gov/xml/relationship_types/provides_data_for.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Sensing Element | Processing Element |

#### Measures

http://xml.noaa.gov/xml/relationship_types/measures.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Actual Measurement | Environmental Parameter |

#### Supports

http://xml.noaa.gov/xml/relationship_types/supports.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Observing System | Goal |
| Actual Measurement | Goal |
| Required Measurement | Goal |
| Organization | Observing System |

#### Operates

http://xml.noaa.gov/xml/relationship_types/operates.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Platform/Station |
| Organization | Observing System |

#### Characterizes

http://xml.noaa.gov/xml/relationship_types/characterizes.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Data Handling System | Environmental Phenomenon |
| Environmental Phenomenon | Actual Measurement |
| Environmental Phenomenon | Environmental Parameter |

#### Drives

http://xml.noaa.gov/xml/relationship_types/drives.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Environmental Phenomenon |
| Objective | Environmental Phenomenon |
| Goal | Environmental Phenomenon |
| Organization | Environmental Parameter |
| Objective | Environmental Parameter |
| Goal | Environmental Parameter |

#### Reported to

http://xml.noaa.gov/xml/relationship_types/reported_to.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Environmental Phenomenon | Environmental Product |

#### Located at

http://xml.noaa.gov/xml/relationship_types/located_at.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Platform/Station | Mobile Location Data |
| Platform/Station | Fixed Location Data |

#### Secondary stakeholder

http://xml.noaa.gov/xml/relationship_types/secondary_stakeholder.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Observing System |
| Group | Observing System |

#### Downstream user

http://xml.noaa.gov/xml/relationship_types/downstream_user.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Observing System |
| Group | Observing System |

#### Acquires

http://xml.noaa.gov/xml/relationship_types/acquires.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Organization | Observing System |

#### Provides data directly to

http://xml.noaa.gov/xml/relationship_types/provides_data_directly_to.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Processing Element | Organization |

#### Is controlled by

http://xml.noaa.gov/xml/relationship_types/is_controlled_by.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Observing System | Observation Control System |

#### Part of

http://xml.noaa.gov/xml/relationship_types/part_of.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Observing System | Observing System |

#### Is type of

http://xml.noaa.gov/xml/relationship_types/is_type_of.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Platform/Station | Platform/Station Type |
| Sensing Element | Sensor Type |

#### Leads to

http://xml.noaa.gov/xml/relationship_types/leads_to.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Characterization requirement | Environmental Phenomenon |

#### Takes

http://xml.noaa.gov/xml/relationship_types/takes_measurement.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Sensing Element | Actual Measurement |

#### Sent to

http://xml.noaa.gov/xml/relationship_types/sent_to.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Message | Data Handling System |

#### Met by

http://xml.noaa.gov/xml/relationship_types/met_by.kmd#oid1

Basetype

Relationship with shared from and to texts

Constraints

| From | To |
| --- | --- |
| Required Measurement | Actual Measurement |

## Not Found

The object type is not found in this reference guide.

Background

Relationship types may have constraints with references to object types that are part of the Common Modeling Library, but not available from any of the domains in this template. These object types are not documented in this reference guide.

Index

C

Criterium

Find Observing Systems (Environmental Parameter)	57

Find Owned Observing Systems (Organization)	29

Find Primary Stakeholder Of Organization (Organization)	29

M

Metamodel

NOSA Version 0.5	4

Variable classification	7

Method

getContactPerson (Actual Measurement)	13

getDrivingRequirements (Environmental Phenomenon)	55

getEnvironmentalParameter (Sensing Element)	19

getEnvironmentalPhenomenon (Environmental Parameter)	57

getEnvironmentalProducts (Environmental Phenomenon)	55

getGoal (Actual Measurement)	13

getGoal (Required Measurement)	44

getHigherSystem (Observing System)	32

getOrganizationAcquires (Observing System)	32

getOrganizationDownstreamUser (Observing System)	32

getOrganizationFunds (Observing System)	32

getOrganizationOperator (Platform/Station)	23

getOrganizationOwner (Platform/Station)	23

getOrganizationPrimaryStakeholder (Observing System)	32

getOrganizationSecondaryStakeholder (Observing System)	32

getOrganizationUser (Observing System)	32

getPlatformStationType (Platform/Station)	23

getSensorType (Sensing Element)	19

Open Block Diagram (Observing System)	32

Open Reference Material 1 (Sensing Element)	19

Open Reference Material 2 (Sensing Element)	19

Open Reference URL (Observing System)	32

Open Schedule (Observing System)	32

Open Source Document (Phenomenon Requirement)	27

Open System Picture (Observing System)	32

Open Webpage (Platform/Station)	23

O

Object Type

Actual Measurement	13

Category	8

Characterization requirement	62

Data Handling System	59

Environmental Parameter	57

Environmental Phenomenon	55

Environmental Product	54

Fixed Location Data	52

Group	51

Location	48

Message	64

Mobile Location Data	42

Observation Control System	41

Observing System	32

Organization	29

Phenomenon Requirement	27

Platform/Station	23

Platform/Station Type	63

Processing Element	22

Required Measurement	44

Sensing Element	19

Sensor Type	18

Term	9

Topic	10

Variable	11

R

Relationship Type

Acquires	92

Applies to	68

Area of Operations	72

Characterizes	86

Contains	81

Downstream user	91

Drives	87

Funds	76

Has contact person	73

Is controlled by	94

Is type of	96

Leads to	97

Located at	89

Measures	83

Met by	100

Operates	85

Owns	79

Part of	95

Primary stakeholder	78

Provides data directly to	93

Provides data for	82

Provides data to	75

Provides info to	74

Reported to	88

Secondary stakeholder	90

Sent to	99

Situated on	80

Supplied by	70

Supports	84

Takes	98

Transmits	69

Use	71

User	77

V

Viewstyle

NOSA View Style	12