DEPARTMENT OF TRANSPORTATION

FEDERAL AVIATION ADMINISTRATION

E14EA

Revision 20

Lycoming Engines

TIO-540-A1A, -A1B, -A2A,

-A2B, -A1C, -A2C, -C1A,

-E1A, -F2BD, -G1A, -H1A,

-J2B, -J2BD, -K1AD, -N2BD,

-R2AD, -S1AD, -T2AD, -U2A,

-V2AD, -W2A, -AA1AD,

-AB1AD, -AB1BD -AE2A,

-AF1A, -AF1B, -AG1A,

-AH1A, -AJ1A, AK1A

LTIO-540-F2BD-J2B, -J2BD,

-K1AD, -N2BD, -R2AD, -U2A,

-V2AD, -W2A

September 26, 2006

TYPE CERTIFICATE DATA SHEET NO. E14EA

Engines of models described herein conforming with this data sheet (which is a part of type certificate No. E14EA) and other approved data on file with the Federal Aviation Administration meet the minimum standards for use in certification aircraft in accordance with pertinent aircraft data sheets and applicable portions of the Civil Air Regulations/Federal Aviation Regulations provided they are installed, operated, and maintained as prescribed by the approved manufacturer’s manual and other approved instructions.

Type Certificate Holder	Lycoming Engines,

An operating Division of Avco Corporation

Williamsport, Pennsylvania  17701

Type Certificate Holder Record	Textron Lycoming, AVCO Corporation transferred TC E14EA to Lycoming Engines, An Operating Division of Avco Corporation on December 17, 2003.

| Model Lycoming | TIO-540-A1A, -A1B, -A2A, -A2B | TIO-540-A1C, -A2C | TIO-540-E1A |
| --- | --- | --- | --- |
| Type 6HOA Direct Drive Turbocharged |  |  |  |
| Rating (See Notes 4 and 13) |  |  |  |
| Maximum continuous hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 310-2575-40.2-15,000 | 310-2575-42.-15,000 | 260-2575-36.0-15000 |
| Standard density sea level alt. ft. | 310-2575-38.6-S.L. | 310-2575-40.0-S.L. | 260-2575-34.1-S.L. |
| Takeoff (5 min.) hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 310-2575-40.2-15,000 | 310-2575-41.5-15,000 | 260-2575-36.0-15000 |
| Standard density sea level alt., ft. | 310-2575-38.6-S.L. | 310-2575-40.0-S.L. | 260-2575-34.1-S.L. |
| Fuel (minimum grade aviation gasoline) | 100/100LL | - - | - - |
| Lubricating oil (lubrication should conform to the specification as listed or to subsequent revisions thereto) | Lycoming Spec. No. 301-F Service Instruction No. 1014 | - - | - - |
| Bore and stroke, in. | 5.125 X 4.375 | - - | - - |
| Displacement, cu. in. | 541.5 | - - | - - |
| Compression ratio | See Note 10. | - - | - - |
| Weight (dry) lb. | See Note 10. | - - | - - |
| C.G. location (with starter and alternator installed) |  |  |  |
| From front face of prop. mounting flange, in. | 22.22 | - - | 19.47 |
| Off crankshaft C.L., in. | 0.88 below & 0.18 right | - - | 3.00 below & 0.11 left |
| Propeller shaft flange, SAE A.S. 127 | Type 2 modified | - - | - - |
| Crankshaft dampers (torsional) | One 5th order One 6th order | - - - - | - - One 6th order (Note 8) |
| Fuel injection | Note 10 | - - | - - |
| Turbocharger | Note 10 | - - | - - |
| Ignition, dual | Note 10 | - - | - - |
| Ignition timing °BTC | 20 | - - | - - |
| Spark plugs | Note 6 | - - | - - |
| Oil sump capacity, qt. | 12 | - - | - - |
| Usable oil, qt. (20° nose up or down) | 9 1/4 | - - | - - |
| NOTES - as applicable | 1-7, 9, 10 | 1-7, 9, 10 | 1-10 |

“- -” indicates “same as preceding model.”

| Model Lycoming | TIO-540-H1A | TIO-540-F2BD LTIO-540-F2BD | TIO-540-J2B, -J2BD, -N2BD, LTIO-540-J2B, -J2BD, -N2BD |
| --- | --- | --- | --- |
| Type 6HOA Direct Drive Turbocharged |  |  |  |
| Rating (See Notes 4 and 13) |  |  |  |
| Maximum continuous hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 270-2575-37.5-15,000 | 325-2575-46.0-15000 | 350-2575-46.0-15000 |
| Standard density sea level alt. ft. | 270-2575-35.9-S.L. | 325-2575-43.5-S.L. | 350-2575-43.0-S.L. |
| Takeoff (5 min.) hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 270-2575-37.5-15,000 | 325-2575-46.0-15000 | 350-2575-46.0-15000 |
| Standard density sea level alt., ft. | 270-2575-35.9-S.L. | 325-2575-43.5-S.L. | 350-2575-43.0-15000 |
| Fuel (minimum grade aviation gasoline) | 100/100LL | - - | - - |
| Lubricating oil (lubrication should conform to the specification as listed or to subsequent revisions thereto) | Lycoming Spec. No. 301-F Service Instruction No. 1014 | - - | - - |
| Bore and stroke, in. | 5.125 X 4.375 | - - | - - |
| Displacement, cu. in. | 541.5 | - - | - - |
| Compression ratio | See Note 10. | - - | - - |
| Weight (dry) lb. | See Note 10. | - - | - - |
| C.G. location (with starter and alternator installed) |  |  |  |
| From front face of prop. mounting flange, in. | 19.47 | 22.19 | 22.75 |
| Off crankshaft C.L., in. | 3.00 below & 0.11 left | 0.88 below & 0.18 right | 0.75 below & 0.16 right |
| Propeller shaft flange, SAE A.S. 127 | Type 2 modified | - - | - - |
| Crankshaft dampers (torsional) | One 5th order One 6th order (Note 8) | - - One 6th order | - - - - |
| Fuel injection | Note 10 | - - | - - |
| Turbocharger | Note 10 | - - | - - |
| Ignition, dual | Note 10 | - - | - - |
| Ignition timing °BTC | 20 | - - | - - |
| Spark plugs | Note 6 | - - | - - |
| Oil sump capacity, qt. | 12 | - - | - - |
| Usable oil, qt. (20° nose up or down) | 9 1/4 | - - | - - |
| NOTES -- as applicable | 1-10 | 1-7, 9, 10 | 1-7, 9, 10 |

“- -” indicates “same as preceding model.”

| Model Lycoming | TIO-540-R2AD LTIO-540-R2AD | TIO-540-C1A, -G1A | TIO-540-K1AD LTIO-540-K1AD |
| --- | --- | --- | --- |
| Type 6HOA Direct Drive Turbocharged |  |  |  |
| Rating (See Note 4) |  |  |  |
| Maximum continuous hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 340-2575-44.0-15000 | 250-2575-34.0-15000 | 250-2575-34.0-15000 |
| Standard density sea level alt. ft. | 350-2575-44.0-S.L. | 250-2575-33.0-S.L. | 250-2575-34.0-S.L. |
| Takeoff (5 min.) hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 340-2575-44.0-15000 | 250-2575-34.0-15000 | 250-2575-34.0-15000 |
| Standard density sea level alt., ft. | 350-2575-44.0-S.L. | 250-2575-33.0-S.L. | 250-2575-34.0-S.L. |
| Fuel (minimum grade aviation gasoline) | 100/100LL | - - | - - |
| Lubricating oil (lubrication should conform to the specification as listed or to subsequent revisions thereto) | Lycoming Spec. No. 301-F Service Instruction No. 1014 | - - | - - |
| Bore and stroke, in. | 5.125 X 4.375 | - - | - - |
| Displacement, cu. in. | 541.5 | - - | - - |
| Compression ratio | See Note 10. | - - | - - |
| Weight (dry) lb. | See Note 10. | - - | - - |
| C.G. location (with starter and alternator installed) |  |  |  |
| From front face of prop. mounting flange, in. | 22.75 | 19.27 | 21.0 |
| Off crankshaft C.L., in. | 0.75 below & 0.16 right | 2.30 below & 0.23 left | 0.50 below & 0.31 left |
| Propeller shaft flange, SAE A.S. 127 | Type 2 modified |  | - - |
| Crankshaft dampers (torsional) | One 5th order One 6th order | - - One 6th order (Note 8) | - - - - |
| Fuel injection | Note 10 | - - | - - |
| Turbocharger | Note 10 | - - | - - |
| Ignition, dual | Note 10 | - - | - - |
| Ignition timing °BTC | 20 | - - | - - |
| Spark plugs | Note 6 | - - | - - |
| Oil sump capacity, qt. | 12 | - - | 8 |
| Usable oil, qt. (20° nose up or down) | 9 1/4 | - - | 6 |
| NOTES - as applicable | 1-7, 9, 10, 11, 13 | 1-10 | 1-10 |

“- -” indicates “same as preceding model.”

| Model Lycoming | TIO-540-S1AD | TIO-540-T2AD | TIO-540-U2A LTIO-540-U2A |
| --- | --- | --- | --- |
| Type 6HOA Direct Drive Turbocharged |  |  |  |
| Rating (See Note 4) |  |  |  |
| Maximum continuous hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 295-2700-36.0-16000 | 330-2400-43.0-12000 | 350-2500-42.2-12000 |
| Standard density sea level alt. ft. | 300-2700-36.0-S.L. & 12000 ft. | 330-2400-40.5-S.L. | 350-2500-41.2-S.L. |
| Takeoff (5 min.) hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 295-2700-36.0-16000 | 330-2400-43.0-12000 | 350-2500-42.2-12000 |
| Standard density sea level alt., ft. | 300-2700-36.0-S.L. & 12000 ft. | 330-2400-43.0-S.L. | 350-2500-41.2-S.L. |
| Fuel (minimum grade aviation gasoline) | 100/100LL | - - | - - |
| Lubricating oil (lubrication should conform to the specification as listed or to subsequent revisions thereto) | Lycoming Spec. No. 301-F Service Instruction No. 1014 | - - | - - |
| Bore and stroke, in. | 5.125 X 4.375 | - - | - - |
| Displacement, cu. in. | 541.5 | - - | - - |
| Compression ratio | See Note 10. | - - | - - |
| Weight (dry) lb. | See Note 10. | - - | - - |
| C.G. location (with starter and alternator installed) |  |  |  |
| From front face of prop. mounting flange, in. | 18.58 | 22.75 | 20.38 |
| Off crankshaft C.L., in. | 1.31 below & 1.00 right | 0.75 below & 0.16 right | 1.4 below & 0.25 right |
| Propeller shaft flange, SAE A.S. 127 | Type 2 modified | - - | - - |
| Crankshaft dampers (torsional) | One 5th order One 6th order (Note 8) | - - - - | - - - - |
| Fuel injection | Note 10 | - - | - - |
| Turbocharger | Note 10 | - - | - - |
| Ignition, dual | Note 10 | - - | - - |
| Ignition timing °BTC | 20 | - - | - - |
| Spark plugs | Note 6 | - - | - - |
| Oil sump capacity, qt. | 12 | - - | - - |
| Usable oil, qt. (20° nose up or down) | 9 3/4 | - - | - - |
| NOTES - as applicable | 1-10 | 1-7, 9. 10 | 1-7, 9, 10 |

“- -” indicates “same as preceding model.”

| Model Lycoming | TIO-540-V2AD LTIO-540-V2AD (See Note 14) | TIO-540-W2A LTIO-540-W2A (See Note 14) | TIO-540-AA1AD* TIO-540-AB1AD TIO-540-AB1BD |
| --- | --- | --- | --- |
| Type 6HOA Direct Drive Turbocharged |  |  |  |
| Rating (See Note 4) |  |  |  |
| Maximum continuous hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 340-2600-42.0-18000 | 350-2600-44.2-15000 | 250-2575-32.5-15000 |
| Standard density sea level alt. ft. | 350-2600-42.0-S.L. | 350-2600-44.2-S.L. | 250-2575-32.5-S.L. |
| Takeoff (5 min.) hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 340-2600-42.0-18000 | 350-2600-44.2-15000 | 250-2575-32.5-15000 |
| Standard density sea level alt., ft. | 350-2600-42.0-S.L. | 350-2600-44.2-S.L. | 250-2575-32.5-S.L. |
| Fuel (minimum grade aviation gasoline) | 100/100LL | - - | - - |
| Lubricating oil (lubrication should conform to the specification as listed or to subsequent revisions thereto) | Lycoming Spec. No. 301-F Service Instruction No. 1014 | - - | - - |
| Bore and stroke, in. | 5.125 X 4.375 | - - | - - |
| Displacement, cu. in. | 541.5 | - - | - - |
| Compression ratio | See Note 10. | - - | - - |
| Weight (dry) lb. | See Note 10. | - - | - - |
| C.G. location (with starter and alternator installed) |  |  |  |
| From front face of prop. mounting flange, in. | 23.05 | 22.10 | 18.91 |
| Off crankshaft C.L., in. | 0.59 below & 0.14 right | 0.10 below & 0.48 right | 2.20 below & 0.67 left |
| Propeller shaft flange, SAE A.S. 127 | Type 2 modified | - - | - - |
| Crankshaft dampers (torsional) | One 5th order One 6th order | - - - - | - - - - |
| Fuel injection | Note 10 | - - | - - |
| Turbocharger | Note 10 | - - | - - |
| Ignition, dual | Note 10 | - - | - - |
| Ignition timing °BTC | 20 | - - | - - |
| Spark plugs | Note 6 | - - | - - |
| Oil sump capacity, qt. | 12 | - - | - - ** |
| Usable oil, qt. (20° nose up or down) | 9 1/4 | - - | - - *** |
| NOTES - as applicable | 1-7, 9, 10, 14 | 1-7, 9, 10, 14 | 1-7, 9, 10, 15 |

“- -” indicates “same as preceding model.”

*TIO-540-AA1AD Model is approved with an alternate rating of 270hp/2575 rpm from S.L. to 15,000 ft.

**  10 for AA1AD

*** 7 1/4 for AA1AD.

| Model Lycoming | TIO-540-AE2A | TIO-540-AF1A TIO-540-AF1B | TIO-540-AG1A |
| --- | --- | --- | --- |
| Type 6HOA Direct Drive Turbocharged |  |  |  |
| Rating (See Note 4) |  |  |  |
| Maximum continuous hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 350-2500-42.0-20000 | 270-2575-36.5-20000 | 270 - 2575 - 36.5 -15000 |
| Standard density sea level alt. ft. | 350-2500-42.0-S.L. | 270-2575-35.0-S.L. | 270 - 2575 - 36.5 - SL |
| Takeoff (5 min.) hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 350-2500-42.0-20000 | 270-2575-36.5-20000 | 270 - 2575 - 36.5 -15000 |
| Standard density sea level alt., ft. | 350-2500-42.0-S.L. | 270-2575-35.0-S.L. | 270 - 2575 - 36.5 - SL |
| Fuel (minimum grade aviation gasoline) | 100/100LL | - - | - - |
| Lubricating oil (lubrication should conform to the specification as listed or to subsequent revisions thereto) | Lycoming Spec. No. 301-F Service Instruction No. 1014 | - - | - - |
| Bore and stroke, in. | 5.125 X 4.375 | - - | - - |
| Displacement, cu. in. | 541.5 | - - | - - |
| Compression ratio | See Note 10. | - - | - - |
| Weight (dry) lb. | See Note 10. | - - | - - |
| C.G. location (with starter and alternator installed) |  |  |  |
| From front face of prop. mounting flange, in. | 18.88 | 18.30 | 19.00 |
| Off crankshaft C.L., in. | 0.95 below & 0.63 left | 2.30 below & 0.64 left | 1.75 below & 1.20 left |
| Propeller shaft flange, SAE A.S. 127 | Type 2 modified | - - | - - |
| Crankshaft dampers (torsional) | One 5th order One 6th order | - - - - | - - - - |
| Fuel injection | Note 10 | - - | - - |
| Turbocharger | Note 10 | - - | - - |
| Ignition, dual | Note 10 | - - | - - |
| Ignition timing °BTC | 20 | - - | - - |
| Spark plugs | Note 6 | - - | - - |
| Oil sump capacity, qt. | 12 | 10 | 10 |
| Usable oil, qt. (20° nose up or down) | 9 1/4 | 7 1/4 | 5 |
| NOTES - as applicable | 1-7, 9, 10, 11 | 1-7, 9, 10 | 1-10, 12 |

“- -” indicates “same as preceding model.”

| Model Lycoming | TIO-540-AH1A | TIO-540-AJ1A | TIO-540-AK1A |
| --- | --- | --- | --- |
| Type 6HOA Direct Drive Turbocharged |  |  |  |
| Rating (See Note 4) |  |  |  |
| Maximum continuous hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 300 - 2500 - 38.0 -12000 | 310 - 2500 - 39.0 – 14000 | 235 – 2400 – 32.0 -15000 |
| Standard density sea level alt. ft. | 300 - 2500 - 38.0 - SL | 310 - 2500 - 39.0 - SL | 235 – 2400 – 32.0 - SL |
| Takeoff (5 min.) hp., rpm, in. Hg. at: |  |  |  |
| Standard density critical alt. ft. | 300 - 2500 - 38.0 –12000 | 310 - 2500 - 39.0 – 14000 | 235 – 2400 – 32.0 -15000 |
| Standard density sea level alt., ft. | 300 - 2500 - 38.0 - SL | 310 - 2500 - 39.0 - SL | 235 – 2400 – 32.0 - SL |
| Fuel (minimum grade aviation gasoline) | 100/100LL | - - | -- |
| Lubricating oil (lubrication should conform to the specification as listed or to subsequent revisions thereto) | Lycoming Spec. No. 301-F Service Instruction No. 1014 | - - | -- |
| Bore and stroke, in. | 5.125 | - - | -- |
| Displacement, cu. in. | 4.375 | - - | -- |
| Compression ratio | See Note 10. | - - | -- |
| Weight (dry) lb. | See Note 10. | - - | -- |
| C.G. location (with starter and alternator installed) |  |  |  |
| From front face of prop. mounting flange, in. | 18.70 | 18.50* | 18.94* |
| Off crankshaft C.L., in. | 1.87 below & 1.00 right | 0.75 below & 1.13 left* | 1.50 below & 1.25 left* |
| Propeller shaft flange, SAE A.S. 127 | Type 2 modified | - - | -- |
| Crankshaft dampers (torsional) | One 5th order One 6th order | - - | -- |
| Fuel injection | Note 10 | - - | -- |
| Turbocharger | Note 10 | - - | -- |
| Ignition, dual | Note 10 | - - | -- |
| Ignition timing °BTC | 20 | - - | 23** |
| Spark plugs | Note 6 | - - | -- |
| Oil sump capacity, qt. | 12 | 11 | 8 |
| Usable oil, qt. (20° nose up or down) | 9 1/4 | 5 1/2 | 4 |
| NOTES - as applicable | 1-10, 12 | - - | -- |

“- -” indicates “same as preceding model.”

* No alternator installed

** See Note 16

Certification Basis:

Regulations and Amendments			Date Type Certificate

Model	Date of Application	E14EA Issued/Revised

CAR 13 effective June 15, 1956	TIO-540-A1A	May 26, 1964	December 6, 1965

as amended by 13-1, 13-2, 13-3, 13-4	TIO-540-A1B	October 6, 1967	October 9, 1967

TIO-540-A2A	March 26, 1968	April 1, 1968

TIO-540-A2B	March 26, 1968	April 1, 1968

TIO-540-C1A	September 7, 1967	June 6, 1968

TIO-540-E1A	September 3, 1970	September 18, 1970

TIO-540-G1A	September 2, 1970	September 18, 1970

TIO-540-A1C	November 17, 1970	January 6, 1971

TIO-540-A2C	November 17, 1970	January 6, 1971

TIO-540-H1A	April 8, 1971	April 16, 1971

TIO-540-F2BD	April 20, 1970	July 23, 1971

Certification Basis:    (cont’d)

Regulations and Amendments			Date Type Certificate

Model	Date of Application	E14EA Issued/Revised

LTIO-540-F2BD	February 9, 1972	February 16, 1972

L/TIO-540-J2BD	August 25, 1971	April 7, 1972

L/TIO-540-K1AD	December 4, 1973	February 28, 1974

L/TIO-540-N2BD	January 29, 1973	February 16, 1973

TIO-540-R2AD	April 23, 1975	September 25, 1975

LTIO-540-R2AD	October 7, 1977	October 13, 1977

TIO-540-S1AD	January 31, 1977	June 14, 1977

TIO-540-T2AD	February 17, 1981	March 5, 1981

L/TIO-540-U2A	June 2, 1981	February 12, 1982

L/TIO-540-J2B	August 20, 1981	September 9, 1981

L/TIO-540-V2AD	December 22, 1981	January 31, 1983

L/TIO-540-W2A	October 29, 1982	August 17, 1984

TIO-540-AB1AD	June 14, 1984	January 16, 1985

TIO-540-AA1AD	February 20, 1985	June 25, 1985

TIO-540-AE2A	March 9, 1987	August 12, 1988

TIO-540-AF1A	September 13, 1988	June 23, 1989

TIO-540-AB1BD	May 18, 1993	August 11, 1993

TIO-540-AG1A	December 1,1994	June 20, 1995

TIO-540-AF1B	December 7, 1995	January 25, 1996

TIO-540-AH1A	February 12, 1997	June 25, 1997

TIO-540-AJ1A	March 5, 1998	April 30, 1998

TIO-540-AK1A	February 22, 2000	February 2, 2001

Production basis:	Production Certificate No. 3.

NOTE 1.	Maximum permissible temperatures:

Cylinder head (well type thermocouple)	500°F (480°F model - W2A, -V2AD, -AJ1A)

Cylinder base	Note 5

Oil inlet	245°F

Exhaust gas	*1650° F (See Note 4 for turbo inlet location)

Fuel injector inlet air	400°F

Compressor temperature rise	Not applicable  (see Note 4)

*TIO-540-AE2A, -AF1A, -AF1B  models are approved for 1750°F  E.G.T.

model TIO-540-AJ1A approved for 1675°F  E.G.T., model TIO-540-AK1A approved for 1685°F  E.G.T.

NOTE 2.	Pressure Limits:

| Fuel pressure (p.s.i. above injector inlet air pressure) | Minimum (p.s.i.) | Maximum (p.s.i.) | Idle (min p.s.i.) |
| --- | --- | --- | --- |
| at inlet to fuel injector (-A models except -A1C, -A2C) | 25 | 45 | 12 |
| at inlet to fuel injector (-A1C, -A2C , -AJ1A models) | 25 | 55 | 12 |
| at inlet to fuel injector (-C, -E, -G, -H models) | 18 | 45 | 12 |
| at inlet to fuel injector (-F model) | 30 | 65 | 12 |
| at inlet to fuel injector (-J, -N model) | 34 | 65 | 12 |
| at inlet to fuel injector (-K model) | 20 | 45 | 12 |
| at inlet to fuel injector (-R model) | 34 | 65 | 12 |
| at inlet to fuel injector (-S model) | 25 | 65 | 12 |
| at inlet to fuel injector (-T model) | 34 | 65 | 12 |
| at inlet to fuel injector (-U model) | 30 | 65 | 12 |
| at inlet to fuel injector (-V, -AH models) | 27 | 65 | 12 |
| at inlet to fuel injector (-W model) | 31 | 65 | 12 |
| at inlet to fuel injector (-AB1AD, -AB1BD models) | 20 | 65 | 12 |
| at inlet to fuel injector (-AA1AD model) | 20 | 65 | 12 |
| at inlet to fuel injector (-AE2A model) | 29 | 65 | 12 |
| at inlet to fuel injector (-AF1A, -AF1B models) | 24 | 55 | 12 |
| at inlet to fuel injector (-AG1A, -AK1A models) | 20 | 55 | 12 |

NOTE 2.  (cont’d)

| Fuel Pressure (p.s.i. above ambient air pressure) | Minimum (p.s.i.) | Maximum (p.s.i.) | Idle (min p.s.i.) |
| --- | --- | --- | --- |
| at inlet to engine fuel pump (-A, -F, -R, -S, -U, -V, -W) | -2 | 65 | -- |
| (-J, -N, -T, -AE2A, -AH1A, -AJ1A models) |  |  |  |
| at inlet to engine fuel pump (-C, -E, -G, -H, -K -AF1A,) (-AF1B, -AG1A, -AK1A models) | -2 | 55 | -- |

| Manifold Pressure (cumulative total with altitude adjustment) | Maximum (in. Hg Abs) |  |  |
| --- | --- | --- | --- |
| -A models (except -A1C, -A2C) | 45 |  |  |
| -A1C, -A2C models | 46 |  |  |
| -C, -E, -G models | 39.5 |  |  |
| -H models | 40.7 |  |  |
| -F, -J, -N models | 49 |  |  |
| -K models | 34.5 |  |  |
| -R models | 44 |  |  |
| -S models | 36 |  |  |
| -T models | 45.2 |  |  |
| -U models, AE2A | 42.0 |  |  |
| -W models | 50.6 |  |  |
| -AA1AD, -AB1AD, -AB1BD, -AF1A, -AF1B, -AH1A models | 38 |  |  |
| -AG1A, -AJ1A models | 39 |  |  |
| -AK1A model | 32 |  |  |
| Oil Pressure | Minimum (p.s.i.) | Maximum (p.s.i.) |  |
| Normal (all models) | 55 | 95 |  |
| Idle (all models) | 25 | -- |  |
| Starting and warm-up (all models) | -- | 115 |  |
| Turbocharger exhaust back pressure |  | 0.5 |  |

NOTE 3.	The following accessory provisions are available:

|  | -A1A, -A1B, -A2A, -A2B, -A1C, -A2C, -C1A, -E1A, -G1A, -H1A, -J2B, -U2A, | AK1A |  | -AF1A | -F2BD, -J2BD, -N2BD, -R2AD, -S1AD, -T2AD, -U2A, -V2AD, -W2A, -J2B, -AB1AD, -AA1AD, |  |  | Rotating Facing Drive Pad |  | Speed Ratio to | Maximum Torque in. lb. |  |  | Maximum Overhang Moment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accessory | -W2A | -AJ1A | -AE2A | -AF1B | -AG1A, -AH1A | -AB1BD | -K1AD | TIO | LTIO | Crankshaft | Cont. |  | Static | in.-lb. |
| Starter | * | * | * | * | * | * | * | CC | C | 16.556:1 | --- |  | 450 | 150 |
| Alternator | * | -- | -- | -- | * | * | * | C | CC | 3.200:1 | 60 |  | 120 | 175 |
| Alternator | - | -- | * | -- | - | - | - | C | - | 3.800:1 | 60 |  | 120 | 175 |
| Vacuum pump | ** | * | * | * | * | * | * | CC | C | 1.300:1 | 70 |  | 450 | 25 |
| Hydraulic pump | ** | * | * | * | -- | -- | -- | C | CC | 1.385:1 | 100 |  | 800 | 40 |
| Hydraulic pump | -- | -- | -- | -- | * | -- | * | C | CC | 1.300:1 | 100 |  | 800 | 40 |
| Tachometer | * | * | -- | * | * | * | * | C | CC | 0.500:1 | 7 |  | 50 | 5 |
| Propeller governor*** | * | -- | -- | -- | -- | -- | * | C | CC | 0.895:1 | 125 |  | 1200 | 25 |
| Propeller governor**** | * | * | * | * | * | -- | -- | C | CC | 0.947:1 | 125 |  | 2200 | 25 |
| Propeller governor**** | -- | -- | -- | -- | -- | * | -- | C | CC | 1.3000:1 | 125 |  | 2200 | 25 |
| Fuel pump | - | * | - | - | * | * | * | C | CC | 1.000:1 | 25 |  | 450 | 25 |
| Fuel pump | * | -- | * | * | -- | -- | -- | CC | C | 1.000:1 | 25 |  | 450 | 25 |
| Freon comp. | -- | -- | - | - | -- | -- | ** | C | CC | 1.300:1 | 180 |  | 2200 | 150 |
| Freon comp. | -- | -- | * | * | ** | * | -- | C | CC | 1.462:1***** |  | Limited by belt. |  |  |

“C” Clockwise, “CC” Counter Clockwise

*Standard

**Optional

***Narrow deck engines

****Wide deck engines

*****With compressor pulley diameter of 6.00 inches.

NOTE 4.	The Turbocharger meets the containment requirements of CAR 13.116 and does not require external

protection.

Measure exhaust gas temperature at inlet location shown on Lycoming Drawing No. 63232(-A1A, -A2A,

-A1B, -A2B, -A1C, -A2C), 63281 (-C1A, -E1A, -G1A, -H1A), 63393 (-F2BD), 63397 (-J2B/D), 63433

(-N2BD), 63416 (K1AD), 63444(-R2AD), 63517 (-T2AD), 63533 (-U2A), 63534 (-V2AD), 63537 (-W2A),

63545 (-AB1AD), 63582 (-AB1BD),  63546 (-AA1AD), 04D63570(-AE2A), 04063573 (-AF1A, -AF1B), 			63583 (-AG1A), 63592 (-AH1A), O4D63594 (-AJ1A), 04D63601 (-AK1A).

Performance data for these engines are presented on Lycoming Curves No. 12993 (-A1A, -A2A, -A1B,

-A2B), 13195-B(-A1C, -A2C), 13107-B(-C1A), 13172(-E1A), 13190-A(-G1A), 13208 (-H1A),

13201-A (-F2BD), 13215 (-J2BD, -N2BD), 13257-A (-K1AD), 13296 (-R2AD), and 13329(-S1AD),

13395 (-T2AD), 13399 (-U2A), 13415 (-V2AD), 13441 (-W2A), 13455 (-AB1AD, -AB1BD, -AA1AD),

13482(-AE2A), 13491(-AF1A -AF1B), 13509 (-AG1A), 13525 (-AH1A), 13538 (-AJ1A), 13556 (AK1A).

Maximum turbocharger speed is now being governed by manifold pressure rather than temperature rise.

Maximum manifold pressure versus altitude are presented on Lycoming Curve Numbers 13166-A (-A1A),

-A2A, -A1B, -A2B), 13196-C (-A1C, -A2C), 13197-A (-C1A, -G1A), 13209 (-H1A), 13202-B (-F2BD),

13216-C (-J2B/D, -N2BD), 13297 (-R2AD), 13263 (-K1AD), 13330 (-S1AD), 13396 (-T2AD), 13416

(-V2AD), 13442 (-W2A), 13456 (-AB1AD, -AB1BD, -AA1AD), 13483 (-AE2A), 13492 (-AF1A,-AF1B), 			13510 (-AG1A), 13537 (AH1A), 13540-A (-AJ1A), 13558 (AK1A).

NOTE 5.	Cylinder base temperature limits are not applicable to engine models which incorporate internal piston

cooling oil jets.

NOTE 6.	Spark plugs approved for use on these engines are listed in the latest revision of Textron Lycoming Service

Instruction No. 1042.

NOTE 7.	These engines incorporate the following similarities or differences:

TIO-540-A1A	Basic model.  Six cylinder, air-cooled, horizontally-opposed, direct drive, fuel injected,

top exhaust, turbocharged engine incorporating internal piston cooling oil jets.  Uses

two-blade propeller.

TIO-540-A1B	Similar to -A1A but has density controller with faster temperature response.

TIO-540-A2A	Similar to -A1A except has propeller flange bushings which accommodate either a

two-blade or three-blade propeller.

TIO-540-A2B	Similar to -A1B except has propeller flange bushings which accommodate either a

two-blade or three-blade propeller.

TIO-540-A1C	Similar to -A1B except BSFC is increased to .67/bhp-hr and the manifold pressure

increased to 40.0 inches Hg.

TIO-540-A2C	Similar to -A2B except BSFC is increased to .67/bhp-hr and the manifold pressure

increased to 40.0 inches Hg.

TIO-540-C1A	Differs from -A1A with lower compression ratio, different counterweights, and parallel

valve cylinders.

TIO-540-E1A	Similar to -C1A except rating is increased, different turbocharger control setting and 				uses Bendix S6LN-1227 magneto instead of S6LN-1208 magneto.

TIO-540-G1A	Similar to -C1A except for higher compression ratio.

TIO-540-H1A	Similar to -E1A except rating is increased.

TIO-540-F2BD	Similar to -A2B except rated power is increased and incorporates the Bendix Dual

Magneto.

LTIO-540-F2BD	Similar to -F2DB except that it has counter-clockwise (reversed) rotation.

TIO-540-J2BD	Similar to TIO-540-F2BD except rated power is increased and is equipped with an Air

Research Model THO8460 turbocharger.

LTIO-540-H2BD	Similar to TIO-540-J2BD except that it has counter-clockwise (reverse) rotation.

L/TIO-540-J2B	Similar to L/TIO-J2BD except for two single Bendix Magnetos instead of a dual Bendix

Magneto.

TIO-540-K1AD	Similar to -E1A except increased compression ratio, dual magneto, pressure controller,

provision for cabin bleed air has the turbocharger mounted to the rear of the engine.

LTIO-540-K1AD	Similar to TIO-540-K1AD except that it has counter-clockwise (reverse) rotation.

TIO-540-N2BD	Similar to -J2BD except turbocharger is moved 1/2 inch to the left when viewed from 				the rear.

LTIO-540-N2BD	Similar to TIO-540-N2BD except that it has counter-clockwise (reverse) rotation.

TIO-540-R2AD	Similar to -J2BD except lower rated speed and power, provision for cabin bleed and 				type of turbocharger controllers.

LTIO-540-R2AD	Similar to TIO-540-R2AD except that it has counter-clockwise (reverse) rotation.

TIO-540-S1AD	Similar to IO-540-M2A5D (1E4) except has a manually controllable turbocharger, front air inlet, lower compression ratio and provision for a controllable propeller.

TIO-540-T2AD	Similar to TIO-540-J2BD except for modified turbocharger transition section and lower

rated power.

TIO-540-U2A	Similar to IO-540-AA1A5 except has thin wall, high crush bearings, narrow connecting

rods, an intercooler (remote mounted) and a Rotomaster (Rajay) turbocharger package.

LTIO-540-U2A	Similar to TIO-540-U2A except that it has counter-clockwise (reverse) rotation.

TIO-540-V2AD	Similar to -J2BD except has down exhaust and top intake cylinder heads, employs an

intercooler and the crankshaft, connecting rods and bearing combination similar to the

-U2A.

LTIO-540-V2AD	Similar to the TIO-540-V2AD except that it has counter-clockwise (reverse) rotation.

TIO-540-W2A	Similar to the -V2AD except equipped with (2) Bendix S-1200 series magnetos, a

density differential controller, not equipped with an induction intercooler nor any

provisions for cabin bleed air.

LTIO-540-W2A	Similar to the TIO-540-W2A except that it has counter-clockwise (reverse) rotation.

TIO-540-AB1AD	Similar to the -K1AD except for the relocation of the turbocharger and has a density-

differential controller.  Also, the dual magneto is equipped with an impulse coupling

instead of retard breaker.

TIO-540-AB1BD	Similar to the -AB1AD except the prop governor is on the accessory housing, scavenger 				pump on upper accessory pad and more effective counterweights.

TIO-540-AA1AD	Similar to the model -AB1AD except for a rear mounted prop governor drive, uses the

-K1AD turbocharger mounting and has no provisions for cabin bleed.

TIO-540-AE2A	Similar to the TIO-540-U2A except with (2) Slick retard breaker, pressurized magnetos,

twin AiResearch turbochargers with individual intercoolers, and a single wastegate

modulated by a variable absolute pressure controller.  Two sonic nozzles are provided

for cabin pressurization.

TIO-540-AF1A	Similar to the TIO-540-AA1AD except equipped with (2) Slick pressurized magnetos,

an intercooler, drives for (2) alternators and a different model Garrett turbocharger.

TIO-540-AF1B	Similar to the TIO-540-AF1A except incorporates exhaust valve guide oilers

TIO-540-AG1A	Similar to the TIO-540-AA1AD except equipped with (2) Slick pressurized magnetos,

and a different model Garrett turbocharger which has been relocated.

TIO-540-AH1A	Similar to the TIO-540-A except equipped with (2) Slick pressurized magnetos,

down exhaust heads and a different model Garrett turbocharger which has been 				relocated.

TIO-540-AJ1A	Similar to the TIO-540-W2A except equipped with a different model Garrett 					turbocharger and slope controller.

TIO-540-AK1A	Similar to TIO-540-AG1A except that it has a relocated AiResearch model TAO411 turbocharger, bottom mounted fuel injector and a lower rating

NOTE 8.	The TIO-540-C1A, -E1A, -G1A, -H1A, -K1AD and LTIO-540-K1AD are equipped with special 5th and 6th

order counterweights making them eligible for use with Hartzell “compact” propeller.  When the

TIO-540-G1A is used with a Hartzell HC-E2YK-2R/8465-7R propeller, the manifold pressure is limited

in the 2000 rpm speed range to a maximum of 27 inches Hg.

NOTE 9.	Starter, generators, and alternators approved for use on these engines are listed in the latest revision of

Textron Lycoming Service Instruction No. 1154.

NOTE 10.	The following tabulation shows turbocharger, fuel injector, weight, compression ratio and ignition.

| Model | Turbocharger | Fuel Injection | Weight (dry) lb. | Compression Ratio | Ignition, Dual * |
| --- | --- | --- | --- | --- | --- |
| TIO-540-A1A | TE0659 | RSA-10AD1 | 509 | 7.30:1 | TCM** S6LN-1208, S6LN-1209 |
| -A1B | TE0659 | RSA-10AD1 | 509 | 7.30:1 | S6LN-1208, S6LN-1209 |
| -A2A | TE0659 | RSA-10AD1 | 509 | 7.30:1 | S6LN-1208, S6LN-1209 |
| -A2B | TE0659 | RSA-10AD1 | 509 | 7.30:1 | S6LN-1208, S6LN-1209 |
| -A1C | TE0659 | RSA-10AD1 | 511 | 7.30:1 | S6LN-1208, S6LN-1209 |
| -A2C | TE0659 | RSA-10AD1 | 511 | 7.30:1 | # S6LN-1208, S6LN-1209 |
| -C1A | TE0659 | RSA-5AD1 | 456 | 7.20:1 | S6LN-1208, S6LN-1209 |

NOTE 10. (cont’d)	The following tabulation shows turbocharger, fuel injector, weight, compression ratio and ignition.

| Model | Turbocharger | Fuel Injection | Weight (dry) lb. | Compression Ratio | Ignition, Dual * |
| --- | --- | --- | --- | --- | --- |
| -E1A | TE0659 | RSA-5AD1 | 453 | 7.20:1 | TCM** S6LN-1227, S6LN-1209 |
| -G1A | TE0659 | RSA-5AD1 | 456 | 8.50:1 | S6LN-1208, S6LN-1209 |
| -H1A | TE0659 | RSA-5AD1 | 453 | 7.20:1 | S6LN-1227, S6LN-1209 |
| -F2BD | TE0659 | RSA-10ED1 | 514 | 7.30:1 | D6LN-3200, /2230 |
| -J2B | TH08A60 | RSA-10ED1 | 558 | 7.30:1 | S61N-1208, S6LN-1209 |
| -J2BD | TH08A60 | RSA-10ED1 | 518 | 7.30:1 | D6LN-3200, /2230 |
| -K1AD | TE0695 | RSA-5AD1 | 459 | 8.00:1 | D6LN-3200, /2230 |
| -N2BD | TE08A60 | RSA-10ED1 | 518 | 7.30:1 | D6LN-3200, /2230 |
| -S1AD | TE0659 | RSA-10ED1 | 502 | 7.30:1 | D6LN-3000, /2231 |
| -R2AD | TH08A69 | RSA-10EB1 | 524 | 7.30:1 | D6LN-3200, /2230 |
| -T2AD | TH08A60 | RSA-10ED1 | 549 | 7.30:1 | D6LN-3200, /2230 |
| -U2A | 3AU(V)2ME60H3 | RSA-10ED1 | 578 | 7.30:1 | S6LN-1258, S6LN-1259 |
| -J2B | TH08A60 | RSA-10ED1 | 558 | 7.30:1 | S6LN-1208, S6LN-1209 |
| -V2AD | TH08A69 | RSA-10DB2 | 565 | 7.30:1 | D6LN-3200 |
| -W2A | TH08A69 | RSA-10ED1 | 554 | 7.30:1 | S6LN-1208, S6LN-1209 |
| -AB1AD | TE0659 | RSA-5AD1 | 474 | 8.00:1 | D6LN-3000 |
| -AB1BD | TE0659 | RSA-5AD1 | 474 | 8.00:1 | D6LN-3000 |
| -AA1AD | TE0659 | RSA-5AD1 | 483 | 8.00:1 | D6LN-3200 |
| -AE2A | TA0411(2) | RSA-10ED1 | 595 | 7.30:1 | Slick 6263, 6260 (right) |
| -AF1A | TA0413 | RSA-10ED1 | 473 | 8.00:1 | Slick 6261, 6260 (right) |
| -AF1B | TA0413 | RSA-10ED1 | 473 | 8.00:1 | Slick 6261, 6260 (right) |
| -AG1A | TAO413 | RSA-5AD1 | 485 | 8.00:1 | Slick 6361, 6360 (right) |
| -AH1A | TAO413 | RSA-10ED1 or FM-250 | 542 | 7.30:1 | Slick 6361, 6360 (right) |
| -AJ1A | TA6102 | RSA-10ED1 | 532 | 7.30:1 | Slick 6361, 6360 (right) |
| -AK1A | TA0411 | RSA-5AD1 | 464 | 8.00:1 | Slick 6361 (2) |
| LTIO-540-F2BD | TE0659 | RSA-10ED1 | 514 | 7.30:1 | D6RN-3200, /2230 |
| -J2BD | TH08A60 | RSA-10ED1 | 518 | 7.30:1 | D6RN-3200, /2230 |
| -K1AD | TE0695 | RSA-5AD1 | 459 | 8.00:1 | D6LN-3200, /2230 |
| -N2BD | TH08A60 | RSA-10ED1 | 518 | 7.30:1 | D6RN-3200, /2230 |
| -U2A | 3AU(V)2ME60H3 | RSA-10ED1 | 578 | 7.30:1 | S6LN-1258, S6LN-1259 |
| -J2B | TH08A60 | RSA-10ED1 | 558 | 7.30:1 | S6LN-1208, S6LN-1209 |
| -V2AD | TH08A69 | RSA-10DB2 | 565 | 7.30:1 | D6LN-3200 |
| -W2A | TH08A69 | RSA-10ED1 | 549 | 7.30:1 | S6LN-1208, S6LN-1209 |

* For alternate magnetos see latest edition of Textron Lycoming Service Instruction No. 1443

** TCM formally Bendix

NOTE 11.	Air from the compressor of the AiResearch Turbocharger is suitable for cabin pressurization.  The

installation must provide for cabin air temperature control from 300°F to 20,000 feet hot day condition

under maximum engine power.  For cabin air outlet flange dimensions see Textron Lycoming Drawing No.

63444 for -R2AD model.

A sonic nozzle must be provided to preclude affecting engine performance by cabin air bleed.

TIO-540-AE2A model is equipped with two sonic nozzles to supply air for cabin pressurization,

reference Lycoming Drawing No. 04D63570.

NOTE 12.	All models equipped with one impulse coupling magneto may use two impulse coupling magnetos as

optional equipment.

NOTE 13.	TIO-540-R2AD and LTIO-540-R2AD have an alt. rating of 340 hp at 2500 rpm and 44.0 in.Hg at S.L. and

325 hp at 2500 rpm and 44.0 in HG at 15,000 feet.

NOTE 14.	The maximum continuous ratings for these engines have been specified with 10 horsepower extracted at

the accessory drives.

NOTE 15.	The TIO-540-AB1BD has more effective counterweights for use with a McCauley propeller.

NOTE 16.	Engines produced prior to September 25, 2006 may have 20 degree timing. Owners and/or operators should refer to the engine dataplate for the correct timing setting.

.....END.....