Springfield Armory National Historic Site

| Object Name/Title: | CANNON, HAND |
| --- | --- |
| Catalog Number: | SPAR 710 |
| Classification: | HISTORY T&E FOR SCIENCE & TECHNOLOGY ARMAMENT-FIREARM |
| Artist/Maker: |  |
| Eminent Figure/ Organization: |  |
| Place of Manufacture: |  |
| Medium/Material(s): | METAL, WOOD |
| Measurements: | TUBE: 39.3CM 15 1/2" SOCKET: 18.4CM 7 1/4" __ __ |
| Date Manufactured: | c 1400 |
| Use Date: |  |
| Historical/Cultural Period: | European |
| Cultural ID: |  |
| Key Descriptor: |  |
| Description: | EUROPEAN HAND CANNON Maker unknown - Early (late 14th, early 15th century?) wrought iron hand cannon believed to be of European origin and consisting of a wrought iron tube with a socket and mounted on the end of a wood staff. The tube is made of one piece of iron wrapped around and forge-welded with a longitudinal seam. There are five reinforcing bands around it. The tube has a bore diameter of 2 1/8" at the muzzle. The tube itself is 15 1/2" long and the socket portion is an additional 7 1/4 inches. Fitted into the socket is a round wooden staff four feet long approximately 1/4" in diameter. The small vent at the rear of the tube is not enlarged and is clear. Bore is extremely smooth. The wood staff appears to be a replacement. No visible markings. Army #0652 - See drawing attached SA8112. In photo with multiple photos this hand cannon is on top. This hand cannon was loaned to West Point in 1967. Exhibit label: "One of the very earliest firearms, dating from the late fourteenth century, was simply a tube mounted on a pole that could be pointed toward the enemy. A match had to be held in the hand, making it awkward to aim and fire the piece with any accuracy." Notes: "Those blessed ages were fortunate which wanted the dreadful fury of the devilish and murdering pieces of ordnance, to whose inventor I am verily persuaded that they render in hell an eternal guerdon (reward) for his diabolical invention, by which he hath given power to an infamous, base, vile, and dastardly arm to bereave the most valorous knight." - Don Quixote This is a "gonne" in its earliest and most simple form - essentially a piece of pipe on the end of a stick. Ammunition for these pieces consisted of stones, or possibly stones wrapped in leather or sheet lead and propelled by "white powder." Their range and effectiveness was extremely limited, but they depended on their noise, flash and smoke to frighten the enemy, particularly the horses. Undoubtedly, a man could probably throw a stone as far as these would fire one, and other weapons such as crossbows were certainly more effective. The date of the origin of small or portable arms is as difficult to fix as is that of the cannon. One writer says they were first produced by the Italians in 1430, but mention is made in the Chronicles of Boulogne in 1397 of "hand cannon," or, as they are designated, "sclopos," and subsequently "sclopeto," and hence "escopette," "escopeta," etc. Valturius, who wrote in Latin, speaks of them as machines of war used by the Romans of antiquity, and he called ballista, from the name of the ancient ballistes of the Roman armies. They were also termed bombards, hand cannon, and culverins. At first they were simply small cannon fastened to a stick and fired from the ground, their front end supported by a crutch or rest. "The first hand cannons, which appeared around the middle of the fourteenth century, were neither portable nor reliable compared to handguns of the nineteenth and twentieth centuries. Yet these hand-held firearms represented a first step toward the invention of guns designed to be fired with one hand. The earliest hand cannons were short tubes of either cast bronze or wrought iron. At first, the cannons were made with a touchhole, which permitted application of a glowing ember of some smoldering tinder to ignite the black gunpowder. Evidence indicates that early hand cannons were clumsy, two-handed weapons, and most required a wooden pole, called a tiller or haft, to help aim and control it. By the mid-fourteenth century, however, small guns were becoming increasingly popular. In 1364, the town government of Perugia, on the Italian peninsula, ordered 500 hand cannons, each to be no longer that the palm of a person's hand ("500 bombards of a span in length, which can be carried in the hand"). These early hand firearms were made in a variety of sizes, shapes, and bore diameters. Swedish examples dating from the late 1300s range in length from 193 millimeters (bronze hand cannon from Morko) to 300 millimeters (bronze hand cannon for Loshult). The wrought-iron hand cannon found at Tannenberg castle in Hesse was about 320 millimeters long, with a muzzle bore diameter of 17.5 millimeters. It was fabricated in 1399, the year the castle was destroyed during a siege. Clearly, the fourteenth century pistol - the word 'pistol' was possibly derived from the Bohemian pist'ala or from the Italian pistola - was not a handgun as we define it today. The length of the tiller depended on the length of the barrel, with the rudimentary stock averaging one meter long. The hand cannon was the common ancestor of the handgun and the shoulder-fired weapon, and many years passed before a clear differentiation was made. Although, they followed separate but related paths of development, the handgun and the shoulder-fired weapon shared a common objective: defeating the enemy." - Edward C. Ezell LOAN HISTORY: Army# 0652 - Loaned to Major Samuel Gordon, Holyoke, Ma. from May 6, 1959 to May 7, 1959. Army# 0652 - Weapon loaned to the Sahara Antique Gun Show, Hotel Sahara, Las Vegas, Nev. from 20 January 1969 to 2 February 1969. At that time weapon was appraised at $200.00. References: Ezell, Edward C. HANDGUNS OF THE WORLD. Stackpole Books. Harrisburg, Pa. 1981. |