Documentation Guide for:     CHANNEL MODIFICATION

BACKGROUND

The NRCS has stream channel guidelines to aid their personnel in identifying when and where channel modification may be used as a technique for implementing water and related land resource projects.  They will be used in the planning of all NRCS projects or measures which qualify for either technical, financial, and/or credit assistance under the authorities for flood prevention projects, small watershed projects, and RC&D projects.

Channel modification can be thought of in two generalized concepts: 1) restoration of the channel, and 2) straightening of the channel. Restoration can include, but is not limited to, the restoration of channel meanders patterns to improve geomorphic conditions.  Straightening the channel or moving a channel to a different location (far to one side of its floodplain) is considered stream channelization.  The latter category can also include clearing and snagging, widening and deepening the channel.

Channel modification is an alternative that may be utilized in solving specific water management problems.  It may be needed to restore a watercourse impaired or damaged naturally or through improper management of associated uplands.  A thorough knowledge of stream dynamics is essential in order to be able to identify existing and potential problems and evaluate the viability of all alternatives.

High flows in rivers and streams and periodic overflows are dynamic natural processes that periodically revitalize riparian habitats by altering the characteristics of floodplains by redistributing substrates in the process of moving water and sediments through a watershed.  Erosion, flooding, and sediment deposition are products of this process, and can cause damage to the rivers and streams of the United States where humans have developed infrastructure in or near these fluvial systems.  These occurrences may result in loss of life, property damage, and may have short-term negative impacts to water quality and fish and wildlife habitat.  Flooding is an essential component of stream systems, and generally result in improved stream conditions except in areas that have been destabilized due to human impacts or recent natural disasters (such as wildfire).

Channelization of streams causes significant damage to fish and wildlife resources.  In addition to direct environmental impacts, the practice can directly or indirectly involve the drainage of wetlands, clearing of bottomland forests, and increase flooding and siltation in downstream areas.  Channelized streams are not naturally stable and therefore require armoring of both the bed and banks of the channel.  Annual maintenance is generally required.

POLICY

It is the policy of NRCS that care and effort will be made to maintain and restore streams, wetlands, and riparian vegetation as functioning parts of a viable ecosystem upon which fish and wildlife resources depend.  An interdisciplinary planning process will be used which will permit a balancing of the need to maintain a viable, naturally functioning ecosystem, food and fiber, economic, and other social needs.  The application of these guidelines, resource inventory, interpretation, and planning assistance provided by NRCS shall ensure identification and consideration of alternatives to channel modification.

An EA is required for projects that include restoring stream channel meander bends or other similar activities.  An EIS is required for projects that include stream channel realignment (channelization) or work to modify channel capacity by deepening or widening where significant aquatic or wildlife habitat exists.  The EE will determine if the channel supports significant aquatic or wildlife habitat (GM Part 410.7).  Channel realignment is defined in the General Manual 190, 410.27 (c)(2)(i) as actions including the construction of a new channel or a new alignment and may include the clearing, snagging, widening, and/or deepening of the existing channel.

Channelization will not be considered if a practical alternative exists.  A practical alternative is defined as one that: 1)  makes a significant contribution to project objectives, and 2) results in less damage to fish and wildlife habitat.

The following categories of alternatives may be considered singly or in combination:

- Soil and water conservation practices.

- Nonstructural measures may include but are not limited to, land use regulation, land acquisition, the maintenance of aquatic areas, floodplain zoning, flood-proofing existing buildings, flood forecasting, flood warning, flood hazard information, flood insurance, tax adjustments, emergency assistance, and relocation of properties and people.

- Structural alternatives may include but are not limited to, dams, floodways, dikes, levees, flood walls, pumping plants, diversions, wetland development, maintenance, and restoration

Formulation of alternatives will consider inputs from interested agencies, organizations, or individuals interacting with sponsoring local organizations.  Compliance with NEPA is required .  Alternatives to channelization will be developed, evaluated and recommended when the action is expected to cause measurable losses of fish and wildlife resources.

Channel modification, if used, will be the minimum required, either alone or in combination with other measures.  It will be accomplished using the least damaging construction techniques and equipment in order to retain as much of the existing characteristics of the channel and riparian habitat.  Construction practices include seasonal construction, minimum clearing, reshaping spoil, limiting bank modification to one or alternating banks (to maintain a riparian corridor), and prompt revegetation of disturbed areas.

Channel modification may be considered as an alternative, providing it does not jeopardize the continued existence of State or Federally listed endangered and threatened species, result in restricted access to use of stream segments developed specifically for recreation or fish and wildlife use by the public, and its intended purpose is not to alter wetlands.

Channel modification will not be considered as an alternative unless it can be accomplished with little or no direct or indirect adverse effect on: 1) streams proposed or designated as Wild and Scenic Rivers, or officially designated by Federal or State legislative actions for their important natural, esthetic, or recreational values; 2) streams located in, or flowing through, or contiguous to, established wilderness areas, parks, refuges, or other areas that have been set aside pursuant to Federal or State legislative actions for fish and wildlife esthetic or recreational values; 3) important fish and wildlife habitat values in the project area, State or Nation after providing for all appropriate mitigation, compensation, or preservation measures.

Detailed coordination procedures shown in GM 410.28 will be used in the planning of water resource projects.

| Project: |  |
| --- | --- |
| Date: |  |
| Planner: |  |

Casefile Documentation:   CHANNEL MODIFICATION

When correctly and completely evaluated this will provide adequate documentation for compliance with the National Environmental Policy Act (NEPA).  Attach to NM-ENV-1 when completed.

STEP 1.  Is the proposed action or activity scheduled to occur, involve channelization of a stream or restoration of a stream, wetland, or riparian area?

Neither			Channelization	 		Restoration

- If your answer is “Neither”, no additional documentation is needed concerning channel modification.  Proceed with planning.

- If your answer is “Channelization”, go to step 3.

- If your answer is “Restoration”, go to step 2.

STEP 2.  What effect (short-term and long-term) will the proposed restoration action or activity have on fish and wildlife resources?  Effects determinations must be reviewed by a Wildlife Biologist.

POSITIVE			NEGATIVE

- If there are "Positive" effects and the effects are consistent with the maintaining, protecting and preserving fish and wildlife resource characteristics, document (describe) the positive effects and attach it to the Environmental Evaluation and Resource Inventory Checklist (NM-ENV-1) send it to a Wildlife Biologist for review and concurrence.  Positive effects can be long-term, short-term or both.  An Environmental Assessment (EA) will need to be completed.  Contact the State Biologist as soon as possible.  An EA involves public scoping to determine if the public has any concerns with the project, several 30-day comment periods to allow the public to review the EA and to provide comments, and the determination of whether or not the project will significantly impact the human environment.

- If there are "Negative" effects, document (describe) the effects and attach it to the Environmental Evaluation and Resource Inventory Checklist (NM-ENV-1) refer to the Documentation Guide regarding the NRCS policy on Natural Area consultations with other groups.  Send this to the State Biologist for review and concurrence.  Negative effects can be long-term, short-term or both.  An Environmental Assessment will need to be completed.  Contact the State Biologist as soon as possible.  Detailed coordination procedures are shown in GM 410.28.  Contact the Team Leader about a need for an Environmental Assessment according to NRCS regulations in the General Manual (GM 190 Part 410).  An EA involves public scoping to determine if the public has any concerns with the project, several 30-day comment periods to allow the public to review the EA and to provide comments, and the determination of whether or not the project will significantly impact the human environment.

STEP 3. What effect (short-term and long-term) will the proposed restoration action or activity have on fish and wildlife resources?  Effects determinations must be reviewed by State Biologist.

POSITIVE			NEGATIVE

- If there are "Positive" effects and the effects are consistent with the maintaining, protecting and preserving fish and wildlife resource characteristics, document (describe) the positive effects and attach it to the Environmental Evaluation and Resource Inventory Checklist (NM-ENV-1) send it to the State Biologist for review and concurrence.  Positive effects can be long-term, short-term or both.  An Environmental Impact Statement (EIS) will need to be completed.  Contact the State Biologist as soon as possible.  An EIS involves a Notice of Intent to prepare and EIS, public scoping to determine if the public has any concerns with the project, preparation of a document with several alternatives, several 30-day comment periods to allow the public to review the EIS and to provide comments, and a Record of Decision.

- If there are "Negative" effects, document (describe) the effects and attach it to the Environmental Evaluation and Resource Inventory Checklist (NM-ENV-1) refer to the Documentation Guide regarding the NRCS policy on Natural Area consultations with other groups.  Send this to the State Biologist for review and concurrence.  Negative effects can be long-term, short-term or both.  An Environmental Impact Statement will need to be completed.  Contact the State Biologist as soon as possible.  Detailed coordination procedures are shown in GM 410.28.  Contact the Team Leader about a need for an Environmental Impact Statement according to NRCS regulations in the General Manual (GM 190 Part 410).  An EIS involves a Notice of Intent to prepare and EIS, public scoping to determine if the public has any concerns with the project, preparation of a document with several alternatives, several 30-day comment periods to allow the public to review the EIS and to provide comments, and a Record of Decision.

Documentation Notes: