POLAR Spacecraft Status As of: Weds July 6, 16:00 EST

(changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM: 1651=AN STIP data only | NOAA-14J AM: 2039=AN STIP data only | NOAA-15K AM: 1806=AN GAC data only | NOAA-16L PM: 1441=AN GAC&LAC data | ##### NOAA-17M AM: 2225=AN GAC&LAC data | ##### NOAA-18N OV PM; 14:00=AN APT/HRPT/GAC |
| --- | --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 12/30/94 | 05/13/98 | 09/21/00 | 6/24/02 | 5/20/05 |
| CNTL OBP | OBP2 | OBP2 OBP1 inoperative | OBP2 | OBP2 | OBP2 | OBP2 |
| BUS | B-BUS XSU-1 | B-BUS XSU2 (XSU1 inop) | B-BUS XSU-1 | B-BUS XSU-1 XSU1 TOAR 444 | B-BUS XSU-1 | B-BUS XSU-1 |
| BUS VETO | NO | NO | NO | NO | NO | NO |
| ADACS X=Yaw Y=Roll Z=Pitch | Nominal Mode RGmode=Monitor Gyro chans=XYZ | Nominal Mode Rgmode=Disabled Gyro chans=XYZ SS check Disabled when mode=YGC | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAA | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB Gyro-2 current spike under investigation | Nominal Mode Gyro-A MIMU-2 Z-wheel -Pitch TOAR 447 |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI BU random enables daily bias–7.0 mS | PRI daily bias -8 mS +100 mS 31Aug04 | PRI Daily Bias –5.5 mS -100 mS 31Aug04 | PRI Daily Bias –3 mS | PRI 23 May05 |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 | 2 |
| MIRP | External Synch Rephase Enabled | External Synch Rephase 0730 Daily | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled | External Synch Rephase Enabled |
| TIP SIDE | 1 | 1 | 2 (since 4/23/03) 3 Channels inop | 2 since 4/18/05 | 1 Analog telem drifting | 1 |
| DTRs | 3A, 4, 5B Safestate 5B | 1, 2B, 5 Safestate- 1B | 1, 2, 3, 4 Safestate 1B EOT | 1,2,3,4A Safestate 3A 4B STIP only | 1,2,3,4 Safestate=1B DTR2-SSR | 1,2,3,4,5 All solid state digital. Current spikes under investigation TOAR 450 |
| VTX (APT data) | #1 ON 137.50 MHZ | OFF Since 8/14/02 | #1 ON 137.50 MHZ | INOP OFF since 11/15/00 | #2 ON 137.62 MHZ Powered off over CDA’s 3-5 Jul | #2 ON 137.9125 MHZ |
| BTX | #1 136.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ | OFF Since 30May05 | #2 ON 137.77 MHZ | #2 137.77 MHZ |
| STX #1=1698.0 mhz #2=1702.5 mhz #3=1707.0 mhz #4=2247.5 mhz | 1 – HRPT; 2,3 - Playback | 3 – HRPT 2, 1 – Playback | 2-Omni HRPT Power drop TOAR 446 4-Playback 1,2,3-Degraded | 1 – HRPT; 2– Playback; 4 – Playback 3 low power unused | 3 – HRPT TOAR424 low power 2 & 4 Playback 1-Standby STX-1 output power drop TOAR 443. | 3 - HRPT 1&2 Playback STX1&3 swap assignments occurred 28June 4-Standby |
| ### POWER (Array offset and CANT angle, charge & V/T rates, eclipse states and sun angle) | ArrayOff +60 since 6/23/04 Eclipse season 21Apr-17Aug Sun Angle 37 start downtrend to 12 mid-Oct 05 Batts 1&2=LRC V/T=4 Shunt degraded SA CANT=37 degs | Array Off -55 Since 8/21/03 Eclipse all year Sun angle 51 Steady trend Batts 1-3= LRC Batts 1&3V/T=7/11 Batt2 VT=8/11 SS trigger =18.98 Shunt Degraded SA CANT=22 degs | Array Off -45 Since 1/25/01 Eclipse Season 16 May-23 Jul Sun Angle 30 downtrend to 2 degs on 10 Oct 05 Batts 1-3= LRC V/T=7/9 SA CANT=37 degs | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 58 Start downtrend to 41 on 30 Oct 05 Batts 1-3= LRC V/T=4/8 SA CANT=22 degs | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 70 Steady trend Batts 1-3= LRC V/T= 4/8 SA CANT=37 degs | Array Off. -40 Since 6/6/05 Eclipse all year Batts 1-3=LRC Batts 1-3 V/T=4/8 SA CANT=22 PSSS enabled 30May |
| AVHRR | Nominal | INOP Scan motor stalled 14 Apr 04 | Nominal Rare major scan motor surges. Last Incident 11/18/04 | Operational TCE24 OFF 4/14/04 Bar coded imagery typical since 29June05 Channel 3B only. | ### Nominal 3A-B switch enabled | Nominal 3A-B switch Enabled 6/23/2005 |
| AMSU-A1 |  |  | Operational Ch14 inoperative Ch11 inoperative | Operational Primary PLLO chans 9-14 bias shift. Backup PLLO inop. Channel 9-14 noise | OFF/INOP Since 10/30/03 Survival heat on. | Nominal Space view 1 Since 6/7/05 |
| AMSU-A2 |  |  | Nominal | Nominal Space Position-2 | Nominal | Nominal Space view-2 since 6/08/05 |
| AMSU-B |  |  | Operational Bias in All Chans. Motor surges most recent 11/14/04 IPD Reports of degraded data on Chan-3 since 4/30 | Operational Space Position-3 IPD reports channel 8 degradation | Nominal | Replaced by MHS |
| ### HIRS | OFF/Inop Turned off 6/23/05 For power balance Filter Wheel turned OFF 10/17/02. | NOMINAL Operational Occasional loss of Fwheel synch | INPROVED Upgraded to Yellow status 16 May 05 FW Abrupt return to nomial at 09:00Z TOAR 445 open FW mode=Hi Powr FW Heater OFF Frequent Filter | Operational High levels of imagery radiometric noise TOAR 441 open Occasional major FM surges, most recent 12/9/04 | NOMINAL 1 Pixel cross track misalignment . | ON TOAR 448 Noise in long wave channels 1-12. Some improvement reported. Investigation ongoing |
| ### MHS/MIU |  |  |  |  |  | Nominal Final config on 6/4 Now Operational |
| MSU | OFF/Inop Turned off for power balance 6/23/05 | Operational Scan motor and antenna problems |  |  |  |  |
| SSU |  | Nominal |  |  |  |  |
| DCS | Nominal | Nominal | Nominal | Nominal | Nominal | ON DRU’s 1-7 On DRU-8 TOAR 449 turned off 6/13 |
| SARR |  | Nominal | Operational A-Side, AGC Mode Intermittent failures for 243MHz A&B | Operational Switched back to mode AA 26Apr05 14:10Z A-side, AGC Mode (B-side in FG mode) 243 MHz = INOP. | Nominal A-side All AGC Mode | ON A-Side AGC-mode (correction from last week) Since 27 June |
| SARP |  | OFF Power Failure | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal | ON A-side 23 May All DRU’s on |
| SEM | ON/Nominal Turned on 8/30/04 Elec-CH-HVPS=4 Prot-CH-HVPS=0 | Operational Elec-CH-HVPS=0 Prot-CH-HVPS=0 1 of 4 Tscopes inop | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=5 P-CDEM-HVPS=4 | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=3 | ON Turned on 6 June at 14:05z |
| SBUV |  | Degraded Grate motor sticks Flex memory 3 (giant step) ops suspended 12/2/03 Reduced operations |  | Operational Degraded Ozone data Backup Diffuser inop Primary Diffuser deployment problem TOAR 426 AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode Plan sweep mode Standard Ops Procedure change starting 17 May | ON since 27 May Commanded to Cathode mode 5 July 17:15z Back to anode mode 6 July 13:30z OV testing continues |
| SADPOS |  |  | N/A | ON Enabled MFactor50% Timer = 12 mins | ON ENABLED Timer = 12mins | ON Enabled |
| THERMAL Heaters, louvers TCE’s | Nominal TCE’s back to nominal configuration following 7/25/04 UVtrip. | Nominal | TCE26 & 15H Telem invalid TOAR 432 HIRS FW Heat tuned ON 17 Apr and OFF on 18 Apr TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 | Nominal TCE24 OFF 4/14 (AVHRR H&L) HIRS Filter heater turned OFF 1/18/05 | TCE3H Batt 2A heater failed ON, turned off 8/26/03 TOAR 428 CLOSED 9/14/04 no corrective action possible. AMSU-A1 survival heaters on 10/29/03 | All TCE’s active |
|  | N12 | N14 | N15 | N16 | N17 | ### N18 |

COMMENTS/CHANGES:

NOAA-12	The N12 battery’s maximum discharge voltage began to slowly trend upward (exiting eclipse at approximately 19.7

volts) during the past week in conjunction with the start of its eclipse season waning.  Shunts also improved over past weeks (from a workload perspective) although as eclipse season continues to wind down additional loading will be necessary to maintain these reduced shunt workloads.  On the other hand, battery temps remained relatively elevated  (averaging 10.5 degs) during the past week.  Additional reconfigurations are anticipated for the N12 power subsystem in the coming weeks although, other than turning HIRS and MSU back on, the specific operations are not yet defined.  Carl Gliniak is EMOSS POC for this situation.

NOAA-14:	Spacecraft is operating nominally and power subsystem remains stable and above safe state threshold values as the

percentage darkness time per orbit has stabilize at or slightly below 20% darkness per orbit which will be sustained for the remainder of the year.   Otherwise there were no changes to N14 during the past reporting period.

NOAA-15:	The N15 AMSU’s were all generally stable and nominal during the past week (illustration 1) including the AMSU-B which quickly recovered from a scan motor current surge the previous week.  The N15 AVHRR and the HIRS were also stable and nominal throughout the week (illustration 2) with the AVHRR scan motor current dropping abruptly several mAmps on 5 July (typically a positive development).  N15 continues progressing nominally through its eclipse season which recently peaked and is now waning (illustration 3).  No power subsystem reconfigurations appear even remotely necessary.  There has been no change to the STX-2 situation on N15 which experienced a significant drop on 27 May.  The transmitter continues to transmit HRPT data at approximately 5.6 watts down from the previously nominal 7.5 watts.  EMOSS engineers have seen no indication of HRPT imagery degradation as a result of this event nor have any HRPT user complaints been received.

NOAA-16:	The N16 AVHRR scan motor current continued its slow increase into higher mean values for the 12th week in a row

(illustration 4) and is now operating within the threshold where bar coded imagery appears.  In contrast, the synch delta values have actually started to slightly improve, that is they are not always pegged as they were last week (illustration 5) although they remain far from nominal at this time.  Bar codes started to appear on HRPT, GAC and LAC images starting on 29 June and, with each passing day,  have become more prevalent (illustration 6 & 7).  At the present time almost all N16 AVHRR images contain at least partial bar codes on them and in many cases it is continuous.  In addition, AVHRR temperatures remain nominal and the TCE’s that control AVHRR temperatures remain turned off, primarily to keep the AVHRR cooler.  On the positive side, the AVHRR situation does not seem to have influenced the N16 HIRS in any way (illustration 8) contrary to what occurred the previous time that the AVHRR was anomalous.   Although there have been spurious reports of degraded N16 HIRS products during the past several months, from an engineering point of view the N16 HIRS has been stable and nominal during that entire period.

NOAA-17:	The remaining two operational AMSU’s, the AVHRR and the HIRS on N17 were nominal during the past week

(illustrations 9 -11).  Gyro-2 spin motor current was relatively stable at its new higher level although it did show some slight signs of less than perfect stability (illustration 12).  The spin motor notwithstanding, N17’s ADACS also appeared to operate nominally during the last reporting period.

On 3 July at 08:50z, the N17 VTX-2 transmitter, used for APT transmissions, began turning off over each CDA support and then powering back on when the support was completed.  This commanding was initiated by the stored command table with the turnoff commands only occurring during CDA supports, but not during blind periods (illustration 13).  Numerous reports were received by NOAA from outside users concerning this situation which was corrected on 5 July at 14:00z by taking the VTX-2 off commands out of the stored command table and commanding VTX-2 back on continuously.  Along with the N16 BTX turnoff (which remains in effect) concerns regarding this VTX situation are centering on the SOMS program and the current configuration of its RF priority algorithms.

As stated last week, starting 11 July, N17 will be used to test Svalbards ability to receive GAC (or any playback) data and re-transmit that data back to the SOCC.   Should this concept prove to be viable, Svalbard could then be incorporated into POES data dump scheduling thus reducing data latency and enhancing spacecraft analysis capabilities.

NOAA-18:	The OV period officially ended on 3 July for N18.  However, N18 is not expected to become “fully operational” until

late summer.  Therefore, until that day, the operational status of N18 can be considered “transitionally operational”.   Some OV testing continues primarily on SBUV while other tests which are being performed to investigate TOARS that were generated by NASA during the OV period (HIRS, Z-axis control, DCS-DRU8) .  During the past week NASA opened one more TOAR on the DDR current spikes which have been discussed in this report previously.  HIRS testing continued under the coordination of a tiger team that is investigating the higher then desired noise levels in channels 1-12.   Last week EMOSS did received reports that the noise problem was improving but had not yet reached nominal levels.  This week the latest HIRS test on behalf of this investigation had EMOSS engineers dwell on the Z-reaction wheel current, then switch SATCU from Mode-B (normal on-orbit configuration) to Mode-A (primarily a launch/deploy configuration), then point HIRS to Space View for 2 orbits after which it was returned to full scan mode.

This operation was nominally implemented by EMOSS engineers Jim Walters and Sally House although the test results produced by this operation are unknown by EMOSS at this time.

One lesson learned from the SATCU operation was that when switching SATCU to Mode-A SADPOS should be disabled before the switch occurs and remain disabled until the switch back to Mode-B.  The reason for implementing this reconfiguration is that Mode-A replaces two of the three illumination sensor values (CSCLI1 and 2) used by SADPOS to determine if the sun was inadvertently lost with two other, non SADPOS related values (array hinge deployments).  However, while in Mode-A SADPOS remains unaware that these illumination sensors readings have been replaced with the hinge readings and continues reading the raw hinge values as CSCl1 and 2 illumination readings.  Fortunately, the two hinge deployment raw values (which should never change after array deployment) are in conflict relative to how they are read by SADPOS (one hinge raw value is the raw equivalent of spacecraft lit the other hinge is equivalent to spacecraft dark).  Therefore, SADPOS will “break the tie” by using SADSCI (which is not replaced in SATCU mode switches) to determine spacecraft illumination status.  As long as SADSCI remains nominal, operational and/or not “locked out”, the worst case scenario of switching to SATCU Mode-A with SADPOS enabled is that an illumination sensor miscompare will be reported (the conflicting hinge readings). However, if for any reason, the SADSCI sensor is non-operational and/or is locked out, switching to SATCU mode-A with SADPOS enabled would immediately result in the SADPOS timer starting its 8 minute (to be changed to 15 minutes prior to the next solar eclipse) countdown timer after which, without intervention by the ground, SADPOS would take over control of the solar array and move the offset to zero degrees.  (Refer to illustration 14.)

Also on 5 July, the SBUV was switched to cathode mode for one day (at approximately 17:00z) and then back to anode mode at 13:30z on 6 July.  Jim Walters and  Sally House were the EMOSS engineers in charge of this implementation.

SBUV testing out of stored command also continued during the past week on Jday 180, 181, 182, and 183) the results of which are assumed to have been nominal.  (Refer to Procedure 050620-1505:  NOAA-18 SBUV Activities J172-183.)

Because of projected frequent and planned RF conflicts with N16, N18 had its STX assignments swapped on 28 June at 01:30z.  The swap made STX-3, previously a playback STX, into the HRPT transmitter and reassigning STX-1, previously HRPT transmitter to playback operations.   Additional transmitter re-assignments for a variety of spacecraft and transmitter types are planned for August of 2005.  Will Chadwick is EMOSS POC for these operations.

See illustrations 15-21 for a sample of N18 performance over the past week.

Illustration 1:  N15 AMSU’s Analysis:

(Nominal and stable)

Illustration 2:  N15 AVHRR & HIRS Analysis

(Nominal and stable. Perhaps even a little improvement in the AVHRR scan motor)

Illustration 3:  N15 EPS Analysis

(Eclipse season beginning to wane, nominal behavior)

Illustration 4:  N16 AVHRR Analysis

(As has been a concern reported in this report over the past several weeks, scan motor current degradation has now breeched the bar code imagery threshold.)

Illustration 5:  N16 AVHRR Synch Delta Analysis

(Not as bad as last week but not at all good either.  Still rephasing each orbit with the synch delta typically pegging out again within 15-30 minutes.)

Illustration 6:  N16 AVHRR GAC & LAC Analysis

(Note on the GAC’s how the rephase (RE) does clean up the imagery.  Unfortunately, as depicted by the 6 July GAC, the clean up doesn’t last long.)

Illustration 7:  N16 AVHRR HRPT ANALYSIS

(First bar code showed up on 29 June, by 3 July over 50% occurrence was noted.  By today almost every image is bar coded..)

Illustration 8:  N16 HIRS Health Analysis

(Nominal and stable. No apparent influences from the AVHRR problems)

Illustration 9:  N17 AMSU Health Analysis

(Nominal and stable.)

Illustration 10:  N17 AVHRR Analysis

(Nominal and stable except for some spikes in the electronics current.)

Illustration 11:  N17 HIRS Health Analysis

(Nominal and stable)

Illustration 12:  N17 ADACS Performance Analysis

(Generally nominal and stable although a reduction is the fluctuation in Gyro-2 current would certainly be preferred.)

Illustration 13:  N17 VTX-2 (APT data) Analysis

(Note the turnoffs that inadvertently occurred on 3, 4 and 5 July.  Problem has since been resolved.)

(data source=ABE)

Illustration 14:  N18 SATCU Mode Swap Analysis

(Note the change in CSCLI1&2 and no change in SADSCI.  Also note how one hinge is relatively high, which is read by SADPOS as spacecraft illuminated while the other is below the darkness threshold and is read by SADPOS as spacecraft dark.  When such conflict occurs, SADSCI is the deciding factor regarding the initiation of the SADPOS timer.)

Illustration 15:  N18 AMSU Health Analysis

(Nominal and stable)

Illustration 16:  N18 HIRS Health Analysis

(Nominal and stable from an engineering perspective.  Testing and tiger team coordination continue. )

Illustration 17:  N18 AVHRR Analysis.

(Stable and nominal)

Illustration 18:  N18 SBUV Analysis

(Stable and nominal.  Testing continued last week with a one day switch to cathode mode on 5 July and back to anode mode on 6 July.)

Illustration 19:  N18 MHS Analysis

(Data spikes cause not yet ascertained.  Possible ground processing problem. Source of data = ABE.  )

Illustration 20:  N18 DHS/Comm Health Analysis

(DDR spikes not as frequent as previous weeks although a new TOAR was submitted last week.  STX transmitter performance following swap appears to be nominal.)

# Illustration 21:  N18 ADACS Health Analysis

(Y-wheel current showing additional increases beyond that from previous weeks.  Otherwise performance looks nominal and stable.)

# N17 MEMORY SCRUB REPORT

# Last week.

# POLAR SPACECRAFT TIP CLOCK ERROR

## WEEK ENDING July 1, 2005

| DAY/DATE | NOAA-12 | NOAA-14 | NOAA-15 | NOAA-16 | NOAA-17 | NOAA-18 |
| --- | --- | --- | --- | --- | --- | --- |
| FRI. 06/24 | -761 msec | +202 msec | -202 msec | +121 msec | +060 msec | -076 msec |
| SAT. 06/25 | -763 msec | +203 msec | -204 msec | +121 msec | +063 msec | -074 msec |
| SUN. 06/26 | -764 msec | +209 msec | -204 msec | +122 msec | +064 msec | -073 msec |
| MON. 06/27 | N/A | +209 msec | -205 msec | +122 msec | +064 msec | -072 msec |
| TUE. 06/28 | -766 msec | +206 msec | -205 msec | +122 msec | +065 msec | -072 msec |
| WED. 06/29 | -766 msec | +206 msec | -207 msec | +123 msec | +065 msec | -070 msec |
| THU. 06/30 | -768 msec | +207 msec | -208 msec | +126 msec | +065 msec | -073 msec |

EMOSS POC for this chart is JIM SHEPHERD

NOAA-14:  As of 12/23/03 the TIP clock is corrected daily by –7.0 msec to compensate for drift.

NOAA-15:  As of 04/22/03 the TIP clock is corrected daily by –8.0 msec to compensate for drift.

NOAA-16:  As of 12/23/03 the TIP clock is corrected daily by -5.5 msec to compensate for drift.

NOAA-17:  As of 12/23/03 the TIP clock is corrected daily by –3.0 msec to compensate for drift.

NOAA-12:  As of 12/10/03 the TIP clock is corrected daily by +6.5 msec to compensate for drift.

*    A clock jump of +500 milliseconds on 01/31/05 due to fractional second daily ETCUP. A clock

jump of +500 milliseconds on 04/17/05 due to fractional second daily ETCUP. Alternating +/-

500 millisecond clock jumps will ccur at 76-day intervals as long as fractional second daily

ETCUP is utilized to manage clock drift. The next +500 millisecond jump is expected on 07/02/05.

POES Engineering Contact List

Government

Renee Smith-Dearing  NOAA POES Constellation Lead 		301-817-4117

Dong Han:  NOAA, POES Payloads				301-817-4127

Micky Fitzmaurice:  NOAA, POES Bus, IJPS	 	    	301-817-4016

Nicolaie Todirita (Todi): NOAA Ground Systems			301-817-4229

EMOSS-2 Contractor

Jeff Devine: EMOSS POES Lead	 				301-817-4020

Jim Walters:  EMOSS, POES Payload Engineer			301-817-4033

Carl Gliniak, EMOSS POES Power and Thermal 			301-817-4207

Jim Sheperd, EMOSS POES Attitude Control			301-817-4030

Jon Woodward EMOSS, POES Flight Software C&CS		301-817-4019

Sally House EMOSS POES Payloads 				301-817-4026

William Chadwick EMOSS, POES Comm and DHS			301-817-4015