Answers

Testing for Machine-Cleaned Foreign Material

- A No. 3, 12/64 round-hole sieve.

- Foreign Material.

- Approximately 1000 grams.

- Step 1 - Sieve approximately 1000 grams of the original work sample with the

No. 6 sieve by using the Carter Dockage Tester.

Step 2 - Handpick the foreign material and other grains that remain on the No. 6 sieve from not less than a 30-gram portion.

Step 3 - Add the two percentages, in hundredths, and round to tenths.

- An 8/64 -inch round-hole sieve, and an 8/64, 9/64, or 10/64 by 3/4-inch slotted sieve.

- By placing the 8/64-inch round-hole sieve on a bottom pan, and then placing the 8/64, 9/64, or 10/64 by 3/4-inch slotted sieve on top of the round-hole sieve.

- The perforations must be parallel to the motion of the sieving action.

- All matter, including soybeans and pieces of soybeans, which will pass readily through the 8/64-inch round-hole sieve, and all matter other than soybeans remaining on top of the sieve after sieving.

- Five (5) strokes.

- By adding the percentage of coarse foreign material removed by handpicking to the percentage of fine foreign material removed by sieving.

- Approximately 125 grams.

- 28 g  1020 g = 2.74 percent coarse foreign material.

3.26 g  125 g = 2.60 percent fine foreign material.

2.74 - Coarse

+ 2.60 - Fine

5.34 rounded to 5.3 percent total foreign material.

- 1.25  52 = 2.4 percent foreign material.

- A Carter Dockage Machine.

- It uses aspiration (air) to remove light-weight material and a combination of riddles and sieves to remove material of similar size and weight.