1.	Objectives

Test all equipment and cabling to handle operational, minimum, and maximum return data rates from "end-to-end".

2.	Requirements to be Tested

a.	The return patch panel and all the cabling to and from it can input "telemetry" data from a clock and data synthesizer, output it to a receiver-driver, input it from a receiver-driver, and output it to a Monarch board at various return data rates.

b.	Each receiver-driver can input "telemetry" data from the return patch panel and output it to the return patch panel at various return data rates.

c.	Each Monarch board can lock onto input "telemetry" data at various return data rates.

d.	The PTP's can output "telemetry" data received from the return patch panel to a "MOC" PTP via an IP socket at various return data rates.

e.	"Telemetry" data sent from a clock and data synthesizer into the return patch panel, then to a receiver-driver, then to Monarch boards, and finally to a "MOC" PTP via IP socket will exhibit no more than 1 bit error per 106 bits (a Bit Error Rate, or BER, of 1.E-06) at the following return data rates:  100, 1000, 2000, 4000, 4864, 5180, 6000, 8000, 16000, 32000, and 512000 bps.  Refer to figure 1.

3.	Configuration for Testing

a.	Create a test desktop on a "MOC" PTP with the following modules:

1)	Network Sockets Module for PTP3

a)	Socket Type	= TCP Server

b)	PTP Address	= 0.0.0.0

c)	PTP Port	= 20013

d)	Remote Address	= (not applicable)

e)	Remote Port	= (not applicable)

f)	TCP Rx Length	= 1264

g)	Minimum Delay	= No (unchecked)

h)	Keep Alive	= No (unchecked)

i)	TCP Safe Mode	= No (unchecked)

*	Data output port 1 to Bit Error Rate Tester module for PTP3.

2)	Bit Error Rate Tester Module for PTP3

3)	Network Sockets Module for PTP4

a)	Socket Type	= TCP Server

b)	PTP Address	= 0.0.0.0

c)	PTP Port	= 20014

d)	Remote Address	= (not applicable)

e)	Remote Port	= (not applicable)

f)	TCP Rx Length	= 1264

g)	Minimum Delay	= No (unchecked)

h)	Keep Alive	= No (unchecked)

i)	TCP Safe Mode	= No (unchecked)

*	Data output port 1 to Bit Error Rate Tester module for PTP4.

4)	Bit Error Rate Tester Module for PTP4

b.	Create test desktops on PTP3 and PTP4 with the following modules:

1)	Avtec Serial Input Module

a)	Board number	= (for board to be tested)

b)	Polarity	= True

c)	APC	= (not applicable)

d)	Source	= Ext 422

e)	Code	= NRZ-L

f)	Orientation	= MSB

g)	Clock Phase	= True

h)	Bits/Wd	= 8

i)	PipeLine	= No (unchecked)

j)	Frame Len	= 1264

k)	FramesPerBuffer	= 1

l)	User GRT Offset msec	= 0

m)	Synchronizer	= Disable

n)	Sync Pattern	= (not applicable)

o)	Mask Pattern	= (not applicable)

p)	Sync Location	= (not applicable)

q)	Slip Size	= (not applicable)

r)	Search Errors	= (not applicable)

s)	Lock Errors	= (not applicable)

t)	Check Frames	= (not applicable)

u)	Fly Frames	= (not applicable)

v)	Use Time Board	= Yes (checked)

w)	Tag First Bit	= Yes (checked)

x)	Extended Options	= (default values)

*	Data output port 1 to Network Sockets module.

2) Network Sockets Module

a)	Socket Type	= TCP Client

b)	PTP Address	= 0.0.0.0

c)	PTP Port	= 20013 for PTP3, 20014 for PTP4

d)	Remote Address	= (IP address for "MOC" PTP)

e)	Remote Port	= 20013 for PTP3, 20014 for PTP4

f)	TCP Rx Length	= 1264

g)	Minimum Delay	= No (unchecked)

h)	Keep Alive	= No (unchecked)

i)	TCP Safe Mode	= No (unchecked)

c.	Configure a clock and data synthesizer to generate a 2047 pseudo-random bit pattern at 10112 bits (1264 bytes) per frame for the data rate to be tested.  "Block" and "frame" are synonymous.

d.	Connect clock and data cables from the output of the clock and data synthesizer to the return patch panel for each Monarch board to be tested as follows (where board#1 corresponds to BRIDGE-1, board#2 to BRIDGE-2, and board#3 to BRIDGE-3):

Clock and Data Synthesizer to BRIDGE-1 INPUT Load

Clock and Data Synthesizer to BRIDGE-2 INPUT Load

Clock and Data Synthesizer to BRIDGE-3 INPUT Load

e.	On the return patch panel, bottle plug the data/clock receptacles from:

BRIDGE-1 OUTPUT 1 Source to PTP-3 RTN SVC 1 Load

BRIDGE-1 OUTPUT 2 Source to PTP-4 RTN SVC 1 Load

BRIDGE-2 OUTPUT 1 Source to PTP-3 RTN SVC 2 Load

BRIDGE-2 OUTPUT 2 Source to PTP-4 RTN SVC 2 Load

BRIDGE-3 OUTPUT 1 Source to PTP-3 RTN SVC 3 Load

BRIDGE-3 OUTPUT 2 Source to PTP-4 RTN SVC 3 Load

4.	Test Procedure

a.	On the "MOC" PTP, start a Console.exe on any Avtec server, load the test desktop, and enable all streams for the desktop.

b.	On PTP3, start Console.exe on the Avtec server corresponding to the Monarch board being tested (port 5000 for board #1, port 5001 for board #2, or port 5002 for board #3), load the test desktop, and enable all streams for the desktop.

c.	On PTP4, start Console.exe on the Avtec server corresponding to the Monarch board being tested (port 5000 for board #1, port 5001 for board #2, or port 5002 for board #3), load the test desktop, and enable all streams for the desktop.

d.	Test the connections:

1)	From the clock and data synthesizer, insert some bit errors.

2)	From PTP3 and PTP4, verify:

a)	The Network Sockets module status shows a connection has been made to the "MOC" PTP.

b)	The Avtec Serial Input module status shows a lock state, clock present, frames incrementing, and an Rx BitRate of approximately the data rate configured at the synthesizer.

3)	From the "MOC" PTP, verify:

a)	The Network Sockets module for PTP3 has a status that shows a connection has been made from PTP3.

b)	The Network Sockets module for PTP4 has a status that shows a connection has been made from PTP4.

c)	The Bit Error Rate Tester module for PTP3 has a status that shows frames are being received with frame bit errors.

d)	The Bit Error Rate Tester module for PTP4 has a status that shows frames are being received with frame bit errors.

e.	Test the return data rates:

1)	From the clock and data synthesizer, configure the data rate to be tested.

2)	From PTP3 and PTP4, verify that the Avtec Serial Input module status shows a lock state, clock present, frames incrementing, and an Rx BitRate of approximately the data rate configured at the synthesizer.

3)	From the "MOC" PTP:

a)	Zero out the counts for the Bit Error Rate Tester module for PTP3 and verify that its status shows frames are being received with no frame bit errors and no frames are being dropped.

b)	Zero out the counts for the Bit Error Rate Tester module for PTP4 and verify that its status shows frames are being received with no frame bit errors and no frames are being dropped.

4)	Run the test until at least 5,000,000 bits have been tested and record the number of frames received with frame bit errors and the number of frames dropped at the "MOC" PTP from PTP3 and PTP4.

f.	Test the other two Monarch boards by repeating previous steps d and e.

5.	Test Results

The requirements were successfully tested from 8/3/03 through 9/9/03 in GSFC building 13, room 262, test berth 3, for the three Monarch boards in each of the five WDISC expansion PTP's for each return data rate.  PTP5 was tested after it replaced STGT PTP3 in the rack on 8/13/03.

Data	Frames	Bit

Rate	Frames	Bits	w/Bit	Frames	Error

PTP	Board	(bps)	Checked	Checked	Errors	Dropped	Rate

STGT PTP3	1	100	1236	1.2 E07	0	0	0. E-07

1000	5216	5.3 E07	0	0	0. E-07

2000	11105	1.1 E08	0	0	0. E-08

4000	14407	1.5 E08	0	0	0. E-08

4864	1914	1.9 E07	0	0	0. E-07

5180	2964	3.0 E07	0	0	0. E-07

6000	1632	1.7 E07	0	0	0. E-07

8000	1007	1.0 E07	0	0	0. E-07

16000	1517	1.5 E07	0	0	0. E-07

32000	2878	2.9 E07	0	0	0. E-07

512000	41906	4.2 E08	0	0	0. E-08

2	100	1136	1.1 E07	0	0	0. E-07

1000	5218	5.3 E07	0	0	0. E-07

2000	1316	1.3 E07	0	0	0. E-07

4000	1067	1.1 E07	0	0	0. E-07

4864	972	9.8 E06	0	0	0. E-06

5180	1763	1.8 E07	0	0	0. E-07

6000	1236	1.2 E07	0	0	0. E-07

8000	784	7.9 E06	0	0	0. E-06

16000	978	9.9 E06	0	0	0. E-06

32000	2045	2.1 E07	0	0	0. E-07

512000	32472	3.3 E08	0	0	0. E-08

3	100	1974	2.0 E07	0	0	0. E-07

1000	591	6.0 E06	0	0	0. E-06

2000	1202	1.2 E07	0	0	0. E-07

4000	530	5.4 E06	0	0	0. E-06

4864	2240	2.3 E07	0	0	0. E-07

5180	28617	2.9 E08	0	0	0. E-08

6000	1977	2.0 E07	0	0	0. E-07

8000	3977	4.0 E07	0	0	0. E-07

16000	1079	1.1 E07	0	0	0. E-07

32000	1713	1.7 E07	0	0	0. E-07

512000	5002	5.1 E07	0	0	0. E-07

Data	Frames	Bit

Rate	Frames	Bits	w/Bit	Frames	Error

PTP	Board	(bps)	Checked	Checked	Errors	Dropped	Rate

STGT PTP4	1	100	1233	1.2 E07	0	0	0. E-07

1000	5216	5.3 E07	0	0	0. E-07

2000	11105	1.1 E08	0	0	0. E-08

4000	14407	1.5 E08	0	0	0. E-08

4864	1751	1.8 E07	0	0	0. E-07

5180	2964	3.0 E07	0	0	0. E-07

6000	1632	1.7 E07	0	0	0. E-07

8000	1007	1.0 E07	0	0	0. E-07

16000	1517	1.5 E07	0	0	0. E-07

32000	2878	2.9 E07	0	0	0. E-07

512000	41906	4.2 E08	0	0	0. E-08

2	100	1137	1.1 E07	0	0	0. E-07

1000	5218	5.3 E07	0	0	0. E-07

2000	1316	1.3 E07	0	0	0. E-07

4000	1067	1.1 E07	0	0	0. E-07

4864	972	9.8 E06	0	0	0. E-06

5180	1763	1.8 E07	0	0	0. E-07

6000	1240	1.3 E07	0	0	0. E-07

8000	787	8.0 E06	0	0	0. E-06

16000	990	1.0 E07	0	0	0. E-07

32000	2073	2.1 E07	0	0	0. E-07

512000	32472	3.3 E08	0	0	0. E-08

3	100	1974	2.0 E07	0	0	0. E-07

1000	591	6.0 E06	0	0	0. E-06

2000	1202	1.2 E07	0	0	0. E-07

4000	529	5.3 E06	0	0	0. E-06

4864	2240	2.3 E07	0	0	0. E-07

5180	28617	2.9 E08	0	0	0. E-08

6000	1977	2.0 E07	0	0	0. E-07

8000	3977	4.0 E07	0	0	0. E-07

16000	1079	1.1 E07	0	0	0. E-07

32000	1713	1.7 E07	0	0	0. E-07

512000	5002	5.1 E07	0	0	0. E-07

Data	Frames	Bit

Rate	Frames	Bits	w/Bit	Frames	Error

PTP	Board	(bps)	Checked	Checked	Errors	Dropped	Rate

WSGT PTP3	1	100	1093	1.1 E07	0	0	0. E-07

1000	1033	1.1 E07	0	0	0. E-07

2000	1421	1.4 E07	0	0	0. E-07

4000	1220	1.2 E07	0	0	0. E-07

4864	866	8.8 E06	0	0	0. E-06

5180	670	6.8 E06	0	0	0. E-06

6000	1411	1.4 E07	0	0	0. E-07

8000	2409	2.4 E07	0	0	0. E-07

16000	1865	1.9 E07	0	0	0. E-07

32000	1002	1.0 E07	0	0	0. E-07

512000	10205	1.0 E08	0	0	0. E-08

2	100	3173	3.2 E07	0	0	0. E-07

1000	12646	1.3 E08	0	0	0. E-08

2000	1117	1.1 E07	0	0	0. E-07

4000	1161	1.2 E07	0	0	0. E-07

4864	1249	1.3 E07	0	0	0. E-07

5180	1452	1.5 E07	0	0	0. E-07

6000	1014	1.0 E07	0	0	0. E-07

8000	1002	1.0 E07	0	0	0. E-07

16000	5366	5.4 E07	0	0	0. E-07

32000	1281	1.3 E07	0	0	0. E-07

512000	3974	4.0 E07	0	0	0. E-07

3	100	1831	1.9 E07	0	0	0. E-07

1000	3345	3.4 E07	0	0	0. E-07

2000	1381	1.4 E07	0	0	0. E-07

4000	1060	1.1 E07	0	0	0. E-07

4864	1013	1.0 E07	0	0	0. E-07

5180	8465	8.6 E07	0	0	0. E-07

6000	4514	4.6 E07	0	0	0. E-07

8000	2185	2.2 E07	0	0	0. E-07

16000	1170	1.2 E07	0	0	0. E-07

32000	1226	1.2 E07	0	0	0. E-07

512000	3525	3.6 E07	0	0	0. E-07

Data	Frames	Bit

Rate	Frames	Bits	w/Bit	Frames	Error

PTP	Board	(bps)	Checked	Checked	Errors	Dropped	Rate

WSGT PTP4	1	100	1093	1.1 E07	0	0	0. E-07

1000	1022	1.0 E07	0	0	0. E-07

2000	1421	1.4 E07	0	0	0. E-07

4000	1220	1.2 E07	0	0	0. E-07

4864	866	8.8 E06	0	0	0. E-06

5180	670	6.8 E06	0	0	0. E-06

6000	1402	1.4 E07	0	0	0. E-07

8000	2409	2.4 E07	0	0	0. E-07

16000	1866	1.9 E07	0	0	0. E-07

32000	1001	1.0 E07	0	0	0. E-07

512000	10207	1.0 E08	0	0	0. E-08

2	100	3173	3.2 E07	0	0	0. E-07

1000	12647	1.3 E08	0	0	0. E-08

2000	1117	1.1 E07	0	0	0. E-07

4000	1163	1.2 E07	0	0	0. E-07

4864	1251	1.3 E07	0	0	0. E-07

5180	1452	1.5 E07	0	0	0. E-07

6000	1014	1.0 E07	0	0	0. E-07

8000	1002	1.0 E07	0	0	0. E-07

16000	5366	5.4 E07	0	0	0. E-07

32000	1281	1.3 E07	0	0	0. E-07

512000	3974	4.0 E07	0	0	0. E-07

3	100	1831	1.9 E07	0	0	0. E-07

1000	3345	3.4 E07	0	0	0. E-07

2000	1381	1.4 E07	0	0	0. E-07

4000	1060	1.1 E07	0	0	0. E-07

4864	1014	1.0 E07	0	0	0. E-07

5180	8465	8.6 E07	0	0	0. E-07

6000	4515	4.6 E07	0	0	0. E-07

8000	2189	2.2 E07	0	0	0. E-07

16000	1174	1.2 E07	0	0	0. E-07

32000	1245	1.3 E07	0	0	0. E-07

512000	3526	3.6 E07	0	0	0. E-07

Data	Frames	Bit

Rate	Frames	Bits	w/Bit	Frames	Error

PTP	Board	(bps)	Checked	Checked	Errors	Dropped	Rate

PTP5	1	100	781	7.9 E06	0	0	0. E-06

1000	640	6.5 E06	0	0	0. E-06

2000	554	5.6 E06	0	0	0. E-06

4000	543	5.5 E06	0	0	0. E-06

4864	535	5.4 E06	0	0	0. E-06

5180	516	5.2 E06	0	0	0. E-06

6000	538	5.4 E06	0	0	0. E-06

8000	11774	1.2 E08	0	0	0. E-08

16000	512	5.2 E06	0	0	0. E-06

32000	1004	1.0 E07	0	0	0. E-07

512000	4880	4.9 E07	0	0	0. E-07

2	100	583	5.9 E06	0	0	0. E-06

1000	614	6.2 E06	0	0	0. E-06

2000	1067	1.1 E07	0	0	0. E-07

4000	1025	1.1 E07	0	0	0. E-07

4864	521	5.3 E06	0	0	0. E-06

5180	1596	1.6 E07	0	0	0. E-07

6000	734	7.4 E06	0	0	0. E-06

8000	573	5.8 E06	0	0	0. E-06

16000	609	6.2 E06	0	0	0. E-06

32000	1027	1.0 E07	0	0	0. E-07

512000	2799	2.8 E07	0	0	0. E-07

3	100	714	7.2 E06	0	0	0. E-06

1000	5152	5.2 E07	0	0	0. E-07

2000	602	6.1 E06	0	0	0. E-06

4000	1257	1.3 E07	0	0	0. E-07

4864	841	8.5 E06	0	0	0. E-06

5180	2216	2.2 E07	0	0	0. E-07

6000	2786	2.8 E07	0	0	0. E-07

8000	2004	2.0 E07	0	0	0. E-07

16000	602	6.1 E06	0	0	0. E-06

32000	1002	1.0 E07	0	0	0. E-07

512000	1430	1.4 E07	0	0	0. E-07