### Bacterial genomic DNA isolation using CTAB

Version Number:		2

Start Production Date:		8-25-04

Stop Production Date:

Author:			William S. and Helene Feil

Reviewed by:

Signature:

# Summary

This scaled up CTAB method can be used to extract large quantities of large molecular weight DNA from bacteria and other microbes.

# Materials & Reagents

Materials/Reagents/Equipment				Vendor		Stock number

##### Disposables

1.5-mL microcentrifuge tube					Eppendorf	22 36 320-4

50-mL Nalgene Oak Ridge polypropylene centrifuge tube			VWR		21010-568

10-mL pipette							Falcon		357551

1-mL pipette tips							MBP		3781

##### Reagents

CTAB								Sigma		H-6269

NaCl								Sigma		S-3014

TE buffer (10mM tris; 1 mM EDTA, pH 8.0).   				Ambion		9858

Lysozyme							Sigma		L-6876

Proteinase K							Qiagen		19131

5M NaCl							Ambion		9759

10% SDS							Sigma		L-4522

Chloroform							Sigma		C-2432

IsoAmyl alcohol							Sigma		I-9392

Phenol								Sigma		P-4557

Isopropanol							VWR		PX-1835-14

Ethanol							AAPER		---------

RNAse A (100mg/mL)						Qiagen		19101

##### Equipment

Hot Plate

250 mL glass beaker

Magnetic stir rod

Thermometer

Automatic pipette dispenser

Sorval 500 Plus centrifuge (DuPont, Newtown, CT)

65°C water bath

37°C incubator

# Procedure

Cell preparation and extraction techniques.

(Modification of CTAB method, Current Protocols in Molecular Biology)

1.5ml	30ml	60ml

- Grow cells in broth and pellet at 10,000 rpm for 5 min or scrape from plate.

- Transfer bacterial suspension to the appropriate centrifuge tube.

- Spin down cells in microfuge or centrifuge at 10,000 rpm for 5 minute.

- Discard the supernatant.

- Resuspend cells in TE.

- Adjust to OD600  1.0 with TE buffer (10mM tris; 1 mM EDTA, pH 8.0)

- Transfer given amount of cell suspension to a clean centrifuge tube. -------   	740µl	14.8ml	29.6ml

- Add lysozyme (conc. 100mg/ml).  Mix well.     -------------------------------------	20µl	400µl	800µl

This step is necessary for hard to lyse gram (+) and some gram (–) bacteria.

- Incubate for 5 min. at room temperature.

- Add 10% SDS. Mix well.          ---------------------------------------------------------	40µl	800µl	1.6ml

- Add Proteinase K (10mg/ml). Mix well.        ----------------------------------------	8µl	160µl	320µl

- Incubate for 1 hr at 37°C.

- Add 5 M NaCl. Mix well.        -----------------------------------------------------------	100µl	2ml	4ml

- Add CTAB/NaCl (heated to 65°C). Mix well.         --------------------------------		100µl	2ml	4ml

- Incubate at 65°C for 10 min.

- Add chloroform:isoamyl alcohol (24:1). Mix well.       ----------------------------	0.5ml	10ml	20ml

- Spin at max speed for 10 min at room temperature.

- Transfer aqueous phase to clean eppendorf (should not be viscous).

- Add phenol:chloroform:isoamyl alcohol (25:24:1). Mix well.      ---------------	0.5ml	10ml	20ml

- Spin at max speed for 10 min at room temperature.

- Transfer aqueous phase and add 0.6 vol isopropanol (-20°C).

(e.g. if 400 l of aqueous phase is transferred, add 240 l of isopropanol.             ---- Add 0.6 of vol. ----

- Incubate at room temp for 30 min.

- Spin at max speed for 15 min.

- Wash pellet with 70% ethanol, spin at max speed for 5 min.

- Discard the supernatant and and let pellet dry for 5 – 10 min at room temp.

- Resuspend in TE plus RNAse (99 l TE + 1 l RNAse (10 mg/ml)). --------	20µl	400µl	800µl

- Transfer to sterile microcentrifuge tubes.

- Incubate at 37°C for 20 min.

- Run 1 l in a 1% agarose gel with concentration standards.

Notes and precautions.

In step 1, do not use too many bacterial cells (an OD600 of not more than 1.2 is recommended), or DNA does not separate well from the protein.

Most of the time, inverting several times is sufficient to mix well.  Shaking too hard will shear the DNA.

Use any protocol for DNA precipitation, the one in this protocol works well.

# Reagent/Stock Preparation

CTAB/NaCl (hexadecyltrimethyl ammonium bromide)

Dissolve 4.1 g NaCl in 80 ml of water and slowly add 10 g CTAB while heating (65°C) and stirring.  This takes more than 3 hrs to dissolve CTAB.  Adjust final volume to 100 ml and sterilize by filter or autoclave.