EPA420-R-98-020

Commercial Marine Emissions Inventory for EPA Category 2 and 3 Compression Ignition Marine Engines in United States Continental and Inland Waterways

21 August 1998

Prepared for:

U.S. Environmental Protection Agency

Office of Mobile Sources

2000 Traverwood

Ann Arbor, MI 48105

Prepared by:

James J. Corbett, Jr., P.E.

Paul S. Fischbeck, Ph.D.

Department of Engineering and Public Policy

Carnegie Mellon University

Pittsburgh, PA 15213

Order No. 8A-0516-NATX

Table of Contents

EXECUTIVE SUMMARY	E2

1.	INTRODUCTION	1

2.	ESTIMATION AND METHODOLOGY FOR SHIP EMISSIONS	1

2.1	Method A:  U.S. Emissions Estimated Using Ship Registry Data	5

2.2	Method B:  Foreign and Domestic Cargo Transport Fleet Inventory	9

2.3	Comparing the U.S. Emissions Inventory with a Global Inventory	16

3.	ENGINE TYPES IN EPA CATEGORIES	18

4.	ENGINE REPLACEMENT RATES	19

5.	SUMMARY	21

6.	REFERENCES	21

7.	APPENDICES	A1

7.1	Method A: Ship Operations Method for 1996 (Pages A2-A8)	A1

7.2	Method B: Cargo Method using 1993 Cargo Trade Data (Pages A9-A18)	A1

7.3	Engine Manufactures and Engine Models in U.S. Flag Fleet (Pages A19-A31)	A1

# EXECUTIVE SUMMARY

The U.S. Environmental Protection Agency (EPA) is currently developing the first national emission regulations applicable to compression ignition (CI) marine engines above 37 kilowatts (kW).  As part of the proposed rulemaking, EPA must develop an estimate of the current (i.e., baseline) CI marine engine emissions inventory.  EPA has defined three categories for CI marine engines, according to cylinder displacement.  These categories are shown in Table E1.

Table E1.  EPA Categories for CI Marine Engines

| EPA Category | Propulsion Engine Displacement per Cylinder (liters) |
| --- | --- |
| 1 | Less than 5 |
| 2 | Equal to 5, less than 20 |
| 3 | Equal to or greater than 20 |

Calculations for Category 2 and 3 engines are presented in this report.  This report provides estimates of oxides of nitrogen (NOx), particulate matter (PM), hydrocarbons (HC), and carbon monoxide (CO) from all ships (domestic and foreign) operating in U.S. navigable waterways (i.e., within approximately 200 miles of U.S. coastlines, inland waterways, and the Great Lakes). Emissions are estimated using ship characteristics from 1996 fleet registry data [LMIS, 1996], which include vessels 100 gross registered tons (GRT) and greater.  Two analytical methods were used to estimate the emissions:  Method A is based on general ship operations, and Method B is based on cargo transportation profiles.  Method A estimates emissions for the U.S. flag fleet (excluding those operating outside U.S. waters) directly from main engine data, and these emissions are characterized by engine category.  Method B estimates emissions from foreign vessels operating in U.S. waters from information about foreign cargo transported on foreign vessels into U.S. ports; they were then characterized by engine category.  Method B also estimates regional emissions for ships involved in trade (not including fishing vessels, passenger vessels, tugs, and utility ships) using 1996 fleet registry data, and 1993 data describing waterway transport and trade [USACE, 1995].

Total nationwide emissions equal the sum of emissions from U.S. flag vessels operating in U.S. waters, and emissions from foreign flag vessels carrying cargo on U.S. navigable waterways.  Total nationwide emissions from main propulsion CI marine engines on commercial ships (100 GRT and greater) operating in U.S. waters are shown in Table E2.

Table E2.  Total Nationwide Ship Emissions From Main Engines in U.S. Watersa

| 103 Metric tons | Engine Category 2 | Engine Category 3 | Total |
| --- | --- | --- | --- |
| U.S. Flag NOx PM HC CO | 220.5 4.6 9.3 28.6 | 118.8 9.6 3.4 10.6 | 339.3 14.2 12.7 39.2 |
| Foreign Flag NOx PM HC CO | 9.9 0.7 0.3 0.9 | 116.9 8.7 3.6 11.0 | 126.9 9.4 3.9 11.9 |
| Grand Total NOx PM HC CO | 230.4 5.4 9.6 29.6 | 235.7 18.3 7.0 21.6 | 466.1 23.7 16.6 51.2 |

a. There may be some differences in totals due to rounding.

This report considers emissions only from main propulsion engines, and does not consider auxiliary engine emissions.  For CI marine engines used in main propulsion, available engine-specific data is provided, including engine manufacturer and model number.

# INTRODUCTION

The U.S. Environmental Protection Agency (EPA) is currently developing the first national emission regulations applicable to compression ignition (CI) marine engines above 37 kilowatts (kW).  As part of the proposed rulemaking, EPA must develop an estimate of the current (i.e., baseline) CI marine engine emissions inventory. EPA is considering dividing CI marine engines into three categories, each with unique emission regulations.  Under the most likely approach to be taken by EPA, these categories would be defined by engine cylinder displacement, as shown in Table 1.

Table 1.  EPA Categories for CI Marine Engines

| EPA Category | Propulsion Engine Displacement per Cylinder (liters) |
| --- | --- |
| 1 | Less than 5 |
| 2 | Equal to 5, less than 20 |
| 3 | Equal to or greater than 20 |

Calculations for Category 2 and 3 engines are presented in this report.  This report provides estimates of oxides of nitrogen (NOx), particulate matter (PM), hydrocarbons (HC), and carbon monoxide (CO) from all ships (domestic and foreign) operating in U.S. navigable waterways (i.e., within approximately 200 miles of U.S. coastlines, inland waterways, and the Great Lakes).   Emissions are estimated for the U.S. flag fleet directly from main engine data, and these emissions are characterized by engine category.  Emissions from foreign vessels operating in U.S. waters were estimated from information about foreign cargo transported on foreign vessels into U.S. ports; they were then characterized by engine category.

# ESTIMATION AND METHODOLOGY FOR SHIP EMISSIONS

No single approach with currently available data can produce a comprehensive inventory of ship emissions in U.S. waters.  Therefore, this analysis developed and applied two different methods to estimate baseline emissions from commercial CI marine engines from 1996 ship inventory data [LMIS, 1996].  Method A produces an inventory of emissions from commercial U.S. flag vessels with CI marine engines by considering engine types, duty cycles, and other operational factors.  Method B estimates emissions from foreign and domestic ships transporting cargo in U.S. waters by estimating the emissions per ton-mile of cargo moved, and deriving the pollution emitted from cargo transport in U.S. waters.  Since foreign shipping carries significant volumes of cargo to and from U.S. ports, this method provides an estimate of emissions from foreign shipping in U.S. waterways; additionally, regional characteristics of ship emissions are identified by this method.

By combining the inventory of emissions from U.S. flag vessels (Method A) with the estimated emissions from foreign cargo ships in Method B, an estimate of the total nationwide emissions from commercial ships in U.S. waters results.  Method A captures emissions from U.S. flag vessels, and Method B captures emissions from foreign cargo vessels.  This analysis integrates these distinct estimation methods to develop total nationwide emissions from CI marine engines used in main propulsion systems for commercial shipping.  Table 2 presents the summary estimate for total nationwide emissions for EPA Categories 2 and 3.

Table 2.  Total Nationwide Emissions from Ship Main Engines in U.S. Watersa

| 103 Metric tons | Engine Category 2 | Engine Category 3 | Total Categories 2 and 3 Engines |
| --- | --- | --- | --- |
| U.S. Flag NOx PM HC CO | 220.5 4.6 9.3 28.6 | 118.8 9.6 3.4 10.6 | 339.3 14.2 12.7 39.2 |
| Foreign Flag NOx PM HC CO | 9.9 0.7 0.3 0.9 | 116.9 8.7 3.6 11.0 | 126.9 9.4 3.9 11.9 |
| Grand Total NOx PM HC CO | 230.4 5.4 9.6 29.6 | 235.7 18.3 7.0 21.6 | 466.1 23.7 16.6 51.2 |

a. There may be some differences in totals due to rounding.

Table 3 shows regional detail, derived from Method B, for each pollutant to be addressed by EPA regulations.  Total nationwide Category 2 and 3 emissions reported in Table 2 are simply the sum of the emissions from U.S. flag CI engines in Method A (e.g., 339.3 thousand metric tons NOx) and total emissions from foreign CI engines estimated by Method B (e.g., 126.9 thousand metric tons for NOx).  A notable regional insight is that emissions from Category 2 and 3 CI marine engines on the inland rivers (e.g., 106.5 thousand metric tons NOx) equal nearly 70% of the combined emissions from oceangoing (foreign) and coastwise (domestic) shipping (e.g., 156.3 thousand metric tons NOx).  The primary reason for this is that the ton-miles of cargo moved over inland rivers equal approximately 65% of the ton-miles of cargo transported in U.S. coastal waters.  Moreover, since the navigable miles on the inland rivers equal only about one third of the navigable ocean miles, emissions from these cargo movements occur in a much smaller area.

Table 3.  Regional Annual Emissions from CI Marine Engines from Cargo Transport Using Method B (EPA Categories 2 and 3)

| U.S. Region | NOx Emissions (103 metric tons NOx/year) | PM Emissions (103 metric tons PM/year) | HC Emissions (103 metric tons HC/year) | CO Emissions (103 metric tons CO/year) |
| --- | --- | --- | --- | --- |
| Oceangoing (foreign cargo) US Flag CI Engines Category 2 CI Engines Category 3 CI Engines Foreign CI Engines Category 2 CI Engines Category 3 CI Engines Total | 8.0 4.9 3.1 96.3 7.5 88.8 104.3 | 0.4 0.1 0.3 7.8 0.6 7.2 8.2 | 0.2 0.15 0.07 2.8 0.2 2.6 3.0 | 0.7 0.5 0.2 8.6 0.7 7.9 9.3 |
| Coastwise (domestic cargo) Category 2 CI Engines Category 3 CI Engines | 52.0 31.7 20.3 | 2.6 0.8 1.8 | 1.4 1.0 0.4 | 4.6 3.2 1.4 |
| Inland Rivers US Flag CI Engines Category 2 CI Engines Category 3 CI Engines Foreign CI Engines Category 2 CI Engines Category 3 CI Engines Total | 76.8 46.8 30.0 29.7 2.3 27.4 106.5 | 2.4 0.7 1.7 1.5 0.1 1.4 3.9 | 2.6 1.8 0.8 1.1 0.1 1.0 3.6 | 8.4 5.8 2.6 3.3 0.3 3.0 11.6 |
| Great Lakes US Flag Engines Category 2 CI Engines Category 3 CI Engines Foreign CI Engines Category 2 CI Engines Category 3 CI Engines Total | 9.4 5.7 3.7 0.8 0.06 0.77 10.2 | 0.5 0.1 0.4 0.1 0.005 0.060 0.5 | 0.3 0.2 0.1 0.02 0.002 0.022 0.3 | 0.8 0.6 0.2 0.1 0.005 0.060 0.9 |
| Total U.S. Emissionsa,b US Flag Engines Category 2 CI Engines Category 3 CI Engines Foreign CI Engines Category 2 CI Engines Category 3 CI Engines Grand Total for Cargo Category 2 CI Engines Category 3 CI Engines | 146.1 89.0 57.1 126.9 9.9 117.0 273.0 99.0 174.0 | 5.8 1.7 4.1 9.4 0.7 8.7 15.2 2.4 12.8 | 4.4 3.1 1.3 3.9 0.3 3.6 8.3 3.4 4.9 | 14,5 10.0 4.4 11.9 0.9 11.0 26.4 11.0 15.4 |

- There may be some differences in totals due to rounding of numbers.

- Emissions estimated to a distance of 200 miles from shore; this is approximately the distance from shore as most of the major shipping lanes.

These methods rely on a common database of registered ships containing such information as propulsion type, brake horsepower, and engine data [LMIS, 1996].  In addition, both methods used marine diesel emission factors reported by Lloyd's Register Engineering Services in the Marine Exhaust Emissions Research Programme [Carlton et al., 1995].  These provide the most current emissions factors for large CI marine engines in commercial use today.  These emission factors are similar to currently available EPA emission factors in AP-42 [EPA, 1997] for similar CI engines, as shown in Table 4.  (The units of the AP-42 factors (lb/MMBtu ) were converted to match the fuel-based units (kg/tonne fuel) reported by Lloyds.)

Aside from these two common databases, the estimation methods are quite different.  The methodologies are explained in detail in the next sections.

Table 4.  Comparison of CI Marine Engine Emission Factors with AP-42

| Pollutant | Emission Factor (fuel-based) (kg pollutant/tonne fuel) |
| --- | --- |
| Oxides of Nitrogen (NOx) as NO2 Lloyds: Slow Speed Lloyds: Medium Speed AP-42 Emission Factors: Large Stationary Diesel Uncontrolled Diesel Industrial Engine Non-Road CI Engine (steady state) | 87 57 53 73 50 |
| Particulate Matter (PM) Lloyds: Slow Speed Lloyds: Medium Speed AP-42 Emission Factors: Large Stationary Diesel Uncontrolled Diesel Industrial Engine Non-Road CI Engine (steady state) | 7.6 1.2 2.0 5.0 1.0 |
| Hydrocarbons (HC) Lloyds: All CI Marine Engines AP-42 Emission Factors: Large Stationary Diesel Uncontrolled Diesel Industrial Engine Non-Road CI Engine (steady state) | 2.4 1.0 6.0 4.0 |
| Carbon Monoxide (CO) as CO Lloyds: All CI Marine Engines AP-42 Emission Factors: Large Stationary Diesel Uncontrolled Diesel Industrial Engine Non-Road CI Engine (steady state) | 7.4 14.0 16.0 16.0 |

## Method A:  U.S. Emissions Estimated Using Ship Registry Data

Method A (Ship Operations Method) is a theoretical approach using estimates of daily fuel consumption calculated from ship brake horsepower (BHP) and engine brake-specific fuel consumption (BSFC).  Fuel-based emission factors were applied to get emissions per day, making adjustments for usage using marine duty cycles from ISO [ISO, 1996], estimated time in service, and time underway per year.  This approach allows for a direct estimate of emissions for all types of vessels, including fishing vessels, tugs, and utility ships.

The numbers of ships in the U.S. commercial fleet with engines in each category were estimated.  This estimate was based on engine manufacturer and model data gathered for commercial ships greater than 100 gross registered tons (GRT) and listed in Lloyds Maritime Information System database of registered vessels [LMIS, 1996].  Engine data, particularly cylinder displacement, was available for 42% of the ships with CI engines in the data set.  A statistical test (chi-squared test) showed that the distribution of unassigned engines was not different from the distribution of categorized engines.  Based on this information, the remaining engines were distributed by EPA category proportionally, as shown in Table 5.

Table 5.  Estimated Number of U.S. Flag Ships by Type and EPA Category

|  | Category |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | 1 | 2 | 3 | Not Diesel | Grand Total |
| Container | - | 1 | 49 | 37 | 87 |
| Fishing | 2,498 | 241 | 40 | - | 2,778 |
| Passenger | 8 | - | 6 | 27 | 41 |
| RoRoa | 6 | 21 | 39 | 32 | 97 |
| Transport | 6 | 17 | 49 | 347 | 419 |
| Tug | 244 | 975 | 42 | 7 | 1,269 |
| Utility | 420 | 171 | 12 | 47 | 650 |
| Grand Total | 3,182 | 1,426 | 236 | 498 | 5,341 |

a.  RoRo: Roll-on, Roll-off vessel; this includes ships carrying vehicles and other large, non-containerized items.

This study does not include ships less than 100 GRT, because the Lloyds database does not contain smaller vessels and it was not possible to find comparable data for these vessels.  One source for smaller vessels may be the US Coast Guard vessel data collected by the Waterborne Commerce Statistics Center (WCSC) [USACE, 1995].  This data indicates 8,319 U.S. flag vessels (ships and boats) are available for service, compared to the 5,341 ships in the Lloyds registry; fishing vessels appear to be excluded from the WCSC data.  However, engine data is not provided for these ships.  Another alternate source for information on smaller vessels may be the Merchant Vessels of the United States CD-ROM [DOT, 1998].   This is a data file of merchant and recreational vessels documented under the laws of the United States by the U.S. Coast Guard.  The file contains the vessel name, official number, managing owner name, particulars such as tonnage and length, port of documentation (homeport), and authorized trade endorsements.  Once again, engine-specific data is not provided.  This analysis does not use this source in estimating emissions for this report, although some 63,809 vessels (boats and ships) are listed that may be used in commerce (171,663 recreational vessels are listed as well).  A significant number may not be powered by CI marine engines at all.  Of the remainder, most, if not all, of these vessels would likely fall into EPA Category 1, and would not be included in this study.

Brake horsepower for nearly all of these engines is reported in the Lloyds registry data.  The BHP for each engine was multiplied by a typical brake-specific-fuel-consumption (BSFC) factor for the horsepower range indicated.  Generalized fuel-consumption characteristics for direct-drive, geared, and diesel-electric CI marine engines illustrate the relationship between fuel-consumption and engine rating for these propulsion-plant configurations [Harrington, 1992].  These fuel-consumption characteristics were confirmed by comparison with other sources [Heywood, 1988; Osbourne, 1943; Turpin and McEwen, 1965]. Units were converted to tons-fuel per day (tpd).  The rated BHP values used in the fuel-consumption estimates were adjusted downward, in consideration of the fact that ship engines generally operate at maximum loads no higher than 80% of rated load.  Then marine duty-cycle load factors were applied, to make the fuel-consumption estimates more realistic.  The E3 duty cycle for heavy-duty marine engines was used [ISO, 1996; Markle and Brown, 1996].  This duty cycle was developed to represent typical overall engine loads for exhaust emission measurement; no better duty cycle representing actual operations is avaible.  A summary of the U.S. flag fleet fuel consumption in tpd is presented in Table 6.  These estimates are not unreasonable when compared with fuel consumption reported by other sources [Evans and Marlow, 1990; Ewart, 1982].

Category 3 engines were divided into two subcategories by cylinder displacement; engines with cylinder displacements greater than 20 liters but less than 60 liters were assigned to subcategory 3A.  Those greater than 60 liters were assigned to subcategory 3B.  The reason for this is that emission factors for NOx and PM are different for medium speed and slow speed engines [Carlton et al., 1995] (see Table 4).  All Category 2 engines are assumed to be medium-speed engines.  The larger engines in Category 3 are assumed to be slow-speed engines, but this is not true of the smaller engine sizes.  In the absence of better information characterizing these engines, this analysis assumed that 50% of the engines in subcategory 3A were medium-speed engines, and half were slow-speed engines.  Therefore, the fuel-consumption estimates shown in Table 6 estimate fuel usage for subcategories 3A and 3B separately.  The fuel-based emission factors for CI marine engines were then multiplied by the estimated fuel-consumption to obtain an estimate for the daily emissions for each pollutant in kg per day.

Table 6.  Estimated Fuel-Consumption for CI Engines in the U.S. Fleet (tpd)

|  | Category |  |  |
| --- | --- | --- | --- |
| Vessel Service | 2 | 3A | 3B |
| Container | 5 | - | 56 |
| Fishing | 5 | 13 | 10 |
| Passenger | - | 10 | 20 |
| RoRo | 6 | 48 | 40 |
| Transport | 6 | 31 | 29 |
| Tug | 5 | 24 | 31 |
| Utility | 4 | 20 | 12 |

The daily emissions per ship were multiplied by the number of ships, and by the number of days per year to estimate annual emissions.  However, this estimate represented the unrealistic scenario where vessels were underway 100% of the time during the year.  This study assumed a conservatively high underway factor of 80% to obtain a final estimate of annual emissions from U.S. flag ships, based on the authors’ knowledge of and experience with oceangoing ships.  An informal telephone survey of several commercial tug and barge operators (Crowley Maritime Corporation, Foss Maritime Company, American Commercial Barge Lines Company, National Marine Inc., and the Port of Pittsburgh) confirmed that this assumption was also reasonable for tugs and utility vessels.

In estimating total nationwide emissions by Method A, two errors are present.  This inventory only includes U.S. flag vessels, and does not address foreign vessels operating in and around U.S. waterways.  This also includes U.S. flag vessels operating anywhere in the world.  The problem of including foreign shipping is addressed through Method B, described in the next section.  Analysis of vessels arriving in U.S. ports carrying foreign cargo [USACE, 1995] showed that 410 U.S. cargo ships (68% of the 597 cargo ships in the U.S. fleet – nearly all of which have Category 2 and 3 engines) made approximately 13,500 visits from foreign ports (15% of the ship visits reported by U.S. Customs).  This represents an average of 33 visits per ship-year (about 1 every 10 days).  The problem of removing U.S. flag vessels that are not operating within U.S. waterways was corrected by assuming that these U.S. flag vessels spend approximately 30% of their operating time in U.S. waters (about one day inbound in U.S. waters, one day in harbor, and one day outbound in U.S. waters).  Therefore, estimated emissions were reduced by approximately 48% (68% - [68% * 30%] = 48%) to “back out” these emissions occurring outside of U.S. waters.  This represents a reasonable upper bound for the number of U.S. vessels operating outside of U.S. waters, especially for ships that transport both foreign and domestic cargoes.  Estimates for each pollutant, by type of vessel and by EPA Category are presented for U.S. flag vessels that operate in U.S. waters in Table 7.

## Method B:  Foreign and Domestic Cargo Transport Fleet Inventory

Method B (Cargo Method) used cargo movements and waterway data to calculate the total tons and ton-miles moved by ships annually in and around the U.S.  Then the numbers of “average” cargo ships needed to carry these volumes were estimated and emissions per ton-mile derived for these hypothetical cargo ships.  The annual emissions estimated by this approach are directly applicable to U.S. trade regions (for both foreign and domestic cargo) and national shipping lanes, which include the Great Lakes, inland waterways, and coastal shipping lanes extending to approximately 200 miles from the coasts.   Most navigable U.S. waterways are inventoried in the U.S. Army Corps of Engineers (USACE) Waterway Link Network.  Shipping lanes and open water passage lanes are represented by over 5,000 line segments, or “links.”  The 1993 National Waterway Network (NWN) geographic database, developed by the Oak Ridge National Laboratory, Vanderbilt University, and the National Waterway GIS Design Committee, includes physical and location information about each of these links [USACE, 1995].  Ocean shipping lanes (waterway links) in the NWN data were within 200 miles of U.S. coastlines.  The USACE Waterborne Commerce Statistics Center (WCSC) tracks commodity movements across these links by conducting on-going surveys of companies and government agencies [USACE, 1995].   For this report, 1993 NWN data on the USACE waterway links were merged with 1993 WCSC shipment data to create a comprehensive picture of cargo movement in and around the

Table 7.  Estimated Annual Pollution for CI Engines in the U.S. Fleet (metric tons)

| Vessel Service | Category |  | Categories |
| --- | --- | --- | --- |
| Pollutant | 2 | 3 | 2 and 3 Total |
| Container NOx (as NOx) PM HC CO (as CO) | 92 2 4 12 | 36,085 3,152 995 3,069 | 36,177 3,154 999 3,081 |
| Fishing NOx (as NOx) PM HC CO (as CO) | 37,193 783 1,566 4,828 | 9,937 823 284 876 | 47,129 1,606 1,850 5,704 |
| Passenger NOx (as NOx) PM HC CO (as CO) | - | 2,362 196 67 208 | 2,362 196 67 208 |
| RoRo NOx (as NOx) PM HC CO (as CO) | 2,255 47 95 293 | 20,912 1,621 622 1,918 | 23,167 1,668 717 2,211 |
| Transport NOx (as NOx) PM HC CO (as CO) | 1,823 38 77 237 | 18,019 1,442 526 1,622 | 19,842 1,480 603 1,859 |
| Tug NOx (as NOx) PM HC CO (as CO) | 154,898 3,261 6,522 20,110 | 26,921 2,052 808 2,492 | 181,820 5,313 7,330 22,602 |
| Utility NOx (as NOx) PM HC CO (as CO) | 24,247 510 1,021 3,148 | 4,521 319 141 436 | 28,768 829 1,162 3,583 |
| Total NOx (as NOx) PM HC CO (as CO) | 220,507 4,642 9,285 28,627 | 118,759 9,604 3,445 10,621 | 339,266 14,247 12,729 39,248 |

U.S.  The cargo movements in ton-miles for each region were then calculated by summing the product of the number of tons shipped along each link in a region by the length of that link, as shown below:

WhereCG=i=15,000LGi∗TGi

Equation 1

C 	=	Cargo movement in ton-miles

G	=	Geographic Region (Ocean, Inland, Great Lakes)

L 	=	Length (in miles) of link i

T 	=	Tonnage (total tons shipped in 1993) of link i

i 	=	Each link in the NWN and WCSC data for a given geographic region

A summary of these calculations is shown in Table 8.  Interestingly, the number of tons moved on the inland river systems in the U.S. exceeds the number moved on the other waterways, including the oceans bordering the U.S. (by a factor of 2).  Alternatively, the distances described by waterway links for the oceans exceed the distances on the inland rivers by nearly a factor of 3.  The fact that the ton-miles of cargo moved over inland rivers equal approximately 65% of the ton-miles of cargo transported in U.S. coastal waters implies that emissions on the inland waters may be significant.  Moreover, since the navigable miles on the inland rivers equal only about one third of the navigable ocean miles, emissions from these cargo movements occur in a much smaller area.

Table 8 .  Summary of Cargo Movements in 1993 By Major Waterway

| Region | Distance (miles)a | 106 Tons movedb | 109 Ton-miles |
| --- | --- | --- | --- |
| Ocean | 116,411 | 15,968.6 | 764.9 |
| Great Lakes | 8,382 | 7,662.2 | 85.3 |
| Inland Rivers | 43,566 | 33,055.5 | 494.4 |
| Unassigned | 120 | 91.9 | 2.3 |
| Total | 168,478 | 56,778.2 | 1,346.9 |

- From NWN data [USACE, 1995].

- From WCSC data [USACE, 1995].  Note that tonnage moved is not the same as tons received or sent by a given port; the same tons may move across several links, and therefore can be counted in the ton-miles more than once.

To estimate emissions per ton-mile from commercial marine vessels, the estimated daily emissions per ship from Method A were used, along with vessel average deadweight tonnage (DWT) and speed data from Lloyds registry.  Deadweight tonnage is a measurement of total contents of a ship including cargo, fuel, crew, passengers, food, and water aside from boiler water.  Because DWT describes more than the cargo carrying capacity of a ship, the DWT reported in Lloyds was multiplied by 80% to obtain an estimate of the maximum cargo tons that could be carried; this is consistent with typical voyage estimating factors [Packard, 1991].  Average DWT for cargo vessels (i.e., containers, roll-on-roll-off  ships (RoRo’s), and other transport ships (including dry bulk, tankers, etc.) was computed for all vessels in Lloyds registry (including U.S. flag) and separately for U.S. flag vessels.  The average DWT for cargo ships in the world fleet is 34,387 DWT; the average DWT for cargo ships in the U.S. flag fleet is 15,454 DWT.

However, ships do not typically operate fully loaded with cargo.  Many ships, particularly tankers, may carry cargo in one direction and return empty (or with ballast only).  Other ships carry cargo both directions, but rarely carry their full capacity.  Most ships carry cargo loads that average 50% to 65% capacity; when cargo capacities exceed 70%, it can be indication that too few ships are available for the route [Abrams, 1997].  This analysis applied a cargo capacity factor of 50% to vessels operating on ocean routes, and a 60% cargo capacity factor to inland rivers and Great Lakes vessels.  This is consistent with reported statistics [Wilde Mathews, 1998a].  The higher capacity factor for inland river cargo transport follows from the understanding that inland vessels are smaller, with shallower drafts and smaller total capacities per vessel.  This fact, combined with the large tonnages moved on inland rivers, implies that these vessels are loaded to higher capacities than oceangoing and coastwise transport.  Moreover, barges can be added or removed from a group of as many as 35 barges [Wilde Mathews, 1998b], and therefore barge towboats/pushboats may also transport higher average capacities.

Speed data reported in Lloyds represents the rated design speed of the vessel, similar to the BHP data.  Therefore, marine duty-cycle load factors were applied to speed, in the same way as for BHP in Section 2.1.  The E3 duty cycle was used for heavy-duty marine engines for all engines [ISO, 1996; Markle and Brown, 1996].  To be consistent with adjustments made to the maximum BHP load to be 80% of rated BHP, the maximum speed was adjusted by employing the relationship between horsepower and the cube of the speed [Laurence, 1984].  This resulted in a maximum speed that is 93% of the rated design speed of the vessel.  Average speeds for cargo ships in the U.S. flag fleet are shown in Table 9.  Average speeds for the world fleet are presented in Table 10.

From this information, the emissions per ton-mile and emissions per year can be calculated, according to the following equations:

ETM=Ed÷DWT∗CCF∗V∗24

Equation 2

Where

ETM 	=	Emissions per ton-mile

Ed 	=	Emissions per day (from Method A)

DWT 	=	Average DWT per ship

CCF 	=	Cargo capacity factor

(0.5 for oceans, 0.6 for inland rivers and Great Lakes)

V 	=	Average speed of vessel across duty cycle (adjusted for max BHP)

24 	=	Hours per day to convert ship speed to ship miles per day

EY=ETM∗TMY

Equation 3

Where

EY 	=	Emissions per year

TMY 	=	Ton-miles per year (from Table 8)

Table 9.  Average Speed (in knots) of U.S. Fleet Cargo Ships (adjusted for duty cycle)a

|  | Category |  |  |  |  | Weighted |
| --- | --- | --- | --- | --- | --- | --- |
| Vessel Service |  | 1 | 2 | 3A | 3B | Average |
| Container |  | - | 5 | - | 10 | 10 |
| RoRo |  | 6 | 8 | 10 | 10 | 9 |
| Transport |  | 5 | 7 | 8 | 8 | 7 |
| Weighted Average |  | 5 | 7 | 9 | 9 | 9 |

a. The average speed of tugs moving barges on inland waters equals 79% of the speed of U.S. cargo vessels; this is taken as the average speed on the inland rivers.

Table 10.  Average Speed (in knots) of World Fleet Cargo Ships (adjusted for duty cycle)

|  | Category |  |  |  |  | Weighted |
| --- | --- | --- | --- | --- | --- | --- |
| Vessel Service |  | 1 | 2 | 3A | 3B | Average |
| Container |  | - | 6 | 13 | 16 | 16 |
| RoRo |  | 6 | 8 | 14 | 14 | 12 |
| Transport |  | 5 | 6 | 12 | 12 | 11 |
| Weighted Average |  | 5 | 6 | 12 | 12 | 11 |

To distinguish between domestic and foreign trade, port data indicating the amount of domestic and foreign cargo was taken from the WCSC data on the US Waterways CD-ROM [USACE, 1995].  Here, the cargo tons delivered or shipped from the 179 ports reported in the data was used, rather than the tons moved along the waterway links.  In 1993, 49% of the cargo delivered to ports in U.S. ocean regions (Atlantic, Pacific, and Gulf Coasts) was domestic. In Great Lakes, 80% of the trade was domestic; 70% of trade shipped on the inland rivers was domestic.

To estimate the number of U.S. ships engaged in foreign trade, this analysis used U.S. Census Bureau data indicating the number of port visits for foreign commerce (i.e., that involved U.S. Customs) in 1993 [USACE, 1995].  Some 90,000 ship visits nationwide were reported.  Approximately 15% of these vessels making port visits were registered as U.S. flag ships; this factor was applied to separate foreign trade into foreign trade carried by U.S. ships and foreign trade carried by foreign ships.  (While 15% of the vessels making visits with foreign cargo were U.S. flag vessels, this amounted to 68% of the cargo ships in U.S. registered fleet as discussed in Section 2.1.)

This emission estimate assumes CI marine engines propel all vessels.  This assumption is generally valid for non-U.S.-flag cargo vessels (which are nearly all diesel-propelled), but it is not true for U.S. flag cargo ships.  While ships with steam-turbine engines only account for 6% of the U.S. flag fleet, they equal 52% of the U.S. flag cargo-carrying vessels.  Marine steam-turbine engines emit significantly less NOx, HC, and CO, and about the same PM as CI marine engines [Booz Allen & Hamilton, 1991].  This analysis corrected the raw calculations to account for U.S. steam ships.  This was done by taking 52% of the raw estimate and multiplying it by the ratio of steam-engine-emission factor to diesel-engine-emission factor for each pollutant.

Lastly, these emissions were characterized by engine category.  World fleet characteristics for cargo vessels were assumed to apply to foreign flag vessels transporting cargo in U.S. waters.  Emissions estimated for foreign ships by Method B were distributed according to the percent of foreign cargo vessels with engines in EPA Categories 1, 2 and 3.  Estimates for emissions from cargo transport are shown by region, domestic versus foreign, in Table 11.  Steam engines are shown separately from diesel engines, and diesel engines in Categories 2 and 3 are shown as a subset of diesel engines.

Table 11.  Emissions from Maritime Transport of Cargo in U.S. Waterways a

| U.S. Region | NOx (metric tons NOx/year) | PM (metric tons PM/year) | HC (metric tons HC/year) | CO (metric tons CO/year) |
| --- | --- | --- | --- | --- |
| Oceangoing (foreign cargo) US Ship Engines All US CI Engines US Cat. 2 and 3 US Steam/other Foreign Ship Engines Foreign Cat. 2 and 3 Oceangoing Total All Cat. 2 and 3 | 9,836 8,695 7,993 1,141 103,803 96,325 113,639 104,318 | 801 414 397 387 8,448 7,840 9,249 8,237 | 284 238 217 46 2,999 2,783 3,283 3,000 | 876 775 705 101 9,248 8,581 10,124 9,286 |
| Coastwise (domestic cargo) All US CI Engines US Cat. 2 and 3 US Steam/other Coastwise Total | 56,535 51,971 7,417 63,952 | 2,690 2,582 2,515 5,205 | 1,550 1,409 298 1,848 | 5,041 4,583 656 5,697 |
| Inland Rivers US Ship Engines All US CI Engines US Cat. 2 and 3 US Steam/other Foreign Ship Engines Foreign Cat. 2 and 3 Inland Rivers Total All Cat. 2 and 3 | 94,499 83,539 76,795 10,960 32,020 29,713 126,518 106,508 | 4,777 2,468 2,370 2,308 1,619 1,502 6,395 3,872 | 3,368 2,826 2,569 543 1,141 1,059 4,510 3,628 | 10,386 9,190 8,354 1,196 3,519 3,266 13,906 11,620 |
| Great Lakes US Ship Engines All US CI Engines US Cat. 2 and 3 US Steam/other Foreign Ship Engines Foreign Cat. 2 and 3 Great Lakes Total All Cat. 2 and 3 | 11,556 10,216 9,391 1,340 901 836 12,457 10,227 | 940 486 467 454 73 68 1,014 535 | 334 280 255 54 26 24 360 279 | 1,029 911 828 119 80 74 1,110 903 |
| Total U.S. Emissionsa,b US Ship Engines All US CI Engines US Cat. 2 and 3 US Steam/other Foreign Ship Engines Foreign Cat. 2 and 3a Grand Total (All Ships) All Cat. 2 and 3 | 179,842 158,984 146,151 20,858 136,723 126,874 316,566 273,025 | 11,723 6,058 5,816 5,665 10,140 9,410 21,863 15,226 | 5,834 4,895 4,405 940 4,167 3,866 10,001 8,316 | 17,989 15,917 14,470 2,072 12,847 11,922 30,836 26,392 |

- Foreign ship Category 2 and 3 numbers appear in Tables E2 and Table 2 as the estimates of total foreign flag emissions in U.S. waters.

- There may be some differences in totals due to rounding of numbers.

- Emissions estimated to a distance of 200 miles from shore; this is approximately the distance from shore as most of the major shipping lanes.

By assuming that all foreign flag vessels operating in U.S. waters are cargo vessels, the emissions estimated for foreign flag cargo vessels can be added to the emissions estimated for U.S. flag vessels (calculated in Section 2.1) to provide a single nationwide estimate for emissions from commercial shipping.  This relies on the understanding that fishing vessels, tugs, utility vessels, and most passenger/ferry vessels operating in U.S. waters are U.S. flag vessels.  The possible exception to this understanding would be foreign passenger cruise ships.  Emissions from these relatively few ships are not expected to change the nationwide inventory of ship emissions.  In fact, emissions from these ships may only impact port emissions inventories that are dominated by tourism-related vessel activity.

## Comparing the U.S. Emissions Inventory with a Global Inventory

Previous work can be used to verify that the estimate of nationwide emissions from ships in U.S. waters provide is reasonable.  A global inventory of ship emissions that was developed independent of the analysis for this report [Corbett and Fischbeck, 1997] can provide a point of comparison.  Because this method derives the traffic densities for ships from the Comprehensive Ocean-Atmosphere Data Set (COADS), this approach estimates only the coastal emissions (both oceangoing and some coastwise domestic); it cannot reliably estimate emissions on the Great Lakes, or the inland waterway emissions.  In addition, this method was not intended to include detailed coastwise domestic operations very near the coastlines.  Also, the COADS data filtered out reports from large fishing fleets.  Therefore, this provides only a rough check of the sum of oceangoing and coastwise emissions estimated by Method B.

Global P=i=1nEi∗F∗Aiemissions calculations used the following equation to obtain the total emissions from international shipping:

Equation 4

Where:

P   =	Total propulsion emissions.

EI  =	Fuel-based pollutant emission factor based on engine type.

F   =	Annual marine fuel (bunkers) used.

Ai  = 	Percent of all vessels with each engine type.

i    =	Engine type (1 = slow-speed, 2 = medium-speed, and 3 = steam/other.

n   =  	3 engine types for NOx calculation.

Emission rates from Lloyd’s were applied to diesel engines (see Table 4).  A steam-turbine emission rate of 8.8 kg NOx/tonne fuel was derived from data in a 1991 study by TRC Environmental Consultants, Inc. [Hottenstein, 1991].  Since more than 95% of ships in the world fleet use CI marine engines and the emission rates are higher for CI engines than for boilers, the resulting estimates reasonably represent CI emission estimates for practical purposes.  The annual estimates for global NOx emissions were calculated by weighting the yearly marine-fuel use by the actual percent of marine engines of each type (slow-speed diesel, medium-speed diesel, and steam turbine).  The average estimate for annual NOx emissions is simply the average of the estimates for each year: 10.12 million metric tons of NOx/yr.  Global emissions (average estimates) were divided by the total annual COADS ship observations to get the average emissions per observation.  This value was then multiplied by the number of observations in each 2o x 2o grid location to map the distribution of global marine emissions.  (More discussion of this method is available [Corbett and Fischbeck, 1997; Corbett et al., 1998].)

U.S. coastal emissions are simply the sum of the emissions estimated in each grid cell adjoining the coasts in the Atlantic, Pacific, and Gulf regions.  Table 12 illustrates this approach for NOx emissions, including approximate distances from the U.S. coastlines in kilometers and miles.  Shipping links described in Method B generally correspond to a distance between 1 and 1½ grid cells of the U.S. coastline.  Table 13 shows all pollutants considered.  If 1½ grid cells are used, these estimates are within 13% to 25% of the overall coastal emissions (the sum of oceangoing and coastwise emissions for all engine types) from cargo ships in U.S. waters presented in Table 11.

Table 12.  Summary of NOx Emissions Near U.S. Coastlines From Previous Work

| Number of Cells from U.S. Coasts | NOx Emissions (103 tons NOx/yr) | Approximate Distance (km) | Approximate Distance (mi) | Emissions as a % of Global |
| --- | --- | --- | --- | --- |
| 3 Grid Cells | 516.2 | 667 | 414 | 5% |
| 2 Grid Cells | 319.0 | 444 | 276 | 3% |
| 1½ Grid Cells | 203.9 | 333 | 207 | 2% |
| 1 Grid Cell | 88.7 | 222 | 138 | 1% |

Table 13.  Emissions Within Approximately 200 Miles of U.S. Coastline

|  | 103 kg NOx/yr | 103 kg PM/yr | 103 kg HC/yr | 103 kg CO/yr |
| --- | --- | --- | --- | --- |
| Oceangoing/Coastwise | 203.9 | 17.2 | 6.9 | 21.2 |

# ENGINE TYPES IN EPA CATEGORIES

Engine manufacturer and model data were available for some of the U.S. flag vessels in the Lloyds register data [LMIS, 1996].  Appendix 7.3 presents this data by vessel type and EPA Category.  Ships with engines in EPA Category 2 are listed in Table 16.  Those with engines in EPA Category 3 (shown as Category 3A, corresponding to cylinder displacements equal to 20 liters and less than 60 liters, and as Category 3B, corresponding to cylinder displacements equal to and greater than 60 liters) in Table 17.  It should be noted that these tables show the number of vessels with each engine type; the number of engines per vessel was not reported in the data available.  No other engine-specific data is provided in the Lloyds data fields that we use in our research, although Lloyds Registry may have that information.  Engine-specific information, such as bore, stroke, rpm, etc., was obtained from manufacturer sources such as websites, and telephone conversations with manufacturer representatives.  Some engine manufacturers were not located, perhaps because the engines are foreign, or no longer manufactured; in some cases engine manufacturers have consolidated or split into separate corporations.  Engines that were not categorized are presented in Table 18.  This analysis focused on defining cylinder displacement so that engines could be classified by EPA Category.  Other data (e.g., turbocharging, aftercooling, etc.) were not consistently available and are not reported.

# ENGINE REPLACEMENT RATES

Important considerations in developing regulations for CI marine engines include how quickly ships and/or their engines are replaced, and how fast the fleet may be growing.  If emissions regulations apply primarily to new engines, then the impact on actual emissions depends on the rate at which emission controls are introduced through engine replacements.  Engine replacement is a function of two primary factors:  1)  The number of ships retired from service and scrapped; and 2)  The number of new vessels built and placed in service.  (A secondary factor in estimating engine replacement rates is replacement of engines on existing vessels without scrapping the ship itself.  As of now, we have no direct information on engine replacement rates of this sort.)  In recent years, approximately 1% of the world fleet greater than 1,000 GRT was demolished (broken-up) annually while new ships in this group accounted for approximately 3% of the world fleet [UNCTAD, 1995]; therefore the net growth of the world fleet is currently about 2% per year.

Moreover, the U.S. fleet has not been building at the same rate as the world fleet (see Figure 1).  The U.S. flag fleet has lagged the pace of the world fleet in both of these aspects during the last decade, at least.  From data shown in Figure 1, it can be seen that the U.S. fleet construction rate has been less than 15 ships per year since 1983, whereas world fleet construction has averaged about 1,500 ships per year.  In other words, U.S. fleet construction has equaled ~0.5% of the approximately 5,300 ships in the U.S. fleet compared to world fleet construction of ~2% of the approximately 86,000 ships in the world fleet.

The world-fleet average age of a vessel when demolished is shown for recent years in Table 14 [UNCTAD, 1995].  The average age of the U.S. Fleet, by vessel type, is shown in Table 15; it is clear that the U.S. fleet is significantly older than the world average.  It is reasonable to believe the U.S. flag fleet may increase its rate of fleet replacement in coming years, to stay competitive with the world fleet vessel construction.  However, there is insufficient evidence to expect the U.S. fleet replacement rate to exceed the world fleet replacement rate of approximately 2% per year.  Under current market and policy conditions (e.g., U.S. versus foreign labor rates for merchant vessel crews, and the Jones Act), there are limited economic incentives to modernize the U.S. flag fleet at the world fleet replacement rate.  Therefore, the 2% per year world fleet replacement rate assumption would imply at most the construction of 1,000 additional ships of 100 GRT or greater over a ten-year period (2000-2010).  However, only 503 ships have been built in the U.S. during the twenty-two year period of 1975 to 1996 (see Figure 1).  Therefore, a more realistic upper bound might be 250 ships (100 GRT or greater) built over a ten-year period (2000-2010).

Figure 1.  Comparison of New Shipbuilding in U.S. Flag and World Fleets [Lloyd's_Register, 1970-1994; MARAD, 1996]

Table 14.  Average Age of Broken-up Ships Greater Than 1,000 GRT by Vessel Type During 1986-1994

| Year | Tankers | Dry Bulk | Containers | General Cargo |
| --- | --- | --- | --- | --- |
| 1986 | 21.3 | 19.4 | 21.7 | 23.6 |
| 1987 | 24.4 | 19.8 | 24.9 | 23.8 |
| 1988 | 24.6 | 22.4 | 25.1 | 24.2 |
| 1989 | 24.9 | 23.1 | 27.2 | 25.5 |
| 1990 | 26.4 | 21.7 | 19.5 | 25.1 |
| 1991 | 25.3 | 22.0 | 19.0 | 24.8 |
| 1992 | 25.8 | 22.9 | 19.1 | 25.7 |
| 1993 | 24.7 | 24.0 | 22.9 | 26.4 |
| 1994 | 24.6 | 24.1 | 24.0 | 27.1 |

Table 15.  Average Year Built for the 1996 U.S. Fleet Greater Than 100 GRT [LMIS, 1996]

| Vessel Service | Average Year Built | Average Age in 1996 |
| --- | --- | --- |
| Container | 1978 | 18 |
| Fishing | 1975 | 21 |
| Passenger | 1959 | 37 |
| RoRo | 1974 | 22 |
| Transport | 1964 | 32 |
| Tug | 1969 | 27 |
| Utility | 1976 | 20 |
| U.S. Fleet Average | 1973 | 23 |

# SUMMARY

A commercial marine emissions inventory was developed that estimates the emissions of NOx, PM, HC, and CO from U.S. flag and foreign flag vessels operating in U.S. waters.  This inventory is characterized by vessel service type and by EPA Category (representing engine cylinder displacement).  Emissions from commercial vessels that carry cargo were estimated by region, including oceangoing (international), coastwise (domestic), inland river system, and Great Lakes.  Emissions from cargo transport on the U.S. inland rivers equals nearly 70% of the total emissions from oceangoing and coastwise cargo transportation on all three coastlines.  Oceangoing and coastal emissions estimated for cargo ships were compared with emission derived from previous work estimating global shipping emissions.  Auxiliary engine emissions were not considered in this analysis.  Engine-specific data available from the Lloyds dataset is provided, including engine manufacturer and model number.  The U.S. fleet replacement rate is lower than the rate of replacement in the world fleet, and is not expected to increase construction beyond the world fleet replacement rate of approximately 2% per year under current market and policy conditions.

# REFERENCES

Abrams, A., Ship Cargo Capacity Tightens in 2nd Quarter, in Journal of Commerce, Washington, DC, 1997.

Booz Allen & Hamilton, Commercial Marine Vessel Contributions to Emission Inventories: Draft Final Report, Booz, Allen & Hamilton, Inc., Los Angeles, CA, 1991.

Carlton, J.S., S.D. Danton, R.W. Gawen, K.A. Lavender, N.M. Mathieson, A.G. Newell, G.L. Reynolds, A.D. Webster, C.M.R. Wills, and A.A. Wright, Marine Exhaust Emissions Research Programme, Lloyd's Register Engineering Services, London, 1995.

Corbett, J.J., and P.S. Fischbeck, Emissions From Ships, Science, 278 (5339), 823-824, 1997.

Corbett, J.J., P.S. Fischbeck, and S.N. Pandis, Global Nitrogen and Sulfur Emissions Inventories for Oceangoing Ships, Journal of Geophysical Research, under review, 1998.

DOT, Merchant Vessels of the United States (Raw data only) (on CD-ROM), NTIS, National Technical Information Service, Washington, DC, 1998.

EPA, AP-42: Compilation of Air Pollutant Emission Factors, U.S. Environmental Protection Agency, Research Triangle Park, NC, 1997.

Evans, J.J., and P.B. Marlow, Quantitative Methods in Maritime Economics, 282 pp., Fairplay Publications Ltd., Coulsdon, Surrey, UK, 1990.

Ewart, W.D., Bunkers, 72 pp., Fairplay Publications Ltd., Surrey CR3 2HR, UK, 1982.

Harrington, R.L., Marine Engineering, pp. 953, Society of Naval Architects and Marine Engineers, Jersey City, NJ, 1992.

Heywood, J.B., Internal Combustin Engine Fundamentals, 930 pp., McGraw-Hill, Inc., New York, NY, 1988.

Hottenstein, L.N., Ship Emissions Control Study for the Ports of Long Beach and Los Angeles: Marine Vessel Emissions While Hotelling in Port and Evaluation of Alternative NOx Control Technologies, TRC Environmental Consultants, Inc., Mission Viejo, CA, 1991.

ISO, Reciprocating Internal Combustion Engines -- Exhaust Emission Measurement -- Part 4: Test Cycles for Different Engine Applications, International Organization for Standardization, Geneva, Switzerland, 1996.

Laurence, C.A., Vessel Operating Economies, 55 pp., Fairplay Publications Ltd., Coulsdon, Surrey, UK, 1984.

LMIS, Dataset of Ships 100 GRT or Greater, Lloyd's Maritime Information Services, Stamford, CT, 1996.

Lloyd's Register, Merchant Shipbuilding Return, Lloyd's Register of Shipping, London, UK, 1970-1994.

MARAD, Outlook for the U.S. Shipbuilding and Repair Industry, Maritime Administration, U.S. Department of Transportation, Office of Ship Construction, Washington, DC, 1996.

Markle, S.P., and A.J. Brown, Naval Ship Engine Exhaust Emission Characterization, Naval Engineers Journal, 108 (5), 37-47, 1996.

Osborne, A., Modern Marine Engineer's Manual, Cornell Maritime Press, Centreville, MD, 1943.

Packard, W.V., Voyage Estimating, 79 pp., Fairplay Publications Ltd., Coulsdon, Surrey, UK, 1991.

Turpin, E.A., and W.A. McEwen, Merchant Marine Officer's Handbook, Cornell Maritime Press, Inc., Centreville, MD, 1965.

UNCTAD, Review of Maritime Transport 1994, United Nations, New York and Geneva, 1995.

USACE, United States Waterway Data, Bureau of Transportation Statistics, U.S. Department of Transportation, Oak Ridge, Tennessee, 1995.

Wilde Mathews, A., Cargo from Asia Overwhelms Transport, in Wall Street Journal, New York, NY, 1998a.

Wilde Mathews, A., Jet-Age Anomalies, Slowpoke Barges Do Brisk Business, in Wall Street Journal, New York, NY, 1998b.

# APPENDICES

## Method A: Ship Operations Method for 1996 (Pages A2-A8)

## Method B: Cargo Method using 1993 Cargo Trade Data (Pages A9-A18)

## Engine Manufactures and Engine Models in U.S. Flag Fleet (Pages A19-A31)

7.1  METHOD A: Ship Operations Method

1996 U.S. Flag Fleet CI Marine Engines, By EPA Category

| Count of Ships | Category |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vessel Service | Uncategorized |  | 1 | 2 | 3A | 3B | Not Diesel | Grand Total |
| CONTAINER | 3 |  |  | 1 |  | 47 | 36 | 87 |
| FISHING | 1926 |  | 766 | 74 | 2 | 10 |  | 2778 |
| PASSENGER | 20 |  | 4 |  | 1 | 2 | 14 | 41 |
| RORO | 45 |  | 3 | 11 | 8 | 13 | 17 | 97 |
| TRANSPORT | 82 |  | 5 | 14 | 12 | 27 | 279 | 419 |
| TUG | 760 |  | 98 | 391 | 9 | 8 | 3 | 1269 |
| UTILITY | 277 |  | 241 | 98 | 4 | 3 | 27 | 650 |
| Grand Total | 3113 |  | 1117 | 589 | 36 | 110 | 376 | 5341 |
|  | 58% |  | 21% | 11% | 1% | 2% | 7% |  |
| Expected distribution of uncategorized vessels with CI Marine Engines by EPA Category |  |  |  |  |  |  |  |  |
| Count of Ships | Category |  |  |  |  |  |  |  |
| Vessel Service |  | 1 |  | 2 | 3A | 3B | Count of ships |  |
| CONTAINER |  | - |  | 1 | - | 49 | 50 |  |
| FISHING |  | 2,498 |  | 241 | 7 | 33 | 2,778 |  |
| PASSENGER |  | 8 |  | - | 2 | 4 | 14 |  |
| RORO |  | 6 |  | 21 | 15 | 24 | 65 |  |
| TRANSPORT |  | 6 |  | 17 | 15 | 34 | 72 |  |
| TUG |  | 244 |  | 975 | 22 | 20 | 1,262 |  |
| UTILITY |  | 420 |  | 171 | 7 | 5 | 603 |  |
| Count of ships |  | 3,182 |  | 1,426 | 68 | 168 | 4,843 |  |

Category 3A engines are equal to or greater than 20 liters per cylinder and less than 60 liters per cylinder; Category 3B engines are greater than or equal to 60 liters per cylinder.

1996 U.S. Flag Fleet Fuel-Consumption Estimates (from BHP in metric tons per day)

|  | Average of tpd | Category |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Vessel Service | Uncategorized | 1 | 2 |  | 3A |  | 3B |  | Weighted Ave |  |  |
|  | CONTAINER | 36 |  | 19 |  |  |  | 101 |  | 96 |  |  |
|  | FISHING | 12 | 9 | 17 |  | 23 |  | 18 |  | 14 |  |  |
|  | PASSENGER | 36 | 17 |  |  | 18 |  | 36 |  | 32 |  |  |
|  | RORO | 45 |  | 23 |  | 87 |  | 74 |  | 53 |  |  |
|  | TRANSPORT | 33 | 15 | 22 |  | 56 |  | 53 |  | 40 |  |  |
|  | TUG | 14 | 10 | 17 |  | 44 |  | 56 |  | 16 |  |  |
|  | UTILITY | 15 | 10 | 16 |  | 36 |  | 22 |  | 13 |  |  |
|  | Weighted Ave | 17 | 10 | 17 |  | 55 |  | 72 |  | 20 |  |  |
| fuel use per day-ship (adjusted for max load) |  |  | Category |  |  |  |  |  |  |  | Weighted Average | Max load = 80% max power |  |
| Vessel Service | Average of tpd |  | 1 | 2 |  | 3A |  | 3B |  |  |  |  |  |
| CONTAINER |  |  | - | 15 |  | - |  | 81 |  | 77 | 80% |  |
| FISHING |  |  | 7 | 13 |  | 18 |  | 14 |  | 11 | 80% |  |
| PASSENGER |  |  | 13 | - |  | 14 |  | 29 |  | 25 | 80% |  |
| RORO |  |  | - | 18 |  | 69 |  | 59 |  | 42 | 80% |  |
| TRANSPORT |  |  | 12 | 17 |  | 45 |  | 42 |  | 32 | 80% |  |
| TUG |  |  | 8 | 14 |  | 35 |  | 44 |  | 13 | 80% |  |
| UTILITY |  |  | 8 | 12 |  | 29 |  | 18 |  | 10 | 80% |  |
| fuel use per day-ship (with marine duty cycle) |  |  | Category | E5 Cycle | E5 Cycle |  | E3 Cycle |  | E3 Cycle |  | Weighted Average |  |  |
| Vessel Service | Average of tpd |  | 1 | 2 |  | 3A |  | 3B |  |  |  |  |
| CONTAINER |  |  | - | 10 |  | - |  | 56 |  | 55 |  |  |
| FISHING |  |  | 5 | 9 |  | 13 |  | 10 |  | 5 |  |  |
| PASSENGER |  |  | 9 | - |  | 10 |  | 20 |  | 12 |  |  |
| RORO |  |  | - | 13 |  | 48 |  | 40 |  | 30 |  |  |
| TRANSPORT |  |  | 8 | 12 |  | 31 |  | 29 |  | 23 |  |  |
| TUG |  |  | 6 | 10 |  | 24 |  | 31 |  | 9 |  |  |
| UTILITY |  |  | 5 | 9 |  | 20 |  | 12 |  | 7 |  |  |
| Weighted Ave |  |  | 5 | 9 |  | 29 |  | 34 |  | 8 |  |  |

1996 World Fleet CI Marine Engines, by EPA Category

| World Ships Emission Inventory |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of ships | Category |  |  |  |  |  |
| Vessel Service | ? | 1 | 2 | 3A | 3B | Grand Total |
| CONTAINER | 983 |  | 2 | 38 | 800 | 1,823 |
| FISHING | 19,895 | 2,511 | 619 | 320 | 145 | 23,490 |
| PASSENGER | 2,824 | 544 | 35 | 230 | 39 | 3,672 |
| RORO | 2,116 | 92 | 87 | 210 | 191 | 2,696 |
| TRANSPORT | 30,685 | 371 | 378 | 1,018 | 3,594 | 36,046 |
| TUG | 7,004 | 847 | 1,170 | 180 | 115 | 9,316 |
| UTILITY | 5,603 | 1,190 | 322 | 110 | 67 | 7,292 |
| Grand Total | 69,110 | 5,555 | 2,613 | 2,106 | 4,951 | 84,335 |
| Statistical distribution of world fleet by engine category |  |  |  |  |  |  |
| Number of ships | Category |  |  |  |  |  |
| Vessel Service | ? | 1 | 2 | 3A | 3B | Grand Total |
| CONTAINER |  | - | 4 | 82 | 1,736 | 1,823 |
| FISHING |  | 16,407 | 4,045 | 2,091 | 947 | 23,490 |
| PASSENGER |  | 2,356 | 152 | 996 | 169 | 3,672 |
| RORO |  | 428 | 404 | 976 | 888 | 2,696 |
| TRANSPORT |  | 2,495 | 2,542 | 6,845 | 24,165 | 36,046 |
| TUG |  | 3,413 | 4,714 | 725 | 463 | 9,316 |
| UTILITY |  | 5,138 | 1,390 | 475 | 289 | 7,292 |
| Grand Total |  | 30,235 | 13,251 | 12,190 | 28,658 | 84,335 |

1996 NOx Emission Estimates (fuel-based)

| (kg NOx/tonne fuel) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 87 | Slow Speed emission factor |  |  |  | All Category 1, 2 use Medium-speed emission factors half 3A use Medium-speed, half use Slow-speed factors |  |  |  |
| 57 | Medium Speed emission factor |  |  |  | currently upper bound emissions factors |  |  |  |
|  |  |  |  |  | similar to AP-42 factors |  |  |  |
| 68% ships in U.S. fleet involved in foreign trade, assume they spend 30% time in U.S. waters (from Census data on Waterways CD) |  |  |  |  |  |  |  |  |
| kg NOx per day-ship |  | Category |  |  |  |  | Weighted Average | U.S. Fleet in U.S. Waters |
| Vessel Service |  |  | 1 | 2 | 3A | 3B |  |  |
| CONTAINER |  |  | - | 304 | - | 2,539 | 2,492 | 52% |
| FISHING |  |  | 286 | 528 | 906 | 863 | 315 | 100% |
| PASSENGER |  |  | 521 | - | 709 | 1,717 | 890 | 100% |
| RORO |  |  | - | 376 | 1,802 | 1,844 | 1,215 | 52% |
| TRANSPORT |  |  | 247 | 359 | 1,158 | 1,324 | 964 | 52% |
| TUG |  |  | 315 | 544 | 1,744 | 2,660 | 555 | 100% |
| UTILITY |  |  | 312 | 486 | 1,414 | 1,075 | 381 | 100% |
| Weighted column average |  |  |  | 291 | 530 | 1,483 | 1,821 | 431 |  |
| Assume 80% of the time underway |  |  |  |  |  |  |  |  |
| Estimated annual emissions |  |  | Category |  |  |  |  | Total | Percent underway |
| Vessel Service | kg NOx/yr |  | 1 | 2 | 3A | 3B |  |  |
| CONTAINER |  |  | - | 92,007 | - | 36,085,495 | 36,177,501 | 80% |
| FISHING |  |  | 208,365,719 | 37,192,614 | 1,725,533 | 8,211,745 | 255,495,611 | 80% |
| PASSENGER |  |  | 1,188,750 | - | 404,118 | 1,958,047 | 3,550,915 | 80% |
| RORO |  |  | - | 2,254,833 | 7,851,187 | 13,060,875 | 23,166,895 | 80% |
| TRANSPORT |  |  | 448,424 | 1,822,588 | 5,043,205 | 12,976,166 | 20,290,383 | 80% |
| TUG |  |  | 22,470,502 | 154,898,115 | 11,429,244 | 15,492,259 | 204,290,119 | 80% |
| UTILITY |  |  | 38,240,282 | 24,246,987 | 2,878,947 | 1,641,764 | 67,007,980 | 80% |
| Total |  |  | 270,713,677 | 220,507,144 | 29,332,232 | 89,426,350 | 609,979,403 |  |

1996 PM Emission Estimates (fuel-based)

| (kg PM/tonne fuel) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.6 | Slow Speed emission factor |  |  |  | All Category 1, 2 use Medium-speed emission factors half 3A use Medium-speed, half use Slow-speed factors |  |  |  |
| 1.2 | Medium Speed emission factor |  |  |  | currently upper bound emissions factors |  |  |  |
|  |  |  |  |  | similar to AP-42 factors |  |  |  |
| 68% ships in U.S. fleet involved in foreign trade, assume they spend 30% time in U.S. waters (from Census data) |  |  |  |  |  |  |  |  |
| kg PM per day-ship |  | Category |  |  |  |  | Weighted Average | U.S. Fleet in U.S. Waters |
| Vessel Service | Data |  | 1 | 2 | 3A | 3B |  |  |
| CONTAINER |  |  | - | 6 | - | 222 | 217 | 52% |
| FISHING |  |  | 6 | 11 | 55 | 75 | 7 | 100% |
| PASSENGER |  |  | 11 | - | 43 | 150 | 55 | 100% |
| RORO |  |  | - | 8 | 110 | 161 | 88 | 52% |
| TRANSPORT |  |  | 5 | 8 | 71 | 116 | 71 | 52% |
| TUG |  |  | 7 | 11 | 107 | 232 | 16 | 100% |
| UTILITY |  |  | 7 | 10 | 86 | 94 | 9 | 100% |
| Weighted column average |  |  |  | 6 | 11 | 91 | 159 | 14 |  |
| Assume 80% of the time underway |  |  |  |  |  |  |  |  |
| Estimated annual emissions |  |  | Category |  |  |  |  | Total | Percent underway |
| Vessel Service | kg PM/yr |  | 1 | 2 | 3A | 3B |  |  |
| CONTAINER |  |  | - | 1,937 | - | 3,152,296 | 3,154,233 | 80% |
| FISHING |  |  | 4,386,647 | 783,002 | 105,449 | 717,348 | 5,992,446 | 80% |
| PASSENGER |  |  | 25,026 | - | 24,696 | 171,048 | 220,770 | 80% |
| RORO |  |  | - | 47,470 | 479,795 | 1,140,950 | 1,668,215 | 80% |
| TRANSPORT |  |  | 9,441 | 38,370 | 308,196 | 1,133,550 | 1,489,557 | 80% |
| TUG |  |  | 473,063 | 3,261,013 | 698,454 | 1,353,347 | 5,785,877 | 80% |
| UTILITY |  |  | 805,059 | 510,463 | 175,936 | 143,418 | 1,634,876 | 80% |
| Total |  |  | 5,699,235 | 4,642,256 | 1,792,525 | 7,811,957 | 19,945,973 |  |

1996 HC Emission Estimates (fuel-based)

| (kg HC/tonne fuel) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.4 | Slow Speed emission factor |  |  |  | All Category 1, 2 use Medium-speed emission factors half 3A use Medium-speed, half use Slow-speed factors |  |  |  |
| 2.4 | Medium Speed emission factor |  |  |  | currently upper bound emissions factors |  |  |  |
|  |  |  |  |  | similar to AP-42 factors |  |  |  |
| 68% ships in U.S. fleet involved in foreign trade, assume they spend 30% time in U.S. waters (from Census data) |  |  |  |  |  |  |  |  |
| kg HC per day-ship |  | Category |  |  |  |  | Weighted Average | U.S. Fleet in U.S. Waters |
| Vessel Service | Data |  | 1 | 2 | 3A | 3B |  |  |
| CONTAINER |  |  | - | 13 | - | 70 | 69 | 52% |
| FISHING |  |  | 12 | 22 | 30 | 24 | 13 | 100% |
| PASSENGER |  |  | 22 | - | 24 | 47 | 29 | 100% |
| RORO |  |  | - | 16 | 60 | 51 | 38 | 52% |
| TRANSPORT |  |  | 10 | 15 | 39 | 37 | 30 | 52% |
| TUG |  |  | 13 | 23 | 58 | 73 | 22 | 100% |
| UTILITY |  |  | 13 | 20 | 47 | 30 | 16 | 100% |
| Weighted column average |  |  |  | 12 | 22 | 49 | 50 | 17 |  |
| Assume 80% of the time underway |  |  |  |  |  |  |  |  |
| Estimated annual emissions |  |  | Category |  |  |  |  | Total | Percent underway |
| Vessel Service | kg HC/yr |  | 1 | 2 | 3A | 3B |  |  |
| CONTAINER |  |  | - | 3,874 | - | 995,462 | 999,336 | 80% |
| FISHING |  |  | 8,773,293 | 1,566,005 | 57,518 | 226,531 | 10,623,347 | 80% |
| PASSENGER |  |  | 50,053 | - | 13,471 | 54,015 | 117,538 | 80% |
| RORO |  |  | - | 94,940 | 261,706 | 360,300 | 716,947 | 80% |
| TRANSPORT |  |  | 18,881 | 76,741 | 168,107 | 357,963 | 621,692 | 80% |
| TUG |  |  | 946,126 | 6,522,026 | 380,975 | 427,373 | 8,276,500 | 80% |
| UTILITY |  |  | 1,610,117 | 1,020,926 | 95,965 | 45,290 | 2,772,298 | 80% |
| Total |  |  | 11,398,471 | 9,284,511 | 977,741 | 2,466,934 | 24,127,657 |  |

1996 CO Emission Estimates

| (kg CO/tonne fuel) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.4 | Slow Speed emission factor |  |  |  | All Category 1, 2 use Medium-speed emission factors half 3A use Medium-speed, half use Slow-speed factors |  |  |  |
| 7.4 | Medium Speed emission factor |  |  |  | currently upper bound emissions factors |  |  |  |
|  |  |  |  |  | similar to AP-42 factors |  |  |  |
| 68% ships in U.S. fleet involved in foreign trade, assume they spend 30% time in U.S. waters (from Census data) |  |  |  |  |  |  |  |  |
| kg CO per day-ship |  | Category |  |  |  |  | Weighted Average | U.S. Fleet in U.S. Waters |
| Vessel Service | Data |  | 1 | 2 | 3A | 3B |  |  |
| CONTAINER |  |  | - | 39 | - | 216 | 212 | 52% |
| FISHING |  |  | 37 | 69 | 93 | 73 | 40 | 100% |
| PASSENGER |  |  | 68 | - | 73 | 146 | 91 | 100% |
| RORO |  |  | - | 49 | 185 | 157 | 116 | 52% |
| TRANSPORT |  |  | 32 | 47 | 119 | 113 | 91 | 52% |
| TUG |  |  | 41 | 71 | 179 | 226 | 69 | 100% |
| UTILITY |  |  | 40 | 63 | 145 | 91 | 49 | 100% |
| Weighted column average |  |  |  | 38 | 69 | 152 | 155 | 53 |  |
| Assume 80% of the time underway |  |  |  |  |  |  |  |  |
| Estimated annual emissions |  |  | Category |  |  |  |  |  | Percent underway |
| Vessel Service | kg CO/yr |  | 1 | 2 | 3A | 3B | Total |  |
| CONTAINER |  |  | - | 11,945 | - | 3,069,341 | 3,081,286 | 80% |
| FISHING |  |  | 27,050,988 | 4,828,515 | 177,346 | 698,470 | 32,755,320 | 80% |
| PASSENGER |  |  | 154,329 | - | 41,534 | 166,547 | 362,410 | 80% |
| RORO |  |  | - | 292,733 | 806,928 | 1,110,925 | 2,210,585 | 80% |
| TRANSPORT |  |  | 58,216 | 236,617 | 518,329 | 1,103,720 | 1,916,882 | 80% |
| TUG |  |  | 2,917,223 | 20,109,580 | 1,174,672 | 1,317,732 | 25,519,208 | 80% |
| UTILITY |  |  | 4,964,528 | 3,147,854 | 295,892 | 139,644 | 8,547,918 | 80% |
| Total Average |  |  | 35,145,284 | 28,627,243 | 3,014,702 | 7,606,379 | 74,393,608 |  |

7.2  Method B: Cargo Method using 1993 Cargo Trade Data

NOx Emissions (as NOx) Before Adjustments for Steam (non-CI) Engines in U.S. Fleet

| Oceans (Foreign and Domestic) |  |  |  |
| --- | --- | --- | --- |
| 1,691 | kg NOx per day-cargo-ship |  |  |
| 34,837 | Ave DWT per ship |  |  |
| 0.5 | cargo capacity factor |  |  |
| 308 | miles per day |  |  |
| 0.00031 | kg emissions per ton-mile |  |  |
| 764,927,137,344 | ton-miles on ocean/coastwise |  |  |
| 240,949,607 | kg emissions per year |  |  |
| 49% | Coastwise (domestic tons) |  |  |
| 51% | Oceangoing (foreign tons) |  |  |
| Great Lakes |  |  |  |
| 1,691 | kg NOx per day-cargo-ship |  |  |
| 34,837 | Ave DWT per ship |  |  |
| 0.6 | cargo capacity factor |  |  |
| 308 | miles per day |  |  |
| 0.00026 | kg emissions per ton-mile |  |  |
| 85,256,307,371 | ton-miles on Great Lakes |  |  |
| 22,379,554 | kg emissions per year |  |  |
| 81% | domestic tons |  |  |
| 19% | foreign tons |  |  |
| Inland Rivers (Uses U.S. Fleet Emissions for Category 1, 2, 3A Engines) |  |  |  |  |  |  |
| 744 | kg NOx per day-cargo-ship |  |  |
| 15,454 | Ave DWT per ship |  |  |
| 0.6 | cargo capacity factor |  |  |
| 191 | miles per day |  |  |
| 0.00042 | kg emissions per ton-mile |  |  |
| 494,368,733,957 | ton-miles on Inland Rivers |  |  |
| 207,661,859 | kg emissions per year |  |  |
| 70% | domestic tons |  |  |
| 30% | foreign tons |  |  |
| 470,991,020 | total national |  |  |
| 281,435,600 | total domestic |  |  |
| 189,555,420 | foreign |  |  |

Miles per day were converted from nautical miles to statute miles for these calculations.

PM Emissions Before Adjustments for Steam (non-CI) Engines in U.S. Fleet

| Oceans (Foreign and Domestic) |  |
| --- | --- |
| 138 | kg PM per day-cargo-ship |
| 34,837 | Ave DWT per ship |
| 0.5 | cargo capacity factor |
| 308 | miles per day |
| 0.00003 | kg emissions per ton-mile |
| 764,927,137,344 | ton-miles on ocean/coastwise |
| 19,610,143 | kg emissions per year |
| 49% | Coastwise (domestic tons) |
| 51% | Oceangoing (foreign tons) |
| Great Lakes |  |
| 138 | kg PM per day-cargo-ship |
| 34,837 | Ave DWT per ship |
| 0.6 | cargo capacity factor |
| 308 | miles per day |
| 0.00002 | kg emissions per ton-mile |
| 85,256,307,371 | ton-miles on Great Lakes |
| 1,821,403 | kg emissions per year |
| 81% | domestic tons |
| 19% | foreign tons |
| Inland Rivers (Uses U.S. Fleet Emissions for Category 1, 2, 3A Engines) |  |  |  |  |
| 38 | kg PM per day-cargo-ship |
| 15,454 | Ave DWT per ship |
| 0.6 | cargo capacity factor |
| 191 | miles per day |
| 0.00002 | kg emissions per ton-mile |
| 494,368,733,957 | ton-miles on Inland Rivers |
| 10,496,888 | kg emissions per year |
| 70% | domestic tons |
| 30% | foreign tons |
| 31,928,433 | total national |
| 18,447,527 | total domestic |
| 13,480,906 | foreign |

Miles per day were converted from nautical miles to statute miles for these calculations.

HC Emissions Before Adjustments for Steam (non-CI) Engines in U.S. Fleet

| Oceans (Foreign and Domestic) |  |  |
| --- | --- | --- |
| 49 | kg HC per day-cargo-ship |  |  |
| 34,837 | Ave DWT per ship |  |  |
| 0.5 | cargo capacity factor |  |  |
| 308 | miles per day |  |
| 0.00001 | kg emissions per ton-mile |  |  |
| 764,927,137,344 | ton-miles on ocean/coastwise |  |  |
| 6,961,848 | kg emissions per year |  |  |
| 49% | Coastwise (domestic tons) |  |  |
| 51% | Oceangoing (foreign tons) |  |  |
| Great Lakes |  |  |
| 49 | kg HC per day-cargo-ship |  |  |
| 34,837 | Ave DWT per ship |  |  |
| 0.6 | cargo capacity factor |  |  |
| 308 | miles per day |  |
| 0.00001 | kg emissions per ton-mile |  |  |
| 85,256,307,371 | ton-miles on Great Lakes |  |  |
| 646,621 | kg emissions per year |  |  |
| 81% | domestic tons |  |
| 19% | foreign tons |  |
| Inland Rivers (Uses U.S. Fleet Emissions for Category 1, 2, 3A Engines) |  |  |  |  |
| 27 | kg HC per day-cargo-ship |  |  |
| 15,454 | Ave DWT per ship |  |  |
| 0.6 | cargo capacity factor |  |  |
| 191 | miles per day |  |
| 0.00001 | kg emissions per ton-mile |  |  |
| 494,368,733,957 | ton-miles on Inland Rivers |  |  |
| 7,402,403 | kg emissions per year |  |  |
| 70% | domestic tons |  |
| 30% | foreign tons |  |
| 15,010,872 | total national |  |
| 9,107,752 | total domestic |  |
| 5,903,120 | foreign |  |

Miles per day were converted from nautical miles to statute miles for these calculations.

CO Emissions (as CO) Before Adjustments for Steam (non-CI) Engines in U.S. Fleet

| Oceans (Foreign and Domestic) |  |
| --- | --- |
| 151 | kg CO per day-cargo-ship |
| 34,837 | Ave DWT per ship |
| 0.5 | cargo capacity factor |
| 308 | miles per day |
| 0.00003 | kg emissions per ton-mile |
| 764,927,137,344 | ton-miles on ocean/coastwise |
| 21,465,698 | kg emissions per year |
| 49% | Coastwise (domestic tons) |
| 51% | Oceangoing (foreign tons) |
| Great Lakes |  |
| 151 | kg CO per day-cargo-ship |
| 34,837 | Ave DWT per ship |
| 0.6 | cargo capacity factor |
| 308 | miles per day |
| 0.00002 | kg emissions per ton-mile |
| 85,256,307,371 | ton-miles on Great Lakes |
| 1,993,748 | kg emissions per year |
| 81% | domestic tons |
| 19% | foreign tons |
| Inland Rivers (Uses U.S. Fleet Emissions for Category 1, 2, 3A Engines) |  |  |  |  |
| 82 | kg CO per day-cargo-ship |
| 15,454 | Ave DWT per ship |
| 0.6 | cargo capacity factor |
| 191 | miles per day |
| 0.00005 | kg emissions per ton-mile |
| 494,368,733,957 | ton-miles on Inland Rivers |
| 22,824,077 | kg emissions per year |
| 70% | domestic tons |
| 30% | foreign tons |
| 46,283,524 | total national |
| 28,082,236 | total domestic |
| 18,201,288 | foreign |

Miles per day were converted from nautical miles to statute miles for these calculations.

| Rough Emissions using Cargo Method (assumes all cargo carried by CI Engines) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | kg NOx per year | kg PM per year | kg HC per year | kg CO per year |
|  | Region | NOx | PM | HC | CO |
|  | Oceangoing (foreign tons) | 122,084,298 | 9,936,063 | 3,527,428 | 10,876,236 |
|  | US Flag CI Engines | 18,281,433 | 1,487,869 | 528,212 | 1,628,655 |
|  | Foreign Flag CI Engines | 103,802,866 | 8,448,194 | 2,999,215 | 9,247,581 |
|  | Coastwise (domestic tons) | 118,865,309 | 9,674,080 | 3,434,420 | 10,589,463 |
|  | Inland Rivers | 207,661,859 | 10,496,888 | 7,402,403 | 22,824,077 |
|  | US Flag CI Engines | 175,642,270 | 8,878,362 | 6,261,019 | 19,304,810 |
|  | Foreign Flag CI Engines | 32,019,589 | 1,618,526 | 1,141,384 | 3,519,267 |
|  | Great Lakes | 22,379,554 | 1,821,403 | 646,621 | 1,993,748 |
|  | US Flag CI Engines | 21,478,650 | 1,748,081 | 620,591 | 1,913,488 |
|  | Foreign Flag CI Engines | 900,904 | 73,322 | 26,030 | 80,260 |
|  | Total Nationwide Emissions | 470,991,020 | 31,928,433 | 15,010,872 | 46,283,524 |
|  | US Flag CI Engines | 334,267,661 | 21,788,392 | 10,844,243 | 33,436,416 |
|  | Foreign Flag CI Engines | 136,723,359 | 10,140,041 | 4,166,630 | 12,847,108 |

| Estimated number of cargo ships in world fleet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of cargo ships |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service |  | 1 |  |  | 2 |  |  | 3A |  |  | 3B |  |  | Grand Total |  |  |  |  |  |
| CONTAINER |  | - |  |  | 3 |  |  | 58 |  |  | 1,215 |  |  | 1,276 |  |  |  |  |  |
| RORO |  | 299 |  |  | 283 |  |  | 683 |  |  | 621 |  |  | 1,887 |  |  |  |  |  |
| TRANSPORT |  | 1,746 |  |  | 1,779 |  |  | 4,791 |  |  | 16,916 |  |  | 25,232 |  |  |  |  |  |
| Grand Total |  | 2,046 |  |  | 2,065 |  |  | 5,532 |  |  | 18,752 |  |  | 28,396 |  |  |  |  |  |
|  |  | 7% |  |  | 7% |  |  | 19% |  |  | 66% |  |  |  |  |  |  |  |  |
| Foreign emissions in U.S. estimated by EPA Category |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kg NOx per year |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service |  | 1 |  |  | 2 |  |  | 3A |  |  | 3B |  |  | Grand Total |  |  |  |  |  |
| CONTAINER |  | - |  |  | 14,629 |  |  | 277,960 |  |  | 5,851,788 |  |  | 6,144,378 |  |  |  |  |  |
| RORO |  | 1,441,355 |  |  | 1,363,020 |  |  | 3,290,049 |  |  | 2,992,378 |  |  | 9,086,803 |  |  |  |  |  |
| TRANSPORT |  | 8,407,685 |  |  | 8,566,320 |  |  | 23,070,143 |  |  | 81,448,030 |  |  | 15,231,181 |  |  |  |  |  |
| Grand Total |  | 9,849,040 |  |  | 9,943,970 |  |  | 26,638,152 |  |  | 90,292,196 |  |  | 136,723,359 |  |  |  |  |  |
| Without Category 1 engines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kg NOx per year |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service |  | 1 |  |  | 2 |  |  | 3A |  |  | 3B |  |  | Grand Total |  |  |  |  |  |
| CONTAINER |  |  |  |  | 14,629 |  |  | 277,960 |  |  | 5,851,788 |  |  | 6,144,378 |  |  |  |  |  |
| RORO |  |  |  |  | 1,363,020 |  |  | 3,290,049 |  |  | 2,992,378 |  |  | 7,645,448 |  |  |  |  |  |
| TRANSPORT |  |  |  |  | 8,566,320 |  |  | 23,070,143 |  |  | 81,448,030 |  |  | 13,789,826 |  |  |  |  |  |
| Grand Total |  | - |  |  | 9,943,970 |  |  | 26,638,152 |  |  | 90,292,196 |  |  | 126,874,319 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 93% |  |  |
| Note: There is a potential error due to differences in emission rates for different engine types. This error (which would increase the estimates for Category 3 engine emissions and decrease the estimates for Category 2 engine emissions) is assumed to be offsetting and/or small because of the few numbers of category 1 and 2 engines involved in cargo transport. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Estimated number of cargo ships in world fleet |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of cargo ships |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER | - |  | 3 |  | 58 |  | 1,215 |  | 1,276 |  |  |
| RORO | 299 |  | 283 |  | 683 |  | 621 |  | 1,887 |  |  |
| TRANSPORT | 1,746 |  | 1,779 |  | 4,791 |  | 16,916 |  | 25,232 |  |  |
| Grand Total | 2,046 |  | 2,065 |  | 5,532 |  | 18,752 |  | 28,396 |  |  |
| Foreign emissions in U.S. estimated by EPA Category |  |  |  |  |  |  |  |  |  |  |  |  |
| Kg PM per year |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER | - |  | 1,085 |  | 20,615 |  | 433,996 |  | 455,696 |  |  |
| RORO | 106,898 |  | 101,088 |  | 244,005 |  | 221,929 |  | 673,920 |  |  |
| TRANSPORT | 623,553 |  | 635,318 |  | 1,710,989 |  | 6,040,565 |  | 1,129,615 |  |  |
| Grand Total | 730,451 |  | 737,491 |  | 1,975,610 |  | 6,696,490 |  | 10,140,041 |  |  |
| Without Category 1 engines |  |  |  |  |  |  |  |  |  |  |  |
| Kg PM per year |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER |  |  | 1,085 |  | 20,615 |  | 433,996 |  | 455,696 |  |  |
| RORO |  |  | 101,088 |  | 244,005 |  | 221,929 |  | 567,022 |  |  |
| TRANSPORT |  |  | 635,318 |  | 1,710,989 |  | 6,040,565 |  | 1,022,718 |  |  |
| Grand Total | - |  | 737,491 |  | 1,975,610 |  | 6,696,490 |  | 9,409,591 |  |  |
| Note: There is a potential error due to differences in emission rates for different engine types. This error (which would increase the estimates for Category 3 engine emissions and decrease the estimates for Category 2 engine emissions) is assumed to be offsetting and/or small because of the few numbers of category 1 and 2 engines involved in cargo transport. |  |  |  |  |  |  |  |  |  |  |  |

| Estimated number of cargo ships in world fleet |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of cargo ships |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER | - |  | 3 |  | 58 |  | 1,215 |  | 1,276 |  |  |
| RORO | 299 |  | 283 |  | 683 |  | 621 |  | 1,887 |  |  |
| TRANSPORT | 1,746 |  | 1,779 |  | 4,791 |  | 16,916 |  | 25,232 |  |  |
| Grand Total | 2,046 |  | 2,065 |  | 5,532 |  | 18,752 |  | 28,396 |  |  |
| Foreign emissions in U.S. estimated by EPA Category |  |  |  |  |  |  |  |  |  |  |  |  |
| Kg HC per year |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER | - |  | 446 |  | 8,471 |  | 178,333 |  | 187,249 |  |  |
| RORO | 43,925 |  | 41,538 |  | 100,264 |  | 91,192 |  | 276,919 |  |  |
| TRANSPORT | 256,223 |  | 261,058 |  | 703,060 |  | 2,482,120 |  | 464,169 |  |  |
| Grand Total | 300,148 |  | 303,041 |  | 811,795 |  | 2,751,645 |  | 4,166,630 |  |  |
| Without Category 1 engines |  |  |  |  |  |  |  |  |  |  |  |
| Kg HC per year |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER |  |  | 446 |  | 8,471 |  | 178,333 |  | 187,249 |  |  |
| RORO |  |  | 41,538 |  | 100,264 |  | 91,192 |  | 232,994 |  |  |
| TRANSPORT |  |  | 261,058 |  | 703,060 |  | 2,482,120 |  | 420,243 |  |  |
| Grand Total | - |  | 303,041 |  | 811,795 |  | 2,751,645 |  | 3,866,481 |  |  |

Note:  The potential error described on the previous worksheets does not apply here because calculations for all engines use the same emission factor for this pollutant.

| Estimated number of cargo ships in world fleet |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of cargo ships |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER | - |  | 3 |  | 58 |  | 1,215 |  | 1,276 |  |  |
| RORO | 299 |  | 283 |  | 683 |  | 621 |  | 1,887 |  |  |
| TRANSPORT | 1,746 |  | 1,779 |  | 4,791 |  | 16,916 |  | 25,232 |  |  |
| Grand Total | 2,046 |  | 2,065 |  | 5,532 |  | 18,752 |  | 28,396 |  |  |
| Foreign emissions in U.S. estimated by EPA Category |  |  |  |  |  |  |  |  |  |  |  |  |
| Kg CO per year |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER | - |  | 1,375 |  | 26,118 |  | 549,859 |  | 577,352 |  |  |
| RORO | 135,436 |  | 128,075 |  | 309,147 |  | 281,177 |  | 853,835 |  |  |
| TRANSPORT | 790,022 |  | 804,928 |  | 2,167,769 |  | 7,653,203 |  | 1,431,187 |  |  |
| Grand Total | 925,458 |  | 934,378 |  | 2,503,034 |  | 8,484,239 |  | 12,847,108 |  |  |
| Without Category 1 engines |  |  |  |  |  |  |  |  |  |  |  |
| Kg CO per year |  |  |  |  |  |  |  |  |  |  |  |
| Vessel Service | 1 |  | 2 |  | 3A |  | 3B |  | Grand Total |  |  |
| CONTAINER |  |  | 1,375 |  | 26,118 |  | 549,859 |  | 577,352 |  |  |
| RORO |  |  | 128,075 |  | 309,147 |  | 281,177 |  | 718,399 |  |  |
| TRANSPORT |  |  | 804,928 |  | 2,167,769 |  | 7,653,203 |  | 1,295,751 |  |  |
| Grand Total | - |  | 934,378 |  | 2,503,034 |  | 8,484,239 |  | 11,921,650 |  |  |

Note:  The potential error described on the previous worksheets does not apply here because calculations for all engines use the same emission factor for this pollutant.

Cargo Method emissions adjustments: Previous calculations are adjusted to account for steam plants in U.S. fleet and to highlight Category 2 and 3 engines.  See text for calculation details.

|  | Region | NOx | PM | HC | CO |
| --- | --- | --- | --- | --- | --- |
|  | Oceangoing | 113,638,624 | 9,248,696 | 3,283,404 | 10,123,828 |
|  | Categories 2 and 3 only | 104,318,416 | 8,236,776 | 2,999,898 | 9,286,246 |
|  | US Cargo Ships all engines | 9,835,758 | 800,502 | 284,188 | 876,247 |
|  | Diesel Engines | 8,694,997 | 413,656 | 238,410 | 775,313 |
|  | Categories 2 and 3 only | 7,993,121 | 397,158 | 216,735 | 704,826 |
|  | Steam Engines | 1,140,761 | 386,846 | 45,778 | 100,934 |
|  | Foreign Vessels | 103,802,866 | 8,448,194 | 2,999,215 | 9,247,581 |
|  | Categories 2 and 3 only | 96,325,295 | 7,839,618 | 2,783,163 | 8,581,420 |
|  | Coastwise (domestic) | 63,951,794 | 5,204,839 | 1,847,783 | 5,697,332 |
|  | Diesel Engines | 56,534,599 | 2,689,578 | 1,550,134 | 5,041,063 |
|  | Categories 2 and 3 only | 51,971,025 | 2,582,309 | 1,409,204 | 4,582,758 |
|  | Steam Engines | 7,417,195 | 2,515,261 | 297,650 | 656,269 |
|  | Inland Rivers | 126,518,467 | 6,395,253 | 4,509,931 | 13,905,622 |
|  | Categories 2 and 3 only | 106,508,416 | 3,871,840 | 3,628,172 | 11,620,216 |
|  | US Cargo Ships all engines | 94,498,878 | 4,776,727 | 3,368,547 | 10,386,355 |
|  | Diesel Engines | 83,538,800 | 2,468,353 | 2,825,926 | 9,189,963 |
|  | Categories 2 and 3 only | 76,795,399 | 2,369,907 | 2,569,009 | 8,354,464 |
|  | Steam Engines | 10,960,078 | 2,308,374 | 542,622 | 1,196,392 |
|  | Foreign Vessels | 32,019,589 | 1,618,526 | 1,141,384 | 3,519,267 |
|  | Categories 2 and 3 only | 29,713,018 | 1,501,933 | 1,059,163 | 3,265,752 |
|  | Great Lakes | 12,456,826 | 1,013,822 | 359,920 | 1,109,753 |
|  | Categories 2 and 3 only | 10,227,034 | 534,656 | 278,795 | 902,570 |
|  | US Cargo Ships all engines | 11,555,922 | 940,501 | 333,890 | 1,029,493 |
|  | Diesel Engines | 10,215,654 | 486,000 | 280,105 | 910,907 |
|  | Categories 2 and 3 only | 9,391,028 | 466,616 | 254,640 | 828,092 |
|  | Steam Engines | 1,340,268 | 454,501 | 53,785 | 118,586 |
|  | Foreign Vessels | 900,904 | 73,322 | 26,030 | 80,260 |
|  | Categories 2 and 3 only | 836,006 | 68,040 | 24,155 | 74,478 |
|  | Total Nationwide Emissions | 316,565,710 | 21,862,610 | 10,001,038 | 30,836,535 |
|  | Categories 2 and 3 only | 273,024,892 | 15,225,581 | 8,316,069 | 26,391,791 |
|  | US Cargo Ships all engines | 179,842,352 | 11,722,569 | 5,834,409 | 17,989,427 |
|  | Diesel Engines | 158,984,050 | 6,057,587 | 4,894,574 | 15,917,246 |
|  | Categories 2 and 3 only | 146,150,573 | 5,815,990 | 4,449,588 | 14,470,141 |
|  | Steam Engines | 20,858,302 | 5,664,982 | 939,834 | 2,072,181 |
|  | Foreign Vessels | 136,723,359 | 10,140,041 | 4,166,630 | 12,847,108 |
|  | Categories 2 and 3 only | 126,874,319 | 9,409,591 | 3,866,481 | 11,921,650 |

7.3 Engine Manufactures and Engine Models in the U.S. Flag

Table 16.  Category 2 Engine Manufacturer Data, by Vessel Service

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 2 | Grand Total |
| CONTAINER | Alco | 12V251C | 1 | 1 |
|  | Alco Total |  | 1 | 1 |
| CONTAINER Total |  |  |  | 1 | 1 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 2 | Grand Total |
| FISHING | Alco | 16V251F | 2 | 2 |
|  |  | 18V251F | 2 | 2 |
|  | Alco Total |  | 4 | 4 |
|  | Caterpillar | 3608TA | 1 | 1 |
|  |  | 3612TA | 1 | 1 |
|  | Caterpillar Total |  | 2 | 2 |
|  | Deutz | SBV6M628 | 1 | 1 |
|  | Deutz Total |  | 1 | 1 |
|  | Fairbanks, Morse | 4-38D8-1/8 | 1 | 1 |
|  | Fairbanks, Morse Total |  |  | 1 | 1 |
|  | General Motors | 12-567-BC | 2 | 2 |
|  |  | 12-645-E2 | 3 | 3 |
|  |  | 12-645-E5 | 2 | 2 |
|  |  | 12-645-E6 | 14 | 14 |
|  |  | 16-567-BC | 1 | 1 |
|  |  | 16-645-E2 | 2 | 2 |
|  |  | 16-645-E5 | 3 | 3 |
|  |  | 16-645-E6 | 7 | 7 |
|  |  | 16-645-E7B | 1 | 1 |
|  |  | 20-645-E5 | 2 | 2 |
|  |  | 20-645-E6 | 1 | 1 |
|  |  | 20-645-E7 | 18 | 18 |
|  |  | 20-645-E7B | 4 | 4 |
|  |  | 8-645-E7B | 2 | 2 |
|  | General Motors Total |  |  | 62 | 62 |
|  | MWM | TBD441V16 | 4 | 4 |
|  | MWM Total |  | 4 | 4 |
| FISHING Total |  |  | 74 | 74 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 2 | Grand Total |
| RORO | General Electric | 7FDM12 | 3 | 3 |
|  | General Electric Total |  |  | 3 | 3 |
|  | General Motors | 12-567-BC | 1 | 1 |
|  |  | 12-645-E6 | 3 | 3 |
|  |  | 12-645-E7B | 2 | 2 |
|  |  | 16-645-E7B | 2 | 2 |
|  | General Motors Total |  |  | 8 | 8 |
| RORO Total |  |  | 11 | 11 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 2 | Grand Total |
| TRANSPORT | Alco | 12V251C | 2 | 2 |
|  |  | 12V251E | 1 | 1 |
|  |  | 16V251E | 3 | 3 |
|  |  | 16V251F | 1 | 1 |
|  | Alco Total |  | 7 | 7 |
|  | Fairbanks, Morse | 12-38TD-1/8 | 1 | 1 |
|  | Fairbanks, Morse Total |  |  | 1 | 1 |
|  | General Electric | 7FDS16-A2 | 1 | 1 |
|  | General Electric Total |  |  | 1 | 1 |
|  | General Motors | 12-645-E6 | 1 | 1 |
|  |  | 16-645-E6 | 1 | 1 |
|  |  | 16-645-E7 | 1 | 1 |
|  |  | 20-645-E7 | 1 | 1 |
|  |  | 8-645-E7 | 1 | 1 |
|  | General Motors Total |  |  | 5 | 5 |
| TRANSPORT Total |  |  |  | 14 | 14 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 2 | Grand Total |
| TUG | Alco | 12V251C | 8 | 8 |
|  |  | 12V251E | 17 | 17 |
|  |  | 12V251F | 10 | 10 |
|  |  | 16V251E | 8 | 8 |
|  |  | 16V251F | 6 | 6 |
|  |  | 18V251F | 3 | 3 |
|  | Alco Total |  | 52 | 52 |
|  | Caterpillar | 3608TA | 2 | 2 |
|  | Caterpillar Total |  | 2 | 2 |
|  | Fairbanks, Morse | 10-38D8-1/8 | 1 | 1 |
|  |  | 12-38D8-1/8 | 1 | 1 |
|  |  | 12-38TD-1/8 | 1 | 1 |
|  |  | 8-38D8-1/8 | 1 | 1 |
|  | Fairbanks, Morse Total |  |  | 4 | 4 |
|  | General Electric | 7FDM16 | 1 | 1 |
|  | General Electric Total |  |  | 1 | 1 |
|  | General Motors | 12-567-BC | 4 | 4 |
|  |  | 12-645-C | 3 | 3 |
|  |  | 12-645-E2 | 6 | 6 |
|  |  | 12-645-E5 | 4 | 4 |
|  |  | 12-645-E6 | 66 | 66 |
|  |  | 12-645-E7 | 1 | 1 |
|  |  | 12-645-E7A | 1 | 1 |
|  |  | 12-645-E7B | 22 | 22 |
|  |  | 12-645-F7B | 2 | 2 |
|  |  | 12V-645-E6 | 1 | 1 |
|  |  | 16-567-BC | 23 | 23 |
|  |  | 16-645 | 3 | 3 |
|  |  | 16-645-C | 1 | 1 |
|  |  | 16-645-E2 | 13 | 13 |
|  |  | 16-645-E3 | 2 | 2 |
|  |  | 16-645-E5 | 15 | 15 |
|  |  | 16-645-E6 | 67 | 67 |
|  |  | 16-645-E7 | 33 | 33 |
|  |  | 16-645-E7B | 17 | 17 |
|  |  | 16-645-E8 | 4 | 4 |
|  |  | 16-645-F7B | 4 | 4 |
|  |  | 16-710-GA | 2 | 2 |
|  |  | 20-645-E5 | 7 | 7 |
|  |  | 20-645-E7 | 3 | 3 |
|  |  | 20-645-E7B | 18 | 18 |
|  |  | 8-645-E2 | 1 | 1 |
|  |  | 8-645-E5 | 1 | 1 |
|  |  | 8-645-E6 | 4 | 4 |
|  | General Motors Total |  |  | 328 | 328 |
|  | MWM | TBD441V16 | 1 | 1 |
|  | MWM Total |  | 1 | 1 |
|  | Niigata | 6L25BX | 1 | 1 |
|  | Niigata Total |  | 1 | 1 |
|  | Wartsila | 12V22 | 1 | 1 |
|  |  | 8R22 | 1 | 1 |
|  | Wartsila Total |  | 2 | 2 |
| TUG Total |  |  | 391 | 391 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 2 | Grand Total |
| UTILITY | Alco | 12V251E | 3 | 3 |
|  |  | 12V251F | 1 | 1 |
|  |  | 16V251F | 1 | 1 |
|  |  | 18V251F | 1 | 1 |
|  | Alco Total |  | 6 | 6 |
|  | Caterpillar | 3608TA | 2 | 2 |
|  |  | 3612TA | 1 | 1 |
|  | Caterpillar Total |  | 3 | 3 |
|  | Fairbanks, Morse | 12-38TD-1/8 | 1 | 1 |
|  | Fairbanks, Morse Total |  |  | 1 | 1 |
|  | General Motors | 12-567-BC | 7 | 7 |
|  |  | 12-645-C | 1 | 1 |
|  |  | 12-645-E2 | 10 | 10 |
|  |  | 12-645-E6 | 25 | 25 |
|  |  | 12-645-E7 | 1 | 1 |
|  |  | 12-645-E7A | 6 | 6 |
|  |  | 12-645-E7B | 2 | 2 |
|  |  | 12-645-F7B | 1 | 1 |
|  |  | 16-645-E2 | 3 | 3 |
|  |  | 16-645-E5 | 6 | 6 |
|  |  | 16-645-E6 | 13 | 13 |
|  |  | 16-645-E7 | 2 | 2 |
|  |  | 16-645-E7B | 5 | 5 |
|  |  | 16-645-E8 | 1 | 1 |
|  |  | 16-645-F7B | 1 | 1 |
|  |  | 8-645-E6 | 3 | 3 |
|  | General Motors Total |  |  | 87 | 87 |
|  | MAN | 6L20/27 | 1 | 1 |
|  | MAN Total |  | 1 | 1 |
| UTILITY Total |  |  | 98 | 98 |

Table 17.  Category 3A and 3B Engine Manufacturer Data, by Vessel Service

| Count of Engine |  |  | Category |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 3A | 3B | Grand Total |
| CONTAINER | B&W | 12L90GFCA |  | 2 | 2 |
|  |  | 7L70MC |  | 3 | 3 |
|  |  | 8L80MC |  | 1 | 1 |
|  |  | 8L90GBE |  | 2 | 2 |
|  |  | 9L80MCE |  | 4 | 4 |
|  | B&W Total |  |  | 12 | 12 |
|  | Sulzer | 12RND90M |  | 5 | 5 |
|  |  | 12RTA84 |  | 5 | 5 |
|  |  | 6RND90 |  | 1 | 1 |
|  |  | 6RTA76 |  | 2 | 2 |
|  |  | 7RLB90 |  | 9 | 9 |
|  |  | 7RTA76 |  | 3 | 3 |
|  |  | 9RND90 |  | 3 | 3 |
|  |  | 9RND90M |  | 7 | 7 |
|  | Sulzer Total |  |  | 35 | 35 |
| CONTAINER Total |  |  |  |  | 47 | 47 |

| Count of Engine |  |  | Category |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 3A | 3B | Grand Total |
| FISHING | MaK | 6M453C |  | 1 | 1 |
|  |  | 6M551AK |  | 1 | 1 |
|  |  | 8M453C |  | 1 | 1 |
|  |  | 8M551AK |  | 1 | 1 |
|  |  | 9M453AK |  | 4 | 4 |
|  |  | 9M453C |  | 1 | 1 |
|  | MaK Total |  |  | 9 | 9 |
|  | Mitsubishi | 8UET45/80D |  | 1 | 1 |
|  | Mitsubishi Total |  |  |  | 1 | 1 |
|  | Wartsila | 12V32E | 1 |  | 1 |
|  |  | 6R32 | 1 |  | 1 |
|  | Wartsila Total |  | 2 |  | 2 |
| FISHING Total |  |  | 2 | 10 | 12 |

| Count of Engine |  |  | Category |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 3A | 3B | Grand Total |
| PASSENGER | Enterprise | DMRV-16-4 |  | 1 | 1 |
|  | Enterprise Total |  |  |  | 1 | 1 |
|  | MWM | TBD604-B12 | 1 |  | 1 |
|  | MWM Total |  | 1 |  | 1 |
|  | Sulzer | 6TD48 |  | 1 | 1 |
|  | Sulzer Total |  |  | 1 | 1 |
| PASSENGER Total |  |  |  | 1 | 2 | 3 |

| Count of Engine |  |  | Category |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 3A | 3B | Grand Total |
| RORO | B&W | 12L90GFCA |  | 1 | 1 |
|  |  | 6K90GF |  | 1 | 1 |
|  |  | 6L60MC |  | 1 | 1 |
|  |  | 6L70MC |  | 1 | 1 |
|  |  | 6S60MCE |  | 1 | 1 |
|  |  | 9K906F |  | 1 | 1 |
|  |  | 9K90GF |  | 1 | 1 |
|  | B&W Total |  |  | 7 | 7 |
|  | Pielstick | 18PC2V-400 | 8 |  | 8 |
|  | Pielstick Total |  | 8 |  | 8 |
|  | Sulzer | 6RND90 |  | 2 | 2 |
|  |  | 6RTA62 |  | 1 | 1 |
|  |  | 8RND90M |  | 2 | 2 |
|  |  | 9RND90 |  | 1 | 1 |
|  | Sulzer Total |  |  | 6 | 6 |
| RORO Total |  |  | 8 | 13 | 21 |

| Count of Engine |  |  | Category |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 3A | 3B | Grand Total |
| TRANSPORT | B&W | 10K84EF |  | 1 | 1 |
|  |  | 4L80MC |  | 2 | 2 |
|  |  | 4L90GBE |  | 2 | 2 |
|  |  | 8K90GF |  | 1 | 1 |
|  |  | Not specified |  | 1 | 1 |
|  | B&W Total |  |  | 7 | 7 |
|  | Enterprise | DMRV-12-4 |  | 3 | 3 |
|  |  | DMRV16-4 |  | 1 | 1 |
|  | Enterprise Total |  |  |  | 4 | 4 |
|  | MaK | 6M453AK |  | 1 | 1 |
|  |  | 6M601AK |  | 1 | 1 |
|  |  | 8M453AK |  | 1 | 1 |
|  |  | 8M601AK |  | 2 | 2 |
|  | MaK Total |  |  | 5 | 5 |
|  | Pielstick | 12PC2V-400 | 1 |  | 1 |
|  |  | 14PC2-2V-400 | 4 |  | 4 |
|  |  | 16PC2V-400 | 5 |  | 5 |
|  |  | 18PC2-5V-400 | 2 |  | 2 |
|  | Pielstick Total |  | 12 |  | 12 |
|  | Sulzer | 5RTA76 |  | 5 | 5 |
|  |  | 7RTA58 |  | 1 | 1 |
|  |  | 8RTA84 |  | 2 | 2 |
|  |  | 9RND90 |  | 2 | 2 |
|  | Sulzer Total |  |  | 10 | 10 |
|  | Wartsila | 8R46 |  | 1 | 1 |
|  | Wartsila Total |  |  | 1 | 1 |
| TRANSPORT Total |  |  |  | 12 | 27 | 39 |

| Count of Engine |  |  | Category |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 3A | 3B | Grand Total |
| TUG | B&W | 7S28LU | 2 |  | 2 |
|  | B&W Total |  | 2 |  | 2 |
|  | Enterprise | DMRV-16-4 |  | 6 | 6 |
|  | Enterprise Total |  |  |  | 6 | 6 |
|  | MaK | 6M551AK |  | 2 | 2 |
|  | MaK Total |  |  | 2 | 2 |
|  | Pielstick | 14PC2-2V-400 | 2 |  | 2 |
|  |  | 14PC2-5V-400 | 2 |  | 2 |
|  |  | 14PC2V-400 | 1 |  | 1 |
|  | Pielstick Total |  | 5 |  | 5 |
|  | Wartsila | 6R32 | 2 |  | 2 |
|  | Wartsila Total |  | 2 |  | 2 |
| TUG Total |  |  | 9 | 8 | 17 |

| Count of Engine |  |  | Category |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | 3A | 3B | Grand Total |
| UTILITY | Cooper Bessemer | KSV16T | 1 |  | 1 |
|  | Cooper Bessemer Total |  |  | 1 |  | 1 |
|  | Daihatsu | 8DSM-32 | 2 |  | 2 |
|  | Daihatsu Total |  | 2 |  | 2 |
|  | Enterprise | DMRV-16-4 |  | 1 | 1 |
|  | Enterprise Total |  |  |  | 1 | 1 |
|  | MaK | 6M453AK |  | 1 | 1 |
|  | MaK Total |  |  | 1 | 1 |
|  | Nordberg | 32112 |  | 1 | 1 |
|  | Nordberg Total |  |  |  | 1 | 1 |
|  | Wartsila | 9R32D | 1 |  | 1 |
|  | Wartsila Total |  | 1 |  | 1 |
| UTILITY Total |  |  | 4 | 3 | 7 |

Table 18.  Un-categorized Engine Manufacturer Data, by Vessel Service

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | Un-categorized | Grand Total |
| CONTAINER | MAN | K9Z78/155E | 2 | 2 |
|  | MAN Total |  | 2 | 2 |
|  | Not specified | Not specified |  |  |
|  | Not specified Total |  |  |  |  |
| CONTAINER Total |  |  |  | 2 | 2 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | Un-categorized | Grand Total |
| FISHING | .... | Not specified | 1071 | 1071 |
|  | .... Total |  | 1071 | 1071 |
|  | Akasaka | AH38 | 1 | 1 |
|  |  | TM6SS | 1 | 1 |
|  |  | UZ6SS | 1 | 1 |
|  |  | Not specified | 1 | 1 |
|  | Akasaka Total |  | 4 | 4 |
|  | Alco | Not specified | 2 | 2 |
|  | Alco Total |  | 2 | 2 |
|  | Alpha | 10V23L-VO | 1 | 1 |
|  |  | 406-24VO | 1 | 1 |
|  |  | 6L28/32 | 1 | 1 |
|  |  | 8V23L-VO | 1 | 1 |
|  |  | Not specified | 3 | 3 |
|  | Alpha Total |  | 7 | 7 |
|  | B&W | 6-33MTF-60 | 1 | 1 |
|  | B&W Total |  | 1 | 1 |
|  | Brons | 12TD200 | 1 | 1 |
|  | Brons Total |  | 1 | 1 |
|  | Buda | Not specified | 1 | 1 |
|  | Buda Total |  | 1 | 1 |
|  | Caterpillar | Not specified | 194 | 194 |
|  | Caterpillar Total |  |  | 194 | 194 |
|  | Cooper Bessemer | Not specified | 4 | 4 |
|  | Cooper Bessemer Total |  |  | 4 | 4 |
|  | Cummins | Not specified | 26 | 26 |
|  | Cummins Total |  |  | 26 | 26 |
|  | Deutz | Not specified | 3 | 3 |
|  | Deutz Total |  | 3 | 3 |
|  | Enterprise | Not specified | 12 | 12 |
|  | Enterprise Total |  |  | 12 | 12 |
|  | Fairbanks, Morse | Not specified | 11 | 11 |
|  | Fairbanks, Morse Total |  |  | 11 | 11 |
|  | General Electric | Not specified | 3 | 3 |
|  | General Electric Total |  |  | 3 | 3 |
|  | General Motors | Not specified | 138 | 138 |
|  | General Motors Total |  |  | 138 | 138 |
|  | Gray | Not specified | 1 | 1 |
|  | Gray Total |  | 1 | 1 |
|  | Kromhout | 8F/SW240 | 1 | 1 |
|  | Kromhout Total |  |  | 1 | 1 |
|  | MaK | 9M452AK | 1 | 1 |
|  |  | Not specified | 1 | 1 |
|  | MaK Total |  | 2 | 2 |
|  | MAN | Not specified |  |  |
|  | MAN Total |  |  |  |
|  | Niigata | Not specified | 1 | 1 |
|  | Niigata Total |  | 1 | 1 |
|  | Nohab | Not specified |  |  |
|  | Nohab Total |  |  |  |
|  | Nordberg | Not specified | 2 | 2 |
|  | Nordberg Total |  |  | 2 | 2 |
|  | Normo | BRM-8 | 2 | 2 |
|  |  | BRM-9 | 1 | 1 |
|  |  | KRMB-9 | 1 | 1 |
|  |  | KVMB-12 | 2 | 2 |
|  |  | KVMB-18 | 1 | 1 |
|  |  | Not specified | 1 | 1 |
|  | Normo Total |  | 8 | 8 |
|  | Penta | TAMD120B | 1 | 1 |
|  | Penta Total |  | 1 | 1 |
|  | Polar | F216V-A | 2 | 2 |
|  |  | F216V-B | 2 | 2 |
|  | Polar Total |  | 4 | 4 |
|  | Not specified | Not specified |  |  |
|  | Not specified Total |  |  |  |  |
|  | Not specified | Not specified | 249 | 249 |
|  | Not specified Total |  |  | 249 | 249 |
|  | Stork | Not specified | 1 | 1 |
|  | Stork Total |  | 1 | 1 |
|  | Stork-Werkspoor | 16SW280 | 1 | 1 |
|  | Stork-Werkspoor Total |  |  | 1 | 1 |
|  | Wartsila | Not specified |  |  |
|  | Wartsila Total |  |  |  |
|  | Westinghouse | Not specified | 1 | 1 |
|  | Westinghouse Total |  |  | 1 | 1 |
|  | White Superior | Not specified | 1 | 1 |
|  | White Superior Total |  |  | 1 | 1 |
|  | Wichmann | 10V28A | 1 | 1 |
|  |  | 4AXA | 2 | 2 |
|  |  | Not specified | 2 | 2 |
|  | Wichmann Total |  |  | 5 | 5 |
| FISHING Total |  |  | 1756 | 1756 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | Un-categorized | Grand Total |
| PASSENGER | .... | Not specified | 1 | 1 |
|  | .... Total |  | 1 | 1 |
|  | Baudouin | Not specified | 1 | 1 |
|  | Baudouin Total |  |  | 1 | 1 |
|  | Caterpillar | Not specified | 1 | 1 |
|  | Caterpillar Total |  |  | 1 | 1 |
|  | De Laval | Not specified |  |  |
|  | De Laval Total |  |  |  |
|  | Enterprise | Not specified | 2 | 2 |
|  | Enterprise Total |  |  | 2 | 2 |
|  | Fairbanks, Morse | Not specified | 1 | 1 |
|  | Fairbanks, Morse Total |  |  | 1 | 1 |
|  | General Electric | Not specified |  |  |
|  | General Electric Total |  |  |  |  |
|  | General Motors | Not specified | 3 | 3 |
|  | General Motors Total |  |  | 3 | 3 |
|  | MaK | Not specified |  |  |
|  | MaK Total |  |  |  |
|  | Not specified | Not specified | 2 | 2 |
|  | Not specified Total |  |  | 2 | 2 |
| PASSENGER Total |  |  |  | 11 | 11 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | Un-categorized | Grand Total |
| RORO | Busch-Sulzer | Not specified | 1 | 1 |
|  | Busch-Sulzer Total |  |  | 1 | 1 |
|  | Caterpillar | Not specified | 1 | 1 |
|  | Caterpillar Total |  |  | 1 | 1 |
|  | Cooper Bessemer | Not specified | 1 | 1 |
|  | Cooper Bessemer Total |  |  | 1 | 1 |
|  | Fairbanks, Morse | Not specified | 3 | 3 |
|  | Fairbanks, Morse Total |  |  | 3 | 3 |
|  | General Electric | Not specified | 3 | 3 |
|  | General Electric Total |  |  | 3 | 3 |
|  | General Motors | Not specified | 1 | 1 |
|  | General Motors Total |  |  | 1 | 1 |
|  | MAN | 14V52/55A | 3 | 3 |
|  |  | 9L52/55A | 3 | 3 |
|  | MAN Total |  | 6 | 6 |
|  | Not specified | Not specified |  |  |
|  | Not specified Total |  |  |  |  |
|  | Stork-Werkspoor | 6TM620 | 2 | 2 |
|  | Stork-Werkspoor Total |  |  | 2 | 2 |
|  | Sulzer | 6RLB66 | 2 | 2 |
|  |  | 6RND68 | 1 | 1 |
|  |  | 6RND68M | 2 | 2 |
|  |  | 7RND76M | 5 | 5 |
|  | Sulzer Total |  | 10 | 10 |
|  | Werkspoor | 18TM410 | 5 | 5 |
|  | Werkspoor Total |  |  | 5 | 5 |
| RORO Total |  |  | 33 | 33 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | Un-categorized | Grand Total |
| TRANSPORT | .... | Not specified | 14 | 14 |
|  | .... Total |  | 14 | 14 |
|  | B&W | 6K67GF | 2 | 2 |
|  |  | 7K67GF | 1 | 1 |
|  |  | 8K45GF | 2 | 2 |
|  |  | 9-84VT2BF-180 | 1 | 1 |
|  | B&W Total |  | 6 | 6 |
|  | Busch-Sulzer | Not specified | 1 | 1 |
|  | Busch-Sulzer Total |  |  | 1 | 1 |
|  | Caterpillar | Not specified | 5 | 5 |
|  | Caterpillar Total |  |  | 5 | 5 |
|  | Cooper Bessemer | Not specified | 3 | 3 |
|  | Cooper Bessemer Total |  |  | 3 | 3 |
|  | Deutz | Not specified | 1 | 1 |
|  | Deutz Total |  | 1 | 1 |
|  | Enterprise | Not specified | 1 | 1 |
|  | Enterprise Total |  |  | 1 | 1 |
|  | Fairbanks, Morse | Not specified | 7 | 7 |
|  | Fairbanks, Morse Total |  |  | 7 | 7 |
|  | General Motors | Not specified | 11 | 11 |
|  | General Motors Total |  |  | 11 | 11 |
|  | Gray | Not specified | 1 | 1 |
|  | Gray Total |  | 1 | 1 |
|  | Hamilton | Not specified | 1 | 1 |
|  | Hamilton Total |  |  | 1 | 1 |
|  | Kromhout | 9FCHD240 | 1 | 1 |
|  | Kromhout Total |  |  | 1 | 1 |
|  | MaK | 6M451AK | 1 | 1 |
|  |  | Not specified | 1 | 1 |
|  | MaK Total |  | 2 | 2 |
|  | MAN | 9L52/55B | 1 | 1 |
|  | MAN Total |  | 1 | 1 |
|  | Nordberg | Not specified | 3 | 3 |
|  | Nordberg Total |  |  | 3 | 3 |
|  | Rathbun-Jones | Not specified | 1 | 1 |
|  | Rathbun-Jones Total |  |  | 1 | 1 |
|  | Smit-Bolnes | 307HD | 1 | 1 |
|  | Smit-Bolnes Total |  |  | 1 | 1 |
|  | Sulzer | 5RLA90 | 1 | 1 |
|  |  | 5RLB90 | 5 | 5 |
|  |  | 5RND76M | 5 | 5 |
|  |  | 6RND76M | 2 | 2 |
|  |  | 7RND68 | 1 | 1 |
|  |  | 7RND76 | 2 | 2 |
|  | Sulzer Total |  | 16 | 16 |
|  | Wichmann | Not specified | 1 | 1 |
|  | Wichmann Total |  |  | 1 | 1 |
|  | Winton | Not specified | 1 | 1 |
|  | Winton Total |  | 1 | 1 |
| TRANSPORT Total |  |  |  | 78 | 78 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | Un-categorized | Grand Total |
| TUG | .... | Not specified | 84 | 84 |
|  | .... Total |  | 84 | 84 |
|  | Alco | Not specified | 2 | 2 |
|  | Alco Total |  | 2 | 2 |
|  | Alpha | 12V23L-VO | 1 | 1 |
|  |  | 14V23L-VO | 2 | 2 |
|  | Alpha Total |  | 3 | 3 |
|  | B&W | 7L67GFCA | 1 | 1 |
|  | B&W Total |  | 1 | 1 |
|  | Bolnes | Not specified | 1 | 1 |
|  | Bolnes Total |  | 1 | 1 |
|  | Brons | 12GV-H | 1 | 1 |
|  |  | Not specified | 2 | 2 |
|  | Brons Total |  | 3 | 3 |
|  | Caterpillar | Not specified | 77 | 77 |
|  | Caterpillar Total |  |  | 77 | 77 |
|  | Cooper Bessemer | Not specified | 2 | 2 |
|  | Cooper Bessemer Total |  |  | 2 | 2 |
|  | Crepelle | 6SN3 | 1 | 1 |
|  | Crepelle Total |  | 1 | 1 |
|  | Cummins | Not specified | 1 | 1 |
|  | Cummins Total |  |  | 1 | 1 |
|  | Enterprise | Not specified | 12 | 12 |
|  | Enterprise Total |  |  | 12 | 12 |
|  | Fairbanks, Morse | Not specified | 49 | 49 |
|  | Fairbanks, Morse Total |  |  | 49 | 49 |
|  | General Electric | Not specified | 1 | 1 |
|  | General Electric Total |  |  | 1 | 1 |
|  | General Motors | Not specified | 323 | 323 |
|  | General Motors Total |  |  | 323 | 323 |
|  | Imperial | Not specified | 1 | 1 |
|  | Imperial Total |  | 1 | 1 |
|  | Kromhout | Not specified | 2 | 2 |
|  | Kromhout Total |  |  | 2 | 2 |
|  | MaK | Not specified | 1 | 1 |
|  | MaK Total |  | 1 | 1 |
|  | Mirrlees | Not specified | 2 | 2 |
|  | Mirrlees Total |  | 2 | 2 |
|  | Nohab | F25.8V | 1 | 1 |
|  |  | Not specified | 4 | 4 |
|  | Nohab Total |  | 5 | 5 |
|  | Nordberg | Not specified | 4 | 4 |
|  | Nordberg Total |  |  | 4 | 4 |
|  | Polar | F212V | 1 | 1 |
|  |  | F212V-D | 1 | 1 |
|  |  | F28V-B | 3 | 3 |
|  |  | F28V-D | 2 | 2 |
|  |  | SF16RS-C | 1 | 1 |
|  |  | SF18VS-F | 1 | 1 |
|  |  | Not specified | 5 | 5 |
|  | Polar Total |  | 14 | 14 |
|  | Not specified | Not specified | 11 | 11 |
|  | Not specified Total |  |  | 11 | 11 |
|  | Rathbun-Jones | Not specified | 1 | 1 |
|  | Rathbun-Jones Total |  |  | 1 | 1 |
|  | Skinner Unaflow | Not specified | 1 | 1 |
|  | Skinner Unaflow Total |  |  | 1 | 1 |
|  | Stork | Not specified | 1 | 1 |
|  | Stork Total |  | 1 | 1 |
|  | Stork-Werkspoor | Not specified | 1 | 1 |
|  | Stork-Werkspoor Total |  |  | 1 | 1 |
|  | Sulzer | Not specified | 1 | 1 |
|  | Sulzer Total |  | 1 | 1 |
|  | Waukesha | Not specified | 1 | 1 |
|  | Waukesha Total |  |  | 1 | 1 |
|  | Westinghouse | Not specified | 1 | 1 |
|  | Westinghouse Total |  |  | 1 | 1 |
|  | White Superior | Not specified | 3 | 3 |
|  | White Superior Total |  |  | 3 | 3 |
|  | Wichmann | 10AXAG | 1 | 1 |
|  |  | 5AXA | 2 | 2 |
|  | Wichmann Total |  |  | 3 | 3 |
|  | Winton | Not specified | 1 | 1 |
|  | Winton Total |  | 1 | 1 |
| TUG Total |  |  | 614 | 614 |

| Count of Engine |  |  | Category |  |
| --- | --- | --- | --- | --- |
| Vessel Service | Engine Design | Engine | Un-categorized | Grand Total |
| UTILITY | .... | Not specified | 51 | 51 |
|  | .... Total |  | 51 | 51 |
|  | Alco | Not specified | 1 | 1 |
|  | Alco Total |  | 1 | 1 |
|  | Alpha | 6SL28L-VO | 1 | 1 |
|  | Alpha Total |  | 1 | 1 |
|  | B&W | 8-35VBF-62 | 1 | 1 |
|  | B&W Total |  | 1 | 1 |
|  | Caterpillar | Not specified | 72 | 72 |
|  | Caterpillar Total |  |  | 72 | 72 |
|  | Cooper Bessemer | Not specified | 1 | 1 |
|  | Cooper Bessemer Total |  |  | 1 | 1 |
|  | Cummins | Not specified | 3 | 3 |
|  | Cummins Total |  |  | 3 | 3 |
|  | De Laval | Not specified | 1 | 1 |
|  | De Laval Total |  | 1 | 1 |
|  | Deutz | Not specified | 2 | 2 |
|  | Deutz Total |  | 2 | 2 |
|  | Enterprise | Not specified | 3 | 3 |
|  | Enterprise Total |  |  | 3 | 3 |
|  | Fairbanks, Morse | Not specified | 8 | 8 |
|  | Fairbanks, Morse Total |  |  | 8 | 8 |
|  | Fiat | Not specified | 1 | 1 |
|  | Fiat Total |  | 1 | 1 |
|  | General Motors | Not specified | 71 | 71 |
|  | General Motors Total |  |  | 71 | 71 |
|  | Kromhout | 8F/SW240 | 1 | 1 |
|  |  | 9F/SW240 | 3 | 3 |
|  |  | Not specified | 2 | 2 |
|  | Kromhout Total |  |  | 6 | 6 |
|  | Lugger | L6125A | 1 | 1 |
|  | Lugger Total |  | 1 | 1 |
|  | M.T.U. | Not specified |  |  |
|  | M.T.U. Total |  |  |  |
|  | MAN | Not specified | 1 | 1 |
|  | MAN Total |  | 1 | 1 |
|  | Mitsubishi | Not specified | 1 | 1 |
|  | Mitsubishi Total |  |  | 1 | 1 |
|  | Nohab | F26R | 1 | 1 |
|  |  | Not specified | 1 | 1 |
|  | Nohab Total |  | 2 | 2 |
|  | Nordberg | Not specified | 4 | 4 |
|  | Nordberg Total |  |  | 4 | 4 |
|  | Polar | F26R-A | 1 | 1 |
|  |  | F26R-B | 1 | 1 |
|  |  | F26R-D | 2 | 2 |
|  |  | SF16RS-D | 1 | 1 |
|  |  | SF16RS-F | 2 | 2 |
|  |  | Not specified | 1 | 1 |
|  | Polar Total |  | 8 | 8 |
|  | Not specified | Not specified |  |  |
|  | Not specified Total |  |  |  |  |
|  | Not specified | Not specified | 5 | 5 |
|  | Not specified Total |  |  | 5 | 5 |
|  | Stork | Not specified | 2 | 2 |
|  | Stork Total |  | 2 | 2 |
|  | Stork-Werkspoor | 8SW280 | 1 | 1 |
|  | Stork-Werkspoor Total |  |  | 1 | 1 |
|  | Waukesha | Not specified | 1 | 1 |
|  | Waukesha Total |  |  | 1 | 1 |
|  | Werkspoor | 6TM410 | 1 | 1 |
|  |  | 9TM410 | 1 | 1 |
|  | Werkspoor Total |  |  | 2 | 2 |
|  | White Superior | Not specified | 1 | 1 |
|  | White Superior Total |  |  | 1 | 1 |
|  | Wichmann | 4AXA | 2 | 2 |
|  |  | 5AXA | 1 | 1 |
|  | Wichmann Total |  |  | 3 | 3 |
| UTILITY Total |  |  | 254 | 254 |

EPA is developing a marine module for its NONROAD emissions model that may ultimately provide such a comprehensive estimate.

This is without including the U.S. flag container ships, of which only three ships out of 87 were in the unassigned category; a chi-squared test for the world fleet – including container ships – showed similar results.

This analysis estimates that less than 2% of the world fleet of passenger ships are propelled by EPA Category 2 and/or 3 CI marine engines.  There are far more passenger ferries than passenger cruise ships in the world fleet.