Service Center

Safety Guide

SARP Completeness

Review Checklist for

NNSA Packages

SG 200

Revision 3

May 2006

TABLE OF CONTENTS

Section											Page

FORWARD 	 iii

1.0  INTRODUCTION 	 iv

2.0  COMPLETENESS REVIEW CHECKLIST	 iv

2.1  Requirements	 iv

2.2  Structure and Use of the Checklist	 iv

3.0  REFERENCES AND SUPPORTING DOCUMENTS	 v

Appendix – SARP Completeness Review Checklist	 1

FOREWORD

The purpose of this safety guide is to establish and disseminate standardized Department of Energy (DOE)/National Nuclear Security Administration (NNSA) processes for establishing and maintaining packaging programs that ensure that off-site shipments of nuclear weapon components, Category I and II special nuclear materials (SNM), nuclear weapon program special assemblies (SAs), and other materials of national security interest are safe, comply with the applicable federal regulatory and DOE Order 461.1A requirements, and present no undue risk to the public, worker safety and health, or the environment.

Comments (recommendations, proposed additions or deletions, or any other suggested improvements) should be sent to:

U.S. Department of Energy/ National Nuclear Security Administration

Service Center

Packaging Certification Division

Attention:  Manager

P.O. Box 5400

Albuquerque, NM 87185-5400

Or by e-mail at the addresses available on the National Nuclear Security Administration Service Center’s Offsite Transportation Authorizations/Certificates Web Site (http://www.doeal.gov/nnsaota/default.htm) Point of Contact page.

1.0  INTRODUCTION

The purpose of this safety guide is to establish a standard for determining the completeness of a Safety Analysis Report for Packaging (SARP) prior to its submittal to the Department of Energy/National Nuclear Security Administration (DOE/NNSA) Packaging Certification Division (PCD).

DOE Order 461.1A (04/26/2004), Packaging and Transfer or Transportation of Materials of National Security Interest, requires that a SARP be submitted in support of a Type B package certification application and the associated request for an Offsite Transportation Certificate (OTC).  This safety guide summarizes, in the form of a checklist, SARP requirements from the applicable regulations, orders, and guidance contained in the documents listed in Section 3.0.  The purpose of this safety guide is to provide a tool that applicants can use to ensure that SARPs submitted to DOE/NNSA are thorough and complete, and therefore contain the information needed to demonstrate the proposed package’s ability to meet the prescribed regulatory and DOE/NNSA requirements.  This safety guide is not a substitute for a formal DOE/NNSA Transportation Safety Review Panel review.

2.0	COMPLETENESS REVIEW CHECKLIST

2.1	Requirements

A SARP must be submitted in support of an application to certify a Type B package and to request the associated OTC.

To ensure that the SARP is complete in addressing the requirements of the applicable regulations, orders, and guidance, the organization submitting the application to certify or recertify a Type B package shall compare its SARP against the “SARP Completeness Review Checklist” found in the appendix of this safety guide before submitting the SARP to DOE/NNSA PCD.

The comparison of the SARP against the Completeness Review Checklist shall be documented. The documentation should consist of filling in the data entries in the Completeness Review Checklist that are to be provided by SARP completeness reviewers, as explained in Section 2.2.  A completed SG-200 checklist must be submitted with all applications for Offsite Transportation Certificates (certification and recertification).

2.2	Structure and Use of the Checklist

The Completeness Review Checklist is organized by SARP chapter and section in the format defined by RG 7.9 to the extent practical, with the addition of a chapter on quality assurance.  RG 7.9 has been updated to be consistent with the current revision of 10 CFR 71; however, where conflicts exist, 10 CFR 71 requirements will apply.  For each SARP section, the requirements basis in regulation, order, or guidance is provided.

The remainder of the checklist consists of three entries that require responses/entries by SARP completeness reviewers:

- The location in the SARP where the requirement is addressed. Note that it is not mandatory for the SARP to address a requirement in the location suggested by the checklist.

- A Y for YES, N for NO, or NA for NOT APPLICABLE response to the question “Does the SARP address the requirement satisfactorily?”

- A reference number for any comment, provided at the end of this Checklist section (SARP chapter), that expands on an entry made in Item 2, above.

3.0	REFERENCES AND SUPPORTING DOCUMENTS

The current revision of a reference should be used to determine requirements unless otherwise directed by DOE/NNSA PCD.  At the time this Guide was issued, the only exception to the requirement to use the current version of a referenced document is that the ANSI/ANS-6.1.1 1977 should be used.  Documents in the list below (e.g., RG 7.6, RG 7.8, etc.), but not explicitly stated in the “Requirement Basis” column, were included to assist the applicant in the SARP preparation process.

10 CFR 71	Packaging and Transportation of Radioactive Materials, U.S. Nuclear Regulatory Commission regulation.

49 CFR, Parts 100-185	Transportation, U.S. Department of Transportation regulation.

10 CFR 20	Standards for Protection Against Radiation, U.S. Nuclear Regulatory Commission regulation.

DOE O 461.1A	Packaging and Transfer or Transportation of Materials of National Security Interest, U.S. Department of Energy order.

RG 7.6	Design Criteria for the Structural Analysis of Shipping Cask Containment Vessels, U.S. Nuclear Regulatory Commission regulatory guide.

RG 7.8	Load Combinations for the Structural Analysis of Shipping Casks, U.S. Nuclear Regulatory Commission regulatory guide.

RG 7.9, Rev. 2	Standard Format and Content of Part 71 Applications for Approval of Packages for Radioactive Material, U.S. Nuclear Regulatory Commission regulatory guide.

RG 7.10	Establishing Quality Assurance Programs for Packaging Used in Transport of Radioactive Material, U.S. Nuclear Regulatory Commission regulatory guide.

RG 7.11	Fracture Toughness Criteria of Base Material for Ferritic Steel Shipping Cask Containment Vessels with a Maximum Wall Thickness of 4 inches (0.1 m), U.S. Nuclear Regulatory Commission regulatory guide.

SG 100	Design and Development Guide for NNSA Type B Packages, U.S. Department of Energy safety guide.

SG 140.1	Combination Test/Analysis Method Used to Demonstrate Compliance to DOE Type B Packaging Thermal Test Requirements, U.S. Department of Energy safety guide.

SG 500	Defense Programs Package Certification and Offsite Transportation Authorization Guide, U.S. Department of Energy safety guide.

SG 600	Design Guide for Testing Type B Packaging, U.S. Department of Energy safety guide.

ANSI N14.5	Leakage Tests on Packages for Shipment, American National Standard for Radioactive Materials.

ASTM E 2230-02	Thermal Qualification of Type B Packages for Radioactive Material, American Society of Testing and Materials.

NUREG/CR-1815	Recommendations for Protecting Against Failure by Brittle Fracture in Ferritic Steel Shipping Containers up to Four Inches Thick (note that this is the basis for RG 7.11).

NUREG/CR-3019	Recommended Welding Criteria for Use in the Fabrication of Shipping Containers for Radioactive Materials.

NUREG/CR-3854	Fabrication Criteria for Shipping Containers.

NUREG/CR-4775	Guide for Preparing Operating Procedures for Shipping Packages.

NUREG/CR-6007		Stress Analysis of Closure Bolts for Shipping Casks.

ANSI/ANS-6.1.1	Neutron and Gamma Ray Flux-to-Dose-Rate Factors, American National Standard.

NNSA QC-1	NNSA Quality Criteria (QC-1).

APPENDIX

SARP COMPLETENESS REVIEW CHECKLIST

NNSA

Type B Package Certification Program

TABLE OF CONTENTS - APPENDIX

Chapter	Page

1	General Information	1-1

2	Structural Evaluation	2-1

3	Thermal Evaluation	3-1

4	Containment 	4-1

5	Shielding Evaluation	5-1

6	Criticality Evaluation	6-1

7	Package Operations	7-1

8	Acceptance Tests and Maintenance Program	8-1

9	Quality Assurance	9-1

| Chapter 1 – General Information Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| Section | Requirement | Requirement Basis | - Requirement SARP Location | Satisfactory? Y/N/NA | Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| 1.0 | General Information This chapter should present an introduction and a general description of the package. | RG 7.9, 1. |  |  |  |
| 1.1 | Introduction This section should identify the following: Proposed use(s) of the package. | - RG 7.9, 1.1 |  |  |  |
|  | - 10 CFR 71.33(a)(3): “Model number.” | - 10 CFR 71.33(a)(3) - RG 7.9, 1.1 |  |  |  |
|  | - 10 CFR 71.33(a)(1): “Classification as Type B(U), B(M), or fissile material packaging.” | - 10 CFR 71.33(a)(1) |  |  |  |
|  | - In the case of fissile packages, the proposed Criticality Safety Index (CSI). | - RG 7.9, 1.1 |  |  |  |
| - 1.2 1.2.1 | Package Description Packaging The general packaging description should include the following information: | - RG 7.9, 1.2 - 10 CFR 71.33(a)(2) - RG 7.9, 1.2.1 |  |  |  |
|  | - Overall dimensions, minimum (empty) weight (if appropriate), and “gross weight.” (10 CFR 71.33(a)(2)). |  |  |  |  |
|  | - 10 CFR 71.33(a)(4): “Identification of the containment system.” This should include the containment features. | - 10 CFR 71.33(a)(4) - RG 7.9, 1.2.1 |  |  |  |
|  | - 10 CFR 71.33(a)(5): “Specific materials of construction, weights, dimensions, and fabrication methods of— | - 10 CFR 71.33(a)(5) - RG 7.9, 1.2.1 |  |  |  |
|  | (i) Receptacles; |  |  |  |  |
|  | (ii) Materials specifically used as nonfissile neutron absorbers or moderators; |  |  |  |  |
|  | (iii) Internal and external structures supporting or protecting receptacles; |  |  |  |  |
|  | (iv) “Valves, sampling ports, lifting devices, and tie-down devices; and |  |  |  |  |
|  | (v) Structural and mechanical means for the transfer and dissipation of heat.” (i.e., heat transfer features) |  |  |  |  |
|  | This section should also describe the following, if applicable: |  |  |  |  |
|  | - Other criticality control features such as spacers, etc. |  |  |  |  |
|  | - Structural features such as impact limiters or other energy absorbing features, internal supporting or positioning features, outer shell or outer packaging, and packaging closure devices. |  |  |  |  |
|  | - 10 CFR 71.33(a)(6): “Identification and volumes of any receptacles containing coolant.” | - 10 CFR 71.33(a)(6) |  |  |  |
|  | - Neutron and gamma shielding features, including personnel barriers. | - RG 7.9, 1.2.1 |  |  |  |
|  | - Packaging markings. | - RG 7.9, 1.2.1 |  |  |  |
| - 1.2.2 | - Contents - This section should state the quantity of radionuclides to be transported. The description should include the following, if appropriate: | - 10 CFR 71.33(b)(1) - RG 7.9, 1.2.2 |  |  |  |
|  | - 10 CFR 71.33(b)(1): “Identification and maximum radioactivity of radioactive constituents.” [RG 7.9, Section 1.2.2: identification and maximum quantity (radioactivity or mass) of the radioactive material.] |  |  |  |  |
|  | - 10 CFR 71.33(b)(2): “Identification and maximum quantities of fissile constituents.” | - 10 CFR 71.33(b)(2) - RG 7.9, 1.2.2 |  |  |  |
|  | - 10 CFR 71.33(b)(3): “Chemical and physical form.” This should also include the density and moisture content, and the presence of any moderating constituents. | - 10 CFR 71.33(b)(3) - RG 7.9, 1.2.2 |  |  |  |
|  | - 10 CFR 71.33(b)(4): “Extent of reflection, the amount and identity of nonfissile materials used as neutron absorbers or moderators, and the atomic ratio of moderator to fissile constituents.” | - 10 CFR 71.33(b)(4) - RG 7.9, 1.2.2 |  |  |  |
|  | - 10 CFR 71.33(b)(5)): “Maximum normal operating pressure.” | - 10 CFR 71.33(b)(5) |  |  |  |
|  | - 10 CFR 71.33(b)(6): “Maximum weight.” In addition to the maximum weight of the radioactive contents, this section should also include the maximum weight of the payload, including secondary containers and packaging (wrapping, shoring, and other material not defined as part of the packaging), if applicable. | - 10 CFR 71.33(b)(6) - RG 7.9, 1.2.2 |  |  |  |
|  | - 10 CFR 71.33(b)(7): “Maximum amount of decay heat.” | - 10 CFR 71.33(b)(7) - RG 7.9, 1.2.2 |  |  |  |
|  | - 10 CFR 71.33(b)(8): “Identification and volumes of any coolants.” | - 10 CFR 71.33(b)(8) |  |  |  |
|  | - Location and configuration of contents within the packaging, including secondary containers, wrapping, shoring, and other material not defined as part of the packaging. | - RG 7.9, 1.2.2 |  |  |  |
|  | - Any material subject to chemical, galvanic, or other reaction, including the generation of gases. | - RG 7.9, 1.2.2 |  |  |  |
|  | - Any loading restrictions. | - RG 7.9, 1.2.2 |  |  |  |
|  | - Include a description of the contents that is suitable for inclusion into the Offsite Transportation Certificate (OTC), including the type and form of material and maximum quantity of material per package. - Note: The information that this RG 7.9 section requires should be contained in other SARP sections. Therefore, for NNSA submissions, the applicants have the option of repeating the information in this section or simply referring to the SARP sections that contain the information in the Requirement SARP Location box in this row. In the case of packages whose proposed contents include a canned subassembly (CSA) or CSAs this section must include the part number(s) for the proposed CSA(s). | - RG 7.9, 1.2.2 |  |  |  |
| - 1.2.3 | - Special Requirements for Plutonium - 10 CFR 71.63: “Shipments containing plutonium must be made with the contents in solid form, if the contents contain greater than 0.74 TBq (20 Ci) of plutonium.” - This section should show that plutonium contents in quantities greater than 0.74 TBq (20 Ci) must be in solid form. | - 10 CFR 71.63 - RG 7.9, 1.2.3 |  |  |  |
| - 1.2.4 | - Operational Features - In the case of complex package systems, this section should describe the operational features of the package, and include a schematic diagram showing all valves, connections, piping, openings, seals, containment boundaries, etc. | - RG 7.9, 1.2.4 |  |  |  |
| - 1.3 | - Appendix - Packages authorized for shipment must conform to the approved design; that is, each package must be fabricated in conformance to the engineering drawings. - The appendix should include engineering drawings that: | - RG 7.9, 1.3 |  |  |  |
|  | - Clearly detail the safety features considered in the package evaluation, and include a materials list, dimensions, valves, fasteners, and welder and welding procedure qualification requirements. - Note: Detailed construction drawings do not have to be submitted. |  |  |  |  |
|  | - Specify, by appropriate weld symbol, the specifications for all packaging weld joints, including the nondestructive examination (NDE) method and the acceptance standard. | - RG 7.9, 1.3 |  |  |  |
|  | - Include containment system gasketed joints in sufficient detail to, as a minimum, show the surface finish and flatness requirements of the closure surfaces, the gasket or O-ring specification and, if appropriate, the method of gasket or O-ring retention. | - RG 7.9, 1.3 |  |  |  |
|  | - Include a list of references, a copy of any reference material not generally available, supporting information on special fabrication procedures, determination of package category, and other appropriate supplemental information. | - RG 7.9, 1.3 |  |  |  |

| Chapter 2 – Structural Evaluation Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| - Section | - Requirement | - Requirement Basis | - Requirement SARP Location | - Satisfactory ? Y/N/NA | - Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| - 2.0 | - Structural Evaluation - This chapter should identify, describe, discuss, and analyze the principal structural design of the packaging, components, and systems important to safety. This chapter should also describe how the package complies with the performance requirements of 10 CFR 71. | - RG 7.9, 2. |  |  |  |
| - 2.1 - 2.1.1 | - Description of Structural Design - Discussion - This section should identify the principal structural members and systems (such as the containment vessel, impact limiters, radiation shielding, closure devices, and ports) that are important to the safe operation of the package. The discussion should reference the locations of these items on the drawings and discuss their structural design and performance. | - RG 7.9, 2.1 - RG 7.9, 2.1.1 |  |  |  |
| - 2.1.2 | - Design Criteria - This section should: - Describe and reference load combinations and factors that serve as design criteria. | - RG 7.9, 2.1.2 |  |  |  |
|  | - State the maximum allowable stresses and strains (as a percentage of the yield or ultimate values for ductile failure) for each criterion. | - RG 7.9, 2.1.2 |  |  |  |
|  | - Describe how the other structural failure modes (e.g., brittle fracture, fatigue, buckling) are considered. | - RG 7.9, 2.1.2 |  |  |  |
|  | - Identify the criteria used for the impact evaluation. | - RG 7.9, 2.1.2 |  |  |  |
|  | - Identify codes and standards that are used to determine material properties, design limits, or methods of combining loads and stresses. | - RG 7.9, 2.1.2 |  |  |  |
|  | - Provide the appropriate value of each criterion used in each case when different design criteria are used in various parts of the packaging or for different conditions. | - RG 7.9, 2.1.2 |  |  |  |
|  | - Provide a detailed description of the design criteria used as substitutes anytime the design criteria deviate from those specified by standard codes, or if such code does not cover certain components. | - RG 7.9, 2.1.2 |  |  |  |
|  | - If different design criteria are allowed in various parts of the packaging or for different conditions, this section should indicate the appropriate values for each case. | - RG 7.9, 2.1.2 |  |  |  |
| - 2.1.3 | - Weights and Centers of Gravity - This section should: - Identify the location of the package’s center of gravity and any other centers of gravity referred to in the SARP. | - RG 7.9, 2.1.3 |  |  |  |
|  | - List the total weight of the packaging and contents, and tabulate the weights of major individual subassemblies such that the sum of the parts equals the total of the package. | - RG 7.9, 2.1.3 |  |  |  |
|  | - Include a sketch or drawing that clearly shows the individual subassembly referred to and the reference point for locating its center of gravity. | - RG 7.9, 2.1.3 |  |  |  |
| - 2.1.4 | - Identification of Codes and Standards for Package Design - The section should: - Identify established codes and standards proposed for use in package design, fabrication, assembly, testing, maintenance, and use. | - RG 7.9, 2.1.4 |  |  |  |
|  | - Include an assessment of the applicability of the codes and standards. | - RG 7.9, 2.1.4 |  |  |  |
|  | - Show that the proposed codes or standards are appropriate for the package category. The package categories are identified in RG 7.9, Table 2-1 (from RG 7.11), NUREG/CR-3854, and SG 100. The structural design criteria based on the ASME BPVC should be addressed; i.e., the appropriate ASME BPVC section should be selected based on the number of A2s of the proposed contents. Note that any other codes and/or standards can be used as design criteria provided that they can be justified to be as conservative as the ASME BPVC. Note: Justification for deviations from the ASME BPVC must be extremely comprehensive and detailed. Therefore, compliance with the ASME BPVC is strongly recommended. | - RG 7.9, 2.1.4 - RG 7.11 - NUREG/CR-3854 - SG 100 |  |  |  |
| - 2.2 - 2.2.1 | - Materials - Material Properties and Specifications - This section should: - Identify the source(s) of all information by publication and page number. | - RG 7.9, 2.2 - RG 7.9, 2.2.1 |  |  |  |
|  | - List the material mechanical properties used in the structural evaluation. These may include yield stress, ultimate stress, modulus of elasticity, ultimate strain, Poisson’s ratio, density, and coefficient of thermal expansion. | - RG 7.9, 2.2.1 |  |  |  |
|  | - If impact limiters are used, include either a compression stress-strain curve for the material or the force-deformation relationship for the impact limiter, as appropriate. | - RG 7.9, 2.2.1 |  |  |  |
|  | - For materials that are subjected to elevated temperatures, specify the appropriate mechanical properties under those conditions. | - RG 7.9, 2.2.1 |  |  |  |
|  | - Where material properties are determined by testing, describe the test procedures, conditions, and measurements in sufficient detail to enable the reviewer to evaluate the validity of the results. | - RG 7.9, 2.2.1 |  |  |  |
|  | - This section should also document the results of the assessment of the behavior of packaging materials whose primary function is to provide thermal insulation and/or impact energy absorption. These materials must be assessed under the following conditions: | - SG-100, 2.5.5, 2.7, and 3.5 |  |  |  |
|  | - The material must be evaluated for NCT compliance under low temperature (-40 ºF) conditions; |  |  |  |  |
|  | - The effect of moisture content on crush strength must be evaluated or a reasoned argument as to why the moisture content of the material cannot change from those conditions already considered must be provided; and |  |  |  |  |
|  | - The effect of high temperature on the insulating material’s performance should be addressed since some materials may start to decompose at a particular temperature. - Note: The impact absorbers are not required to maintain their energy absorbing properties after exposure to the thermal test specified in 10 CFR 71.73(c)(4). |  |  |  |  |
| - 2.2.2 | - Chemical, Galvanic, or Other Reactions - 10 CFR 71.43(d): “A package must be made of materials and construction that assure that there will be no significant chemical, galvanic, or other reaction among the packaging components, among package contents, or between the packaging components and the package contents, including possible reaction resulting from inleakage of water, to the maximum credible extent. Account must be taken of the behavior of materials under irradiation.” | - 10 CFR 71.43(d) - RG 7.9, 2.2.2 |  |  |  |
|  | - This section should: |  |  |  |  |
|  | - Describe the possible chemical, galvanic, or other reactions among the packaging components, among package contents, or between the packaging components and the package contents, including possible reaction resulting from inleakage of water, to the maximum extent. |  |  |  |  |
|  | - For each component material (material of construction) of the packaging, all chemically or galvanically dissimilar materials in contact with the component material (material of construction) should be listed. | - RG 7.9, 2.2.2 |  |  |  |
|  | - Describe methods used to prevent significant reactions. Include coatings used on internal or external package surfaces. | - RG 7.9, 2.2.2 |  |  |  |
|  | - Include the possible generation of hydrogen or other gases from chemical or radiolytic interactions. Also consider galvanic interactions and the formation of a eutectic for components that are, or may be, in physical contact. | - RG 7.9, 2.2.2 |  |  |  |
| - 2.2.3 | - Effects of Radiation on Materials - This section should account for the behavior of materials under irradiation. It should describe any aging or damaging effects of radiation on the packaging materials. These effects may include degradation of seals, sealing materials, coatings, adhesives, and structural materials. | - 10 CFR 71.43(d) - RG 7.9, 2.2.3 |  |  |  |
| - 2.3 - 2.3.1 | - Fabrication and Examination - Fabrication - This section should describe the fabrication processes used for the package, such as fitting, aligning, welding and brazing, heat treatment, and foam and lead pouring. | - RG 7.9, 2.3 - RG 7.9, 2.3.1 |  |  |  |
|  | - For fabrication specifications prescribed by an acceptable code or standard (e.g., those promulgated by the ASME or the AWS), the code or standard should be clearly specified on the engineering drawings. Unless the SARP justifies otherwise, specifications of the same code or standard used for design should be used for fabrication. | - RG 7.9, 2.3.1 |  |  |  |
|  | - For components for which no code or standard is applicable, the SARP should identify the specifications on which the evaluation depends, and describe the method of control to ensure that these specifications are achieved. This description may reference QA or other appropriate specification documents, which should also be identified on the engineering drawings. | - RG 7.9, 2.3.1 |  |  |  |
| - 2.3.2 | - Examination - This section should describe the methods and criteria by which the fabrication is determined to be acceptable. Unless the SARP justifies otherwise, specifications of the same code or standard used for fabrication should also be used for examination. | - RG 7.9, 2.3.2 |  |  |  |
|  | - For components that have no applicable fabrication code or standard, this section should summarize the examination methods and acceptance criteria contained in Chapter 8 (Acceptance Tests and Maintenance Program). |  |  |  |  |
| - 2.4 | - General Requirements for All Packages - Note: Only the general requirements of minimum package size, tamper-indicating feature, and positive closure are included in this section. The requirements of §71.43(d) (chemical and galvanic reactions) are addressed in Section 2.2.2 of this checklist; the requirements of §71.43(e) (package valve or other device) are addressed in Section 4.1, bullet 3 of this checklist; and the requirements of §71.43(h) (venting during transport) are addressed in Section 4.1, bullet 5 of this checklist. | - RG 7.9, 2.4 - 10 CFR 71.43(a) - RG 7.9, 2.4.1 |  |  |  |
| - 2.4.1 | - Minimum Package Size - 10 CFR 71.43(a): “The smallest overall dimension of a package may not be less than 10 cm (4 in).” - The overall dimensions of the package should be discussed and shown to meet §71.43(a) requirements. |  |  |  |  |
| - 2.4.2 | - Tamper-Indicating Feature - 10 CFR 71.43(b): “The outside of the package must incorporate a feature, such as a seal, that is not readily breakable and that, while intact, would be evidence that the package has not been opened by unauthorized persons.” | - 10 CFR 71.43(b) - RG 7.9, 2.4.2 |  |  |  |
|  | - This section should describe the package closure system in sufficient detail to show that it meets the §71.43(b) requirements. The description should include covers, ports, or other access that must be closed during normal transportation. Tamper indicators and their locations should be described. |  |  |  |  |
| - 2.4.3 | - Positive Closure - 10 CFR 71.43(c): “Each package must include a containment system securely closed by a positive fastening device that cannot be opened unintentionally or by a pressure that may arise within the package.” | - 10 CFR 71.43(c) - RG 7.9, 2.4.3 |  |  |  |
|  | - This section should describe the package closure system in sufficient detail to show that it cannot be inadvertently opened or opened by a pressure that may arise within the package. This description should include covers, valves, or any other access that must be closed during normal transportation. |  |  |  |  |
| - 2.5 - 2.5.1 | - Lifting and Tiedown Standards for All Packages - Lifting Devices - 10 CFR 71.45(a): “Any lifting attachment that is a structural part of a package must be designed with a minimum safety factor of three against yielding when used to lift the package in the intended manner, and it must be designed so that failure of any lifting device under excessive load would not impair the ability of the package to meet other requirements of” 10 CFR 71 Subpart E. “Any other structural part of the package that could be used to lift the package must be capable of being rendered inoperable for lifting the package during transport, or must be designed with strength equivalent to that required for lifting attachments.” | - RG 7.9, 2.5 - 10 CFR 71.45(a) - RG 7.9, 2.5.1 |  |  |  |
|  | - This section should: |  |  |  |  |
|  | - Identify all devices and attachments that could be used to lift the package or its lid. |  |  |  |  |
|  | - Show by testing or analysis that the devices comply with §71.45(a) requirements. | - RG 7.9, 2.5.1 |  |  |  |
|  | - Include drawings or sketches that show the location and construction of these devices. | - RG 7.9, 2.5.1 |  |  |  |
|  | - Show the effects of forces imposed by the lifting devices on other packaging surfaces. | - RG 7.9, 2.5.1 |  |  |  |
|  | - Show that the documented values of the yield stresses of the materials were used as the criteria to demonstrate compliance with §71.45(a), including failure under excessive load. | - RG 7.9, 2.5.1 |  |  |  |
| - 2.5.2 | - Tiedown Devices | - 10 CFR 71.45(b) - RG 7.9, 2.5.2 |  |  |  |
|  | - 10 CFR 71.45(b)(1): “If there is a system of tie-down devices that is a structural part of the package, the system must be capable of withstanding, without generating stress in any material of the package in excess of its yield strength, a static force applied to the center of gravity of the package having a vertical component of 2 times the weight of the package with its contents, a horizontal component along the direction in which the vehicle travels of 10 times the weight of the package with its contents, and a horizontal component in the transverse direction of 5 times the weight of the package with its contents.” |  |  |  |  |
|  | - 10 CFR 71.45(b)(2): “Any other structural part of the package that could be used to tie down the package must be capable of being rendered inoperable for tying down the package during transport, or must be designed with strength equivalent to that required for tie-down devices.” |  |  |  |  |
|  | - 10 CFR 71.45(b)(3): “Each tie-down device that is a structural part of the package must be designed so that failure of the device under excessive load would not impair the ability of the package to meet other requirements of” 10 CFR 71. - This section should: |  |  |  |  |
|  | - Describe the overall tiedown system for the package. This should also include a discussion on how any other structural part of the package that is not intended for tiedown purposes but could be used to tie down the package is rendered inoperable for tying down the package during transport. |  |  |  |  |
|  | - Identify any device that is a structural part of the package and can be used for tiedown. | - RG 7.9, 2.5.2 |  |  |  |
|  | - Discuss the testing or analysis that show these devices comply with the requirements of §71.45(b). | - RG 7.9, 2.5.2 |  |  |  |
|  | - Show the effect of imposed forces of vital package components, including the interfaces between the tiedown devices and other packaging surfaces. | - RG 7.9, 2.5.2 |  |  |  |
|  | - Show that documented values of the yield stresses of the materials were used as the criteria for demonstrating compliance with §71.45(b), including failure under excessive load. | - RG 7.9, 2.5.2 |  |  |  |
|  | - Provide drawings or sketches that show the location and construction of the overall tiedown system and individual devices. Note: The development of the tiedown procedures used to secure the package to the transportation conveyance(s) should have started early in the package development process. The DOE/NNSA Office of Secure Transportation (NA-15) is responsible for the development and approval of tiedown procedures used in the Transportation Safeguards System. Therefore, package developers should provide NA-15 the package information required to support this process early in the package development phase. | - RG 7.9, 2.5.2 |  |  |  |
| - 2.6 | - Normal Conditions of Transport 10 CFR 71.71(a): “Evaluation. Evaluation of each package design under normal conditions of transport must include a determination of the effect on that design of the conditions and tests specified in” 10 CFR 71.71. “Separate specimens may be used for the free drop test, the compression test, and the penetration test, if each specimen is subjected to the water spray test before being subjected to any of the other tests.” | - 10 CFR 71.71 - RG 7.9, 2.6 |  |  |  |
|  | - 10 CFR 71.71(b): “Initial conditions. With respect to the initial conditions for the tests in” 10 CFR 71.71, “the demonstration of compliance with the requirements of” 10 CFR 71 “must be based on the ambient temperature preceding and following the tests remaining constant at that value between -29 C (-20 F) and +38 C (+100 F) which is most unfavorable for the feature under consideration. The initial internal pressure within the containment system must be considered to be the maximum normal operating pressure, unless a lower internal pressure consistent with the ambient temperature considered to precede and follow the tests is more unfavorable.” |  |  |  |  |
|  | - This section should describe the evaluation that shows the package meets the standards specified in §71.43 (General requirements for all packages) and §71.51 (Additional requirements for Type B packages) when it is subjected to the tests and conditions specified in §71.71 (Normal conditions of transport). |  |  |  |  |
|  | The documentation must show that the package was evaluated against each condition specified in §71.71 separately, and that the package satisfies the applicable performance requirements specified in the regulations. Note: The requirements of 10 CFR 71.43 and 10 CFR 71.51 are quoted in Sections 2.4 and 2.7 of this checklist. |  |  |  |  |
|  | - Structural evaluation of the package under NCT may be performed by analysis or test or a combination of both. This section should clearly show that the most limiting initial test conditions and most damaging orientations have been considered, and that the evaluation methods are appropriate and properly applied. | - RG 7.9, 2.6 |  |  |  |
|  | - The following general information should be considered and included, as appropriate: |  |  |  |  |
|  | - For evaluation by test, the test method, procedures, equipment, and facilities that were used should be described. | - RG 7.9, 2.6 |  |  |  |
|  | - Package orientations that were evaluated for the tests should be clearly identified and justified as being the most damaging, if applicable. | - RG 7.9, 2.6 |  |  |  |
|  | - If the package tested was not identical in all respects to the package described in the SARP, the differences should be identified and a justification given to show that the differences will not affect the test results. | - RG 7.9, 2.6 |  |  |  |
|  | - Materials used as substitutes for the radioactive contents during the tests should be described and a justification that shows that this substitution would not affect the results should be provided. The justification should include an assessment of the effects of internal decay heat and pressure buildup, if appropriate. | - RG 7.9, 2.6 |  |  |  |
|  | - A detailed and quantitative description of the damage caused by the tests should be provided, along with the results of any measurements that were made, including both interior and exterior damage, as well as photographs of the damaged packaging. | - RG 7.9, 2.6 |  |  |  |
|  | - For prototype and model testing, this section should provide a complete description of the test specimen, including detailed drawings that show its dimensions and materials of construction and dimensional tolerances to which the prototype or model was fabricated. Fabrication tolerances of the test specimen should be compared to those that will be used for the package. | - RG 7.9, 2.6 |  |  |  |
|  | - For scale models, this section should identify the scale factor that was used, and provide a detailed description of the laws of similitude that were used for testing, considering time scale, material density, velocity at impact, and kinetic energy. Information should be provided to show that the model test would give conservative results for peak g-force, maximum deformation, and dissipated energy. In addition, the damage done to the model should be correlated to damage that could occur to the package had the package undergone the same test(s). | - RG 7.9, 2.6 |  |  |  |
|  | - For evaluation by analysis, this section should describe the methods and calculations used in the package evaluation in sufficient detail to enable the reviewer to verify the results. This section should clearly describe and justify all assumptions used in the analysis, and include adequate narration, sketches, and free body force diagrams. In addition, for equations that were used in the analysis, this section should either cite the source or include the derivation. | - RG 7.9, 2.6 |  |  |  |
|  | - Computer programs should be identified and described, and should be shown to be well benchmarked, widely used for structural analysis, and applicable to the evaluation. | - RG 7.9, 2.6 |  |  |  |
|  | - Computer models and related details should be well described and justified. For example, the number of discrete finite elements used in the model should reflect the type of analysis performed and should be appropriate considering such factors as stress or displacement. | - RG 7.9, 2.6 |  |  |  |
|  | - Sensitivity studies used to determine the appropriate number of nodes or elements for a particular model should be provided. | - RG 7.9, 2.6 |  |  |  |
|  | - A detailed description of the modeling of the bolted connections, including element types, modeling technique, and material properties should be included. | - RG 7.9, 2.6 |  |  |  |
|  | - For impact analysis, information should be provided that shows how all of the kinetic energy would be dissipated, and what local deformation and dynamic forces would occur during impact, the package response in terms of stress and strain to components and structural members, the structural stability of individual members, stresses attributable to impact combined with those stresses caused by temperature gradients, differential thermal expansions, pressure, and other loads. | - RG 7.9, 2.6 |  |  |  |
|  | - Analytical results should be directly compared with the acceptance criteria. | - RG 7.9, 2.6 |  |  |  |
|  | - An assessment that shows that the NCT do not reduce the effectiveness of the packaging should be included. 10 CFR 71.43(f): “A package must be designed, constructed, and prepared for shipment so that under the tests specified in §71.71 (“Normal conditions of transport”) there would be no loss or dispersal of radioactive contents, no significant increase in external surface radiation levels, and no substantial reduction in the effectiveness of the packaging.” | - 10 CFR 71.43(f) - 10 CFR 71.51(a)(1) - RG 7.9, 2.6 |  |  |  |
|  | 10 CFR 71.51(a)(1): “Section 71.71 (“Normal conditions of transport”), there would be no loss or dispersal of radioactive contents—as demonstrated to a sensitivity of 10-6 A2 per hour, no significant increase in external surface radiation levels, and no substantial reduction in the effectiveness of the packaging.” |  |  |  |  |
|  | - 10 CFR 71.51(c): “Compliance with the permitted activity release limits of” 10 CFR 71.51(a) “may not depend on filters or on a mechanical cooling system.” This section should discuss how the package meets the requirements specified in §71.51(c). | - 10 CFR 71.51(c) |  |  |  |
| - 2.6.1 | - 10 CFR 71.71(c): “Conditions and tests.” Heat 10 CFR 71.71(c)(1): “Heat. An ambient temperature of 38 C (100 F) in still air, and insolation according to the” Insolation Data table in 10 CFR 71.71(c)(1). | - 10 CFR 71.71(c) - 10 CFR 71.71(c)(1) - RG 7.9, 2.6.1 |  |  |  |
|  | The thermal evaluation for the heat test should be described and reported in Chapter 3 (Thermal Evaluation) of the SARP. The results of the Thermal Evaluation should be used as input to SARP Section 2.6.1.1 through Section 2.6.1.4, and this section should include a discussion to show that the package meets the §71.71(c)(1) requirements. |  |  |  |  |
| - 2.6.1.1 | - Summary of Pressures and Temperatures This section should summarize all of the temperatures and pressures, as determined in the Thermal Evaluation (Chapter 3.0), that will be used to perform calculations in SARP Section 2.6.1.2 through Section 2.6.1.4. | - RG 7.9, 2.6.1.1 |  |  |  |
| - 2.6.1.2 | - Differential Thermal Expansion This section should present calculations of the circumferential and axial deformations and stresses (if any) that result from differential thermal expansion. Steady state and transient conditions should be considered. The calculations should be sufficiently comprehensive to demonstrate package integrity under NCT. | - RG 7.9, 2.6.1.2 |  |  |  |
| - 2.6.1.3 | - Stress Calculations This section should present calculations of the stresses that are attributable to the combined effects of thermal gradients, pressure, and mechanical loads (including fabrication stresses from lead pour and lead cooldown, if applicable). | - RG 7.9, 2.6.1.3 |  |  |  |
|  | - This section should provide sketches that show the configuration and dimensions of the members or systems being analyzed, and the points at which stresses are calculated. | - RG 7.9, 2.6.1.3 |  |  |  |
|  | - The analysis should consider whether repeated cycles of thermal loadings, together with other loadings, would cause fatigue failure or extensive accumulations of deformation. | - RG 7.9, 2.6.1.3 |  |  |  |
| - 2.6.1.4 | - Comparison with Allowable Stresses This section should present the appropriate stress combinations and compare the resulting stresses with the design criteria specified in the SARP. | - RG 7.9, 2.6.1.4 |  |  |  |
|  | - This section should show that all the relevant performance requirements have been satisfied as specified in the regulations. | - RG 7.9, 2.6.1.4 |  |  |  |
| - 2.6.2 | - Cold 10 CFR 71.71(c)(2): “Cold. An ambient temperature of -40 C (-40 F) in still air and shade.” | - 10 CFR 71.71(c)(2) - RG 7.9, 2.6.2 |  |  |  |
|  | The thermal evaluation for the cold test should be described and reported in Chapter 3 (Thermal Evaluation) of the SARP. |  |  |  |  |
|  | This section should include a discussion to show that the package meets the §71.71(c)(2) requirements. |  |  |  |  |
|  | - Using the results from SARP Chapter 3 (Thermal Evaluation), this section should assess the effects that the cold condition has on the package, including material properties and possible liquid freezing and lead shrinkage. The resulting temperatures and their effects on package components and operation of the package should be reported. | - RG 7.9, 2.6.2 |  |  |  |
|  | - Stresses should be within the limits for normal condition loads. | - RG 7.9, 2.6.2 |  |  |  |
|  | - For the sequential hypothetical accident test series, -29 C (-20 F) is the lowest service temperature that needs to be considered as specified in §71.73(b). | - RG 7.9, 2.6.2 |  |  |  |
|  | - Brittle fracture should be evaluated. | - RG 7.9, 2.6.2 |  |  |  |
| - 2.6.3 | - Reduced External Pressure 10 CFR 71.71(c)(3): “Reduced external pressure. An external pressure of 25 kPa (3.5 lbf/in2) absolute.” This section should describe the evaluation of the package and show that it meets the requirements for the effects of reduced external pressure as described in §71.71(c)(3). | - 10 CFR 71.71(c)(3) - RG 7.9, 2.6.3 |  |  |  |
|  | - The evaluation should include the greatest pressure difference between the inside and outside of the package, as well as the inside and outside of the containment system, and evaluate this condition in combination with the maximum normal operating pressure. | - RG 7.9, 2.6.3 |  |  |  |
| - 2.6.4 | - Increased External Pressure 10 CFR 71.71(c)(4): “Increased external pressure. An external pressure of 140 kPa (20 lbf/in2) absolute.” This section should describe the evaluation of the package and show that it meets the requirements for the effects of increased external pressure as specified in §71.71(c)(4). | - 10 CFR 71.71(c)(4) - RG 7.9, 2.6.4 |  |  |  |
|  | - The evaluation should include the greatest pressure difference between the inside and outside of the package, as well as the inside and outside of the containment system, and evaluate this condition in combination with the minimum internal pressure. | - RG 7.9, 2.6.4 |  |  |  |
|  | - A buckling evaluation should be included. | - RG 7.9, 2.6.4 |  |  |  |
| - 2.6.5 | - Vibration 10 CFR 71.71(c)(5): “Vibration. Vibration normally incident to transport.” This section should describe the evaluation of the package and show that it meets the requirements for the effects of vibrations that are normally incident to transport. The combined stresses attributable to vibration, temperature, and pressure loads should be considered and a fatigue analysis should be included, if applicable. | - 10 CFR 71.71(c)(5) - RG 7.9, 2.6.5 |  |  |  |
|  | - If closure bolts are reused, the bolt preload should be considered in the fatigue evaluation. | - RG 7.9, 2.6.5 |  |  |  |
|  | - Packaging components, including internals, should be evaluated for resonant vibration conditions that could cause rapid fatigue damage. | - RG 7.9, 2.6.5 |  |  |  |
| - 2.6.6 | - Water Spray 10 CFR 71.71(c)(6): “Water spray. A water spray that simulates exposure to rainfall of approximately 5 cm/h (2 in/h) for at least 1 hour.” The effects of the water spray test should be assessed. This section should show that the water spray test has no significant effect on the package and that it meets the requirements of §71.71(c)(6). | - 10 CFR 71.71(c)(6) - RG 7.9, 2.6.6 |  |  |  |
| - 2.6.7 | - Free Drop 10 CFR 71.71(c)(7): “Free drop. Between 1.5 and 2.5 hours after the conclusion of the water spray test, a free drop through the distance specified” in the Criteria for Free Drop Test (Weight/Distance) table of 10 CFR 71.71(c)(7) “onto a flat, essentially unyielding, horizontal surface, striking the surface in a position for which maximum damage is expected.” This section should describe the test performed to meet the requirements specified in §71.71(c)(7). This section should describe the package evaluation for the effects of a free drop. | - 10 CFR 71.71(c)(7) - RG 7.9, 2.6.7 |  |  |  |
|  | - This section should also address such factors as drop orientation, effects of free drop in combination with pressure, heat, and cold temperatures and other factors discussed in Section 2.6 (NCT). - Note: The general comments of Section 2.7.1 (HAC Free Drop) may also apply to this section. | - RG 7.9, 2.6.7 |  |  |  |
| - 2.6.8 | - Corner Drop 10 CFR 71.71(c)(8): “Corner drop. A free drop onto each corner of the package in succession, or in the case of a cylindrical package onto each quarter of each rim, from a height of 0.3 m (1 ft) onto a flat, essentially unyielding, horizontal surface. This test applies only to fiberboard, wood, or fissile material rectangular packages not exceeding 50 kg (110 lbs) and fiberboard, wood, or fissile material cylindrical packages not exceeding 100 kg (220 lbs).” If applicable, this section should describe the test performed and provide the assessment of the effects of the corner drop test on the package. | - 10 CFR 71.71(c)(8) - RG 7.9, 2.6.8 |  |  |  |
| - 2.6.9 | - Compression 10 CFR 71.71(c)(9): “Compression. For packages weighing up to 5000 kg (11,000 lbs), the package must be subjected, for a period of 24 hours, to a compressive load applied uniformly to the top and bottom of the package in the position in which the package would normally be transported. The compressive load must be the greater of the following: (i) The equivalent of 5 times the weight of the package; or (ii) The equivalent of 13 kPa (2 lbf/in2) multiplied by the vertically projected area of the package.” If applicable, this section should describe the test performed and provide an assessment of the effects of the test specified in §71.71(c)(9). | - 10 CFR 71.71(c)(9) - RG 7.9, 2.6.9 |  |  |  |
| - 2.6.10 | - Penetration 10 CFR 71.71(c)(10): “Penetration. Impact of the hemispherical end of a vertical steel cylinder of 3.2 cm (1.25 in) diameter and 6 kg (13 lbs) mass, dropped from a height of 1 m (40 in) onto the exposed surface of the package that is expected to be most vulnerable to puncture. The long axis of the cylinder must be perpendicular to the package surface.” This section should describe the effects of the penetration test on the package and identify the most vulnerable location on the package surface based on the test specified in §71.71(c)(10). | - 10 CFR 71.71(c)(10) - RG 7.9, 2.6.10 |  |  |  |
| - 2.7 | - Hypothetical Accident Conditions 10 CFR 71.73(a): “Test procedures. Evaluation for hypothetical accident conditions is to be based on sequential application of the tests specified in” 10 CFR 71.73, “in the order indicated, to determine their cumulative effect on a package or array of packages. An undamaged specimen may be used for the water immersion tests specified in” 10 CFR 71.73(c)(6). 10 CFR 71.73(b): “Test conditions. With respect to the initial conditions for the tests, except for the water immersion tests, to demonstrate compliance with the requirements of” 10 CFR 71 “during testing, the ambient air temperature before and after the tests must remain constant at that value between -29 C (-20 F) and +38 C (+100 F) which is most unfavorable for the feature under consideration. The initial internal pressure within the containment system must be the maximum normal operating pressure, unless a lower internal pressure, consistent with the ambient temperature assumed to precede and follow the tests, is more unfavorable.” This section should describe the evaluation that shows the package meets the standards specified in §71.51, §71.55(e), and §71.59(a)(2), when subjected to the tests specified in §71.73. 10 CFR 71.51: “Additional requirements for Type B packages. (a) A Type B package, in addition to satisfying the requirements of §§71.41 through 71.47, must be designed, constructed, and prepared for shipment so that under the tests specified in: - (1) Section 71.71 (“Normal conditions of transport”), there would be no loss or dispersal of radioactive contents—as demonstrated to a sensitivity of 10-6 A2 per hour, no significant increase in external surface radiation levels, and no substantial reduction in the effectiveness of the packaging; and - (2) Section 71.73 (“Hypothetical accident conditions”), there would be no escape of krypton-85 exceeding 10 A2 in 1 week, no escape of other radioactive material exceeding a total amount A2 in 1 week, and no external radiation dose rate exceeding 10 mSv/h (1 rem/h) at 1 m (40 in) from the external surface of the package. - (b) Where mixtures of different radionuclides are present, the provisions of appendix A, paragraph IV of” 10 CFR 71 “shall apply, except that for Krypton-85, an effective A2 value equal to 10 A2 may be used. - (c) Compliance with the permitted activity release limits of paragraph (a) of this section may not depend on filters or on a mechanical cooling system. - (d) For packages which contain radioactive contents with activity greater than 105 A2, the requirements of §71.61 must be met.” Note: The requirements of 10 CFR 71.55(e) and 10 CFR 71.59(a)(2) are quoted in Section 6.0 of this checklist. This section should document the structural evaluation performed to determine the cumulative effect of the HAC tests, performed in the specified sequence, on the package. Damage caused by each test is cumulative, and an evaluation of the package’s ability to withstand any one test must consider the damage that resulted from the previous tests. | - 10 CFR 71.73(a) - 10 CFR 71.73(b) - 10 CFR 71.51 - 10 CFR 71.55(e) - 10 CFR 71.59(a)(2) - RG 7.9, 2.7 |  |  |  |
|  | - This section should confirm that the package effectiveness has not been reduced as a result of the NCT as indicated in SARP Section 2.6. This section should include the applicable information regarding tests and analyses, as described in Section 2.6 of this checklist. | - RG 7.9, 2.7 |  |  |  |
|  | - Brittle fracture should be considered. | - RG 7.9, 2.7 |  |  |  |
|  | - Unlike the previous version of RG 7.9, RG 7.9 Revision 2 dated March 2005, does not provide an option for assessing a package’s performance by comparison to a similar currently certified package. Nevertheless, NNSA does provide this assessment option for drum-type packages. More specifically, NNSA will accept comparisons to a similar currently certified drum-type package for demonstrating bounding case package orientations and acceptable minimum weight for the HAC Free Drop, Puncture, Crush and Thermal performance tests. Applicants who choose this assessment option must ensure that the SARP includes a comparison that demonstrates that the proposed package is, in all respects, better than or equal to the package previously certified by NNSA and that the proposed package can meet all the regulatory performance requirements. | - SG 100, 2.6.5 |  |  |  |
|  | - The following information should be provided: |  |  |  |  |
|  | - The dimensions, materials, and configurations of each package. |  |  |  |  |
|  | - The overall weight of each package. |  |  |  |  |
|  | - The weight and form of the contents of each package. The comparison should demonstrate the following: |  |  |  |  |
|  | - That the packages would have similar responses to the specified tests. |  |  |  |  |
|  | - That the proposed package’s structural response is similar to or bounded by the currently certified package at both the maximum and minimum specified weights. |  |  |  |  |
|  | - That the forces acting on all vital safety systems and components of the proposed package are less than the currently certified package or that all vital safety systems and components of the proposed package have sufficient structural integrity. |  |  |  |  |
|  | - That the proposed package will meet all the regulatory performance requirements. |  |  |  |  |
| - 2.7.1 | - 10 CFR 71.73(c): “Tests. Tests for hypothetical accident conditions must be conducted as follows.” Free Drop 10 CFR 71.73(c)(1): “Free drop. A free drop of the specimen through a distance of 9 m (30 ft) onto a flat, essentially unyielding, horizontal surface, striking the surface in a position for which maximum damage is expected.” The SARP should discuss performance of the test and demonstrate that testing meets the requirements specified in §71.73(a), §71.73(b) and §71.73(c)(1). | - 10 CFR 71.73(c) - 10 CFR 71.73(c)(1) - RG 7.9, 2.7.1 |  |  |  |
|  | - The performance and structural integrity of the package should be evaluated for the drop orientations that cause the most severe damage, including center-of-gravity-over-corner, oblique orientation with secondary impact (slap down), side drop, and drop onto the closure. Orientations for which the center of gravity is directly over the point of impact should also be considered. An orientation that results in the most damage to one system or component may not be the most damaging for other systems and components. If a feature such as a tiedown component is a structural part of the package, it should be - considered in selecting the drop test configurations and drop orientation. For these reasons, it is usually necessary to consider several drop orientations. | - RG 7.9, 2.7.1 |  |  |  |
|  | - The following items should be addressed, as applicable: - For packages with lead shielding, the package should be evaluated for the effects of lead slump. This evaluation of lead slump should be consistent with that used in the shielding evaluation. | - RG 7.9, 2.7.1 |  |  |  |
|  | - The closure lid bolt (or closure system for designs using something other than closure lid bolts) design should be assessed for the combined effects of free drop impact force, internal pressures, thermal stress, O-ring compression force, and bolt preload. | - RG 7.9, 2.7.1 |  |  |  |
|  | - Buckling of package components should be evaluated. | - RG 7.9, 2.7.1 |  |  |  |
|  | - Other package components, such as port covers, port cover plates, and shield enclosures should be evaluated for the combined effects of the package drop impact force, puncture, internal pressures, and thermal stress. | - RG 7.9, 2.7.1 |  |  |  |
| - 2.7.1.1 | - End Drop This section should describe the effects of the end drop test on the package. | - RG 7.9, 2.7.1.1 |  |  |  |
| - 2.7.1.2 | - Side Drop This section should describe the effects of the side drop test on the package. | - RG 7.9, 2.7.1.2 |  |  |  |
| - 2.7.1.3 | - Corner Drop This section should describe the effects of the corner drop test on the package. | - RG 7.9, 2.7.1.3 |  |  |  |
| - 2.7.1.4 | - Oblique Drops This section should describe the effects of oblique drops or provide information that shows that the end, side, and corner drops are more damaging to all systems and components that are vital to safety. | - RG 7.9, 2.7.1.4 |  |  |  |
| - 2.7.1.5 | - Summary of Results This section should describe the condition of the package after each drop test, and describe the damage for each orientation. | - RG 7.9, 2.7.1.5 |  |  |  |
| - 2.7.2 | - Crush 10 CFR 71.73(c)(2): “Crush. Subjection of the specimen to a dynamic crush test by positioning the specimen on a flat, essentially unyielding horizontal surface so as to suffer maximum damage by the drop of a 500-kg (1100-lb) mass from 9 m (30 ft) onto the specimen. The mass must consist of a solid mild steel plate 1 m (40 in) by 1 m (40 in) and must fall in a horizontal attitude. The crush test is required only when the specimen has a mass not greater than 500 kg (1100 lb), an overall density not greater than 1000 kg/m3 (62.4 lb/ft3) based on external dimension, and radioactive contents greater than 1000 A2 not a special form radioactive material. For packages containing fissile material, the radioactive contents greater than 1000 A2 criterion does not apply.” If applicable, this section should discuss the performance of the dynamic crush test, describe the effects of the test on the package, demonstrate that the test(s) performed meets the requirements specified in §71.73(a), §71.73(b) and §71.73(c)(2), and the evaluation of the package’s ability to meet the specified requirements. If the applicant determines that the test is not applicable, the SARP should explain why the applicant believes the test is not applicable. | - 10 CFR 71.73(c)(2) - RG 7.9, 2.7.2 |  |  |  |
| - 2.7.3 | - Puncture 10 CFR 71.73(c)(3): “Puncture. A free drop of the specimen through a distance of 1 m (40 in) in a position for which maximum damage is expected, onto the upper end of a solid, vertical, cylindrical, mild steel bar mounted on an essentially unyielding, horizontal surface. The bar must be 15 cm (6 in) in diameter, with the top horizontal and its edge rounded to a radius of not more than 6 mm (0.25 in), and of a length as to cause maximum damage to the package, but not less than 20 cm (8 in) long. The long axis of the bar must be vertical.” The SARP should discuss performance of the test and demonstrate that the test(s) performed meets the requirements of §71.73(c)(3). | - 10 CFR 71.73(c)(3) - RG 7.9, 2.7.3 |  |  |  |
|  | This section should describe the effects of puncture on the package, and identify and justify that the orientations for which maximum damage would be expected have been evaluated. - The description should consider any damage resulting from the free drop and crush tests, as well as both local damage near the point of impact of the puncture bar and the overall effect on the package. | RG 7.9, 2.7.3 |  |  |  |
|  | - Containment system valves and fittings should be addressed. Punctures at oblique angles, near a support valve, at the package closure, and at a penetration should be considered, as appropriate. General comments provided in Sections 2.6 (NCT) and 2.7.1 (Free Drop) of this checklist may also apply to this test condition. |  |  |  |  |
|  | - Although analytical methods are available for predicting puncture, empirical formulas derived from puncture test results of laminated panels are usually used for package design. The Nelm’s formula developed specifically for package design provides the minimum thickness needed for preventing the puncture of the steel surface layer of a typical steel-lead-steel laminated cask wall. |  |  |  |  |
| - 2.7.4 | - Thermal 10 CFR 71.73(c)(4): “Thermal. Exposure of the specimen fully engulfed, except for a simple support system, in a hydrocarbon fuel/air fire of sufficient extent, and in sufficiently quiescent ambient conditions, to provide an average emissivity coefficient of at least 0.9, with an average flame temperature of at least 800 C (1475 F) for a period of 30 minutes, or any other thermal test that provides the equivalent total heat input to the package and which provides a time averaged environmental temperature of 800 C. The fuel source must extend horizontal at least 1 m (40 in), but may not extend more than 3 m (10 ft), beyond any external surface of the specimen, and the specimen must be positioned 1 m (40 in) above the surface of the fuel source. For purposes of calculation, the surface absorptivity coefficient must be either that value which the package may be expected to possess if exposed to the fire specified or 0.8, whichever is greater; and the convective coefficient must be that value which may be demonstrated to exist if the package were exposed to the fire specified. Artificial cooling may not be applied after cessation of external heat input, and any combustion of materials of construction, must be allowed to proceed until it terminates naturally.” Except as noted in Section 2.7.4.1 through Section 2.7.4.4 of this checklist, detailed results of the thermal test should be reported in Chapter 3 (Thermal Evaluation) of the SARP. | - 10 CFR 71.73(c)(4) - RG 7.9, 2.7.3 |  |  |  |
|  | - This section should evaluate the structural design for the effects of a fully engulfing fire as specified in §71.73(c)(4). Any damage resulting from the free drop, crush, and puncture conditions should be incorporated into the initial condition of the package for the fire test. | - RG 7.9, 2.7.4 |  |  |  |
|  | - The temperatures resulting from the fire and any increase in gas inventory caused by combustion or decomposition processes should be considered when determining the maximum pressure in the package during or after the test. The maximum thermal stresses, which can occur during or after the fire, should be assessed. | - RG 7.9, 2.7.4 |  |  |  |
| - 2.7.4.1 | - Summary of Pressures and Temperatures This section should provide a summary of all the temperatures and pressures, as determined in SARP Chapter 3 (Thermal Evaluation). | - RG 7.9, 2.7.4.1 |  |  |  |
| - 2.7.4.2 | - Differential Thermal Expansion This section should include calculations for the circumferential and axial deformations and stresses (if there are any) that result from differential thermal expansion. Peak conditions, post-fire steady-state conditions, and all transient conditions should be considered. Note: Consideration of the conditions specified in the last sentence above can be demonstrated by addressing and justifying the maximum stresses from differential thermal expansion and/or temperature gradients on vital components. | - RG 7.9, 2.7.4.2 |  |  |  |
| - 2.7.4.3 | - Stress Calculations This section should include calculations of the stresses caused by thermal gradients, differential expansion, pressure, and other mechanical loads. | - RG 7.9, 2.7.4.3 |  |  |  |
|  | - Sketches showing the configuration and dimensions of the members of systems under investigation, and locations of the points at which the stresses are being calculated, should be included. | - RG 7.9, 2.7.4.3 |  |  |  |
| - 2.7.4.4 | - Comparison with Allowable Stresses This section should make the appropriate stress combinations and compare the resulting stresses with the design criteria in SARP Section 2.1.2. This section should demonstrate that the performance requirements specified in the regulations have been satisfied. | - RG 7.9, 2.7.4.4 |  |  |  |
| - 2.7.5 | - Immersion -- Fissile Material 10 CFR 71.73(c)(5): “Immersion – fissile material. For fissile material subject to §71.55, in those cases where water inleakage has not been assumed for criticality analysis, immersion under a head of water of at least 0.9 m (3 ft) in the attitude for which maximum leakage is expected.” If the contents include fissile material that is subject to the requirements of §71.55, and if water leakage has not been assumed for the criticality analysis, this section should assess the effects and consequences of the water immersion test condition. - The SARP should discuss performance of the test and demonstrate that the testing meets the requirements specified in §71.73(a), §71.73(b) and §71.73(c)(5). | - 10 CFR 71.73(c)(5) - RG 7.9, 2.7.5 |  |  |  |
| - 2.7.6 | - Immersion -- All Packages 10 CFR 71.73(c)(6): “Immersion – all packages. A separate, undamaged specimen must be subjected to water pressure equivalent to immersion under a head of water of at least 15 m (50 ft). For test purposes, an external pressure of water of 150 kPa (21.7 lbf/in2) gauge is considered to meet these conditions.” This section should evaluate the undamaged package for water pressure equivalent to immersion under a head of water of at least 15 m (50 ft) for 8 hours. The SARP should discuss performance of the test and demonstrate that testing meets the requirements specified in §71.73(a), §71.73(b) and §71.73(c)(6). | - 10 CFR 71.73(c)(6) - RG 7.9, 2.7.6 |  |  |  |
| - 2.7.7 | - Deep Water Immersion Test (for Type B Packages Containing More than 105 A2) 10 CFR 71.61: “A Type B package containing more than 105 A2 must be designed so that its undamaged containment system can withstand an external water pressure of 2 MPa (290 psi) for a period of not less than 1 hour without collapse, buckling, or inleakage of water.” If applicable, this section should evaluate the package for an external water pressure of 2 MPa (290 psi) for a period of not less than 1 hour, as specified in §71.61. Note: The pressure should be assumed to be gauge, and the internal pressure should be assumed to be 0 atm. Although 10 CFR 71.61 is silent on this issue, this guidance is consistent with the pressure requirement specified in 10 CFR 71.73(c)(6). | - 10 CFR 71.61 - RG 7.9, 2.7.7 |  |  |  |
| - 2.7.8 | - Summary of Damage This section should describe the condition of the package after the accident test sequence. The description should address the extent to which safety systems and components have been damaged, and relate the package condition to the acceptance standards. | - RG 7.9, 2.7.8 |  |  |  |
| - 2.8 | - Accident Conditions for Air Transport of Plutonium If applicable, this section should address the accident conditions in §71.74. Note: §71.74 is not presented in this section since NNSA currently does not have responsibility for transporting plutonium packages by air. Note: If this requirement does not apply, the SARP should simply state that the requirement does not apply and why. | - 10 CFR 71.74 - RG 7.9, 2.8 |  |  |  |
| - 2.9 | - Accident Conditions for Fissile Material Packages for Air Transport If applicable, this section should address the accident conditions in §71.55(f). Note: §71.55(f) is not presented in this section since NNSA currently does not transport fissile material by air. The SARP should state that these requirements do not apply. For packages proposed for certification where this is not the case, the applicant should follow the guidance provided in RG 7.9, Section 2.9. | - 10 CFR 71.55(f) - RG 7.9, 2.9 |  |  |  |
| - 2.10 | - Special Form For packages designed to transport radioactive material only in special form, this section should state that the contents meet the requirements specified in §71.75 when subjected to the applicable test conditions specified in §71.77. Note: The contents of NNSA packages are not normally special form. Therefore, this SARP Section should state that the contents are normal form and that special form requirements do not apply. If this is not the case, the content of this SARP section should conform to that specified in RG 7.9, Section 2.10. | - 10 CFR 71.4 - 10 CFR 71.75 - RG 7.9, 2.10 |  |  |  |
| - 2.11 | - Fuel Rods Note: NNSA packaging currently does not have responsibility for any packages with fuel rods content. This SARP section should state that these requirements do not apply. If the package proposed for certification does include fuel rods as a proposed content, SARP Chapter 4 (Containment) should provide the information specified in RG 7.9, Section 2.11. | - RG 7.9, 2.11 |  |  |  |
| - 2.12 | - Appendix The appendix should include a list of references, copies of applicable references if not generally available, computer code descriptions, input and output files, test results, test reports, and other appropriate supplemental information. The appendix should also include materials and manufacturing specifications for items that are significant with respect to safety, but are not produced to generally recognized standards. | - RG 7.9, 2.12 |  |  |  |

| Chapter 3 – Thermal Evaluation Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| - Section | - Requirement | - Requirement Basis | - Requirement SARP Location | - Satisfactory? Y/N/NA | - Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| - 3.0 | - Thermal Evaluation This chapter should identify, describe, discuss, and analyze the principal thermal engineering design of the packaging, components, and systems that are important to safety, and describe how the package complies with the performance requirements of 10 CFR 71. | - RG 7.9, 3. |  |  |  |
| - 3.1 | - Description of Thermal Design 10 CFR 71.33(a)(5)(v): “Structural and mechanical means for the transfer and dissipation of heat.” This section should describe the significant thermal design features and operating characteristics of the package, and discuss the operation of all subsystems. | - 10 CFR 71.33(a)(5)(v) - RG 7.9, 3.1 |  |  |  |
|  | - 10 CFR 71.33(b)(8): “Identification and volumes of any coolants.” This section should identify the volumes of any coolants used in the package and should document the evaluation of their effects on the packaging’s performance. | - 10 CFR 71.33(b)(8) |  |  |  |
|  | - The thermal criteria that will be applied directly to thermal results (e.g., maximum fuel temperature, shield temperature not to exceed melt) should be identified. | - RG 7.9, 3.1 |  |  |  |
|  | - The significant results of the thermal analysis or tests and the implication of these results on the overall package should be summarized. | - RG 7.9, 3.1 |  |  |  |
|  | - 10 CFR 71.33(b)(7): “Maximum amount of decay heat.” The minimum and maximum decay heat loads assumed in the thermal evaluation should be specified. The maximum decay heat load assumed should be consistent with the source terms assumed in the shielding and containment analyses. | - 10 CFR 71.33(b)(7) - RG 7.9, 3.1 |  |  |  |
|  | - Properties evaluated in this chapter of the SARP that are used to support other evaluations (e.g., pressure, temperature, distributions relative to thermal stress) should be identified. | - RG 7.9, 3.1 |  |  |  |
| - 3.1.1 | - Design Features This section should describe design features important to thermal performance, including the following: - Package geometry and materials of construction. | - RG 7.9, 3.1.1 |  |  |  |
|  | - The structural and mechanical features that may affect heat transfer, such as cooling fins, insulating materials, surface conditions of the package components, and gaps or physical contacts between internal components. | - RG 7.9, 3.1.1 |  |  |  |
| - 3.1.2 | - Content’s Decay Heat The maximum decay heat and radioactivity of the contents should be specified. This section should show the derivation of decay heat and that it is consistent with the maximum quantity of radioactive contents. | - RG 7.9, 3.1.2 |  |  |  |
| - 3.1.3 | - Summary Tables of Temperatures This section should present summary tables of the maximum or minimum temperatures that affect structural integrity, containment, shielding, and criticality under both NCT and HAC. | - RG 7.9, 3.1.3 |  |  |  |
|  | - For the thermal test condition, the tables should also include: - The maximum temperatures of various package components and the time that they occur after fire initiation. | - RG 7.9, 3.1.3 |  |  |  |
|  | - The maximum temperatures of the post-fire steady-state condition. | - RG 7.9, 3.1.3 |  |  |  |
| - 3.1.4 | - Summary Tables of Maximum Pressures The summary tables should include the maximum normal operating pressure and the maximum pressure under HAC. | - RG 7.9, 3.1.4 |  |  |  |
| - 3.2 3.2.1 | - Material Properties and Component Specifications Material Properties This section should specify the appropriate thermal properties for materials that affect heat transfer both within the package and from the package to the environment. | - RG 7.9, 3.2 - RG 7.9, 3.2.1 |  |  |  |
|  | - Liquids or gases within the package and gases external to the package for HAC should be included. | - RG 7.9, 3.2.1 |  |  |  |
|  | - The thermal absorptivities and emissivities should be appropriate for the package surface conditions and each thermal condition. | - RG 7.9, 3.2.1 |  |  |  |
|  | - When reporting a property as a single value, the evaluation should show that this value bounds the equivalent temperature-dependent property. | - RG 7.9, 3.2.1 |  |  |  |
|  | - This section should include references for the data cited. | - RG 7.9, 3.2.1 |  |  |  |
| - 3.2.2 | - Component Specifications This section should include the technical specifications of components that are important to the thermal performance of the package, as illustrated by the following examples: - In the case of valves or seals, the operating pressure range and temperature limits. | - RG 7.9, 3.2.2 |  |  |  |
|  | - Properties of fabricated insulation and coatings, including a summary of test data that supports their performance specifications. | - RG 7.9, 3.2.2 |  |  |  |
|  | - The maximum allowable service temperatures or pressures for each package component. | - RG 7.9, 3.2.2 |  |  |  |
|  | - The minimum allowable service temperature of all components, which should be less than or equal to -40 C (-40 F). | - RG 7.9, 3.2.2 |  |  |  |
|  | This section should also document the results of the assessment of the behavior of packaging materials whose primary function is to provide thermal insulation and/or impact energy absorption. These materials must be assessed under the following conditions: | - SG 100, 2.5.5, 2.7, and 3.5 |  |  |  |
|  | - The material must be evaluated for NCT compliance under low temperature (-40 ºF) conditions; |  |  |  |  |
|  | - The effect of moisture content on crush strength must be evaluated or a reasoned argument as to why the moisture content of the material cannot change from those conditions already considered must be provided; and |  |  |  |  |
|  | - The effect of high temperature on the insulating material’s performance should be addressed since some materials may start to decompose at a particular temperature. |  |  |  |  |
| - 3.3 | - Thermal Evaluation Under Normal Conditions of Transport This section of the SARP should demonstrate compliance with the heat and cold conditions specified for NCT. 10 CFR 71.71(c)(1) : “Heat. An ambient temperature of 38 C (100 F) in still air, and insolation according to the” Insolation Data table in 10 CFR 71.71(c)(1). 10 CFR 71.71(c)(2): “Cold. An ambient temperature of -40 C (-40 F) in still air and shade.” | - 10 CFR 71.71(c)(1) and (c)(2) |  |  |  |
|  | - This section should describe the thermal evaluation of the system and subsystem operation under NCT. The temperature ranges bounded by the minimum and maximum ambient temperatures and the minimum and maximum decay heat loads should be considered. The results should be compared with allowable limits of temperature, pressure, etc., for the package components. | - RG 7.9, 3.3 |  |  |  |
|  | - The information should be presented in summary tables, along with statements and appropriate comments. Information that is to be used in other sections of the SARP should be identified. | - RG 7.9, 3.3 |  |  |  |
|  | - The margins of safety for package temperatures, pressures, and thermal stresses, including the effects of uncertainties in thermal properties, test conditions and diagnostics, and analytical methods, should be addressed. The analysis or test results should be shown to be reliable and repeatable. - Note: The reliability and repeatability of analysis or test can be demonstrated by ensuring that computer codes are benchmarked against actual data, and by deriving the thermal properties of the materials from nationally or internationally recognized sources or by deriving them from tests conducted in accordance with an appropriate quality assurance program as outlined in ASTM E 2230-02. In addition, any HAC thermal testing of the package or components should be performed in accordance with the procedures outlined in ASTM E 2230-02. | - RG 7.9, 3.3 |  |  |  |
|  | - The following general information should be considered and included, as appropriate, when addressing this section: - For thermal evaluation by analyses, the methods and calculations used in the package thermal evaluation should be described in sufficient detail to enable the reviewer to verify the results. | - RG 7.9, 3.3 |  |  |  |
|  | - Assumptions used in the analysis should be clearly described and justified. | - RG 7.9, 3.3 |  |  |  |
|  | - For computer analyses that include finite element analyses, the computer program should be described and shown to be well benchmarked and widely used for thermal analyses and applicable to the evaluation. | - RG 7.9, 3.3 |  |  |  |
|  | - The models and modeling details should be clearly described. | - RG 7.9, 3.3 |  |  |  |
|  | - For thermal evaluation by test, the test method, procedures, equipment, and facilities that were used should be described. | - RG 7.9, 3.3 |  |  |  |
|  | - If the specimen tested is not identical in all respects to the package described in the SARP, the differences should be described and a justification that explains that the differences between the specimen tested and the package proposed for certification will not affect the test results should be included. | - RG 7.9, 3.3 |  |  |  |
|  | - Temperature data should be reported at gaskets, valves, and other containment boundaries, particularly for temperature-sensitive materials, as well as for the overall package. | - RG 7.9, 3.3 |  |  |  |
|  | - Some conditions, such as ambient temperature, decay heat of contents, or package emissivity or absorptivity, may not be exactly represented by a thermal test. Appropriate corrections or evaluations that account for these differences must be made and should be fully described. | - RG 7.9, 3.3 |  |  |  |
|  | - Both interior and exterior temperatures should be included. Note: External: Maximum surface temperature. Interior: Maximum temperatures at specified internal locations such as O-rings, CV, and/or other internal location/package component that may be of concern. | - RG 7.9, 3.3 |  |  |  |
|  | - The damage caused by the tests and the results of any measurements that were made should be reported in detail, including photographs of the testing and the test specimen. | - RG 7.9, 3.3 |  |  |  |
| - 3.3.1 | - Heat and Cold This section should demonstrate that the tests for NCT do not result in significant reduction in packaging effectiveness. - The following items should be considered and addressed: - Degradation of the heat-transfer capability of the packaging (such as creation of new gaps between components). | - RG 7.9, 3.3.1 |  |  |  |
|  | - Changes in material conditions or properties (e.g., expansion, contraction, gas generation, and thermal stresses) that affect the structural performance. | - RG 7.9, 3.3.1 |  |  |  |
|  | - Changes in the packaging that affect containment, shielding, or criticality (such as thermal decomposition or melting of materials). | - RG 7.9, 3.3.1 |  |  |  |
|  | - Ability of the packaging to withstand the tests under HAC. | - RG 7.9, 3.3.1 |  |  |  |
|  | - The component temperatures and pressures should be compared with their allowable values. This section should explicitly show that the package meets the maximum surface temperature requirements specified in §71.43(g). 10 CFR 71.43(g): “A package must be designed, constructed, and prepared for transport so that in still air at 38 C (100 F) and in the shade, no accessible surface of a package would have a temperature exceeding 50 C (122 F) in a nonexclusive use shipment, or 85 C (185 F) in an exclusive use shipment.” | - 10 CFR 71.43(g) - RG 7.9, 3.3.1 |  |  |  |
| - 3.3.2 | - Maximum Normal Operating Pressure This section should report the maximum normal operating pressure and show how it was calculated, assuming the package has been subjected to the heat condition for one year. The calculation should consider all possible sources of gases, including the following: | - RG 7.9, 3.3.2 |  |  |  |
|  | - Gases initially present in the package. |  |  |  |  |
|  | - Saturated vapor, including water vapor from the contents or packaging. | - RG 7.9, 3.3.2 |  |  |  |
|  | - Helium from the radioactive decay of the contents. | - RG 7.9, 3.3.2 |  |  |  |
|  | - Hydrogen or other gases resulting from thermal- or radiation-induced decomposition of materials such as water or plastics. | - RG 7.9, 3.3.2 |  |  |  |
|  | - Fuel rod failure. | - RG 7.9, 3.3.2 |  |  |  |
|  | - It should be demonstrated that hydrogen and other flammable gases would not result in a flammable mixture within any confined volume of the package. | - RG 7.9, 3.3.2 |  |  |  |
| - 3.4 | - Thermal Evaluation Under Hypothetical Accident Conditions This section should describe the thermal evaluation of the package under HAC. The HAC conditions in §71.73 should be applied sequentially. For the accident condition thermal evaluation, general comments in Section 3.3 (Thermal Evaluation Under NCT) should be considered and addressed, as appropriate. Note: The HAC conditions (10 CFR 71.73) are quoted in Section 2.7 of this checklist. | - 10 CFR 71.73(c)(4) - RG 7.9, 3.4 |  |  |  |
|  | - This section of the SARP should demonstrate compliance with ASTM E 2230-02, Thermal Qualification of Type B Packages for Radioactive Material. - Note: SARPs, for packages undergoing recertification, that document the packages’ compliance with SG 140.1,Combination Test Analysis/Method Used to Demonstrate Compliance to DOE Type B Packaging Thermal Test Requirements, but whose existing test data are insufficient to demonstrate compliance with ASTM E 2230-02 may continue to demonstrate compliance with SG-140.1. | - ASTM E 2230-02 - SG 140.1 |  |  |  |
|  | - The effects of solar insolation must be evaluated following the HAC thermal test, but does not have to be considered prior to or during the thermal test. | - SG-100, 3.5 and Note (a) of Table 2-8 |  |  |  |
| - 3.4.1 | - Initial Conditions The thermal evaluation should consider the effects of the drop, crush (if applicable), and puncture tests. This section should identify the initial conditions and justify that they are the most unfavorable, including initial ambient temperature, insolation, internal pressure, decay heat, etc. Note: NNSA authorized deviations from the guidance contained in RG 7.8 Table 1 are contained in SG-100 Table 2-8. | - RG 7.9, 3.4.1 |  |  |  |
| - 3.4.2 | - Fire Test Conditions The test or analysis used to evaluate the package under the thermal test conditions should be described in detail. The evaluation should address the requirements in §71.73(c). Note: The HAC test conditions specified by 10 CFR 71.73 are quoted in Section 2.7 of this checklist. | - 10 CFR 71.73(c)(4) - RG 7.9, 3.4.2 |  |  |  |
| - 3.4.3 | - Maximum Temperatures and Pressure This section should report the transient peak temperatures of the package components as a function of time both during and after the thermal test, as well as the maximum temperatures from the post-thermal test, steady-state condition. This section should include those temperatures at locations in the package that are significant to the safety analysis and review. In particular, the temperatures for items such as the contents, gaskets, valves, and shielding should be reported. The calculations of the transient temperatures should trace the temperature-time history up to and past the time at which maximum temperatures are achieved and begin to fall. Note: When conformance to this specification is demonstrated through analysis, full conformance to this specification is required. When conformance to this specification is demonstrated through testing, determining the transient peak temperatures of the package components as a function of time both during and after the thermal test, as well as the maximum temperatures from the post-thermal test, steady-state condition may be extremely difficult or impossible. Therefore, when conformance to this specification is demonstrated through testing, it is acceptable to document the maximum adjusted peak temperatures and associated locations (at or near all vital components (e.g., seals or criticality control features, etc.)), and to use these temperatures in the required calculations. | - RG 7.9, 3.4.3 |  |  |  |
|  | - The evaluation of the maximum pressure in the package should be based on the maximum normal operating pressure, and should consider fire-induced increases in package temperatures, thermal combustion or decomposition processes, fuel rod failure, phase changes, etc. | - RG 7.9, 3.4.3 |  |  |  |
|  | - This section should provide a general description of the package performance and should compare the results of the thermal test with allowable limits of temperature, pressure, etc., for the package components. Damage to the package from either the interpretation of the analysis or from test observation should be considered and described. The assessment should include structural damage, breach of containment, and loss of shielding. | - RG 7.9, 3.4.3 |  |  |  |
| - 3.4.4 | - Maximum Thermal Stresses This section should evaluate the most severe thermal stress conditions that result during the fire test and subsequent cool-down. The temperatures corresponding to the maximum thermal stresses should be reported. | - RG 7.9, 3.4.4 |  |  |  |
| - 3.4.5 | - Accident Conditions for Fissile Material Packages for Air Transport If applicable, address the expanded fire tests conditions specified in §71.55(f). Note: NNSA currently does not transport fissile material by air. The SARP should state that these requirements do not apply. For packages proposed for certification where this is not the case, the applicant should follow the guidance provided in RG 7.9, Section 3.4.5. | - RG 7.9, 3.4.5 |  |  |  |
| - 3.5 | - Appendix The appendix should include a list of references, applicable pages from referenced documents, justification of assumptions or analytical procedures, test results, photographs, computer program descriptions and input and output files, specifications of O-rings and other components, detailed materials test data, and other supplemental information. | - RG 7.9, 3.5 |  |  |  |

| Chapter 4 – Containment Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| - Section | - Requirement | - Requirement Basis | - Requirement SARP Location | - Satisfactory? Y/N/NA | - Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| - 4.0 | - Containment - This chapter should identify the package containment system and describe how the package complies with the containment requirements of 10 CFR 71. | - RG 7.9, 4. |  |  |  |
| - 4.1 | - Description of the Containment System - 10 CFR 71.33(a)(4): “Identification of the containment system.” - This section should define and describe the containment system, including components such as the containment vessel, welds, seals, lids, cover plates, valves, and other closure devices. The description should include materials of construction and applicable codes and standards. The containment boundary of the package should be explicitly identified, including the containment vessel, welds, drain or fill ports, valves, seals, test ports, pressure relief devices, lids, cover plates, and other closure devices. If multiple seals are used for a single closure, this section should identify the seal defined as the containment system seal. This section should also include a sketch of the containment system. - Note: If a sketch of the containment system is provided elsewhere in the SARP, this section may simply state the following: “The sketch of the containment system is located in Section X.X.” | - 10 CFR 71.33(a)(4) - RG 7.9, 4.1 |  |  |  |
|  | - This section should address the following items: - The containment system penetrations and their method of closure. | - RG 7.9, 4.1 |  |  |  |
|  | - The performance specifications for components such as valves and pressure relief devices. | - RG 7.9, 4.1 |  |  |  |
|  | - The method used to protect any valve or similar device on the packaging against unauthorized operation and the enclosure used to retain any leakage (except for a pressure relief valve). Note: This may be discussed in Chapter 2 (Structural Evaluation) of the SARP. - 10 CFR 71.43(e): “A package valve or other device, the failure of which would allow radioactive contents to escape, must be protected against unauthorized operation and, except for a pressure relief device, must be provided with an enclosure to retain any leakage.” | - 10 CFR 71.43(e) - RG 7.9, 4.1 |  |  |  |
|  | - How the containment system is securely closed with a positive fastening device that cannot be opened unintentionally or by a pressure that may arise within the package. Note: This may be discussed in Chapter 2 (Structural Evaluation) of the SARP. 10 CFR 71.43(c): “Each package must include a containment system securely closed by a positive fastening device that cannot be opened unintentionally or by a pressure that may arise within the package.” | - 10 CFR 71.43(c) - RG 7.9, 4.1 |  |  |  |
|  | - The features that ensure continuous venting are precluded. Note: This may be discussed in Chapter 2 (Structural Evaluation) of the SARP. - 10 CFR 71.43(h): “A package may not incorporate a feature intended to allow continuous venting during transport.” | - 10 CFR 71.43(h) - RG 7.9, 4.1 |  |  |  |
| - 4.2 | - Containment Under Normal Conditions of Transport - This section should address the effects of NCT on hazardous nonradioactive materials present in proposed weapon component content, and determine their effects on the package’s containment boundary. | - SG 100, 4.7 |  |  |  |
|  | - This section should include the evaluation of the containment system under NCT. The evaluation should be performed for the most limiting chemical and physical forms of the contents. Significant daughter products should be included. The constituents of the releasable source term, including radioactive gases, liquids, and powder aerosols, should be identified. | - RG 7.9, 4.2 |  |  |  |
|  | - The evaluation should address the following: - The maximum internal pressures. | - RG 7.9, 4.2 |  |  |  |
|  | - The structural performance of the containment system, including seals, closure bolts, and penetrations. | - RG 7.9, 4.2 |  |  |  |
|  | - The leakage rate testing of the containment system. | - RG 7.9, 4.2 |  |  |  |
|  | - For Type B packages, the evaluation should show that there is no release under NCT to the required sensitivity. - 10 CFR 71.51(a)(1): “Section 71.71 (“Normal conditions of transport”), there would be no loss or dispersal of radioactive contents – as demonstrated to a sensitivity of 10-6 A2 per hour, no significant increase in external surface radiation levels, and no substantial reduction in the effectiveness of the packaging.” - This section should summarize and use the pertinent results of the structural and thermal evaluations to demonstrate that the package meets containment requirements under NCT. | - 10 CFR 71.51(a)(1) - RG 7.9, 4.2 |  |  |  |
|  | - 10 CFR 71.51(c): “Compliance with the permitted activity release limits” of 10 CFR 71.51(a) “may not depend on filters or on a mechanical cooling system.” - This section should document the package’s compliance with §71.51(c). | - 10 CFR 71.51(c) |  |  |  |
| - 4.3 | - Containment Under Hypothetical Accident Conditions - This section should include the evaluation of the containment system under HAC, considering factors given in Section 4.2 (Containment Under NCT). | - RG 7.9, 4.3 |  |  |  |
|  | ##### This section should demonstrate that the package meets the containment requirements of 10 CFR 71.51(a)(2) under HAC. In particular, the structural performance of the containment system should be addressed, including seals, closure bolts, and penetrations, as well as leakage rate testing of the containment system. - 10 CFR 71.51(a)(2): “Section 71.73 (“Hypothetical accident conditions”), there would be no escape of krypton-85 exceeding 10 A2 in 1 week, no escape of other radioactive material exceeding a total amount A2 in 1 week, and no external radiation dose rate exceeding 10 mSv/h (1 rem/h) at 1 m (40 in) from the external surface of the package.” | - 10 CFR 71.51(a)(2) - RG 7.9, 4.3 |  |  |  |
|  | - This section should address the effects of HAC on hazardous nonradioactive materials present in proposed weapon component content, and determine their effects on the package’s containment boundary. | - SG-100, 4.7 |  |  |  |
|  | - 10 CFR 71.51(c): “Compliance with the permitted activity release limits” of 10 CFR 71.51(a) “may not depend on filters or on a mechanical cooling system.” - This section should document the package’s compliance with §71.51(c). | - 10 CFR 71.51(c) |  |  |  |
| - 4.4 | - Leakage Rate Tests for Type B Packages - This section should describe leakage rate tests that are used to show the package meets the containment requirements of 10 CFR 71.51. This may include leakage rate tests of test units, newly fabricated packagings, periodic tests, and pre-shipment tests. | - RG 7.9, 4.4 |  |  |  |
| - 4.5 | - Appendix - Appropriate supporting information and analysis should be included in the analysis. If not included in the body of the SARP, the calculations for the following should be included: | - SG-100, 4.3, 4.6, 4.7, et al. |  |  |  |
|  | - Determination of the source term for each of the proposed contents |  |  |  |  |
|  | - Determination of A2s for each of the proposed contents |  |  |  |  |
|  | - Determination of the allowable leakage rate per ANSI N14.5. This calculation is not required if the package is demonstrated to be leaktight. |  |  |  |  |
|  | - The appendix should include a list of references, applicable pages from referenced documents, supporting information and analysis, test results, and other appropriate supplemental information. | - RG 7.9, 4.5 |  |  |  |

| Chapter 5 – Shielding Evaluation Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| - Section | - Requirement | - Requirement Basis | - Requirement SARP Location | - Satisfactory? Y/N/NA | - Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| - 5.0 | - Shielding Evaluation - This chapter should identify, describe, discuss, and analyze the principal radiation shielding design of the packaging, components, and systems that are important to safety. | - RG 7.9, 5. |  |  |  |
| - 5.1 - 5.1.1 | - Description of Shielding Design - Design Features - This section should describe the radiation shielding design features of the package, including dimensions, tolerances, materials of construction, and densities of material for neutron and gamma shielding. | - RG 7.9, 5.1 - RG 7.9, 5.1.1 |  |  |  |
| - 5.1.2 | - Summary Table of Maximum Radiation Levels - This section should present the maximum dose rates for NCT and HAC at the appropriate locations for non-exclusive and exclusive use shipments, as applicable. Table 5-1 of RG 7.9, Revision 2 provides an example of the information that should be provided. - 10 CFR 71.47(a): “Except as provided in” 10 CFR 71.47(b), “each package of radioactive materials offered for transportation must be designed and prepared for shipment so that under conditions normally incident to transportation the radiation level does not exceed 2 mSv/h (200 mrem/h) at any point on the external surface of the package, and the transport index does not exceed 10.” - 10 CFR 71.51(a)(2): “Section 71.73 (“Hypothetical accident conditions”), there would be no escape of krypton-85 exceeding 10 A2 in 1 week, no escape of other radioactive material exceeding a total amount A2 in 1 week, and no external radiation dose rate exceeding 10 mSv/h (1 rem/h) at 1 m (40 in) from the external surface of the package.” | - 10 CFR 71.47(a) - 10 CFR 71.51(a)(2) - RG 7.9, 5.1.2 |  |  |  |
| - 5.2 | - Source Specification - This section should describe the contents, as well as the gamma and neutron source terms used in the shielding analysis. Any increase in source terms over time should be addressed. | - RG 7.9, 5.2 |  |  |  |
|  | - For packages designed for spent fuel transport, this section should also state the assumed fuel burnup, power density, and cooling times. - Note: NNSA currently does not have responsibility for certifying spent fuel packages. Therefore, the entry in “Satisfactory?” column of this line should be NA. | - RG 7.9, 5.2 |  |  |  |
| - 5.2.1 | - Gamma Source - This section should specify the quantity of radioactive material included as contents and tabulate the gamma decay source strength (MeV/sec and photons/sec) as a function of photon energy. | - RG 7.9, 5.2.1 |  |  |  |
|  | - A detailed description of the method used to determine the gamma source strength and distribution should be provided. | - RG 7.9, 5.2.1 |  |  |  |
|  | - If dose rate requirements of 10 CFR 71.47(a) cannot be met, requirements for exclusive use shipments must be addressed. - 10 CFR 71.47(b): “A package that exceeds the radiation level limits specified in” §71.47(a) “must be transported by exclusive use shipment only, and the radiation levels for such shipment must not exceed the following during transportation: - (1) 2 mSv/h (200 mrem/h) on the external surface of the package, unless the following conditions are met, in which case the limit is 10 mSv/h (1000 mrem/h): - (i) The shipment is made in a closed transport vehicle; - (ii) The package is secured within the vehicle so that its position remains fixed during transportation; and (iii) There are no loading or unloading operations between the beginning and end of the transportation; - (2) 2 mSv/h (200 mrem/h) at any point on the outer surface of the vehicle, including the top and underside of the vehicle; or in the case of a flat-bed style vehicle, at any point on the vertical planes projected from the outer edges of the vehicle, on the upper surface of the load or enclosure, if used, and on the lower external surface of the vehicle; and - (3) 0.1 mSv/h (10 mrem/h) at any point 2 meters (80 in) from the outer lateral surfaces of the vehicle (excluding the top and underside of the vehicle); or in the case of a flat-bed style vehicle, at any point 2 meters (6.6 feet) from the vertical planes projected by the outer edges of the vehicle (excluding the top and underside of the vehicle); and - (4) 0.2 mSv/h (2 mrem/h) in any normally occupied space, except that this provision does not apply to private carriers, if exposed personnel under their control wear radiation dosimetry devices in conformance with 10 CFR 20.1502.” | - 10 CFR 71.47(b) |  |  |  |
| - 5.2.2 | - Neutron Source - This section should specify the quantity of radioactive material included as contents and tabulate the neutron source strength (neutron/sec) as a function of energy. | - RG 7.9, 5.2.2 |  |  |  |
|  | - A detailed description of the method used to determine the neutron source strength and distribution should be provided. | - RG 7.9, 5.2.2 |  |  |  |
| - 5.3 - 5.3.1 | - Shielding Model - Configuration of Source and Shielding - This section should provide a detailed description of the model used in the shielding evaluation. The effects of the tests on the packaging and its contents under NCT and HAC should be evaluated. The models used in the shielding calculation should be consistent with these effects. | - RG 7.9, 5.3 - RG 7.9, 5.3.1 |  |  |  |
|  | - This section should include sketches (to scale) and dimensions of the radial and axial shielding materials. The dimensions of the transport vehicle and package location for exclusive-use shipments for which the analysis is based on the radiation levels in 10 CFR 71.47(b) should be included, as appropriate. - Note: The requirements of §71.47(b) are quoted in Section 5.2.1 of this checklist. | - 10 CFR 71.47(b) - RG 7.9, 5.3.1 |  |  |  |
|  | - The dose point locations in the shielding model, including all locations prescribed in 10 CFR 71.47(a) or 71.47(b) and 10 CFR 71.51(a)(2) should be identified. These points should be chosen to identify the locations of maximum radiation levels. Voids, streaming paths, and irregular geometries in the model should be included or otherwise treated in an adequate manner. - Note: The requirements of 10 CFR 71.47(a), 10 CFR 71.47(b), and 10 CFR 71.51(a)(2) are quoted in Sections 5.1.2, 5.2.1, and 5.1.2 of this checklist. | - 10 CFR 71.47(a) - 10 CFR 71.47(b) - 10 CFR 71.51(a)(2) - RG 7.9, 5.3.1 |  |  |  |
| - 5.3.2 | - Material Properties - This section should describe the material properties (e.g., mass densities and atom densities) in the shielding models of the packaging and contents. Changes resulting under NCT or HAC should be included, as appropriate. | - RG 7.9, 5.3.2 |  |  |  |
|  | - The sources of data should be referenced for uncommon materials. | - RG 7.9, 5.3.2 |  |  |  |
| - 5.4 - 5.4.1 | - Shielding Evaluation - Methods - This section should provide a general description of the basic method used to determine the gamma and neutron dose rates at the selected points outside the package for NCT and HAC. This should include a description of the spatial source distribution and any computer program used, with its referenced documentation. | - RG 7.9, 5.4 - RG 7.9, 5.4.1 |  |  |  |
|  | - The shielding evaluation should include a detailed description of the basic input parameters, as well as the bases for selecting the program, attenuation and removal cross-sections, and buildup factors. | - RG 7.9, 5.4.1 |  |  |  |
| - 5.4.2 | - Input and Output Data - This section should identify key input data for the shielding calculations and show that information from the shielding models is properly input into the code. | - RG 7.9, 5.4.2 |  |  |  |
|  | - At least one representative input file and output file (or key sections of those files) should be included. | - RG 7.9, 5.4.2 |  |  |  |
|  | - This section should show that the code achieved proper convergence. | - RG 7.9, 5.4.2 |  |  |  |
| - 5.4.3 | - Flux-to-Dose-Rate Conversion - This section should include a tabulation of the flux-to-dose-rate conversion factors as a function of energy and should cite the appropriate references to support the data. | - RG 7.9, 5.4.3 |  |  |  |
| - 5.4.4 | - External Radiation Levels - This section should describe the results of the radiation analysis in detail. These results should agree with the summary tables. The locations of maximum dose rates for the analysis should be identified and sufficient data provided to show that the radiation levels are reasonable and their variations with location are consistent with the geometry and shielding characteristics of the package. | - RG 7.9, 5.4.4 |  |  |  |
|  | - The results should address NCT and HAC. | - RG 7.9, 5.4.4 |  |  |  |
| - 5.5 | - Appendix - The appendix should include a list of references, applicable pages from referenced documents, justification of assumptions or analytical procedures, test results, photographs, computer program descriptions, input and output files, and other supplemental information. | - RG 7.9, 5.5 |  |  |  |

| Chapter 6 – Criticality Evaluation Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| - Section | - Requirement | - Requirement Basis | - Requirement SARP Location | - Satisfactory? Y/N/NA | - Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| - 6.0 | - Criticality Evaluation - This chapter should identify, describe, discuss, and analyze the principal criticality safety design of the package, components, and systems important to safety, and describe how the package complies with the requirements of 10 CFR 71.55 and 71.59. - 10 CFR 71.55: “General requirements for fissile material packages. - (a) A package used for the shipment of fissile material must be designed and constructed in accordance with §§71.41 through 71.47. When required by the total amount of radioactive material, a package used for the shipment of fissile material must also be designed and constructed in accordance with §71.51. - (b) Except as provided in paragraph (c) or (g) of this section, a package used for the shipment of fissile material must be so designed and constructed and its contents so limited that it would be subcritical if water were to leak into the containment system, or liquid contents were to leak out of the containment system so that, under the following conditions, maximum reactivity of the fissile material would be attained: - (1) The most reactive credible configuration consistent with the chemical and physical form of the material; - (2) Moderation by water to the most reactive credible extent; and - (3) Close full reflection of the containment system by water on all sides, or such greater reflection of the containment system as - may additionally be provided by the surrounding material of the packaging. - (c) The” NNSA “may approve exceptions to the requirements of paragraph (b) of this section if the package incorporates special design features that ensure that no single packaging error would permit leakage, and if appropriate measures are taken before each shipment to ensure that the containment system does not leak. - (d) A package used for the shipment of fissile material must be so designed and constructed and its contents so limited that under the tests specified in §71.71 (“Normal conditions of transport”)— - (1) The contents would be subcritical; - (2) The geometric form of the package contents would not be substantially altered; - (3) There would be no leakage of water into the containment system unless, in the evaluation of undamaged packages under §71.59(a)(1), it has been assumed that moderation is present to such an extent as to cause maximum reactivity consistent with the chemical and physical form of the material; and - (4) There will be no substantial reduction in the effectiveness of the packaging, including: - (i) No more than 5 percent reduction in the total effective volume of the packaging on which nuclear safety is assessed; (ii) No more than 5 percent reduction in the effective spacing between the fissile contents and the outer surface of the packaging; and (iii) No occurrence of an aperture in the outer surface of the packaging large enough to permit the entry of a 10 cm (4 in.) cube. - (e) A package used for shipment of fissile material must be so designed and constructed and its contents so limited that under the tests specified in §71.73 (“Hypothetical accident conditions”), the package would be subcritical. For this determination, it must be assumed that: - (1) The fissile material is in the most reactive credible configuration consistent with the damaged condition of the package and the chemical and physical form of the contents; - (2) Water moderation occurs to the most reactive credible extent consistent with the damaged condition of the package and the chemical and physical form of the contents; and (3) There is full reflection by water on all sides, as close as is consistent with the damaged condition of the package.” - Note: 10 CFR 71.55(f) and (g) are not quoted in this section. NNSA does not currently transport fissile material packages by air, and does not currently have the responsibility for certifying packages containing uranium hexafluoride. - 10 CFR 71.59: “Standards for arrays of fissile material packages. - (a) A fissile material package must be controlled by either the shipper or the carrier during transport to assure that an array of such packages remains subcritical. To enable this control, the designer of a fissile material package shall derive a number “N” based on all the following conditions being satisfied, assuming packages are stacked together in any arrangement and with close full reflection on all sides of the stack by water: - (1) Five times “N” undamaged packages with nothing between the packages would be subcritical; - (2) Two times “N” damaged packages, if each package were subjected to the tests specified in §71.73 (“Hypothetical accident conditions”) would be subcritical with optimum interspersed hydrogenous moderation; and - (3) The value of “N” cannot be less than 0.5. - (b) The CSI must be determined by dividing the number 50 by the value of “N” derived using the procedures specified in paragraph (a) of this section. The value of the CSI may be zero provided that an unlimited number of packages are subcritical, such that the value of “N” is effectively equal to infinity under the procedures specified in paragraph (a) of this section. Any CSI greater than zero must be rounded up to the first decimal place. - (c) For a fissile material package which is assigned a CSI value— - (1) Less than or equal to 50, that package may be shipped by a carrier in a nonexclusive use conveyance, provided the sum of the CSIs is limited to less than or equal to 50. - (2) Less than or equal to 50, that package may be shipped by a carrier in an exclusive use conveyance, provided the sum of the CSIs is limited to less than or equal to 100. - (3) Greater than 50, that package must be shipped by a carrier in an exclusive use conveyance, provided the sum of the CSIs is limited to less than or equal to 100.” | - 10 CFR 71.55 - 10 CFR 71.59 - RG 7.9, 6. |  |  |  |
| - 6.1 - 6.1.1 | - Description of Criticality Design - Design Features - This section should describe the design features of the package that are important for criticality control. This should include such information as the confinement system for fissile material, neutron absorbing and moderating materials, flux traps, spacers, etc. | - RG 7.9, 6.1 - RG 7.9, 6.1.1 |  |  |  |
| - 6.1.2 | - Summary Table of Criticality Evaluation - This section should provide a summary table of criticality analysis results for the package for the following cases, as described in Section 6.4 through Section 6.6: - A single package, under the conditions of 10 CFR 71.55(b), (d), and (e). | - 10 CFR 71.55(b) - 10 CFR 71.55(d) - 10 CFR 71.55(e) - 10 CFR 71.59(a)(1) - 10 CFR 71.59(a)(2) - RG 7.9, 6.1.2 |  |  |  |
|  | - An array of undamaged packages, under the conditions of 10 CFR 71.59(a)(1). |  |  |  |  |
|  | - An array of damaged packages, under the conditions of 10 CFR 71.59(a)(2). - Note: The requirements of §71.55(b), §71.55(d), §71.55(e), §71.59(a)(1), and §71.59(a)(2) are quoted in Section 6.0 of this checklist. |  |  |  |  |
|  | - The maximum value of the effective neutron multiplication factor (keff), the uncertainty, the bias, and the number of packages evaluated in the arrays should be specified in the table. | - RG 7.9, 6.1.2 |  |  |  |
| - 6.1.3 | - Criticality Safety Index - This section should provide the Criticality Safety Index (CSI) based on the number of packages evaluated in the arrays and show how it was calculated. | - RG 7.9, 6.1.3 |  |  |  |
| - 6.2 | - Fissile Material Contents - This section should describe in detail the fissile materials in the package. The mass, dimensions, enrichment, physical and chemical composition, density, moisture, and other characteristics should be defined. | - RG 7.9, 6.2 |  |  |  |
| - 6.3 | - General Considerations - This section should address the general considerations used to evaluate criticality of the package. These may apply to the criticality evaluations of a single package and arrays of packages under both NCT and HAC. | - RG 7.9, 6.3 |  |  |  |
| - 6.3.1 | - Model Configuration - This section should describe and provide sketches of the analytical model used in the calculations. The sketches should identify the materials used in all regions of the model. | - RG 7.9, 6.3.1 |  |  |  |
|  | - Any differences between the actual package configuration and the model should be identified, and justification given that the model is conservative. | - RG 7.9, 6.3.1 |  |  |  |
|  | - Differences between the models for NCT and HAC should be clearly identified. | - RG 7.9, 6.3.1 |  |  |  |
| - 6.3.2 | - Material Properties - This section should provide the appropriate mass densities and atomic number densities for materials used in the models of the packaging and contents. Material properties should be consistent with the condition of the package under the tests specified in 10 CFR 71.71 and 71.73. - Note: The requirements of §71.71 and §71.73 are quoted in Sections 2.6 and 2.7 of this checklist. | - RG 7.9, 6.3.2 |  |  |  |
|  | - All material property differences between NCT and HAC should be clearly identified. Materials relevant to the criticality design such as poisons, foams, plastics, and other hydrocarbons should specifically be addressed. | - RG 7.9, 6.3.2 |  |  |  |
| - 6.3.3 | - Computer Codes and Cross-Section Libraries - This section should describe the basic methods used to calculate the effective neutron multiplication constant of the package to demonstrate compliance with the fissile material package standards. This should address the following: - A description of the computer program and neutron cross-sections used. | - RG 7.9, 6.3.3 |  |  |  |
|  | - The basis for selecting the specific program and cross-sections. | - RG 7.9, 6.3.3 |  |  |  |
|  | - Key input data for the criticality calculations, such as neutrons per generation, number of generations, convergence criteria, mesh selection, etc. | - RG 7.9, 6.3.3 |  |  |  |
| - 6.3.4 | - Demonstration of Maximum Reactivity - This section should include a demonstration that the most reactive configuration of each case listed in Sections 6.4, 6.5, and 6.6 (single package, arrays of undamaged packages, and arrays of damaged packages, respectively) have been evaluated. All assumptions and approximations should be clearly identified and justified. | - RG 7.9, 6.3.4 |  |  |  |
|  | - This section should identify the optimum combination of internal moderation (within the package) and interspersed moderation (between packages), as applicable. The following should be considered: - Moderation by water and any hydrogen-containing packaging materials, such as polyethylene. | - RG 7.9, 6.3.4 |  |  |  |
|  | - Preferential flooding of different regions within the package. | - RG 7.9, 6.3.4 |  |  |  |
|  | - Partial loadings (i.e., fissile masses less than the maximum allowable mass). | - RG 7.9, 6.3.4 |  |  |  |
| - 6.4 - 6.4.1 | - Single Package Evaluation - Configuration - This section should demonstrate that a single package is subcritical under NCT and HAC. The evaluation should consider the following factors: - Fissile material in its most reactive credible configuration consistent with the condition of the package and the chemical and physical form of the contents. | - RG 7.9, 6.4 - RG 7.9, 6.4.1 |  |  |  |
|  | - Water moderation to the most reactive credible extent, including water inleakage to the containment system as specified in 10 CFR 71.55(b). - Note: The 10 CFR 71.55(b) requirements are quoted in Section 6.0 of this checklist. | - 10 CFR 71.55(b) - RG 7.9, 6.4.1 |  |  |  |
|  | - Full water reflection on all sides of the containment system as specified in 10 CFR 71.55(b)(3), or reflection by the package materials, whichever results in the maximum reactivity. - Note: The 10 CFR 71.55(b)(3) requirements are quoted in Section 6.0 of this checklist. | - 10 CFR 71.55(b)(3) - RG 7.9, 6.4.1 |  |  |  |
| - 6.4.2 | - Results - The results of the single package evaluation should be presented. The additional specifications of 10 CFR 71.55(d)(2) through 71.55(d)(4) under NCT should also be addressed. - Note: The 10 CFR 71.55(d)(2) through 71.55(d)(4) requirements are quoted in Section 6.0 of this checklist. | - 10 CFR 71.55(d)(2) - 10 CFR 71.55(d)(3) - 10 CFR 71.55(d)(4) - RG 7.9, 6.4.2 |  |  |  |
| - 6.5 - 6.5.1 | - Evaluation of Package Arrays Under Normal Conditions of Transport - Configuration - This section should evaluate an array of 5N (or more) packages under NCT. Note: The 10 CFR 71.59(a)(1) requirements are quoted in Section 6.0 of this checklist. | - RG 7.9, 6.5 - 10 CFR 71.59(a)(1) - RG 7.9, 6.5.1 |  |  |  |
|  | - The evaluation should consider the following factors: - The most reactive configuration of the array (e.g., pitch and package orientation) with nothing between the packages. |  |  |  |  |
|  | - The most reactive credible configuration of the packaging and its contents under NCT. If the water spray test has demonstrated that water would not leak into the package, water inleakage need not be assumed. | - RG 7.9, 6.5.1 |  |  |  |
|  | - Full water reflection on all sides of a finite array. | - RG 7.9, 6.5.1 |  |  |  |
| - 6.5.2 | - Results - This section should present the results of the analyses for arrays and identify the most reactive array conditions. | - RG 7.9, 6.5.2 |  |  |  |
| - 6.6 - 6.6.1 | - Package Arrays Under Hypothetical Accident Conditions - Configuration - This section should evaluate an array of 2N (or more) packages under HAC. - Note: The 10 CFR 71.59(a)(2) requirements are quoted in Section 6.0 of this checklist. - The evaluation should consider the following factors: - The most reactive configuration of the array (e.g., pitch, package orientation, and internal moderation). | - RG 7.9, 6.6 - 10 CFR 71.59(a)(2) - RG 7.9, 6.6.1 |  |  |  |
|  | - Optimum interspersed hydrogenous moderation. | - RG 7.9, 6.6.1 |  |  |  |
|  | - The most reactive credible configuration of the packaging and its contents under HAC, including inleakage of water. | - RG 7.9, 6.6.1 |  |  |  |
|  | - Full water reflection on all sides of a finite array. | - RG 7.9, 6.6.1 |  |  |  |
| - 6.6.2 | - Results - This section should present the results of the analyses for arrays and identify the most reactive array conditions. | - RG 7.9, 6.6.2 |  |  |  |
| - 6.7 | - Fissile Material Packages for Air Transport - Note: NNSA currently does not transport fissile material by air. The SARP should state that these requirements do not apply. For packages proposed for certification where this is not the case, the applicant should follow the guidance provided in RG 7.9, Section 6.7. | - RG 7.9, 6.7 |  |  |  |
| - 6.8 | - Benchmark Evaluations - This section should include a description of the methods used to benchmark the criticality calculations. The computer codes for criticality evaluations should be benchmarked against critical experiments. | - RG 7.9, 6.8 |  |  |  |
|  | - The same computer code, hardware, and cross-section library used to calculate the effective multiplication factor values for the package should be used in the benchmark experiments. This section should present the results of the calculations for selected critical benchmark experiments to justify the validity of the calculational method and neutron cross-section values used in the analysis, and to demonstrate that the hardware and software used were appropriate for the application. | - RG 7.9, 6.8 |  |  |  |
| - 6.8.1 | - Applicability of Benchmark Experiments - This section should describe selected critical benchmark experiments that are to be analyzed using the method and cross-sections given in Section 6.3 (General Considerations). This section should show the applicability of the benchmarks in relation to the package and its contents, noting all similarities and resolving all differences. References that give full documentation on these experiments should be provided. The overall quality of the benchmark experiments and any uncertainties in experimental data should be addressed. Results of the benchmark calculations, as well as the actual nuclear and geometric input parameters used for those calculations, should be provided. | - RG 7.9, 6.8.1 |  |  |  |
| - 6.8.2 | - Bias Determination - This section should present the results of the benchmark calculations and the method used to account for biases, including the contribution from uncertainties in the experimental data. | - RG 7.9, 6.8.2 |  |  |  |
|  | - This section should show a sufficient number of appropriate benchmark experiments and that the results of the benchmark calculations were appropriate to determine the bias for the package calculations. Parameters such as pitch-to-rod diameter, assembly separation, neutron absorber material performance, etc., should be considered. Statistical and convergence uncertainties should be addressed. | - RG 7.9, 6.8.2 |  |  |  |
| - 6.9 | - Appendix - The appendix should include a list of references, applicable pages from referenced documents (if not generally available), justification of assumptions or analytical procedures, test results, photographs, computer code descriptions, input and output files, and other supplemental information. The following input files should be included in the criticality evaluation for all contents analyzed: - Content only (if included in the SARP) - Content in the containment vessel, i.e., no package—(if included in the SARP) - Single undamaged package for NCT - Single damaged package for HAC - Array of undamaged packages for NCT - Array of damaged packages for HAC. - Note: If the “Worm” program is used to generate Keno input files, please include appropriate examples of the “Worm” input in addition to the Keno input files listed above that were created using the “Worm” program. | - RG 7.9, 6.9 |  |  |  |

| Chapter 7 – Package Operations Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| - Section | - Requirement | - Requirement Basis | - Requirement SARP Location | - Satisfactory? Y/N/NA | - Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| - 7.0 | - Package Operations - This chapter should describe the operations used to load a package and prepare it for transport, presenting the steps sequentially in the actual order in which they are performed. The operations should describe the fundamental steps needed to ensure that the package is properly prepared for transport consistent with the package evaluation in Chapters 2 through 6 of the SARP. - Chapter 7 should demonstrate the ability to comply with operating procedures requirements in 10 CFR 71, Subpart G. - Note: Many of the Chapter 7s of SARPs that have been submitted to DOE/NNSA were written in a format specified by NUREG/CR-4775 (Guide for Preparing Operating Procedures for Shipping Packages). The format provided in NUREG/CR-4775 or other alternate formats are acceptable provided that Chapter 7 contains all of the required information. | - 10 CFR 71, Subpart G - RG 7.9, 7. |  |  |  |
|  | - The package should be operated in accordance with detailed written procedures that are based on and consistent with the operations described in this chapter. | - RG 7.9, 7. |  |  |  |
|  | - Package operations should be consistent with maintaining occupational radiation exposures as low as reasonably achievable (ALARA) as required by 10 CFR 20.1101(b) of 10 CFR 20, “Standards for Protection Against Radiation.” - 10 CFR 20.1101(b): “The licensee shall use, to the extent practical, procedures and engineering controls based upon sound radiation protection principles to achieve occupational doses and doses to members of the public that are as low as is reasonably achievable (ALARA).” | - 10 CFR 20.1101(b) - RG 7.9, 7. |  |  |  |
|  | - The SARP should state that users of the package must develop site-specific operating procedures that have guidance and requirements equivalent to those in Chapter 7 of the SARP. | - DOE Order 461.1A |  |  |  |
| - 7.1 | - Package Loading - This section should describe the loading-related preparations, tests, and inspections of the package, including the inspections made before loading the package to determine that the package is not damaged and radiation and surface contamination levels are within allowable limits of the regulations. | - RG 7.9, 7.1 |  |  |  |
|  | - The SARP must demonstrate compliance with the routine determination prior to each shipment as specified in 10 CFR 71.87. - 10 CFR 71.87: “Routine determinations. Before each shipment of licensed material, the licensee shall ensure that the package with its contents satisfies the applicable requirements of” 10 CFR 71 “and of the license. The licensee shall determine that – | - 10 CFR 71.87 - 49 CFR 173.443 |  |  |  |
|  | - (a) The package is proper for the contents to be shipped; |  |  |  |  |
|  | - (b) The package is in unimpaired physical condition except for superficial defects such as marks or dents; |  |  |  |  |
|  | - (c) Each closure device of the packaging, including any required gasket, is properly installed and secured and free of defects; |  |  |  |  |
|  | - (d) Any system for containing liquid is adequately sealed and has adequate space or other specified provision for expansion of the liquid; |  |  |  |  |
|  | - (e) Any pressure relief device is operable and set in accordance with written procedures; |  |  |  |  |
|  | - (f) The package has been loaded and closed in accordance with written procedures; |  |  |  |  |
|  | - (g) For fissile material, any moderator or neutron absorber, if required, is present and in proper condition; |  |  |  |  |
|  | - (h) Any structural part of the package that could be used to lift or tie down the package during transport is rendered inoperable for that purpose, unless it satisfies the design requirements of §71.45; |  |  |  |  |
|  | - (i) The level of non-fixed (removable) radioactive contamination on the external surfaces of each package offered for shipment is as low as reasonably achievable, and within the limits specified in DOT regulations in 49 CFR 173.443; |  |  |  |  |
|  | - (j) External radiation levels around the package and around the vehicle, if applicable, will not exceed the limits specified in §71.47 at any time during transportation; |  |  |  |  |
|  | - (k) Accessible package surface temperatures will not exceed the limits specified in §71.43(g) at any time during transportation.” |  |  |  |  |
|  | - Note: 10 CFR 71.45 requirements are quoted in Section 2.5 of this checklist; 10 CFR 71.47(a) and 10 CFR 71.47(b) requirements are quoted in Sections 5.1.2 and 5.2.1, respectively, of this checklist; and 10 CFR 71.43(g) requirements are quoted in Section 3.3.1 of this checklist. - Note: One acceptable method of meeting the requirements specified in 10 CFR 71.87(k) is to perform a thermal analysis of the package at steady-state, in the shade (i.e., no solar insolation), in still air of an ambient temperature of 38 °C (100 °F), and using the maximum decay heat load of the proposed content. The heat transfer between the package’s outer surface and its surroundings should be modeled as a combination of natural convection with the ambient air and radiant heat transfer. |  |  |  |  |
|  | - 49 CFR 173.443: “Contamination Control. - (a) The level of non-fixed (removable) radioactive contamination on the external surfaces of each package offered for transport must be kept as low as reasonable achievable. The level of non-fixed radioactive contamination may not exceed the limits set forth in Table 9” of 49 CFR 173.443 “and must be determined by either: |  |  |  |  |
|  | - (1) Wiping an area of 300 cm2 of the surface concerned with an absorbent material, using moderate pressure, and measuring the activity on the wiping material. Sufficient measurements must be taken in the most appropriate locations to yield a representative assessment of the non-fixed contamination levels. The amount of radioactivity measured on any single wiping material, divided by the surface area wiped and divided by the efficiency of the wipe procedure (the fraction of removable contamination transferred from the surface to the absorbent material), may not exceed the limits set forth in Table 9” of 49 CFR 173.443 “at any time during transport. For this purpose the actual wipe efficiency may be used, or the wipe efficiency may be assumed to be 0.10; or |  |  |  |  |
|  | - (2) Alternatively, the level of non-fixed radioactive contamination may be determined by using other methods of equal or great efficiency.” |  |  |  |  |
|  | - Table 9 is located in 49 CFR 173.443. - “(b) Except as provided in paragraph (d) of this section, in the case of packages transported as exclusive use shipments by rail or public highway only, the removable (non-fixed) radioactive contamination on any package at any time during transport may not exceed ten times the levels prescribed in paragraph (a) of this section. The levels at the beginning of transport may not exceed the levels prescribed in paragraph (a) of this section. |  |  |  |  |
|  | - (c) Except as provided in paragraph (d) of this section, each transport vehicle used for transporting Class 7 (radioactive) materials as an exclusive use shipment that utilizes the provisions of paragraph (b) of this section must be surveyed with appropriate radiation detection instruments after each use. A vehicle may not be returned to service until the radiation dose rate at each accessible surface is 0.005 mSv per hour (0.5 mrem per hour) or less, and there is no significant removable (non-fixed) radioactive surface contamination as specified in paragraph (a) of this section. |  |  |  |  |
|  | - (d) Paragraphs (b) and (c) of this section do not apply to any closed transport vehicle used solely for the transportation by highway or rail of Class 7 (radioactive) material packages with contamination levels that do not exceed 10 times the levels prescribed in paragraph (a) of this section if— |  |  |  |  |
|  | - (1) A survey of the interior surfaces of the empty vehicle shows that the radiation dose rate at any point does not exceed 0.1 mSv per hour (10 mrem per hour) at the surface or 0.02 mSv per hour (2 mrem per hour) at 1 m (3.3 feet) from the surface; |  |  |  |  |
|  | - (2) Each vehicle is stenciled with the words ‘For Radioactive Materials Use Only’ in letters at least 76 millimeters (3 inches) high in a conspicuous place on both sides of the exterior of the vehicle; and |  |  |  |  |
|  | - (3) Each vehicle is kept closed except for loading or unloading.” |  |  |  |  |
| - 7.1.1 | - Preparation for Loading - At a minimum, the operations for preparing the package for loading should specify that the package is loaded and closed in accordance with detailed written procedures, that the contents are authorized in the package approval, that the package is in unimpaired physical condition, and that any required moderator or neutron absorber is present and in proper condition. The operations should also include any special controls and precautions for handling. The operations should describe the inspection of gaskets, criteria for replacement, the inspection of each closure device and criteria for replacement, and the processes for repair, of gaskets, if applicable. | - RG 7.9, 7.1.1 |  |  |  |
| - 7.1.2 | - Loading of Contents - At a minimum, the operations for loading the contents should describe how the contents are loaded and how the package is closed. | - RG 7.9, 7.1.2 |  |  |  |
| - 7.1.3 | - Preparation for Transport - The operations for preparing the package for transport should address the radiation and contamination surveys of the package, leakage rate testing of the package, measurement of the package surface temperature, package tiedown, and the application of tamper-indicating devices. | - RG 7.9, 7.1.3 |  |  |  |
| - 7.2 | - Package Unloading - This section should include inspections, tests, and special preparations of the package for unloading. As applicable, this section should also describe the operations used to ensure safe removal of fission gases, contaminated coolant, and solid contaminants. | - RG 7.9, 7.2 |  |  |  |
|  | - Compliance with the requirements of 10 CFR 71.89 should be demonstrated. This section should specify that the shipper must provide any special instructions needed to safely open the package to the consignee in accordance with 10 CFR 20.1906(e). - 10 CFR 71.89: “Opening Instructions. Before delivery of a package to a carrier for transport, the licensee shall ensure that any special instructions needed to safely open the package have been sent to, or otherwise made available to, the consignee for the consignee’s use in accordance with 10 CFR 20.1906(e).” - Note: The 10 CFR 20.1906(e) requirements are quoted in Section 7.2.1 of this checklist. | - 10 CFR 71.89 |  |  |  |
| 7.2.1 | Receipt of Package from Carrier - The process for receiving the package should address radiation and contamination surveys and inspection of the tamper-indicating device(s). Any proposed special controls and precautions for handling and unloading should be described. The appropriate requirements of 10 CFR 20.1906 (“Procedures for receiving and opening packages”) should be addressed. - 10 CFR 20.1906: “Procedures for receiving and opening packages. (a) Each licensee who expects to receive a package containing quantities of radioactive material in excess of a Type A quantity, as defined in §71.4 and appendix A to part 71” of 10 CFR, “shall make arrangements to receive— (1) The package when the carrier offers it for delivery; or | 10 CFR 20.1906 RG 7.9, 7.2.1 |  |  |  |
|  | (2) Notification of the arrival of the package at the carrier’s terminal and to take possession of the package expeditiously. |  |  |  |  |
|  | (b) Each licensee shall— (1) Monitor the external surfaces of a labeled package for radioactive contamination unless the package contains only radioactive material in the form of a gas or in special form as defined in 10 CFR 71.4; (2) Monitor the external surfaces of a labeled package for radiation levels unless the package contains quantities of radioactive material that are less than or equal to the Type A quantity, as defined in §71.4 and appendix A to part 71 of” 10 CFR;” and |  |  |  |  |
|  | (3) Monitor all packages known to contain radioactive material for radioactive contamination and radiation levels if there is evidence of degradation of package integrity, such as packages that are crushed, wet, or damaged. |  |  |  |  |
|  | (c) The licensee shall perform the monitoring required by paragraph (b) of this section as soon as practical after receipt of the package, but not later than 3 hours after the package is received at the licensee’s facility if it is received during the licensee’s normal working hours, or not later than 3 hours from the beginning of the next working day if it is received after working hours. |  |  |  |  |
|  | (d) The licensee shall immediately notify the final delivery carrier and the NRC Operations Center (301-816-5100), by telephone, when— (1) Removable radioactive surface contamination exceeds the limits of §71.87(i) of” 10 CFR; “or (2) External radiation levels exceed the limits of §71.47” of 10 CFR. - Note: 10 CFR 20.1906(d) is not applicable for packages shipped in the TSS. The shippers and receivers work closely with the Office of Secure Transportation elements to communicate issues such as these and others that may arise during the shipment and receipt of packages transported in the TSS. - “(e) Each licensee shall— (1) Establish, maintain, and retain written procedures for safely opening packages in which radioactive material is received; and (2) Ensure that the procedures are followed and that due consideration is given to special instructions for the type of package being opened. - (f) Licensees transferring special form sources in licensee-owned or licensee-operated vehicles to and from a work site are exempt from the contamination monitoring requirements of paragraph (b) of this section, but are not exempt from the survey requirement in paragraph (b) of this section for measuring radiation levels that is required to ensure that the source is still properly lodged in its shield.” |  |  |  |  |
| 7.2.2 | Removal of Contents - This section should describe the appropriate operations and method for opening and removing contents from the package. | RG 7.9, 7.2.2 |  |  |  |
| 7.3 | Preparation of Empty Package for Transport - This section should describe the inspections, tests, and special preparations needed to ensure that the packaging is verified to be empty and is properly closed, and that the radiation and contamination levels are within allowable limits. | RG 7.9, 7.3 |  |  |  |
|  | This section should address the appropriate requirements of 49 CFR 173.428, “Empty Class 7 (radioactive) materials packaging.” - 49 CFR 173.428: “Empty Class 7 (radioactive) materials packaging. A packaging which previously contained Class 7 (radioactive) materials and has been emptied of contents as far as practical, is excepted from the shipping paper and marking (except for the UN identification number marking requirement described in §173.422(a)) requirements of” 49 CFR, “provided that— | 49 CFR 173.428 RG 7.9, 7.3 |  |  |  |
|  | (a) The packaging meets the requirements of §173.421(a)(2), (3), and (5) of” 49 CFR; |  |  |  |  |
|  | “(b) The packaging is in unimpaired condition and is securely closed so that there will be no leakage of Class 7 (radioactive) material under conditions normally incident to transportation; |  |  |  |  |
|  | (c) The outer surface of any uranium or thorium in its structure is covered with an inactive sheath made of metal or some other substantial material; |  |  |  |  |
|  | (d) Internal contamination does not exceed 100 times the limits in §173.443(a); |  |  |  |  |
|  | (e) Any labels previously applied in conformance with subpart E of part 172 of” 49 CFR “are removed, obliterated, or covered and the ‘Empty’ label prescribed in §172.450 of” 49 CFR “is affixed to the packaging; and |  |  |  |  |
|  | (f) The packaging is prepared for shipment as specified in §173.422.” |  |  |  |  |
| 7.4 | Other Operations - This section should include the provisions for any special operational controls (e.g., route, weather, shipping time restrictions, etc.). | RG 7.9, 7.4 |  |  |  |
| 7.5 | Appendix This appendix should include a list of references, copies of applicable references (if not generally available), detailed descriptions and analysis of processes or protocols, graphic presentations, test results, and other appropriate supplemental information. | RG 7.9, 7.5 |  |  |  |

| Chapter 8 – Acceptance Tests and Maintenance Program Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| Section | Requirement | Requirement Basis | Requirement SARP Location | Satisfactory? Y/N/NA | Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| 8.0 | Acceptance Tests and Maintenance Program | RG 7.9, 8. |  |  |  |
|  | - This chapter should describe the acceptance tests and maintenance program to be used for the packaging in compliance with 10 CFR 71, Subpart G. |  |  |  |  |
| 8.1 | Acceptance Tests - Prior to applying the model number to the package, the package owner must determine that the container was fabricated in accordance with the approved design. - 10 CFR 71.85: “Preliminary determinations. Before the first use of any packaging for the shipment of licensed material – | 10 CFR 71.85 |  |  |  |
|  | (a) The licensee shall ascertain that there are no cracks, pinholes, uncontrolled voids, or other defects that could significantly reduce the effectiveness of the packaging; |  |  |  |  |
|  | (b) Where the maximum normal operating pressure will exceed 35 kPa (5 lbf/in2) gauge, the licensee shall test the containment system at an internal pressure at least 50 percent higher than the maximum normal operating pressure, to verify the capability of that system to maintain its structural integrity at that pressure. |  |  |  |  |
|  | (c) The licensee shall conspicuously and durably mark the packaging with its model number, serial number, gross weight, and a package identification number assigned by the” NNSA. “Before applying the model number, the licensee shall determine that the packaging has been fabricated in accordance with the design-approved by” the NNSA.” |  |  |  |  |
|  | This section should describe the tests to be performed before the first use of each packaging. Each test and its acceptance criteria should be described. The acceptance tests should confirm that each packaging is fabricated in accordance with the drawings referenced in the package approval. | RG 7.9, 8.1 |  |  |  |
| 8.1.1 | Visual Inspections and Measurements - This section should describe the visual inspections to be performed and the intended purpose of each inspection. | RG 7.9, 8.1.1 |  |  |  |
|  | The criteria for acceptance of each inspection, as well as the action to be taken if noncompliance is encountered, should be described. | RG 7.9, 8.1.1 |  |  |  |
|  | The inspections should verify that the packaging has been fabricated and assembled in accordance with the drawings, and that all dimensions and tolerances specified on the drawings are confirmed by measurement. | RG 7.9, 8.1.1 |  |  |  |
| 8.1.2 | Weld Examinations - This section should describe welding examinations used to verify fabrication in accordance with the drawings, codes and standards specified in the SARP. The location, type, and size of the welds should be confirmed by measurement. Other applicable specifications for weld performance, NDE, and acceptance should be identified. | RG 7.9, 8.1.2 |  |  |  |
| 8.1.3 | Structural and Pressure Tests - This section should identify and describe the structural and pressure tests. Such tests should comply with 10 CFR 71.85(b), as well as applicable codes or standards specified. The sensitivity of the tests, and the actions taken when the prescribed criteria are not met, should be specified. - Note: The 10 CFR 71.85(b) requirements are quoted in Section 8.1 of this checklist. | 10 CFR 71.85(b) RG 7.9, 8.1.3 |  |  |  |
| 8.1.4 | Leakage Tests - This section should describe the leakage rate tests to be performed on the containment vessel, as well as auxiliary equipment. | RG 7.9, 8.1.4 |  |  |  |
|  | The sensitivity of the tests, including the basis of this value, the criteria for acceptance, and the action to be taken if the criteria are not met, should be specified. | RG 7.9, 8.1.4 |  |  |  |
|  | Compliance with leakage rate test requirements specified in ANSI N14.5 should be addressed. | SG-100, Chapter 4 ANSI N14.5 |  |  |  |
| 8.1.5 | Component and Material Tests - This section should specify the appropriate tests and acceptance criteria for components that affect package performance. In addition, this section should specify test sensitivity (if applicable), provide acceptance criteria, and describe the action to be taken if those criteria are not met. | RG 7.9, 8.1.5 |  |  |  |
|  | This section should specify the appropriate tests and acceptance criteria for packaging materials. Tests should include those components, such as gaskets, under conditions that simulate the most severe service conditions under which they are to perform, including performance at pressure and under high and low temperatures. | RG 7.9, 8.1.5 |  |  |  |
|  | Tests for neutron absorbers (e.g., boron) and insulating materials (e.g., foams, fiberboard) should ensure that minimum specifications for density and isotopic content are achieved. | RG 7.9, 8.1.5 |  |  |  |
|  | Tests that demonstrate the ability of the materials to meet the performance specifications shown on the engineering drawings should be described. | RG 7.9, 8.1.5 |  |  |  |
| 8.1.6 | Shielding Tests - This section should specify the appropriate shielding tests for both gamma and neutron radiation. These tests and acceptance criteria should be sufficient to ensure that no defects, voids, or streaming paths exist in the shielding. - Note: The types of contents currently shipped under the authority of DOE/NNSA generally result in low dose rates and do not require material specific for shielding. | RG 7.9, 8.1.6 |  |  |  |
| 8.1.7 | Thermal Tests - This section should specify the appropriate tests to demonstrate the heat transfer capability of the packaging. These tests should confirm that the heat transfer performance determined in the thermal evaluation (SARP Chapter 3) is achieved in the fabrication process. - Note: The guidance provided in RG 7.9, 8.1.7 is generally not applicable for the types of contents currently shipped in NNSA certified packages. Generally, verification of insulation material properties is sufficient. | RG 7.9, 8.1.7 |  |  |  |
| 8.1.8 | Miscellaneous Tests - This section should describe any additional tests to be performed prior to use of the packaging. | RG 7.9, 8.1.8 |  |  |  |
| 8.2 | Maintenance Program - This section should describe the maintenance program used to ensure continued performance of the packaging. This program should include periodic testing, inspection, and replacement schedules, as well as criteria for replacement and repair of components and subsystems on an as-needed basis. | RG 7.9, 8.2 |  |  |  |
| 8.2.1 | Structural and Pressure Tests - This section should identify and describe any periodic structural or pressure tests. Such tests would generally be applicable to codes, standards, or other procedures specified in the application. | RG 7.9, 8.2.1 |  |  |  |
| 8.2.2 | Leakage Tests - This section should describe the tests to be performed, the frequency with which those tests are performed, and the sensitivity of each test. For most systems, this description should include a test of the package before each shipment and annually. In general, this section should specify that elastomeric seals should be replaced and leakage rate tested within the 12-month period before shipment, and that metallic seals should also be replaced and tested before each shipment. The SARP should demonstrate that the prescribed leakage rate tests are in accordance with ANSI N14.5. | RG 7.9, 8.2.2 SG 100, Chapter 4 ANSI N14.5 |  |  |  |
| 8.2.3 | Component and Material Tests - This section should describe the periodic tests and replacement schedules for components. Any process that could result in the deterioration of packaging materials, including loss of neutron absorbers, reduction in hydrogen content of shields, and density changes of insulating materials, should be addressed. Replacement intervals for components, such as bolts, that are susceptible to fatigue should be specified. | RG 7.9, 8.2.3 |  |  |  |
| 8.2.4 | Thermal Tests - This section should describe the periodic tests used to ensure heat-transfer capability during the service life of the packaging. This section should describe periodic thermal tests, similar to the acceptance tests discussed in Section 8.1.7, and the interval for the tests, which is typically 5 years. - Note: The guidance provided in RG 7.9, 8.2.4 is generally not applicable for the types of contents currently shipped in NNSA certified packages. Generally, verification of insulation material properties is sufficient. | RG 7.9, 8.2.4 |  |  |  |
| 8.2.5 | Miscellaneous Tests - Any additional tests to be performed periodically on the package or its components should be described. | RG 7.9, 8.2.5 |  |  |  |
| 8.3 | Appendix - The appendix should include a list of references, copies of applicable references (if not generally available), test data and reports, and other appropriate supplemental information. | RG 7.9, 8.3 |  |  |  |

| Chapter 9 – Quality Assurance Safety Analysis Report for Packaging Completeness Review Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name(s) of Reviewer(s): |  |  |  |  | Date of Review: |  |  |  |
| SARP No: |  |  | Title: |  |  | Date: |  |  |

| Section | Requirement | Requirement Basis | Requirement SARP Location | Satisfactory? Y/N/NA | Comment Ref. No. |
| --- | --- | --- | --- | --- | --- |
| 9 | Quality Assurance - DOE contractors are required to develop and implement a Quality Assurance Program Plan (QAPP) that meets the requirements of 10 CFR 71, Subpart H for packaging design through certification and use, as applicable; QC-1 for product acceptance; and 10 CFR 830 (Nuclear Safety Management) for facilities and records. The Plan must also comply with the guidance provided in SG 500. The SARP should provide a description of the QA Program in place. The QAPP would not normally be part of the SARP. | 49 CFR 173.7(d) 10 CFR 71.37 10 CFR 71, Subpart H 10 CFR 830 SG 500 DOE Order 461.1A |  |  |  |
|  | 49 CFR 173.7(d): “Notwithstanding the requirements of §§173.416 and 173.417 of” 49 CFR, “packagings made by or under the direction of the U.S. Department of Energy may be used for the transportation of Class 7 materials when evaluated, approved, and certified by the Department of Energy against packaging standards equivalent to those specified in 10 CFR part 71. Packages shipped in accordance with this paragraph shall be marked and otherwise prepared for shipment in a manner equivalent to that required by” 49 CFR “for packagings approved by the Nuclear Regulatory Commission.” - 10 CFR 71.37: “Quality assurance. |  |  |  |  |
|  | (a) The applicant shall describe the quality assurance program (see Subpart H” of 10 CFR 71) “for the design, fabrication, assembly, testing, maintenance, repair, modification, and use of the proposed package. |  |  |  |  |
|  | (b) The applicant shall identify any specific provisions of the quality assurance program that are applicable to the particular package design under consideration, including a description of the leak testing procedures.” |  |  |  |  |