BROOKHAVEN NATIONAL LABORATORY

PROCESS ASSESSMENT FORM

I.	General Information

| Process ID: | SM-551-ESS |  |  |  |
| --- | --- | --- | --- | --- |
| Process Name: | Employee Service Station |  |  |  |
| Process Flow Diagrams: | SM-551-ESS-01 |  |  |  |
| Process Description: | This process includes the on-site automotive service station and fueling operations for BNL employee vehicles. The following activities are included under the Employee Service Station Operations: minor repairs, oil changes, engine fluid replacement, tire replacement, battery replacement, etc. The fueling operation is attendant service on either regular, plus or super unleaded gasoline. Section II and the above referenced Process Flow Diagram provide more details on the Employee Service Station. The environmental aspects associated with this process include: regulated industrial waste generation, hazardous waste generation, atmospheric discharges, and storage and use of chemicals. The relevant subject areas include: hazardous waste management, oil management, pollution prevention and waste minimization, spill response, and storage and transfer of hazardous. |  |  |  |
| Dept./Div.: | Finance & Administration Division |  |  |  |
| Dept. Code: | SS |  |  |  |
| Building(s): | 630 |  |  |  |
| Room(s): | N/A |  |  |  |
| Point of Contact: | K. Mohring | x2526 |  |  |
| Prepared by: | P. D. Pohlot | Reviewed by: | ##### J. Swenson | K. Mohring |
| Approved by: |  | J. Swenson | ##### Date: |  |

II.	Detailed Process Descriptions and Waste Determination

##### The BNL Employee Service Station performs automotive repairs, NYS vehicle inspections and fueling services for BNL employees and guests.  Operations include engine repair, engine fluid replacement, tire replacement and battery replacement.  These activities are conducted by the Employee Service Station  (ESS) in the vehicle maintenance shop in Building 630. The service shop consists of four bays (three of which are equipped with hydraulic lifts), a spare parts storeroom and an office area. Bodywork and/or painting are operations not conducted by the ESS.  The activities of BNL Employee Service Station have been graphically depicted in Process Flow Diagram SM-551-ESS-01, provided in attachment 1

Vehicle repair and maintenance involves the replacement of broken or worn engine components.  The ESS removes, repairs, and installs engine parts.  Metal components that cannot be reserviced (broken or worn parts, mufflers, etc.) are disposed of as scrap metal.  Some electrical parts may be sent off-site for reservice and reinstallation.

Engine fluids (oil, water, antifreeze, washer fluid, etc.) are replaced as needed to insure proper operation and emission control.  Used oil is stored in a satellite 55-gallon drum.  When the drum is full it is emptied into a 450-gallon storage tank located in back of the service area in Building 630.  In agreement with a local laundry cleaning operation (Sterebel’s), the used oil is hauled off-site and used as fuel.  Used oil filters are sent to building 423 (Facility Vehicle Maintenance Shop) for crushing and are then directed to the HWMF for disposal.  Used antifreeze is also stored in a 55-gallon drum.  When full, the drum is emptied into an outside vendor’s truck and taken off-site for disposal.  Unused oil and antifreeze are contained in 1-quart and 1-gallon containers, stored in the parts room and the trailer located adjacent to the service station.

After replacement, used tires are put in a dumpster outside before removal by Plant Engineering’s Custodial Services.  Removal and recycling of used tires is discussed in detail in PAF EP-330-CS.

Wetcell batteries are picked up and taken off-site for recycling or disposal by an outside vendor (NAPA).

A System Onetm parts cleaner is located in the service shop.  Greasy engine parts and tools are placed on a tray in the cleaner, which is filled with a petroleum solvent.  Over time, the solvent fluid becomes contaminated with oil and dirt is condensed and recycled by the parts cleaner.  The subsequent dirty oil and sludge is removed and mixed with waste oil.  Various shop cleaning items in aerosol cans are used with reusable cloth towels.  Empty aerosol cans are recycled through the Town of Brookhaven as low grade metal and the used towels are cleaned by an outside vendor (Cintas) and reused.

The refueling operation is manual attendant service on either regular, plus or super unleaded gasoline.  The pumps are equipped with VOC recirculation nozzles.  The tanks are three (3) 8,000-gallon underground tanks, equipped with double walled construction, leak detection and a computerized system status reporting function.

Regulatory Determination of Process Outputs

- Employee Service Station Operations

The ESS performs all repair and maintenance operations for BNL employee and guest vehicles.  The Service Shop contains a variety of electric power tools and equipment.  All waste streams generated by the Service Shop for Vehicle Maintenance Operations are listed below.

| Waste ID | Waste Description | Determination/Basis | Waste Handling | Corrective Action Required |
| --- | --- | --- | --- | --- |
| 1.1 | ## Broken Parts | Non-hazardous waste as determined by process knowledge | Waste is discarded as scrap metal | ### None |
| 1.2 | ## Broken Tools | Non-hazardous waste as determined by process knowledge | Waste is discarded as scrap metal | ### None |
| 1.3 | Used Oil | Non-hazardous waste as determined by process knowledge | Waste is hauled and used off-site by outside source | None |
| 1.4 | ## Used Oil Filters | Non-hazardous waste as determined by process knowledge | Waste is sent to the HWMF for disposal | ### None |
| 1.5 | Used Antifreeze | Non-hazardous waste as determined by process knowledge | Hauled off-site for recycling | None |
| 1.6 | ## Used Tires | Non-hazardous waste as determined by process knowledge | Waste is hauled off-site by outside contractor | ### None |
| 1.7 | ## Used Wetcell Batteries | Hazardous waste as determined by process knowledge | Waste is hauled off-site by outside contractor for recycling under the Universal Waste Rule | ### None |
| 1.8 | Used Parts Cleaner Petroleum Solvent | Non-hazardous waste as determined by process knowledge | Recycled within the system Onetm parts cleaner | None |
| 1.9 | ## Empty Aerosol Cans | Non-hazardous waste as determined by process knowledge | Waste is collected and is sent to the TOB for recycling | ### None |
| 1.10 | ## Non-empty Aerosol Cans | Hazardous waste as determined by Regulatory guidelines | Waste is sent to the HWMF for disposal | ### None |
| 1.11 | ## Oily Rags | Non-hazardous waste as determined by process knowledge | Waste is sent to the HWMF for disposal | ### None |
| 1.12 | ## Used Towels | Non-hazardous waste as determined by process knowledge | Towels are cleaned and reused | ### None |
| 1.13 | ## Used Speedy Dry | Non-hazardous waste as determined by process knowledge | Waste is sent to the HWMF for disposal | ### None |
| 1.14 | Deminimus releases of CFC’s/HCFC’s from auto refrigerant recovery/recycling devices | Controlled substance - used with proper controls | Released to ambient air | None |
| 1.15 | ## Vapors from parts cleaning tank | Non-hazardous waste as determined by process knowledge | Released to ambient air | ### None – tank lid is kept closed when not in operation |

- Vehicle Fueling Operations

The Service Station provides fueling services.  The facility is not a self-serve operation.  An attendant operates the pumps and pumps the requested amount of fuel into the vehicle. The pumps are equipped with VOC recirculation nozzles and automatic shut off handles, however minimal amounts of VOCs escape during the refueling operation.  The pumps are equipped with inline filters, which get changed every 6-months.  The spent filters are crushed and sent to the HWMF for disposal.

| Waste ID | Waste Description | Determination/Basis | Waste Handling | Corrective Action Required |
| --- | --- | --- | --- | --- |
| 2.1 | ## Gasoline vapors | Hazardous waste as determined by process knowledge | Release is minimized by current applied technology | ### None |
| 2.2 | ## Inline Fuel Filters | Non-hazardous waste as determined by process knowledge | Waste is discarded as Industrial Waste | ### None |

###### III.	Waste Minimization, Opportunity for Pollution Prevention

None

IV.	Assessment Prevention and Control

- Vehicle and Equipment Maintenance

The three hydraulic lifts operate with their own hydraulic reservoirs. The hydraulic fluid they contain is probably DT light.  The service station should look into the economic feasibility of using a vegetable-based hydraulic fluid.  This would include looking into the manufactures specifications and warranties for use of alternative hydraulic fluids.

Some hydraulic equipment has been switched over to the vegetable based hydraulic fluid and its integration is being evaluated.

During calendar year 2001 there was a hydraulic leak detected in Bay #3 lift.  This was the only remaining hydraulic lift whose ancillary piping and cylinder were under ground. The replacement system was installed above ground.  Pending evaluation of the switchover to canola based hydraulic oil at the Motor pool shop, SS will evaluate a comparable switchover for the Employee Service Station.

ATTACHMENT 1

PROCESS FLOW DIAGRAM