Industrial Hygiene Findings and Recommendations

- Development of a written plan and compliance with the Hazard Communication (HAZCOM) Standard (29 CFR 1910.1200) should be a priority at the Museum Resource Center.  Hazard Communication focuses on direct and indirect issues relating to chemical exposure.  Employees who handle artifacts should understand that a potential health hazard may exist from exposure to known chemicals applied to artifacts, and also to unknown chemical application.  The Museum Resource Center did not have a written hazard communication program.

Implement the Hazard Communication Program. The basic goal of a HAZCOM Program is to ensure that employees are aware of their “right to know” the identities and the hazards of the chemicals that they are exposed to in the workplace, and how to protect themselves from such hazards.  There are several elements to HAZCOM and all elements must be implemented for the program to be effective and compliant.

Elements of the Hazard Communication Program include:

- Written Hazard Communication Program

- Material Safety Data Sheets (MSDS).  MSDS’s must be obtained and maintained for each chemical product on site and known chemicals that have been applied to artifacts.  The MSDS must be reviewed at the time the chemical is brought on site and/or prior to use.  The MSDS is the first step toward protection as it lists the components of the chemical and the hazards associated, the personal protective equipment recommended, first aid procedures, etc.  In addition, the MSDS must be readily available to employees for review.

- A Chemical inventory must be developed for all chemicals and maintained on site.  The inventory must be updated as chemicals are purchased and/or depleted. It is recommended that a centralized purchasing program be established for the park to monitor chemical purchases and avoid duplicate orders.  Chemicals should be ordered in the amount to be used to avoid long term storage and disposal issues.

- Chemical labeling of all chemicals to indicate the identity of the material.  Manufacturer labels are sufficient, however, if the label is defaced the chemical container must be re-labeled to indicate the chemical name, manufacturer, hazard category, and target organs.  Chemicals transferred to secondary containers must also be labeled and should only contain the information for the chemical in the container.  The only exception to the requirement for labeling of secondary containers is if the chemical in the secondary container will be under complete control by the employee who made the transfer and will be used in its entirety by that employee during that work shift.  Labeling is recommended for secondary containers (even when the intent is to use the chemical immediately) to eliminate the possibility of chemicals being left unattended in the event of an emergency.

- Employee Information and Training must be provided to all employees who use hazardous chemicals or may be exposed to them and documentation of training must be maintained.  Information and Training must be provided at the time of their initial assignment, whenever a new physical or health hazard is introduced into the workplace, or when there is evidence that training is needed.

- Compliance with the Personal Protective Equipment (PPE) standard (29 CFR 1910.132 –General Requirements, .133 – Eye and Face Protection, .138 – Hand Protection) is essential for employee protection from occupational hazards. Employees at MRC handle various artifacts during museum preservation work that may be encrusted with dust, dirt, surface scrapings, and unknown chemicals.  Dermal contact with these materials presents not only a contact hazard (dermatitis), but may also present an inhalation hazard.

Develop and maintain a PPE program.  The purpose of a PPE program is to protect employees from the risk of injury by creating a barrier against workplace hazards. Personal protective equipment is not a substitute for good engineering or administrative controls or good work practices, but is used in conjunction with these controls to ensure the safety and health of employees. Elements of an effective PPE program include:

- Hazard Assessment and equipment selection.

- Written certification of hazard assessment.

- Training to recognize what PPE is necessary, when PPE is necessary, how to properly wear PPE, the limitations of the PPE, and the proper care, maintenance, useful life and disposal of PPE.   It is important that all PPE be kept clean and properly maintained. Cleaning is particularly important for eye and face protection where dirty or fogged lenses could impair vision. PPE should be inspected, cleaned, and maintained at regular intervals so that the PPE provides the required protection. Personal protective equipment shall not be shared between employees until it has been properly cleaned and sanitized.

The types of PPE and some engineering, administrative, and/or good work practice guidelines include:

- Nitrile gloves while handling artifacts.  When removing the gloves, do not touch the exterior surface of the gloves.

- Wash hands with soap and water after handling objects, and especially before eating, drinking or smoking.

- Wear a lab coat or other protective clothing to keep dust off clothing.  Do not wear coat outside work area.

- Ensure that there is adequate ventilation when working with materials.

- Do not permit eating and/or drinking in store room or work area.

- Ensure that work surfaces are well cleaned after they have been in contact with artifacts.