POLAR Spacecraft Status As of: Weds, 28 Jul 04  16:00 EST

(changes from last week in bold border)

| Subsystem/ Component | NOAA-12D AM: AN=1644Z STIP data only | NOAA-14J AM: AN=1930Z STIP data only | NOAA-15K AM: AN=1837Z GAC data only | NOAA-16L PM: AN=1437Z GAC&LAC data | ##### NOAA-17M AM: AN=2220Z GAC&LAC data | ##### NOAA-N18 Planned PM Plan AN=14:00L |
| --- | --- | --- | --- | --- | --- | --- |
| LAUNCHED | 05/14/91 | 12/30/94 | 05/13/98 | 09/21/00 | 6/24/02 | Planned NET 2/11/05 |
| CNTL OBP | OBP2 | OBP2 OBP1 inoperative | OBP2 | OBP2 | OBP2 CPU1&2 Errr TOAR |  |
| BUS | B-BUS XSU-1 | B-BUS XSU2 (XSU1 inop) | B-BUS XSU-1 | B-BUS XSU-1 | B-BUS XSU-1 |  |
| BUS VETO | NO | NO | NO | NO | NO |  |
| ADACS X=Yaw Y=Roll Z=Pitch | Nominal Mode RGmode=Monitor Gyro chans=XYZ | Nominal Mode Rgmode=Disabled Gyro chans=XYZ SS check Disabled when mode=YGC | Nominal Mode Rgmode=Monitor Gyro-3 Off 6/26/00 Gyro chans=AAB N2 Low PSI invalid | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAA Gyro3 TOAR 427 | ###### Nominal Mode Rgmode=Monitor Gyro-3 Off 6/10/04 Gyro chans=AAB Gyro3 TOAR 436 | New ring laser gyro |
| RXO | PRI BU random enables daily bias+6.5 mS | PRI BU random enables daily bias–7.0 mS | PRI daily bias -8 mSec | PRI Daily Bias –5.5 mS | PRI Daily Bias –3 mSec |  |
| CLOCK DIV | 2 | 2 | 2 | 2 | 2 |  |
| MIRP | External Synch Rephase Enabled | External Synch Rephase 0730 Daily | External Synch Rephase 0730 daily | External Sync Rephase each orbit | External Sync Rephase Enabled |  |
| TIP SIDE | 1 | 1 | 2 (since 4/23/03) Channels 224 -227 invalid TOAR 432 Dwell test 7/26 | 1 Analog telem drifting | 1 Analog telem drift |  |
| DTRs | 3, 4, 5B Safestate 3B since 9 Jun | 1, 2B, 3A, 5 Safestate- 1B | 1, 2, 3, 4 Safestate 1B EOT | 1,2,3,4A Safestate 3A 4B TOAR 418 (STIP only) | 1,2,3,4 Safestate=1B DTR2-SSR DTR5 TOAR 420 | All solid state digital recorders |
| VTX (APT data) | #1 ON 137.50 MHZ | OFF Since 8/14/02 | #1 ON Turned on 7/7/04 | INOP OFF since 11/15/00 | #2 ON 137.62 MHZ |  |
| BTX | #1 136.77 MHZ | #2 137.77 MHZ | #1 137.35 MHZ | #1 137.35 MHZ | #2 137.77 MHZ |  |
| STX #1=1698.0 mhz #2=1702.5 mhz #3=1707.0 mhz #4=2247.5 mhz | 1 – HRPT; 2,3 - Playback | 3 – HRPT 2, 1 – Playback | 2-Omni HRPT 4-Playback 1,2,3-Degraded | 1 – HRPT; 2– Playback; 4 – Playback 3 low power unused | 3 – HRPT TOAR424 low power 1&2 Playback 4 – TIP |  |
| ### POWER (Array offset, charge and eclipse states, sun angle) | ArrayOff -50 since 7/27/04 Battery Undervol t event 25Jul. Sun Angle 36 down to 15 mid-Oct Eclipse Season peak 7/1 ends 8/19 Shunt degraded Batts 1&2=LRC V/T=4 (26Jul) | Array Off -55 Since 8/21/03 Eclipse all year Sun Angle at 40 steady trend Batts 1-3= LRC Batts 1&3V/T=8/11 Batt2 VT=9/11 ) Shunt Degraded | Array Off -45 Since 1/25/01 Eclipse Season Peak 15 June ends 28 July Sun Angle 30 down trend to 8 in early Oct Batts 1-3= LRC V/T=7/9 | Array Off -40 Since 7/16/02 Eclipse all year Sun Angle 68 Steady Trend Batts 1-3= LRC V/T=4/8 | Array Off. –40 Since 1/29/04 Eclipse all year Sun Angle at 70 Steady trend Batts 1-3= LRC V/T= 4/8 |  |
| AVHRR | Nominal UVTRIP turned off power 7/25 at 23:15Z restored 7/28 at 18:00Z | INOP Scan motor stalled 14 Apr 04 | Nominal Rare Scan motor surges. Last Incident 8/9/03 | DEGRADED Typically erratic performance/quality TCE24 OFF 4/14 Chan 3B only. | ### Nominal 3A-B switch enabled | New brushless scan motors |
| AMSU-A1 |  |  | Operational Ch14 inoperative Ch11 inoperative | Operational TOAR 415 on primary PLLO chans 9-14 bias shift. TOAR 417 on Backup PLLO inop Diffuser test 7/16. negative | OFF/INOP Since 10/30/03 Inoperable Red in status. Survival heat on. |  |
| AMSU-A2 |  |  | Nominal | Nominal Space Position-2 | Nominal |  |
| AMSU-B |  |  | Operational Bias in All Chans. Motor surges ,most recent 5/19/03 IPD Reports of degraded data on Chan-3 since 4/30 | Operational Space Position-3 IPD reports channel 8 degradation | Nominal |  |
| ### HIRS | INOP Filter Wheel turned OFF 10/17/02. Electronics OFF 7/25 because of UVTRIP | NOMINAL Operational Occasional loss of Fwheel synch | VARIABLE FW mode=Hi Powr FW Heater OFF Frequent Filter Motor current “mini” surges, most recent 7/11/04 Occasional major FM current surges, most recent 6/3/04 | Operational Occasional major FM surges, most recent 4/29/04 IPD Reports of excess noise in space view channels 21May04 | NOMINAL 1 Pixel cross track misalignment . |  |
| ### MHS/MIU |  |  |  |  |  | At factory Replaces AMSUB |
| MSU | INOP Scan Motor OFF since 3/12/03 Electronics OFF 7/25 )UVTRIP). | Operational Scan motor and antenna problems |  |  |  |  |
| SSU |  | Nominal |  |  |  |  |
| DCS | Nominal | Nominal | Nominal | Nominal | Nominal |  |
| SARR |  | Nominal | Operational A-Side All AGC Mode Intermittent failures for 243MHz A&B | Operational A-side All AGC Mode (B-side in FG mode) 243 MhZ = INOP. | Nominal A-side All AGC Mode |  |
| SARP |  | OFF Power Failure | Nominal RCVR B/W = 2 | Nominal RCVR B/W = 2 | Nominal |  |
| SEM | Nominal Elec-CH-HVPS=4 Prot-CH-HVPS=0 UVTRIP turned off power 7/25, Plan restored 7/29 | Operational Elec-CH-HVPS=0 Prot-CH-HVPS=0 1 of 4 Telescopes inoperative | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=0 | Nominal E-CDEM-HVPS=5 P-CDEM-HVPS=4 | Nominal E-CDEM-HVPS=2 P-CDEM-HVPS=3 |  |
| SBUV |  | Degraded Grating motor stuck at position –930 on 7/8/04 recovery ops successful 7/9 Flex memory 3 (giant step) ops suspended indefinitely on 12/2/03 Reduced operations started 6/5/03 |  | Operational Degraded Ozone data reported TOAR 435 Primary & Backup diffuser =TOAR 426 and TOAR 433 Both have Antenna deploy problems AMSU EMI causing erratic Behavior in PMT cathod, range 3 | Nominal Anode Mode |  |
| SADPOS |  |  | N/A | ON Enabled MFactor=50% | ON ENABLED |  |
| THERMAL Heaters, louvers TCE’s | Nominal UVTRIP on 25Jul powered off and cycle powered various TCE’s. Nominal config restored 27Jul | Nominal | TCE26 & 15H Telem invalid TOAR 432 HIRS FW Heat OFF TCE24L&25L (HIRS & AVHRR) failed 5/13/98 RCE thermostat failed 5/13/98 | Nominal TCE24 OFF 4/14 (AVHRR H&L) | TCE3H Batt 2A heater failed ON, turned off 8/26/03 TOAR 428 AMSU-A1 survival heaters on 10/29/03 |  |
|  | N12 | N14 | N15 | N16 | N17 | ### NOAA-N18 |

COMMENTS/CHANGES:

NOAA-12	On 25 July at 23:18Z battery 2 voltage on N12 fell to 19.05 volts while exiting its orbital eclipse.  This value was

below the 19.25 volts threshold for triggering autonomous power survival safestate and STESM routines on the spacecraft.  (Note: Battery 1 also fell to a similar value at the same time but it was confirmed in telemetry that battery 2 actually triggered the safestate mode. N12 has only 2 batteries.)  When this trigger was reached, Safestate mode autonomously induced the spacecraft to perform an electrical power load shed that include 62 items including the only two remaining operational payloads on N12;  SEM and AVHRR.  In addition, this safing routine changed the N12 battery V/T levels to 2 (from 4) stopped the recursive stored command table from running and powered off most TCE’s then began cycling power on and off to some of the more critical TCE’s.  A number of interrelated situations contributed to this anomaly including the following:  (1) The relatively minimal amount of engineering data available for analysis from N12 because of its limited number of daily supports.   (2) The possibility that the battery cells have degraded over the past year.  (3)  Concerns about degraded shunt performance and possible shunt failure in July 2003 that resulted in the need to reduce power generation by backing off the solar array offset to -60 degrees (shunt failure is a mission ending anomaly, refer to N10).  Shunt load reduction via reduced power generation was augmented by concurrently maintaining an increased power demand on the bus (and off the shunts) by keeping two inoperative payloads (HIRS and MSU) powered on for use as “pseudo resisters”.  This configuration did indeed significantly reduce shunt stress during the past year but slowly led to a lower than optimum charge rate during the N12 eclipse season (which started in mid-April, peaked in early July 04 began to wane in mid-July and ends in mid-August).  During this eclipse season this reduced charge rate and increased consumption resulted in orbital minimum battery voltages (which occur just before exiting darkness) that while very stable and predictable, were less than 0.3 volt above the safe state trigger threshold, therefore providing a minimal margin for error in the event of unexpected power demands.  (4)  A Wallops pass on 25 July at 22:00Z where this margin was breached when commanding problems at the CDA resulted in an inability to turn off STX-2 before the spacecraft faded.  This additional load was enough to bring the minimum voltage down to trigger the safestate situation which was then detected on the next scheduled pass on 26 Jul at 10:25Z.

Recovery operations began on 26 Jul with a team of NOAA, NASA and EMOSS engineers performing in depth analysis from data obtained on a significant number of call up passes and discussing various operational options.  The following summarizes the options that were implemented:  At 18:30Z, the V/T level was commanded back to its nominal value of 4 (from 2).  On 26 Jul at 20:10Z the Safe State trigger word was reset and the recursive table re-initiated.    On 27 Jul at 13:15Z the TCE cycling process was stopped and on the next pass at 14:55Z the all TCE’s were commanded back to their nominal configuration.  Throughout these passes STIP recorders were dumped and engineering data analyzed to assess the impact of each operation on the power situation which ultimately drove the next reconfiguration decision.  On 27 July at 19:55 the solar array offset was decreased to -50 degrees (from -60) in order to increase power generation.  On 28 July at 18:05Z, the AVHRR was powered back on with imagery returning to normal (ILLUSTRATION 1) and the triggers for battery undervoltage trip were lowered to 18.7 (from 19.25 volts) thus providing additional margin for unexpected power draws (like STX-2 remaining on).  Additional analysis will be conducted before the SEM will be powered back on but that is not expected to occur until 29 July at the earliest.  Into the foreseeable future, turning HIRS and MSU back on for power consumption purposes is not expected although this too may change as N12’s eclipse season comes to an end in the next 2-3 weeks.  EMOSS engineers Jon Woodward, Jim Walters, Carl Gliniak Angelique Riley, Jim Sheperd and Jeff Devine were all involved in these recovery operations.  While the spacecraft and its two operational payloads can still be considered relatively nominal at the present time (notwithstanding additional recovery operations and 15 years on-orbit) operational concepts regarding N12 power balance, shunt degradation and battery capacity concerns remain to be completely worked out which may be further complicated by the impending end of eclipse season.  Once eclipse season ends, concerns about Attitude Control operational concepts during low sun angle season (starting early Sept) will also need to be addressed.  In response to this anomaly, and the acknowledged importance of the N12 AVHRR, scheduling will be increasing the number of N12 passes to 4 per day (from its current 1-2) effective immediately.

NOAA-14:	Prior to the N12 power anomaly, on Thursday, 22 July, EMOSS engineers Jeff Devine and Carl Gliniak detected a “power starvation” potential on N14 that was/is similar to that on N12.  In both cases, the solar array has been “backed off” significantly and inoperative payloads have been left on as power draws in order to protect a degraded shunt against overloading. (N14 actually experienced a major anomaly because of overloaded shunt in April of 2003.)  In the case of N14, which became a permanently eclipsing spacecraft in April of 2004, it has 3 batteries (vs N12’s two) with which to provide the extra power when needed and those batteries have maintained a better “margin or error” of at least .75 volt above undervoltage threshold since it began eclipsing (N12 was approx 0.25).  Even though N14 is maintaining a better margin of error and has an extra battery, the concern regarding N14 is that it is now operating at record low battery voltages and will continue eclipsing with lower power generation and higher loads than when first reconfigured in May 2003.  The concern is that this could lead to a chronic undercharging that could subtly lead to increased undervoltage trip potential.  Therefore, EMOSS engineering have requested additional analysis by NASA engineers and solicited their opinion regarding the necessity of increasing power, decreasing loading, changing VT or lowering the UV Trip threshold number (which is current 19.4) to 19.0.  Carl Gliniak is POC for this issue.  See illustration 2-3.

NOAA-14:	(cont)

Because of concerns about SBUV giant step operations as a source of the grating motor becoming stuck, the giant step operation has been permanently removed from the stored command table.  No other reconfigurations or plans regarding this payload are currently under discussion with EMOSS engineers.  Jim Walters is POC.

NOAA-15:	In support of TOAR 432 investigation (TIP channel 224-227 failure), the EMOSS team instituted an operation which

performed one second dwells for one orbit each on TIP-2 channels 224, 225, 226, 227, 228, 229, 230, and 231.  The EMOSS team had long ago detected and confirmed that channels 224, 226 and 227 had failed while 228-231 remained nominal.  Since all of these channels are on the same mux the primary question for investigating this anomaly is whether channel 225 is working or not.  The main problem with this determination is that channel 225 does not have any associated telemetry.  Therefore, it is hoped that by using the 1 second dwell mode and detecting any changes of any kind across this channel theories as to the origin of this anomaly can be pursued or eliminated.  This operation consisted of generating and loading a macro to place each channel in 1 second dwell mode for one orbit then switching to the next channel.  This was done over a single JDAY (209).  Jon Woodward and Angelique Riley were EMOSS  POC’s for the operation.

The N15 AMSU-B were nominal during the past reporting period.  However, the N15 AVHRR scan motor experienced a current surge of 15-20 mAmps on 25 July starting at approximately 11:30Z and the current remains at this elevated level.  Imagery does not appear to have been impacted and no reports of imagery degradation have been received by EMOSS engineers and synch deltas remain relatively stable.  N15 AVHRR component temps are in a strong uptrend which is seasonal in nature and has not yet approached record high levels.  EMOSS will be maintaining a close watch on this subsystem in the coming days.  Jim Walters is POC.  See illustrations 4-6

The N15 HIRS continued its typical pattern of experiencing mini filter motor current surges followed by a recovery after several days.  No reports of imagery degradation have been received by EMOSS engineers.  Jim Walters is POC for this issue (illustration 7).

N15 eclipse season is rapidly waning, battery voltages are nominal at this time (illustration 8)

NOAA-16:	 N16 AVHRR remained erratic and generally more elevated than in previous weeks with numerous reports of bar codes being received and detected by EMOSS engineers and pegged synch delta values at all times since 15 July.  Conversely, the AVHRR component temperatures remain generally stable and nominal.  The MIRP rephrase operations performed once per orbit continue to be very successful at increasing the percentage of good imagery over the previous method of rephasing one per day.  However, as long as the scan motor current remains elevated and synch delta always pegged imagery will be impacted.  (See illustrations 9-12.)

NOAA-17:	No changes since previous report.  A new matrix for CPU single event upsets is being generated and the information

will be included in this report as soon as it is completed.

NOAA-N	No changes since previous report.

MISC:	New computers were received by EMOSS engineers this week.  They will take some getting used to….

This is the final weekly report to be published under the EMOSS-1 contract

28 July 19:30Z

Illustration 1:  N12 HRPT Image following 28 July AVHRR turn-on (back to nominal operations)

Illustration 2:  N14 Battery Voltage Analysis

Illustration  3:  N14 Battery Trend Analysis

(Record low sustained minimum voltage which will continue indefinitely since eclipse season is now permanent)

Illustration  4:  N15 AVHRR Scan Motor & Imagery Analysis during scan motor surge

J-208			   		J-209				J-210

12:00Z				21:45Z			16:40Z

Illustration 5:  N15 AVHRR Synch Delta Analysis Recent and Past

(generally stable during this most recent scan motor current surge)

Illustration 6:  N15 AVHRR Component Temps Analysis

(A strong but seasonal uptrend with several degrees to go before reaching new highs)

Illustration 7:  N15 HIRS Analysis (yet another mini surge)

Illustration 8:  N15 Battery Voltage Analysis

(improving daily as eclipse season wanes.)

Illustration 9:  N16 AVHRR Scan Motor Analysis

(somewhat degraded since previous report)

28 Jul		28 Jul 					28 Jul

02:00Z		07:30Z					17:30Z

Illustration 10:  N16 AVHRR Imagery Analysis

(would be a lot worse if MIRP rephasing wasn’t done each orbit)

Illustration 11:  N16 Synch Delta Analysis

(pegged for a couple of weeks, impacting imagery

Illustration 12:  N16 AVHRR Thermal Analysis

(stable and nominal)

# POLAR SPACECRAFT TIP CLOCK ERROR

## WEEK ENDING July 23, 2004

| DAY/DATE | NOAA-12 | NOAA-14 | NOAA-15 | NOAA-16 | NOAA-17 |
| --- | --- | --- | --- | --- | --- |
| FRI. 07/16 | -559 msec | -026 msec | -045 msec | +033 msec | -005 msec |
| SAT. 07/17 | -558 msec | -028 msec | -046 msec | +032 msec | -005 msec |
| SUN. 07/18 | -557 msec | -026 msec | -047 msec | +033 msec | -007 msec |
| MON. 07/19 | -558 msec | -025 msec | -048 msec | +032 msec | -007 msec |
| TUE. 07/20 | -557 msec | -024 msec | -049 msec | +032 msec | -006 msec |
| WED. 07/21 | -563 msec | -026 msec | -050 msec | +032 msec | -007 msec |
| THU. 07/22 | -559 msec | -025 msec | -048 msec | +032 msec | -008 msec |

EMOSS POC for this chart is JIM SHEPHERD

NOAA-12  	Autonomous -500mSec jump on 17 Jun

Daily ETCUP bias of +6.5 mSec started on 12/10/03.

NOAA-14:  	As of 1223/03 the TIP clock is corrected daily by –-7.0 msec to compensate for drift.

Subtracted 150 mSecs on 30 June

NOAA-15:  	As of 04/23/03 the TIP clock is corrected daily by –-8 msec to compensate for drift.

Added 100 msec to spacecraft command clock on 10 June 03.

NOAA-16:  	As of 12/23/03 the TIP clock is corrected daily by –5.5 msec to compensate for drift.

Subtracted 100 msec from spacecraft command clock on 23 Dec 03.

NOAA-17:  	As of 12/23/03 the TIP clock is corrected daily by –3 msec to compensate for drift.

Added 125 Msecs3 Feb 04.

CPU Single Event Upsets, new matrix under construction

PACS DATA AVAILABILITY (%)

Not available in time for this publication

DTR Inf. (*):

NOAA-12	DTR 5A -Inoperative.

DTR 2A/B -Occasionally noisy and short

DTR 1A/B - Inoperative

NOAA-14	DTR 4A/B -Inoperative.

DTR 3B -Unreliable playbacks, Removed from operations.

DTR 2A -Inoperative, DOY/200

NOAA-15	DTR 5A/B - Inoperative

NOAA-16            DTR 5A/B -Inoperative

DTR 3A -safe state change 1B

DTR 4B –Inoperative 1/12/03

NOAA-17	DTR 5A/B-Inoperative 2/16/03 Anomaly

Acronyms:

DL – Drop lock(s)	PE - Parity Error(s)			DQR - Data Quality Report

BE - Bit Error(s)		DAR - Data Accountability Report		MFR - Multi-Function Receiver