UNITED STATES ENVIRONMENTAL PROTECTION AGENCY

WASHINGTON, D.C. 20460

OFFICE OF

PREVENTION, PESTICIDES AND

TOXIC SUBSTANCES

October 24, 2007

MEMORANDUM

SUBJECT:		Ecological Hazard and Environmental Risk Assessment Chapter for the Copper 8-Quinolinolate Reregistration Eligibility Decision (RED) Document (Case No.: 4026)

FROM:		Genevieve Angle, Biologist

Risk Assessment and Science Support Branch (RASSB)

Antimicrobials Division (7510P)

TO:			Mark Hartman, Branch Chief

Diane Isbell, Team Leader

ShaRon Carlisle, Chemical Review Manager

Sanyvette Williams, Risk Assessor

Regulatory Management Branch II

Antimicrobials Division (7510P)

THRU:		Nader Elkassabany, Team Leader, Team Two

Risk Assessment and Science Support Branch (RASSB)

Antimicrobials Division (7510P)

Norman Cook, Branch Chief

Risk Assessment and Science Support Branch (RASSB)

Antimicrobials Division (7510P)

Chemical Names					PC Codes	CAS Nos.

Copper 8-quinolinolate				024002	10380-28-6

Attached is the Ecological Hazard and Environmental Risk Assessment chapter for the copper 8-quinolinolate RED Document.

ECOLOGICAL HAZARD AND ENVIRONMENTAL

RISK ASSESSMENT CHAPTER

Copper 8-Quinolinolate

### PC Code 024002

### CASE No.: 4026

10/24/07

Genevieve Angle, Biologist

Antimicrobials Division

Office of Pesticide Programs

U.S. Environmental Protection Agency

1200 Pennsylvania Avenue, NW

Washington, DC 20460

Table of Contents

Page

Executive Summary ……………………………………………………………………………… 1

1.  Ecological Toxicity Data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .3

A.  Toxicity to Terrestrial Animals . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . ...3

1.  Birds, Acute  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3

2.  Birds, Subacute. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

3.  Mammals, Acute and Chronic . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .5

B.  Toxicity to Aquatic Animals . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .5

1.  Freshwater Fish, Acute . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .5

2.  Freshwater Invertebrates, Acute . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .6

3.  Estuarine and Marine Organisms, Acute . . . . . . . . . . . . . . . . . . . . . . . . . . ... . . . . .7

4.  Aquatic Organisms, Chronic . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .8

C.  Toxicity to Plants . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . ..8

II.  Risk Assessment and Risk Characterization . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . ..9

A.  Environmental Fate Assessment Summary . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. . . . ..10

B.  Environmental Exposure And Ecological Risk Assessment. . . . . . . . . . . . . . . . .. . . . . . .10

C.  Endangered Species Considerations . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  . . . . . . . . . . 14

III.  Confirmatory Data Required. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. . . . . . . . . . 15

IV.  Label Hazard Statements for Terrestrial and Aquatic Organisms. . . . . . . . . . .  . . . . . . . . . .16

V.  References . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  . . . . . . . . . . 17

LIST OF TABLES

Page

Table 1 – Acute Oral Toxicity of Copper 8-Quinolinolate to Birds. . . . . . . . . . . . . .  . . . . . . . . . .3

Table 2 – Subacute Oral Toxicity of Copper 8-Quinolinolate to Birds. . . . . . . . . . .. . . . . . . . . . . 4

Table 3 – Acute Toxicity of Copper 8-Quinolinolate to Freshwater Fish. . . . . . . . . . .. . . . . . . . . 4

Table 4 – Acute Toxicity of Copper 8-Quinolinolate to Freshwater Invertebrates. . . . .. . . . . . . . 5

Table 5 – Acute Toxicity of Copper 8-Quinolinolate to Estuarine and Marine Organisms. . . . . . 6

Table 6 – Chronic Toxicity of Copper 8-Quinolinolate to Freshwater Organisms. . .. . . . . . . . . . 7

Table 7 – Toxicity of Copper 8-Quinolinolate to Aquatic Plants. . . . . . . . . . .. . . . . . . . . . . . .. . ..8

Ecological Hazard and Environment Risk Assessment

### For Copper 8-Quinolinolate

Executive Summary:

Copper 8-quinolinolate (copper 8) is currently registered as a fungicide, algaecide and bacteriocide for materials preservation, wood preservation and food handling/storage establishments.  Materials preservative uses include:  industrial textiles, webbing, rope, canvas, leather, kraft paper, paperboard, adhesives and paint.  Wood preservative uses include: log homes, shingle roofs, siding, fences, poles, posts, decks, furniture, playground equipment, sills and baseboards and structural building lumber.  Food handling/storage establishment uses include potato processing, storage and transportation facilities.

Copper 8-quinolinolate is hydrolytically stable at pH 5, 7 and 9. More than 80% of it is stable in aerobic and anaerobic soils. In aerobic soils its half-life is 16 weeks, but it may be over one year in anaerobic soils.  It does not show any tendency to migrate from top soil. It is therefore likely to contaminate surface water through surface water run-off. Its degradation pathway appears to be aqueous photolysis with a half-life of 60 to 96 hours.  The estimated log Kow for copper 8-quinolinolate is 2.5, which indicates that it is not likely to bioaccumulate in aquatic organisms like fish.

The wood treatment uses of copper 8 have high potential for environmental exposure and thus require an ecological risk assessment.  Uses in rope, webbing, and roofing shingles are likely to provide for some environmental exposures as well; however, only the wood treatment uses have been examined in this assessment.  All other uses are considered indoor and have minimal to no environmental exposure potential following use.

EPA has performed an environmental risk assessment using estimated environmental concentrations (EECs) for copper 8 developed by modeling its release from a dock into water and toxicity values from the tables in section I to develop risk quotients (RQs) and compare them to levels of concern (LOCs) for copper 8.  LOCs were not exceeded for fish, freshwater invertebrates, the eastern oyster or aquatic plants in bodies of water 6 acre feet in size or greater, whereas risks to endangered freshwater fish and the eastern oyster as well as risks to aquatic plants were of concern in bodies of water 1 acre foot in size or less.  Since it is unlikely that a dock of the size used in the calculations for EEC will be present on a body of water less than 6 acre feet in size, the risks to aquatic organisms from copper 8 in this assessment appear to be small.  There were no acceptable acute toxicity studies for other estuarine and marine organisms nor were there any acceptable chronic toxicity studies available for aquatic organisms.  Therefore, risk to these species could not be assessed.

Data Gaps:

Confirmatory Data Required For Wood Treatment Risk Assessment:

- Estuarine/marine fish acute study (850.1075),

- Estuarine/marine shrimp acute study (850.1035),

- Acute sediment toxicity to freshwater invertebrates (850.1735),

- Acute sediment toxicity to estuarine invertebrates (850.1740),

- Aquatic invertebrate (freshwater) life-cycle study (850.1300),

- Fish early life-stage (freshwater) study (850.1400),1

- Fish early life-stage (estuarine/marine) study (850.1400),1

- Chronic mysid shrimp study (850.1350),1

- Freshwater diatom (850.5400),

- Blue-green cyanobacteria (850.5400),

- Freshwater green alga (850.5400),

- Freshwater floating macrophyte duckweed (850.4400),

- Freshwater rooted macrophyte rice seedling emergence (850.4225),

- Freshwater rooted macrophyte rice vegetative vigor (850.4250),

- Wood leaching study (AWPA E11-06),

- Environmental runoff monitoring study (None).  This test can be waived provided labels that include anti-sapstain treatment are amended as outlined below for such wood preservative labels,

- Residues in honey/beeswax and toxicity of treated wood residues to bees – combination of Guideline 860.1500 and 850.3030 (“Honey Bee Toxicity of Residues on Foliage.”).  The toxicity portion of this study is in lieu of the honeybee contact LD50 test 850.3020.  The residue and toxicity test can be waived provided the label is amended to prohibit the use of treated wood for beehive construction, with a statement such as, “Wood treated with copper 8-quinolinolate shall not be used in the construction of beehives.”

Label Hazard Statements/Use Recommendations:

Copper 8-quinolinolate labels must state:

“This pesticide is toxic to fish, aquatic invertebrates, oysters, and shrimp.”

"Do not discharge effluent containing this product into lakes, streams, ponds, estuaries, oceans, or other waters unless in accordance with the requirements of a National Pollutant Discharge Elimination System (NPDES) permit and the permitting authorities are notified in writing prior to discharge.  Do not discharge effluent containing this product to sewer systems without previously notifying the local sewage treatment plant authority.  For guidance contact your State Water Board or Regional Office of the EPA."

Wood preservative labels that include anti-sapstain treatment must state:  "Treated lumber must be stored under cover, indoors, or at least 100 feet from any pond, lake, stream, wetland, or river to prevent possible runoff of the product into the waterway.  Treated lumber stored within 100 feet of a pond, lake, stream, wetland, or river must be either covered with plastic or surrounded by a berm to prevent surface water runoff into the nearby waterway.  If a berm or curb is used around the site, it should consist of impermeable material (clay, asphalt, concrete) and be of sufficient height to prevent runoff during heavy rainfall events.”

All wood preservative labels must state: The honey bee residue and toxicity test can be waived provided the label is amended to prohibit the use of treated wood for beehive construction, with a statement such as, “Wood treated with copper 8-quinolinolate shall not be used in the construction of beehives.

I.	Ecological Toxicity Data

The toxicity endpoints presented below are based on the results of ecotoxicity studies submitted to EPA to meet the Agency’s data requirements for the uses of copper 8-quinolinolate.

A.	Toxicity to Terrestrial Animals

(1)	Birds, Acute

In order to establish the toxicity of copper 8 to avian species, the Agency requires an acute oral toxicity study using the technical grade active ingredient (TGAI).  The preferred test species is either mallard duck (a waterfowl) or bobwhite quail (an upland game bird).  The results of two acute oral toxicity studies, submitted for copper 8, are provided in the following table (Table 1).

# Table 1.  Acute Oral Toxicity of Copper 8 to Birds

| Species | Chemical, % Active Ingredient (a.i.) Tested | Endpoint (mg/kg) | Toxicity Category | Satisfies Guidelines/ Comments | Reference (MRID No.) |
| --- | --- | --- | --- | --- | --- |
| Bobwhite quail (Colinus virginianus) | Copper 8 99.5% | LD50 = 618 | Slightly toxic | Yes (core) - 14-day test duration - 4-5 months of age | 429271-01 |
| Mallard duck (Anas platyrhynchos) | Copper 8 99.5% | LD50 = >2000 NOAEL = 2000 | Relatively nontoxic | Yes (core) - 14-day test duration - 15 months of age | 429271-02 |

These acceptable acute oral toxicity studies indicate that copper 8 is slightly toxic to relatively nontoxic to birds on an acute oral basis. The guideline requirement OPPTS 850.2100/(71-1) is satisfied.

(2)	Birds, Subacute

A subacute dietary study using the TGAI may be required on a case-by-case basis depending on the results of lower-tier ecological studies and pertinent environmental fate characteristics in order to establish the toxicity of a chemical to avian species.  This testing was required for copper 8.  The preferred test species is either the mallard duck or bobwhite quail.  The results of two subacute dietary toxicity studies, submitted for copper 8, are provided in the following table (Table 2).

# Table 2.  Subacute Oral Toxicity of Copper 8 to Birds

| Species | Chemical, % Active Ingredient (a.i.) Tested | Endpoint (ppm) | Toxicity Category | Satisfies Guidelines/ Comments | Reference (MRID No.) |
| --- | --- | --- | --- | --- | --- |
| Bobwhite quail (Colinus virginianus) | Copper 8 99.5% | LC50 (diet) = 3248 NOAEC = 1300 | Slightly toxic | Yes (core) - 8-day test duration - 14 days of age | 429271-03 |
| Mallard duck (Anas platyrhynchos) | Copper 8 99.5% | LC50 (diet) = >5200 NOAEC = 2600 | Relatively nontoxic | Yes (core) - 8-day test duration - 10 days of age | 429271-04 |

The results from these acceptable studies indicate that copper 8 is slightly toxic to relatively nontoxic to avian species through subacute dietary exposure. These studies fulfill guideline requirements OPPTS 850.2100/ (71-2a – Bobwhite quail and 71-2b – Mallard duck).

(3)	Mammals, Acute and Chronic Toxicity

Wild mammal testing is not required by the Agency.  In most cases, rat toxicity values obtained from studies conducted to support data requirements for human health risk assessments substitute for wild mammal testing.  Refer to the human toxicology chapter of this RED for mammalian toxicity data.

B.	Toxicity to Aquatic Animals

The Agency requested that aquatic toxicity studies be conducted with copper 8-quinolinolate since, under typical use conditions, it can be introduced into the aquatic environment.

(1)	Freshwater Fish, Acute

In order to establish the acute toxicity of copper 8 to freshwater fish, the Agency requires freshwater fish toxicity studies using the TGAI.  The preferred test species are rainbow trout (a coldwater fish) and bluegill sunfish (a warmwater fish).  The results of four freshwater fish acute studies submitted for copper 8, 3 coldwater and 1 warmwater, are presented in Table 3.

# Table 3.  Acute Toxicity of Copper 8 to Freshwater Fish

| Species | Chemical, % Active Ingredient (a.i.) Tested | Endpoint (mg/L) | Toxicity Category | Satisfies Guidelines/ Comments | Reference (MRID No.) |
| --- | --- | --- | --- | --- | --- |
| Bluegill Sunfish (Lepomis macrochirus) | Copper 8 100% | LC50 = 0.0216 NOAEC = 0.0108 | Very highly toxic | Yes (core) - 96-hr test duration - flow-through test system | 428990-03 |
| Rainbow Trout (Oncorhynchus mykiss) | Copper 8 100% | LC50 = 0.0089 NOEC = 0.0062 | Very highly toxic | Yes (core) - 96-hr test duration - flow-through test system | 428990-02 |
| Rainbow Trout (Oncorhynchus mykiss) | Copper 8 80% | LC50 = 0.0097 NOAEC = 0.0071 | Very highly toxic | Yes (core) - 96-hr test duration - static renewal test system | 435637-01 |
| Coho Salmon (Oncorhynchus kisutch) | Copper 8 100% | LC50 = 0.0139 NOAEC = 0.0066 | Very highly toxic | Yes (core) - 96-hr test duration - flow-through test system | 429024-01 |

Freshwater acute toxicity tests indicate that copper 8 is very highly toxic to the coldwater rainbow trout and very highly toxic to the warmwater bluegill sunfish on an acute basis.  Guideline requirement OPPTS 850.1075 (72-1a&b) is fulfilled.  Because acute toxicity to fish is <1.0 mg/L the environmental hazard section of copper 8 labels must state: “This pesticide is toxic to fish.”

(2)	Freshwater Invertebrates, Acute

The Agency requires a freshwater aquatic invertebrate study using the TGAI to establish the acute toxicity to freshwater invertebrates.  The preferred test species is Daphnia magna.  The results of one study submitted for copper 8 is provided in the following table (Table 4).

Table 4.  Acute Toxicity of Copper 8 to Freshwater Invertebrates

| Species | Chemical, % Active Ingredient (a.i.) Tested | Endpoint (mg/L) | Toxicity Category | Satisfies Guidelines/ Comments | Reference (MRID No.) |
| --- | --- | --- | --- | --- | --- |
| Waterflea (Daphnia magna) | Copper 8 98% | EC50 = 0.162 NOAEC = < 0.036 | Highly toxic | Yes (core) - 48-hr test duration - flow-through test system | 432284-01 |

The results of MRID 432284-01 indicate that copper 8 is highly toxic to freshwater invertebrates.  This study fulfills guideline requirement OPPTS 850.1010 (72.2a).  Because the acute aquatic invertebrate toxicity value is < 1.0 mg/L, the environmental hazard section of copper 8 labels must state:  “This pesticide is toxic to aquatic invertebrates.”

(3)	Estuarine and Marine Organisms, Acute

Acute toxicity testing with estuarine and marine organisms using the TGAI is required when the end-use product is intended for direct application to the marine/estuarine environment or effluent containing the active ingredient is expected to reach this environment.  The preferred fish test species is the sheepshead minnow.  The preferred invertebrate test species are mysid shrimp and eastern oysters.  This testing is required for copper 8 based on the chemical’s potential to reach estuarine and marine environments.  The results of one toxicity study submitted for copper 8 is presented in Table 5.

# Table 5.  Acute Toxicity of Copper 8 to Estuarine and Marine Organisms

| Species | Chemical, % Active Ingredient (a.i.) Tested | Endpoint (mg/L) | Toxicity Category | Satisfies Guidelines/ Comments | Reference (MRID No.) |
| --- | --- | --- | --- | --- | --- |
| Eastern oyster (Crassostrea virginica) | Copper 8 100% | LC50 = 0.0363 EC50 = 0.0111 NOAEC = 0.003 | Very highly toxic | Yes (core) - 48-hr test duration - static test system | 428990-04 |

The results of the one core study indicate that copper 8 is very highly toxic to estuarine/marine invertebrates on an acute basis.  The Eastern oyster study (428990-04) fulfills the guideline requirement OPPTS 850.1025/(72-3b) for an acute estuarine/marine mollusk study. Study MRID 429024-02 for mysid shrimp is INVALID and must be repeated.  The guideline requirements OPPTS 850.1075/(72-3a) for an acute estuarine/marine fish study and OPPTS 850.1035/(72-3c) for an estuarine/marine shrimp study are not fulfilled.  Because estuarine/marine acute toxicity values are < 1.0 mg/L, the environmental hazard section of copper 8 labels must state:  “This pesticide is toxic to oysters and shrimp.”

(4)	Aquatic Organisms, Chronic

Chronic toxicity testing (fish early life stage and aquatic invertebrate life cycle) is required for pesticides when certain conditions of use and environmental fate apply.  Preferred freshwater and estuarine/marine fish and invertebrate species include:  fathead minnow, rainbow trout, Daphnia magna, and mysid shrimp.  Two studies were submitted for copper 8, but are INVALID and must be repeated.  The rainbow trout study (429024-03) does not fulfill the guideline requirement for a fish early life stage toxicity test (72-4a/OPPTS 850.1400) due to the percent RSD of weight in one replicate of the solvent control being >40%. The study on the waterflea (428990-05) also does not fulfill the guideline requirement for a chronic aquatic invertebrate because dissolved oxygen concentrations fell to extremely low levels during the test and the highest measured concentration in one exposure solution was greater than twice the lowest measured concentration at the same level.

- Toxicity to Plants

Non-target plant phytotoxicity testing is required for pesticides when certain conditions of use and environmental fate apply.  The use of copper 8 as a wood treatment may result in chemical leachate from treated wood into the aquatic environment.  Aquatic plant toxicity data are necessary for a non-target plant risk assessment.  Testing is conducted with one species of aquatic vascular plant (Lemna gibba) and four species of algae:  (1) freshwater green alga, Selenastrum capricornutum, (2) marine diatom, Skeletonema costatum, (3) freshwater diatom, Navicula pelliculosa, and (4) bluegreen cyanobacteria, Anabaena flos-aquae.  The rooted aquatic macrophyte rice (Oryza sativa) is also tested in seedling emergence and vegetative vigor tests.

Two marine diatom studies and a saltwater green algae study were reviewed. Results of these studies are presented in Table 7.

Table 7.  Toxicity of Copper 8 to Aquatic Plants

| Species | Chemical, % Active Ingredient (a.i.) Tested | Endpoint (mg/L) | Satisfies Guidelines/ Comments | Reference (MRID No.) |
| --- | --- | --- | --- | --- |
| Marine diatom (Nitzschia punctata) | Copper 8 100% | EC50 = 0.0073 | No (supplemental) - 5-day test duration - static test system - NOEC not determined | 429024-04 |
| Marine diatom (Skeletonema costatum) | Copper 8 98% | EC50 = 0.0019 NOEC = < 0.0007 | Yes (core) - 5-day test duration - static test system | 430735-01 |
| Saltwater Green alga (Dunaliella tertiolecta) | Copper 8 100% | EC50 = 0.0154 NOEC = 0.009 | Yes (core) - 5-day test duration - static test system | 429024-05 |

The studies above are classified as acceptable and partially fulfill the guideline requirements for an algae toxicity test (850.5400, 123-2).  Three additional algal toxicity tests under 850.5400 are outstanding:  freshwater diatom (Navicula pelliculosa), blue-green cyanobacteria (Anabeana flos-aquae), and freshwater green alga (Selenastrum capricornutum).  Other outstanding non-target aquatic plant toxicity tests are:  floating freshwater aquatic macrophyte duckweed (Lemna gibba) – 850.4400 and rooted freshwater macrophyte rice (Oryza sativa) – 850.4225 and 850.4250 (2 tests on seedling emergence and vegetative vigor).

II.	Risk Assessment and Characterization

Copper 8-quinolinolate uses are classified as “indoor” with the exception of the wood treatment uses.  An ecological risk assessment is not typically conducted for “indoor” uses.  A risk assessment for the wood preservative uses is conducted below using the toxicity values from section I and estimated environmental concentrations (EECs) calculated by modeling copper 8 leaching into a body of water from a dock.  These values were used to develop risk quotients (RQs) and compare them to levels of concern (LOCs) in section B below.

A.        Environmental Fate Assessment Summary

Copper 8-quinolinolate is hydrolytically stable at pH 5, 7 and 9. More than 80% of it is stable in aerobic and anaerobic soils. In aerobic soils its half-life is 16 weeks, but it may be over one year in anaerobic soils.  It does not show any tendency to migrate from top soil. It is therefore likely to contaminate surface water through surface water run-off. Its degradation pathway appears to be aqueous photolysis with a half-life of 60 to 96 hours.  The estimated log Kow for copper 8-quinolinolate is 2.5, which indicates that it is not likely to bioaccumulate in aquatic organisms like fish.

B.	Environmental Exposure and Ecological Risk Assessment

Risk assessment integrates the results of the exposure and ecotoxicity data to evaluate the likelihood of adverse ecological effects. One method of integrating the results of exposure and ecotoxicity data is called the quotient method.  For this method, risk quotients (RQs) are calculated by dividing exposure estimates by ecotoxicity values, both acute and chronic:

RQ = EXPOSURE/TOXICITY

RQs are then compared to levels of concern (LOCs).  These LOCs are criteria used by OPP to indicate potential risk to nontarget organisms and the need to consider regulatory action.  The criteria indicate that a pesticide used as directed has the potential to cause adverse effects on nontarget organisms.  LOCs currently address the following risk presumption categories: (1) acute - the potential for acute risk is high, regulatory action may be warranted in addition to restricted use classification; (2) acute restricted use - the potential for acute risk is high, but this may be mitigated through restricted use classification; (3) acute endangered species - the potential for acute risk to endangered species is high, and regulatory action may be warranted, and (4) chronic risk - the potential for chronic risk is high, and regulatory action may be warranted, (5) non-endangered plant risk – potential for effects in non-target plants, and (6) endangered plant risk – potential for effects in endangered plants.   Currently, EPA does not perform assessments for chronic risk to plants, acute or chronic risks to nontarget insects, or chronic risk from granular/bait formulations to birds or mammals.

The ecotoxicity test values (measurement endpoints) used in the acute and chronic risk quotients are derived from required studies.  Examples of ecotoxicity values derived from short-term laboratory studies that assess acute effects are: (1) LC50 (fish and birds), (2) LD50 (birds and mammals), (3) EC50 (aquatic plants and aquatic invertebrates) and (4) EC25 (terrestrial plants).  Examples of toxicity test effect levels derived from the results of long-term laboratory studies that assess chronic effects are: (1) LOAEC (birds, fish, and aquatic invertebrates), and (2) NOAEC (birds, fish and aquatic invertebrates). For birds and mammals, the NOAEC generally is used as the ecotoxicity test value in assessing chronic effects, although other values may be used when justified. However, the NOAEC is used if the measurement endpoint is production of offspring or survival.  Risk presumptions, along with the corresponding RQs and LOCs are tabulated below.

Risk Presumptions for Terrestrial Animals

| Risk Presumption | RQ | LOC |
| --- | --- | --- |

Birds and Wild Mammals

| Acute Risk | EEC1/LC50 or LD50/sqft2 or LD50/day3 | 0.5 |
| --- | --- | --- |
| Acute Restricted Use | EEC/LC50 or LD50/sqft or LD50/day (or LD50 < 50 mg/kg) | 0.2 |
| Acute Endangered Species | EEC/LC50 or LD50/sqft or LD50/day | 0.1 |
| Chronic Risk | EEC/NOAEC | 1 |

1  abbreviation for Estimated Environmental Concentration (ppm) on avian/mammalian food items

2    mg/ft2             	3  mg of toxicant consumed/day

LD50 * wt. of bird             	LD50 * wt. of bird

Risk Presumptions for Aquatic Animals

| Risk Presumption | RQ | LOC |
| --- | --- | --- |
| Acute Risk | EEC1/LC50 or EC50 | 0.5 |
| Acute Restricted Use | EEC/LC50 or EC50 | 0.1 |
| Acute Endangered Species | EEC/LC50 or EC50 | 0.05 |
| Chronic Risk | EEC/MATC2 or NOAEC | 1 |

1  EEC = (ppm or ppb) in water

2  MATC = maximum allowable toxicant concentration

| Risk Presumptions for Plants |  |  |
| --- | --- | --- |
| Risk Presumption | RQ | LOC |

| ## Terrestrial and Semi-Aquatic Plants |  |  |
| --- | --- | --- |
| Acute Risk | EEC/EC25 | 1 |
| Acute Endangered Species | EEC/EC05 or NOAEC | 1 |

| ## Aquatic Plants |  |  |
| --- | --- | --- |
| Acute Risk | EEC1/EC50 | 1 |
| Acute Endangered Species | EEC/EC05 or NOAEC | 1 |

- EEC = (ppb/ppm) in water

Maximum Expected Environmental Concentrations:

EPA calculated the leaching of copper 8 from a dock into water.  It was assumed that 4% of the total applied copper 8 would leach from the wood into the water.  The retention rate of the wood was assumed to be 22 g/cm2.  The length and width of the dock was assumed to be 30 meters and 10 meters, respectively, and the thickness of the wood was assumed to be 0.1 meters.  The number of poles underneath the dock was assumed to be 18 and the dimensions of the poles were assumed to be 2 meters (length) x 0.15 meters (width) x 0.15 meters (height).  The poles were assumed to be 0.5 meters inserted into the sediment.  Based on these specifications, copper 8 EECs were calculated for water body sizes ranging from 1 acre foot to 24 acre feet.  The highest EEC calculated for the smallest body of water (1 acre foot) was 0.00226 mg copper 8 per liter of water.  The calculated EEC for a slightly larger body of water (6 acre feet) is 0.00038 mg copper 8 per liter of water.  For details on the assumptions made and calculations conducted to arrive at this EEC as well as the uncertainties and limitations of the calculations, consult Siroos Mostaghimi’s 4/25/2007 memo “Estimated Environmental Concentrations for Copper 8-Quinolinolate (Cu8Q) from Treated Wood Used to Build Docks.”

Maximum Ecotoxicity Values:

From the toxicity tables in section I above, the highest toxicity in an acceptable fish study was achieved in a study on the rainbow trout (Oncorhynchus mykiss).  The LC50 value obtained in this study was 0.0089 mg/L (MRID 428990-02).  The EC50 value obtained in the study on the freshwater invertebrate (Daphnia magna) was 0.162 mg/L (MRID 432284-01).  The EC50 value obtained in the study on the eastern oyster (Crassostrea virginica) was 0.0111 mg/L (MRID 428990-04).  There were no acceptable acute toxicity studies for estuarine and marine fish or shrimp nor were there any acceptable chronic toxicity studies available for aquatic organisms.  Therefore, risk to these species cannot be assessed.  The highest toxicity in an acceptable aquatic plant toxicity study was achieved in a study on the marine diatom (Skeletonema costatum).  The EC50 value obtained in this study was 0.0019 mg/L and the NOEC was < 0.0007 (MRID 430735-01).   Due to outstanding plant toxicity studies, the non-target plant risk assessment is incomplete.

Acute Freshwater Fish LOC’s:

Comparing the maximum calculated EEC (0.00226 mg/L) to the highest toxicity found in a fish acute study (0.0089 mg/L), an RQ of 0.254 is obtained.  This is less than the LOCs for acute and chronic risks to aquatic animals but greater than the LOCs for endangered species and acute restricted use.  Therefore, there is the potential for copper 8 to cause adverse effects to endangered freshwater fish and these risks may need to be mitigated.  The calculated EEC for a slightly larger body of water (6 acre feet) is 0.00038 mg copper 8 per liter of water.  This EEC brings the RQ for freshwater fish down to 0.043, therefore the risk to endangered fish species is no longer considered significant in this size body of water.  However, as discussed below an endangered species effect determination will not be made at this time.

Acute Freshwater Aquatic Invertebrate LOC’s:

Comparing the maximum calculated EEC to the toxicity value found in the study on the freshwater invertebrate (0.162 mg/L), an RQ of 0.014 is obtained.  This is less than all LOCs for aquatic organisms, therefore the potential for risks from copper 8 to aquatic invertebrates is not high.  The calculated EEC for a slightly larger body of water (6 acre feet) is 0.00038 mg copper 8 per liter of water.  The calculated RQ is less than 0.00 and therefore no risk is triggered for a larger water body.  However, as discussed below an endangered species effect determination will not be made at this time.

Acute Aquatic Estuarine/Marine Species:

For the eastern oyster, the RQ is 0.204.  This is less than the LOCs for acute and chronic risks to aquatic animals but greater than the LOCs for endangered species and acute restricted use.  Therefore, there is the potential for copper 8 to cause adverse effects to endangered estuarine and marine organisms and these risks may need to be mitigated.  The calculated EEC for a slightly larger body of water (6 acre feet) is 0.00038 mg copper 8 per liter of water.  This EEC brings the RQ for the eastern oyster down to 0.034 and the RQ for aquatic plants down to 0.2 so the risks to endangered estuarine/marine organisms and aquatic plants are no longer considered significant either.  This risk assessment is incomplete due to outstanding fish and shrimp toxicity studies.

Chronic Aquatic Toxicity Studies:

Chronic freshwater fish and invertebrate studies are triggered based on acute toxicity.  No valid studies are available for copper 8.  Estuarine/marine chronic toxicity studies for fish and invertebrates are required.  This risk assessment is incomplete.

Plant Toxicity Studies:

Comparing the maximum calculated EEC (0.00226 mg/L) to the highest toxicity value found for aquatic plants (0.0019 mg/L), an RQ of 1.189 is obtained.  This is greater than the LOC of >1.0 and therefore there is the potential for acute risk to aquatic plants, including endangered species, from copper 8.  The calculated EEC for a slightly larger body of water (6 acre feet) is 0.00038 mg copper 8 per liter of water.  This EEC brings the RQ for aquatic plants down to 0.2 so the risks to endangered estuarine/marine organisms and aquatic plants are no longer considered significant for the larger body of water.  This risk assessment is incomplete due to outstanding plant toxicity studies.

Avian and Mammalian Species:

Based on available avian toxicity data for copper 8, the various wood treatments are not expected to be acutely toxic.

Uncertainties and Limitations:

Fate and Environmental Modeling:  There are a number of uncertainties and limitations with this preliminary environmental risk assessment.  Extrapolating risk conclusions from the pond scenario used in the environmental modeling may either underestimate or overestimate potential exposures and risks. Numerous uncertainties exist with the modeling used since environmental properties are likely to be regionally specific because of local hydrogeological conditions.  Further, any alteration in water quality parameters may impact the environmental behavior of the pesticide.  Additionally, there are pertinent data (wood leaching) lacking; such data would be useful in refining this preliminary risk assessment.

Toxicity Data:  This screening level risk assessment relies on selected toxicity endpoints from what are believed to be the most sensitive species tested, but it does not necessarily mean that the selected toxicity endpoints reflect sensitivity of the most sensitive species existing in a given environment.  Surrogates were used to predict potential risks for various species, and data for several species are lacking.  Submission of additional non-target organism data will refine this screening level assessment.

Stakeholder Modeling Efforts:  The American Chemistry Council (ACC) Biocides Panel Copper Reregistration Task Force is presently developing modeling pertinent to the wood preservative and roofing uses of various copper products.  This modeling effort is intended to provide estimated environmental exposures which can be used by the Agency to refine environmental assessments.  Such data would be useful in refining this screening level assessment.

- Endangered Species Considerations

Section 7 of the Endangered Species Act, 16 U.S.C. Section 1536(a)(2), requires all federal agencies to consult with the National Marine Fisheries Service (NMFS) for marine and anadromous listed species, or the United States Fish and Wildlife Services (FWS) for listed wildlife and freshwater organisms, if they are proposing an "action" that may affect listed species or their designated habitat.  Each federal agency is required under the Act to insure that any action they authorize, fund, or carry out is not likely to jeopardize the continued existence of a listed species or result in the destruction or adverse modification of designated critical habitat.  To jeopardize the continued existence of a listed species means "to engage in an action that reasonably would be expected, directly or indirectly, to reduce appreciably the likelihood of both the survival and recovery of a listed species in the wild by reducing the reproduction, numbers, or distribution of the species." 50 C.F.R. ' 402.02.

To facilitate compliance with the requirements of the Endangered Species Act subsection (a)(2) the Environmental Protection Agency, Office of Pesticide Programs has established procedures to evaluate whether a proposed registration action may directly or indirectly reduce appreciably the likelihood of both the survival and recovery of a listed species in the wild by reducing the reproduction, numbers, or distribution of any listed species (U.S. EPA 2004).  After the Agency’s screening-level risk assessment is performed, if any of the Agency’s Listed Species LOC Criteria are exceeded for either direct or indirect effects, a determination is made to identify if any listed or candidate species may co-occur in the area of the proposed pesticide use.  If determined that listed or candidate species may be present in the proposed use areas, further biological assessment is undertaken.  The extent to which listed species may be at risk then determines the need for the development of a more comprehensive consultation package as required by the Endangered Species Act.

For certain use categories, the Agency assumes there will be minimal environmental exposure, and only a minimal toxicity data set is required (Overview of the Ecological Risk Assessment Process in the Office of Pesticide Programs U.S. Environmental Protection Agency - Endangered and Threatened Species Effects Determinations, 1/23/04, Appendix A, Section IIB, pg.81).  Chemicals in these categories therefore do not undergo a full screening-level risk assessment, and are considered to fall under a “No Effect” determination.  The materials preservative uses of copper 8-quinolinolate fall into this category.

The preliminary analysis for wood treatment uses indicates that there is a potential for copper 8 use to overlap with listed species and that a more refined assessment is warranted, to include direct, indirect and habitat effects.  The more refined assessment should involve clear delineation of the action area associated with proposed use of copper 8 and best available information on the temporal and spatial co-location of listed species with respect to the action area.  This analysis has not been conducted for this assessment.  An endangered species effect determination will not be made at this time.  The wood runoff label statement is expected to provide some level of mitigation until such time as a full endangered species assessment is possible.

- Confirmatory Data Required To Support Wood Treatment Use:

- Estuarine/marine fish acute study (850.1075),

- Estuarine/marine shrimp acute study (850.1035),

- Acute sediment toxicity to freshwater invertebrates (850.1735),

- Acute sediment toxicity to estuarine invertebrates (850.1740),

- Aquatic invertebrate (freshwater) life-cycle study (850.1300),

- Fish early life-stage (freshwater) study (850.1400),1

- Fish early life-stage (estuarine/marine) study (850.1400),1

- Chronic mysid shrimp study (850.1350),1

- Freshwater diatom (850.5400),

- Blue-green cyanobacteria (850.5400),

- Freshwater green alga (850.5400),

- Freshwater floating macrophyte duckweed (850.4400),

- Freshwater rooted macrophyte rice seedling emergence (850.4225),

- Freshwater rooted macrophyte rice vegetative vigor (850.4250),

- Wood leaching study (AWPA E11-06),

- Environmental runoff monitoring study (None).  This test can be waived provided labels that include anti-sapstain treatment are amended as outlined below for such wood preservative labels,

- Residues in honey/beeswax and toxicity of treated wood residues to bees – combination of Guideline 860.1500 and 850.3030 (“Honey Bee Toxicity of Residues on Foliage.”).  The toxicity portion of this study is in lieu of the honeybee contact LD50 test 850.3020.  The residue and toxicity test can be waived provided the label is amended to prohibit the use of treated wood for beehive construction, with a statement such as, “Wood treated with copper 8-quinolinolate shall not be used in the construction of beehives.”

IV.	 Label Hazard Statements for Terrestrial and Aquatic Organisms and Use                              Recommendations

Copper 8-quinolinolate labels must state:

“This pesticide is toxic to fish, aquatic invertebrates, oysters, and shrimp.”

"Do not discharge effluent containing this product into lakes, streams, ponds, estuaries, oceans, or other waters unless in accordance with the requirements of a National Pollutant Discharge Elimination System (NPDES) permit and the permitting authorities are notified in writing prior to discharge.  Do not discharge effluent containing this product to sewer systems without previously notifying the local sewage treatment plant authority.  For guidance contact your State Water Board or Regional Office of the EPA."

Wood preservative labels that include anti-sapstain treatment must state:

"Treated lumber must be stored under cover, indoors, or at least 100 feet from any pond, lake, stream, wetland, or river to prevent possible runoff of the product into the waterway.  Treated lumber stored within 100 feet of a pond, lake, stream, wetland, or river must be either covered with plastic or surrounded by a berm to prevent surface water runoff into the nearby waterway.  If a berm or curb is used around the site, it should consist of impermeable material (clay, asphalt, concrete) and be of sufficient height to prevent runoff during heavy rainfall events.”

All wood preservative labels must state:

“Wood treated with copper 8-quinolinolate shall not be used in the construction of beehives.”  (Note:  If this statement is not added to wood preservation labels, bee studies are required as noted above.)

###### REFERENCES

42899002  Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to Rainbow Trout, Oncorhynchus mykiss, Under Flow-Through Test Conditions: Lab Project Number: J9006014A.  Unpublished study prepared by Toxikon Environmental Sciences.  43p.

42899003  Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to Bluegill, Lepomis macrochirus, Under Flow-Through Test Conditions: Lab Project Number: J9006014B.  Unpublished study prepared by Toxikon Environmental Sciences.  44p.

42899004  Ward, G.; Davis, J. (1993) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to Embryos and Larvae of the Eastern Oyster, Crassostrea virginica, Under Static Test Conditions: Lab Project Number: J9006014I.  Unpublished study prepared by Toxikon Environmental Sciences.  45p.

42899005  Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Chronic Toxicity to the Water Flea, Daphnia magna, Under Flow-Through Test Conditions: Lab Project Number: J9006014F.  Unpublished study prepared by Toxikon Environmental Sciences.  52p.

42902401  Carr, K.; Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to Coho Salmon, Oncorhynchus kisutch, Under Flow-Through Test Conditions: Lab Project Number: J9006014C.  Unpublished study prepared by Toxikon Environmental Sciences.  44p.

42902402  Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to the Mysid, Mysidopsis bahia, Under Flow-Through Test Conditions: Lab Project Number: J9006014J.  Unpublished study prepared by Toxikon Environmental Sciences.  42p.

42902403  Lintott, D.; Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Toxicity to Embryos and Lavae of the Rainbow Trout, Oncorhynchus mykiss, Under Flow-Through Test Conditions: Lab Project Number: J9006014E.  Unpublished study prepared by Toxikon Environmental Sciences.  64p.

42902404  Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Toxicity to the Saltwater Alga, Nitzschia punctata, Under Static Test Conditions: Lab Project Number: J9006014M.  Unpublished study prepared by Toxikon Environmental Sciences.  46p.

42902405  Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to the Saltwater Green Alga, Dunaliella tertiolecta, Under Static Test Conditions: Lab Project Number: J9006014L.  Unpublished study prepared by Toxikon Environmental Sciences.  46p.

42927101  Hakin, B.; Rodgers, M.; Grutzner, I. (1991) Ro 17-0099/000 (Copper 8-Quinolinolate): Acute Oral Toxicity (LD50) to the Bobwhite Quail: Lab Project Number: HLR 184-901854: RCC 284253.  Unpublished study prepared by Huntingdon Research Centre Ltd.; RCC UMWELTCHEMIE AG. 53p.

42927102  Hakin, B.; Rodgers, M.; Grutzner, I. (1991) Ro 17-0099/000 (Copper 8-Quinolinolate): Acute Oral Toxicity to Mallard Duck: Lab Project Number: HLR 185-901733: RCC 284264.  Unpublished study prepared by Huntingdon Research Centre Ltd.; RCC UMWELTCHEMIE AG. 38p.

42927103  Hakin, B.; Rodgers, M.; Grutzner, I. (1991) Ro 17-0099/000 (Copper 8-Quinolinolate): Dietary Toxicity (LC50) to Bobwhite Quail: Lab Project Number: HLR 187-901685: RCC 279854.  Unpublished study prepared by Huntingdon Research Centre Ltd.; RCC UMWELTCHEMIE AG. 39p.

42927104  Hakin, B.; Rodgers, M.; Grutzner, I. (1991) Ro 17-0099/000 (Copper 8-Quinolinolate): Dietary Toxicity (LC50) to the Mallard Duck: Lab Project Number: HLR 186-901684: RCC 274228.  Unpublished study prepared by Huntingdon Research Centre Ltd.; RCC UMWELTCHEMIE AG. 39p.

43073501  Ward, G. (1993) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to the Freshwater Alga, Skeletonema costatum, Under Static Test Conditions: Lab Project Number: J9006014K.  Unpublished study prepared by Toxikon Environmental Sciences.  45p.

43228401  Ward, G. (1994) Oxine Copper (Copper 8-Quinolinolate): Acute Toxicity to the Water Flea, Daphnia magna, Under Flow-Through Test Conditions: Lab Project Number: J9006014D.  Unpublished study prepared by Toxikon Environmental Sciences.  44p.

43563701  Nozaka, T. (1995) Acute Toxicity of Copper 8-Quinolinolate to Rainbow Trout: English Version: Lab Project Number: 90753:E89-0753.  Unpublished study prepared by Kurume Research Labs.  12p.

Testing with the more sensitive species (freshwater or estuarine/marine) based on acute toxicity test results is required.

Testing with the more sensitive species (freshwater or estuarine/marine) based on acute toxicity test results is required.