RCS WRC –07 Agenda Item Scope and High-Level Work Plan

Agenda Item 1.20: to consider the results of studies, and proposals for regulatory measures, if appropriate, regarding the protection of the Earth exploration-satellite service (passive) from unwanted emissions of active services in accordance with Resolution 738 (WRC–03).

Short Description:  Resolution 738 (WRC–03) calls for studies of compatibility between the Earth exploration-satellite (passive) service and unwanted emissions from particular active services in nine specified pairs of active and passive service frequency bands. It also resolves to recommend that WRC–07 review the studies and consider regulatory measures, if appropriate, to protect the passive service, considering the impact on the concerned services.

Background: Under part of Agenda Item 1.8.2, WRC–03 considered compatibility between unwanted emissions and the EESS (passive) in combinations of active and passive service bands. Proposals from APT and CEPT favored hard limits on active service unwanted emissions to provide the needed protection. These proposals were based primarily, but not exclusively, on Task Group 1/7 studies that had been documented in Recommendation ITU–R SM.1633. The United States, however, proposed that no changes be made to the Radio Regulations.

With nearly all administrations supporting continuation of studies at a minimum—the United States and a few others opposed this—the Conference approved Resolution 738 along with inclusion of the EESS (passive) protection issue in the agenda for WRC–07.

At the fourteenth plenary meeting of WRC–03, the delegate of the United States made the following statement regarding Agenda Item 1.20 of the proposed agenda for WRC–07 (§ 17.13 of Document 410):

The United States continues to believe that the only regulatory measures that could possibly result from the studies under WRC-07 agenda item 1.20 and its associated Resolution [738] are hard limits for terrestrial radio services. The imposition of such regulatory limits on communications equipment including aircraft communications, defence radars, and cellular handsets, among others, would represent a significant burden on equipment manufacturers and national regulators. Because of this, we believe that this agenda item will be extremely difficult to resolve at WRC-07.

CPM06–1 designated a task group of Study Group 1 as the responsible group for Agenda Item 1.20. At its November 2003 meeting, Study Group 1 established Task Group 1/9 to conduct studies and draft CPM text for this agenda item. The first CPM also designated Working Party 7C as the contributing group, and Study Group 6, along with Working Parties 4A, 8D, 9B and 9D as the interested groups.

Scope of Agenda Item:  Whereas WRC–03 Agenda Item 1.8.2 was not specific regarding the bands to be studied, Resolution 738 (WRC–03) for WRC–07 applies to six EESS (passive) bands and nine adjacent or nearby active service bands. For two combinations of active and passive service bands near 30 and 50 GHz, Resolution 738 limits studies to consideration of the impact of recommended fixed service unwanted emission limits (those resulting from studies documented in Recommendation ITU–R SM.1633) in Regions 2 and 3.

Resolution 745 (WRC–03), to be addressed under WRC–07 Agenda Item 1.17, calls for studies of a number of compatibility issues near 1.4 GHz. Apparently included in these studies are the effect of fixed service unwanted emissions on EESS (passive) operation in the 1400–1427 MHz band.

Key technical/regulatory issues to be resolved:

- Continuation of studies in Recommendation ITU–R SM.1633 to consider the probability of interference

- Impact of the increased sensitivity of remote sensing systems reflected in the May 2003 revision of Recommendation ITU–R SA.1029

- Applicability of No. 5.340.1 of the Radio Regulations

- Overlap of studies in support of WRC–07 Agenda Items 1.17 and 1.20

Key policy issues to be resolved:

- U.S. policy regarding methods to address Agenda Item 1.20

- Possibility of a solution not involving “hard limits”

- Effect of the U.S. statement at WRC–03 on the resolution of this agenda item

Work Plan:

- Engage the FCC in discussions to explore suitable solutions to this agenda item from a private sector perspective

- Develop an initial RCS position on the agenda item considering views of all affected Federal Agencies

- Contribute to and participate in Task Group 1/9 studies on the compatibility between EESS (passive) and unwanted emissions from active services

- Develop a proposal that would avoid unacceptable “hard limits” on the active services, but would offer a remedy to the EESS (passive) that the United States and other administrations could support

- Obtain support of CITEL and other administrations on the U.S. proposal

Reference Material:

- Resolution 738 (WRC–03)

- Recommendations ITU–R SM.1633 and SA.1029

attachment

RESOLUTION 738 (WRC-03)

Compatibility analyses between the Earth exploration-satellite 
service (passive) and active services

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that primary allocations have been made to various space services in the Earth-to-space direction such as the fixed-satellite service (FSS), mobile-satellite service and broadcasting-satellite service and/or to terrestrial services such as the fixed service, mobile service and broadcasting service, hereafter referred to as “active services”, in bands adjacent or nearby to bands allocated to the Earth exploration-satellite service (passive) (EESS);

b)	that unwanted emissions from active services may cause unacceptable interference to the EESS (passive);

c)	that for technical or operational reasons, the general limits in Appendix 3 may be insufficient in protecting the EESS (passive) in specific bands;

d)	that, in many cases, the frequencies used by the EESS (passive) sensors are chosen to study natural phenomena producing radio emissions at frequencies fixed by the laws of nature, and therefore shifting frequency to avoid or mitigate interference problems may not be possible;

e)	that Recommendation ITU-R SM.1633 provides a list of band-pairs and a methodology for conducting, and a framework for documenting the results of, the compatibility studies between active and passive services operating in adjacent and nearby bands;

f)	that according to Recommendation ITU-R SM.1633, the EESS (passive) in the band 31.3-31.5 GHz can be protected if the unwanted emissions of fixed-service systems (except high-altitude platform stations (HAPS)) operating in the band 31.0-31.3 GHz do not exceed –38 dBW in a 100 MHz reference bandwidth in the band 31.3-31.5 GHz;

g)	that according to Recommendation ITU-R SM.1633, the EESS (passive) in the band 52.6-54.25 GHz can be protected if the unwanted emissions of fixed-service systems operating in the band 51.4-52.6 GHz do not exceed –33 dBW in a 100 MHz reference bandwidth in the band 52.6-54.25 GHz;

h)	that it is necessary to ensure an equitable burden sharing for achieving compatibility between active and passive services,

recognizing

a)	that Recommendation ITU-R SM.1633 addresses the compatibility between the EESS (passive) and active services operating in adjacent or nearby bands;

b)	that the relevant annexes in Recommendation ITU-R SM.1633 need further refinement;

c)	that Recommendation ITU-R SM.1633 does not address all band-pairs in the Table below where compatibility analyses need to be conducted;

d)	that additional measures may be required to protect the EESS (passive) from unwanted emissions of active services for the band-pairs listed in the Table,

resolves

1	to invite ITU-R to continue or to initiate studies on the compatibility analyses between EESS (passive) and the corresponding active services as listed in the Table with a view to updating Recommendation ITU-R SM.1633 or developing additional Recommendations;

Table

| EESS (passive) band | Active service band | Active service |
| --- | --- | --- |
| 1 400-1 427 MHz | 1 350-1 400 MHz | Fixed service (FS) Mobile service (MS) Radiolocation service |
| 1 400-1 427 MHz | 1 427-1 429 MHz | FS, MS (except aeronautical mobile service) and space research service (Earth-to-space) |
| 1 400-1 427 MHz | 1 429-1 452 MHz | FS and MS |
| 23.6-24 GHz | 22.55-23.55 GHz | Inter-satellite service |
| 31.3-31.5 GHz | 30-31 GHz | FSS (Earth-to-space) |
| 50.2-50.4 GHz1 | 50.4-51.4 GHz1 | FSS (Earth-to-space)1 |
| 50.2-50.4 GHz1 | 47.2-50.2 GHz (Regions 2 and 3) 49.44-50.2 GHz1 (Region 1) | FSS1 |
| 1 Studies in this band must take into account No. 5.340.1 of the Radio Regulations. |  |  |

2	to invite ITU-R to further study the impact of implementing the values provided in considering f) and g) for unwanted emissions of fixed-service systems operating in Regions 2 and 3, taking into account that the impact on fixed-service systems in Region 1 has already been investigated;

3	to recommend that WRC-07 review the results of the studies identified in resolves 1 and 2 in order to consider regulatory measures, if appropriate, to ensure the protection of the EESS (passive) operating in the bands listed in the Table from unwanted emissions of active services operating in the corresponding bands while taking into account the impact on all concerned services of implementing or not implementing such measures,

invites administrations

1	to provide the relevant characteristics of active and passive service systems operating in the bands identified in the Table;

2	to actively participate in the studies identified in resolves 1 and 2.

Author: Rob Haines   Document Date:  November 26, 2003 (Rev1)