Navy Response to OSHA’s Request for Comments

Section 1915.81: Housekeeping

- OSHA requests comment on the extent to which shipyard employers currently pay for waterproof footgear, how much such footgear costs to purchase and maintain, and whether OSHA should include a specific employer payment requirement in the final rule.(Page 72459)

Navy Response:  Waterproof footgear is provided when job assignment warrants.  Cost varies significantly with type of footgear provided.  For this reason we recommend that footgear be provided to personnel as necessary in lieu of cash payments or allowances.  Footgear is purchased from multiple sources and in various lots.  Cost figures are not currently available.

Section 1915.82: Lighting

- OSHA solicits comments on the proposal as well as alternative approaches such as the one used in the Longshoring Standard or the requirements of the ANSI/IESNA standard.  Are the proposed lighting intensities adequate?  Does the table adequately address all areas of shipyard employment?  If not, what areas need to be added? (Page 72460)

Navy Response:  The proposed lighting intensities is considered to be adequate for suitable aisles and routine work areas but consider specifying minimum lighting requirements for all "walking and working surfaces" in shipyards to be unnecessarily burdensome and costly.  The light intensity levels listed in Table F-1 for General landside walkways (5 FC) are not achievable.  It is recommended that this requirement be removed from Table F-1.

- SESAC also recommended adding a requirement that only a “qualified person” be permitted to replace or cap unguarded, damaged bulbs that have exposed filaments (Docket SESAC 1991, Ex. 100X, p. 84).  OSHA has not adopted this suggestion, because the Agency believes that the existing and other proposed standards address this hazard. OSHA requests comment on this recommendation, and whether it is needed, in light of other existing and proposed regulatory provisions that deal with lighting, electrical safety, and guarding of temporary lights. (page 72460)

Navy Response:  We agree with OSHA.  Existing requirements adequately address this requirement and do not need to be addressed in Subpart F.

Proposed paragraph (b)(7) revises the existing standard by requiring that splices have insulation that "exceeds” that of the cable.  The existing provision allows the use of splices where the insulation is "equal" to that of the cable.  OSHA requests comment on

the proposed revision.  Does the proposed requirement provide sufficient protection for employees?  Is weather a factor in determining what insulation to use?  In your establishment and industry, what practices are followed regarding insulation of splices?  Should OSHA propose a more specific requirement, for example that splices have insulation at 1-½ times greater than that of the cable? (Page 72461)

Navy Response:  The existing standard 1915.92(b)(2) is adequate. This standard currently complies with the NFPA, NEC 70, Article 400.9 Splices. This article allows splicing of hard-service and junior hard-service cords 14 AWG and larger that comply with Article 110.14(B) and the completed splice retains the insulation, outer sheath properties, and usage characteristics of the cord being spliced. Article 110.14(B) requires both mechanical and electrical securing of the cord. Increasing the insulation value provides no added value to employee protection.  Weather plays no factor in determining the insulation used. We comply with the NEC for splicing. OSHA should not propose a specific requirement for the reasons listed above.

- Proposed paragraph (c) addresses the use of handheld portable lights in work areas that do not have permanent or temporary lighting or such lighting is not working or is not readily accessible.  OSHA requests comment on the proposed provision.  In your establishment, when are employees provided with and required to use handheld portable lights to enter an area?  Are there other situations where handheld portable lights are needed? (Page 72461)

Navy Response:  Handheld portable lights are provided when conditions warrant and/or as requested by employees. They are always available for distribution as needed. Handheld portable lights are also used  for tank inspections.

5. 	The proposal also makes explicit that the employer must ensure that handheld portable lights are readily available in the immediate area where employees are working.  Implicit in the proposal is the obligation that the employer provide handheld portable lights in numbers that are adequate to ensure that all employees are able to move about and exit the area safely.  OSHA requests comment on the proposed provision.  Should OSHA apply this provision to any area where landside or shore-based lighting provides the only illumination?  Should OSHA include an exception to the rule when natural sunlight suffices? (Page 72461)

Navy Response:  Ships are inherently dark and the use of handheld portable lights is beneficial.  Landside or shore based lighting is provided to work areas outside of industrial areas ashore to include office spaces.  It is felt that providing handheld portable lights in these areas should not be required.  We also agree that there should be an exception to the rule where natural sunlight provides adequate illumination.

Section 1915.84: Work in Confined and Isolated Spaces

- Specifically, OSHA requests comment on whether the section should be limited to employees working alone in either a confined or isolated space.  Should OSHA address the hazards associated with working in confined spaces in subpart B confined and enclosed spaces instead of subpart F?  In your establishment and industry, are employees working in confined spaces or alone in isolated spaces checked frequently during the work shift and accounted for at the end of the work shift?  OSHA requests data and information on any injuries, fatalities, or near-misses that have occurred during the last five years due to an employee working in a confined space or alone in an isolated area.  If any incidents have occurred, what measures have been instituted to ensure that employees working in these areas are safe? (Page 72463)

Navy Response:  This section should be limited to personnel working alone in (non-enclosed/non-confined) isolated spaces. We also believe that enclosed/confined space requirements as contained in Subpart B adequately address those issues and do not need to be included in Subpart F.  Periodicity of checks is based on supervisory judgment and personnel are accounted for at the end of the shift.  Routine periodic checks and supervisory oversight have proven adequate to meet this requirement.

- OSHA also requests comment on whether the section should require that employers establish a system or some form of a signal to indicate when a single employee enters a confined space or a cofferdam to perform work.  For example, should OSHA require employers to have a system where employees leave their picture identification (or some other easily identifiable flag) outside the entrance to alert other employees that someone is inside working? (Page 72463)

Navy Response:  We believe frequent checks and proper supervision are adequate protection for employees, and are a more practical solution than adopting the suggested system.. Some isolated spaces have multiple access points requiring a signal device at all entrance/exit points.  When leaving the area all signals would have to be removed.  This could result in two possible failures; 1) False negatives: A little used access point is missed when posting  "signal flags" indicating falsely that the spaces is unoccupied or; 2) False positives: One of the many "signal flags" is not removed when the space is emptied indicating that someone is not accounted for.

Section 1915.87 Medical and First Aid Services

- OSHA requests comment on this provision.  In your establishment and industry, what measures are in place to ensure that first aid and medical services are readily accessible?  Should the final standard specify a maximum time within which first aid and medical services must be available?  For example, should the final standard specify that employers must ensure that first aid and medical services are initiated within three to five minutes of the discovery or report of an injury? (Page 72465)

Navy Response:  Do not set a pre-determined time by which first aid must be available.  There are numerous circumstances that can arise to slow response; however, backup response plans are established.  Setting a maximum time will require additional resources to train/hire first aid responders as ‘backups’ to established Navy first responders.

- OSHA requests comment on the proposed first aid training requirement.  Should the final standard require that first aid providers have a valid first aid and CPR certificate?  Should the final rule specify the areas in which first aid providers must be trained?  Should OSHA include an appendix similar to that in §1910.266 or 1918.97 in the final rule?  If not, why not?  If so, what should the program include?  Should the program include hands-on exercises?  Should the final rule include a requirement that whatever first aid training program and trainer/provider the employer uses, that the program and/or trainer be certified by a nationally recognized first aid organization?  Please explain.   In your establishment and/or industry, what training and certification do first aid providers have and does it include CPR training?  What organizations, if any, conduct the first aid training and certification?  How frequently do first aid providers have refresher training? (Page 72466)

Navy Response:  Revision should say something to the effect of “ensure there are employees qualified to provide first aid at each work location if medical services are not readily available.”  Emergency Medical Services are believed to meet the definition of supplying qualified employees at each work site.  First aid requirements and thus the content of a qualified training program to meet these requirements will vary greatly.  A non-mandatory appendix outlining basic first aid training in CPR, assessing and stabilizing injured personnel and wound treatment would be helpful.

3. 	OSHA proposed to revise the existing standard to require that first aid supplies be provided “at each work location.” (Page 72467)

Navy Response:  Fire Emergency Services are considered to be the “first aid providers.”  Unless ship first aid kits meet the intent, personnel would have to carry a first aid kit with them, unless kits were placed at multiple locations on each pier. Revision should say something to the effect of “if medical services are not readily available, first aid kits will be readily available to each work location.”

4.	OSHA requests comment on whether shipyards should be required to have AEDs as part of their first aid and medical services.  If not, why not?  If so, should the requirement apply to all shipyards or be limited to certain types of work or work locations (e.g., remote work areas, work where employees are exposed to electrical hazards, shift work)?  What criteria should employers use to determine whether and how many AEDs should be provided and where they should be located?  In your establishment and industry are AEDs provided?  If not, why not?  If so, how many are provided and what criteria were considered in making that determination?  Who is trained and authorized to operate the AEDs? (Page 72471)

Navy Response:  The Navy does not believe AEDs should be “required” as part of their first aid and medical services.  Rather, Naval Shipyards have the discretion to decide whether AEDs should be installed at their shore facilities because: (1) this approach follows Congressional intent plainly stated in the Cardiac Arrest Survival Act of 2000 (CASA, 42 U.S.C. 238p(a) & 238q(c)(1)(A)) whereby Congress expressly does not require Federal agencies to place AEDs in Federal facilities; (2) in CASA, Congress tasked the Secretary of Health and Human Services (HHS) to issue the guidelines for Public Access Defibrillator Programs in Federal facilities consistent with the statute; and (3) relevant factors at each Naval Shipyard vary, which may affect the decision whether to install AEDs.

To determine whether and how many AEDs should be installed, the Navy believes a needs assessment should be conducted that includes but is not necessarily limited to the following factors: adequacy of emergency response time, demographics of the workforce, history of incidence of cardiac episodes, the type of work performed, and the physical layout of the Naval shipyard.  These factors are consistent with the “Guidelines for Public Access Defibrillation Programs in Federal Facilities” (66 Fed. Reg. 28495 (May 23, 2001)) promulgated by HHS and the General Services Administration under CASA.

Portsmouth, Norfolk, Puget Sound and Pearl Harbor Naval  Shipyards already completed a needs assessment for installation of AEDs at their shore facilities.  Presently, AEDs have been installed in at least one shore facility.  The decision to install AEDs at the remaining Navy shipyard shore facilities will be made based on the results of their needs assessments.

Section 1915.88: Sanitation

- OSHA requests comment on the proposed requirement for location of sanitation facilities.  In particular, OSHA requests comment on whether the final rule should contain more specific requirements for the location of sanitation facilities, especially toilet facilities.  For example, should the final rule specify maximum distances, maximum walking times (e.g., 5 or 10 minutes), or other objective criteria for determining where sanitation facilities must be located in the workplace?  Should different specifications be developed for specific types of sanitation facilities?  OSHA seeks information on where sanitation facilities are located and what criteria are used to make this determination. (Page 72481)

Navy Response:  We do not believe more prescriptive requirements are needed.  Labor/Management negotiations and productivity issues results in ample facilities being provided.

2.	OSHA requests comment on this provision.  OSHA seeks information on the measures in place to ensure that sanitation facilities and supplies are maintained in clean, sanitary and serviceable condition.  How often are sanitation facilities inspected, cleaned,

and restocked?  Are there different procedures and/or schedules for portable toilet facilities as opposed to other sanitation facilities? (Page 72482)

Navy Response:  The frequency of servicing and cleaning varies from daily to weekly, based on the type of facility, number of employees serviced and location and is addressed via contracts with janitorial services and portable toilet vendors.  Contract requirements are established, reviewed and enforced by regional contracting organizations.

3.	OSHA requests comment on the proposal.  Where and to what extent is single use drinking water bottles used in your shipyard? (Page 72482)

Navy Response:  Single use drinking water bottles should be a recognized option.  Single use drinking water bottles are provided to supplement permanent facilities on a case by case basis as needed (e.g. in remote locations during dry-docking evolutions during summer months).

4.	OSHA is also considering adding a requirement to the final standard requiring employers to ensure that drinking water is “suitably cool.”  OSHA requests comment on this issue.  OSHA seeks information on the measures that have been implemented to ensure that drinking water is cool, especially for employees working on board vessels or in hot and humid weather. (Page 72482)

Navy Response:  The term "suitably cool" is too subjective and should not be part of the requirement.  Water is supplied for fluids replenishment and is kept shaded or in thermal coolers to prevent overheating prior to use.

5.	OSHA requests comment on the proposal to retain the 1:15 toilet ratio from the existing standard.  Should OSHA adopt the 1:9 ratio in the current ANSI Z4.1 and IPC 2003 standards?  Would such adoption significantly improve OSHA’s protection of employee health, and in what manner?  What costs, if any, would result?  If OSHA were to adopt the ANSI/IPC table, should its application be limited in any way, such as to facilities built after a certain date (e.g., the date the ANSI or IPC standards were adopted)? (Page 72482)

Navy Response:  In general, facilities (including industrial and support areas to which the standard applies and other areas (e.g. housing) to which it does not) are designed to meet or exceed the current version of the international plumbing code (IPC) and are upgraded accordingly during normal renovation cycles.  Therefore no significant cost/impact would be expected from proposed change.

Recommend, however, the standard reference the international plumbing code versus providing specific requirements and be clear that immediate retroactive compliance is not intended.  Cost estimates for upgrading facilities are not currently available.

6.	OSHA requests comment on whether the final rule should contain specific requirements for the location of toilet facilities in shipyards.  If not, why not?  If so, what specifications should OSHA use?  Should the same or different specifications apply for both sewered and portable toilets?  Please explain. (Page 72482)

Navy Response:  We do not believe more prescriptive requirements are needed.  Labor/Management negotiations and productivity issues results in ample facilities at our shipyards.

7.	OSHA requests comment on the proposed requirements for portable toilet facilities.  What additional requirements, if any, should the final rule include in order to ensure that portable toilet facilities provide a level of service close to that provided by sewered toilet facilities? (Page 72483)

Navy Response:  We believe the proposed requirements are adequate.

8.	OSHA requests comment on whether the final rule should require employers to provide portable toilet facilities in these types of situations.  If not, why not?  If so, in what situations should they be required?  How many portable toilets, at a minimum, should employers be required to provide?  For instance, should OSHA adopt the ratios (i.e., toilets per employees) established in the ANSI Z4.1 standard?

OSHA requests comment on the use of portable toilet facilities in shipyards.  When and where are portable toilet facilities used?  What factors determine how many to provide and when and where to provide them? (Page 72483)

Navy Response:  Portable toilets are used to handle the flux in the number personnel related to increases and decreases in shipyard activity at different locations and ship's availability to supplement sewered facilities.  The numbers provided are based on the ANSI Z4.1 and IPC standards.

9.	OSHA requests comment on the proposed exemption.  Should OSHA limit these exemptions in any way?  For example, with the increasing availability of waterless cleaning agents, should OSHA require that mobile crews be provided with such supplies?  What measures do shipyards currently use to ensure that mobile crews have immediate access to transportation to nearby toilet facilities? (Page 72483)

Navy Response:  We believe the proposed exemptions are adequate.

10.	In paragraph (e)(1), OSHA proposes to add a requirement that hand washing facilities be located "at or adjacent to each toilet facility," sewered and portable toilet facilities alike.   OSHA requests comment on the proposal. (Page 72484)

Navy Response:  We agree that requiring provision of hand washing facilities at or adjacent to toilet facilities is reasonable and appropriate.

11.	OSHA requests comment on the proposal to allow the optional use of waterless cleaning agents.   In your establishment, to what extent are waterless cleaning agents used?  If waterless cleaners are used, have they been received favorably by employees, and have employees experienced any problems with the cleaners (e.g., allergic reaction)? (Page 72484)

Navy Response:  The use of waterless cleaning agents is a viable option, enabling the provision of hand washing facilities at all toilet facilities.  Some waterless hand cleaners are in limited use in the shipyards, but data is not currently available on employee’s acceptance of this alternative.

Sections 1915.89 and 1910.147, Control of Hazardous Energy (Lockout/Tagout)

1.	‘‘Shipyard employment’’ is defined in § 1915.4(i) to mean ‘‘ship repairing, shipbuilding, shipbreaking, and related employment.’’  The proposal also adds language to clarify OSHA’s longstanding position that subpart F applies to shipyard employment ‘‘regardless of geographic location’’ of the shipyard activity.  Thus, if employees are performing shipyard employment activities, including but not limited to performing them onboard vessels and vessel sections and in landside facilities on navigable waters, the proposed requirements would apply. Likewise, if employees are performing shipyard employment activities at a location that is not contiguous to a vessel, the proposed requirements also would apply.  The proposal also clarifies that subpart F applies to any employer, regardless of whether the employer owns the vessel or shipyard, whose employees perform shipyard employment activities. The existing policy will continue to apply under the revised rule.

Recently published injury statistics on the commercial fishing industry also support the need to address hazardous energy during servicing operations onboard floating fish factories.  In light of these incidents, OSHA proposes to change its existing policy on the coverage of servicing and maintenance activities onboard commercial vessels, particularly fish processing vessels. In short, OSHA proposes adding language to § 1915.89 and § 1910.147 specifying that: • Proposed § 1915.89 applies to the servicing of ship’s systems by any employee, including but not limited to, ship’s officers and crew of the vessel (see proposed   §  1915.89(a)(2)(i)(A)); • Proposed § 1915.89 applies to the servicing of machines, equipment and systems that employees use in the course of performing shipyard employment operations (see proposed § 1915.89(a)(2)(i)(B)); and • Existing § 1910.147, and not proposed § 1915.89, applies to the servicing of equipment onboard vessels that is used for inherently general industry operations such as fish processing (see § 1910.147(a) and proposed § 1915.89(a)(2)(iii)(C)).

Background and current policy.

In order to fully explain OSHA’s proposed changes, it is important to understand OSHA’s current policy on the coverage of commercial vessels. This section discusses OSHA and U.S. Coast Guard authority over vessels, OSHA’s current exemption of maritime employment from § 1910.147, and OSHA’s current policy concerning application of § 1910.147 to floating fish processors.

Coast Guard/OSHA authority over vessels.

Both OSHA and the U.S. Coast Guard have authority for the safety and health of employees onboard vessels. The Coast Guard has statutory authority to prescribe and enforce regulations affecting safety and health onboard inspected vessels and has exercised that authority. Therefore, OSHA does not have authority over those vessels...; however, OSHA does have authority over uninspected vessels (hereafter ‘‘commercial vessels’’) to the extent that the U.S. Coast Guard has not regulated a specific hazard or working condition …. Moreover, to date, the Coast Guard has not regulated the control of hazardous energy during the servicing and maintenance of equipment on commercial vessels. Therefore, OSHA has authority to regulate hazardous energy onboard commercial vessels. (Page 72488)

Navy Response: Navy vessels are not regulated by the U.S. Coast Guard, and are not commercial vessels.   Further, the Navy has reservations about the statement that the Proposed Rule applies to shipyard employment ‘‘regardless of geographic location’’ because of the potential for the regulation to infringe on the authority of a Commanding Officer of a Navy vessel during a military operation.

2.	Interpretation of § 1910.147.  OSHA requests comment on the proposal to apply § 1910.147 to the servicing of fish processing and other equipment onboard vessels that is used for performing inherently general industry operations. What are the advantages and disadvantages of this proposed approach? Who services equipment onboard vessels that is used to perform inherently general industry operations? How frequently, if ever, do shipyard employees service general industry equipment onboard vessels and when does such servicing occur? What equipment onboard vessels, other than fish processing equipment, should OSHA classify as being used to perform inherently general industry operations? Should § 1915.89 or § 1910.147 apply to the servicing of inherently general industry equipment during an overhaul of the entire vessel? Please explain.  (Page 72490)

Navy Response:  The term “commercial” is not used under the § 1910.147 proposed rule, yet the term is used under the § 1915.89 proposal.  This would mean that the standard would apply to all U.S. Coast Guard inspected and un-inspected (commercial) vessels.  When Navy vessels are engaged in what could be considered general industry operations, such as, humanitarian relief or homeland mine countermeasures, all equipment and systems are still considered ship systems.  The use of two OSHA standards aboard ships would mean two LOTO programs aboard ship, contrary to OSHA’s rules for one employer system – intended not to confuse employees.  To minimize risk and confusion, all equipment on Navy vessels is controlled using one program.  The use of two hazardous energy control systems on ships would increase risk by increasing confusion.  Therefore, recommend the use of a single standard, CFR 1915.89, for servicing operations on all shipboard systems and equipment.

3.	OSHA asks for comment on the proposed definition of ship's systems. What machines, equipment and systems should the definition include? Does the proposed definition adequately distinguish between systems that are part of a vessel and equipment that is used for inherently general industry operations? Are there other approaches that would more clearly differentiate between those types of equipment and systems? (Page 72490)

Navy Response:  On Navy vessels, galley systems, waste processing, machinery repair shops, intermediate maintenance depots, temporary alterations, non-permanent mission modules, etc. are inherently shipboard vice general industry.  Some additional Warship’s specific systems to be included are weapons, surveillance, plastic waste processing, medical/dental facilities, motor rewind shops, etc.  Detachments and expeditionary forces which are not considered part of the crew are provided the same care as the crew.  If the Warship’s operational mission changes to "general industry" humanitarian, such as, providing for the care of refugees aboard, and electrical power and drinking water ashore, it is not prudent to change system definitions to general industry systems.  To minimize risk and confusion, all equipment on Navy vessels is controlled using one program.

4.	2nd Column OSHA question whether to use electric power generation transmission and distribution standard 1910.269 to avoid confusion over whether outside contractors could bring their own energy control program. (Page 72498)

Navy Response:  Recommend using the electric power generation transmission and distribution standard wording.  This provides one single standard method on board Navy vessels to decrease confusion and risk.  Recommend using the 1910.269 words in this area.

5.	Bottom of 3rd Column, OSHA questions the compatibility of the general industry approach for shipyard employment (Shipboard), vice a different plain language standard that is specifically tailored to shipyard employment (shipboard). (Page 72498)

Navy Response:   OSHA has used language in many interpretations under the electric power generation transmission and distribution standard 1910.269 that allows a tagout program, as long as “Tags Plus” is in affect.  The Navy vessels’ expert based tags plus system under the control of the ship Commanding Officer provides the Commanding Officer ultimate control of what is happening aboard his/her ship in accordance with U. S. Navy Regulations.  Navy vessels want to keep the hazardous energy control procedure standard and as simple as possible for the military sailors aboard ship by maintaining a single standard, which should flow from one OSHA rule.  Our group Tags Plus expert -based hazardous energy control program involves an interaction of expert systems operators and shipyard maintenance personnel.  The responsibility for hazardous energy control rests with both the ship’s crew and with the shipyard employed hazardous energy control expert(s).    The shipyard servicing personnel (e.g., craftsmen, tradesmen) are not systems experts.  This is similar to the electric power generation transmission and

distribution model under 1910.269, with operators providing the isolation for the maintenance personnel.

Recommend OSHA provide a section for this standard (as well as all LOTO standards) to provide detailed requirements for a “Tags Plus” Hazardous Energy Control Program.

6.	Bottom of 3rd Column, OSHA questions if incident investigation should be required. (Page 72498)

Navy Response:   The Navy has a robust program for formal investigation of energy control problems on board Navy vessels.  In addition to general Navy requirements for detailed investigation, cause analysis, and determination of corrective actions for problems resulting in injuries, Naval shipyards have implemented formal investigation of process errors which indicate significant weakness in the execution of energy control processes.  It is this intense focus on and formal resolution of smaller problems that results in the elimination of more serious problems.  All safety programs need to include a formal investigation process which should include documented problem definition, cause analysis and corrective action determination.  The Navy agrees that incident investigation is an appropriate requirement to be included in the standard.  In order to maintain the level of quality and frankness necessary to assist in the continuation of a successful proactive program, the Navy will continue to maintain the information as privileged information.

Section V., Executive Summary of the Preliminary Economic and Initial Regulatory Flexibility Screening Analysis

1.	Economic analysis - Neither the benefits nor the cost of this proposed rule exceed $100 million.  OSHA estimates that the proposal will cost employers in the shipyard employment industry about $0.6 million annually to implement. (Pages 72504, 72506)

Navy Response:  When proposing new regulations, OSHA performs an industry cost impact assessment.  Each US Navy program with a ship under contract is likely to get a large bill when these regulations are put into effect.  The Portsmouth and Norfolk Naval Shipyards, and Puget Sound and Pearl Harbor Naval Shipyards and IMFs provided estimated cost impact to change from the current Warship shipboard expert based group Tags Plus system to a LOTO program with off-the-shelf procedures for civilian maintenance personnel to lock out their own isolations.  This worst case estimate was that over $30 million per shipyard per year would be required. This estimate alone exceeds the annual $100 million threshold at these four shipyards.  Therefore, we disagree that the costs associated with the rule will not exceed $100 million nationwide, and disagree that the cost per shipyard is only about $0.6 million annually.

2.	OSHA examined several data sources to identify and characterize the risks shipyard employees face from the hazards this proposal addresses. These data show, for

example, that the shipyard industry has one of the highest rates and severity of workplace injury of all private sector industries. (Page 72505)

Navy Response:  Public sector (Naval Industrial Activities) data was not included, yet this proposal is being applied to the public sector as well as the private sector.  The US Navy has many authorized civilian employees servicing Commissioned Ships.  This includes over 24,000 at Portsmouth and Norfolk Naval Shipyards, and Puget Sound and Pearl Harbor Naval Shipyards and IMFs  and thousands from other Naval/Defense Commands.  Also, hundreds of Naval/contractor alteration installation teams and other System Specialists, who have not previously been considered “shipyard” employers, would now be related shipyard employers.

Navy Comments on Proposed Changes to Subpart F

The Navy in general is supportive of the Proposed Rule, with the changes or interpretations provided below.

Section 1915.80: Scope and Application; other General Areas

1.	“The provisions of this subpart apply to general working conditions in shipyard employment, regardless of geographic location, including work onboard vessels, vessel sections, and landside operations.” (Page 72511)

Navy Response:  Due to the unique complexity of systems on Navy vessels, shipboard energy control should be excluded from application.  The Navy has developed shipboard energy control process requirements codified in formal Naval Instructions.  These instructions were specifically designed to provide for work safety when dealing with energy control of complex systems which require specialized system qualification, knowledge and experience as well as multi-layered technical reviews to ensure proper isolation of work areas is established.  Work isolation is often directly linked to maintaining combat system requirements and ship safety (fire protection, list, trim, buoyancy, etc) on combat ships with redundant systems.  This required the development of an energy control process that utilizes system experts and trained work control professionals.  The lockout/tagout guidance of this proposed rule, if applied as written to Navy vessels, would degrade the safety of shipboard work.  Placing responsibility for adequate isolation and system conditions in the hands of personnel performing work is unsafe when the equipment and systems are so complicated that workers could not be reasonably expected to correctly determine safe isolation or it’s affect on critical ship systems.

The point might be made that some systems and equipment on Navy vessels are less complicated than others and therefore this rule could be applied in those areas.  While this is true, it would lead to having two fundamentally different energy control processes being executed by the same crew and workforce depending on which system or equipment was involved.  This would lead to confusion and errors that would degrade safety.

While the current Warship process meets and in most ways exceeds the requirements in this proposed rule, the fundamental approach where an authorized employee (servicing personnel) applies personalized locking devices to shipboard components is different enough that making this rule applicable to Navy vessels may require cumbersome changes to the Navy process with no associated benefits in worker safety.  Such changes would encumber proven and very successful Navy work and energy control processes leading to inefficiency and increased risk of process errors resulting in degraded safety.

Recommend 1915.80 be modified to say “The provisions of this subpart do not apply to energy control for any work performed on Navy vessels (new construction and decommissioned included) where such work is covered by Warship work control and tag-out procedures.”

Section 1915.81: Housekeeping

1.	Throughout the proposed section OSHA uses the term “walking and working surfaces” in place of the list of the specific areas and surfaces contained in the existing section.  In proposed §1915.95, OSHA defines “walking and working surfaces” to mean any surface on or through which employees gain access to or perform job tasks.  Walking and working surfaces also include any surface upon or through which employees are required or allowed to walk or work in the workplace.  Walking and working surfaces include, but are not limited to, work areas, access ways, aisles, exits, gangways, ladders, ramps, stairs, steps and walkways.  Paragraph 1915.81 (a) reads; “The employer shall maintain good housekeeping conditions to ensure that walking and working surfaces do not create a hazard for employees.  The employer shall ensure that these conditions are maintained at all times.  (Page 72511)

Navy Response:  Replace “at all times” with "as necessary to provide hazard free access”.  This would allow temporary services to lie on decks and floors as long as adequate aisles and walkways are provided and maintained.

Delete "maintain good housekeeping."  Rather than "maintain good housekeeping conditions," the Navy recommends that the standard more simply read “maintain conditions to ensure walking working surfaces do not create a hazard for employees."  "Good housekeeping" adds an ambiguity without apparent benefit.

- Proposed paragraph (k) adds a requirement that combustible scrap be removed from the work area as soon as possible to reduce fire hazards. (Page 72511)

Navy Response:  Paragraph (k) need only apply to areas where hot work is performed.  Hot work requirements are covered elsewhere and this paragraph should be removed from subpart F or should clearly indicate it applies only to work areas where hot work is being performed.

3.	Paragraph (g): The employer shall ensure that slippery conditions, such as snow and ice, on walking and working surfaces are eliminated as they occur. (Page 72511)

Navy Response:  Add “as soon as practicable” in place of "as they occur".  In northern climates, elimination of snow and ice cannot be reasonably removed as it occurs.

Section 1915.82: Lighting

- Proposed paragraph (a) sets forth requirements that apply to lighting in all areas of shipyard employment.  The proposed general requirements would apply regardless of whether permanent or temporary lights are used.  The lighting intensity levels that would be required by table F-1 would not apply to emergency lighting or portable handheld lights. (Page 72511)

Navy Response:  We have large facilities that are comprised of many types of non-working areas such as parking lots, recreational facilities and outdoor walkways.  The light intensity levels listed in Table F-1 are not achievable for General landside walkways.  It is recommended that this general walking area requirement be removed from Table F-1.

2.	Paragraph (b) Temporary lights.  The employer shall ensure that temporary lights meet the following requirements:

(1) Lights with bulbs that are not completely recessed are equipped with guards to prevent accidental contact. (Page 72512)

Navy Response:  The term “completely recessed” needs to be more clearly defined.  We recommend to change “are not completely recessed” be changed to “extend beyond the plane of the lighting fixture opening.”

3.	Proposed paragraph (b)(7) revises the existing standard by requiring that splices have insulation that "exceeds” that of the cable.  The existing provision allows the use of splices where the insulation is "equal" to that of the cable. (Page 72512)

Navy Response:  The existing standard 1915.92(b)(2), equal to that of the cable, is adequate. This standard currently complies with the NFPA, NEC 70, and Article 400.9 Splices. This article allows splicing of hard-service and junior hard-service cords 14 AWG and larger that comply. Article 110.14(B) and the completed splice retain the insulation, outer sheath properties, and usage characteristics of the cord being spliced. Article 110.14(B) requires both mechanical and electrical securing of the cord.

4.	Proposed paragraph (c)(3) retains and simplifies the existing requirement (§1915.92(e)) on the use of handheld portable lights in any area that is not gas-free.  In such areas, the proposal would require that the employer ensure that only “explosion-

proof, self-contained” handheld portable lights are used (or other equipment approved by a nationally recognized testing laboratory (NRTL)). (Page 72512)

Navy Response:   Concur.  Only on rare occasions, in an emergency, would anyone enter a confined space that is not certified gas free. This would typically be emergency

response personnel (Fire Department), who would use appropriate lights.  We do not believe that the intent of this paragraph would apply to a Navy Competent person who inspect and certify spaces gas free.  These personnel test the atmosphere before entering and continue to test ahead of themselves.  They do not enter spaces, or sections of spaces, that do not meet the criteria of Safe for Entry.

Section 1915.83: Utilities

1.	Proposed paragraph (b) revises the requirement that ‘short bights’ be used when hanging steam hoses to reduce chafing and tension on Steam hoses which are hung overhead. (Page 72512)

Navy Response: Recommend “short bight” be more precisely defined by providing wording such as, “"When steam hose is hung in a bight or bights, the weight shall be relieved by appropriate lines that are spaced not to exceed 8 feet maximum along the entire run", so that definitive guidance can be developed and promulgated to workers.

2.		Proposed paragraph (b)(4) revises the existing requirement that steam hoses and temporary piping passing through walking or working areas be shielded to protect employees from injury due to accidental contact. (Page 72512)

Navy Response:  This requirement, coupled with the expanded definition of “walking and working surfaces”, would require the shielding of all steam hoses and temporary piping in our facilities.  We feel that this is not practical.

Recommend that paragraph 1915.83 (b)(4) be revised to read: “Each steam hose or temporary piping passing within close proximity to routine walking or working area is shielded to protect employees from contact.”  “Close proximity” should be determined by the local facility.

Section 1915.87: Medical Services and First Aid

1.		OSHA proposes in paragraph (e) to retain and expand the existing provision to require that quick drench facilities be provided where employees could be splashed with hazardous or toxic substances. (Page 72513)

Navy Response:  ANSI Z358.1 2004 establishes minimum performance and use requirements for eyewash and shower equipment for the emergency treatment of the eyes or body of a person who has been exposed to injurious materials.

Recommend that paragraph (e) be revised to incorporate requirements and language that is not more restrictive than the ANSI Z358.1 2004 standard for quick drenching or flushing the eyes and body.

Section 1915.89 of Proposed Subpart F, Control of Hazardous Energy (Lockout/Tagout)

1.	1915.89 (a)(2)(iii)(C)  Exception for inherently general industry operations equipment servicing to use a 1910.147 process. (Page 72515)

Navy Response:  Delete this exclusion.  Excluding inherently general industry operations creates a situation where two different standards are applied by the same personnel on the same ship.  Requiring personnel to apply different standards based on which equipment they are working on increases risk of confusion.

2.	1915.89 (b)(2)(iii)  Whenever the owner performs replacement, major repair or modification, the owner shall ensure that replacement is designed to accept a lockout device. (Page 72515)

Navy Response:  Under this provision all executed Warship work would require significant redesign and material modification to allow installation of locking mechanisms.  This would not be required for the same work executed at Private Shipyards.  The Navy Tags Plus system has been highly successful without the use of locking devices due to the unique operational controls in effect on board Navy vessels.  Only trained and qualified crew members and civilian system experts are allowed to operate ship’s equipment.  Equipment is operated in strict compliance with formal operating instructions and under the oversight and direction of their chain of command.  For those systems which do not readily accept a lock, the Navy would have to research, specify, procure and stock, readily available lock-out devices per component.  In addition to shipyard employers who do not own vessels being serviced, this requirement should not apply to Navy vessels.  Recommend add the following to the end of this paragraph “or to Navy vessels”.

3.	1915.89 (b)(3)(ii) Tagout Program equivalent safety measures. (Page 72515)

Navy Response:  The examples provided for achieving a level of safety with a tag-out program that is equivalent to a lockout program all relate only to physical modifications such as extra isolation points or hand wheel removal.  Such language could lead to an interpretation that only additional physical controls would suffice to meet this standard.  The Warship program has demonstrated that a Tags Plus system can meet safety levels equivalent to lockout programs using measures such as restricting operational authority to only trained crew or shipyard operators, and thoroughly training the workforce to not violate Danger Tags.

Recommend OSHA provide additional examples of measures that can be used to meet the equivalency standard.  The examples may include non-hardware related controls.  Some examples that the Navy vessels already use are two valve tagged isolations for high energy, single valve tested, tagged and locked for high energy with operator

representative approval, and the electric power generation transmission and distribution industry use of single tagged valve with a telltale drain/vent path opened.

4.	1915.89 (b)(4)(i)  Energy Control Procedures. (Pages 72510, 72511, 72514 - 72518)

Navy Response:  Warship systems and equipment are extremely complex with multiple energy sources. The servicing requirements vary considerably based on the nature and extent of repairs required.  To support this, the Navy Tags Plus and work authorization processes require the development of specific isolation and deenergization plans within prescribed standards which match the specific work scope and requirement.  It is not practical to maintain procedures for every piece of equipment, portion of a system, and maintenance scenario.  Doing so would be costly both for initial development and maintenance.  The electrical power industry under CFR 1910.269 is allowed to have a similar process to develop specific plans for each work order. The Navy uses a combination of documents to control hazardous energy.  The final procedures include a Work Authorization Form (WAF) linked to a Technical Work Document (TWD) and an associated Tag-out Record Sheet or Tag-out Line Item.  These WAFs and TWDs depend on procedures derived from the Tag-out Users Manual, Naval Ships' Technical Manual Chapters, Engineering Plant Manuals, Vendor Manuals, Engineering Operating Sequencing Systems, Combat System Operating Sequencing Systems, and other technical direction for proper operation, isolation, & testing.  The Navy asserts that the collection of the documents listed above constitutes the totality of and fulfills the need for energy control procedures.

5.	1915.89(b)(5)(ii)  Protective Materials (Danger Tags) singularly identified and not used for other purposes. (Page 72515)

Navy Response:  There is one standard Navy Danger Tag approved for shipboard use.  It is used to isolate energy for specific work items with authorized employees, and also to maintain safe ship conditions without specific work items.  The Navy asserts that a ship and ship equipment may present a danger (fire, flooding, overspeed, overheating, inadvertent movement, etc.) to any shipboard personnel, and therefore anyone on or in the ship may be an affected employee even if there is no authorized employee for that tag-out.  Therefore, Navy Danger Tags are only used for the stated purpose of controlling energy to prevent injury to (authorized and/or affected) employees.

6.	1915.89 (b)(5)(ii)(B)  Standardized tagout devices within the facility. (Page 72516)

Navy Response:  The Navy considers ship and base/shipyard as two separate facilities.  For a ship within a drydock at a Naval shipyard, a tagout (Navy group tags-plus) system is used on the shipboard facility, while a general industry lockout/tagout system is used for maintenance on the Naval shipyard facility (piers, drydocks, buildings, etc.).  Different Naval shipyards currently use lockout devices per Navy requirement ashore for the control of hazardous energy, which are intended to meet the requirements of 29 CFR 1910.147, general industry standard.  Similarly, a private shipyard may have a lockout/tagout program in shops, but aboard vessels, the Navy vessels tagout program is used.  To avoid confusion in the workforce, and create a clear visible indication of which process is being applied, the Naval shipyard utilizes different devices/tags for each

program.  Where a significant process difference is needed to safely execute work, the use of different devices/tags should be allowed to reduce the risk of personnel executing to the wrong process controls.  Shipboard and shipyard should be considered separate facilities, and standard devices allowed for a particular shipyard  hazardous energy control program may follow 29 CFR Part 1910.

7.	1915.89(b)(5) (ii) (C) (2) Durability of tag attachments device. (Page 72516)

Navy Response:  The Navy agrees; however, the standard must be adjusted due to circumstances where alternate means of attachment will be required as stated under 1910.269. The Navy will change Warship Danger Tag attachment device from waxed twine to one piece nylon cable tie or similar.  Relief for circuit breaker eyelets should be provided in the shipyard employment standard.  Note that the electric power generation, transmission and distribution standard interpretations allow a smaller cable tie with lower strength requirements – the best strength that can be accomplished as a non-citable violation.

8.	1915.89 (b)(6)  Periodic inspection. (Page 72516)

Navy Response:  Periodic compliance inspections are warranted and the Navy processes currently involve routine auditing.  However, the details contained within this standard address a simple work model which involves a specific procedure written for a specific component and worked on repeatedly by the same work team or group of authorized and affected employees.  The Navy updates/validates the hazardous energy control procedure - procedure as clarified in our comments to Page 72515, 1915.89 (b)(4)(i) - annually OR prior to each use.  The Navy training is not specific to the particular isolation, however, affected and authorized employees are briefed that they have the right to question and be shown the specific isolation for their job.

Recommend modify the standard to allow for audits and inspections of general process application when applicable in lieu of annual inspections.  Regarding inspecting the use

of energy control procedures, the standard should allow a general sampling surveillance and interview of authorized and affected employees found aboard ship by the organizations Quality, Safety, or other Hazardous Energy Control Experts as part of routine auditing.  Routine auditing can be prescribed as required per corrective actions.

9.	1915.89 (b)(7)(i)-(iii)  Tagout System Training. (Page 72516)

Navy Response:  Warship shipboard hazardous energy control program requires specific training of all personnel who execute process steps.  It also requires general training for all workers on generic energy control issues which could be affected by any worker.

Requiring all workers to be trained in aspects of the program for which they have no involvement or authority to apply is cumbersome.  Examples would be:

1)	tag selection, material, and legibility requirements – the Navy employs standard tags which employees do not have authority to affect.

2)	Secure attachment requirements – only personnel authorized to attach tags should require this training.

Also requiring training on tags evoking a false sense of security is inappropriate in that it implies that a tag based system is not safe.

10.	1915.89 (b)(7)(ii)(A) ; (iii) Expert authorized employee training; Retraining. (Page 72516)

Navy Response:  Recommend add: When using an expert representative as the authorized employee for group tags-plus applications, these experts will require training on ship’s systems and equipments, and the energy control process, including device and tag attachment applications.  To maintain an expert based authorized employee as qualified, re-training of these elements must occur every two years at minimum.

In the re-training section the words “whenever there is a change to their job assignment” is too ambiguous.  Recommend adding to this – whenever there is a change to their job assignment that changes their role or responsibility in performance of the energy program.

11.	1915.89 (b)(8) Energy Isolation by those who service/maintain. (Page 72516)

Navy Response:  Due to the complexity of systems and work requirements on Navy vessels, the safe execution of the Tags Plus program requires the application of Ship’s Force qualified system experts and full time shipyard work control professionals to communicate work scope versus ship system status requirements, and safely determine and execute the required conditions.  Note that the electric power generation, transmission and distribution industry operators provide the isolation for the servicing personnel.  The operators are similar to the shipyard expert authorized employee.  The servicing employee has the right and is given the opportunity to review their isolations.  On a large ship, this could mean several decks, spaces, and bulkheads – which may be more confusing than value added.  There is no value added for accounting for people on a

shiftly basis tied to several work authorizations/tag-outs.  Workers are assigned several work locations based upon availability of worksites, due to site interference management of other workers.  Recommend the standard allow system operators and work control experts to perform these functions when necessary due to system and work complexity.

12.	1915.89 (b)(9)  Notification of employees of application or removal of tagout devices. (Page 72516)

Navy Response: The requirement to notify employees prior to the attachment of tag-out devices is inappropriate.  This would only be appropriate if the employee had direct control of the associated equipment and needed to be involved in the decision to disable

that equipment.  There is also no merit in notification of employees after such devices have been removed since this would imply that there would be a period in time when such devices had been removed but before an affected employee was notified.  This is an unsafe order in that it would risk unprotected work from the time the first tags or devices were removed until the notification was completed.  Employees need to know when

lock-out/tag-out has been established and that they can now work safely.  They also need to know when they are no longer allowed to work and energy is to be restored to machine, equipment or system.

Recommend adding "Ship’s Departments and/or the authorized employee representative (s) make(s) a determination that needs to know about the shift in isolation and any precautions that need to be taken.”

13.	1915.89 (c)(5)(i)  Stored Energy removal sequence. (Page 72517)

Navy Response:  Due to the arrangement of systems, it is often not possible to remove stored energy after tag-out devices are applied.

Recommend delete the words “Following the application of lockout or tagout devices to energy isolating devices,”.  This leaves the key requirement that all stored energy must be relieved, but without a required order of performance which is not always possible.

14.	1915.89 (d)(3) , (e)(2)(ii), and (e)(3)(ii)(D) Tagout devices removal by the same employee who applied the tag. (Page 72517)

Navy Response:  A qualified repair activity person representing authorized employees shall concur with the removal of tagout devices prior to removal by qualified operators.  Due to the complexity of Warship systems, often with multiple affected employees performing maintenance and/or testing on interfacing/inter-connected systems, an expert based group tagout process is warranted.  The system knowledge level to correctly secure and isolate complex ship’s equipment is often beyond the capability of servicing personnel that perform work on the equipment.  Due to this complexity, even the expert group tagout approach of the Navy does not rely on the expertise of only one individual.

The Warship process requires multiple layers of expert reviews to be accomplished to ensure all energy control points are properly accounted for and secured for the specific described work areas.  The correct positioning of energy points and

attachment of closure devices and/or attachment of tags also requires multiple experts to validate. This expert group constitutes the “authorized employees” category.  This “authorized” group affix the tagouts prior to work being authorized to start and remove them only after each individual employee’s work is complete.

Recommend adding statement to paragraph 1915.89 (d)(3), “When system complexity exceeds the ability of servicing personnel to accurately identify all necessary energy control points and/or when each individual authorized employee does not have the necessary level of system knowledge and/or can not be allowed operational control of a system, an expert with specific systems and isolation methods qualifications can

represent the group as the “VESTED authorized employee”.  The authorized employee will utilize the VESTED authorized employee experts (via ship crew customers when necessary) to accomplish the draining, de-energizing and depressurizing of the equipment and/or systems, identify and establish isolations at all necessary energy control points and communicate these conditions to the authorized employees prior to the start of work.  The authorized employee will communicate to the VESTED authorized employee expert

when the work is done that required isolation.  The VESTED authorized employee expert will direct when the energy control can be removed.”

15.	1915.162 and 1915.163, Ship’s Boilers and Ship’s Piping. (Page 72520)

Navy Response:  This no longer needs to be included as 1915.89 now promulgates energy control.  It duplicates the requirements.

If this is not adopted, then the below will need included:

This states that any high temperature piping system with bolted joints must have a blank installed vice using a shut valve to provide isolation to perform work.  This appears to regard the maintenance work differently from the work required to break the piping joints to install blanks.  Since the system would have to be shut valve isolated to perform the work to un-bolt the joint and then install a blank to be tagged, the regulation appears to self conflict.  Breaking the flange and blanking a functional valve provides additional safety concerns over just shutting it.   The necessary premise in acceptable barrier criteria is that when a valve is used as a barrier it must be proven as an acceptable barrier.  This premise applies regardless of if the valve is bolted or welded into the piping system.  The standard should be modified to delete the requirement to always use blanks with bolted in joints.  The proposed rule will allow tagging vice mandatory locking of two welded in valves.  The Navy recommends that the standard be changed (similar to the electric power generation, transmission and distribution standard) to not require locking two welded-in valves, and to not require the disassembly and blanking of mechanically attached (flanged) valves as isolation for work from high temperature fluids/steam.  The Navy will require danger tagging two valves (welded in or mechanically attached (flanged)), and in cases where the danger tagging of two valves is not practical, danger

tagging and locking one valve.  The Navy uses an existing procedure to obtain Warship Commanding Officer permission to deviate by locking and tagging a single valve vice two valve isolation.

Recommend that the rule be changed to allow tagging two valves, a locked and tagged single valve isolation, or tagged single valve isolation with telltale (open drain or loosened flange).

Section 1915.90: Safety Color Code for Marking Physical Hazards

- OSHA proposes to incorporate the General Industry Standard 1910.144.

Navy Response:  This should apply only to shore facilities and temporary systems placed shipboard during repair. (Page 72518)

Section 1915.93: Motor Vehicle Safety Equipment, Operation and Maintenance

1.	OSHA proposes in paragraph (b)(2) that “The employer shall ensure that each employee uses the safety belt, securely and tightly fastened, at all times while operating or riding in a motor vehicle.” (Page 72519)

Navy Response:  For vehicles in Section (b)(1) where seatbelts have been provided employees will be required to utilize them.

Section 1915.95: Definitions

1.	Motor vehicle.  Any motor-driven vehicle operated by an employee that is used to transport employees, material, or property.  For the purposes of this subpart, motor vehicles include passenger cars, light trucks, vans, motorcycles, all-terrain-vehicles, powered industrial trucks and other similar vehicles.  Motor vehicle does not include boats or vehicles operated exclusively on a rail or rails. (Page 72519)

Navy Response:  Add “busses” to the list of included vehicles.