PPD/MD/Engineering Analysis Group

Fermilab

MSG-EAR-01-295

July 12, 2001

Dipole/Toroid Field Interaction

Bob Wands

Summary

A dipole magnet is placed in the bore of a series of toroid magnets. The clearance between the outer boundary of the dipole iron and the inner boundary of the toroid iron is ~ 1 cm. There are concerns that the field of each magnet may be unacceptably distorted by the presence of the other magnet.

A 2-d ANSYS finite element model of the two-magnet assembly was created. The BH used was the same that was applied to the most recent Minos toroid studies, and is based on measurements of Minos toroid steel. The BH data is shown in Table I. The BH curve is plotted in Figs. 1 and 2.

The total NI/coil for the dipole and toroids magnets were 36000 A-t, and 28800 A-t, respectively.

The field in the dipole across the air gap (line A-B of Fig. 3) is shown in Fig. 5. No asymmetry of field can be observed on the plot. Fig. 6 shows the field vectors in the dipole iron. Without the toroid, this field would be symmetric top-to-bottom in the figure. But the toroid coil forces much of the dipole flux downward, producing the field asymmetry in the dipole iron.

The toroid field was plotted along the two radii indicated by lines A-B and C-D of Fig. 4. Line C-D lies in the plane of the toroid coils; line A-B is midway between the two planes.  Figs 7 and 8 show the field with and without the dipole. Because the hole in the center of the toroid iron is not square, there is about 4% more steel available for flux at theta = 0, which accounts for the consistently lower field along that line when the dipole is not present.

With the dipole in place, the primary effect is a suppression of the field at theta = 90, and an augmentation of the field at theta = 0.

Table I. BH Curve for Minos Toroid Analysis

| H (A-T/m) | B (Tesla) |
| --- | --- |
| 0.399179 | 0.106296 |
| 24.19925 | 0.151608 |
| 39.90442 | 0.202609 |
| 48.37524 | 0.257182 |
| 57.58945 | 0.30322 |
| 62.86961 | 0.351813 |
| 68.89665 | 0.402339 |
| 74.65029 | 0.459434 |
| 83.44129 | 0.504142 |
| 88.50487 | 0.555545 |
| 95.59025 | 0.601927 |
| 110.5865 | 0.650416 |
| 124.4171 | 0.706708 |
| 137.0773 | 0.750248 |
| 150.1383 | 0.802527 |
| 167.3008 | 0.85276 |
| 183.6754 | 0.902306 |
| 211.0142 | 0.953658 |
| 238.6406 | 1.005324 |
| 263.6361 | 1.051782 |
| 299.0317 | 1.101619 |
| 334.14 | 1.152131 |
| 379.1933 | 1.201794 |
| 443.0544 | 1.251806 |
| 513.8499 | 1.301553 |
| 612.0904 | 1.351628 |
| 744.7709 | 1.40154 |
| 948.7924 | 1.451394 |
| 1292.537 | 1.501308 |
| 1885.41 | 1.551123 |
| 2880.497 | 1.601317 |
| 4310.95 | 1.651106 |
| 6168.308 | 1.701096 |
| 8435.199 | 1.751365 |
| 11062.14 | 1.801329 |
| 14145.06 | 1.851199 |
| 17729.06 | 1.901095 |
| 21916.56 | 1.951278 |
| 26949.63 | 2.001226 |
| 33486.56 | 2.05101 |
| 43749.89 | 2.101159 |
| 62908.96 | 2.151141 |

Figure 3. Dipole Cross section, and line A-B for Dipole Field Chart

Figure 4. Lines for Toroid Field Chart

Figure 6. Field Vectors in Dipole Iron Showing Distortion Due to Toroid

Toroid Coil