Aircrew Member Checklist – Duties & Responsibilities

Preflight Equipment Inspection

- Crew – Flight Gear under wing, on your side of aircraft - Observer – Untie Aircraft - Observer – Remove Pitot tube cover - Observer – Remove chocks - Scanner – Inspect Survival Gear (Aircraft and individual equip) - Scanner – Place Survival Gear with in arms reach of scanner or of observer if in two person crew - Scanner – Inspect Baggage compartment/Tie down or remove excess items (all lose items under net) - Crew – verify location of ELT Antennas (top or bottom)

Crew – Preflight Walk Around Aircraft Inspection (If deficiency found notify pilot)

- Baggage Door – Secure - Left Fuselage – No wrinkles in skin, Antennas secure - Left Elevator – No significant dents - Rudder – No significant dents – VOR antenna present - Right Elevator – No significant dents - Right Fuselage – No wrinkles in skin - Right Wing – No significant dents - Right Main Landing Gear – No leaks, bald spots - Pitot Tube – No cracks - Nose wheel – No bald spots & two fingers of inflation on landing gear - Windshield – Generally free of bugs and clean - Propeller – No significant burs - Left Wing – No Significant dents - Left Main Landing Gear – No leaks, bald spots

Aircrew Member Checklist – Duties & Responsibilities

Preflight Interior Equipment Inspection

- Crew – Headset plugged in - Crew – Log started – Hobbs recorded - Crew – Sectional open to area flying over - Observer – Read Aircraft Before Start Checklist to Pilot – Challenge Response - Observer – Program Communication radios – a. IAW Mission Pilot’s preference - Crew - Visually clear aircraft of traffic

Observer Run Up Checklist

- Program ELT – tune to 121.5 run ELT checklist - Program GPS – Run GPS checklist - Program NDB – Nearest NDB to search - Program VOR – Two nearest to search - Program CAP radio – Set to freq briefed at Mission base - CAP Radio – Radio check w/mission base - Transponder – Set to 1200 or ATC squak code

Before Take off

- Observer – Read Aircraft Before Take off Checklist to Pilot – Challenge Response - Scanner – Check 360deg of Aircraft - Crew – Announce any traffic

Cruise

- Observer – Contact Mission base with time wheels up – number of souls on board - Crew – Maintain situational awareness with sectional – announce traffic as seen

Aircrew Member Checklist – Duties & Responsibilities

Before Landing Checklist

- Observer – Dial up ATIS/AWOS - Crew – Seatbelts/shoulder harness secure - Crew – Store mission gear - Crew – Scan for announce traffic - Scanner – Maintain visual contact with traffic once announced

After Landing Checklist

- Crew – Visually clear aircraft as taxing – Announce Clear left/right - Observer – Contact mission base and report RTB time - Crew – Exit Aircraft – remove all personal gear - Crew – Record Hobbs time - Crew – Tie down Aircraft - Crew – Seatbelts returned to original configuration - Scanner – Return any non mission related gear to the aircraft - Crew – Close Flight Plan - Crew – Debrief

Refueling Operations

- Crew – Exit Aircraft - Scanner – Attach grounding wire - Scanner – Place ladder (if available) - Observer – Pull fuel hose into place - Observer – Fuel aircraft main tanks

Hand Push Operations

- Pilot – Operate steering bar - Observer – Push from right side of aircraft - Scanner – Push from left side of aircraft

Aircrew Member Checklist – Duties & Responsibilities

Emergency Checklist (Only after Pilot declares emergency) (Must be done within 20 sec) {Need to memorize}

- Crew – Secure seatbelts/shoulder harness - Scanner – Secure and Lap load aircraft survival equipment - Scanner – Scan for place to set aircraft down - Scanner – Scan for visual landmark to assist in position determination - Observer – Set Transponder to 7700 - Observer – Set GPS to current LAT/LONG - Observer – Set Comm 2 to 121.5 - Observer – On order from pilot switch to Comm 2 and broadcast MAYDAY to ATC - Observer – Contact Hi-bird or mission base and report MAYDAY with current LAT/LONG

Emergency Landing Checklist

- Crew – 50 feet before touchdown open door - Scanner – Make one attempt to wedge aircraft book into Observers door - Observer – Continue to broadcast MAYDAY with Current LAT/LONG until aircraft stops moving

Emergency Egress

- Crew – On command of “EGRESS, EGRESS, EGRESS” exit aircraft - Scanner(s) – Exit pilots side (unless in MAULE) - Observer – Exit observer side - Pilot – Exit observer side of aircraft - Scanner – Takes survival equipment with them when they exit - Crew – Link up at rear of aircraft 25meter from tail