MSDS Information

JSC MSDS #33056

=============================================================================

Section 1.                 CHEMICAL PRODUCT SECTION

Product Name:   Zero Charge Static Dissipative Floor Coating

Product Number: 1720 BULK

General Use:

Product Description:

MANUFACTURER: Tech Spray, Inc.        For Chemical Emergency, Spill, Leak, Fire

P.O. Box 949               Exposure, or Accident Call CHEMTREC

Amarillo, TX 79105-0949        DAY OR NIGHT 1-800-424-9300.

PHONE: 806/372-8523

FAX: 806/372-8750

=============================================================================

Section 2.          COMPOSITION / INFORMATION ON INGREDIENTS

CHEMICAL                                          C.A.S. Number    Weight %

Tributoxyethyl Phosphate                             78-51-3         < 1

Diethylene Glycol Monomethyl                        111-77-3         3-4

Ether

Dipropylene Glycol Methyl                         34590-94-8         < 1

Ether

Octylphenoxpolyethoxyethanol                       9036-19-5         << 1

Nonylphenoxypolyethoxyethanol                     68412-54-4         << 1

Zinc Oxide Complex                                 1314-13-2         << 1

Ammonium Hydroxide                                 1336-21-6         .1

Dibutyl Phthalate                                    84-74-2         .89

Acrylic Copolymer                                 25987-66-0         10-15

NJ trd. sec.#00850201001-5391P                                       < 5

Perfluoroalkyl Phosphates                                            << 1

Deionized Water                                    7732-18-5         75-85

Polyethylene Polymer                                                 < 3

OSHA HAZARDOUS COMPONENTS (29 CFR 1910.1200):

Exposure Limits 8 Hours TWA (PPM)

OSHA PEL   ACGIH TLV Supplier

Tributoxyethyl Phosphate                                      NIF

Diethylene Glycol Monomethyl                                  NIF

Dipropylene Glycol Methyl                        100          100

Ether

Ammonium Hydroxide                                             25

Dibutyl Phthalate                               5mg/m3

Perfluoroalkyl Phosphates                        NIF

=============================================================================

Section 3.                    HAZARD IDENTIFICATION

Emergency Overview:

Potential Health Effects:

INHALATION: Inhalation of vapors will not likely cause any effects of

exposure.

EYES: Due to the high pH, this product is a moderate eye irritant.

SKIN: Slight irritation possible.

INGESTION: Nausea and diarrhea are possible.

=============================================================================

Section 4.                    FIRST AID MEASURES

Inhalation:

Move to fresh air in case of accidental inhalation of vapors.  If

victim has stopped breathing, give artificial respiration.  Call  fffffffffffor

prompt medical attention.

Eye Contact:

Flush eyes with large amounts of water for 15 minutes or until

irritation subsides.  If irritation persists, get medical aaaaaaaaaaattention.

Skin Contact:

Remove contaminated clothing (including shoes) and wash before rrrrrrrrrrreuse.

Flush with large amounts of water.  Use soap if available.  If

irritation persists, seek medical attention.

Ingestion:

Do not induce vomiting unless directed by a physician.  If ccccccccccconscious

and alert, give two glasses of water.  Seek medical attention

immediately.

=============================================================================

Section 5.                   FIRE FIGHTING MEASURES

Flash Point & Method:     None TCC Method

Flammable Limits:         LEL: NA     UEL: NA

Autoignition Temperature:

GENERAL HAZARD:

None

FIRE FIGHTING INSTRUCTIONS:

Fire fighters should wear self contained, positive-pressure breathing

apparatus and avoid skin contact.

FIRE FIGHTING EQUIPMENT:

Water, foam, dry chemical, carbon dioxide.

HAZARDOUS COMBUSTION PRODUCTS:

Smoke, fumes and oxides of carbon.

=============================================================================

Section 6.                ACCIDENTAL RELEASE MEASURES

LAND SPILL:

Pick up material and place in sealed container for disposal.

WATER SPILL:

=============================================================================

Section 7.                   HANDLING AND STORAGE

STORAGE TEMPERATURE: Ambient

STORAGE PRESSURE:    Atmospheric

GENERAL:

Keep container closed when not in use.  Store in cool, well vvvvvvvvvventilated

place out of direct sunlight and away from incompatible materials.

(See STABILITY AND REACTIVITY Section 10.)  Follow all MSD Sheet and

Label warnings even after container is emptied.

=============================================================================

Section 8.          EXPOSURE CONTROL / PERSONAL PROTECTION

Engineering Controls:

(X)    Local Exhaust ventilation acceptable.

( )    Mechanical ventilation recommended.

( )    Use explosion-proof ventilation equipment.

( )    Do not use in confined spaces without mechanical ventilation

equipment.

See section 2 for component exposure guidelines.

Personal Protection:

RESPIRATOR:

If concentrations are over the exposure limit and are known, air

purifying respirator with Organic Vapor Cartridges may be aaaaaaaaaacceptable.

Refer to cartridges for acceptable levels.  If concentrations are

over exposure limit and are unknown, use a supplied air respirator.

HAND PROTECTION:

(X)    Gloves recommended

(X) Solvex            (X) Neoprene

(X) Butyl             (X) Buna

(X) Natural Latex     ( ) Cotton/Jersey

EYE PROTECTION:

(X) Safety Glasses    ( ) Chemical Goggles    ( ) Full Face Shield

OTHER RECOMMENDATIONS:

( ) Rubber Boots      ( ) Splash-proof chemical resistant suit/apron

=============================================================================

Section 9.              PHYSICAL AND CHEMICAL PROPERTIES

Density.................  1.004         pH......................... 8.6

Boiling Point...........  100C / 212F   % Volatile................. 80

Freezing Point.......... NIF            % Solids................... 20

Vapor Density (Air=1):..  NIF           Evaporation Rate (H2O=1)... NIF

Solubility in Water.....  50            Viscosity.................. N/A

Molecular Weight........ N/A            Physical State............ LIQUID

Non-Exempt VOC (g/l).... 54.1           Odor....................... NIF

Appearance: Opaque white emulsion with ammonical odor.

=============================================================================

Section 10.                STABILITY AND REACTIVITY

GENERAL:

STABLE

INCOMPATIBLE MATERIALS AND CONDITIONS TO AVOID:

None

Reactive alkali metals, strong acids & bases.

HAZARDOUS DECOMPOSITION:

Forced combustion yields carbon oxides.

=============================================================================

Section 11.                 TOXICOLOGICAL INFORMATION

RESULTS OF COMPONENT TOXICITY TEST PERFORMED:

Information not available.

HUMAN EXPERIENCE:

Information not available.

=============================================================================

Section 12.                  ECOLOGICAL INFORMATION

FURTHER INFORMATION:

Information not available.

=============================================================================

Section 13.                DISPOSAL CONSIDERATIONS

RCRA 40 CFR 261 Classification:

Federal, State, and Local laws governing disposal of materials can sssssdiffer.

Ensure proper disposal compliance with proper authorities before sssssdisposal.

=============================================================================

Section 14.               TRANSPORTATION INFORMATION

U.S. DOT Information

Basic Description: NON HAZARDOUS MATERIAL

Proper Shipping Name: N/A

Hazard Class: N/A

Packaging Group: N/A

UN Number: N/A

Limitations: 1720-K ships as CONSUMER COMMODITY  ORM-D

IATA

Proper Shipping Name: NON HAZARDOUS MATERIAL

Hazard Class: N/A

Packing Group: N/A

UN Number: N/A

Limitations: 1720-K ships as POTASSIUM HYDROXIDE SOLUTION, 8,

UN1814, II. FOR DOMESTIC SHIPMENTS ONLY.  WHEN

SHIPPING INTERNATIONAL CONTACT THE TECH SPRAY

SHIPPING DEPT.

IMO

Proper Shipping Name: NON HAZARDOUS MATERIAL

Class: N/A

UN Number: N/A

Packaging Group: N/A

EMS: N/A

MFAG: N/A

Marine Pollutant: N/A

Canadian TDG: N/A

Limitations: 1720-K ships as DANGEROUS GOODS IN LIMITED QUANTITIES

OF CLASS 8 (POTASSIUM HYDROXIDE), 8, UN1814, II,

EMS: 8-06, MFAG: 705, IMDG: 148

=============================================================================

Section 15.               REGULATORY INFORMATION

UNITED STATES FEDERAL REGULATIONS:

MSDS complies with OSHAs Hazard Communication Rule, 29 CFR 1910.1200.

CERCLA/SUPERFUND, 40 CFR 117, 302:

--- None of the chemicals are Superfund hazards ---

SARA SUPERFUND AND REAUTHORIZATION ACT OF 1986

TITLE III Sections 302, 311, 312 and 313:

Section 302 - Extremely hazardous substances (40 CFR 355):

--- None of the chemicals are Section 302 hazards ---

Section 311/312-Material Safety Data Sheet Requirements (40 CFR 370)

( ) By our hazard evaluation, this product is non-hazardous.

(X) By our hazard evaluation, this product is hazardous.  It

should be reported under the following EPA hazard.

(X) Immediate (acute) health hazard

( ) Delayed (chronic) chronic health hazard

( ) Sudden release of pressure hazard

( ) Reactive hazard

( ) FLAMMABLE

Section 313 - List of Toxic Chemicals (40 CFC 372)

This product contains the following chemicals (at levels of 1% oooooooooooooor greater) which are found on the 313 list of Toxic Chemicals.

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

--- None of the chemicals are 313 Toxic Chemicals ---

TOXIC SUBSTANCE CONTROL ACT (TSCA): All substances are TSCA Listed.

RESOURCE CONSERVATION AND RECOVERY ACT (RCRA 40 CFR 261) Subpart C & D:

Refer to Section 11. for RCRA classification.

FEDERAL WATER POLLUTION CONTROL ACT, CLEAN WATER ACT, 40 CFR 401.15

(FORMERLY SECTION 307), 40 CFR 116 (FORMERLY SECTION 311)

This product contains the following chemicals which are listed:

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

CLEAN AIR ACT:  --- No Information ---

STATE REGULATIONS:

CALIFORNIA PROPOSITION 65:

This product contains the following ingredients which appear on the

California proposition 65 list:

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

--- None of the chemicals are on the Proposition 65 list ---

INTERNATIONAL REGULATIONS:

CANADA WHMIS: NIF

EUROPE EINECS NUMBERS: NIF

=============================================================================

Section 16.                   OTHER INFORMATION

LABEL INFORMATION:

European risk and Safety Phrases: S2, S7, S24/25

European Symbols Needed: NIF

Canadian WHMIS Symbols: NIF

NFPA HAZARD RATING:

(0) Fire   (1) Health   (0) Reactivity

REVISION DATES, SECTIONS, REVISED BY:

27-JULY-94, CONVERTED TO ANSI STANDARD, B. RIFFEL

26-APR-95, Updated Section 16, B. Riffel

09-MAY-95, Corrected CAS Number Section 2, B. Riffel

18-AUG-97, Reviewed, B. Riffel

ABBREVIATIONS USED IN THIS DOCUMENT:

NE - Not Established, NA - Not Applicable, NIF - No Information Found

REFERENCES:

Code of Federal Regulations (CFR)

The Sigma-Aldrich Library of Regulatory and Safety Data

Chemical Guide and OSHA Hazard Communication Standard

Various Federal, State & Local Regulations

To the best of our knowledge, the information contained herein is accurate.

However, neither Tech Spray, Inc. or any of its subsidiaries assumes any

liability whatsoever for the accuracy or completeness of the information

contained herein.  Final determination of suitability of any material is the

sole responsibility of the user.  All materials may present unknown hazards and should be used with caution.  Although certain hazards are described herein,we cannot guarantee that these are the only hazards which exist.

MSDS Information

JSC MSDS #33057

=============================================================================

Section 1.                 CHEMICAL PRODUCT SECTION

Product Name:   Zero Charge Anti-Static Floor Stripper

Product Number: 1721 BULK

General Use:

Product Description:

MANUFACTURER: Tech Spray, Inc.        For Chemical Emergency, Spill, Leak, Fire

P.O. Box 949               Exposure, or Accident Call CHEMTREC

Amarillo, TX 79105-0949        DAY OR NIGHT 1-800-424-9300.

PHONE: 806/372-8523

FAX: 806/372-8750

=============================================================================

Section 2.          COMPOSITION / INFORMATION ON INGREDIENTS

CHEMICAL                                          C.A.S. Number    Weight %

Potassium Hydroxide                                1310-58-3         4-4.5

Water                                                                > 70%

Diethylene Glycol Monomethyl

ether                             111-77-3         < 6

Sodium Metasilicate                               68349-20-0         < 5

Tetrasodium Ethylendiamine-

Tetraacetate                                        < 1

Diethanol amide of coconutte

fatty acid                                               < 3

Phosphated Alkyl Ethoxilate

Potassium Salt                       68071-17-0         2-6

OSHA HAZARDOUS COMPONENTS (29 CFR 1910.1200):

Exposure Limits 8 Hours TWA (PPM)

OSHA PEL   ACGIH TLV   Supplier

Potassium Hydroxide                                         2mg/m3

ether                                           NIF

Sodium Metasilicate                                           NIF

Tetraacetate                                      NIF

Potassium Salt				           NIF

=============================================================================

Section 3.                    HAZARD IDENTIFICATION

Emergency Overview:

Potential Health Effects:

INHALATION: Inhalation of mist can cause irritation and damage to the

upper respiratory tract and lungs.  Effects will vary,

depending on the severity of the exposure, from mild irritation

of nasal membranes to severe pneumonitis.

EYES: Contact with eyes rapidly causes burning. Damage to eye possible if

material is not quickly removed.

SKIN: Severe irritant. Burning and redness caused by contact with liquid.

Prolonged or repeated exposure can cause scar tissue to form. Small

amounts of liquid on skin may not cause irritation for several

minutes; however, the material is still causing damage to tissues

which will be felt later. Keep skin clean from all contact and

frequently wash hands.

INGESTION: Ingestion results in severe damage to mucous membranes and to

underlying tissues with which contact is made. Perforation of

these tissues may follow; severe and extensive scar formation

may occur. Death may occur if material penetrates to vital

areas.

=============================================================================

Section 4.                    FIRST AID MEASURES

Inhalation:

Move to fresh air in case of accidental inhalation of vapors.  If

victim has stopped breathing, give artificial respiration.  Call fffffffffffor

prompt medical attention.

Eye Contact:

Flush eyes with large amounts of water for 15 minutes or until

irritation subsides.  If irritation persists, get medical attention.

Skin Contact:

Remove contaminated clothing (including shoes) and wash before rrrrrrrrrrreuse.

Flush with large amounts of water.  Use soap if available.  If

irritation persists, seek medical attention.

Ingestion:

Do not induce vomiting unless directed by a physician.  If conscious

and alert, give two glasses of water.  Seek medical attention

immediately.

=============================================================================

Section 5.                   FIRE FIGHTING MEASURES

Flash Point & Method:     None, TCC Method

Flammable Limits:         LEL: NA     UEL: NA

Autoignition Temperature:

GENERAL HAZARD:

None

FIRE FIGHTING INSTRUCTIONS:

Fire fighters should wear self contained, positive-pressure breathing

apparatus and avoid skin contact.

FIRE FIGHTING EQUIPMENT:

Water, foam, dry chemical, carbon dioxide.

HAZARDOUS COMBUSTION PRODUCTS:

Smoke, fumes and oxides of carbon.

=============================================================================

Section 6.                ACCIDENTAL RELEASE MEASURES

LAND SPILL:

Evacuate area. Personnel should wear full protective clothing including

rubber boots and protective gloves. Dike up area and prevent runoff to

sewer or surface waters. Pump or absorb liquid with suitable absorbent and

place in sealed containers for disposal. Remove all possible sources of

ignition from area.

WATER SPILL:

=============================================================================

Section 7.                   HANDLING AND STORAGE

STORAGE TEMPERATURE: Ambient

STORAGE PRESSURE:    Atmospheric

GENERAL:

Keep container closed when not in use.  Store in cool, well ventilated

place out of direct sunlight and away from incompatible materials.

(See STABILITY AND REACTIVITY Section 10.)  Follow all MSD Sheet and

Label warnings even after container is emptied.

=============================================================================

Section 8.          EXPOSURE CONTROL / PERSONAL PROTECTION

Engineering Controls:

(X)    Local Exhaust ventilation acceptable.

( )    Mechanical ventilation recommended.

( )    Use explosion-proof ventilation equipment.

( )    Do not use in confined spaces without mechanical ventilation

equipment.

See section 2 for component exposure guidelines.

Personal Protection:

RESPIRATOR:

If concentrations are over the exposure limit and are known, air

purifying respirator with Organic Vapor Cartridges may be acceptable.

Refer to cartridges for acceptable levels.  If concentrations are

over exposure limit and are unknown, use a supplied air respirator.

HAND PROTECTION:

(X)    Gloves recommended

(X) Solvex            (X) Neoprene

(X) Butyl             (X) Buna

(X) Natural Latex     ( ) Cotton/Jersey

EYE PROTECTION:

( ) Safety Glasses    (X) Chemical Goggles    ( ) Full Face Shield

OTHER RECOMMENDATIONS:

(X) Rubber Boots      ( ) Splash-proof chemical resistant suit/apron

=============================================================================

Section 9.              PHYSICAL AND CHEMICAL PROPERTIES

Density.................  1.050         pH......................... 12

Boiling Point...........  100C / 212F   % Volatile................. >80

Freezing Point.......... NIF            % Solids................... <20

Vapor Density (Air=1):..  NE            Evaporation Rate (H2O=1)... NIF

Solubility in Water.....  100           Viscosity.................. N/A

Molecular Weight........ N/A            Physical State............. LIQUID

Non-Exempt VOC (g/l).... 59.8           Odor....................... NIF

Appearance: Fluoroscene colored, mobile liquid with low odor.

=============================================================================

Section 10.                STABILITY AND REACTIVITY

GENERAL:

STABLE

INCOMPATIBLE MATERIALS AND CONDITIONS TO AVOID:

Contact with open flame, heat.

Material will react with acids producing heat.

HAZARDOUS DECOMPOSITION:

Combustion yields carbon oxides.

=============================================================================

Section 11.                 TOXICOLOGICAL INFORMATION

RESULTS OF COMPONENT TOXICITY TEST PERFORMED:

Glycol Ether: LD50, oral (Guinea Pigs) 4160 mg/kg

Potassium Hydroxide, LD 50, oral (rats) 365 mg/Kg

HUMAN EXPERIENCE:

Material is corrosive. Contact with skin or eyes can cause iiiiiiiiiirritation

and burns. Glycol Ethers may be absorbed through skin.

=============================================================================

Section 12.                  ECOLOGICAL INFORMATION

FURTHER INFORMATION:

Caustic materials can have adverse effects on ecological systems in high

concentrations.

=============================================================================

Section 13.                DISPOSAL CONSIDERATIONS

RCRA 40 CFR 261 Classification:

Federal, State, and Local laws governing disposal of materials can dDDDDdiffer.

Ensure proper disposal compliance with proper authorities before ddddddisposal.

=============================================================================

Section 14.               TRANSPORTATION INFORMATION

U.S. DOT Information

Proper Shipping Name: POTASSIUM HYDROXIDE, SOLUTION

Hazard Class: 8

Packaging Group: II

UN Number: UN1814

Limitations: N/A

IATA

Proper Shipping Name: POTASSIUM HYDROXIDE, SOLUTION

Hazard Class: 8

Packing Group: II

UN Number: UN1814

Limitations: CARGO AIRCRAFT ONLY     OVERPACK MUST BE USED.

Domestic shipments only. When shipping International

please contact Tech Spray shipping department.

IMO

Proper Shipping Name: POTASSIUM HYDROXIDE, SOLUTION

Class: 8

UN Number: UN1814

Packaging Group: II

EMS: 8-06

MFAG: 705

Marine Pollutant: N/A

Canadian TDG: N/A

IMDG Page: 8214

=============================================================================

Section 15.               REGULATORY INFORMATION

UNITED STATES FEDERAL REGULATIONS:

MSDS complies with OSHAs Hazard Communication Rule, 29 CFR 1910.1200.

CERCLA/SUPERFUND, 40 CFR 117, 302:

--- None of the chemicals are Superfund hazards ---

SARA SUPERFUND AND REAUTHORIZATION ACT OF 1986

TITLE III Sections 302, 311, 312 and 313:

Section 302 - Extremely hazardous substances (40 CFR 355):

--- None of the chemicals are Section 302 hazards ---

Section 311/312-Material Safety Data Sheet Requirements (40 CFR 370)

( ) By our hazard evaluation, this product is non-hazardous.

(X) By our hazard evaluation, this product is hazardous.  It

should be reported under the following EPA hazard.

(X) Immediate (acute) health hazard

( ) Delayed (chronic) chronic health hazard

( ) Sudden release of pressure hazard

( ) Reactive hazard

Section 313 - List of Toxic Chemicals (40 CFC 372)

This product contains the following chemicals (at levels of 1%   OOOOOOOOOOOOOor greater) which are found on the 313 list of Toxic Chemicals.

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

--- None of the chemicals are 313 Toxic Chemicals ---

TOXIC SUBSTANCE CONTROL ACT (TSCA): All substances are TSCA Listed.

RESOURCE CONSERVATION AND RECOVERY ACT (RCRA 40 CFR 261) Subpart C & D:

Refer to Section 11. for RCRA classification.

FEDERAL WATER POLLUTION CONTROL ACT, CLEAN WATER ACT, 40 CFR 401.15

(FORMERLY SECTION 307), 40 CFR 116 (FORMERLY SECTION 311)

This product contains the following chemicals which are listed:

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

CLEAN AIR ACT:  --- No Information ---

STATE REGULATIONS:

CALIFORNIA PROPOSITION 65:

This product contains the following ingredients which appear on the

California proposition 65 list:

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

--- None of the chemicals are on the Proposition 65 list ---

INTERNATIONAL REGULATIONS:

CANADA WHMIS: NIF

EUROPE EINECS NUMBERS: NIF

=============================================================================

Section 16.                   OTHER INFORMATION

LABEL INFORMATION:

European risk and Safety Phrases: S2, S26, S27, S37/39, R35

European Symbols Needed: CORROSIVE

Canadian WHMIS Symbols: Corrosive

NFPA HAZARD RATING:

(0) Fire   (3) Health   (1) Reactivity

REVISION DATES, SECTIONS, REVISED BY:

28-MAY-96, Updated Section 8, B. Riffel

14-NOV-96, Updated Section 12 and 13, B. Riffel

18-AUG-97, Updated Section 16, B. Riffel

ABBREVIATIONS USED IN THIS DOCUMENT:

NE - Not Established, NA - Not Applicable, NIF - No Information Found

REFERENCES:

Code of Federal Regulations (CFR)

The Sigma-Aldrich Library of Regulatory and Safety Data

Chemical Guide and OSHA Hazard Communication Standard

Various Federal, State & Local Regulations

To the best of our knowledge, the information contained herein is accurate.

However, neither Tech Spray, Inc. or any of its subsidiaries assumes any

liability whatsoever for the accuracy or completeness of the information

contained herein.  Final determination of suitability of any material is the

sole responsibility of the user.  All materials may present unknown hazards and

should be used with caution.  Although certain hazards are described herein,

we cannot guarantee that these are the only hazards which exist.

MSDS Information

JSC MSDS #33058

=============================================================================

Section 1.                 CHEMICAL PRODUCT SECTION

Product Name:   Zero Charge Anti-Static Cleaner/Neutralizer

Product Number: 1722 BULK

General Use:

Product Description:

MANUFACTURER: Tech Spray, Inc.        For Chemical Emergency, Spill, Leak, Fire

P.O. Box 949               Exposure, or Accident Call CHEMTREC

Amarillo, TX 79105-0949        DAY OR NIGHT 1-800-424-9300.

PHONE: 806/372-8523

FAX: 806/372-8750

=============================================================================

Section 2.          COMPOSITION / INFORMATION ON INGREDIENTS

CHEMICAL                                          C.A.S. Number    Weight %

Deionized Water                                     7732-18-5        70-80

Isopropyl Alcohol                                    67-63-0         1-5

Tetrasodium Ethylenediamine-

tetraacetate                                         64-02-8 < 1

Sodium Hydroxide                                   1310-73-2         <2

poly(oxy-1,2-ehtanediyl)                           9016-45-9         < 5

Dodecylbenzene Sulfonic Acid                      27176-87-0         10-15

Quaternary Ammonium Compounds                     68071-95-4         < 5

Diethanol amide of coconut

Fatty acid                                111-42-2    < 5

OSHA HAZARDOUS COMPONENTS (29 CFR 1910.1200):

Exposure Limits 8 Hours TWA (PPM)

OSHA PEL   ACGIH TLV   Supplier

Isopropyl Alcohol                                400         400

tetraacetate                     NIF

Sodium Hydroxide                                2mg/m3

Dodecylbenzene Sulfonic Acid                     NIF         NIF

Quaternary Ammonium Compounds                    NIF         NIF

Fatty acid

=============================================================================

Section 3.                    HAZARD IDENTIFICATION

Emergency Overview:

Potential Health Effects:

INHALATION: No observable effects of overexposure.

EYES: Mild irritation.

SKIN: Very mild, if any, irritation.

INGESTION: Nausea and diarrhea are possible.

=============================================================================

Section 4.                    FIRST AID MEASURES

Inhalation:

Move to fresh air in case of accidental inhalation of vapors.  If

victim has stopped breathing, give artificial respiration.  Call f          o	  for prompt medical attention.

Eye Contact:

Flush eyes with large amounts of water for 15 minutes or until

irritation subsides.  If irritation persists, get medical a	  t	   attention.

Skin Contact:

Remove contaminated clothing (including shoes) and wash before r             e	  reuse.

Flush with large amounts of water.  Use soap if available.  If

irritation persists, seek medical attention.

Ingestion:

Do not induce vomiting unless directed by a physician.  If c o	  n	   conscious

and alert, give two glasses of water.  Seek medical attention

immediately.

=============================================================================

Section 5.                   FIRE FIGHTING MEASURES

Flash Point & Method:     None, T.C.C Meth

Flammable Limits:         LEL: NA     UEL: NA

Autoignition Temperature:

GENERAL HAZARD:

None

FIRE FIGHTING INSTRUCTIONS:

Fire fighters should wear self contained positive-pressure breathing

apparatus.

FIRE FIGHTING EQUIPMENT:

Water, foam, dry chemical, carbon dioxide.

HAZARDOUS COMBUSTION PRODUCTS:

Smoke, fumes and oxides of carbon.

=============================================================================

Section 6.                ACCIDENTAL RELEASE MEASURES

LAND SPILL:

Pick up and place in appropriate container.

WATER SPILL:

=============================================================================

Section 7.                   HANDLING AND STORAGE

STORAGE TEMPERATURE: Ambient

STORAGE PRESSURE:    Atmospheric

GENERAL:

Keep container closed when not in use.  Store in cool, well ventilated

place out of direct sunlight and away from incompatible materials.

(See STABILITY AND REACTIVITY Section 10.)  Follow all MSD Sheet and

Label warnings even after container is emptied.

=============================================================================

Section 8.          EXPOSURE CONTROL / PERSONAL PROTECTION

Engineering Controls:

(X)    Local Exhaust ventilation acceptable.

( )    Mechanical ventilation recommended.

( )    Use explosion-proof ventilation equipment.

( )    Do not use in confined spaces without mechanical ventilation

equipment.

See section 2 for component exposure guidelines.

Personal Protection:

RESPIRATOR:

If concentrations are over the exposure limit and are known, air

purifying respirator with Organic Vapor Cartridges may be  a     ccept    acceptable.

Refer to cartridges for acceptable levels.  If concentrations are

over exposure limit and are unknown, use a supplied air respirator.

HAND PROTECTION:

(X)    Gloves recommended

(X) Solvex            (X) Neoprene

(X) Butyl             (X) Buna

(X) Natural Latex     ( ) Cotton/Jersey

EYE PROTECTION:

(X) Safety Glasses    ( ) Chemical Goggles    ( ) Full Face Shield

OTHER RECOMMENDATIONS:

( ) Rubber Boots      ( ) Splash-proof chemical resistant suit/apron

=============================================================================

Section 9.              PHYSICAL AND CHEMICAL PROPERTIES

Density.................  1.030         pH......................... 7.2

Boiling Point...........  100C / 212F   % Volatile................. >70

Freezing Point.......... NIF            % Solids................... <30

Vapor Density (Air=1):..  NE            Evaporation Rate (H2O=1)... NE

Solubility in Water.....  100           Viscosity.................. N/A

Molecular Weight........ N/A            Physical State............. LIQUID

Non-Exempt VOC (g/l).... 14.48          Odor....................... NIF

Appearance: Clear blue liquid with pleasant odor.

=============================================================================

Section 10.                STABILITY AND REACTIVITY

GENERAL:

STABLE

INCOMPATIBLE MATERIALS AND CONDITIONS TO AVOID:

None

None

HAZARDOUS DECOMPOSITION:

Forced combustion yields carbon and silicone oxides.

=============================================================================

Section 11.                 TOXICOLOGICAL INFORMATION

RESULTS OF COMPONENT TOXICITY TEST PERFORMED:

Information not available.

HUMAN EXPERIENCE:

Information not available.

=============================================================================

Section 12.                  ECOLOGICAL INFORMATION

FURTHER INFORMATION:

Information not available.

=============================================================================

Section 13.                DISPOSAL CONSIDERATIONS

RCRA 40 CFR 261 Classification:  Non-hazardous

Federal, State, and Local laws governing disposal of materials can  skdkddiffer.

Ensure proper disposal compliance with proper authorities before sdddddisposal.

=============================================================================

Section 14.               TRANSPORTATION INFORMATION

U.S. DOT Information

Basic Description: NON HAZARDOUS MATERIAL

Proper Shipping Name: N/A

Hazard Class: N/A

Packaging Group: N/A

UN Number: N/A

Limitations: N/A

IATA

Proper Shipping Name: NON HAZARDOUS MATERIAL

Hazard Class: N/A

Packing Group: N/A

UN Number: N/A

Limitations: N/A

IMO

Proper Shipping Name: NON HAZARDOUS MATERIAL

Class: N/A

UN Number: N/A

Packaging Group: N/A

EMS: N/A

MFAG: N/A

Marine Pollutant: N/A

Canadian TDG: N/A

=============================================================================

Section 15.               REGULATORY INFORMATION

UNITED STATES FEDERAL REGULATIONS:

MSDS complies with OSHAs Hazard Communication Rule, 29 CFR 1910.1200.

CERCLA/SUPERFUND, 40 CFR 117, 302:

--- None of the chemicals are Superfund hazards ---

SARA SUPERFUND AND REAUTHORIZATION ACT OF 1986

TITLE III Sections 302, 311, 312 and 313:

Section 302-Extremely hazardous substances (40 CFR 355):

--- None of the chemicals are Section 302 hazards ---

Section 311/312-Material Safety Data Sheet Requirements (40 CFR 370)

( ) By our hazard evaluation, this product is non-hazardous.

(X) By our hazard evaluation, this product is hazardous.  It

should be reported under the following EPA hazard.

(X) Immediate (acute) health hazard

( ) Delayed (chronic) chronic health hazard

( ) Sudden release of pressure hazard

( ) Reactive hazard

Section 313 - List of Toxic Chemicals (40 CFC 372)

This product contains the following chemicals (at levels of 1% or           or greater) which are found on the 313 list of Toxic Chemicals.

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

--- None of the chemicals are 313 Toxic Chemicals ---

TOXIC SUBSTANCE CONTROL ACT (TSCA): All substances are TSCA Listed.

RESOURCE CONSERVATION AND RECOVERY ACT (RCRA 40 CFR 261) Subpart C & D:

Refer to Section 11. for RCRA classification.

FEDERAL WATER POLLUTION CONTROL ACT, CLEAN WATER ACT, 40 CFR 401.15

(FORMERLY SECTION 307), 40 CFR 116 (FORMERLY SECTION 311)

This product contains the following chemicals which are listed:

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

CLEAN AIR ACT:  --- No Information ---

STATE REGULATIONS:

CALIFORNIA PROPOSITION 65:

This product contains the following ingredients which appear on the

California proposition 65 list:

CHEMICAL                             C.A.S. NUMBER    WEIGHT %

--- None of the chemicals are on the Proposition 65 list ---

INTERNATIONAL REGULATIONS:

CANADA WHIMS: NIF

EUROPE EINECS NUMBERS: NIF

=============================================================================

Section 16.                   OTHER INFORMATION

LABEL INFORMATION:

European risk and Safety Phrases: S2

European Symbols Needed: NIF

Canadian WHIMS Symbols: NIF

NFPA HAZARD RATING:

(0) Fire   (1) Health   (0) Reactivity

REVISION DATES, SECTIONS, REVISED BY:

27-JLY-94, CONVERTED TO ANSI STANDARD, B. RIFFEL

28-May-96, Updated Section 8, B. Riffel

18-AUG-97, Updated Section 13, B. Riffel

28-SEP-99, Reviewed MSDS, B. Riffel

ABBREVIATIONS USED IN THIS DOCUMENT:

NE - Not Established, NA - Not Applicable, NIF - No Information Found

REFERENCES:

Code of Federal Regulations (CFR)

The Sigma-Aldrich Library of Regulatory and Safety Data

Chemical Guide and OSHA Hazard Communication Standard

Various Federal, State & Local Regulations

To the best of our knowledge, the information contained herein is accurate.

However, neither Tech Spray, Inc. or any of its subsidiaries assumes any

liability whatsoever for the accuracy or completeness of the information

contained herein.  Final determination of suitability of any material is the

sole responsibility of the user.  All materials may present unknown hazards and should be used with caution.  Although certain hazards are described herein,we cannot guarantee that these are the only hazards which exist.