MSFC Problem Reporting and Corrective Action (PRACA) System

WHOLE RECORD REPORT( + ADDENDUM)

| MSFC Record # A08085 | In-Flight Anomaly Number -- | Contractor Report Number E-008 | JSC# -- | KSC# -- |  |
| --- | --- | --- | --- | --- | --- |
| Problem Title MULTIPLEXER VIBRATION FAILURE |  |  |  |  |  |
| EICN# -- | ELEMENT ET | Contractor MMMSS | FSCM# -- | FCRIT 3 |  |
| HCRIT -- | Sys_Lvl N | Misc Codes A B C D E F G H I J K L M N O |  |  |  |
| HARDWARE EIM | NOMENCLATURE N/A | PART# N/A | SER/LOT# N/A | MANUFACTURER N/A |  |
| HARDWARE LRU | NOMENCLATURE N/A | PART# N/A | SER/LOT# N/A | MANUFACTURER N/A |  |
| HARDWARE NCA | NOMENCLATURE PCM MULTIPLEXER | PART# PD6400478 | SER/LOT# 0000001 | MANUFACTURER GULTON |  |
| Test/Operation Q - QAL | Prevailing Condtion -- | F / U F | Fail Mode -- | Cause -- |  |
| System ELECTRICAL | Defect -- | Material -- | Work Contact R. JACKSON | Fail Date 09/28/1978 |  |
| Received at MSFC 07/01/1979 | Date Isolated -- | FMEA Reference 3.3.2 | IFA: Mission Phase -- | Mission Elapsed Time -- |  |
| Location GULTON |  | Symptom ER - ERRONEOUS |  | Time Cycle -- |  |
| Effectivity Text NONE |  |  |  |  |  |
| Vehicle Effectivity Codes |  |  |  |  |  |
| Vehicle 1 -- | Vehicle 2 -- | Vehicle 3 -- | Vehicle 4 -- | Vehicle 5 -- |  |
| Mission Effectivity Codes |  |  |  |  |  |
| Mssn 1 -- | Mssn 2 -- | Mssn 3 -- | Mssn 4 -- | Mssn 5 -- |  |
| Estimated Completion Dates |  |  |  |  |  |
| MSFC Approved Defer Until Date -- | Contractor Req Defer Until Date -- | LVL 3 Close -- | Remark / Action -- |  |  |
| Investigation / Resolution Summary |  |  |  |  |  |
| Last MSFC Update 10/07/1987 | CN RSLV SBMT 07/01/1979 | Defer Date -- | Add Date -- | R/C Codes 0 - EXPL -- -- |  |
| Assignee |  |  |  |  |  |
| Design G. PLATT | Chief Engineer -- | S & MA D. NEWMAN | Project R. ABRAHAM | Project MGR -- |  |
| Approval |  |  |  |  |  |
| Design N/A | Chief Engineer -- | S & MA N/A | Project N/A | Project MGR -- |  |
| PAC Assignee M. GLASS | PAC Review Complete MG | MSFC Closure Date 07/01/1979 | Status C - CLOSED | F/A Completion -- |  |
| Problem Type -- | SEV -- | Program Name -- | REVL -- | OPRINC -- |  |
| FUNC MOD -- | Software Effectivity -- -- -- -- -- | Software Fail CD -- |  | SUBTYPE -- | Software Closure CD -- |
| RES PERSON L2 -- | Approval Signature L3 -- |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Contractor Status Summary |  |  |  |  |  |
| Reliability/Quality Assurance Concerns, Recommendations: |  |  |  |  |  |
| Problem Description 1. S/N 1 EXPERIENCED MULTIPLE VIB. FAILURES OVER EXTENDED TEST PERIOD A. A HARD FAILURE WAS ISOLATED TO 1 PART AT THE BOARD LEVEL; HOWEVER, FAILURE ANALY OF REMOVED PARTS FAILED TO VERIFY FAILURES. B. UNIT INSTALLED ON MPTA AT NSTL FOR TANKING USE ONLY, FAILURES ACCEPTED AS UNVERIFIED. 2. S/N 1 FAILED IN 40% TANKING AT NSTL BY A LOSS OF ALL ANALOG DATA AT -15 DEG F. 3. S/N 2 - 7 FAILURES IN PRE-ATP, 2 IN ATP & 2 IN POST-ATP. FAILURES WERE ACCEPTED AS UNVERIFIED |  |  |  |  |  |
| Contractor Investigation/Resolution 1. REFER TO ATTACHED MEMO 3532-78-204 FOR THE SUMMARY OF ACTIONS TAKEN TO IMPROVE THE MULTIPLEXER RELIABILITY. SIGNIFICANT ACTIONS ARE: A UPGRADING OF SOLDERING. B. MECHANICAL DESIGN REVIEW, VERIFICATION, & IMPROVEMENTS. C. CRIMP PROCESS IMPROVEMENTS. D. PIND TESTING OF COMPONENTS. 2. THE FAILURE OF S/N 0000001 ON MPTA WAS UNVERIFIED EXTENSIVE FAILURE ANALYSIS & ENGR'G ASSESS- MENT HAVE ESTABLISHED THE MOST PROBABLE CAUSE TO BE A DEFECTIVE SOLDER JOINT. 3. S/N 0000001 WAS PREVIOUSLY DISPOSITIONED FOR MPTA USE ONLY BY MARS T00735 & IS CURRENTLY LOCATED AT NSTL FOR USE AS A SPARE. THE TROUBLESHOOTING & RETEST FOR THE 40% TANKING FAILURE IS DOCUMENTED ON MARS T13228. 4. S/N 0000002 COMPLETED ATP & WITH UNVERIFIED FAILURES DISPOSITIONED USE-AS-IS BY THE MRB. S/N 0000002 IS CURRENTLY INSTALLED ON MPTA |  |  |  |  |  |
| MSFC Response/Concurrence |  |  |  |  |  |

MSFC Problem Reporting and Corrective Action (PRACA) System

ASSESSMENT ADDENDUM REPORT

| MSFC Report# A08085 | IFA# -- | Contractor RPT# E-008 | JSC# -- | KSC# -- | EICN# -- |
| --- | --- | --- | --- | --- | --- |
| Asmnt Part# -- | Asmnt Part Name -- | Asmnt Serial/Lot# -- |  |  |  |
| HCRIT CD -- | FCRIT CD -- | CAUSE CD -- | FAIL MODE ER - ERRONEOUS |  |  |
| Asmnt FMEA -- | Asmnt FM -- | FMEA CSE -- | FMEA SCSE -- |  |  |
| Asmnt FMEA -- | Asmnt FM -- | FMEA CSE -- | FMEA SCSE -- |  |  |
| Asmnt FMEA -- | Asmnt FM -- | FMEA CSE -- | FMEA SCSE -- |  |  |
| Correlated Part# -- | Correlated Part# -- | Correlated Part# -- |  |  |  |
| Associated LRU# -- | Associated LRU# -- | Associated LRU# -- |  |  |  |
| MAJOR DESIGN CHANGES |  |  |  |  |  |
| APRV DATE -- | DESCRIPTION OF CHANGES -- |  |  |  |  |
| ASSESSMENT TEXT |  |  |  |  |  |