- 8. Scientific Payload

The strawman payload discussed in this chapter contains sufficient detail for the system design but may not reflect the finally selected suite of instruments.

- 8.1  Measurement requirements

The Helios, Ulysses, Yohkoh, SOHO and TRACE missions provide the expertise and experience for designing the payload of the Solar Orbiter. It must be state-of-the-art, withstand the considerable thermal load at 45 Rs, comply with the general requirements of a low-mass, compact and integrated design, make use of on-board data compression/storage and require a modest data transmission rate. The payload meeting the solar and heliospheric science objectives encompasses two instrument packages:

- In-situ  heliospheric instruments (see Table 8.1 below): Solar wind plasma analyser measuring electrons and ions, magnetometer for DC and AC fields, particle detector for solar energetic ions and electrons, interplanetary dust detector, neutral particle detector, radio instruments. Heritage for such instruments is from Helios, SOHO, Ulysses, Wind,  ACE.

- Remote-sensing solar instruments (see Table 8.4 below): EUV/X-ray full-Sun and high resolution imager; high-resolution EUV spectrometer covering selected emission lines from the chromosphere to corona; high-resolution visible light telescope and magnetograph; EUV and visible-light coronagraph; solar neutron and -ray detector and a radiometer. Heritage for such instruments is from Yohkoh, SOHO, TRACE.

- 8.2 In-Situ heliospheric instruments

- 8.2.1 Solar Wind Plasma Analyser (SWA)  Still old prop. Version!

The SWA will measure separately the three-dimensional distribution functions of the major solar wind constituents: protons, alpha particles and electrons. The basic moments of the distributions, such as density, velocity, temperature tensor, and heat flux vector will be obtained under all solar wind conditions and be sampled fast enough to characterise fully the fluid and kinetic state of the wind. In this way we will be able to determine possible non-gyrotropic features of the distributions, ion beams, temperature anisotropies and particle signatures of wave excitation and dissipation.

The SWA will measure protons, alpha particles and electrons. In addition, measurements of representative high-FIP elements (the C, O, N  group) and of low-FIP elements (such as Fe, Si or Mg) will be carried out, to obtain their abundances, velocities, temperature anisotropies and charge states, and in order to probe the wave-particle couplings (heavy-ion wave surfing) and determine the freeze-in temperatures (as a proxy for the coronal electron temperature). Also, pick-up ions of various origins, such as singly-ionised ions stemming from interplanetary dust, will be measured.

The SWA will unambiguously separate ion species and provide M/Q determination with an energy resolution of at least 5 % in (E/Q) /(E/Q) and range between 10 eV/Q and 30 keV/Q. The angular resolution will be better than 5° in azimuth and elevation angle. The electron analyser will be capable of resolving the strahl (heat flux) and cover a large fraction of 4 in solid angle, with a pitch-angle resolution better than 22.5° and an energy range between a few eV up to 10 keV. To cope with fast wave-particle effects to be expected in the velocity distributions, the sensors will have time resolutions of about 1s or even shorter. The specifications can be met by existing state-of-the-art plasma instruments, using time-of-flight techniques with post-acceleration for particle discrimination.

- 8.2.2 Plasma Waves Analyser (PWA)   Still old prop. Version!

The PWA will identify the various plasma waves and kinetic modes comprising the high-frequency part of the fluctuation and turbulence spectra. The instrument will cover a broad band in frequencies, extending from some Hz into the MHz range. The expected field strength may range between a few V/m and mV/m, and perhaps up to a V/m for the convection electric field. The magnetic field strength is expected to vary between a few nT and T, with large differences between the longitudinal and transverse components with respect to the mean magnetic field and the solar wind flow direction (substantial Doppler shifts are to be expected).

The electromagnetic fluctuations are measured with search coil magnetometers and electric field dipole antennas, minimised in size and arranged in a compact configuration. The PWA will perform on-board processing of the data and deliver selectable wave forms, to identify nonlinear coherent fluctuations. To resolve the vector components of the electric and magnetic fields, which is scientifically highly desirable and required to determine wave modes unambiguously, the search-coils are to be mounted on short booms required for magnetic cleanliness reasons. More than one electric field antenna is needed, which will be short enough to adjust to the variable Debye length of about one meter.

8.2.3 Magnetometers (MAG)   RS, June 5, 2000

The magnetometer will measure the interplanetary magnetic field components over a broad dynamic range from a few nT up to the T range, and for frequencies well below the local heavy-ion or proton gyrofrequencies. The magnetometer may consist of a triaxial identical sensor system. A compromise has to be found between the requirements of the spacecraft and payload systems and the demand of the magnetometer for magnetically clean conditions.

The heritage of MAG includes dozens of similar instruments flown on various magnetospheric, planetary and deep space missions. Expertise has been built up in several European countries, e.g. the UK, Germany, Russia. After all, the MAG sensor and electronics for the Solar Orbiter requires no new development. State of the art instruments can be built for less than 1 kg. Further mass savings appear possible.

MAG might consist of a 3-axis fluxgate magnetometer with the sensor mounted at the end of an extendable boom (length tbd) which is positioned in the shadow of the Orbiter. Other mounting locations such as on the high-gain antenna structure appear possible, provided MAG stays in permanent shade. The MAG proposed for the STEREO mission might serve as a guideline. It is in turn based on the design used for the NEAR and ACE missions. It measures fields in 8 ranges (automatically selected) from 4 nT to 65536 nT with a resolution of 30 ppm, from DC up to AC frequencies of 500 Hz. The parameters can be adjusted according to the particular needs o the Solar Orbiter mission. The DPU design might resemble the ones developed, e.g., for the AMPTE, Ulxsses and Rosetta missions. It could be shared with either the plasma instruments or the wave instruments.

The mass of the sensor package will be less than 1 kg, the average power consumption is about 1 W, and the required telemetry is of the order of 200 bits/s.

Magnetic cleanliness of a spacecraft as required by a magnetometer on board has been a cost driver for some missions in the past. However, more recent experience has shown that there are in fact acceptable low-cost engineering solutions possible. A reasonable magnetic cleanliness program can be performed, if the MAG experimenters work in close contact with the all other teams in order to ensure proper engineering at an early phase and to make other precautions without impacting the mission schedule.

Similar schemes were applied in the past for several successful missions.

- 8.2.4  Energetic Particle Detector (EPD)  Still old prop. Version!

The EPD measurements on the Solar Orbiter will allow to study the sources, acceleration and propagation of solar energetic particles in association with coronal and interplanetary shocks. Energetic pick-up particles originating from outgassing or sputtering of near-Sun dust should be measured as well in conjunction with the SWA measurements. The EPD will determine chemical and charge composition of ions in a wide energy range, from about the typical solar wind energies of a few keV to several 100 MeV/nucleon for protons and heavy ions. Electrons should be measured from 10 keV to 10 MeV.

The combination of electrostatic E/Q-analysis with time-of-flight E/M-determination and subsequent direct energy measurement in a solid state detector has been employed in many EPDs in the past. Ongoing research in solid state detector technology is promising and will certainly lead to further improvements in energy thresholds, and noise level as well as to size reduction. It is essential that all measurements are done fast, typically at 1 s, with complete angular coverage to resolve the pitch-angle distributions. These requirements can readily be met by a low-mass and compact sensor assembly with different look directions and modest aperture sizes. We propose to place several detectors at different positions onboard the spacecraft, in order to obtain a sufficient field of view and angle-of-incidence coverage without using a scanning platform.

- 8.2.5 Dust Detector (DUD) Still old prop. Version!

The DUD will analyse interplanetary dust particles with respect to their composition and orbital characteristics. The spatial distribution of particles with masses ranging between 10-16 g and 10-6 g should be determined. These objectives can be reached with an instrument of Ulysses, Galileo or Cassini heritage. The minimal dust experiment may consist of a multiple sensor assembly, with each sensor looking in a different direction to resolve incidence angles. A low-mass dust-detecting element could consist of a thin polarised polyvinylidene fluoride foil with conducting electrodes on opposite sides. The particle impact irreversibly depolarises this foil in the penetration crater volume and thus creates a fast (ns) current pulse as input to the electronics. The calibrated detector signal carries information about mass and velocity of the impact particle. Its mass can be determined by the foil thickness setting a threshold value. The differential dust particle fluxes are directly obtained from the differences of the integral fluxes measured with foils of various thickness. Full chemical analysis and mass resolution of the important elements H, C, N, O of carbonaceous material and the metals like Na, Mg, Al, Si, Ca and Fe contained in chondritic silicates is desirable.

- 8.2.6  Radio Instruments   Still old prop. Version!

Radio Spectrometer (RAS)   The RAS will measure the solar and interplanetary radio waves in the frequency range from 100 kHz  to 1 GHz, with a sweep period of 1 s and a high spectral resolution (f/f  0.01). The RAS will observe plasma processes associated with energetic electrons from the low corona up to 50 Rs. It will probe the plasma at distances ranging from the solar surface to the spacecraft location, thereby connecting the low-altitude coronal regions observed by the optical instruments with the near-Sun heliospheric conditions specified by the in-situ measurements. The instrument could be divided into four sub-spectrometers according to a frequency ratio of 1:10. Each sub-spectrometer may consist of 128 channels, leading to a total data rate of 4 kb/s.

#### Coronal Radio Sounding (CRS)  A passive radio science experiment can be carried out using the available radio links. The  S/C will pass behind the Sun (superior solar conjunction) on many occasions, making it possible to investigate the solar corona by radio sounding at solar distances much less than the perihelion distance down to at least 2 Rs. Integrated line-of-sight parameters such as electron content (densities), Faraday rotation, scintillations and angular broadening can be recorded. The CRS experiment with a two-way radio link via the S/C high-gain antenna (ranging capability) would require a radio subsystem with dual-frequency phase coherent downlinks at X-band and Ka-band.  Linear polarisation of the downlink signals would enable Faraday rotation measurements as an option. The RF power for both radio links (X and Ka) planned for this mission is sufficient for this investigation. A two-way dual-frequency coherent radio link (X-band uplink; X-band and Ka-band simultaneous coherent downlinks) is considered as the optimal configuration for a sufficiently stable link and for detecting signatures of CME events traversing the radio ray path. If only a one-way radio link (S/C to ground) is feasible due to operational constraints during solar conjunction, the transmitted radio signal must be stabilised by an onboard ultra-stable oscillator (frequency stability 10-11 to 10-12 at 3 s integration time, 200g , 3 W).

- 8.2.7  Neutral Particle Detector (NPD)

## Instrument characteristics

The Lyman- line of neutral hydrogen is used in spectroscopic observations on SOHO to determine the solar wind velocity profile in the corona up to several solar radii. The NPD on the Solar Orbiter will measure primarily neutral hydrogen and thus render possible the detection of the interplanetary "neutral solar wind". The NPD will also measure energetic neutral atoms emitted from various coronal sources, and thus enable images of these coronal emission regions to be constructed from rays of neutral atoms with different velocities. As a baseline, the instrument parameters could be: Energy/mass range: 0.6 to 100 keV/nucleon, moderate velocity resolution; mass resolution is a secondary objective. The field of view (for the imaging capability) of the NPD should be centred at about 17° off the spacecraft-Sun-centre line .

The instrument must be capable of detecting neutral atom particle populations originating in the vicinity of the Sun. Therefore, the instrument entrance system must block radiation emerging from the solar disk. The UV scattered in the solar corona, i.e. Ly-, must be suppressed by a factor of more than 10-10. The entrance system must reduce ions and electrons of the solar wind plasma by a factor of about 10-8 while transmitting neutral atoms. The instrument should detect neutral atoms in the typical solar-wind and supra-thermal velocity range between about 350 and 5000 km/s. The neutral atom flux is expected to be about 100-1000 atoms /(cm2 s) at 0.21 AU, but it could be up to (estimated) 106 atoms/(cm2 s) in a CME.

## Instrument design

A feasible neutral atom detector may consist of two sections: The first compartment accommodates the baffle plates keeping out solar radiation and scattered UV and the deflection unit for charged particles. The second section contains the time-of-flight (TOF) double-coincidence detector. Mid of the two sections is the pinhole/slit that defines the optical properties of the neutral atom detector.

The direct solar radiation is  blocked by a collimator. For the suppression of scattered light within the collimator, the plates are covered with saw-teeth like structures. The collimator consists of, alternatively positive and negative, high-voltage biased plates and therefore acts a charged-particle deflector. The collimator plate length, distance and potential determine the cut-off of the charged-particle transmission (about 100 keV/q). UV originating from the solar corona, i.e. Ly-, is transmitted through the collimator. The grid in front of the pinhole reflects the UV. The grid consists of 2 gratings with a grating constant matched to the Ly- peak (gratings are perpendicular to account for different polarisation angles of the UV light). The UV photons are scattered according to diffraction theory. The grids are more transparent for particles than for UV photons (ratio up to 105 per grating, in dependence on the UV wavelength).

The neutral atoms pass through the pinhole (or slit) and a 0.5 to 1.1 g/cm2 carbon foil. As they passed the carbon foil, there is some probability to release secondary electrons which are accelerated towards the START MCP (multi-channel plate). The atoms travel through the 3-cm-wide TOF drift region and hit the rear STOP MCP. The time of flight for 1 keV/n-100 keV/n atoms is about 8 to 80 ns. The position resolution is 1-D for the START MCP and 2-D for the STOP MCP. The flight pass of the neutral atom can be calculated from these measurements by an inversion of the instrument function. The detector determines the velocity (energy/nucleon) and the flight pass (imaging) of the neutral atoms.

Figure 8.1  The instrument schematics of the Neutral Particle Detector.

An alternative instrument design approach could consist of a solar disc baffle, the pinhole, the charge particle deflection system and the TOF unit. While this approach has superior qualities subduing the solar disc radiation, the length of the charged particle deflection system is very limited and the TOF section and the detectors are much larger and therefore require more mass.

## Resources, mass budget and envelope

-  Volume envelope:  10 x 10 x 20 cm3  including mounting lugs.

-  Mass:  Housing, collimator, TOF structure: up to 300 g MCPs; MCP mounting: 150 g

Electronic boards: HV supply, TOF and interface to DPU:  350 g

Connectors: 	     50 g

Instrument mass:  0.85 kg

DPU share: 	    150 g

Total mass: 1 kg;   total mass with 20% margin: 1.2 kg

-  Power: less than 2 W

- Data/DPU: It is envisaged to use a common DPU with the solar wind plasma instrument. The required tasks of the DPU are command transfer, data handling and the provision of memory for the in-flight look-up tables and software.

-  Field of view: 17° off spacecraft-Sun-centre line

(due to the up to 10° aberration for a 1 keV/n particle at 0.21 AU)

-  Telemetry: less than 0.5 kb/sec

-  Possible accommodation: near the dust detector

### Heritage

Neutral atom detectors have been flown on SOHO, CASSINI and IMAGE. Time-of-flight instruments are for example flown on ULYSSES, SOHO and ACE. The later two made extensive use of the shared DPU concept for different sensors.

- Table 8.1: In-situ  heliospheric instrumentation

| Name | Abbre-viation | Measurement | Specifica-tions | Mass kg | Size cm x cm x cm | Power Watt | Telemetry kb/s |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solar Wind Plasma Analyser | SWA | Thermal ions and electrons | 0-30 keV/Q; 0-10 keV | 6 | 20 x 20 x 20 | 5 | 5 |
| Plasma Wave Analyser | PWA | AC Electric and magnetic fields | V/m - V/m 0.1nT - T | 3 | 20 x 20 x 20 | 3 | 4 |
| Magneto-meter | MAG | DC magnetic field | to 500 Hz | 1 | 10 x 10 x10 | 1 | 0.5 |
| Energetic Particle Detector | EPD | Solar and cosmic-ray particles | Ions and electrons .01-10 MeV | 4 | 20x 20 x20 | 3 | 2.5 |
| Neutral Particle Detector | NPD | Neutral hydrogen and atoms | 0.6 - 100 keV | 1 | 10 x 10 x 20 | 2 | 0.5 |
| Dust Detector | DUD | Interplanetary dust particles | Mass (g): 10-16-10-6 | 2 | 10 x 10 x 10 | 1 | 0.5 |
| Radio Spectrometer | RAS | Coronal radio waves | 100 kHz - 1 GHz | 4 | 10 x 10 x 10 antennae | 10 | 1 |
| Radio Sounding | CRS | Wind density and velocity | X-band Ka-band | (USO 200g) | 5 x 5 x 5 | 3 | 0 |

- 8.3  Remote sensing solar instruments

- 8.3.1  General considerations

An integrated ensemble of high resolution remote sensing instruments is proposed for the Solar Orbiter. Emphasis is put here on the scientific requirements and their feasibility. All the instruments will resolve the small-scale dynamical processes in the solar atmosphere. The combination of instruments proposed here will provide a complete set of multi-wavelength measurements required to understand the Sun’s magnetised atmosphere – from below the photosphere up into the extended corona.

Critical issues like heat input into the instruments and their mechanical structures, mass and power requirements have been investigated.  The feasibility of the instruments has been demonstrated.

- 8.3.2  Visible-light imager and magnetograph (VIM)

Scientific Drivers

The purpose of the VIM is to determine the magnetic field boundary conditions for the MHD processes observed by other remote sensing instruments. It will observe the morphology, dynamics, and strength of the magnetic elements and flux tubes at the photospheric level with a resolution that is consistent with the resolution of the EUV telescopes. It will also provide images, Dopplergrams and magnetograms from the side of the Sun which is invisible from Earth.

VIM will be a vector magnetograph. Having vector capabilities is of fundamental importance to understand the nature of photospheric fields (Are they vertical unipolar fields? Do they harbour complex neutral lines with horizontal sheared fields?). Having vector capabilities is also the only way in which quantitative inferences of the magnetic field in the transition region and corona can be made (from force-free or full 3D MHD extrapolations). The importance of such data for the understanding of coronal processes (like e.g. the initiation of CMEs) has been amply demonstrated by SOHO.

VIM will also produce line-of-sight velocity maps by observing two points at each side of the spectral line. These maps can be used, through local helioseismology techniques, to understand subsurface flows. The internal structure and dynamics of the near polar regions of the Sun is of paramount importance and perhaps the key to our understanding of the solar cycle.

The key scientific goals of the VIM are:

- To resolve solar magnetism down to its fundamental length scale (100 km).

- To provide measurements of the `magnetic carpet` which drives chromospheric and coronal activity.

- To observe and accurately quantify for the first time the surface polar magnetic field of the sun.

- To unveil the small scale photospheric dynamo.

- To provide the first magneto- and Dopplergrammes of the far side of the Sun.

- To measure rotation and flows near the Sun’s poles using techniques of local area helio-seismology and thereby provide crucial constraints on solar dynamo theories.

Instrument Concept

The optics of the VIM include

- the 25 cm diameter High Resolution Telescope (HRT),

- the 2 to 5 cm diameter Full Disk Telescope (FDT), and

- the Filtergraph Optics (FO).

Both telescopes are used alternatively and share the filtergraph optics with the only science detector of the VIM. While the HRT delivers an image of a part of the solar surface with a resolution at its diffraction limit near 0.5 arcsec, the FDT is capable to adjust its focal length such that the solar disk fills entirely the detector at all orbital phases. The HRT will provide high cadence filtergrams, Doppler maps and magnetograms for limited times during the phases of closest approach and polar proximity, while the FDT provides low cadence filtergrams and magnetograms during all orbital phases.

The filtergraph consists of a two Fabry-Perot interferometers with fixed gap widths and a narrow band interference prefilter. The whole assembly is mounted on a rotating stage in a telecentric beam. Rotation of the etalons and the filter provides tuning of a 5 mĂ passband around the magnetically sensitive Fe I 630.2 nm line, also allowing for Doppler shifts of up to 40 km/s resulting from the spacecraft orbit.

High Resolution Telescope

The HRT is a Gregorian reflector with 250 mm entrance aperture and an

effective focal length of 4250 mm. A broad-band interference filter covers the entire telescope and reflects most of the incident sunlight, with the exception of the spectral range of interest. A polished field stop at the primary focus reflects most of the sunlight which is incident on the primary mirror outside the spacecraft. A 1.2 mm diameter hole in the field stop transmits the science field of view. The size of the field stop is sufficient to permit pointing of the telescope anywhere on the solar disk. An external cover shields the telescope during off-sun pointing conditions.

The F ratio of the parabolic primary is F/2. The elliptical secondary mirror has a focal length of 134 mm and provides a magnification of 8.5 for the F/17 telescope focus. The secondary is actively mounted with three translation degrees of freedom for telescope alignment control and pointing.

Two flat mirrors fold the beam in the back focus area of the primary. The first mirror provides tip-tilt control in order to maintain the image position within 0.01 arcsec relative alignment in the presence of 3 arcsec rms spacecraft pointing error. The second mirror is a dichroic which reflects the light longwards of 600 nm towards the telescope focus, while transmitting the shorter wavelengths towards the image position and alignment sensor. The concept for the alignment sensor is described below.

The field of view covers a square with 10 mm side. It contains only a few percent of the total incident power and presents no further thermal problem.

Primary Mirror

Entrance Aperture

Modulator

Telescope Focus

FPI focus

Detector

Primary focus with field stop

Figure 1: perspective view of the optical layout of the high resolution telescope and filtergraph.

The optics of the HRT are made from SiC which combines excellent thermostability with lightweighting capability. Astrium has just completed under contract with the KIS a 360 mm all C/SiC mirror which meets optical surface quality specifications. The possibility to fabricate the entire telescope structure from SiC is under investigation.

Full Disk Telescope

The FDT is mounted on the same optical bench as the FO. It consists of four singlet lenses in a zoom configuration, a modulator and two folding mirrors. The second folding mirror can be moved into the optical beam of the HRT just prior to the focal plane in order to direct the FDT light towards the filtergraph.

The axial position of two elements of the Zoom optics is changed during the orbit in order to adjust the focal length between 233 mm and 888 mm. A fixed aperture stop results in variable diameter of the entrance pupil which changes from 50 mm for the long focal setting to 20 mm for the shortest setting. An interference filter at the entrance transmits the spectral region which is used by the filtergraph and the image position control system and reflects the rest outside the spacecraft.

Figure 2 shows the annotated optical layout of the FDT from a similar point of view as the HRT in Figure 1. Note that the layout is "horizontal" (in the XZ plane) compared to the vertical (YZ plane) layout of the HRT proper. A separate modulator is necessary for the FDT in order to perform polarization modulation within the centered optical path before the first oblique reflection.

Telescope focus

Movable folding mirror

Zoom lens arrangement

Modulator

Figure 2: perspective view of the optical layout of the full disk telescope.

The FDT has its own pointing alignment control system, which consists of a simple limb sensor and the same combination of tip-tilt mirror and dichroic as the HRT. It shares the control electronics with the HRT.

Filtergraph Optics

The FO consists of two singlet lenses which combine the functions of collimator and camera with a 1:2 magnification. Two off-axis mirrors in

between provide a 1:1 pupil relay function and an F/66 telecentric

intermediate image plane. Two 50 mm diameter  Fabry-Pérot etalons at the telecentric image plane provide the required spectral resolution of typically 50mĂ. If necessary, a separate narrow band interference filter will block the secondary transmission maxima of the combined etalons. About  1 of rotation shifts the passband of the two filtergraphs by 1Ă, which is sufficient to scan the spectral line and to account for the S/C velocity effects. The principle proposed here is the same as used in the TESOS filtergraph which operates at the Vacuum Tower Telescope of the Kiepenheuer-Institut on Tenerife.

The FO are mounted together with the FDT on a common optical bench.

Figure 3 shows a plane view of the filtergraph optics. The etalons are not included but their position indicated.

Telescope focus

Detector focus

Off-axis paraboloid

Off-axis paraboloid

FPI etalon position

Figure 3: optical arrangement of filtergraph optics.

Overall Configuration

The combination of all optical subsystems is shown in Figure 4. The HRT is the large cylinder on top of the Y and Z views. The FDT is to the lower left of the Z view and near the bottom of the X view. The FO are to the lower left of the Z view and near the top of the X view.

Figure 4: plain views (top left: Y, top right: Z, bottom: X) of the combined VIM. The HRT is arranged above the FDT and the FO, both of which are mounted on a common optical bench. The optics fit a 300 mm x 400 mm x 1600 mm envelope.

Polarization Modulation Package

The Polarization Modulation Package (PMP) will allow VIM to provide vector magnetograms of the region being observed. The PMP will produce the modulation of the intensity at the CCD as a function of the input polarization state. These intensity changes of the CCD measurements will be used to recover the Stokes vector of the solar light.

The PMP will be located before any mirror reflection of the light beam. In this way the polarization measurements will be free of the instrumental polarization problems induced by the reflections. In the present configuration, VIM will use two PMPs, one for the HRT and one for the FDT. Each PMP will be composed of a rotating retarder and followed by a fixed linear polarizer. The retarder follows the design of the Solar-B instrument, which is based on the Advanced Stokes Polarimeter (ASP) used at Sacramento Peak observatory. The retarder will be positioned at 8 different positions per half rotation, each separated by 22.5 degrees. At each of these positions the CCD will be read and the 8 readouts combined in different ways to determine the final Stokes parameters (I,Q,U,V). Depending on the light flux budget, a number of such independent determinations of the Stokes vector will be added on four memory buffers to reduce the noise to the level demanded by the scientific objectives (of around

10-3 times the continuum intensity level). The retarders will have a retardance at the VIM nominal wavelength of 123 degrees. This value ensures that the three Stokes parameters, Q, U and V, are all measured with similar and large efficiencies. The rotating retarder will be designed with a wedge system that eliminates beam wobbling and fringe patterns. The exact waveplate rotation frequency needs to be defined, but similar devices proposed for other S/C have rotation frequencies of around 0.5 Hz. In this case the CCD (which must be synchronized with the rotation of the waveplate) will be read at 8 Hz.

There are two options to simplify the actual design of the PMP. Both will have to be studied further in a Phase A study. First, instead of using two PMP, only one could be used located after the M4 mirror (that selects the light from either the high-resolution or full-disk telescope [HRT and FDT]). In this case, fixed half-wave retarders should be inserted in each beam to compensate the instrumental polarization produced by the mirror reflections. The second option to simplify the PMP, will be the use of electro-optical devices instead of rotating retarders. Liquid Crystal based retarders offer the possibility to produce the modulation of the Stokes parameters without having to rotate any mechanism. This Liquid Crystal retarders have been tested successfully on the ground for solar polarization studies. Their advantages are the use of a simple voltage signal to produce the Stokes modulation and the lack of moving parts (saving mass and power). Their disadvantage is their sensitivity to UV light. If Liquid Crystals are to become an option, this last issue needs to be carefully studied.

Optical Alignment and Image Stabilization

The optical alignment of the VIM is an important and critical issue. Due to large temperature gradients during the mission, it is important to have reliable and accurate in-flight alignment capabilities. To this end, we propose a wavefront control system which is capable to detect low order modes of wavefront deformations in the telescope. A wavefront sensor measures the actual state of the optical alignment and generates an appropriate error signal. A control system converts this error signal into actuation signals which are used to drive the position of the secondary mirror M2 and the tip-tilt mirror (M3).

The detection principle is based on a correlation tracker which produces tip and tilt error signals.  However, instead of sensing the position of a single image which is derived from the entire pupil of the telescope, seven subapertures sense the local wavefront tilt in two zones of the pupil. The information derived from the seven independently analyzsed images of the same solar scene suffices to determine the coefficients of a Zernike function decomposition of the wavefront error up to the third radial degree. The coefficients for tip and tilt, defocus, and Seidel coma are used as error signals for the control system.  The system is capable to measure and compensate axial and lateral displacement of the secondary mirror M2 as well as dynamic image displacement errors, i.e. it includes the functionality of a correlation tracker.

Telescope focus (F2)

Lenslet array at pupil image

Detector

Figure 5: Optical configuration of wavefront sensor

Figure 5 shows a sketch of the wavefront sensor which is located in the vicinity of the secondary focus. The sensor consists of a collimating lens, a hexagonal lenslet array with seven - one central and six peripheral - elements which cover the pupil of the telescope, and lenses which reimage the lenslet array focal plane onto a matrix array detector (CCD camera). The image scale of the pupil on the array is chosen such that the illuminated areas of the lenslets are equal, taking also into account the central obstruction. All lenslets are partially illuminated. Figure 6 shows the configuration of the lenslet array.

Figure 6: Hexagonal 7-element lenslet array of wavefront control unit. The dashed lines indicate the position of the pupil image.

CCD cameras

A variety of CCD detectors exist on the market that are fully adequate for the VIM. The HRT and FDT telescopes have been designed for a CCD with 2k x 2k pixels of around 10 micron size. Such CCDs are common on the market today. Slight modifications of the existing EEV CCD42-40 (13.5 micron pixels) meet our requirements and they are proposed for use on other missions (e.g. STEREO). These CCDs are thinned, back illuminated low dark current cameras with typical quantum efficiencies of 85 % (AR-coated) at the wavelength of interest for VIM. Shuttered and shutterless (frame transfer) modes are available and it has to be investigated which one suites our application better. As a result different observing modes may be available (slow but large FOV, fast but small FOV). Tradeoffs between photon statistics, image stabilization system bandwidth, rotating frequency of the PMP waveplate and CCD pixel read rate (1MHz is anticipated) will have to be studied.  Marconi (EEV) is presently developing custom CCDs for the Japanese led Solar-B project that will be used in both imaging and spectroscopic modes. Their experience can be of great value for VIM.

Resources and Requirements

Telemetry needs – Data compression.

VIM will take advantage of the high rate observing mode by using a high cadence data acquisition necessary for local helioseismology and magnetic field dynamics. For example, in this mode one can read only 1024 x 1024 pixels (smaller FOV, but still large enough for magnetic field studies) or readout the CCD with 2 x 2 on-chip summing (preserving FOV but with slightly worse spatial resolution that is still adequate for local helioseismology). In this case, 1 full frame (1024 x 1024, 12 bits/pixel) at a cadence of 1 per minute produces a data rate of 200 kb/s. By using efficient  lossy compression algorithms with a reduction factor of 10, the final rate will be around 20 kb/s. In this mode, we are considering storing only one physical quantity each time (e.g. the LOS magnetic field or the LOS velocity). Or one can read all 2048 x 2048 pixels, and send 4 physical magnitudes (for example all 4 Stokes parameters) at a cadence of 1 frame each 15 minutes using a similar data rate. In the low rate observing mode, sending 2 (4) physical magnitudes over the full frame can be done at a cadence of 1 per hour and with a data rate of 3 (6) kb/s.

Mass breakdown and power requirements

A rough breakdown of the estimated mass and power requirements is given in the two tables below.

|  | Mass (g) |
| --- | --- |
| 1. High Res Telescope (incl. ISS, re-imaging, filter, CCD) | 19500 |
| - 25 cm telescope (mirrors + structure) | 4000 |
| - prefilter (@ primary hole) | 400 |
| - rotating quarter wave plate | 1000 |
| - linear polarizer | 200 |
| - folding mirror | 400 |
| - tip-tilt mirror + PZT (incl. electronics) | 1500 |
| - cube beam-splitter | 200 |
| - ISS lens | 200 |
| - ISS detector | 1000 |
| - re-imaging system (4x200) | 800 |
| - Fabry-Perots and Oven | 3000 |
| - shutter | 400 |
| - CCD camera (incl. electronics) | 2500 |
| - harness (25% of total above) | 3900 |
| 2. Full Disk Telescope | 3250 |
| - prefilter | 200 |
| - focal lenses (4*200) | 800 |
| - rotating quarter wave plate | 1000 |
| - linear polarizer | 200 |
| - folding mirror | 400 |
| - harness (25% of total above) | 650 |
| 3. VIM enclosure | 6000 |
| 4. VIM TOTAL | 28750 |

Table xx: VIM mass break-down

|  | Power (W) |
| --- | --- |
| CCD | 5 |
| Image Stabilization System | 5 |
| Fabry Perots & Oven | 10 |
| PMP (x2) | 4 |
| TOTAL | 28 |

Table xx: VIM power requirements.

Instrument Summary

The basic design of the instrument is given in the table below.

| Telescope | HRT: Gregorian reflector - 250 mm diameter - effective focal length 4250 mm FDT: zoom lens - aperture: 20 – 50 mm - effective focal length: 233 – 888 mm |
| --- | --- |
| Spatial resolving element | 0.28 arcsec pixel size (40 km on Sun at 0.2 AU) |
| Spectrometer | - Fabry-Periot double etalon - tuning by rotation |
| Spectral resolution | FWHM: < 50 mÅ |
| Field of View/Pointing | 80x80 Mm2; pointing to anywhere on Sun by tilting and decentering secondary mirror. |
| Detector | CCD: - 10 micron pixels - 2048x2048 array - back thinned |
| Stability | Active image stabilisation system and low order wavefront correction. |
| Telemetry | 20 kbit/s |
| Mass | 28 kg |
| Power | 28 W |
| Size | Optics box: 160 cm x 40 cm x 30 cm |

8.3.3  EUV spectrometer (EUS)

Scientific Driver

Observations of the UV/EUV spectral range are critical for the determination of plasma diagnostics in the solar atmosphere across the broad temperature range from tens of thousands to several million K. Analysis of the emission lines, mainly from trace elements in the Sun’s atmosphere, provides information on plasma density, temperature, element/ion abundances, flow speeds and the structure and evolution of atmospheric phenomena. Such information provides a foundation for understanding the physics behind a large range of solar phenomena.

Current spacecraft instrumentation (SOHO) provides EUV spatial and spectral resolving elements of order 2-3 arcsec and 0.1 Å, respectively, and UV resolutions of 1 arcsec and 0.02 Å.  There are no plans for EUV or UV spectroscopic measurements from the NASA STEREO mission (2004 launch), the NASA Solar Dynamics Observatory (~2005 launch) and the Solar Probe (~2007 launch). The only planned EUV spectrometer for a future mission is the EIS instrument on Solar-B with 1 arcsec (750 km on the Sun) and 0.01 Å resolving elements. However, this instrument is tuned to active region observations with almost no transition region capability.

The European solar physics community has a well established expertise in solar EUV/UV spectroscopy as illustrated by the successful CDS and SUMER instruments on SOHO.

Instrument Concept

Whilst recognising that an EUV spectrometer is an essential component the Solar Orbiter, we must be aware that it must be compact and light-weight, must not be too telemetry ‘thirsty’ and must be able to cope with the thermal and particle environment of such an orbit.

A Ritchey-Chretien telescope design, feeding a spectrometer, has been chosen to minimise the size of the EUS instrument. It has the disadvantage of one extra reflection over, for example, a single-mirror off axis paraboloid system, but the wavelength range selected is geared to bright solar EUV lines.

The instrument structure would be made of carbon fibre, with silicon carbide (SiC) optical components. Multilayers will be considered if the final wavelength selection requires it.

Basic Optical Layout

In the current design, the primary and secondary mirrors are separated by 900mm, with the focal plane 200 mm beyond the primary, making a total telescope length of 1.1 m. The telescope diameter is 120 mm, making an overall diameter of the telescope ‘tube’ of about 150 mm. The telescope effective focal length is 3.7 m.

The slit assembly lies at the focal plane and beyond this is the spectrometer, with a toroidal grating forming a focus at a 2-D detector. The grating ruling spacing is yet to be decided but we take 4800 l/mm as a guide for the current calculations. For the wavelength bands selected (see table), and assuming a spectrometer magnification of 1.5, the spectrometer will project 1.2 m beyond the focal plane with a 1.76 m distance to the detector. The detector will lie 0.55 m off-axis, adjacent to the primary mirror. Thus, without a spectrometer, the instrument cross-section would be of order 15x15 cm2 but the off-axis path of the spectrometer will ensure that the back-end of the instrument is wider, making an overall boot-shaped instrument of maximum dimensions of order 230 cm x 15 cm x 55 cm. The 55 cm dimension is dependent on the choice of grating and the wavelength selection.

The secondary presents a portion of the Sun at the slit and can be rotated to allow rastered images (i.e. exposures interlaced with mechanism movements to build up images simultaneously in selected wavelengths). Only some 20% of the Sun will be visible to the secondary. The instrument can be re-pointed to acquire particular targets.

Calculations of the spot-size at the focus from ray-tracing codes have been performed for various angles of the secondary.  This confirms that the concept of rastering with the secondary is workable for the scientific requirements over the planned range. We plan a 9 micron detector pixel size and a field of view of 34x34 arcmin.

The heritage of this instrument comes from the SOHO/CDS, SOHO/SUMER and Solar-B/EIS projects.

Resolution/Detector

We size the instrument to an observing distance of 0.2 AU (perihelion).  A spatial resolving element of 0.1 arcsec represents 75 km on the Sun from the Earth; the same resolution can be achieved at 0.5 arcsec at 0.2 AU. With regard to spectral resolution, our basic aim is to return sufficient numbers of well separated spectral lines to allow thorough spectroscopic studies.

A detector array of 4k x 4k 9 micron pixels is baselined. Thus, the EUS has a spectral range of 40 Å at 0.01 Å/pixel. The same array will give a spatial extent (vertical distance on the detector = slit length) of 0.5 arcsec x 4096 = 2048 arcsec = 34 arcmin. The solar diameter at 0.2 AU is 170 arcmin, i.e. we have a slit length of 0.2 of the solar diameter. Thus, a pointing mechanism is required. For a given pointing location, rastered imaging will be made up from movement of the secondary mirror in only one direction.

We baseline an APS (Active Pixel Sensor) detector system. The SOHO CCD detectors receive many 'cosmic ray' particle hits, occasionally have 'bleeding' of charge for the brightest intensities (e.g. EIT flares and LASCO planets) and there must be some degradation due to protons causing damage to the silicon lattice (this creates 'traps' that can steal charge which can be transferred to other parts of the image).

In an APS device, the signal charge is sensed by an amplifier within the pixel (each pixel has its own). Therefore, it avoids the charge transfer problem. Pixels will still be hit, but the damage will not propagate because of the transfer process used for CCDs.

This degradation problem has been experienced by Chandra, mainly because the CCDs have a direct line of sight to space for protons and the damage was done mainly by relatively low energy protons (~100 keV) of which there are many. In SOHO, there is a lot more shielding. However, this effectively means that the degradation will be at a slower rate, not that there will be no degradation. For a long duration mission (7 years) heading into a more intense particle environment, we have to consider this seriously. We expect a moore intense particle environment because Solar Orbiter is more likely to encounter accelerated protons from solar events.

Intense sources display bleeding on CCD devices. This is not the case for the APS detector system. This alone could be a tremendous advantage.

The APS detectors are still silicon chip devices, it is just that they incorporate individual pixel amplification and charge extraction. The EUV sensitivity will be provided in the same way as with CCDs, in this case with back-thinned devices. There is no mass overhead in using such a system, relative to the CCD approach.

Resources and requirements

Telemetry

For EUS we baseline 20 kb/s telemetry. The instrument detector has 4096x4096 pixels. It is clear that data from individual exposures will have to be selected carefully, as given by pre-defined observing sequences – i.e. only selected fractions of the full range would be returned in any exposure. In addition, a number of compression schemes should be available with a common (‘lossless’) compression of order 3-5 but other schemes up to a factor of 10 would be available.

In order to cope with the limited telemetry resources, a combination of novel compression schemes, large dedicated instrument memory and careful emission line selection will be applied. In the light of the successful operation of the CDS instrument on SOHO – where a similar selection and compression consideration was applied - and the new opportunities given by the Orbiter’s location, it is anticipated that this instrument can make significant advances with a telemetry rate of order 20 kbit/s.

Stability/Pointing

Given our plan to achieve 0.5 arcsec resolution elements, we must choose one of the following options:

- do not include an image stabilisation system, assuming that the variations of the spacecraft stability occur on timescales much less than the exposure time of the spectrometer and thus any corrections could be done on the ground, or

- include an image stabilisation system possibly making use of VIM limb sensor error signals to drive adjustments to the secondary mirror.

The EUS instrument would require an independent pointing system to enable independent observation of any solar location.

Thermal Environment

At 1 AU the average solar intensity is 1,371 W/m2.  During the nominal phase, in each 149 day period, the spacecraft will encounter a range from 2142 W/m2 (0.8 AU) to 34275 W/m2 (0.2 AU – 25 times the value at 1 AU).  This presents a severe thermal challenge which we tackle in a number of ways.

The SiC primary mirror ‘sees’ the full Sun. SiC optical components can run hotter than traditional components, but the primary receives 310.6 W at 0.2 AU. Gold-coating is used to reduce the absorption (to 0.2). We run the primary at 70oC, and require a dedicated radiator of 0.11 m2 (33 cm x 33 cm).

Some 248.5 W are reflected from the primary. The secondary is sized such that it catches only 20% of this, i.e. it only sees 20% of the solar image and much of the image is reflected back out of the aperture. Thus, 49.7 W is received by the secondary mirror which will also be gold coated. Thus it absorbs 9.9 W which will require dissipation via a small radiator of area about 11 cm x 11 cm. We run the secondary at 83oC.

The reflected component, now 39.8 W continues toward the spectrometer aperture in the primary. This beam can be reduced considerably by stops; only about 1/100 of this incoming beam is required for each exposure.

Finally, the APS detector system should run cold and this would require a cold finger to a further radiator.

For the current design we assume passive radiators to space.

With regard to the remaining structure, the design would include thermal blankets and the front panel could include a solar shield. Heaters will be required to maintain the correct temperature gradients throughout the orbit.

Instrument Pointing/Spacecraft Shield

The EUS can view a 34 x 34 arcmin area of the Sun, which is about 170 arcmin across at 0.2 AU. Thus an independent pointing mechanism is required which needs to cover the range about +/- 3 degrees. The instrument has an aperture of 120 mm. This must match a viewing port in the spacecraft +X-plate shield. To minimise the viewing port size, the instrument will be mounted on six legs (like the SOHO/CDS instrument) with the actuators at the back and the pivot near the front of the instrument. Thus, we envisage a port size of approximately 130 mm. The front shield of the EUS instrument will act as a secondary to the spacecraft shield when the aperture edge and  viewing port edge are not aligned.

Mass Breakdown

A rough breakdown of the estimated mass is given here. We take the view that this is a new generation instrument made of light-weight materials which really ought to be significantly lighter than, for example, the equivalent (100 kg) instruments on SOHO.

| ## Component | Mass (kg) |
| --- | --- |
| Primary mirror | 0.5 |
| Mirror support | 0.3 |
| Secondary mirror | 0.1 |
| Mirror scan mechanism | 0.6 |
| Slit assembly | 0.3 |
| Grating assembly | 0.6 |
| OPS | 1.5 |
| Detector | 1 |
| Detector electronics. | 1.5 |
| Baffles | 0.5 |
| Structure | 5.4 |
| Thermal subsystem | 3.5 |
| Harness | 1.2 |
| Main electronics incl PSU | 6 |
| GRAND TOTAL | 23 |

Instrument Summary

The basic design of the instrument is given in the table.

| Telescope | Ritchey-Chretien type: - 120 mm diameter - 900 mm primary to secondary - 1100 mm secondary to focus - effective focal length 3.7 m - gold-coated SiC mirrors |
| --- | --- |
| Spectrometer | Normal Incidence: - 4800 l/mm toroidal grating - slit/grating distance 1200 mm - grating/detector distance 1760mm |
| Spatial resolving element | 0.5 arcsec (75 km on Sun at 0.2 AU) |
| Spectral resolving element | 0.01 Å/pixel (5 km/s) |
| Raster Mechanism | Through motion of the secondary |
| Detector | Active Pixel Sensor (APS): - 9 micron pixels - 4096x4096 array - back thinned |
| Preliminary Wavelength Selection | - First Order: 580-620 Å - Second Order: 290-310 Å (e.g. Fe XII 291, Si X 293, He I 584, Si IX 296, O III 599, He II 304, Mg X 610, Fe XIX 592). |
| Slit Length/Width | 34 arcmin length; width <9 micron |
| Field of View/Pointing | Raster over 34 arcmin; pointing to anywhere on Sun with independent pointing mechanism. |
| Telemetry | 20 kbit/s |
| Mass | 25 kg |
| Power | 30 W |
| Size | Instrument = 230 cm x 15 cm x 55 cm Electronics box = 20cm x 20cm x 20cm |
| Thermal | Mirror operating temperatures 70 and 83oC, both with radiators. APS detector cooled with cold finger and radiator. Possible shield at front. Standard thermal blankets with heaters for thermal control. |
| Operation | On board deferred command store in which can be loaded time-tagged observation sequences designed on the ground. |
| Stability | Active image stabilisation system at secondary may be considered if jitter is significant over common exposure times. |

- 8.3.4. EUV and X-ray imager (EXI)

Scientific Drivers

- The EIT/SOHO and TRACE imagers have recently exemplified the power of EUV imaging for the understanding of the solar corona at all different spatial and temporal scales.

- The Full-Sun Imager (FSI) EIT not only allowed to survey the general state of the corona (growing and decaying of active regions, coronal holes, ..) but also to pinpoint special fine features such as plumes or bright points and to record impulsive events such as flare, prominence eruption and the start of CMEs (EIT waves).

- With TRACE, the limited field of view (FOV) allowed an increased spatial resolution which revealed a wealth of new features such as the filamentary structure of coronal loops and their (apparent) constant cross-section which puzzles theoreticians. It also appears that the 17.1 nm images are more filamentary than the 19.5 nm images. Unfortunately, for TRACE (as for EIT), images taken at different wavelengths are also taken at different times.

- In the quiet Sun, a lot of "events" of various sizes (bright points, explosive events, jets, blinkers,… ) all evidence heating probably related to magnetic reconnection. Their spatio-temporal behavior (distribution functions exhibiting power laws) is interpreted in terms of small-scale intermittency due to, e.g., MHD turbulence. As for the reconnection, it appears to be pervasive whether the magnetic field lines are closed (quiet Sun) or open (coronal holes). In the last case, it probably takes place low in the atmosphere (Axford & McKenzie 1991, Parker 1991).

- The results obtained lead to new questions concerning the structuring of the corona above the poles, the real basal dimension of coronal structures, the rôle played by nanoflares in the heating of the quiet solar corona. Moreover, SOHO results point at the rôle of an activated network from where high frequency waves, emitted to the outward corona, could resonate with protons and heavy ions and provide the momentum for the solar wind.

- Dramatic new results can be obtained from increasing the spatial resolution by about one order of magnitude on one hand, from connecting chromospheric/coronal phenomena with close in-situ measurements and also from exploring the open field regions on the other hand.

- This is where an EUV Imager on SO can fully exploit the unique capabilities of the mission. Such an imager must provide EUV images with an angular resolution increased by about one order of magnitude in order to reveal the fine structure of most coronal features. To do so, it benefits from the closeness of the Sun. It must also provide full disk EUV images of the Sun in order to reveal the global structure of regions inaccessible such as the "far side" of the Sun and the polar regions.

The European solar physics community has a well established expertise in solar EUV/UV imaging as illustrated by the successful EIT instrument on SOHO. Some studies concerning the thermal (radiative) load on the optics have been shared with Europeans teams involved in the Remote Sensing Package of the Solar Probe mission.

Instrument Concept

A unique imager faces two difficulties : 1/ the best spatial resolution cannot be obtained with a full disk imager for various reasons, the less not being the limited number of pixels on current detectors, a limitation coinciding with the telemetry constraints; 2/ it must cope with a heavy thermal (radiative) load when at perihelion (25 solar constants on the first optical element if the field-of-view (fov) is not reduced). (It also must cope with the particle environment of such an orbit).

This consideration leads to a separation between a High Resolution Image (HRI) and a Full Sun Imager (FSI) all being part of EXI. In this way, one can find specific solutions for the thermal load and have realistic detectors.

HRI

The driver of the design is the need to get rid of the unwanted solar radiation which goes along the direct light path. A first step consists in reducing the entrance aperture of the instrument to about 2 cm and to fully use the available length of the Payload Module. In doing so, one can cluster a few telescopes together, each one dedicated to one wavelength, which eliminates any moving part, takes full use of the aperture and allows for simultaneous observations in different wavelengths.

Essentially one diminishes the FOV through the use of an entrance baffle and one sets the FOV to 70 000 km (or 500 arcsec when at perihelion) for a 2*2 K detector. This sets the equivalent pixel size to 35 km on the Sun (or 0.25 arcsec). The pixel size of the detector is baselined at 9 microns, so the equivalent focal length is 7.2m.

We baseline a set of three telescopes working at 13.3, 17.4, and 30.4 nm. These three wavelengths cover a very wide range of temperatures (from 1.6 107 down to 5 104 K) and targets (from flares to the quiet Sun). Note that the 13.3 band includes a very hot line (Fe XXIII) visible only during flares and that the 17.4 band only selects the 17.4 Fe X line, excluding the cooler Fe IX line at 17.1.

The basic feature of each telescope relies upon the use of a long baffle to reduce the FOV and the stray light, an entrance metallic filter and an off-axis Gregory telescope with multilayers fitted to each wavelength. The Gregory concept allows to put stops at the focal plane and at the image of the entrance aperture; it leads to a major reduction of straylight.

A carbon-carbon composite structure is planned.

Figure X shows the optical scheme of the HRI.

Figure X: HRI optical scheme : B = baffle, M1 = primary mirror, M2 = secondary mirror, F = entrance filter, D = detector. The figure also shows two possible implementations of the three telescopes (boxes on the right-hand side).

2kx2k APS detectors (see EUS section in 8.3.X) are baselined. Diamond or GaN new generation detectors (robust, solar-blind, ..) may be interesting alternatives.

The electronics (including DPU) is common to the three detectors and to the FSI telescope. Its volume should fit within the overall volume given in the Table.

Counts number

For the 17.4 and 30.4 channels, the small aperture allows for a few counts only per pixel per second. One can cope with this low number by increasing the exposure time up to at least 10 s (as compatible with the pointing stability) and/or on-board binning (at least a factor 2*2). (the counts number for the 13.3 channel can be much higher but only on very active regions and flares).

FSI

The Full Sun Imager needs a wide field (about 5°) when at perihelion in order to cover the full lower solar corona with about 50 % margin in each direction. For a 2kx2k detector, this means a pixel size of 9 arcsec (or 1250 km on the Sun at perihelion). In order to solve the issue of thermal (radiative) load on the entrance filter of the Imager, different schemes have been envisaged. We found that the only safe possibility is to move back the entrance filter as much as possible (1800 mm) and as close as possible to the M1 mirror. Then the mirrors combination (all off-axis) allows for an image quality of the order of the pixel size. The entrance filter is crossed twice by the light. This system can work e.g. in the 17.4 line with the appropriate multilayer (Mo/Si). Another solution exists if one works above 120 nm (e.g. in the Lyman alpha line at 121.6 nm). In any case, the dimensions of the instrument are of the order of 2000 x 250 mm x 400 mm, i.e. shorter than HRI but wider.

Figure 4 shows the optical scheme of the FSI.

Figure 4:  FSI optical scheme : B = baffle, M1 = primary mirror, M2 = secondary mirror, F = entrance filter, D = detector.

Operations

Observing sequences should be defined/designed well ahead of time and include target coordinates, sequence of operating telescopes, along with the transmission of a few sample data. These sequences would be uplinked and stored on board in a deferred command store. The operations for a defined period would be stored in this way and sequences would be run autonomously. Planning will be done more on a monthly and weekly basis; however, we think useful to have daily samples of the data (at least a full-Sun image and an image in each wavelength, i.e. the equivalent of 4 Mbits) in order to check the acquisition of scientific data and possibly to change the pointing or the sequence of observations.

Resources and Requirements

HRI

The average rate of 20 kbit/s can be obtained by observing with the three telescopes about one day during the "nominal mission". This duration may be increased by on-board binning (e.g. 2*2) and adopting an observing scheme different from strictly simultaneous observations.

One can also envisage a set of two basic oberving modes : high time resolution (short) sequences followed by low time resolution (long) sequences.

For a typical exposure time (a few ten seconds), the bit rate for each telescope is very high and it is assumed that we can compress data by a factor 20. Actually, with a Rosetta-type compression scheme (Langevin, 2000), tests performed on solar images seem to indicate that rates as high as 50 could be achieved.

Mass budget

The overall mass budget is given in the Table below.

| Component | Mass (kg) |
| --- | --- |
| Structure (including the baffle) : | 6.3 |
| Optics (including mounts) : | 0.2 |
| Detector (including electronics and shielding) : | 0.5 |
| Miscellaneous (harness, ..) | 0.1 |
| Margin | 0.8 |
| Total | 7.9 |
| Total 3 telescopes | 23.7 |
| Pointing mechanism | 2. |
| Electronics shared between the 4 telescopes | 1.5 |
| Grand total for the HRI | 27.2 |

Power

The power requirements concern the detector feeding (it should be remembered that voltage conversion is not lossless), electronics and the pointing actuator (common to the three telescopes). For a given telescope, we anticipate about permanently 4 W for the cycling [power on, integration, dump to memory]. With margin and power for pointing, we reach no less than 5 W. If the three telescopes are working simultaneously, they will consume about 15 W for basic functions (above) to which one should add about 2 W for data compression and dump by the common DPU.

Non-simultaneous (sequential) observations of course lead to a smaller power consumption.

Stability/Pointing

Given our plan to achieve 0.25 arcsec resolution elements and the fact that exposure times are of the order of 10 seconds, an image stabilisation system should be included, possibly making use of the VIM limb sensor error signals to drive adjustments to the secondary mirror.

The three telescopes cluster has its own pointing mechanism since the limitation of the FOV prevents any independent internal pointing. The pointing is performed by a joint displacement of the three telescopes.  We can mention that the S/C having an offset capability of about 10 °, much larger than what is required (2.5 ° at perihelion), it is well possible to point a solar target with the whole S/C.

FSI

An average rate of 1 kbit/s is sufficient to transmit one full sun image every 2400 s (about every 40 minutes) with compression.

The mass is of the order of 13 kg (weight of the structure : 8 kg; optics and mountings : 4 kg; detector and associated electronics : 1 kg).

The power needed is only related to the detector (4 W) since the electronics is common with HRI.

The FSI also relies on the pointing stability of the PLM but with constraints much less stringent than for the HRI.

Instrument Summary

The characteristics of HRI are summarized in the following table.

| 3 Telescopes | Off-axis Gregory; 20 mm diam. |
| --- | --- |
| Spatial resolving element | 0.25 arcsec pixel size (35 km on Sun at perihelion) |
| Pointing | Common pointing mechanism |
| Detector | Baselined on a 9 microns, 2000*2000 |
| Field of View | 70 000 km at perihelion |
| Peak Telemetry Average Telemetry | 720 kbit/s (with compression 20) >20 kbit/s |
| Mass | 24 kg |
| Power | 15 W (essentially detector; shared electronics) |
| Size | Instrument = 2500 mm x 150 mm x 240 mm |
| Thermal | Operating temperature 20oC with passive cooling; 140 cm long baffle at the entrance of each telescope |
| Stability | We assume a pointing stability of about 0.2 arcsec during a typical exposure time of 10s |

The characteristics of FSI are summarized in the following table.

| 3 Telescopes | Off-axis Gregory; 20 mm diam. |
| --- | --- |
| Spatial resolving element | 9 arcsec (1300 km on Sun at perihelion) |
| Pointing | Pointed at Sun center |
| Detector | Baselined on a 9 microns, 2000*2000 detector |
| Field of View | 4*4 solar radii at perihelion |
| Peak Telemetry Average Telemetry | 240 kbit/s (with compression 20) 1 kbit/s |
| Mass | 13 kg |
| Power | 4 W (essentially detector; electronics shared with HRI) |
| Size | Instrument = 2000 mm x 250 mm x 400 mm |
| Thermal | Operating temperature 20oC with passive cooling; 600 mm long baffle at the entrance of the telescope |
| Stability | The constraint is defined by HRI. |

Axford W.I. & McKenzie, J.F., 1991, Solar Wind 7, 1

Langevin, Y., 2000, S.P.I.E., submitted

Parker E. N., 1991, ApJ, 372, 719

8.3.5. Ultraviolet and Visible-light Coronagraph (UVC)

Scientific Drivers

The Solar and Heliospheric Observatory (SOHO) has broadened our ability to investigate the outer solar corona by gaining access to an unexplored electromagnetic domain, the ultraviolet, and to a much wider field of view. This was achieved by flying a set of two coronagraphs, the Ultraviolet Coronagraph Spectrometer (UVCS) and the Large Angle Spectroscopic Coronagraph (LASCO).

The Ultraviolet and Visible-light Coronagraph (UVC) proposed for the Solar Orbiter is designed to combine and extend the imaging capabilities of the SOHO coronagraphs. This instrument can at the same time image the visible and ultraviolet coronal emissions   and  diagnose with  unprecedented temporal coverage and space resolution the full  solar corona.

UVC can provide monochromatic images in the neutral hydrogen HI Ly alpha 1216 Å and single-inonized helium HeII Ly alpha 304 Å lines and measure the polarized brightness (pB) of the K-corona.  These observational quantities are sufficient to diagnose crucial physical parameters, such as the electron density, the outflow velocity through Doppler dimming techniques, and the abundance of the major components of the solar atmosphere.

Corotation, during the helio-synchronous phase of the orbit, will freeze for many days coronal structures in the plane of the sky.  This condition will be most favourable to investigate the evolution of the magnetic configuration of streamers in order to test the hypothesis of magnetic reconnection as one of the main processes leading to the formation of the slow solar wind.

The out-of-ecliptic vantage point will allow for the first time a unique view of the plasma distribution and solar wind expansion in the coronal low-latitude/equatorial belt. Therefore it will be possible to measure the longitudinal extent of coronal streamers and coronal mass ejections.   These parameters, that at present are unknown, are essential to determining the magnetic flux carried by plasmoids and coronal mass ejections in the heliosphere.

UVC will allow the first determination of the absolute abundance (i.e., relative to hydrogen) of helium in corona.  Helium, the second largest contributor to the density of coronal plasma, is important for the dynamics of solar wind, and it may act as a regulator to maintain a nearly constant solar wind mass flux.

UVC will also determine the differential outflow speed of the major components (H, He) of the solar wind and discriminate the mechanisms of solar wind acceleration through  different  mass-charge ions observations.

The European solar community has a well established expertise in visible light and UV coronagraphy as illustrated by the successful LASCO and UVCS instruments on SOHO.

Instrument Concept

UVC is an externally occulted telescope designed for narrow-band imaging of the EUV corona in the He II 304 Å and H I 1216 Å lines, and for broad-band polarization imaging of the visible K-corona, in an annular field of view between 1.2 and 3.5 solar radii, when the Solar Orbiter perihelion is 0.21 AU. When it moves to 0.3 AU, at the end of the nominal  mission the UVC field-of-view  is within 1.8 and 5.3  solar radii.

The telescope optical configuration is an off-axis Gregorian. The EUV L lines are separated with multi-layer mirror coatings and EUV transmission filters. These mirrors with coating optimized for 304 Å still have a good reflectivity at 1216 Å and in the visible. The visible light channel includes an achromatic polarimeter, based on electro-optically modulated liquid crystals.

Optical Design

Telescope:  The external occulter ensures both thermal protection for the optics and better stray-light rejection. A Gregorian telescope design has been chosen because it gives real images of the external occulter and the edges of the telescope primary mirror.  Field and Lyot stops will be used to reduce stray light and diffracted radiation from the telescope primary mirror.  In addition, use of a annular secondary mirror and light trap captures and removes diffracted light from the external occulting disk and its supports.  A light trap behind the primary mirror ensures that only the light reflected by the mirror enters the filters / detectors assembly structure.  The direct light from the solar disk is mostly dumped into space, though a portion of it may be used for radiometery and / or coarse imaging of the solar disk in the three wavelength bandpasses.

Fig.xx: Two views of the optical layout of UVC.

UV and Visible-Light Channels: The instrument structure is made of carbon fiber, with Zerodur optical components. Zerodur is used for the mirrors' substrate because of its extremely small thermal expansion coefficient. Optics made with this material is also easy to polish into non-spherical figures with sub-angstrom rms surface roughnesses.

The basic specifications and drivers of the instrument are summarized in the following table (Table I).

Structure

The instrument structure is of carbon fiber/cyanate or carbon-carbon composite, designed for zero coefficient of thermal expansion in the axial direction.  It is in the form of a rigid rectangular baseplate to which are bonded appropriately placed brackets (also of composite) for the support of the optical and detector elements.  The focus and orientation of each element is set and maintained by a precision spacer located between the element and its support bracket. The optical system is surrounded by a lightweight rectangular composite enclosure and baffle structure that attaches to and further strengthens the baseplate. Removable covers on the top of the enclosure allow for installation and servicing of the optical and detector elements. An external door mechanism shields the telescope aperture when the instrument is not observing.

Thermal

The thermal control approach is based on shielding the instrument from most direct solar radiation, rejecting light from the solar disk that enters the annular entrance aperture, and an opto-mechanical design that is insensitive to bulk temperature changes. The use of a carbon fiber composite structural system allows the optical system to be almost completely athermalized. Moreover, the use of a low secondary magnification factor in the telescope (i.e.3:1) makes the optical system less sensitive to dimensional changes in comparison with more traditional space instruments such as TRACE. Preliminary estimates suggest that the allowable temperature range of the opto-mechanical system may be as much as +/- 50 C. Local temperature control will be needed for the liquid crystal polarization module, which has a smaller allowable temperature range. Thus we consider it prudent to include a small power allocation for thermal control heaters (e.g. 1 watt) pending further analysis.

The occulter is the element of the coronagraph, which reaches the highest temperature since it is directly exposed to the Sun, particularly when the Solar Orbiter is near perihelion.  The occulter has an exiguous conductive link (represented by the thin rods of the support) with the rest of the structure.  The occulter and supporting links are made of titanium and are arranged in a conical geometry such that thermally induced strain is absorbed without de-centering the occulter disk.

The temperature achieved by the occulter has been estimated by assuming a purely radiative heat transfer and an external coating of the same type as that utilised for the sunshield of the payload module.

The thermal flux incident on the occulter is 31 kW/m2 at 0.21 AU and 5.5 kW/m2  at 0.5 AU. The temperature difference experienced by the occulter along the observation phase is at maximum 266.5 C. A small reduction of the temperature will be caused by the transport of part of the heat through the occulter supports.

Detectors

There are two detectors, optimized for the Visible region (4500-6000 Å) and for the Extreme UV (304-1216 Å). Both detectors have 15 micron pixels and an array size of 4096x4096. The baseline visible detector is an Active Pixel Sensor (APS). This detector system is based on CMOS technology, which is attractive for high radiation dose environment. In addition, the APS architecture has the potential to allow on-chip differencing of the polarimetric signals, yielding substantial improvements in signal-to-noise performance in comparison with conventional CCD detectors.

The baseline EUV detector is a Charge Injection Device (CID) array. This detection system is highly resistant to space radiation. It can easily withstand more than 100 times the lethal dose of a typical CCD and radiation-hardened versions can tolerate doses in excess of 1 MegaRad (Si total dosage). CID detectors have randomly addressable pixels, permitting the dynamically programmable read-out of individual pixels and sub-arrays. This feature will be used in UVC for limiting the readout area to the solar corona (excluding the occulted portion field) and for defining "Regions of Interest" to be observed with higher time resolution. EUV sensitivity will be provided by a Micro-Channel Plate image intensifier coupled to the CID array with fiber-optics. This approach results in a detector that is blind in the visible, substantially improving the stray light rejection performance of the EUV channel.

We note that both detector approaches are based to some extent on anticipated developments in rapidly evolving technologies (e.g. array sizes, APS noise performance). Heavily shielded CCD detectors are suitable alternatives in either case.

Resources and Requirements

Data Rates and Volume

The primary driver of the data rate is UVC efficiency in the three channels. In the two EUV lines, the coronal signal is weaker and the instrument efficiency lower than in the visible band. Therefore, longer exposure times will be required for the EUV coronal observations.   On the basis of the count-rate for H I and He II Lyman-alpha emission estimated from a coronal hole (i.e., worst case), we assume an average exposure time of 600 sec.  A single image  (4096 x 4096) pxl2 with 16 bit (b) per pixel takes about 270 Mbit of memory. Assuming that only 50% of the image is used (the rest is occulted disk, extreme corners of the square matrix, etc.), and that a factor of 3 lossless compression can be achieved, we expect a data rate of 0.07 Mbps per detector. Since the Visible-light detector works simultaneously with the H I Lyman-alpha (therefore, about 50% of the time), the data volume accumulated per orbit is about 260 Gbit.  A factor 10 image compression is needed to reach a data rate of about 5 kbps.  This level of compression can be achieved with an acceptable loss of information with schemes such as, for instance, the ``Adaptive Discrete Cosine Transform'' (ADCT). The ADCT technique is being used in the Large-Angle Spectroscopic Coronagraph (LASCO) on SOHO.

Mass

The UVC mass breakdown is given in the table.

| ## Component | Mass (kg) |  |
| --- | --- | --- |
| ## Structure | 6 |  |
| ## Optics | 2 |  |
| ## Mechanisms | 1 |  |
| ## Detectors (2) | 2 |  |
| ## CPU & Interface | 2 |  |
| ## DC/DC converter | 1 |  |
| ## ADC | 1 |  |
| ## Data compressor | 1 |  |
| ## Motor drive | 2 |  |
| TOTAL instrument | 18 |  |

Instrument Summary

| Telescope | Off-axis Gregorian |
| --- | --- |
| Occultation | External occulter: 60 mm diam. |
| Optics | Primary mirror: 33 mm diam. Secondary mirror: 94 mm diam. (multilayer coated mirrors) |
| Eff. Focal Length | 490 mm |
| Field of View | Annular, Sun-centered. Coverage: - R at 0.21 A.U. 1.8-5.3 R at 0.30 A.U. |
| Spatial Resolution | UV (304 Å, 1216Å): 8 arcsec at 1.2 R, 40 arcsec at 3.5 R Visible(5000 Å): 40 arcse entire FOV |
| Stray-light | < 10-8 (visible light); < 10-7 (304 Å, 1216 Å) |
| Wavel. Bandpasses | 1) He II  (304  20) Å 2) H I  (1216  100) Å 3) Visible  (4500 - 6000) Å |
| Detectors | Visible-light: Active Pixel Sensor (APS) EUV (304 Å, 1216 Å): Charge Injection Device (CID) (EUV intensified); 4096 x 4096 array, 15 micron pixels |
| Data Rate | 10 kb/s |
| Phys. Dimensions | (80 x 30 x 30) cm3 |
| Mass (kg) | 22 Total |
| Power (W) | 35 (Operational mode, peak); 25 (Stand-by mode) |

- 8.3.6  Neutron and -ray detector (NGD)

The Solar Orbiter provides a unique opportunity to measure many, and also low-energy (few MeV), neutrons in the perihelion passages. Although the simultaneously produced   rays have been measured frequently on other spacecraft, co-ordinated measurements are possible in scintillation detectors via pulse-height and shape analysis of the signals. Proton-recoil spectrometers or inorganic scintillators allow neutrons to be detected in the energy range between 0.5 and 100 MeV and  rays between 0.1 and 10 MeV, with a large dynamic range, energy resolution of a few percent, and time resolution in the s range. A serious problem is the large mass of several kg of conventional detectors. Mass reduction to less than about 1 kg is technologically feasible (e.g. with a fission detector as used on Pioneers 10/11). For example, McKibben et al. (1997) have suggested  a low-mass design based on detection of neutron-induced fission of heavy nuclei. Their instrument will allow measurements down to very low neutron fluxes for a mass of 1-2 kg and power less than 2 W.

- 8.3.7  Radiometer (RAD)

Scientific Drivers

The irradiance of the Sun (i.e. its brightness as measured above the Earth’s atmosphere) is known to vary by 0.1% over the solar cycle. There is also some evidence for a longer term, secular variation. In spite of its small magnitude, the irradiance variation is a potential cause for climate change. Two basic questions need to be answered before we can reach an understanding of the causes of this variability:

- How does the solar luminosity (i.e. the radiation escaping in all directions) vary? Does it change or is a brightening at the equator compensated by a darkening over the poles?

- Why is the irradiance variability of the Sun a factor of three smaller than that of Sun-like stars?

By carrying out irradiance studies from out of the ecliptic and on the side away from the Earth, the Solar Orbiter will provide a unique opportunity to answer these questions. For example, one major difference between the Sun and Sun-like stars is that whereas we observe the Sun nearly equator on, stars are on average seen from a latitude of approximately 30o. The Solar Orbiter will reach this latitude in the course of the extended mission and test whether this inclination effect really is responsible for the Sun's anomalous behaviour.

The mission profile of the Solar Orbiter is ideally suited for studies to improve our understanding of the angular redistribution of the radiance in active regions of the photosphere. This can be done by observing e.g. the influence of faculae on TSI from different angles: from near the equator (from the Earth-bound  mission) and from higher latitudes from the Solar Orbiter.  Similar studies can also be done and have been done from one platform (e.g. SOHO) using the fact that the Sun rotates, which allows to view the same feature from different angles at different times; the uncertainty, however, is limited by the evolution of the active regions themselves which are known to change on time scales of days at the level of up to 0.1%. From our present understanding, an uncertainty of about 1 part 10000 is needed for such comparative studies. Thus, we have to ensure that IPSIR can be traced back to 1 AU radiometers for all its operational modes and at all times with such an uncertainty. This seems feasible, as shown below, but has to be studied in detail and confirmed during a Phase A.

Instrument Concept

IPSIR is a new generation, room temperature radiometer with four channels, which can be operated independently or in groups of two for internal comparisons. Each channel has its shutter and an independent cover for protection. IPSIR has electrically calibrated cavity receivers. Two cavities are operated together with one closed and the other with cyclic shutter phases of 50s open and 50s closed durations. Phase sensitive detection at the fundamental shutter frequency is used to analyze the signal. This operational mode has many advantages over the normal “active cavity” operation in e.g. SOHO/VIRGO, which relies on reaching a steady state before measurements can be taken (for a description of this novel way of operating radiometers see e.g. George M. Laurence et al. , 2000, “The Total Solar Irradiance Monitor: TIM”, Metrologia, 37, in press).

In order to cope with the large change in solar irradiance from 1 to 0.2 AU three different apertures with 8, 4 and 2 mm diameter will be used at different phases of the mission. A set of four of these apertures is mounted on an aperture wheel which allows the selection of the same aperture size for each of the four cavities. These determine the receiver area (the m2 of the Wm-2 ) and about 80mm back from these apertures are the view-limiting apertures of 10mm diameter in front of each conical-cylindrical cavity (12mm diameter and about 38mm long) yielding a full-angle field of view of 70 and a slope angle of 0.70, 2.10 and 2.90 for 8, 4 and 2mm apertures. This reversed arrangement of the field-of-view apertures has the advantage of less energy in the radiometer and a simpler treatment of the diffraction correction.  At 1 AU with the 8mm aperture an area of 0.5cm2 is illuminated with about 70mW with a radial fall-off to zero within 0.4mm, whereas at 0.2 AU (2mm aperture) a central area of 0.03cm2 is illuminated with about 110mW and the radial fall-off to zero is within 1.7mm. Thus, the spatial irradiance distribution within the cavity changes substantially for the two cases and thus the non-equivalence between the homogenous electrical heating and the inhomogeneous radiative heating will probably be the most important limiting factor of the uncertainty of the radiometer.

The operation will be mostly digital and done by software (S/W) in the instrument's computer. There are essentially only two analog circuits: (i) the amplifier of the thermistor bridge measuring the temperature difference along the thermal impedance and between two cavities (one measuring irradiance the other as thermal reference) and (ii) the highly stable voltage source for the electrical power fed to the cavity. The bridge is powered by a sine wave at about 100Hz produced by S/W and its output is A-D converted and synchronously demodulated by S/W. The demodulated signal is used to PID control the heater power of the shuttered cavity to compensate for the extra radiant power during the open periods. The heater power is deduced from the highly stable voltage source which is pulse-width modulated by the output of the PID controller. This power signal is frequency analyzed and the amplitudes and phases of the shutter fundamental frequency and the first few harmonics of it (up to about the 8th) are transmitted to ground. Only the in-phase signal at the shutter fundamental is needed to calculate the irradiance, the harmonics are for the in-flight characterization of the temporal behavior of the detectors. Some of the correction factors, needed for the final evaluation of the irradiance can be determined and/or checked in flight.

# Instrument Summary

- 4-Channel Radiometer with an uncertainty of <500ppm (goal)

- Three precision aperture diameters: 8, 4 and 2mm corresponding to areas of 0.5, 0.12 and 0.03cm2; the full-angle field of view is 70 and the slope angles are 0.70, 2.10 and 2.90 for 8, 4 and 2mm apertures

- Volume: 110x110x220mm with the 110x110mm face looking to the sun; in front there will be some (probably 2) radiation shields which will extend the length of the box by 40mm and the area of the front to 140x140mm.

- Mass: 4.0 kg including a contingency of 10%

- Power: total power consumption is estimated to 6.6W including a contingency of 10%. This estimate is still uncertain, as the computer type has not yet been selected.

- Normal data rate (measuring with one radiometer) is 750 b/s. During a backup operation (about once a week) 280 b/s have to added. During check-out a total of 3500 b/s will be needed; this mode is during commissioning and if problems have to be analyzed.

- 8.3.8  Other potential solar instruments  Still old prop. Version!

A valid option to be considered in future studies of the Solar Orbiter is the addition of three corona-related instruments:

- A wide-angle white-light instrument with a FOV from 1.5 to 6 Rs, equivalent to the LASCO-C2 on SOHO, would allow one to see extended coronal structures and evolving CMEs. It would cover the cusp regions of helmet streamers, where the slow solar wind is released and begins to accelerate. The highly successful LASCO-C3 instrument (FOV from 3 to 32 Rs at 1 AU) could serve as a design baseline. The required mass should not exceed 4 kg (assuming common electronic systems with the other imagers).

- A heliospheric imager might extend the view of the heliosphere to almost a full hemisphere, from about 3 Rs (at 0.2 AU) to as much as the anti-solar direction. Thus, it would have almost the full Earth-Sun line within its FOV for major parts of the observation time, depending on the spacecraft attitude. This instrument would allow for the first time to observe the propagation of transient disturbances towards the Earth from various distances and aspect angles, in particular from outside the ecliptic plane. Preliminary design studies have demonstrated that such an instrument can probably be built for less than 2 kg. A crucial requirement would be that the hemisphere to be watched must be kept free of Sun-illuminated S/C appendages.

- An X-ray instrument dedicated to the high-temperature portion of the coronal emission could excellently complement the Solar Orbiter payload. A novel configuration for an imaging spectroscopic instrument would operate in the wavelength range from 40 Å to 6 Å and detect emissions from plasma at temperatures between a few 106 K and a few 107  K. Such an instrument working in extremely-grazing incidence will obtain very high spectral and spatial resolution. It would also be able to measure densities equal to or greater than 109 cm-3  (through density-sensitive lines in this high-temperature range).

- Table 8.4: Solar Instrumentation (*depth x width x length)

| Name | Abbre- viation | Measurement | Specifications | Mass kg | Size cm x cm x cm | Power W | Telemetry kb/s |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Visible-light Imager and Magnetograph | VIM | High-res. disk imaging and polarimetry | Na D1 Na D2 | 35 | 30 x 40 x 120 | 30 | 20 |
| EUV Imager and Spectrometer | EUS | Imaging and diagnostics of TR and corona | EUV emission lines | 35 | 30 x 15 x 140 | 30 | 20 |
| X-ray / EUV Imager | EXI | Corona imaging | He and Fe Ion lines | 15 | 10 x 10 x 30 | 5 | 5 |
| Ultraviolet and Visible Light Coronagraph | UVC | Imaging and diagnostics of the corona | Coated mirror coronagraph CCD detector | 25 | 20 x 20 x 50 | 10 | 10 |
| Neutron and -ray Detector | NGD | Solar neutrons gamma rays | 0.5-100 MeV 0.1-10 MeV | 1 | 10 x 10 x 10 | 2 | 0.5 |
| Radiometer | RAD | Solar constant | Visible light | 4 | 15 x 15 x 25 | 5 | 0.5 |