May 1, 2002

Hard Disk Write Block Tool Specification

Version 2.0 Draft May 1, 2002

Hard Disk Write Block Tool Specification	1

Version 2.0 Draft May 1, 2002	1

1	INTRODUCTION	3

2	PURPOSE	3

3	SCOPE	3

4	Technical Background	3

4.1	Hard Disk Attachment and Access	3

4.2	Technical Terminology	4

4.3	Terminology Example	5

5	REQUIREMENTS	5

5.1	Mandatory Requirements	5

5.2	Optional Requirements	6

6	ASSERTIONS	6

6.1	Mandatory Assertions	6

6.2	Optional Assertions	7

7	ABSTRACT TEST CASES	7

8	Interrupt 13 BIOS Access	8

# INTRODUCTION

Accurate and dependable computer forensics tools are required for a reliable means of investigating crimes that involve computers. In order to insure a measure of reliability and assurance that the results are accurate, the tools used in these investigations should be tested. The Computer Forensics Tool Testing project at the National Institute of Standards and Technology (NIST), an agency of the United States Department of Commerce, provides a measure of confidence in the software tools used in computer forensics investigations. It provides law enforcement personnel with a means of deciding whether the tools in consideration for use should be applied to the purposes required.

The central requirement of a sound forensic examination of digital evidence is that the original evidence must not be modified, i.e., the examination or capture of digital data from the hard disks of a seized computer must be performed so that the disk contents are not changed. The investigator follows a set of procedures designed to prevent the execution of any program that might modify the disk contents. These procedures involve a layered defense against any modifications to the source disk using the following strategies:

- Where possible, set a hardware jumper to make the disk read only.

- Use an operating system and other software that are trusted not to write to the disk unless given explicit instructions.

- Use a hard disk write block tool to intercept any inadvertent disk writes.

# PURPOSE

This document defines requirements for hard disk write block tools used in computer forensics investigations and the test methods used to ascertain whether a specific tool meets the requirements.

The requirements are used to derive assertions that will be tested. The assertions are described as general statements of conditions that can be checked after a test is executed. Each assertion generates one or more test cases consisting of a test protocol and the expected test results. The test protocol specifies detailed procedures for setting up the test, executing the test, and measuring the test results.

The requirements and test methods were developed by a focus group of individuals who are expert in the use of hard disk write block tools and have performed investigations that have depended on the results of these tools. As this document evolves through comments from the focus group and others, new versions will be posted to our web site at http://www.cftt.nist.gov.

# SCOPE

The scope of this specification is limited to software tools that protect a hard disk attached to a PC from unintended modification. The specifications are general and could be adapted to other write blocking tools. However, actual testing is currently confined to tools that protect disk access through the interrupt 13 BIOS interface of a PC. Not included are tools that protect a hard disk from modification through other mechanisms such as replacing device drives or hardware write blocking tools. Definitions for hard disk drive related terms can be found in NCITS 347:2001 “American National Standard for Information Technology – BIOS Enhanced Disk Drive Services.”

# Technical Background

This section  provides technical background for a discussion of write blocking technology. The first subsection presents an overview of how hard disks are physically attached to a computer and then accessed by programs running on the computer. The second subsection defines terminology related to write block tools. The last subsection is an example illustrating how the terminology relates to an actual PC.

## Hard Disk Attachment and Access

Before a hard disk drive can be used it must be physically attached to a computer. A hard disk is attached to a computer by one of several available physical interfaces. A disk is usually connected by a cable to an interface controller located either on the mother board or on a separate adapter card. The most common physical interface is the ATA (AT Attachment or IDE) interface. This includes variants such as EIDE or ATA-2, ATA-3, etc. Some of the other physical interfaces include, but are not limited to SCSI (Small Computer System Interface), IEEE 1394 (aka FireWire or i-Link), and USB (Universal Serial Bus).

All access to a disk is accomplished by commands sent from a computer to a disk through the interface controller. However, since the low level programming required for direct access through the interface controller is difficult and tedious, each operating system usually provides other access interfaces. For example, programs running in the DOS environment can, in addition to direct access via the disk controller, use two other interfaces: DOS service interface (interrupt 21) or BIOS service interface (interrupt 13). The DOS service operates at the logical level of files and records while the BIOS service operates at the physical disk sector level. More sophisticated operating systems, for example Windows NT or a UNIX variant (e.g., Linux), may disallow any low level interface (through the BIOS or controller) and only allow user programs access to a hard disk through a device driver, a component of the operating system that manages all access to a device.

## Technical Terminology

A hard disk write block tool replaces or monitors a hard disk access interface on a general purpose host computer with hard disks attached by a physical interface. A hard disk access interface is defined as a method used by a program to access a hard disk. For a program to access a disk, the program issues a high level command to the access interface that is translated by the access interface into the corresponding low level command and is sent to the disk drive through the physical interface controller. For each command issued, the access interface indicates command results (e.g., command completion, error status) by a return value. A hard disk write block tool operates by monitoring disk I/O commands sent from the PC through a given access interface. Any commands that could modify a hard disk are intercepted and not passed on to the hard disk. As of the end of 2001, the most common access interfaces are as follows: hard disk device driver, interrupt 13 BIOS (Basic Input Output Services), ATA (AT Attachment) direct controller, ASPI (Advanced SCSI Programming Interface), USB (Universal Serial Bus) and IEEE 1394 (also known as Firewire). Each interface has its own command set and access protocol. The command set for a given interface is partitioned into the following categories:

- Read: commands that transfer data from the disk to the computer memory.

- Write: commands that transfer data from the computer memory to the disk.

- Information: commands that return information about the disk.

- Control: commands that request the disk to do a nondestructive operation, for example: reset or seek.

- Configuration: commands that change how the disk is presented to the host computer. These commands often destroy data on the disk or make data inaccessible.

- Miscellaneous: commands that do not fit into the other categories.

Appendix A presents two tables: a categorization of the typical interrupt 13 BIOS commands, and a catalog of commands and widely known extensions to the typical command set.

The following terms are defined for convenience in specifying the tool requirements.

- Covered interface: a disk access interface that is controlled by the tool.

- Covered disk: a disk attached to a covered interface.

- Protected disk: a disk designated for protection from modification when accessed by a covered interface.

- Unprotected disk: a disk that is not protected from modification through a specified access interface.

## Terminology Example

Consider a computer with the following four disks attached:

| Example Configuration |  |  |  |
| --- | --- | --- | --- |
| Model | Drive | Physical Interface | Access Interfaces |
| Fujitsu MPF3153AT | 0x80 | EIDE | ATA, BIOS |
| WDC WD200BB-00AUA1 | 0x81 | EIDE | ATA, BIOS |
| QUANTUM ATLAS10K2 | 0x82 | SCSI | ASPI, BIOS |
| SEAGATE ST318404LC | 0x83 | SCSI | ASPI, BIOS |

Each disk is attached to one physical interface, in this case, either EIDE or SCSI. Each EIDE disk can be accessed in either of two ways. The EIDE disks (attached as drives 0x80 and 0x81) can be accessed directly by the AT-Attachment (ATA) interface to the disk controller, or the disk can be accessed through the BIOS (by interrupt 13). The SCSI disks (attached as drives 0x82 and 0x83) can be accessed through either an ASPI driver or through the BIOS (by interrupt 13).

Consider a write block tool that covers only BIOS disk access through the interrupt 13 interface. All of the disks can be accessed through the BIOS, but suppose that the tool is executed with drives 0x81 and 0x82 specified for protection. The covered interface is the interrupt 13 BIOS access. The tool does not cover the ASPI or ATA interfaces. All the disks are covered disks when accessed by the BIOS. None of the disks are covered disks when accessed by either the ATA or ASPI interfaces. Disks on drives 0x81 and 0x82 are protected disks when accessed by the BIOS. All the disks are unprotected when accessed by either ATA or ASPI.

| Protection Specified for BIOS interrupt 13 access to 0x81 & 0x82 |  |  |
| --- | --- | --- |
| Disk/access interface | Covered | Protected |
| 0x80/BIOS | yes | no |
| 0x80/ATA | no | no |
| 0x81/BIOS | yes | yes |
| 0x81/ATA | no | no |
| 0x82/BIOS | yes | yes |
| 0x82/ASPI | no | no |
| 0x83/BIOS | yes | no |
| 0x83/ASPI | no | no |

Note that even though disks on drives 0x81 and 0x82 are protected from modification by access through the BIOS interface, these disks could still be modified if accessed through the ATA or ASPI interfaces.

# REQUIREMENTS

This section presents mandatory requirements that all write block tools must meet and a list of optional requirements that some write block tools may provide.

## Mandatory Requirements

The informal hard disk write block tool requirements are the following:

- The tool shall not allow a protected disk to be changed.

- The tool shall not prevent obtaining any information from or about any disk.

- The tool shall not prevent any changes to a disk that is not protected.

The three informal requirements are the essence of a write blocking tool: protect the evidence from alteration while allowing a complete examination of the evidence. A formal statement of these requirements follows:

- The tool shall block any commands to a protected disk in the write, configuration, or miscellaneous categories.

- The tool shall not block any commands to an unprotected disk.

- The tool shall not block any commands to a protected disk in the read or information categories.

- The tool shall give an indication to the user that the tool is active.

- The tool shall report all disks accessible by the covered interfaces.

- The tool shall report the protection status of all disks.

- The tool shall, if so configured, adjust the return value of any blocked commands to indicate that the operation was carried out successfully even though the operation was blocked.

- The tool shall, if so configured, adjust the return value of any blocked commands to indicate that the operation failed.

- Return values of information commands shall be consistent with return values of any blocked commands. (For example, a command to return status of last command after a blocked command shall return the same value as returned by the blocked command.)

## Optional Requirements

The following requirements define optional tool features. If a tool provides the capability defined, the tool is tested as if the requirement were mandatory. If the tool does not provide the capability defined, the requirement does not apply.

- The tool shall alert the user when a command is blocked, either by an audio or a visual signal.

- The tool shall be able to uninstall itself if requested.

- The user shall be able to specify a subset of the covered disks for protection.

- The tool shall log a subset of command executions that have been blocked.

# ASSERTIONS

Each assertion provides a specific class of conditions that can be tested and the result that is expected.

## Mandatory Assertions

- If a disk is protected and a command from the write category is issued for the protected disk then the tool shall block the command.

- If a disk is protected and a command from the configuration category is issued for the protected disk then the tool shall block the command.

- If a disk is protected and a command from the miscellaneous category is issued for the protected disk then the tool shall block the command.

- If a disk is protected and a command from the read category is issued for the protected disk then the tool shall not block the command.

- If a disk is protected and a command from the information category is issued for the protected disk then the tool shall not block the command.

- If a disk is not protected and a command from any category is issued for the protected disk then the tool shall not block the command.

- If the tool is executed then the tool shall issue a message indicating that the tool is active.

- If the tool is executed then the tool shall issue a message indicating all disks accessible by the covered interfaces.

- If the tool is executed then the tool shall issue a message indicating the protection status of each disk attached to a covered interface.

- If the tool is configured to return success on blocked commands and a command is blocked by the tool then the return code shall indicate successful command execution.

- If the tool is configured to return fail on blocked commands and a command is blocked by the tool then the return code shall indicate unsuccessful command execution.

- If the tool is active and a command is blocked and the next command issued is a return status of last command then the value returned shall match the value returned by the blocked command.

## Optional Assertions

- If the tool blocks a command then the tool shall issue either an audio or a visual signal.

- If the tool is active and the tool is then uninstalled then no commands to any disk shall be blocked.

- If a subset of all covered disks is specified then commands from the write, configuration and miscellaneous categories shall be blocked for disks in the selected subset.

- If a subset of all covered disks is specified then commands from the read and information categories shall not be blocked for disks in the selected subset.

- If a subset of all covered disks is specified then no commands from any category shall be blocked for disks not in the selected subset.

- If the tool is active and command logging is specified then the tool shall create a log of commands blocked.

# ABSTRACT TEST CASES

[Editor’s note: Test cases will be added after the requirements are in final form.]

Abstract test cases describe the combinations of test parameters required to fully test each assertion. They are abstract in that they do not prescribe the exact environment in which the tests are to be performed. They are written at the next level above the environment. This allows different environments to be substituted under the test cases for testing different products and options.

A set of test parameters is chosen to cover the assertions from various aspects. Not all possible tests will be specified since this number could run into the hundreds or thousands based on the combinations of parameters that could be used. Exhaustive testing, in most cases, is not economically feasible. Instead, a subset of parameters will be used to define the set of test cases needed to evaluate tools against the requirements.

- Interrupt 13 BIOS Access

A typical set of interrupt 13 BIOS disk access commands can be categorized as follows.  Other BIOS vendors or software installed on a PC may add or change functionality for the interrupt 13 interface.

| Categorization of Interrupt 13 BIOS Commands |  |  |
| --- | --- | --- |
| Command | Code | Category |
| Reset | 00h | control |
| Get last status | 01h | information |
| read sectors | 02h | read |
| Write sectors | 03h | Write |
| Verify sectors | 04h | information (or read or control) |
| Format Cylinder | 05h | Configuration |
| Read Drive Parameters | 08h | Information |
| Initialize Drive Parameters | 09h | Configuration |
| Read Long Sector | 0Ah | Read |
| Write Long Sector | 0Bh | Write |
| Seek Drive | 0Ch | Control |
| Alternate disk reset | 0Dh | Control |
| Test drive ready | 10h | Information |
| Recalibrate drive | 11h | Configuration |
| Controller diagnostic | 14h | Configuration |
| Read drive type | 15h | Information |
| Check extensions present | 41h | Information |
| Extended read | 42h | Read |
| Extended write | 43h | Write |
| Verify sectors | 44h | Information |
| Extended seek | 47h | Control |
| Get drive parameters | 48h | Information |

The following table is a list of common interrupt 13 BIOS disk access commands and widely known extensions derived from http://www.ctyme.com/rbrown.htm (August 23, 2000). NIST does not make any claims for the accuracy of the contents but has included this to illustrate commands that should be used for test cases of interrupt 13 based write blocker tools. Some commands that might modify hard disk contents are highlighted with a gray background.

| Common Interrupt 13 BIOS Commands |  |
| --- | --- |
| Command | Description |
| AH = 00h | DISK - RESET DISK SYSTEM |
| AH = 01h | DISK - GET STATUS OF LAST OPERATION |
| AH = 02h | DISK - READ SECTOR(S) INTO MEMORY |
| AH = 03h | DISK - WRITE DISK SECTOR(S) |
| AH = 04h | DISK - VERIFY DISK SECTOR(S) |
| AH = 05h | FLOPPY - FORMAT TRACK |
| AH = 05h | FIXED DISK - FORMAT TRACK |
| AH = 05h | Future Domain SCSI BIOS - SEND SCSI MODE SELECT COMMAND |
| AX = 057Fh | 2M - FORMAT TRACK |
| AH = 06h | FIXED DISK - FORMAT TRACK AND SET BAD SECTOR FLAGS (XT,PORT) |
| AH = 06h | Future Domain SCSI BIOS - FORMAT DRIVE WITH BAD SECTOR MAPPING |
| AH = 06h | Adaptec AHA-154xA/Bustek BT-542 BIOS - IDENTIFY SCSI DEVICES |
| AH = 06h | V10DISK.SYS - READ DELETED SECTORS |
| AH = 07h | FIXED DISK - FORMAT DRIVE STARTING AT GIVEN TRACK (XT,PORT) |
| AH = 07h | Future Domain SCSI BIOS - FORMAT DRIVE |
| AH = 07h | V10DISK.SYS - WRITE DELETED SECTORS |
| AH = 08h | DISK - GET DRIVE PARAMETERS (PC,XT286,CONV,PS,ESDI,SCSI) |
| AH = 08h | V10DISK.SYS - SET FORMAT |
| AX = 08000h | SecureDrive - INSTALLATION CHECK |
| AH = 09h | HARD DISK - INITIALIZE CONTROLLER WITH DRIVE PARAMETERS (AT,PS) |
| AH = 0Ah | HARD DISK - READ LONG SECTOR(S) (AT and later) |
| AH = 0Bh | HARD DISK - WRITE LONG SECTOR(S) (AT and later) |
| AH = 0Ch | HARD DISK - SEEK TO CYLINDER |
| AH = 0Dh | HARD DISK - RESET HARD DISKS |
| AH = 0Eh | HARD DISK - READ SECTOR BUFFER (XT only) |
| AH = 0Fh | HARD DISK - WRITE SECTOR BUFFER (XT only) |
| AH = 10h | HARD DISK - CHECK IF DRIVE READY |
| AH = 11h | HARD DISK - RECALIBRATE DRIVE |
| AH = 12h | HARD DISK - CONTROLLER RAM DIAGNOSTIC (XT,PS) |
| AH = 12h | Future Domain SCSI CONTROLLER - STOP SCSI DISK |
| AH = 12h | SyQuest - START/STOP SCSI DISK |
| AH = 13h | HARD DISK - DRIVE DIAGNOSTIC (XT,PS) |
| AH = 13h | SyQuest - READ DRIVE PARAMATERS (for DOS 5+) |
| AH = 14h | HARD DISK - CONTROLLER INTERNAL DIAGNOSTIC |
| AH = 15h | DISK - GET DISK TYPE (XT 1/10/86 or later,XT286,AT,PS) |
| AH = 16h | FLOPPY DISK - DETECT DISK CHANGE (XT 1/10/86 or later,XT286,AT,PS) |
| AH = 17h | FLOPPY DISK - SET DISK TYPE FOR FORMAT (AT,PS) |
| AX = 1700h | Future Domain SCSI CONTROLLER - GET INQUIRY INFO FROM SCSI DEVICE |
| AH = 18h | DISK - SET MEDIA TYPE FOR FORMAT (AT model 3x9,XT2,XT286,PS) |
| AH = 18h | Future Domain SCSI BIOS - GET SCSI CONTROLLER INFORMATION |
| AH = 18h | PU_1700.COM - INSTALLATION CHECK |
| AH = 18h | XDF.COM - API |
| AH = 19h | FIXED DISK - PARK HEADS ON ESDI DRIVE (XT286,PS) |
| AH = 19h | Future Domain SCSI CONTROLLER - REINITIALIZE DRIVE |
| AH = 1Ah | ESDI FIXED DISK - FORMAT UNIT (PS) |
| AH = 1Ah | Future Domain SCSI CONTROLLER - GET SCSI PARTIAL MEDIUM CAPACITY |
| AH = 1Bh | ESDI FIXED DISK - GET MANUFACTURING HEADER |
| AH = 1Bh | Future Domain SCSI CONTROLLER - GET POINTER TO SCSI DISK INFO BLOCK |
| AH = 1Ch | Future Domain SCSI CONTROLLER - GET POINTER TO FREE CONTROLLER RAM |
| AH = 1Ch | ESDI FIXED DISK - ??? |
| AX = 1C08h | ESDI FIXED DISK - GET COMMAND COMPLETION STATUS |
| AX = 1C09h | ESDI FIXED DISK - GET DEVICE STATUS |
| AX = 1C0Ah | ESDI FIXED DISK - GET DEVICE CONFIGURATION |
| AX = 1C0Bh | ESDI FIXED DISK - GET ADAPTER CONFIGURATION |
| AX = 1C0Ch | ESDI FIXED DISK - GET POS INFORMATION |
| AX = 1C0Dh | ESDI FIXED DISK - ??? |
| AX = 1C0Eh | ESDI FIXED DISK - TRANSLATE RBA TO ABA |
| AX = 1C0Fh | ESDI FIXED DISK - ??? |
| AX = 1C12h | ESDI FIXED DISK - ??? |
| AH = 1Dh | IBMCACHE.SYS - CACHE STATUS |
| AH = 1Fh | SyQuest - DOOR LATCH/DOOR BUTTON DETECT |
| AH = 20h | DISK - ??? (Western Digital "Super BIOS") |
| AH = 20h | Compaq, ATAPI Removable Media Device - GET CURRENT MEDIA FORMAT |
| AH = 20h | QUICKCACHE II v4.20 - DISMOUNT |
| AH = 21h | HARD DISK - PS/1 and newer PS/2 - READ MULTIPLE DISK SECTORS |
| AH = 21h | QUICKCACHE II v4.20 - FLUSH CACHE |
| AH = 22h | HARD DISK - PS/1 and newer PS/2 - WRITE MULTIPLE DISK SECTORS |
| AH = 22h | QUICKCACHE II v4.20 - ENABLE/DISABLE CACHE |
| AH = 23h | HARD DISK - PS/1 and newer PS/2 - SET CONTROLLER FEATURES REGISTER |
| AH = 23h | QUICKCACHE II v4.20 - GET ??? ADDRESS |
| AH = 24h | HARD DISK - PS/1 and newer PS/2 - SET MULTIPLE MODE |
| AH = 24h | QUICKCACHE II v4.20 - SET SECTORS |
| AH = 25h | HARD DISK - PS/1 and newer PS/2 - IDENTIFY DRIVE |
| AH = 25h | QUICKCACHE II v4.20 - SET FLUSH INTERVAL |
| AH = 26h | QUICKCACHE II v4.20 - UNINSTALL |
| AH = 27h | QUICKCACHE II v4.20 - INSTALLATION CHECK |
| AH = 28h | QUICKCACHE II v4.20 - SET AUTOMATIC DISMOUNT |
| AH = 29h | QUICKCACHE II v4.20 - NOP |
| AH = 2Ah | QUICKCACHE II v4.20 - SET BUFFER SIZE |
| AH = 2Bh | QUICKCACHE II v4.20 - DRIVE ACCESS SOUNDS |
| AH = 2Ch | QUICKCACHE II v4.20 - SET BUFFERED WRITES |
| AH = 2Dh | QUICKCACHE II v4.20 - SET BUFFERED READ |
| AH = 2Eh | QUICKCACHE II v4.20 - SET FLUSH COUNT |
| AH = 2Fh | QUICKCACHE II v4.20 - FORCE IMMEDIATE INCREMENTAL FLUSH |
| AH = 30h | QUICKCACHE II v4.20 - GET INFO |
| AH = 31h | QUICKCACHE II v4.20 - RESERVE MEMORY |
| AH = 32h | QUICKCACHE II v4.20 - ENABLE CACHING FOR SPECIFIC DRIVE |
| AH = 33h | QUICKCACHE II v4.20 - DISABLE CACHING FOR SPECIFIC DRIVE |
| AH = 34h | QUICKCACHE II v4.20 - SECTOR LOCKING |
| AH = 35h | QUICKCACHE II v4.20 - SET LOCK POOL SIZE |
| AH = 36h | QUICKCACHE II v4.20 - SET TRACE BUFFER SIZE |
| AH = 37h | QUICKCACHE II v4.20 - SET BUFFERED READS FOR SPECIFIC DRIVE |
| AH = 38h | QUICKCACHE II v4.20 - SET BUFFERED WRITES FOR SPECIFIC DRIVE |
| AH = 39h | QUICKCACHE II v4.20 - SET READ BUFFER SIZE FOR SPECIFIC DRIVE |
| AH = 3Ah | QUICKCACHE II v4.20 - SET WRITE BUFFER SIZE FOR SPECIFIC DRIVE |
| AH = 3Bh | QUICKCACHE II v4.20 - ENABLE/DISABLE ??? |
| AH = 3Ch | QUICKCACHE II v4.20 - ENABLE/DISABLE ??? |
| AH = 3Dh | QUICKCACHE II v4.20 - ENABLE/DISABLE CYLINDER FLUSH FOR DRIVE |
| AH = 3Eh | QUICKCACHE II v4.20 - SET SINGLE-SECTOR BONUS |
| AH = 3Fh | QUICKCACHE II v4.20 - SET BONUS THRESHOLD |
| AH = 40h | QUICKCACHE II v4.20 - SET "sticky_max" |
| AH = 41h | IBM/MS INT 13 Extensions - INSTALLATION CHECK |
| AH = 41h | QUICKCACHE II v4.20 - SAVE/RESTORE ??? |
| AH = 42h | IBM/MS INT 13 Extensions - EXTENDED READ |
| AX = 4257h ("BW") | Beame&Whiteside BWLPD - INSTALLATION CHECK |
| AH = 43h | IBM/MS INT 13 Extensions - EXTENDED WRITE |
| AH = 44h | IBM/MS INT 13 Extensions - VERIFY SECTORS |
| AH = 45h | IBM/MS INT 13 Extensions - LOCK/UNLOCK DRIVE |
| AH = 46h | IBM/MS INT 13 Extensions - EJECT MEDIA |
| AH = 47h | IBM/MS INT 13 Extensions - EXTENDED SEEK |
| AH = 48h | IBM/MS INT 13 Extensions - GET DRIVE PARAMETERS |
| AH = 49h | IBM/MS INT 13 Extensions - EXTENDED MEDIA CHANGE |
| AH = 4Ah | Bootable CD-ROM - INITIATE DISK EMULATION |
| AX = 4B00h | Bootable CD-ROM - TERMINATE DISK EMULATION |
| AX = 4B01h | Bootable CD-ROM - GET STATUS |
| AH = 4Ch | Bootable CD-ROM - INITIATE DISK EMULATION AND BOOT |
| AX = 4D00h | Bootable CD-ROM - RETURN BOOT CATALOG |
| AH = 4Eh | IBM/MS INT 13 Extensions v2.1 - SET HARDWARE CONFIGURATION |
| AX = 5001h | Enhanced Disk Drive Spec v3.0 - SEND PACKET COMMAND |
| AX = 5001h | VIRUS - "Andropinis" - INSTALLATION CHECK |
| AX = 5342h ("SB") | ScanBoot - INSTALLATION CHECK |
| AX = 5501h | Seagate ST01/ST02 - Inquiry |
| AX = 5502h | Seagate ST01/ST02 - RESERVED |
| AX = 5503h | Seagate ST01/ST01 - Set Device Type Qualifier (DTQ) |
| AX = 5504h | Seagate - ??? - RETURN IDENTIFICATION |
| AX = 5504h | Seagate ST01/ST02 - RETURN IDENTIFICATION |
| AX = 5505h | Seagate - ??? - PARK HEADS |
| AX = 5505h | Seagate ST01/ST02 - PARK HEADS |
| AX = 5506h | Seagate ST01/ST02 - SCSI Bus Parity |
| AX = 5507h to 550Dh | Seagate ST01/ST02 - RESERVED FUNCTIONS |
| AX = 5514h | Seagate - ??? |
| AX = 5515h | Seagate - PARK HEADS??? |
| AH = 59h | SyQuest - Generic SCSI pass through |
| AH = 70h | Priam EDVR.SYS DISK PARTITIONING SOFTWARE??? |
| AH = 75h | ??? |
| AH = 76h | ??? |
| AX = 7B00h | NOW! v3.05 - GET INFORMATION |
| AX = 7B01h | NOW! v3.05 - ??? |
| AX = 7B02h | NOW! v3.05 - SET INFORMATION |
| AX = 7B03h | NOW! v3.05 - ??? |
| AX = 7B04h | NOW! v3.05 - ??? |
| AX = 7B05h | NOW! v3.05 - GET DISK ACCESSES??? |
| AX = 7B06h | NOW! v3.05 - GET ??? |
| AX = 7B07h | NOW! v3.05 - GET ??? |
| AX = 7B08h | NOW! v3.05 - ??? |
| AH = 80h | FAST! v4.02+ - API |
| AX = 8001h | FAST! v4.02+ - GET CACHE INFORMATION |
| AX = 8006h | FAST! v4.02+ - INSTALLATION CHECK |
| AX = 8007h | FAST! v4.02+ - UNHOOK INTERRUPTS |
| AH = 81h | Super PC-Kwik v3.20+ - ??? |
| AH = 82h | Super PC-Kwik v3.20+ - ??? |
| AH = 83h | Super PC-Kwik v3.20+ - ??? |
| AH = 84h | Super PC-Kwik v3.20+ - ??? |
| AH = 85h | Super PC-Kwik v3.20+ - ??? |
| AH = 86h | Super PC-Kwik v4.00+ - ??? |
| AH = 87h | Super PC-Kwik v4.00+ - ??? |
| AH = 88h | Super PC-Kwik v4.00+ - ??? |
| AH = 89h | Super PC-Kwik v5.10+ - ??? |
| AH = 8Ah | Super PC-Kwik v5.10+ - ??? |
| AX = 8EEDh | HyperDisk v4.01+ - ??? |
| AX = 8EEEh | HyperDisk v4.01+ - ??? |
| AX = 8EEFh | HyperDisk v4.01+ - ??? |
| AH = 92h | Super PC-Kwik v5.10+ - ??? |
| AH = 93h | Super PC-Kwik v5.10+ - ??? |
| AH = 94h | Super PC-Kwik v5.10+ - ??? |
| AH = 95h | Super PC-Kwik v5.10+ - ??? |
| AH = 96h | Super PC-Kwik v5.10+ - ??? |
| AH = 97h | Super PC-Kwik v5.10+ - ??? |
| AH = 98h | Super PC-Kwik v5.10+ - ??? |
| AH = 99h | Super PC-Kwik v5.10+ - ??? |
| AH = 9Ah | Super PC-Kwik v5.10+ - ??? |
| AH = 9Bh | Super PC-Kwik v5.10+ - ??? |
| AH = 9Ch | Super PC-Kwik v5.10+ - ??? |
| AH = 9Dh | Super PC-Kwik v5.10+ - ??? |
| AH = A0h | Super PC-Kwik v3.20+ - GET RESIDENT CODE SEGMENT |
| AH = A1h | Super PC-Kwik v3.20+ - FLUSH CACHE |
| AH = A2h | Super PC-Kwik v3.20+ - ??? |
| AH = A3h | Super PC-Kwik v5.10+ - DISABLE CACHE |
| AH = A4h | Super PC-Kwik v5.10+ - ENABLE CACHE |
| AH = A5h | Super PC-Kwik v5.10+ - PROGRAM TERMINATION NOTIFICATION |
| AH = A6h | Super PC-Kwik v5.10+ - PROGRAM LOAD NOTIFICATION |
| AH = A7h | Super PC-Kwik 5.1 - ??? |
| AX = A759h | Novell DOS 7 - SDRes v27.03 - ??? |
| AH = A8h | Super PC-Kwik 5.1 - ??? |
| AH = A9h | Super PC-Kwik 5.1 - EXITCODE RETRIEVAL NOTIFICATION |
| AH = AAh | Super PC-Kwik v4+ - ??? |
| AH = ABh | Super PC-Kwik v4+ - ??? |
| AH = ACh | Super PC-Kwik v4+ - ??? |
| AH = ADh | Priam HARD DISK CONTROLLER??? |
| AH = ADh | Super PC-Kwik v4+ - ??? |
| AH = AEh | Super PC-Kwik v5.10+ - ??? |
| AH = B0h | Super PC-Kwik v3.20+ - ??? |
| AX = E000h | XBIOS - COMMAND |
| AX = EC00h | VIRUS - "Tiso" - INSTALLATION CHECK |
| AH = EEh | SWBIOS - SET 1024-CYLINDER FLAG |
| AH = EFh | Ontrack Drive Rocket - SET CYLINDER OFFSET |
| AH = F2h | VIRUS - "Neuroquila" - INSTALLATION CHECK |
| AH = F9h | SWBIOS - INSTALLATION CHECK |
| AH = FAh | PC Tools v8+ VSAFE, VWATCH - API |
| AX = FD50h | VIRUS - "Predator" - INSTALLATION CHECK |
| AH = FEh | SWBIOS - GET EXTENDED CYLINDER COUNT |
| AH = FFh | EZ-Drive - INSTALLATION CHECK |
| AH = FFh | IBM SurePath BIOS - Officially "Private" Function |
| AX = FFFFh | UNIQUE UX Turbo Utility - SET TURBO MODE |