Centralized Hydrogen Production from Coal Gasification with Sequestration

The figure represents a process for hydrogen production from coal that uses the gasification/reforming technology with hot-gas cleanup, the water-gas shift process, and carbon dioxide sequestration.  The hydrogen produced is separated at low pressure (~300 psi) and compressed before delivery to the hydrogen distribution system.  Oxygen is used so that a concentrated stream of carbon dioxide is produced for sequestration.

The petroleum energy use and the CO2 emissions from this process are associated with the inability to sequester all of the CO2 produced in the gasification process inefficiency of the sequestration process as well as the energy needed for hydrogen liquefaction/pipeline compression, delivery, and compression of the hydrogen at the forecourt.

| Well-to-Wheels Energy and Greenhouse Gas Emissions Data |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Current (2005) Gasoline ICE Vehicle | Current (2005) Gasoline Hybrid Electric Vehicle | Current (2005) Coal with Sequestration - FCV | Future (2030) Coal with Sequestration - FCV |
| Well-to-Wheels Total Energy Use (Btu/mile) | 5,900 | 4,200 | 5,100 | 3,200 |
| Well-to-Wheels Petroleum Energy Use (Btu/mile) | 5,300 | 3,800 | 100 | 40 |
| Well-to-Wheels Greenhouse Gas Emissions (g/mile) | 470 | 340 | 210 | 60 |
| Cost of Hydrogen ($/gge, Delivered) |  |  | 5.10 | 2.20 |

This analysis is based on the best available technology in the laboratory.  Currently, it has not been validated through demonstration.

Notes: Centralized Hydrogen Production from Coal Gasification with Sequestration

- Source: Well-to-wheels energy, petroleum and greenhouse gas emissions information from the Argonne National Laboratory GREET model, Version 1.7.  Well-to-wheels values represent primary fuel production, electricity production, hydrogen production, and hydrogen delivery. Fossil resource exploration and equipment manufacture is not included.

- Source: Cost, resource requirements, energy requirements, all fuel and feedstock energy contents, and efficiency values for the Current (2005) case is from the National Renewable Energy Laboratory and the H2A model, Version 1.0.9, Central Coal Gasification plant with a capacity of 308,000 kg/day.

- Source: Cost, resource requirements, energy requirements, all fuel and feedstock energy contents, and efficiency values for the Future (2030) case is from the Central Coal Gasification H2A model case, Version 1.0.9.

- Basis is 1 kg of hydrogen, dispensed from filling station for 5,000 psi fills.  A kg of hydrogen contains approximately the same amount of energy as one gallon of gasoline, or one gallon of gasoline equivalent (gge).

- Diagram is for Future (2030) case, showing feedstock and energy consumption levels required to meet technology cost goals.  Flows in diagram represent direct energy and emissions between production and dispensing, and are not based on well-to-wheels calculations.

- The petroleum use and resultant GHG emissions are associated with the grid electricity for delivery, as well as CO2 that could not be captured and sequestered.  85% of the plant CO2 is assumed to be captured and sequestered.  CO2 separation and sequestration costs are $15/metric tonne of carbon.

- Fossil-based power plants generating grid electricity for the future (2030) case are assumed to sequester carbon emissions at a rate of 85%.

- Production plant electricity requirements for the future (2030) case are met through internally-generated power.

- The hydrogen delivery in the Current (2005) case assumes liquid hydrogen delivery by truck from a central plant located 76 miles from the forecourt station.  Liquefier efficiency is 77.4%.  Truck fuel consumption is 6 mi/gallon.  Data were obtained from the H2A Delivery Scenario Model and GREET.  The Future (2030) case assumes hydrogen pipeline delivery over 76 miles.  Delivery costs include necessary compression and/or liquefaction equipment.

- Cost of hydrogen delivery for the Current (2005) and Future (2030) cases is assumed to be $3.50/kg and $1.00/kg, respectively.

- For the Current (2005) case, hydrogen is assumed to be received at the forecourt as a liquid and dispensed as a gas for 5,000 psi fills.  For the Future (2030) case, hydrogen is assumed to be received at the forecourt as gaseous hydrogen at 250 psi by pipeline and dispensed for 5,000 psi fills.  The cost of the forecourt operations is included in the delivery cost.

- Efficiency results are presented in terms of lower heating value (LHV) of hydrogen.

- Future (2030) case assumes pipeline compressed gas delivery to the forecourt station.  Pipeline energy use calculated using the H2A Delivery Models.

- The efficiency of the electric forecourt compressor, which raises the pressure of the gaseous hydrogen for 5,000 psi fills, is 94%.

- The operating capacity factor of the production plant is 90%.

- Coal feedstock prices are based on the 2015 projections for electric utility steam coal by the Department of Energy’s Energy Information Administration Annual Energy Outlook 2005 High A case.  Prices shown in table are in 2005 $.  Feedstock is inflated at 1.9%/year for the 40 year operating life of the plant.

- Electricity is consumed by the process for compression and plant operations.  Electricity prices are based on the 2015 projections for industrial-rate electricity by the Department of Energy’s Energy Information Administration Annual Energy Outlook 2005 High A case.  Prices shown in table are in 2005 $.  Electricity is inflated at 1.9%/year for the 40 year operating life of the plant.

- The levelized capital cost for the Current (2005) and Future (2030) cases are $1.00/kg hydrogen and $0.67/kg of hydrogen

- Cost of hydrogen is the minimum required to obtain a 10% internal rate of return after taxes on the capital investment.

- The data relevant to the Centralized Hydrogen Production from Coal Gasification with Sequestration technology diagram above is provided in the table below.

|  | Current (2005)Coal with Sequestration - FCV | Future (2030) Coal with Sequestration - FCV |
| --- | --- | --- |
| Coal Feedstock Price ($/million Btu LHV) | 1.19 | 1.19 |
| Coal Feedstock Price ($/ton) | 26.70 | 26.70 |
| Energy in Coal Feedstock (Btu) | 194,000 | 189,000 |
| Electricity Price ($/kWh) | 0.052 | N/A - process electricity internally generated |
| Electricity to Process (Btu) | 1,000 | - |
| Energy Losses from Process (Btu) | 79,000 | 73,000 |
| Pressure of Hydrogen from Production (psi) | 300 | 300 |
| Energy Use for Delivery at the Forecourt (Btu) | Negligible | 7,200 |
| Energy Use for Delivery Transport (Btu) | 34,000 | 2,000 |
| Hydrogen Dispensing Fill Pressure (psi) | 5,000 | 5,000 |
| Plant Gate Energy Use Including Coal Feedstock (Btu) | 195,000 | 189,000 |
| Production Process Efficiency | 59% | 61% |
| Pathway Efficiency | 51% | 58% |
| CO2 Sequestered (lb/gge of hydrogen produced) | 38 | 37 |
| Greenhouse Gas Emissions from Production (lb/gge of hydrogen produced) | 7.2 | 7.0 |