Before the

Federal Communications Commission

Washington, D.C. 20554

| In the Matter of Amendment of Part 15 regarding new requirements and measurement guidelines for Access Broadband over Power Line Systems Carrier Current Systems, including Broadband over Power Line Systems | ) ) ) ) ) ) ) ) ) | ET Docket No. 04-37 ET Docket No. 03-104 |
| --- | --- | --- |

ERRATUM

Released:  November 12, 2004

By the Chief, Office of Engineering and Technology:

- On October 28, 2004, the Commission released a Report and Order, FCC 04-245, in the above captioned proceeding.  This erratum makes the following corrections to the Report and Order.

- Paragraph 3 of Appendix B of the Report and Order should read:

3.  Section 15.15 is amended by revising paragraph (b), to read as follows:

Section 15.15  General Technical Requirements

*  *  *  *  *

(b)  Except as follows, an intentional or unintentional radiator must be constructed such that the adjustments of any control that is readily accessible by or intended to be accessible to the user will not cause operation of the device in violation of the regulations.  Access BPL equipment shall comply with the applicable standards at the control adjustment that is employed.  The measurement report used in support of an application for Certification and the user instructions for Access BPL equipment shall clearly specify the user- or installer-control settings that are required for conformance with these regulations.

*  *  *  *  *

- Paragraph 4 of Appendix B of the Report and Order should read:

4.  Section 15.31 is amended by revising paragraph (f)(5), to read as follows:

Section 15.31  Measurement standards.

*  *  *  *  *

(f) *  *  *

*  *  *  *  *

(5)  Measurements shall be performed at a sufficient number of radials around the equipment under test to determine the radial at which the field strength values of the radiated emissions are maximized.  The maximum field strength at the frequency being measured shall be reported in the equipment authorization report.  This paragraph shall not apply to Access BPL equipment on overhead medium voltage lines.  In lieu thereof, the measurement guidelines established by the Commission for Access BPL shall be followed.

*  *  *  *  *

- Paragraph 5 of Appendix B of the Report and Order should read:

5.  Section 15.37 is amended by adding paragraph (m) to read as follows:

Section 15.37  Transition provisions for compliance with the rules.

*  *  *  *  *

(m) All Access BPL devices that are manufactured, imported, marketed or installed on or after [insert date 18 months from date of publication in the Federal Register] shall comply with the requirements specified in subpart G of this part, including certification of the equipment.

- This action is taken pursuant to authority contained in Section  0.241 of the Commission’s rules on delegated authority,  47 C.F.R § 0.241(a), and these corrections shall be included in the Report and Order, prior to its publication in the Federal Register and FCC Record.

FEDERAL COMMUNICATIONS COMMISSION

Edmond J. Thomas

Chief

Office of Engineering and Technology