NATURAL RESOURCES CONSERVATION SERVICE

Idaho

CONSTRUCTION SPECIFICATIONS

FOR

POWER FENCE

_________________________________		_____________________________________

(Owner/Operator)					(Project Title)

GENERAL

Installation shall be in accordance with an approved plan.  Details of construction shown on the drawings but not include herein are considered as part of these specifications.  Construction activities shall be in accordance with applicable OSHA regulations.

Prior to construction the fence lines shall be cleared of any possible obstruction that would hinder the fence placement and operation.

The soil surface along the fence line shall be relatively smooth such that placement of the bottom fencing member does not exceed the maximum fence member to soil surface spacing specified.

The fence materials shall have an expected life of 10 years with routine maintenance.  All wood materials except Orange Osage, Western Red Cedar, Juniper and Black Locust that have contact with the soil shall be treated with an EPA-registered wood preservative.  Wood posts shall be treated from the butt end of the post to distance of at least 30 inches for line posts and 36 inches for all corner, gate and brace posts.  Refer to Table 1 for the life expectancy of treated versus untreated wood posts.

MATERIALS

Fiberglass Posts.  Fiberglass posts shall be of heavy duty specified material with a minimum diameter of 1 inch.

Wood Insultimber.  Iron wood posts shall be a minimum of 1 inch square in dimensions.

Wood Posts.  Line posts shall have a minimum top diameter of 3 inches and shall be a minimum of 6 feet in length.  Corner, gate and brace posts shall have a minimum top diameter of 5 inches and shall be a minimum of 7 feet in length.  Braces shall have a minimum top diameter of 4 inches and shall be a minimum of 6 feet in length.

Table 1.  Life Expectancy of Untreated and Treated Fence Posts (Years)

| Kind of Wood | Un-treated | Pressure Treated | Hot and Cold Bath | Cold Soak |
| --- | --- | --- | --- | --- |
| Western Red Cedar | 12-15 | 20-25 | 20-25 | - |
| Lodgepole & Ponderosa Pine | 2-4 | 20-25 | 15-20 | 10-20 |
| Aspen or Cottonwood | 1-3 | 15-20 | 10-15 | 5-10 |
| Douglas Fir & Western Hemlock | 3-6 | 20-35 | 15-25 | 10-20 |

Wire.  The grade of the wire shall be 12.5-gauge or heavier galvanized high tensile with a minimum tensile strength of 135,000 psi.  Galvanizing shall be no less than Class III (0.8 oz. of zinc per square foot of surface).

Fasteners.  Fasteners may be made of high quality glazed porcelain or UV resistant HDPE or HDPP plastic.  Galvanized 9-gauge wire may be used on fiberglass and insultimber posts.

Stays. Fiberglass stays shall be used; they shall consist of marble, fiberglass and high-polymer resins.

Energizer Units.  The unit shall include a high impact self-insulating weather resist case, a snap-in circuit panel, a safety pace fuse, a lightening arrester, have full power input and reduced power output, be high power/low impedance with 5,000-volt peak output and have a 2.3 millionlombs charge and a pulse that is furnished within 300 millionths of a second.  The unit shall operate on 110-volt, 220-volt electrical power or 12-volt battery or solar–powered system.

INSTALLATION

The fence shall be reasonably straight and shall not deviate more than 12 inches between any corner and gate or line brace assembly.

The line tension for all line wires shall not be less than 175 pounds per wire.

All power fences will be grounded with at least three ½-inch diameter galvanized rods.

Post Depth.  Fiberglass line posts shall be set a minimum depth of 1.5 feet and wood line posts shall be set to a minimum depth of 2 feet, unless otherwise specified.  Gate, corner and brace posts shall be set to a minimum depth of 3 feet, unless otherwise specified.

Post Spacing.  The maximum post spacing interval shall be 50 feet on fences without fence stays and 100 feet with one stay between posts.

Line Bracing.  Line brace assemblies shall be located at all corners, gates and abrupt changes in vertical topography (generally considered as 15 degrees).  On straight reaches of fencing line braces shall be installed at a spacing of no more than 1300 feet.

Wire Spacing.  Wire spacings are as follows, unless otherwise specified:

Table 2.  Wire Spacing (Cattle Only Fence)

| Fence Type | Spacing Measured From Groundline (inches) |  |  |  |
| --- | --- | --- | --- | --- |
| 2-wire | 18 - | 30 + |  |  |
| 3-wire | 18 + | 26 - | 36 + |  |
| 4-wire | 12 - | 20 + | 28 - | 38 + |

Table 3.  Wire Spacing  (Sheep Only Fence)

| Fence Type | Spacing Measured From Groundline (inches) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 4-wire | 10 + | 16 - | 24 + | 32 - |  |
| 5-wire | 6 + | 12 - | 18 + | 24 - | 34 + |

Note:  On spacing (+) denotes charged wire and (-) denotes a ground wire.  Where the free movement of antelope is a concern, the bottom wire shall always be a ground wire and alternate wires should be charged from the bottom wire.

Wire Fasteners.  Fasteners shall be attached to the posts in such a manner so as not to cause grounding of the line wires.

Stays.  Stays shall be uniformly spaced between the posts as required for the specified post spacing.

Grounding Rods.  All energizers must be grounded.  Ground rods for the fence must be driven into the ground a minimum of 6 feet or additional grounding rods shall be added.