ASSESSMENT OF CRITICAL

THIN FILM RESOURCES

RAF-9-29609

TELLURIUM

Introduction

On April 4, 1999 NREL commissioned an assessment of the availability of tellurium resources in relation to the potential substantial increase of tellurium use in photo voltaic (PV) solar cells. The tasks to be accomplished in this study consist of the following:

- Provide general background information on tellurium.

- Identify current suppliers (distributors and producers) of tellurium.

- Describe production methods for tellurium.

- Determine production capacity and output potential

- Quantify solar cell industry tellurium demand

- Identify current and potential tellurium resources.

- Quantify ability of producers to scale up production for tellurium.

- Determine likelihood of new market entrants for tellurium should demand and or price increase.

- Analysis of tellurium market stability at various pricing scenarios (2,4,8 times increase over current price).

Background

Tellurium was discovered in 1782  in a complex group of gold telluride minerals in the ores of Transylvania (Romania) by Austrian chemist Joseph Franz Mueller von Reichenstein. The element was named after the Latin word “tellus” meaning earth. The discovery of the Seebeck effect (conversion of heat to electricity) in 1822 and the Peltier effect (conversion of electricity to heat) in 1834 opened the way for the utilization of tellurium in thermoelectric devices (Ageton, 1969).

Tellurium is one of the least abundant elements and tends mostly to form its own elements in metallic sulfide deposits. Widely distributed in nature, tellurium is a constituent in over 40 minerals. The most significant associations are with sulfide minerals of copper, lead, zinc and iron. Other less common associations are with precious metals and platinum group metals. Tellurium is a nonmetallic element; its atomic number is 52 and atomic weight is 127.6; and it exists in eight stable isotopes. The specific gravity of tellurium is 6.24 and melts at 452 C°. (Davidson, Lakin, 1973)

As of 1997 tellurium was used primarily as free machining additives to steel. Other uses included catalysts, chemical uses, nonferrous alloys, photoreceptors and thermoelectric devices.(Brown, 1997)

Production Methods

The concentration of tellurium in copper, lead, massive sulfides (complexes primarily of copper/lead/zinc) gold and platinum group ores is overall very low. The estimated tellurium content shown in Table 1 is per refined ton of metal from these deposits or regions. It should be noted that the tellurium values in the table are averages and that individual deposits in the different regions may have substantially more or less tellurium than the average.

Table 1

Tellurium Concentration

Deposit/Region					Pounds Per Refined Ton Of Metal

U.S. Porphyry Copper Deposits				0.4

Sudbury Canada Massive Sulfide Deposits			0.06

World Wide Lead Ore Deposits				0.1

Peru Porphyry Copper Deposits				0.2

Chile Porphyry Copper Deposits				0.4

Congo Copper Deposits					0.4

Zambia Copper Deposits					0.4

Russian/CIS Copper Nickel-Copper, Massive Sulfide

Deposits							0.4

Mexican Copper and Massive Sulfide Deposits		0.4

Australia/New Guinea Copper Deposits			0.4

Japan/Philippines/Chinese Copper and Massive Sulfide

Deposits							0.4

European Copper and Massive Sulfide Deposits		0.4

The majority of tellurium produced comes from the precious-metals-rich anode slimes that accumulate during the electrolytic refining of copper.(Jensen, 1985)  Lesser amounts of tellurium are also recovered from soda slag skimmings generated in lead refining (Brown, 1997) ; nickel/copper leach solutions generated in platinum group metals refining (Mining Journal, 1998); and flotation concentrates generated from gold milling (Suttill, 1991).

There are several different methods for recovering tellurium from the various refining processes. Basic recovery methods are detailed in the descriptions to follow. Refinements and modifications of these methods by the operators have taken place and are considered proprietary.

Electrolytic Copper Refining-Anode Slime Processing

Recovery Method 1- Slimes are roasted at high temperatures with sodium bisulfate. Sulfuric acid is used to oxidize copper minerals and to volatilize selenium from the slimes. Copper is then leached from the crushed slime residue with water and a subsequent leach with sodium hydroxide dissolves the tellurium. The insoluble slime residue is smelted in a dore´ furnace to recover the precious metals.(Jensen, 1985)

Recovery Method 2 - An alternative method is to oxidize both selenium and tellurium in concentrated sulfuric acid to the soluble plus VI valence state. The tellurium is then precipitated by adjusting the pH to 7 with sodium hydroxide.(Jensen, 1985)

Lead Refining-Soda Slag Processing

Tellurium and selenium obtained from furnace soda slag are leached with water and sent to neutralization tanks where tellurium is precipitated from the selenium solution.(Jensen, 1985)

Platinum Group Metals Refining - Nickel/Copper Leaching

Copper-nickel rich matte is leached in a series of pressure leach autoclaves and atmospheric leach tanks which leach the nickel, copper, other associated base metals, selenium and tellurium. A copper cement rich in selenium and tellurium is produced from this process. (Mining Journal, 1998)

Gold Milling-Gold/Telluride Processing

Tellurides are refractory to cyanidation and must be removed and treated separately. Ore is crushed and the telluride is floated to produce a concentrate. The concentrate is leached with a 5% solution of calcium oxychloride and a 5% solution of sodium hydroxide to dissolve and precipitate the tellurium as calcium tellurate. (Suttill, 1991)

For all of the processing methods described, approximately 0.17 pounds (43%) to 0.14 pounds (35%) of the 0.4 pounds (average content) of contained tellurium is recovered (Davidson, Lakin, 1973). The main focus of all of the processing methods is to maximize the recoveries of precious metals. If sufficient price and demand incentives were in place for tellurium, recovery percentages could increase.

The final product derived from all of the processing methods is a tellurium oxide of variable purity. Generally, this tellurium sludge or precipitate is sent to specialized refining facilities for final processing into high purity tellurium products. Tellurium and tellurium dioxide sludges or precipitates are purified to commercial grades by repeated dissolution and selective precipitation or by thermal reduction or electrolysis. Commercial grade tellurium is 99.7% tellurium, 0.003% lead, 0.02% selenium, 0.004% silica and 0.015% copper. The high purity tellurium is prepared by distillation and sublimation procedures under vacuum and further purified by a series of zone melting and refining steps. High purity tellurium is 99.9% to 99.9999% pure and is sold as ingots, sticks, granules, powder or lumps. (Jensen, 1985), (Ageton, 1969).

Current Suppliers (Producers, Distributors)

Throughout the world there are numerous processing facilities who produce tellurium products in three categories:

- Tellurium Dioxides - Sludges and precipitates of variable purity around 35%.

- Commercial Grade Tellurium - 99.7% purity

- High Purity Tellurium - 99.9% to 99.9999%+

The organizations who produce tellurium products of variable quality; commercial grade; and high purity are profiled in the following list. Research has indicated that, because of current low demand, several of the facilities listed no longer recover tellurium. If improving market conditions would warrant, these facilities could again recover tellurium with a minimum amount of effort.

Company			: Phelps Dodge

Facility Location		: El Paso, Texas

Phone				: 800-223-8567

Type Operation		: Refinery

Tellurium Product Output	: Variable grade tellurium dioxide

Tellurium Source		: Trace amounts of Te contained in porphyry copper

deposits

Company			: Asarco

Facility Locations		: Amarillo, Texas

Denver, Colorado

Phone				: 806-383-2201 Amarillo

303-296-5900 Denver

Type Operations		: Refinery - Amarillo

High Purity Metals Processing-Globe Plant - Denver

Tellurium Product Output	: Amarillo - Commercial grade tellurium

Tellurium dioxide

Denver - High purity Tellurium

Tellurium Source		:  Trace amounts of Te contained in porphyry copper

deposits and potentially other types of deposits.

Company			: Dowa

Facility Locations		: Japan

Phone				: 03-3201-1062

Type Operation		: Refinery

Tellurium Product Output	: Tellurium Dioxide

Commercial grade tellurium?

Tellurium Source		: Copper and massive sulfide ores and concentrates from

Australia and other parts of the world including some

Japanese production.

Company			: Pacific Rare Metals Industries Inc. (Subsidiary of

Lewer Corporation)

Facility Location		: Manila, Philippines

Phone				: 612-9363-9741 (Sydney Australia Office)

Tellurium Product Output	: Commercial Grade Tellurium

High Purity Tellurium

Tellurium Source		: Copper and massive sulfide slimes and sludges from

refineries around the world.

Company			: Noranda Metallurgy Inc. (Subsidiary of Noranda Inc.)

Facility Locations		: Montreal, Quebec, Canada - CCR Refinery

Saint Laurent, Quebec

Phone				: 416-982-7423 (Toronto Office)

Type Operations		: Refinery - Montreal

High Purity Metals Processing Plant - Saint Laurent

Tellurium Product Output	: CCR Refinery - Tellurium dioxide

Saint Laurent - High Purity Tellurium

Tellurium Source		: Base metal and massive sulfide deposits controlled by

Noranda and possibly other sources

Company			: Inco

Facility Location		: Copper Cliff, Ontario, Canada

Phone				: 705-682-4211

Type Operation		: Smelter and Refinery

Tellurium Product Output	: Tellurium dioxide - variable purity

Tellurium Source		: Copper-nickel deposits controlled by Inco

Company			: Alamalyk Mining and Metallurgical Combine

(AMMC is a Government run company)

Facility Location 		: Uzbekistan - Various locations

Phone				: 988-261-43-03-03

Type operation		: Smelter and refinery

Tellurium Product Output	: Tellurium Oxide - variable purity

Tellurium Source		: Copper-gold- molybdenum mine in Uzbekistan

Company			: Kazakh Polymetals

Facility Location		: Kazakhstan

Phone				: 49-10-00 (Additionally requires intln. access #’s)

Type Operation		: Smelter, refinery, metal processing

Tellurium Product Output	: Commercial grade tellurium

Tellurium Source		: Various complex sulfide deposits in Kazakhstan

Company			: Balkhash Integrated Mining and Metallurgical Works

Facility Location		: Kazakhstan

Phone				: 2-20-35 (Additionally requires intln. access #’s)

Type Operation		: Smelter, refinery, metal processing

Tellurium Product Output	: Commercial grade tellurium

Tellurium Source		: Copper-molybdenum, copper-nickel and massive sulfide

deposits in  Kazakhstan

Company			: Chimkent Lead Works

Facility Location		: Kazakhstan

Phone				: 12-39-35 (Additionally requires intln. access #’s)

Type Operation		: Smelter, refinery

Tellurium Product Output	: Tellurium oxide concentrates

Tellurium Source		: Lead zinc deposits in Kazakhstan

Company			: Norilsk Mining and Metallurgical Combine

Facility Location		: Russia-Krasnoyarsk Region

Phone				: No listed number

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium oxide

Commercial grade tellurium?

Tellurium Source		: Nickel-copper, gold, platinum deposits in the Norilsk

District.

Company			: Krastsvtmet Plant

Facility Location		: Russia

Phone				: No listed number

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Oxide

Commercial grade Tellurium?

Tellurium Source		: Copper and base metal mineral deposits in Russia.

Company			: Uralelectromed  Combine

Facility Location		: Russia - Urals

Phone				: No listed number

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Oxide

Commercial grade tellurium

Telllurium Source		: Copper and base metal deposits in Russia

Company			: Kyshtym Electrolytic Copper Plant

Facility Location		: Russia

Phone				: No listed number

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Oxide

Tellurium Source		: Copper deposits in Russia

Company 			: Centromin (Government Company)

Facility Location		: Central Peru

Phone				:511-476-9264

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Dioxide

Commercial Grade Tellurium

Tellurium Source		: Copper; lead-zinc deposits in Peru

Company			: Southern Peru Copper Corp (Subsidary of a JV in

which Asarco is a majority owner

Facility Location		: West central Peru - Ilo

Phone				:212-510-2147 (Asarco Home Office)

Type Operation		: Refinery

Tellurium Product Output	: Tellurium Dioxide

Tellurium Source		: Asarco controlled copper mines in Peru plus possibly some

other non company controlled sources

Company			: Metallurgie Hoboken Overpelt (Sogem?)

(Believed to have commercial tie in to Union Miniere)

Facility Location		: Belgium

Phone				: 32-2-227-77-77 (Sogem)

Type Operation		: Refinery

Tellurium Product Output	: Tellurium Dioxide

Tellurium Source		: European lead-zinc deposits and others world wide

Company			: Nippon Mining Co. Ltd.

Facility Location		: Japan

Phone				: 8-135573-7332

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Dioxide

Tellurium Source		: Copper and base metal concentrates from around the world

Company			: Mitsubishi Metal Corp.

Facility Location		: Japan

Phone				: 3-5800-9373

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Dioxide

Tellurium Source		: Copper and base metal concentrates from around the world

Company			: Mitsui Mining and Smelting

Facility Location		: Japan

Phone				: 8-133778-6029

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Dioxide

Tellurium Source		: Copper and base metal concentrates from around the world

Company			: Sumitomo Metal Mining

Facility Location		: Japan

Phone				: 8-133436-7801

Type Operation		: Smelter, Refinery

Tellurium Product Output	: Tellurium Dioxide

Tellurium Source		: Copper and base metal concentrates from around the world

Company			:  China Nonferrous Metal Industry Corp.

(Government Controlled)

Facility Locations		: Throughout China

Phone				: No listed number

Type Operations		: Smelters, Refineries

Tellurium Product Output	: Tellurium Dioxide

Commercial grade tellurium

Tellurium Source		: Copper; lead-zinc deposits throughout china

Comment			: The Chinese mineral and processing industry is in a

state of flux with some operations being privatized

and new non government controlled mines and smelters

being developed. The sources for the tellurium coming

from China today are overall not well documented.

Company			: Outokumpu Harjavalta Metals Oy

Facility Location		: Finland

Phone				: 358-2-626-5240

Type Operation		: Refinery

Tellurium Product Output	: Tellurium Dioxide

Tellurium Source		: Smelter products from copper and massive sulfide deposits 				  located throughout the world

Company			: Atomergic Chemetals Corp.

Facility Locations		: Farmingdale, New York

Moscow, Russia

Phone				: 516-694-9000

Type Operation 		: High Purity Metals Processing

Tellurium Product Output	: High Purity Tellurium

Tellurium Source		: Purchased from a variety of sources

Company			: All-Chemie Ltd.

Facility Location		: Mount Pleasant, South Carolina

Phone				: 843-884-4400

Type Operation		: High Purity Metals Procesing

Tellurium Product Output	: High Purity Tellurium

Tellurium Source		: Purchased from China and other sources

Company			: Cabot Performance Materials

Facility Location		: Boyertown, Pennsylvania

Phone				: 800-531-3676

Type Operation		: High Purity Metals Processing

Tellurium Product Output	: High Purity Tellurium

Tellurium Source		: Purchased from a variety of sources

Company			: Metaleurop Group

Facility	 Location		: Langelsheim, Germany

Phone				: 49-0-5326-507-410

Type Operation		: High Purity Metals Processing

Tellurium Product Output	: High Purity Tellurium

Tellurium Source		: Company controlled lead-zinc and copper deposits in

Europe

Company			:  Johnson Matthey Electronics/Alfa Aesar

Facility Locations		: Royston, United Kingdom

Brimsdown, United Kingdom

West Deptford, United States

Phone				: 702-896-3773

Type Operation		: High Purity Metals Processing

Tellurium Product Output	: High Purity Tellurium

Tellurium Source		: Purchased from a supplier

Company			: MCP Group

Facility Location		: United Kingdom

Phone				: 203-384-0331

Type Operation		: High Purity Metals Processing

Tellurium Product Output	: High Purity Tellurium

Tellurium Sources		: Open market purchase from a variety of sources

In the United States four companies purchase and resell tellurium with no additional processing and are listed as follows:

- Stanford Metals - San Mateo, California - 650-348-3482

- Goodfellow Corp. - Berwyn, Pennsylvania - 800-821-2870

- Atlantic Equipment Engineers - Bergenfield, New Jersey -

- 201-384-5606

- Belmont Metals - Brooklyn, New York - 718-342-4900

Production Capacity/Output Potential

The yearly production of tellurium ranged between 200 and 300 metric tons per year in 1997 and did not significantly change in 1998.(Bokovay, 1999). Currently, there is a substantial under utilization of capacity for the production of tellurium. If  there was sufficient demand approximately 50% of all tellurium contained in refined copper and lead could be recovered (the 50% recovery rate assumes process efficiencies over the standard 35 % recovery rates of 15 years ago). Shown in the tables to follow are tons of refined copper (Edelstein, 1997) and lead (Smith, 1997) for the world for 1997. Only countries with production over 100,000 tons of refined copper and 50,000 tons of refined lead are considered in this tellurium content and recovery estimate. Assuming an average tellurium content of 0.4 pounds per refined ton of copper (Davidson, Lankin, 1973) and 0.1 pounds per refined ton of lead (Jensen, 1985) plus a 50% recovery rate; an estimate can be made of total possible tellurium output per year.

Table 2

Total Possible

Tons Production	Contained	Contained Te	   Production/Yr.

Country	Electrolytic Copper	Te (Pounds)	Metric Tons	   (50% Rec.) M/Tons

United States		1,730,000	548,000	     249		  	124

Australia		   490,000	196,000	       89		 	  44

Canada		   654,700	261,880	     118			  59

Chile			2,511,000     1,004,400	     455		     	227

China			   414,000	165,600	       75			  37

Bulgaria		   100,000	  40,000	       18			    9

Indonesia		   529,000	211,648	       96			  48

Iran			   108,000	  43,200	       20			  10

Kazakhstan		   316,000	126,400	       57			  28

Mexico		   342,319	136,928	       62		     	  31

Mongolia		   125,300	  50,120	       23			  11

Paupa New Guinea	   111,700	  44,680	       20			  10

Peru			   391,265	  78,253	       35			  17

Poland			   414,000	165,600	       75			  37

Portugal		   106,500	  42,600	       19		               9

Russia			   505,000	202,000	       92			  46

South Africa		   186,000	  74,400	       34			  17

Zambia		   287,000	114,800	       52			  26

TOTAL	9,321,784			  1,589			795

Note: Tellurium tonnage calculations rounded.

Table 3

Total Possible

Tons Production	Contained	Contained Te	   Production/Yr.

Country	Refined Lead		Te (Pounds)	Metric Tons	   (50% Rec.) M/Tons

United States	    343,000		     34,300		16		8

Australia	    204,000		     20,400		  9		4.5

Belgium	      84,400		       8,440		  4		2

Bulgaria	      60,000		       6,000		  3		1.5

Canada	    162,000		     16,200		  7		3.5

China		    506,000		     50,600		23	          11.5

France		    115,000		     11,500		  5		2.5

Germany	      90,000		       9,000		  4		2

India		      69.000 		       6,900		  3		1.5

Italy		    115,000		     11,500		  5		2.5

Japan		    142,326		     14,233		  6		3

Kazakhstan	      65,000		       6,500		  3		1.5

Korea(North)	      75,000		       7,500		  3		1.5

Korea(Republic) 121,296		     12,130		  6		3

Mexico   	    168,164		     16,816		  8		4

Morocco	      64,200		       6,420		  3		1.5

Peru		      86,105		       8,611		  4		2

Poland		      50,000		       5,000		  2		1

Sweden	      50,000		       5,000		  2		1

United Kingdom  215,000		     21,524		11		5.5

TOTAL 	  2,785,491				          127	           64

Note: Tellurium tonnage calculations rounded.

The combined production potential from copper and lead refining totals 859 metric tons of tellurium for 1997. The actual production of tellurium for 1997 ranges between 200 and 300 metric tons (250 metric tons average). The capacity utilization is estimated to be 29%. It has been indicated that tellurium producers are capable of achieving higher recovery rates if the demand was present. Because of the proprietary nature of the various tellurium recovery processes it is not possible to determine the ultimate recovery efficiencies. With current refining practices for copper and lead coupled with generally consistent yearly metal product output, it is estimated that 859 metric tons to 1,716 metric tons of tellurium could be produced per year if demand warranted.

Solar Cell Industry/Tellurium Demand

Discussions were held with industry and government (NREL) officials concerning the current and estimated future demand of tellurium for use in solar panels. Because solar powered electrical generation is an emerging industry, future demand for tellurium in this application can only be roughly estimated. Considering current and expected efficiencies, it has been estimated that between 130 and 90 kg of tellurium is necessary for each Mw of generated power. Solar cell materials utilization efficiencies in which less tellurium per Mw is used has been factored into the demand estimate. Manufacturing efficiencies have also been considered in this consumption estimate. Current manufacturing techniques result in tellurium losses of around 40% (Zweibel, 1999). As manufacturing techniques improve these losses could drop to approximately 10%. For the purpose of this tellurium demand estimate, no recovery of tellurium lost in the manufacturing process or tellurium potentially recoverable from recycled solar panels is considered. Table 4 shows NREL’s tellurium demand estimate for a slow, medium and fast track solar generated electrical power. Table 5 shows the range of private industry estimates of potential tellurium demand.

Table 4

Development Rate

Year	Item				         Slow	        Medium	           Fast

2000

Mw PV Generated Power			1		5		10

Consumption of Te (Kg/Mw)		130		130		130

% Manufacturing Loss			40		40		40

Consumption of Te in Metric Tons		0.2		1		2

2005	Mw PV Generated Power			5		25		100

Consumption of Te (Kg/Mw)		130		130		130

% Manufacturing Loss			30		30		30

Consumption of Te in Metric Tons		1		5		19

2010	Mw PV Generated Power			25		100		1,000

Consumption of Te (Kg/Mw)		90		90		90

% Manufacturing Loss			20		20		20

Consumption of Te in Metric Tons		3		11		112

2020	Mw PV Generated Power			100		500		10,000

Consumption of Te (Kg/Mw)		90		90		90

% Manufacturing Loss			10		10		10

Consumption of Te  in Metric Tons	10		50		1,000

2030	Mw Pv Generated Power			500		2,000		30,000

Consumption of Te (Kg/Mw)		90		90		90

% Manufacturing Loss			10		10		10

Consumption of Te in Metric Tons		50		200		3,000

Note: Tellurium Tonnage calculations rounded

Table 5

Development Rate

Year	Item			   	          Slow   	       Medium	            Fast

2000	Mw Generated Power			2		5	  	10

Consumption of Te (Kg/Mw)		130		130		130

% Manufacturing Loss 			40		40		40

Consumption of  Te in Metric Tons	0.4		1		2

2005	Mw Generated Power			10		25		500

Consumption of Te (Kg/Mw)		130		130		130

% Manufacturing Loss			20		20		20

Consumption of Te in Metric Tons		1		3		82

2010	Mw Generated Power			25		40		3,000

Consumption of Te (Kg/Mw)		90		90		90

% Manufacturing Loss			10		10		10

Consumption of Te in Metric Tons		2.5		4		300

Note: Tellurium tonnage calculations rounded

If the most optimistic  PV industry development is considered, approximately 300 metric tons of tellurium per year will be required by the year 2010. With current minimum potential production capacity per year at 859 metric tons and non PV tellurium usage estimated at 400 metric tons per year there does not appear to be a short fall of production. The current tellurium sources will be adequate to meet increased demand. By the year 2020 (considering the most optimistic scenario) PV demand per year is estimated to be 1,000 metric tons and non PV tellurium demand is estimated to be 600 metric tons. The 1,600 total metric tons is greater than the 859 metric tons that can be produced at a 50% recovery rate but still within the possible tellurium recovery range (assuming greater recovery efficiencies) of 1,716 metric tons. Beyond the year 2020 new sources of  tellurium will have to be developed in order to meet any increasing demand by the PV industry.

Current Tellurium Resources

Currently, the source of over 80% of the tellurium produced comes from copper mining with most of the remainder coming from lead, lead-zinc and massive sulfide (copper-lead-zinc- precious metals) mining. These active mines world wide have the capability to produce at least 859 metric tons of tellurium per year and most likely an ability to produce closer to the maximum potential of 1,716 metric tons per year if demand was sufficient.

Under current practices, generally only enough tellurium is recovered from the mineral processing in a given year to meet demand. Tellurium not recovered is permanently lost. No economic process exists today for recovering the tellurium that is not initially captured.

Since the majority of tellurium produced comes from copper mining, an estimate of  total contained tellurium from this source has been made. It is believed that copper mining will continue to be the primary source of tellurium because of the in-place tellurium recovery infrastructure. Until tellurium demand exceeds the ability of the copper industry to recover tellurium, no significant changes in this business structure is expected to take place. Shown in Table 6 are the copper reserves estimated by countries. These copper reserve and reserve base estimates were derived from the 1996 USGS Mineral Commodity Summary for copper.

Table 6

Reserves			Reserve Base 1 			Total

Contained Cu	Contained Te     Contained Cu	Contained Te	Contained Te

Country	Metric Tons	Metric Tons	Metric Tons	Metric Tons 	Metric Tons

United States	41,000,000	8,200		82,000,000	16,000		24,200

Australia	  6,000,000	1,300		21,000,000	  4,200		  5,500

Canada		10,000,000	2,000		21,000,000            4,200		  6,200

Chile		80,000,000        16,000	            148,000,000	30,000		46,000

China		  3,000,000	   500		  7,000,000	  1,500		  2,000

Indonesia	10,000,000	2,000		14,000,000      	  2,700		  4,700

Kazakhstan	13,000,000	2,500		18,000,000	  3,600		  6,100

Peru		  6,000,000	   700		22,000,000	  2,200		  2,900

Philippines  	  6,000,000	1,300		10,000,000	  2,000		  3,300

Poland 		18,000,000	3,600		33,000,000	  6,500		10,100

Russia		18,000,000	3,600		27,000,000	  5,400		  9,000

Zaire		  9,000,000	1,800		27,000,000	  5,400		  7,200

Zambia		11,000,000	2,200		34,000,000	  6,200		  8,400

Other Countries	50,000,000        10,000		91,000,000	18,000		28,000

Total	            281,000,000        55,700	            549,000,000        107,900	            163,600

Note: All Figures Rounded

1. Reserve base is the in-place demonstrated (measured + indicated) resource from which reserves are estimated. At some time, depending upon economics, technological changes etc. some or all of the reserve base may be reclassified as reserves.

In 1997 close to 10 million metric tons of electrolytic copper was produced world wide. If the assumption is made that this yearly production rate will remain relatively steady for the foreseeable future, the currently defined copper reserve and associated tellurium will be mined out in approximately 28 years. Assuming conditions will warrant mining the reserve base an additional 55 years of copper/tellurium production is possible. Considering both the copper reserve and the reserve base, a total of 83 years of copper production at a rate of 10 million metric tons per year is considered a reasonable scenario. Since the contained tellurium can only be recovered by mining the copper, the yearly possible recovery of tellurium from copper refining (at current production rates) can approach 1,600 metric tons (see Table 2).

The reserve and reserve base for lead, lead-zinc and massive sulfide mineral deposits is similar to that of copper. If current production and refining rates are maintained for these reserves an additional 100 to 200 metric tons of tellurium could be most likely be produced.

As previously indicated in the Solar Cell Industry/Tellurium Demand section of this report, the combination of  solar cell industry demand and traditional (non solar) demand is estimated to result in a short fall of tellurium production by the year 2020.

Potential Tellurium Resources

A world wide review has been made of potential new sources of tellurium production and include the following categories of occurrences:

- Potential byproduct tellurium recovery from platinum group metals refining.

- Precious metals deposits with variable tellurium content.

- Primary tellurium deposits.

- Mineral Exploration Projects.

- Copper and other complex base metal occurrences not included in the “Current Tellurium Resources” section.

- Miscellaneous other mineral deposits with associated tellurium.

Overall, the current tellurium resources (copper, lead-zinc, etc.) do not overlap the potential tellurium resources. Because of the sparse amount of available data in China, Russia and the CIS countries; some deposits may be considered in both resource categories. Table 7 is a compilation of deposits, districts or regions located throughout the world that are known to contain some variable quantity of tellurium. The heading “Associated Minerals” indicates the dominate minerals in the  deposit or occurrence.  It must be emphasized that there is no implication of economic viability in any potential recovery of tellurium from the listed deposits. The listing of occurrences serves as an indicator of potential sources of tellurium that could be considered for detailed evaluation should demand necessitate development of new resources.

Table 7

Associated

Country	Deposit/District/Region		Location			Minerals

Argentina	Cerro Negro District		La Rioja Province		Ag

Armenia	Ankadzor			Northern Armenia		Cu, sulfides

Australia	Kalgoorlie			Western Australia		Gold

Kingsgate			New South Wales		Polymetallic

Lucky Draw-Bathurst District	New South Wales		Au, Bi

Table 7 Continued

Associated

Country	Deposit/District/Region		Location			Minerals

Australia	Mammoth Region		NW Queensland			Sulfides

Mount Morgan			Queensland			Polymetallic

Azerbaijan	Bitti Bulakh			Lesser Caucasus Mtns.		Cu, As

Kyzylbulag			Lesser Caucasus Mtns.		Cu

Paragachay			Lesser Caucasus Mtns.		Cu, Mo

Vejnalinsk			Lesser Caucasus Mtns.		Cu

Brazil		Cerrito do Pires			Rio Grande do Sul State		Au, As, Sb, Bi, Se

Corrego Criminoso District	Goias State			Au

Bulgaria	Kamilski Dol Region		Eastern Rhodopes		Bi

Madan District			Southern Bulgaria?		Polymetallic

Rossen District			Carpatho-Balken Region		Cu, Mo

Canada		Atlin District			British Columbia			Polymetallic

Beaver Dam			Nova Scotia			Ag, W

Bigstone Bay Region		Ontario				Au, Ag

Donaldson West			Quebec				Ni, Cu

Flin Flon Area			Manitoba			Polymetallic

Geordie Lake			Ontario				Sulfides

Great Bear Lake			Northwest Territories		Ag

Kirkland Lake District		Ontario				Au, Ag

Red Lake			Ontario				Au, Ag

Revelstoke Area			British Columbia			Polymetallic

Rouyn District			Quebec				Polymetallic

Sudbury District			Ontario				Polymetallic

Chile		El Indio				NE Chile			Au, Ag, Cu

China		Antimony Ore Belt		S. Anhui Province		Sb

Chinkuashih			Northern China?			Sulfides

Coastal Region			Southern China			Au, Ag

Dashuigou			W. Sichuan (Shimian)		Primary Te Dep.

Guantian			Guangdong Province		Cu, Sulfides

Guilaizhuang			Shandong (Pingyi County)	Au

Hadamengou(Wulasham Dist.)	Inner Mongolia			Au, Sulfides

Heti				Northern China?			Se

Jinchangyu			Inner Mongolia			Mo, Ag, Sb, Py.

Li Shui				Jiangsu Province			Au

Qibaoshan			Hunan Province			Polymetallic

Xiaoyinpan			Inner Mongolia			Au, Sulfides

Xiong’er			Hunan Province			Sulfides

Zhongshangou			Inner Mongolia			Au, Bi, Sulfides

Congo (Zaire)	Kinshasa			Western Congo			Cu

Table 7 Continued

Associated

Country	Deposit/District/Region		Location			Minerals

Czech Republic	Bohemian Massif		W. Czech Republic		Polymetallic

Petkovic				Orahovac Massif			Co, Ni, Cu

Smolotely			Pribram Area			Bi

Egypt		Eastern Desert Region		Eastern Egypt			Cu, PGM’s

Ethiopia	Legi Dembi			Central Ethiopia			Au/Ag/Pb/Zn/Cu

Federated

States of

Micronesia	Yap 				Yap Island			Au, Ag

Fiji		Emperor			Viti Levu Island			Au, Ag

Vahatala			Viti Levu Island			Au, Ag

Vatukoula District		Viti Levu Island			Au, Ag

Finland		Hattu Schist Belt			Eastern Finland			Au

Ramepuro			Eastern Finland			Au

France		Argentera			French-Italian Alps		Polymetallic

Vialas				SE France?			Polymetallic

Georgia		Misc. Deposits			Country Wide			Sulfides

Germany	Central Rhodope Massif		Eastern Germany		Pb, Zn

Kupferschiefer			Harz Area			Polymetallic

Greece		Hellenic Region			Central Greece			Polymetallic

Macedonia			Northern Greece			Polymetallic

Honduras	Misc. Districts			Country Wide			Polymetallic

India		Bhandara			Maharashtra State		Polymetalic

Khetri				Northern India?			Cu

Nagpur				Maharashtra State		Polymetallic

Rakha				Northern India?			Polymetallic

Indonesia	Misc Districts			Irian Jaya			Polymetallic

Salida				Sumatra?			Au, Ag

Italy		Several Districts			Sardinia				Polymetallic

Japan		Date Mine			Hokkaido, Island			Au

Honshu				Honshu Island			Cu/Pb/Zn/Fe

Iriki Mine			Kagoshima Pref.			Cu/Ag/Mo/Sb

Kawazu Mine			Shizuoka Pref.			NK

Kinkie Mine			Hokkaido, Island			NK

Table 7 Continued

Associated

Country	Deposit/District/Region		Location			Minerals

Japan		Kobetsuzawa Mine		Sapporo, Hokkaido		Nk

Kushikino Mine			Kagoshima Pref.			Ag, Au

Nojiri Mine			Iwate Pref.			Au, Ag

Okuyama Mine			Izu Peninsula			NK

Suttsu Mine			Hokkaido, Island			Ag/Bi/Se/S

Suzaki Mine			Izu Peninsula			Au

Takeno Mine			Hokkaido, Island			NK

Teine Mine			Hokkaido, Island			NK

Tsuomo Mine			SW Japan			NK

Kazakhstan	Aktogay				E. Central Kazakhstan		NK

Asubulak			E. Central Kazakhstan		NK

Aydaryl				E. Central Kazakhstan		NK

Boshchekal			E. Central Kazakhstan		Cu, Mo

Copper-Gold Deposits		Country Wide			Cu, Au

Kalba				Altai Mtns.			Sulfides

Kosmurun			Central Kazakhstan		Cu, S, Se

Ognevka			E. Central Kazakhstan		NK

Ridder-Sokol			Altai Mtns.			Se

Rudny				Altai Mtns.			Polymetallics, Se

Tishin				Altai Mtns.			Se

Zyranovsk			Altai Mtns.			Ag

Korea

(Republic Of)	Imcheon Mine			Western Korea			Au, Ag

Tongyoung			Western Korea			Au, Ag

Malagasy Rep.	Madagascar -Pegmitite		Madagascar Island		Cu, Bi

Mexico		Bambolla Mine			Moctezuma Dist., Sonora		Au, Ag

Francisco I. Madero		Zacatecas State			Polymetallic

Moctezuma District		Sonora State			Au, Ag

Sahuayacan			Sonora State			Au, Ag

San Francisco			Sonora State			Au, Ag

San Miguel			Moctezuma Dist., Sonora		Au, Ag

Sierra Blanca			Moctezuma Dist., Sonora		Au, Ag

Sierra La Cieneguita		Sonora State			Polymetallic

Morocco	Alma-Idouska			Western Anti-Atlas		Cu

Norway		Kells Creek			Oppdal-Central Norway		Polymetallic

Middavarre			Northern Norway		Cu

North Troms			Northern Norway		Polymetallic

Vaddas Rieppe			Northern Norway		Polymetallic

Palau		Palau District			Palau Island			Au, Ag

Table 7 Continued

Associated

Country	Deposit/District/Region		Location			Minerals

Papua New

Guinea		Mt. Kare			Central Highlands		Au, Ag

Porgera				Central Highlands		Au, Ag, Cu

Peru		Ilo Region			S. Central Peru			Cu

Oroya Region			N. Central Peru			Cu

Yauricocha			Central Peru			Au, Ag

Philippines	Baguio District			Negros Occidental?		Au, Ag, Cu

Bulawan			Negros Occidental		Au, Ag

Exciban				Negros Occidental?		Au, Ag, Cu

Poland		Fore-Sudetic Deposit		Lubin Area			Cu

Romania	Deva Area			Transylvania			Polymetallic

Musariu Brad Region		Metaliferi Mtns.			Au, Ag

Nagyag				Central Romania?		Au, Ag

Scaramb Deposit			S. Apuseni Mtns.			Au, Ag

Scaramb Region			S. Apuseni Mtns.			Au, Ag

Russia		Allarechenskiy District		Kola Peninsula			Sulfides

Ardino				Urals?				Ag, Bi

Boshchekul			Urals?				Cu, Mo, Se

Caucasus Region			Southern Russia			Polymetallic

Chatkal Region			Kurama Mtns.			Polymetallic

Eastern Slope Region		Southern Urals			Polymetallic

Eastern Transbaikalia		Urals?				Bi, Au

Evanskoe			Eastern Siberia			NK

Filizchay District			Urals?				NK

Kamchatka District		Eastern Russia			Au

Kremnica District		Urals?				NK

Kuekvun			Eastern Siberia			NK

Kuranakh			Western Russia?			Au, Mn

Makanskoge			Southern Urals			Polymetallic

Monchegorsk District		Northern Russia?			Cu, Ni, Ag, Pb			Northern Caucasus Region	Southern Russia			Cu, Pb, Zn

North Onki			Murmansk Reion			NK

Noril’sk Region			Norhtern Russia			PGM’s, Ni, Cu

Oktyabar			Noril’sk				PGM’s Ni, Cu, Ag

Pelvuntykoinen			Eastern Siberia			NK

Prasolovskoye			Kunashir Island			Au, Ag, Se

Priorsk				Urals?				Se

Sevkar				Sarigyukh Region		Mn

Shkolny				Eastern Siberia			NK

Sultanskoe			Caucasus Region			NK

Talnakh				Noril’sk				Cu, Ni, Sulfides

Transcaucasia Region		Southern Russia			Au

Tyrnyauz			Southern Russia?			Mo, Au, Bi

Table 7 Continued

Associated

Country	Deposit/District/Region		Location			Minerals

Russia		Urals Region			Centeral Urals Mtns.		Au

Uzelginsk			Urals?				Sulfides

Verkhne-Kolyma Region		Northeast Russia			Sulfides, Au

Saudi Arabia	Mahd Adh Dhahab		S. Central Saudi Arabia		Au, Ag

Scotland	Ratagan				Northwest Scotland		Au, Ag

Slovakia	Kramnica			Central Slovakia?		Polymetallic

Stivnica-Hodrusa District		Central Slovakia			Polymetallic

Zlata Bana Region		Slanske Vrchy Mtns.		Polymetallic

South Africa	Merenski Reef Region		Bushveld Complex-Transvaal	PGM’s

Spain		Rodalquilar			Southern Spain			Au, Ag

Sweden		Ballek				Northern Sweden		Polymetallic

Boliden District			North Central Sweden		Polymetallic

Tajikistan	Chashmaniat			Central Tajikistan		Bi

Maykura			Central Tajikistan		Sn, W

Turkey		Murgel District			NE Turkey			Cu

Ukraine	Talnov				Western Ukraine?		As, Bi

United States	American Eagle			Idaho, Cty., ID			Au, Ag

Anacortes Group			Okanogan Cty., WA		Au, Ag

Aniak-Bethl			Alaska				Au, Ag

Badger King			Blaine Cty., MT			Au, Ag

Bear Creek			San Juan Cty., CO		Au, Ag

Bessie G			Montezuma Cty., CO		Au, Ag

Big Grassy Group		Fergus Cty., MT			Au, Ag

Black Mtn.-Miners Gulch		Hill Cty., MT			NK

Bonanza Queen			Snohomish Cty., WA		Au, Ag

Boulder Cty., Deposits		Boulder Cty., CO		Au, Ag

Boundary-Red Mtn.		Whatcom Cty., WA		Au, Ag

Brewer				Chesterfield Cty., SC		Au, Ag

Bridger Mine			Carbon Cty., WY		Au, Ag

Bright Star			Custer Cty., ID			Au, Ag

Bunker Hill			Josephine Cty., OR		Au, Ag

Calaveras District		Calaveras Cty., CA		Au, Ag

Circle District			Alaska				Au, Ag

Cornucopia			Baker Cty., OR			Au, Ag

Cortez-Buckhorn			Eureka Cty., NV			Au, Ag

Cripple Creek District		Teller Cty., CO			Au, Ag

Ely District			White Pine Cty., NV		Au, Ag

Empire District			Clear Creek Cty., CO		Au, Ag

Table 7 Continued

Associated

Country	Deposit/District/Region		Location			Minerals

United States	Eureka-Hamburg			Eureka Cty., NV			Au, Ag

Eureka Mine			Josephine Cty., OR		Au, Ag

Gies Mine			Fergus Cty., MT			Au, Ag, V

Gold Hill			Boulder Cty., CO		Au, Ag

Gold Hill			Yavapai Cty., AZ		Au, Ag

Gold King			White Pine Cty., NV		Au, Ag

Gold Mint			Lyon Cty., NV			Au, Ag

Golden Chariot			San Diego Cty., CA		Au, Ag

Golden Circle			Hancock Cty., ME		NK

Golden Quail			San Bernardino Cty., CA		Polymetallic

Golden Shear			La Plata Cty., CO		Au, Ag

Golden Sunlight			Jefferson Cty., MT		Au, Ag

Goldfield District		Nye Cty., Nevada		Au, Ag

Grandview			Coconino Cty., AZ		Au, Ag

Green Glory			La Paz Cty., AZ			NK

Greenback			Pima Cty., AZ			NK

Griffith				Clear Creek Cty., CO		Au, Ag

Harrison			Deerlodge Cty., MT		Au, Ag

Hazel Mine Group		Montezuma Cty., CO		Au, Ag

Holbrook			Idaho, Cty., ID			Au, Ag

Horseshoe Lode			Douglas Cty., OR		Au, Ag

Idaho Springs District		Clear Creek Cty., CO		Au, Ag

Iditarod				Alaska				NK

Independence			Colfax Cty., NM			Au, Ag

Irwin Mo Prospect		Jackson Cty., OR			Mo

Jamestown			Tuolumne Cty., CA		Au, Ag

Kendall District			Fergus Cty., MT			Au, Ag

Klondyke			Saguache Cty., CO		Au, Ag

Kensington Mine		Alaska				Au, Ag

La Plata Mtns.			La Plata Cty., CO		Au, Ag

Lake City District		Hinsdale Cty., CO		Au, Ag

Leadville District		Lake Cty., CO			Au, Ag

Little Johnny			Jackson Cty., OR			Au, Ag

Lone Pine			Grant Cty, NM			Au, Ag

Lone Pine/Wilcox		Catron Cty., NM			Au, Ag

Lucky Queen			Josephine Cty., OR		Au, Ag

Mayflower Mine			Madison Cty., MT		Au, Ag

McPherson Prospect		Josephine Cty., OR		Au, Ag

Mill Canyon			Eureka/Lander Ctys., NV		Au, Ag

Molybdenum Prospect		Jackson Cty., OR			Mo

Neglected			La Plata Cty., CO		Au, Ag

North Moccasin District		Fergus Cty., MT			Au, Ag

North Star			Alaska				Au, Ag

Nuka Bay-Kenai			Alaska				Au, Ag

Opp Mine			Jackson Cty., OR			NK

Organ District			Dona Ana Cty., NM		Au, Ag, Cu

Oriole Mine			Josephine Cty., OR		Au, Ag

Petsite				Idaho Cty., ID			Au, Ag

Table 7 Continued

Associated

Country	Deposit/District/Region		Location			Minerals

United States	Plainview			Pershing Cty., NV		Au, Ag

Porcupine Mine			Josephine Cty., OR		Au, Ag

Red Top Mine			Eureka Cty., NV			Au, Ag

Rex-Gold Hill			Boulder Cty., CO		Au, Ag

San Juan Nat. Forest		San Juan Cty., CO		Au, Ag

Silver City NW			Grant Cty., NM			NK

Silver Drip			Catron Cty., NM			Au, Ag

Spotted Horse			Fergus Cty., MT			Au, Ag

St. Kevin			Boulder Cty., CO		Au, Ag

Staten Island			Richmond Cty., NY		NK

Stillwater			Stillwater Cty., MT		PGM’s

Sugarloaf			Boulder Cty., CO		Au, Ag

Surprise Mine			Blaine Cty., MT			Au, Ag

Telluride District			San Miguel Cty., CO		Au, Ag

Tellurium Group			Valley Cty., ID			Au, Ag

Tombstone			Cochise Cty., AZ			Au, Ag

Tuluksak-Kuskokwim		Alaska				Au, Ag

Tungsten Reef			Cochise Cty., Az			W, Au, Ag

Valdez Creek			Alaska				Au, Ag

Virgin Islands Group		Virgin Islands			NK

Voght				Alaska				Au, Ag

Vulcan				Gunnison Cty., CO		Polymetallic

Washington			Idaho Cty., ID			Au, Ag

Watchtower			Lemhi Cty, ID			Au, Ag

Wenatchee District		Chelan Cty., WA			Au, Ag

Whitehall			Virginia				Au, Bi

Winters Prospect			Josephine Cty., OR		Au, Ag

Zortman-Landusky		Phillips Cty., MT		Au, Ag

Uzbekistan	Kayragach			Eastern Uzbekistan		Cu

Kochibulak			Eastern Uzbekistan?		Polymetallic

Kurama				Eastern Uzbekistan?		Au

Molodezh			Chatkal Subdivision		Polymetallic

Rabindzhan			Western Uzbekistan		W, Se

Zambia		Zambian Copper Belt		Eastern Zambia			Cu

Zimbabwe	Hartley				Great Dyke-Central Zimbabwe	PGM’s

Widespread Gold Districts	Countrywide			Au, Ag

Market Analysis

Demand Increase (2X-3X)

One of the major concerns of the PV industry is the availability of product if demand for tellurium should increase two to three times over the current demand of approximately 300 metric tons per year to 600 to 900 metric tons per year. As previously indicated, the current capacity utilization (production capacity actually being utilized) is approximately 29%. Increases in demand of two to three times would result in production of tellurium dioxide approaching maximum capacity of near 100%.

Currently, there are at least nine facilities located around the world that have the capacity to produce high purity (99.99+) tellurium. Additionally, there are at least five other facilities with the capability of producing commercial grade (99.7%) tellurium. It is understood that most of these operations are working at less than 50% capacity. (Mike Weddle, Phelps Dodge) If demand should increase at the rates indicated by the PV industry and NREL (see Tables 4 and 5) these facilities should be able to scale up and/or upgrade their facilities to meet this increased 3 to 3 times demand at least for the next twenty years.

The tellurium consumption rates in sectors not related to the PV industry are expected to be flat with little growth. Since the early 1990’s tellurium consumption in these sectors has actually dropped. If tellurium supplies become tight or prices increase there are at least two more readily available and cheaper substitutes (selenium and bismuth) that could be used in many applications. It is not anticipated that these other use sectors will play any significant role in future demand requirements for tellurium.

New Market Entrants

If Prices or demand increase it is likely that new market entrants will consist of the following two types of organizations:

- Metal mining, smelting and or refining companies (copper, PGM’s, Lead-Zinc, massive sulfides etc.)

- Companies currently recovering lower grades of tellurium that could expand processing activities into higher grade production lines i.e. Commercial Grade tellurium producers expanding into High Purity tellurium production.

If tellurium demand by the PV industry progresses as anticipated in the fast track scenarios shown in Tables 4 and 5; organizations that do not have producing metal mines or smelters will not attempt to enter the tellurium recovery market within the next 20 years. Scattered throughout the world are at least 29 companies who either produce or upgrade and refine tellurium. New market entrants would have great difficulty establishing a viable

business with excess production capacity of at least 70% overshadowing the market.

Tellurium Market Stability

In discussions with contacts in the PV industry it was learned that the cost of the tellurium contained in a typical PV panel could represent approximately 3% of the total manufacturing cost (Personal Communications: BP Solar, First Solar, ITN; 1999). The PV industry personnel indicated that  tellurium price increases of 2 to 4 times would not have a significant impact on manufacturing plans. If prices increased 8 times tellurium content would represent approximately 24% of the manufacturing cost of a typical PV panel. This magnitude of price increase would significantly destabilize the PV industry. It is unlikely, however, that prices could rise to these levels within the next 20 years because of the under utilization of current capacity. Dramatic price increases could also be damped by the following factors:

- Increased recovery from of tellurium from known metal deposit resources.

- Development of primary tellurium deposits that are known to exist in China and Mexico (see Table 7).

- Price driven exploration and development efforts for discovery of new tellurium resources.

The mining industry has traditionally responded to mineral price increases by undertaking aggressive worldwide exploration and development efforts. To date these efforts have always been successful and commodity prices have responded to these successes by either stabilizing or dropping. In mining, as in other industries, the marketplace responds to excess demand by providing an additional supply (in this case new deposit discoveries and or technological breakthroughs).

EXECUTIVE SUMMARY

A study was commissioned by the National Renewable Energy Laboratory (NREL) to assess the world wide tellurium market and its ability to supply product to the emerging PV industry. Tellurium, discovered in Transylvania, Romania in 1782, has unique thermoelectric characteristics (Seebeck effect, Peltier effect) that make it an invaluable component for the process of generating solar electric power.

The majority (80%+) of tellurium produced today comes from the processing of anode rich slimes that accumulate during the electrolytic refining of copper. Other, less important sources of  byproduct tellurium include gold milling, lead refining, and platinum group metals (PGM’s) refining. There are no primary tellurium deposits currently in production.

There are 29 companies world wide that recover and or process tellurium into three types of products:

- Tellurium Dioxides - Sludges and precipitates of variable purity of around 35%.

- Commercial Grade Tellurium - 99.7% average purity

- High Purity Tellurium - 99.9% to 99.9999%+

In 1997 the total world production of tellurium ranged between 200 and 300 metric tons;  the great majority of which was a byproduct of electrolytic copper refining. Copper, lead, lead-zinc, PGM’s and other related types of deposits contain trace elements of tellurium that can be recovered if there is sufficient demand. Average yearly world wide output of refined copper and lead totals approximately 12 million metric tons. Approximately 1,700 tons of tellurium is accumulated in this refining process of which only a very small portion is recovered in any given year. Tellurium not recovered in the process is lost to the market. Tellurium recovery technology is proprietary but historic data suggests recoveries of 50 % and more of tellurium per refined ton of metal is viable.

Tellurium demand by the PV industry has been estimated at a slow, medium and fast track development rate for the next 30 years. Considering the most optimistic development rate, the PV industry demand will not exceed current possible tellurium output until after the year 2020. After the year 2020 additional sources of tellurium would have to be developed to meet demand. Review of  literature sources indicates a number of  mineral occurrences world wide (including undeveloped primary tellurium deposits) that contain variable amounts of tellurium that could be considered for development to meet increased demand.

The current tellurium capacity utilization (current capacity actually being used) is approximately 29%. With this magnitude of capacity overhang on the market, the increasing demand by the PV industry over the next two decades will have essentially no impact on available supply.

Currently there are 29 companies located throughout the world who either produce, upgrade and refine tellurium. Considering the excess capacity situation in the market it is anticipated that few, if any, new market entrants will become involved the tellurium business. What is expected, however, that if demand begins to increase, those companies in the business could expand their product lines i.e. some of the Commercial Grade tellurium producers expanding into High Purity tellurium production.

Price increases for tellurium in the range of two to four times will not have an impact on the PV industry panel manufacturing programs. The tellurium contained in a PV panel represents approximately 3% of the total cost of that panel. If  tellurium prices were to increase 8 times, tellurium would represent 24% of the cost of a panel and would significantly impact manufacturing costs and destabilize the market. A mitigating factor to this price increase scenario is that the market (in this case the mining industry) would respond to rising prices by finding and producing more tellurium deposits and/or by developing technological breakthroughs that would allow for better recovery of tellurium in current deposits. Historically, in the mining industry has been successful in responding to tight mineral supply situations by expanding production which have resulted in commodity prices stabilizing or dropping.

CONCLUSIONS

A study has been completed of the world wide tellurium market and its ability to respond to the product demand requirements of the emerging PV industry. The conclusions reached upon completion of this study indicate the following:

- The copper industry is the source of the majority (80%+) of the tellurium produced currently.

- The copper industry will continue to be the source of the majority of the world’s tellurium supply for decades to come.

- Because of lack of demand there is currently an excess tellurium capacity overhang on the market of approximately 70%.

- Assuming the most optimistic development scenario by the PV industry, it will take approximately 20 years or more for tellurium demand to be in balance with supply.

- Worldwide, there are numerous tellurium enriched mineral occurrences that could be developed to meet demand in excess of existing supplies.

- New market entrants are unlikely as long as there is excess capacity in the tellurium market.

- If tellurium demand and prices increase, existing tellurium producers would be likely to increase production and/or expand product lines.

- Tellurium price increases of 2 to 4 times would most likely not impact the PV panel manufacturing plans of the industry.

- Tellurium price increases of 8 times or more would impact the economics of PV panel manufacturing and destabilize the industry.

- The mining industry response to tight tellurium supplies would be increased exploration and development which historically has resulted price stabilization or decline.

ASSESSMENT OF CRITICAL

THIN FILM RESOURCES

TELLURIUM

RAF-9-29609

Prepared By:

Jim Guilinger

World Industrial Minerals

April, 1999

TABLE OF CONTENTS

EXECUTIVE SUMMARY AND CONCLUSIONS

Introduction and Background……………………………………Section 1

Production Methods………………………………………………Section 2

Current Suppliers…………………………………………………Section 3

Production Capacity, Demand, Resources………………………Section 4

Market Analysis…………………………………………………..	Section 5

Tellurium Report Bibliography And Phone Log..………………Appendix

Selected Bibliography…………………………………………….	Appendix

Mr. Danny Cunningham

BP Solar

2300 N. Watney Way

Fairfield, CA 94533				May 20, 1999

Dear Mr. Cunningham:

Per the instructions of Ken Zweibel at NREL,  I have enclosed a copy of a report prepared by World Industrial Minerals on Cd-Te resources titled “Assessment of Critical Thin Film Resources, Cad-Telluride (Cd/Te)”.

If you have any questions, please do not hesitate to contact me.

Sincerely,

Jim Guilinger