STATEMENT OF WORK

The Contractor shall construct a concrete building foundation per the following specifications at the Texas Mid-Coast National Wildlife Refuge Complex:

- Form 40’ 3” x 60’ -3” Metal Building Foundation

- 1’ x 2’ beams (all perimeter and two 40’ interior beam.

- 6- #5 bar, 3 bar at top and 3 bars at bottom #3 ties at 2’ 0” O.C.

- 5 ½” thick slab.

- Slad Steel #4 bars at 16” o.c.e.w.

- Pour 40’ -3 x 60’ – 3” foundation

- Spray curing compound.

- Wreck form after 24 hours.

- Grade dirt around foundation and general clean up.