Proposal for analysis of base fluid 10-day and field drilling mud 4-day Leptocheirus plumulosus sediment toxicity testing results – A new interval arithmetic approach to demonstrate compliance

M.L. Cano 1 and J.T. Louallen 2

1 Shell Chemical Company LP, 2 Shell Global Solutions

##### Prepared for the Offshore Operators Committee

# Summary

This document contains a proposed approach for handling the uncertainty that is present in the computation used to determine compliance with USEPA Gulf of Mexico (GOM) guidelines for sediment toxicity of base fluids used for drilling muds and sediment toxicity of field drilling muds.  The proposed approach is based on interval arithmetic.  The 95% confidence intervals of the test and reference (base fluid or field drilling mud) sediment toxicity test results are used to develop a “compliance” interval for the ratio of the test results.  Fluids and muds might be assessed as “passing” their respective sediment toxicity tests if the lower bound of this compliance interval is  1.0.  Example calculations are shown for some representative base fluid sediment toxicity tests and some field drilling mud sediment toxicity tests.

# Introduction

The current USEPA Gulf of Mexico (GOM) guidelines (USEPA, 2001) and Region 6 General Permit (USEPA, 2002) for demonstration of base fluid sediment toxicity and field drilling mud sediment toxicity compliance involve the use of a K-factor to account for uncertainty in the toxicity tests.  The compliance tests use a ratio of the toxicity of a reference fluid or reference mud (C1618 internal olefin (IO) as base fluid or as basis for reference mud) to the toxicity of the tested fluid or mud.  A fluid or mud is shown to be compliant if the ratio is  1.0.  It is currently believed that when the new GOM General Permit is issued, usage of the K-factor will be eliminated.  This will result in no specific mechanism to address uncertainty in the sediment toxicity tests.  Typical results for the tests include a reported LC50 value for toxicity to Leptocheirus plumulosus with a corresponding statistical confidence interval (usually 95%).

Some level of uncertainty in experimental measurements is always present (including the LC50 results of the 10-day base fluid sediment toxicity test and the 4-day field mud sediment toxicity test).  There are many potential sources of uncertainty, which can include limitations of instrumentation and variability in test samples and procedures.  When computations are carried out with measured numbers that have associated uncertainty, there are statistical techniques that are used to recognize this uncertainty and reflect it in the computed result.  This is typically known as propagation of errors.  Standard mathematical formulas are available to determine error propagation (Taylor, 1982).  However, these techniques usually only have applications in situations when errors are normally distributed (negative and positive errors are equal).  When errors are not normally distributed, the traditional propagation of error formulas do not apply.

For non-normally distributed errors, an alternative way to account for uncertainty is available.  This approach is based on the concept of interval arithmetic (Hayes, 2003).  This document contains a proposal for an alternative mechanism to recognize and account for the uncertainty in sediment toxicity testing results (using the concept of interval arithmetic) when computing a ratio that is to be used for compliance.  This concept and its application are discussed below.

# Interval arithmetic to address uncertainty

One approach to dealing with numerical errors is a technique called interval arithmetic. It does nothing directly to improve the accuracy of calculations, but for every number it provides a certificate of accuracy—or the lack of it. The result of an ordinary (non-interval) computation is a single number, a point on the real number line, which lies at some unknown distance from the true answer. An interval computation yields a pair of numbers, an upper and a lower bound, which are guaranteed to enclose the exact answer.  While the resulting interval does not pinpoint the truth, it does point out how much is actually known.  Work in the area of computer programming uses interval arithmetic to manage and mitigate the effects of round-off error in computational algorithms.

A single formula extends the definition of the four standard arithmetic operations to intervals. If  represents any of the operations +, –,  or ÷, then the corresponding interval operation is defined as:

[x1, x2]  [y1, y2] = [min(x1  y1, x1  y2, x2  y1, x2  y2), max(x1  y1, x1  y2, x2  y1, x2  y2)]	(1)

Thus, one can compute the four possible combinations and choose whichever of these results is the smallest as the new lower bound, and likewise take the largest as the new upper bound.  This interval represents the combined uncertainty in the computation.  If both intervals have the same uncertainty, the computed interval will have the exact same uncertainty.

For the example of a ratio, the interval calculation is as follows:

[x1,x2][y1,y2]=minx1y1,x1y2,x2y1,x2y2,maxx1y1,x1y2,x2y1,x2y2	(2)

In the case of the 10-day Leptocheirus sediment toxicity testing results that are compared via a ratio (toxicity of C1618 reference fluid/toxicity of test fluid), if both results have a 95% confidence level, the computed ratio interval will be based on these 95% confidence levels.  This calculated interval would represent the “compliance” interval for the ratio of the two toxicity test results.  A similar approach could be used to evaluate the results from the 4-day field mud sediment toxicity tests.  This computed interval could be used to assess compliance.

# Example results for Base Fluids

The proposed interval approach has been used (Equation (2)) to assess some specific examples of sediment toxicity tests for base fluids.  These four specific examples are shown in Table 1 below.

Table 1.  Example results from sediment toxicity tests.

Results are shown for two different products.  Three test results are shown for Product 1.  In this example, Product 1 “passes” the compliance test (Ratio  1) two times and fails once.  Product 2 fails the test with a Ratio of 7.8.  Using interval arithmetic to compute the compliance intervals shows the following compliance intervals for the ratios for Product 1:  [0.1, 1.5], [0.5, 0.8], and [0.9, 2.5].  In all cases for Product 1, the required passing value (1.0) falls within the compliance interval.  For Product 2, the compliance interval for the ratio is [5.5, 11,0].  In this case, the required passing value (1.0) falls outside of the compliance interval.

Example results for field muds

Recently, the inter-industry Synthetic Based Mud (SBM) Toxicity Working Group assembled a database of results from ~ 500 field mud sediment toxicity tests.  This database is documented in Louallen et al. (2004), and represents results from field mud 4-day sediment toxicity testing from 2002 and 2003 conducted in 4 different laboratories.  Louallen et al. (2004) carried out an analysis of these data to study the appropriateness of the current K factor for these 4-day sediment toxicity tests (0.25) and concluded that the value of 0.25 was a statistically appropriate value based on the field mud sediment toxicity database.

This same database was used to assess the effects of applying the proposed interval arithmetic approach described herein.  These results are shown in Table 2.

Table 2:  Application of interval arithmetic approach to determine compliance of field mud samples from industrial database.  A total of 409 samples were considered to have adequate data for the analysis (needed to have upper and lower confidence interval about each experimental result).  Results are presented as probabilities ( 1.0).

The data in Table 2 indicate that the interval approach has an effect similar to the use of a K factor equal to 0.25.  Overall, the use of the interval approach resulted in a compliance rate of 96% compared to 95% with the use of a 0.25 K factor and 85% with the use of a 0 K factor (i.e., no K factor).  This analysis shows that the interval approach can adequately account for the variability that is (1) observed in the sediment toxicity tests and (2) propagated in the calculation of the sediment toxicity ratio.

Proposed passing criterion

Given the interval arithmetic approach discussed above, one proposed criterion for passing the 10-day base fluid sediment toxicity test or the 4-day field mud sediment toxicity test could be:

lower bound of compliance interval  1.0 	(3)

In other words, a base fluid or field mud would pass the ecotoxicity test if the lower bound of the compliance interval were  1.0.  This approach would provide some statistical significance to the passing criterion.  If Equation (3) is applied as a passing criterion for the four examples shown in Table 1, Product 1 “passes” in all cases while Product 2 “fails”.

It is possible to consider other passing criteria with this approach.  These may be based on different confidence intervals.  In addition, it may also be possible to define some criteria that would help to define a “good” test as opposed to a “bad” test.  These criteria could be used to objectively drop out (or not consider) some specific tests because of high test variability.  This would be manifested in an abnormally large compliance interval for the ratio.

# Conclusions and recommendations

A new proposed approach for calculating toxicity ratios to determine compliance with the 10-day base fluid sediment toxicity test or the 4-day field drilling mud sediment toxicity test has been presented.  This approach uses interval arithmetic to develop a “compliance” interval for the ratio that is based on the 95% confidence intervals of the specific toxicity tests used to develop the ratio.  A new proposed pass/fail criterion has also been presented.  To further develop and refine this approach, it is recommended that this approach be applied to a larger data set of test results for base fluid sediment toxicity in order to determine the robustness of the proposed method and to determine additional criteria that may be used to help define the “quality” of a set of test results.  Application of the approach to a large database of 4-day mud sediment toxicity tests showed that the method could effectively manage the variability observed in these test results.

# References

Hayes, B.  2003.  A lucid interval.  American Scientist.  Vol. 91. No. 6, pp. 484-488.

Louallen, J.T., D.C.L. Wong, and P.B. Dorn.  2004.  Statistical analysis of the sediment toxicity ratio for synthetic-based drilling muds.  Shell Global Solutions.

Taylor, J.R.  1982.  An introduction to error analysis.  The study of uncertainties in physical measurements.  University Science Books, Mill Valley, CA.

USEPA. 2001. 40 CFR Parts 9 and 435 [FRL-6929-8] RIN 2040-AD14 Effluent Limitations Guidelines and New Source Performance Standards for the Oil and Gas Extraction Point Source Category; OMB Approval Under the Paperwork Reduction Act: Technical Amendment. 66 Federal Register 14. January 22, 6858.  Website address:  http://www.epa.gov/waterscience/guide/sbf/final/finalsbf.pdf.

USEPA.  2002.  Final NPDES General Permit for New and Existing Sources and New Dischargers in the Offshore Subcategory of the Oil and Gas Extraction Category for the Western Portion of the Outer Continental Shelf of the Gulf of Mexico (GMG290000).