Rev 10/16/06

IH60500 Content of Attachment 9.2 for editing:

| Item Name | Description / Example of appropriate level of documentation |
| --- | --- |
| 1. Assign the Project Number (completed by Sampler) |  |
| Survey Title | One line description of the survey event that identities the hazard and location. Example: Lead surface wipe sampling in Building 120 |
| Survey # | Unique identifier number used by sampling organization to track field sampling work. |
| 2. Describe the Event (completed by Sampler) |  |
| First & Last Name of Employee(s) & Life Numbers | Identify person sampled and other potentially exposed personnel or personnel represented by sampling data (i.e. similar exposure group). Example: This sampling was done on plumber John Doe (BNL 9999). Also represented on this day by the results was his companion plumber on the task Mary Smith (BNL 8888). Also represented by this type monitoring are other plumbers of Plant Engineering who are assigned to periodic maintenance of the XYZ unit boiler. |
| Supervisor | Name of management/supervisory personnel responsible for the operation. Example: John Doe of EENS is the owner of the equipment. Mary Smith is the supervisor of work group assigned to perform the episodic replacement of the unit’s window. |
| Task Done | Describe the operation and associated work activities/tasks Example: The work consisted of wet rag wiping of 124 lead brick to remove oxidation, wrapping them with plastic tape, and stacking the bricks on a pallet for shipment. |
| Hazard | List the types and sources of potential health hazards (e.g., chemical, physical, or biological agents and ergonomic stressors). Provide the exact name of chemicals/hazards sampled and other hazards represented by the sampling. |
| Date of sampling | Mm/dd/yyyy |
| Time of Day | Military Time that the work occurred. |
| Task frequency | State whether the sampling captures an everyday occurrence, routine event, periodic event, or one-time event. State the frequency of occurrence. Example: This was a one time event during the preparation for demolition of the building. |
| Task Normalcy | State whether the sampling captures the route of exposure on a typical day or an emergency, spill, accident, unplanned event, etc. Example: This operation represents a frequent event in the maintenance cycle of the pump. It is performed typically every 40 to 60 days in a similar manner to the work done during sampling. |
| Exposure Pattern Normalcy | State whether the exposure represents typical exposure patterns, in terms of frequency, duration, intensity, etc. Example: This operation was conducted in a similar manner to the way the work is typically done. However, only two workers were used on this day and usually three workers participate. Exposure potential may be higher than typical. |
| Building | BNL Building # |
| Location | Location within the building such as room number. Provide additional detail on area within a room as appropriate. |
| Organization Name | 2-digit abbreviation |
| Indoors/outdoors | Does exposure occur in a closed in area or outdoors. If outdoors, state the weather during sampling, including temperature and wind speed and direction). Example: This operation was conducted outdoors on a very windy day. The workers were upwind of the source so exposure potential may be lower than typical. |
| Worker location | State if the worker(s) were sampled at the location of typical operation. Example: The operator was stationed at her typical position throughout the monitoring period. Exposure potential is expected to represent typical work conditions. |
| By-stander Representation | State if the monitoring represents by-stander exposure |
| Compounding Environmental Factors | Describe significant environmental conditions/factors that could affect worker exposure potential, and/or exposure sampling or measurements. Example: This operation was conducted on a very hot day. A fan was used to cool the worker, and the wind from it may cause excessive dust so that exposure potential may be higher than typical. |
| Sample Location | List the specific location of the sample. Example: Sample number 2 was taken four inches from the source and is not representative of worker exposure. Sample 4 and 5 were worn by worker on their lapel and represent their exposure and those of other worker on other days doing this job. |
| Sketch or photo | A hand drawn sketch, floor plan with sampling locations, or digital photo of the sampling area is to be included. |
| Exposure Sampling Period | Describe if the full exposure event was sampled (all the time of exposure) or which portion was sampled. Describe if the sampling includes time when no exposure would be expected to occur (such as set-up or break-down of controls). Example: The sample pumps were worn during the containment set-up and during the sand blasting. Pumps were worn during the entire blasting period and were removed prior to worker decontamination. |
| Sample Duration- Time On/ Sample Duration- Time Off | Time of day to the nearest minute that the sampling pump was turned on and off. Multiple times should be recorded if the pump was turned off during breaks, lunch, or other periods. |
| 3. Describe the Sampling Parameters and Analysis Technique (on media sampling): (Completed by SHSD IH Lab) |  |
| Calibration | State the sampling instrument calibration data. |
| Method | State the sampling and analytical methods and protocols used (e.g., those specified by OSHA or NIOSH or other equivalent methods) |
| Laboratory | State the analytical laboratory used. |
| Results | Provide the results of exposure monitoring/ sampling measurements to the IH Professional or Sampler |
| 4. Analysis of Compliance: (Completed by IH Professional) |  |
| Worker Protection status & future needs | Provide supporting data and assumptions (e.g., consideration of respiratory protection or other PPE attenuation factors). Example: The workers wore full Level C suits, with gloves and boots. Skin route of exposure would be minimal. A full face APR with OVC was worn during the spraying operation. Based on the sampling results, this level of PPE should continue to be used on future operations. |
| OEL | Identify the applicable occupational exposure limits/industrial hygiene standards, including ACGIH, OSHA, and DOE Exposure Standards and Action Levels. |
| Relevance to OELs | Compare personal exposure levels against recognized occupational exposure standards, e.g., 8-hour time-weighted average (TWA) concentration, short-term exposure level (STEL), peak or ceiling concentration, or average sound pressure level, dBA, and Action Levels. Document if the sampling and/or operation was a partial day event Describe the impact of the sample duration on the operation if it was extended to a full day- i.e. would exposure be in compliance or would over-exposure occur. |
| Exposure controls | Interpret monitoring/sampling results worker exposure level(s) relative to applicable occupational standards. Note: where personal protective equipment, such as respirators, hearing protectors, etc., was used by workers to attenuate exposures, the documented record should report the measured or estimated unattenuated level(s) of potential personal exposure, along with the type and protection/attenuation factor of the PPE worn. Example: The workers wore Level C suits, with PAPR, gloves and boots. Based on the sampling results, the workers level of exposure exceeded the PEL. While the PPE would have eliminated some actual exposure, the operation is considered out of compliance with the OEL and corrective actions to reduce employee exposure are needed. Example: The workers wore ear plugs. Determine the adjusted NRR for the actual brand of equipment used and discuss the impact on the exposure. |
| Controls in place at time of sampling | Describe the impact of exposure hazard controls (e.g., engineering, administrative, work practice, personal protective equipment) in-place during sampling. Example: The operation was administratively controlled during this trial period to restrict the work to 2 hour exposure or less. Because of this time limitation on exposure, the OEL was not exceeded. However, if the Administrative limit was not in place and exposure had occurred for 6.5 hours or more, the OEL would have been exceeded. A permanent limitation on exposure to 4 hours or less is required. |
| 5. Describe Expected actions of worker and management (Completed by IH Professional) |  |
| Needed Control Measures | Describe any recommended, additional control measures, (including engineering, administrative, work practice controls, and/or personal protective equipment), to reduce worker exposures to within acceptable exposure levels and as low as practicable. Differentiate between long-term and interim control measures. Recommendations should be for hazard controls based on the following hierarchy: - Elimination or substitution of the hazards where feasible - Engineering controls where feasible - Work practices and administration controls that limit worker exposures, and - Personal Protective Equipment |
| Need for Medical Surveillance | Determine if the OSHA or BNL trigger has been exceeded. State if medical surveillance is required based on the sampling results. |
|  | If medical surveillance is required, inform supervision to identify all the exposure population and to require medical evaluation. Example: The operation has the potential to expose workers above the Action Level. Identify any workers who have the potential to do this operation for 30 or more days in any 365 period and have them evaluated by the OMC under the medical surveillance protocol. |
| Need for new or additional Training | State if additional training is needed for the exposed workers. Example: Worker # 2 had a total dose exceeding the Action Level. The worker needs renewed training in use of ear muffs and plugs and should retake the noise and hearing conservation class. |
|  | If training is required, inform supervision to identify all the exposure population and to require training. Example: The operation has the potential to expose workers above the Occupational Exposure limit or Action Level. Identify any workers who have the potential to do this operation for 30 or more days in any 365 period and have them trained as per BTMS. |
| Other | Describe any other pertinent factors, hazards, controls, precautions, etc. |