16S/18S rRNA PCR Library Creation

Version Number:		1.1

Production Start Date:		4/24/07

Version 1.1 Date:		03/17/08

Author(s):			Tanja Woyke, Doug Smith

Reviewed/Revised by:		Julianna Chow, Karla Ikeda, Eileen Dalin

# Summary

This protocol is to (i) check the purity of DNA used for genome shotgun sequencing of single organisms and (ii) evaluate the diversity of an environmental DNA sample.

# EH&S

JGI employees performing this procedure must wear a lab coat, safety glasses, and gloves at all times.

# Materials & Reagents – PCR amplification and Cloning

| Materials/Reagents/Equipment | Vendor | Stock Number |
| --- | --- | --- |
| Disposables |  |  |
| 384-well PCR Microplate, Blue | Abgene | TF-0384/B |
| Clear Adhesive Plate Sealers | Edge BioSystems | 48461 |
| Reagents |  |  |
| Milli-Q Water | Millipore Milli-Q System | - |
| 27F primer (AGAGTTTGATCCTGGCTCAG) | IDT or Operon or your favorite | - |
| 4aF primer (TCCGGTTGATCCTGCCRG) | IDT or Operon or your favorite | - |
| 1391R primer (GACGGGCRGTGWGTRCA) | IDT or Operon or your favorite | - |
| 515F (GTGCCAAGCAGCCGCGGTAA) | IDT or Operon or your favorite | - |
| 1209R (GGGCATCACAGACCTG) | IDT or Operon or your favorite | - |
| Escherichia coli K-12 genomic DNA | ATCC | 10798D-5 |
| Archaeoglobus fulgidis genomic DNA | ATCC | 49558D |
| Saccharomyces cerevisiae genomic DNA | Isolated In house |  |
| Easy-A High-Fidelity PCR Master Mix | Stratagene | 600640 |
| 6X loading dye | See reagent/stock preparation | - |
| Agarose, for routine use | Sigma | A9539-250G |
| 50X TAE Buffer | Invitrogen | 24710-030 |
| Ultra Pure Ethidium Bromide (10mg/ml) | Invitrogen | 15585011 |
| 1 Kb DNA ladder | Invitrogen | 15615-016 |
| TOPO TA Cloning Kit for Sequencing | Invitrogen | K4580-40 |
| Equipment |  |  |
| CLP Agarose gel box (12cm x 14cm) and two 25 teeth combs | CLP | 75.1214-MT-25D |

# Procedure – PCR amplification and Cloning

NOTE: All reagents/stock solutions should be prepared prior to the start of the procedure.

NOTE: Cloning should be performed on the same day as PCR amplification.

- 16S & 18S rRNA Gene PCR Amplification

  - Add ~100 ng of DNA template to the bottom of a clean, well-labeled 96-well plate and per primer set required.  Less DNA can be used and still produces amplified product.

NOTE: There are three primer sets for rRNA gene amplification as follows:

27F/1391R for bacterial 16S rRNA gene amplification (BAC 16S)

4aF/1391R for archaeal 16S rRNA gene amplification (ARCH 16S)

515F/1209R for eukaryotic 18S rRNA gene amplification (EUK 18S)

NOTE: draw out which samples require amplification with which primer set and use different plates or plate sections for each primer set required.

NOTE: For environmental samples: leave 2 blank rows per row used!

  - Add a positive control and a negative control per primer set used:

Add ~10 ng of control DNA to the bottom of the 96-well plate:

E. coli DNA for BAC 16S, A. fulgidis genomic DNA for ARCH 16S, Saccharomyces cerevisiae genomic DNA for EUK 18S

  - Bring each sample volume up to 18.9 µl with nuclease free H2O.

  - Set-up the following PCR master mix for each primer combination for the amount of samples plus positive and negative controls.  Keep plate on ice.

|  | 1X | 12X |
| --- | --- | --- |
| F primer (10 µM) | 1.8 µl | 21.6 µl |
| R primer (10 µM) | 1.8 µl | 21.6 µl |
| Easy-A Master Mix | 22.5 µl | 270 µl |
| Total Volume: | 26.1 µl | 313.2 µl |

  - Dispense 26.1 µl of the PCR mix into each PCR plate well with template DNA. Keep plate on ice.

  - Spin down the plate.

  - Make a visual check to make sure all of the wells have mix in them (45 µl/ well).

  - Using the multi-channel pipettor, mix the samples by pipetting up and down gently. Be careful of your well location and of contamination.

  - For environmental samples only: using the multi-channel pipettor transfer 2 times 15 µl/ sample into the rows below.

NOTE: You will end up with 3x 15 µl/ environmental sample.

  - Quick spin plate.

  - Set up a PE 9700 with the following PCR program (This is a 1.5 h protocol):

94 oC – 3 min.

94 oC – 30 sec.

55 oC – 30 sec.	         20 cycles

68 oC – 90 sec.

72oC – 10 min.

4 oC – ∞

  - For environmental samples only: after the PCR reaction, pool the samples back into one 45 µl aliquot/ sample and mix the samples by pipetting up and down gently.  Be careful of your well location and of contamination.

- Amplification QC using an Agarose Gel

  - Transfer a 10 µl aliquot per sample to a fresh PCR plate (or tubes).

  - Add 5 µl of 1X loading dye per sample.

  - Vortex and spin quickly to collect the sample.

  - Load 15 µl of sample onto 1% agarose gel containing Ethidium Bromide and save the remaining sample for cloning (on ice).  Load 10 µl of 1 Kb DNA ladder between the samples.

NOTE: use smallest comb size for the agarose gel (i.e. 25 teeth combs for 12cm x 14cm agarose gel box).

CAUTION! Use proper safety precautions while handling Ethidium Bromide.  This is a carcinogen/irritant.

  - Run gel for ~ 40 min at 120V.

  - Image gel and check for proper inserts at ~1.4 kb (BAC16S), ~1.4 kb and/or ~2.1 kb (ARCH16S) and ~0.7 kb (EUK 18S). Make sure there is no non-specific amplification.

NOTE: you should see a single discrete band (exceptions are possible; i.e. archaeal 16S rRNA genes with introns; some variations in rRNA gene size).

If you see more than a discrete band of the expected size or smearing from your PCR, you either (i) optimize your PCR to eliminate non-specific amplification or (ii) gel-purify your fragment prior to cloning (avoid nuclease contamination when gel purifying!)

NOTE: make sure your positive control is positive and your negative controlis  negative.

  - Discard the gel in the appropriately labeled waste.

- Ligation of Amplified Product

  - Add 1-4 µl of fresh PCR product to the bottom of a clean, well-labeled 96-well plate (do not include negative controls).

NOTE: use PCR product volumes based on the intensity of the PCR product on the gel (i.e. use 4 µl, if very faint).

  - Add 1 µl of dilute salt solution. (1:4 dilution of  stock salt solution)

  - Bring the sample volume up to 5 µl with nuclease free H2O.

  - Add 1 µl of TOPO vector.

  - Mix gently and quick spin.

  - Incubate for 20 min at room temperature.

# Materials & Reagents – Transformation

| Materials/Reagents/Equipment | Vendor | Stock Number |
| --- | --- | --- |
| Disposables |  |  |
| Gene Pulse Cuvette 0.1 cm electrode gap | BioRad | 165-2089 |
| Falcon 14 ml Polypropylene Tube | Becton Dickinson | 352059 |
| Cryogenic Vial | Corning | 430289 |
| LB Carb 150 X-gal Plates | Teknova | L4940 |
| Reagents |  |  |
| ElectroMAX DH10B Cells | Invitrogen | 18290015 |
| SOC Medium | Teknova | S1640 |
| SOC Medium | Invitrogen | Supplied with comp cells |
| Glycerol | Sigma | G-6279 |
| Equipment |  |  |
| Gene Pulser II | BioRad | - |
| Pulse Controller Plus | BioRad | - |

# Procedure – Transformation

NOTE: All reagents/stock solutions should be prepared prior to the start of the procedure.

- Equipment Settings (BioRad Pulse Controller):

  - Low range:  200

  - High range:  (not used)

  - Capacitance:  25

  - Voltage:  1.8 kV

- Transformation

  - Place on ice: one well-labeled Eppendorf tube and cuvette for each sample being transformed.

  - Thaw ElectroMax DH10B competent cells on ice (each tube contains 100µl, enough for 2 reactions).  Discard unused cells-DO NOT refreeze!

  - To the appropriately well-labeled, cold Eppendorf tube; add 1 µl of ligation product.

  - Once thawed, mix competent cells by swirling with pipette tip a few times.

  - Add 50 µl eDH10B competent cells to the Eppendorf tube.

  - Mix by swirling the ligation and competent cells together with pipette tip a few times.

  - Transfer solution to the bottom groove of COLD cuvette and tap on tabletop a few times to settle solution to the bottom (must see even levels of cells on each side of the cuvette without bubbles).

  - Electroporate at 1.8 kV.

  - Transfer cell solution immediately to 950 µl of SOC in a 14ml falcon tube at room temperature (make sure SOC is clear, i.e. no growth).

Important! Transfer electroporation within 10 seconds to SOC. Rinse cuvette with 50 µl of the same SOC mixture to which you just added the cells.

  - Incubate within rotating wheel at 37oC for 1 hour.

  - After incubation, place on ice (no more than one hour) until ready to plate on agar plates.

- Plating with Beads:

  - Before transforming, prepare one well-labeled LB/CARB150/IPTG/X-gal agar plate per library by letting them warm to 37˚C in an incubator to dry agar down and open (~ 90 min).

  - After 60 minute transformation incubation, make a 10% glycerol transformation stock (143 µl 80% glycerol + full transformation = ~ 1143 µl glycerol transformation stock).

  - Cap, then mix transformation glycerol stock by inverting several times.  [Can store tube at -80˚C if not plating within 10 min.]

  - Prepare a tube with 1.5 ml of SOC (plating tube).

  - Add 10 μl of the transformation glycerol stock to 1.5ml of SOC (plating tube).

  - Transfer remaining transformation glycerol stock into cryotube and store immediately at –80˚C for long term storage.

  - Mix tube by inverting several times (plating tube).

  - Place 10-15 beads onto bioassay.

  - Pipet entire volume of plating tube onto bioassay with beads (~1510 μl).  Pipet stock over the entire plate.

  - Shake beads around on the bioassay until the stock has been evenly spread  (Be Gentle!).

  - Place bioassay on benchtop allowing stock to soak in completely.

  - When bioassay is dry, pour beads off of bioassay into autoclave waste.

  - Incubate the bioassays agar down in 37˚C incubator for 16-18 hrs.

# Materials & Reagents – QC PCR

| Materials/Reagents/Equipment | Vendor | Stock Number |
| --- | --- | --- |
| Disposables |  |  |
| 96-well PCR plate | USA Scientific | 1402-9708 |
| Reagents |  |  |
| Taq DNA Polymerase | Amersham Biosciences | 27-0799-63 |
| 10X PCR Buffer | Amersham Biosciences | - |
| 10mM dNTP mix | MBI Fermentas | R0192 |
| pUC-F primer (CTTTACACTTTATGCTTCC) | IDT or Operon or your favorite | - |
| pUC-R primer (GCAAGGCGATTAAGTTGG) | IDT or Operon or your favorite | - |
| 1 Kb DNA ladder | Invitrogen | 15615-016 |
| Equipment |  |  |
| GeneAmp PCR System 9700 | Perkin Elmer (Applied Biosystems) | - |

# Procedure – QC PCR

# Materials and Reagents – PCR QC

| Materials/Reagents/Equipment | Vendor | Stock Number |
| --- | --- | --- |
| Disposables |  |  |
| 96-well PCR plate | USA Scientific | 1402-9708 |
| Reagents |  |  |
| Taq DNA Polymerase (10KU) | Amersham Biosciences | 27-0799-63 |
| 10X PCR Buffer | Amersham Biosciences | - |
| 10mM dNTP mix | MBI Fermentas | R0192 |
| pUC-F primer (CTTTACACTTTATGCTTCC) | IDT or Operon or your favorite | - |
| pUC-R primer (GCAAGGCGATTAAGTTGG) | IDT or Operon or your favorite | - |
| FailSafe PCR System (Enzyme mix for high GC) | Epicentre | FS99250 |
| (choose 2x Buffer K) | Epicentre | FSP995K |
| Equipment |  |  |
| GeneAmp PCR System 9700 | Perkin Elmer (Applied Biosystems) | - |

# Procedure – PCR for QC of Library Insert Size

- PCR QC

  - Make up the following PCR master mix for each sample (x120 for a 96-well plate).  Keep plate on ice.

|  | 1X | 120X |
| --- | --- | --- |
| Nuclease free H2O | 16.85 µl | 2022.00 µl |
| 10X PCR Buffer | 2.00 µl | 240.00 µl |
| 10mM dNTP (MBI) | 0.40 µl | 48.00 µl |
| pUC-F primer (10 pmol/µl) | 0.28 µl | 33.60 µl |
| pUC-R primer (10 pmol/µl) | 0.28 µl | 33.60 µl |
| Taq (Amersham) | 0.28 µl | 33.60 µl |
| Total Volume: | 20.09 µl | 2410.8 µl |

  - Dispense 20µl of the PCR mix into each well needed of the PCR plate. Keep plate on ice.

  - Spin down the plate.

  - Make a visual check to make sure all of the wells have mix in them.

  - Using pipette tips only, pick 16-24 colonies per library into their own well. Be careful of your well location and of contamination.  Mix tips in cocktail well.

  - Quickly spin down plate.

  - Set up a PE 9700 with the following colony PCR program (This is a 3 hr 15 min protocol):

94 oC – 4 min.

94 oC – 30 sec.

55 oC – 30 sec.	         35 cycles

68 oC – *2 min.

4oC – Hold

(*modify with 5 sec. added on to each cycle for the 68 oC elongation step.)

  - Add 10µl of 1X loading dye (must be faint so the PCR bands are not hidden) to PCR plate.

  - Vortex & spin down plate to collect.

  - Load 15µl of sample onto 1% agarose gel containing Ethidium Bromide (Carcinogen/Irritant)  and save the remaining sample for later use if gel fails.  Leave a well empty between libraries for loading 5µl of marker 2.

  - Run for ~ 30 min at 120V.

  - Image gel and check for inserts at the appropriate size depending upon primer set used.  ~1.4 kb (BAC16S), ~1.4 kb and/or ~2.1 kb (ARCH16S) and ~0.7 kb (EUK 18S).

  - Discard the gel in the appropriately labeled waste.

# Reagent/Stock Preparation

80% Glycerol Stock Solution

40ml 100% Glycerol (pipette slowly)

10ml Nuclease-free H20

Autoclave

Store at room temperature

6X Loading Dye

75ml 100% glycerol

125ml Nuclease free H20

0.05g Bromphenol Blue

0.05g Xylene Cyanole FF

Store at 4˚C

1X TAE Buffer

To make 20 liters:

400ml 50X TAE Buffer

19.6L Milli-Q H2O

Store at room temperature

# Troubleshooting

(1) Only the positive control is visible.

This may indicate that the wrong concentration of the sample DNA was used for the reaction. The concentration may be too low or too high. Low DNA concentration may produce poor amplification, while too much template DNA may inhibit the amplification reaction due to the presence of PCR inhibitors in the sample. Amplification inhibition will be most commonly be the case.

SUGGESTION:

Run template DNA serial dilutions as follows.

Dilutions:

1/10 dilution: 1 µl of stock DNA to 9 µl of nuclease free H2O

1/100 dilution: 1 µl of (1/10 dilution) to 9 µl of nuclease free H2O

1/1000 dilution: 1 µl of (1/100 dilution) to 9 µl of nuclease free H2O

Use 1 µl from the above dilutions for amplification reaction.

1 µl of each dilution for bacterial 16S rRNA gene amplification

1 µl of each dilution for archaeal 16S rRNA gene amplification

1 µl of each dilution for eukaryotic 18S rRNA gene amplification

Bring volume up to 18.9 µl with nuclease free H2O

Dispense 26.1 µl of the appropriate PCR master mix for each primer combination.

NOTE: For environmental samples, using multi-channel pipettor transfer 2 times 15 µl / sample into rows below.

NOTE:  When the above method still fails to amplify, the given gene of interest and thus superkingdom may not be present in the sample.

(2) Both negative and positive controls are visible.

This indicates contamination.

SUGGESTION:

Toss all amplified products and repeat 16S/18S rRNA gene PCR amplification.

NOTE: Be careful when dispensing samples to ensure no cross contamination.

(3) Multiple bands of different sizes are visible.

This may indicate that the (environmental) sample contains organisms with rRNA genes of different lengths.  Archaeal 16S rRNA genes as well as eukaryotic 18S rRNA genes may contain introns of varying size.

Moreover, unspecific amplification may occur.

SUGGESTIONS:

Repeat amplification, run a positive, negative control, and run 1ul of undiluted DNA. In addition to adding positive and negative controls, 1 µl of DNA will be added as a DNA quality control. If multiple bands do not appear in the 1ul DNA sample, then the library should be cloned and handed off for sequencing.

NOTE: This will help determine if the DNA was affected by the heating and cooling in the thermocycler, causing multiple band patterns.

Add 1 µl of DNA sample to a clean well

Bring volume up to 45 µl with of nuclease free H2O

Do not add any PCR master mix to the sample.

The rRNA Gene library PCR QC show different band sizes.

It is perfectly normal for ARCH 16S and EUK 18S libraries to have different size PCR product.

# SOP Approval

| DEPARTMENT | APPROVED BY | DATE |
| --- | --- | --- |
| Lab Supervisor |  |  |
| Research & Development |  |  |
| Instrumentation |  |  |
| QC |  |  |
| Purchasing |  |  |
| EH & S |  |  |
| Informatics |  |  |
| Seq Assessment & Analysis |  |  |
| Dept Head of Prod Seq |  |  |

# Appendix

AUDIT TRACKING

PROCEDURAL CHANGES