- Hazard Analysis Form

- This form is to be completed by the construction subcontractor or Fermilab Task Manager and must be accepted prior to the issuance of the Notice to Proceed. In addition, this form is to be maintained at the site where the work is being performed. This is a dynamic document, which requires modifications as the project moves from start to finish.

- Job Title	        Perform Survey in the Target Hall on the East and West Walkways of the Pit Liner

- Contract/Change Order Number _  NuMI Installation Phase___   _______________________________

- Job Location      MI-65 Service Building

- Date Prepared       January 08, 2004

- Subcontractor						Fermilab

- Company _N/A_____	__	_	_______		Project Manager	_____	_			_

- Project Manager _______			_		Phone

- Phone ______	__	_ Page  ___	_______		TM/CC

- Superintendent _________	___________			Phone _			__  Page ________	_

- Phone _	___		_ Page  ________	_		ES&H Rep. 	John P. Cassidy

- ESH Rep. _______	__________________		Phone _	___		_ Page  ________

- Phone ____	_______ Page  			_		Other

- Prepared  __John P. Cassidy_		________   Date _January 8, 2004__

- Accepted  _____________		________   Date ___		_

- Accepted as noted ___			_________  Date ____

ESH Admin Form #17			Page 1				01/2002

Hazard Analysis

Using the format below, identify hazards and safety precautions/procedures to mitigate hazards.  Use as many sheets as necessary.

Description of Work:

Perform Survey in the Target Hall on the East and West Walkways of the Pit Liner

| Step/Phase of Job | Safety Hazard | Precautions/Safety Procedures |
| --- | --- | --- |
| - Carry survey equipment from the truck to the underground. - Setting up equipment on the east and west walkways. - Removing railing so survey points can be seen. | Back and muscle injuries. Fall hazards when rails are removed. Fall hazards Muscle and back strains. | Only carry what you can without overexerting yourself. Use two people on equipment that is too heavy or awkward. Set equipment up so that it minimizes the need to remove railing. All personnel with in six feet of the rail opening must use a body harness and lanyard. Retractable lanyards should be used by the Techs removing the railing. The Surveys should use a shock-absorbing lanyard short enough to keep them from edge of the opening. On the east wall personnel should tie off to the handrail along the wall. On the west side personnel should tie off to the struts along the wall. Techs removing railing should keep their bodies close to the load while lifting, pivot your feet do not twist. Communicate with your partner to lift and move the railing at the same time |

| Step/Phase of Job | Safety Hazard | Precautions/Safety Procedures |
| --- | --- | --- |
| - Surveying points on the ledge of the pit liner walls with an extension ladder from the bottom of the pit liner. | Fall hazards Muscle and back strains. | Inspect the ladder before use to ensue there are no defects. Extend the ladder far enough to reach the top of the ledge. Make sure the pitch of the ladder is at a four to one ratio. Hold the ladder in place while the Surveyor is making the survey point. Use two people to move the ladder. Keep the load close to your body, pivot your feet do not twist. Communicate with your partner so you are lifting and moving together. |

My supervisor has reviewed this hazard analysis with me and I understand the hazards and required precautionary actions.  I will follow the requirements of this hazard analysis or notify my supervisor if I am unable to do so.

| Name (please print) | Signature | Date |
| --- | --- | --- |