SUMMARY OF LAYOUT AND NAMING CONVENTIONS FOR

# HV CABLE ASSEMBLIES

Schematic of HV cable assemblies.  Two assemblies are manufactured at same time, AWG36 “inner” wires are cut in middle and attached to different PP0 sets.  Each assembly uses two identical Flex PCB boards (designated “A” – top and “B” – bottom).  Layout of an individual PCB is shown on next page.

Schematic of Flex PCB for HV cables

|  | Wires | Conductor | Insulation | Manufacturer |
| --- | --- | --- | --- | --- |
| Each End | 26 tw pairs | AWG28 7-strand tin plated copper | Polyimide red/green | Raydex |
| Middle | 26 tw pairs (2 bundles) | AWG36 solid copper | Polyimide ML clear//green | MWS |

PP1 Pinout, FlexPCB pad assignments, and Naming Conventions for HV Cables

| F-connector Pin Number | AWG28-Flex A | AWG28-Flex B | Color | Net name | Label PP1-Flex-fcn-PP0 |
| --- | --- | --- | --- | --- | --- |
| 2 | 1H |  | R | HV_1T_A | 2 - A1H - HV - A1T |
| 3 | 1 |  | G | HV_RET_1T_A | 3 - A1 - HVr - A1T |
| 4 | 2H |  | R | HV_2T_A | 4 - A2H - HV - A2T |
| 5 | 2 |  | G | HV_RET_2T_A | 5 - A2 - HVr - A2T |
| 6 |  |  |  | Drain/shield | 6 – Drain/shield |
| 27 | 3H |  | R | HV_3T_A | 27 - A3H - HV - A3T |
| 28 | 3 |  | G | HV_RET_3T_A | 28 - A3 - HVr - A3T |
| 29 | 4H |  | R | HV_4T_A | 29 - A4H - HV - A4T |
| 30 | 4 |  | G | HV_RET_4T_A | 30 - A4 - HVr - A4T |
| 31 | 5H |  | R | HV_5T_A | 31 - A5H - HV - A5T |
| 48 | 5 |  | G | HV_RET_5T_A | 48 - A5 - HVr - A5T |
| 46 | 6H |  | R | HV_6T_A | 46 - A6H - HV - A6T |
| 47 | 6 |  | G | HV_RET_6T_A | 47 - A6 - HVr - A6T |
| 59 |  |  |  | Safety_interlock_A1 |  |
| 60 |  |  |  | Safety_interlock_A2 |  |
| 7 |  |  |  | Drain/shield | 7 – Drain/shield |
| 8 | 7H |  | R | HV_7T_A | 8 - A7H - HV - A7T |
| 9 | 7 |  | G | HV_RET_7T_A | 9 - A7 - HVr - A7T |
| 10 | 8H |  | R | HV_1B_A | 10 - A8H - HV - A1B |
| 11 | 8 |  | G | HV_RET_1B_A | 11 - A8 - HVr - A1B |
| 12 | 9H |  | R | HV_2B_A | 12 - A9H - HV - A2B |
| 13 | 9 |  | G | HV_RET_2B_A | 13 - A9 - HVr - A2B |
| 32 | 10H |  | R | HV_3B_A | 32 - A10H - HV - A3B |
| 33 | 10 |  | G | HV_RET_3B_A | 33 - A10 - HVr - A3B |
| 34 | 11H |  | R | HV_4B_A | 34 - A11H - HV - A4B |
| 35 | 11 |  | G | HV_RET_4B_A | 35 - A11 - HVr - A4B |
| 49 | 12H |  | R | HV_5B_A | 49 - A12H - HV - A5B |
| 50 | 12 |  | G | HV_RET_5B_A | 50 - A12 - HVr - A5B |
| 51 | 13H |  | R | HV_6B_A | 51 - A13H - HV - A6B |
| 61 | 13 |  | G | HV_RET_6B_A | 61 - A13 - HVr - A6B |
| 15 |  | 1H | R | HV_1T_B | 15 - B1H - HV - B1T |
| 16 |  | 1 | G | HV_RET_1T_B | 16 - B1 - HVr - B1T |
| 17 |  | 2H | R | HV_2T_B | 17 - B2H - HV - B2T |
| 18 |  | 2 | G | HV_RET_2T_B | 18 - B2 - HVr - B2T |
| 19 |  |  |  | Drain/shield | 19 – Drain/shield |
| 37 |  | 3H | R | HV_3T_B | 37 - B3H - HV - B3T |
| 38 |  | 3 | G | HV_RET_3T_B | 38 - B3 - HVr - B3T |
| 39 |  | 4H | R | HV_4T_B | 39 - B4H - HV - B4T |
| 40 |  | 4 | G | HV_RET_4T_B | 40 - B4 - HVr - B4T |
| 52 |  | 5H | R | HV_5T_B | 52 - B5H - HV - B5T |
| 53 |  | 5 | G | HV_RET_5T_B | 53 - B5 - HVr - B5T |
| 54 |  | 6H | R | HV_6T_B | 54 - B6H - HV - B6T |
| 55 |  | 6 | G | HV_RET_6T_B | 55 - B6 - HVr - B6T |
| 62 |  |  |  | Safety_interlock_B1 |  |
| 63 |  |  |  | Safety_interlock_B2 |  |
| 20 |  |  |  | Drain/shield | 20 – Drain/shield |
| 21 |  | 7H | R | HV_7T_B | 21 - B7H - HV - B7T |
| 22 |  | 7 | G | HV_RET_7T_B | 22 - B7 - HVr - B7T |
| 23 |  | 8H | R | HV_1B_B | 23 - B8H - HV - B1B |
| 24 |  | 8 | G | HV_RET_1B_B | 24 - B8 - HVr - B1B |
| 25 |  | 9H | R | HV_2B_B | 25 - B9H - HV - B2B |
| 26 |  | 9 | G | HV_RET_2B_B | 26 - B9 - HVr - B2B |
| 41 |  | 10H | R | HV_3B_B | 41 - B10H - HV - B3B |
| 42 |  | 10 | G | HV_RET_3B_B | 42 - B10 - HVr - B3B |
| 43 |  | 11H | R | HV_4B_B | 43 - B11H - HV - B4B |
| 44 |  | 11 | G | HV_RET_4B_B | 44 - B11 - HVr - B4B |
| 56 |  | 12H | R | HV_5B_B | 56 - B12H - HV - B5B |
| 57 |  | 12 | G | HV_RET_5B_B | 57 - B12 - HVr - B5B |
| 58 |  | 13H | R | HV_6B_B | 58 - B13H - HV - B6B |
| 64 |  | 13 | G | HV_RET_6B_B | 64 - B13 - HVr - B6B |