MSFC PRACA : 2003-02-12 08:41

MSFC Problem Reporting and Corrective Action (PRACA) System

WHOLE RECORD REPORT( + ADDENDUM)

| MSFC Record # A15743 | In-Flight Anomaly Number -- | Contractor Report Number P-070 | JSC# -- | KSC# -- |  |
| --- | --- | --- | --- | --- | --- |
| Problem Title LO2 FEEDLINE JOINT 2 LEAKAGE |  |  |  |  |  |
| EICN# -- | ELEMENT ET | Contractor MMMSS | FSCM# -- | FCRIT 1 |  |
| HCRIT -- | Sys_Lvl N | Misc Codes A B C D E F G H I J K L M N O |  |  |  |
| HARDWARE EIM | NOMENCLATURE -- | PART# -- | SER/LOT# -- | MANUFACTURER -- |  |
| HARDWARE LRU | NOMENCLATURE -- | PART# -- | SER/LOT# -- | MANUFACTURER -- |  |
| HARDWARE NCA | NOMENCLATURE EXTERNAL TANK | PART# 80901006000-059 | SER/LOT# NA | MANUFACTURER MAF |  |
| Test/Operation A - ATP | Prevailing Condtion F - FUNCTIONAL | F / U F | Fail Mode MV - EXT LEAK | Cause MAW - MFG-ASY-WORK |  |
| System PROPULSION | Defect MA - ME ADJ | Material P - SEAL | Work Contact D. O'NEAL | Fail Date 12/20/1993 |  |
| Received at MSFC 12/23/1993 | Date Isolated 03/19/1993 | FMEA Reference 2.1.14.1 | IFA: Mission Phase -- | Mission Elapsed Time -- |  |
| Location MAF |  | Symptom MV - EXT LEAK |  | Time Cycle -- |  |
| Effectivity Text -- |  |  |  |  |  |
| Vehicle Effectivity Codes |  |  |  |  |  |
| Vehicle 1 -- | Vehicle 2 -- | Vehicle 3 -- | Vehicle 4 -- | Vehicle 5 -- |  |
| Mission Effectivity Codes |  |  |  |  |  |
| Mssn 1 -- | Mssn 2 -- | Mssn 3 -- | Mssn 4 -- | Mssn 5 -- |  |
| Estimated Completion Dates |  |  |  |  |  |
| MSFC Approved Defer Until Date 01/04/1994 | Contractor Req Defer Until Date 01/31/1994 | LVL 3 Close -- | Remark / Action 01/31/1994 |  |  |
| Investigation / Resolution Summary |  |  |  |  |  |
| Last MSFC Update 04/12/1994 | CN RSLV SBMT 02/01/1994 | Defer Date 01/13/1994 | Add Date 12/23/1993 | R/C Codes 5 - TRNG -- -- |  |
| Assignee |  |  |  |  |  |
| Design W. PATTERSON | Chief Engineer M. PESSIN | S & MA R. JACKSON | Project -- | Project MGR P. COUNTS |  |
| Approval |  |  |  |  |  |
| Design W. PATTERSON | Chief Engineer M. PESSIN | S & MA R. JACKSON | Project -- | Project MGR P. COUNTS |  |
| PAC Assignee B. HURST | PAC Review Complete BH | MSFC Closure Date 02/14/1994 | Status C - CLOSED | F/A Completion -- |  |
| Problem Type -- | SEV -- | Program Name -- | REVL -- | OPRINC -- |  |
| FUNC MOD -- | Software Effectivity -- -- -- -- -- | Software Fail CD -- |  | SUBTYPE -- | Software Closure CD -- |
| RES PERSON L2 -- | Approval Signature L3 -- |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Contractor Status Summary |  |  |  |  |  |
| Reliability/Quality Assurance Concerns, Recommendations: |  |  |  |  |  |
| Problem Description THE LO2 FEEDLINE JOINT 2 LEAKED AT 90 SCCM AND EXCEEDED THE ALLOWABLE LEAKAGE RATE OF 2.7 SCCM PER TP-8C101-FA CRITICALITY: PARA. 2.1.14.1 REFLECTS SEAL LEAKAGE AS A FAILURE MODE WHICH MAY RESULT IN LOSS OF MISSION REV "A", REVISES GENERAL, TASKS I, II, III, AND IV (SEE ASTERISKS) AND CLOSES CAPS |  |  |  |  |  |
| Contractor Investigation/Resolution * GENERAL: DURING THE DUAL SEAL CAVITY LEAK CHECK OF JOINT 2 (LO2 FWD FLEX ASSY TO LO2 FWD ELBOW), A 90 SCCM LEAK WAS DETECTED. THE ALLOWABLE LEAKAGE IS 2.7 SCCM MAXIMUM AT 50.0 +/-.5 PSIG GHE, AS TESTED PER TP-8C101-FA TROUBLE SHOOTING STEPS DETECTED NO TEST EQUIPMENT MALFUNCTION AND VERIFIED THAT THE LEAKAGE WAS ISOLATED TO THE PRIMARY SEAL. UPON JOINT DISASSEMBLY DAMAGE TO THE RACO SEAL (PRIMARY SEAL) WAS FOUND WHICH SHOWED THE SEAL WAS MISALIGNED DURING INSTALLATION. NO DAMAGE WAS FOUND ON EITHER FLANGE SURFACE THE FEEDLINES ARE MANUFACTURED BY ARROWHEAD PRODUCTS. THE FORWARD FLEX ASSEMBLY IS IDENTIFIED AS A PD4800175-099 AND THE FORWARD ELBOW IS IDENTIFIED AS A PD4800175-089. THE RACO SEAL WAS MANUFACTURED BY FLUOROCARBON AND IS IDENTIFIED AS A 55L2-5 SEAL TASK I. PROBLEM/FAILURE INVESTIGATION REVIEW ASSEMBLY PROCEDURES AND FLANGE/SEAL CONFIGURATIONS FOR FACTORS WHICH WOULD HAVE CONTRIBUTED TO THE SEAL MISALIGNMENT DURING INSTALLATION RESPONSIBILITY: D. O'NEAL/3741 -- D. WESTPHAL/3740 COMPLETED 01/04/94 * CLOSURE STATEMENT THE ORIENTATION OF THIS JOINT DURING ASSEMBLY IS SUCH THAT THE FLANGES ARE HORIZONTAL, THE UPPER FLANGE (LO2 FORWARD ELBOW) CONTAINING BOTH THE PRIMARY (RACO) AND SECONDARY (CREAVY) SEALS IN GROOVES, WHICH IS LOWERED ONTO THE SMOOTH FLANGE OF THE LO2 FORWARD FLEX ASSEMBLY. DUE TO THIS ORIENTATION ACLAR STRIPS ARE USED TO ENSURE RETENTION OF THE SEALS IN THEIR GROOVES UNTIL IMMEDIATELY PRIOR TO FLANGE MATE WHERE THE STRIPS ARE REMOVED AND THE FLANGES ARE ALLOWED TO CONTACT. DISCUSSIONS WITH THE PERSONNEL THAT PERFORMED THIS PARTICULAR ASSEMBLY NOTED THAT THIS RACO SEAL TENDED TO SLIP FROM ITS GROOVE, UNUSUAL AS NORMALLY THERE IS ENOUGH OF AN INTERFERENCE FOR TO THE GROOVE FOR THE SEAL TO RETAIN ITSELF BOTH THE RACO SEAL AND THE GROOVE OUTER DIAMETER REQUIREMENTS ARE THE SAME, 17.805 +/- .005 INCHES. NOTE THAT THE TOLERANCES WOULD ALLOW THE LOOSE SEAL CONDITION REPORTED DURING ASSEMBLY A REVIEW OF THE DATA PACK FOR THE FORWARD ELBOW SHOWED THE DIMENSIONAL REQUIREMENT FOR THE FLANGE SEAL GROOVE DIAMETER TO BE ACCEPTABLE PER A DISCRETE VERIFICATION DURING FINAL INSPECTION. THE SEAL DIAMETER IS VERIFIED AT THE SUPPLIER BY FIT CHECKING TO FIXED TOOLING. NO POST-REMOVAL DIMENSIONAL INSPECTION OF THE SEAL WAS POSSIBLE DUE TO ITS DAMAGED CONDITION * CAUSE PRIMARY SEAL DAMAGED DURING INSTALLATION * TASK II. CORRECTIVE ACTION PRODUCTION OPERATIONS AND QUALITY CONTROL PERSONNEL HAVE BEEN MADE AWARE OF THE INSTALLATION ERROR AND CAUTIONED TO USE EXTREME CARE TO ASSURE PROPER SEAL ALIGNMENT PRIOR TO FLANGE MATE. REFERENCE IOM 3725-94-05 NO OTHER CORRECTIVE ACTION IS NECESSARY. THIS TEST FAILURE IS ISOLATED, A DIRECT RESULT OF A DAMAGED PRIMARY SEAL CAUSED BY IMPROPER ALIGNMENT DURING JOINT ASSEMBLY * TASK III. FLEET CLEARANCE ET-71 JOINT NUMBER 2 ASSEMBLY SUCCESSFULLY REWORKED AND RETESTED ALL OTHER JOINTS SUCCESSFULLY COMPLETED LEAK TESTS ALL OTHER ETS CLEARED BASED ON SUCCESSFULLY COMPLETING LEAK TESTS * TASK IV. CAPS CLOSURE SUMMARY THE LEAK DETECTED DURING ACCEPTANCE TESTING OF THE LO2 FEEDLINE JOINT OCCURRED AS A RESULT OF A MISALIGNED SEAL WHICH HAD SLIPPED FROM ITS GROOVE IMMEDIATELY PRIOR TO FLANGE MATE, UNDETECTED BY; ASSEMBLY PERSONNEL. NO REASON WAS FOUND TO INDICATE THAT EITHER THE SEAL OR FLANGE GROOVE WAS AT FAULT AS THE DIAMETERS ARE CHECKED AT THE SUPPLIERS AND DIMENSIONAL TOLERANCES CAN ALLOW A CLEARANCE FIT IN A WORST CASE CONDITION (MAXIMUM GROOVE DIAMETER COUPLED WITH MINIMUM SEAL DIAMETER). THIS ANOMALY IS AN ISOLATED OCCURRENCE, DETECTABLE THROUGH TEST AND AFFECTS NO OTHER ET. ASSEMBLY PERSONNEL ARE AWARE OF THEIR ERROR. NO OTHER CORRECTIVE ACTION IS NECESSARY * THIS PROBLEM CONDITION IS CLEARLY SCREENED BY SPECIAL TESTS AND SHOULD NOT OCCUR FOLLOWING THE TESTS THIS CAPS IS CLOSED. NO FURTHER ACTION IS REQUIRED |  |  |  |  |  |
| MSFC Response/Concurrence 12/23/93 - DEFERRAL RATIONALE: BASED ON THE DEFERRAL RATIONALE IN "TASK III. FLEET CLEARANCE" (ABOVE), THIS REPORT HAS BEEN DEFERRED FOR THE NEXT SIX MONTHS PER NSTS 07700, VOLUME XI, PARAGRAPH 3.4.1, ITEM C AND NSTS 08126, REV. E, PARAGRAPH 3.3.10.1, ITEM D WHICH STATES "THE PROBLEM CONDITION IS CLEARLY SCREENED BY PREFLIGHT CHECKOUT OR SPECIAL TESTS (I.E., FAILURE MODE SHOULD NOT OCCUR FOLLOWING THE TEST)." THE DEFERRAL RATIONALE HAS BEEN APPROVED BY THE EXTERNAL TANK PROJECT MANAGER, MR. PARKER V. COUNTS SIGNED:___PARKER V. COUNTS (SIGNED)_______ DATE:___1/13/94_________ |  |  |  |  |  |

MSFC Problem Reporting and Corrective Action (PRACA) System

ASSESSMENT ADDENDUM REPORT

| MSFC Report# A15743 | IFA# -- | Contractor RPT# P-070 | JSC# -- | KSC# -- | EICN# -- |
| --- | --- | --- | --- | --- | --- |
| Asmnt Part# 80901006000-059 | Asmnt Part Name ET COMPLETE | Asmnt Serial/Lot# N/A |  |  |  |
| HCRIT CD -- | FCRIT CD 1 | CAUSE CD MAW - MFG-ASY-WORK | FAIL MODE MV - EXT LEAK |  |  |
| Asmnt FMEA 2.1.14.1 | Asmnt FM 1 | FMEA CSE A | FMEA SCSE 1 |  |  |
| Asmnt FMEA -- | Asmnt FM -- | FMEA CSE -- | FMEA SCSE -- |  |  |
| Asmnt FMEA -- | Asmnt FM -- | FMEA CSE -- | FMEA SCSE -- |  |  |
| Correlated Part# -- | Correlated Part# -- | Correlated Part# -- |  |  |  |
| Associated LRU# -- | Associated LRU# -- | Associated LRU# -- |  |  |  |
| MAJOR DESIGN CHANGES |  |  |  |  |  |
| APRV DATE -- | DESCRIPTION OF CHANGES -- |  |  |  |  |
| ASSESSMENT TEXT |  |  |  |  |  |