Appendix A.  Screening-Level Ecological Risk Assessment

# Introduction

This section describes the screening-level ecological risk evaluation that was performed for Maurepas Swamp using available chemical data from the study area.  Ecological risk was evaluated in support of EPA’s Environmental Impact Statement (EIS) for the Maurepas Swamp freshwater diversion project, which proposes to divert an estimated 1500 cubic feet per second (cfs) of freshwater from the Mississippi River into Maurepas Swamp (Day et al., 2004).  This screening-level ecological risk assessment is designed to assess if, upon completion of the project, chemicals carried in Mississippi River water and sediments could adversely affect wildlife species in Maurepas Swamp and Lake Maurepas.  This assessment follows EPA’s ecological risk assessment guidance (EPA, 1997) and includes the following components:

- Overview of the biological resources;

- Conceptual site model (CSM);

- Identification of contaminants of concern;

- Selection of receptors of concern;

- Assessment and measurement endpoints;

- Ecological effects assessment;

- Risk characterization; and,

- Uncertainties.

A.1.1  Biological Evaluation

Maurepas Swamp (Figure A-1) lies partly within the 62,500 acres encompassing the Maurepas Swamp Wildlife Management Area, which is owned by the State of Louisiana.  This management area is located 25 miles west of New Orleans and along the south shore of Lake Maurepas to near Sorrento (LDWF, 2003).  Much of the flood control on the Mississippi River has eliminated the natural inputs of freshwater, nutrients, and sediment to the area that historically built and maintained the wetlands (Lee Wilson & Associates, 2001).  Thus, permanent flooding, as well as nutrient and sediment starvation has injured much of the swamp.  Major plant species in the area include stressed and dying cypress-tupelo trees, cattails, swamp maple, and submerged aquatics.  The area is also home to various game species such as whitetail deer, squirrels, bullfrogs, rabbits, and raccoons.  Freshwater fishing for largemouth bass, perch, and crappie is common in the area.  Bald eagles nest in and around the area along with many other species of birds including warblers, ducks, woodpeckers, swallows, and various wading birds and birds of prey (Table A-1).

Figure A-1.  Maurepas Study Area.  Source: Day et al. (2004).

Adjacent to the Maurepas Wildlife Management Area lays the Manchac Swamp Wildlife Management Area.  This area, between Lake Maurepas and Lake Pontchartrain, consists of approximately 8,300 acres of second growth cypress-tupelo swamp.  Bird sightings in the Manchac Swamp in 2004 included pelicans, cormorants, wading birds, bald eagles, shorebirds, gulls, and various other insectivores and song birds (BirdFest, 2004).

Table A-1. Wildlife Species Commonly Found in and around Lake Maurepas, Pass Manchac, and Lake Pontchartrain.  Source: Handley et al., 2002.

| Common Names |  |
| --- | --- |
| Wading Birds | Shorebirds |
| Great Blue Heron | Wilson’s Phalarope |
| Little Blue Heron | Spotted Sandpiper |
| Great Egret | Pectoral Sandpiper |
| Snowy Egret | Least Sandpiper |
| Black-crowned Night-heron | Short-billed Dowitcher |
| Tricolored Heron | Western Sandpiper |
| White Ibis | Common Snipe |
| Clapper Rail | Solitary Sandpiper |
| White-faced Ibis | Reptiles |
| Least Bittern | American Alligator |
| American Bittern | Mammals |
| Virginia Rail | River Otter |
| Sora Rail | Muskrat |
| Common Moorhen | Mink |
| Waterfowl | Nutria |
| Canvasback | Northern Raccoon |
| Canada Goose | Shellfish |
| Snow Goose | Brackish Water Clam |
| Mallard | River Crayfish |
| Northern Pintail | Red Swamp Crayfish |
| Northern Shoveler | Blue Crab |
| Greater Scaup | White Shrimp |
| Lesser Scaup | Brown Shrimp |
| Bufflehead | Marine and Freshwater Fish |
| Red-breasted Merganser | Spotted Seatrout |
| Redhead | Red Drum |
| Ruddy Duck* | Southern Flounder |
| Gadwall | Bay Anchovy |
| American Wigeon | Spot |
| Blue-winged Teal | Black Drum |
| Hooded Merganser | Atlantic Croaker |
| Mottled Duck | Southern Kingfish |
| Diving Birds | Sheepshead |
| Common Loon | Gizzard Shad |
| Horned Grebe | Largemouth Bass |
| Eared Grebe | Black Crappie |
| Double-created Cormorant | Bluegill |
| Anhinga | Blue Catfish |
| Pied-billed Grebe | Channel Catfish |
| Raptors | White Crappie |
| Bald Eagle* | Warmouth |
| Osprey* | Redear Sunfish |
| Peregrine Falcon* | Freshwater Drum |
| American Kestrel* | Spotted Sunfish |
|  | Gulf Menhaden |
|  | Gulf Kingfish |

* indicates threatened or endangered status

# Conceptual Site Model and Exposure Pathway Analysis

The first step in the ecological risk assessment process is to define a conceptual site model (EPA, 1997).  The CSM is a framework for relating ecological wildlife receptors to contaminated media (i.e., water and/or sediment) and determining the degree of completion and significance of exposure pathways.  An exposure pathway describes the route(s) a chemical takes from its source to a receptor of concern (ROC).  An exposure pathway analysis links the source, location, and type of contaminants with a wildlife population, location, and activity patterns to determine the primary means of potential exposure.  If potentially complete and significant exposure pathways exist between contaminants and ROCs, an assessment of potential effects and exposure is conducted.

An exposure pathway is considered complete if all four of the following elements are present:

- A source and mechanism of chemical release to the environment;

- An environmental retention or transport medium (e.g., water or sediment) for the released chemical;

- A point of potential physical contact to a receptor with the contaminated medium (exposure point); and,

- An exposure route (e.g., ingestion of contaminated prey, incidental ingestion of sediment).

In the absence of any one of these components, a complete exposure pathway does not exist, and risk to ecological receptors is not plausible.

As discussed in Section 2.2.3, discharges of contaminants to the Mississippi River, in general, include industrial point sources and non-point source agricultural runoff.  Following discharge to the river, contaminants can partition from water into sediments, and bioavailable contaminants can make their way through the food chain.  After completion of the freshwater diversion, approximately 1500 cfs of water (and suspended sediment) will be diverted to Maurepas Swamp and subsequently into Lake Maurepas (Day et al., 2004).  In addition, there are several oil production sites within Maurepas Swamp.  Potential exposure to contaminants, therefore, may occur to wildlife in the study area through direct exposure of contaminants in the water column, in sediments, and/or in the tissues of prey items.  Upper-trophic level receptors may also be exposed through consumption of contaminated prey.  The CSM for this screening-level risk evaluation is illustrated in Figure A-2.

Figure A-2.    Conceptual Site Model for Screening-Level Risk Evaluation of Maurepas Swamp Diversion Project.

A.2.1 Exposure Pathways

The identified media of concern are surface water, sediment, and the tissues of prey items. Correspondingly, a wide range of ecological receptors are potentially at risk, including benthic invertebrates, fish, and a variety of piscivorous or aquatic avian predator species.

The contaminants of concern represent a range of chemical classes and possess different physicochemical properties that affect their behavior (i.e., partitioning, bioaccumulation) in the environment.  These properties are important considerations in an exposure pathway analysis because they are a primary determinant of whether a detected chemical is likely to possess a risk to ROCs.  For example, the physicochemical properties of polychlorinated biphenyls (PCBs), polycyclic aromatic hydrocarbons (PAHs), and most metals suggest that if these chemicals are detected in sediments at the study area, receptors exposed to this medium will likely be exposed to the chemical (i.e., the exposure pathway is complete).

Ecological receptors may be exposed to chemical contaminants through three major pathways: ingestion, dermal contact, and inhalation.  Dermal exposure to sediment contaminants for birds and mammals, although likely to occur, is considered to be de minimus.  Dermal contact with sediment is typically incidental when foraging for food.  Although established methods are available to assess dermal exposure to humans, limited data are available to quantitatively assess dermal exposure of wildlife to sediments.  In addition, the presence of scales, fur, and feathers, along with grooming and preening activities, reduces contaminant contact with skin.

Organic compounds and metal contaminants typically adsorb to sediments, as such, risk evaluations are frequently based on sediment concentrations of contaminants, which flux into and out of the water column by various chemical and physical process.  Many of these contaminants become bioavailable to benthic organisms, which are then consumed by fish, and subsequently make their way up the food chain to upper-trophic level consumers.

Exposure via inhalation can occur to upper-trophic receptors in terrestrial environments from exposure to contaminants in soil.  It is considered, however, de minimus in Maurepas Swamp due to the high moisture content in the sediments and lack of direct exposure to air due to water cover.

Based on a review of major exposure pathways, the significant complete exposure routes for higher-trophic level organisms are associated with the ingestion of contaminated prey and to a lesser extent, the direct/incidental ingestion of sediment and surface water.

A.2.2   Contaminants of Concern

Available data that could represent future conditions (i.e., contaminant data from the Mississippi River for surface water, sediment, and prey items) was compiled and the maximum concentrations were used to conservatively estimate potential risk to wildlife receptors in the Maurepas Swamp study area.  The following sections describe the available data used for this screening-level risk evaluation.

Surface Water

The most recent surface water contaminant data were collected by Louisiana Department of Environmental Quality (LDEQ) from 1995 to 2001 from six stations that were determined to be in the vicinity of the Maurepas Swamp study area:  Amite River Diversion Canal north of Gramercy, Blind River near confluence with Lake Maurepas, Blind River near Gramercy, Lake Maurepas, Mississippi Bayou north of Reserve, and Mississippi River south of Lutcher.  Between the time periods of 1995 to 2001 these sites were monitored for metals such as arsenic, cadmium, chromium, copper, lead, mercury, nickel, and zinc (Table A-2).

Table A-2.  Summary of Metals in Surface Water from 
the Maurepas Swamp Study Area (µg/L).

| Date | As | Cd | Cr | Cu | Hg | Ni | Pb | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1995 | ND | 0.2 | 0.4 | 3.1 | 0.1 | 1.7 | ND | - |
| 1995 | 2.0 | 0.3 | ND | 4.9 | ND | 2.5 | 1.2 | - |
| 1995 | 2.8 | 0.3 | ND | 8.5 | ND | 1.9 | 1.3 | - |
| 1995 | ND | 0.2 | ND | 3.7 | ND | 0.9 | 0.5 | - |
| 1995 | ND | 0.1 | 0.2 | 6.2 | ND | 2.2 | ND | - |
| 1995 | 2.0 | ND | 0.3 | 2.6 | ND | ND | ND | - |
| 1995 | 2.6 | ND | 0.5 | 5.4 | ND | ND | ND | - |
| 1995 | 3.7 | ND | 0.7 | ND | ND | 1.8 | ND | - |
| 1995 | ND | 0.2 | 0.3 | 5.4 | ND | 2.4 | 0.4 | - |
| 1995 | 2.0 | 0.4 | 0.3 | 3.5 | ND | 1.2 | 0.4 | - |
| 1995 | 3.3 | 0.3 | 0.3 | 2.1 | ND | 2.2 | 0.8 | - |
| 1995 | 1.2 | 0.1 | 0.7 | 2.2 | ND | 2.8 | 0.2 | - |
| 1995 | 1.5 | 0.1 | 0.3 | 3.7 | 0.1 | 1.0 | 0.4 | - |
| 1995 | 0.8 | 0.1 | 0.5 | 2.7 | ND | 1.9 | 0.5 | - |
| 1995 | 1.1 | ND | 0.5 | 2.8 | ND | 1.4 | 0.5 | - |
| 1995 | 2.1 | ND | ND | 1.6 | ND | 2.8 | ND | - |
| 1995 | ND | ND | ND | 1.7 | ND | 2.5 | 0.2 | - |
| 1996 | 0.8 | ND | 0.2 | 0.7 | ND | 1.3 | ND | - |
| 1996 | 1.4 | ND | 0.4 | 3.6 | ND | 2.3 | ND | - |
| 1996 | 1.5 | 0.4 | 0.4 | 3.9 | ND | 1.8 | 0.2 | - |
| 1996 | ND | 0.3 | 0.3 | 1.9 | ND | 3.0 | 0.5 | - |
| 1996 | 1.3 | 0.2 | ND | 3.7 | ND | 5.1 | 1.0 | - |
| 1996 | ND | 0.1 | ND | 1.7 | ND | 7.1 | 0.4 | - |
| 1996 | 3.1 | 0.1 | ND | 2.2 | ND | 1.5 | 0.2 | - |
| 1996 | 3.7 | 0.1 | 0.6 | 3.2 | ND | 4.0 | ND | - |
| 1996 | 0.8 | 0.3 | 0.3 | 0.7 | ND | 2.5 | ND | - |
| 1996 | 0.9 | 0.3 | 0.3 | 3.1 | ND | 1.5 | 0.6 | - |
| 1996 | 1.0 | ND | 0.4 | 1.6 | ND | 2.8 | 0.4 | - |
| 1996 | ND | 0.1 | 0.5 | 1.8 | ND | 2.4 | ND | - |
| 1997 | ND | 0.1 | 0.5 | 2.4 | ND | 2.7 | ND | - |
| 1997 | ND | 0.2 | 0.4 | 6.9 | ND | ND | ND | - |
| 1997 | ND | 0.5 | 0.4 | 0.9 | ND | ND | ND | - |
| 1997 | ND | ND | 0.4 | 0.8 | ND | ND | ND | - |
| 1997 | 5.0 | ND | ND | 2.8 | ND | ND | ND | - |
| 1997 | ND | ND | ND | ND | ND | ND | ND | - |
| 1997 | ND | ND | ND | 5.7 | ND | ND | ND | - |
| 1997 | ND | ND | ND | 3.0 | ND | ND | ND | - |
| 1997 | ND | ND | ND | ND | 0.1 | ND | ND | - |
| 1997 | ND | ND | ND | ND | 0.1 | ND | ND | - |
| 1997 | ND | 1.4 | ND | 11.9 | 0.1 | ND | ND | - |
| 1997 | ND | 6.5 | ND | 24.1 | ND | ND | ND | - |

Table A-2.  Summary of Metals in Surface Water from 
the Maurepas Swamp Study Area (µg/L).

| Date | As | Cd | Cr | Cu | Hg | Ni | Pb | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1998 | ND | ND | ND | ND | ND | ND | ND | - |
| 1998 | ND | ND | ND | ND | 0.1 | ND | ND | - |
| 1998 | ND | ND | ND | ND | 0.2 | ND | ND | - |
| 1998 | ND | ND | ND | 3.1 | 0.1 | ND | ND | - |
| 2001 | 0.3 | 0.01 | 0.1 | 1.4 | 0.001 | 1.1 | ND | ND |
| 2001 | 1.3 | 0.01 | 0.03 | 1.1 | 0.001 | 91.0 | 0.4 | 1.2 |
| 2001 | 1.4 | - | ND | 1.1 | 0.001 | 1.2 | 0.3 | 0.5 |
| 2001 | 1.5 | 0.02 | ND | 0.8 | 0.001 | 1.2 | 0.02 | ND |
| 2001 | 2.0 | 0.01 | 0.2 | 2.2 | 0.006 | 1.2 | 0.4 | 2.3 |
| 2001 | 1.5 | 0.01 | 0.16 | 1.6 | 0.004 | 1.3 | 0.3 | 1.6 |
| 2001 | 1.8 | 0.0 | 0.1 | 1.9 | - | 1.2 | 0.3 | 1.7 |
| 2001 | 0.7 | 0.3 | 0.4 | 2.1 | 0.001 | 1.3 | 4.0 | ND |
| 2001 | 0.9 | - | 0.1 | 1.4 | 0.001 | 0.8 | 0.03 | 0.6 |
| 2001 | 0.9 | - | 0.03 | 0.6 | 0.002 | 0.5 | 0.2 | 0.9 |
| 2001 | 1.3 | 0.01 | 0.1 | 0.9 | 0.002 | 0.5 | 0.1 | 2.1 |

ND = not detected

- = no data available

Sediment

Available data for many chemical contaminants in sediment are lacking for the Maurepas Swamp study area.  As a result, available data from Mississippi River sediments were compiled from a study reported by Macauley and Summers (1998).  Surface sediments were sampled during EPA’s Environmental Monitoring and Assessment Program (EMAP) from 1991 to 1994 for chemical contaminants including metals, several PAHs, two PCB congeners, and several pesticides (Table A-3).

Table A-3.  Contaminant Concentrations in Sediment from the Mississippi River.

| Chemical Parameters | Minimum Concentration | Maximum Concentration |
| --- | --- | --- |
| Metals (ppm) |  |  |
| Arsenic | 2.2 | 15.9 |
| Cadmium | 0.03 | 0.86 |
| Chromium | 6.7 | 84 |
| Copper | 1.5 | 33.7 |
| Lead | 7.4 | 35.7 |
| Mercury | 0.002 | 0.15 |
| Nickel | 9 | 45.5 |
| Selenium | 0.02 | 0.82 |
| Silver | 0.04 | 0.46 |
| Zinc | 12.4 | 165 |
| PAHs (ppb) |  |  |
| 1-Methylphenanthrene | 0.25 | 65.45 |
| 2,3,5-Trimethylnaphthalene | NA | NA |
| 2,6-Dimethylnaphthalene | NA | NA |
| 2-Methylnaphthalene | NA | NA |
| Acenaphthene | NA | NA |
| Acenaphthylene | NA | NA |
| Anthracene | 0.1 | 34.6 |
| Fluorene | 0.25 | 76.29 |
| Naphthalene | 1.08 | 218.7 |
| Phenanthrene | 0.94 | 164.19 |
| Benzo[a]anthracene | 0.29 | 224.21 |
| Benzo[a]pyrene | 0.13 | 307.84 |
| Benzo[b]fluoranthene | 0.3 | 89 |
| Benzo[e]pyrene | 0.19 | 260.36 |
| Benzo[g,h,i]perylene | 0.11 | 252.28 |
| Benzo[k]fluoranthene | 0.09 | 94.74 |
| Chrysene | 0.44 | 294.73 |
| Dibenz[a,h]anthracene | 0.04 | 105.68 |
| Dibenzothiophene | 0.15 | 54.66 |
| Fluoranthene | 0.73 | 95.9 |
| Indeno[1,2,3-c,d]-pyrene | 0.15 | 66.31 |
| Perylene | 4.52 | 263.66 |
| Pyrene | 1.02 | 185.28 |
| PAHs, Total | NA | NA |
| PCB 44 | 0.03 | 3.89 |
| PCB 209 | 0.04 | 0.45 |
| PCBs, Total | NA | NA |

Table A-3.  Contaminant Concentrations in Sediment from the Mississippi River.

| Chemical Parameters | Minimum Concentration | Maximum Concentration |
| --- | --- | --- |
| Pesticides (ppb) |  |  |
| 4,4'-DDD | 0.09 | 5.39 |
| 4,4'-DDE | 0.03 | 2.89 |
| 4,4'-DDT | 0.02 | 18.44 |
| Total DDx compounds (sum of 4,4'- and 2,4'-isomers) | ND | 23.24 |
| Aldrin | 0.028 | 0.09 |
| cis-Nonachlor | 0.01 | 0.29 |
| trans-Nonachlor | 0.04 | 0.55 |
| Atrazine | NA | NA |
| BHC (alpha) | 0.0071 | 1.67 |
| Chlordane | NA | NA |
| Dieldrin | 0.15 | 0.68 |
| Endosulfan | NA | NA |
| Endrin | 0.04 | 0.31 |
| Heptachlor | NA | NA |
| Heptachlor epoxide | 0.03 | 0.48 |
| Methoxychlor | NA | NA |
| Herbicides (ppb) |  |  |
| Acetochlor | NA | NA |
| Alachlor | NA | NA |
| Carbonate | NA | NA |
| Triazine | NA | NA |
| 2-4-dichlorophenoxy acetic acid (2,4-D) | NA | NA |
| 2, 4, 5-trichlorophenoxy acetic acid (2,4,5-T) | NA | NA |
| 4-(2,4-dichlorophenoxy)butyric acid (2,4-DB) | NA | NA |
| 2-(2,4,5-Trichlorophenoxy) propionic acid (2,4,5-TP) | NA | NA |
| Volatile Organic Compounds (ppb) |  |  |
| 1,1,1-Trichloroethane | NA | NA |
| 1,1,2,2-Tetrachloroethane | NA | NA |
| 1,1,2-Trichloroethane | NA | NA |
| 1,1-Dichloroethane | NA | NA |
| 1,1-Dichloroethene | NA | NA |
| 2-Hexanone | NA | NA |

Table A-3.  Contaminant Concentrations in Sediment from the Mississippi River.

| Chemical Parameters | Minimum Concentration | Maximum Concentration |
| --- | --- | --- |
| 4-Methyl-2-Pentanone | NA | NA |
| Benzene | NA | NA |
| Carbon Disulfide | NA | NA |
| Carbon Tetrachloride | NA | NA |
| Chlorobenzene | NA | NA |
| Chloroform | NA | NA |
| cis-1,3-Dichloropropene | NA | NA |
| Ethylbenzene | NA | NA |
| Methylene chloride | NA | NA |
| Toluene | NA | NA |
| Total Xylenes | NA | NA |
| Trans-1,3-dichloropropene | NA | NA |

NA = not available

Fish

To assess potential exposures to upper-trophic level receptors, such as the bald eagle, it is necessary to estimate chemical contaminant levels in prey items, such as fish tissue.  These concentrations are used in a dose-assessment model (Section A2.4) to evaluate exposures.  For this screening-level ecological risk evaluation, available fish tissue data were obtained from USGS (2002) and LDEQ (1997-2004).  Mercury-in-fish tissue data, collected by LDEQ, were grouped according to three distinct areas:  Mississippi River, Lake Maurepas, and Lake Pontchartrain.  Mercury concentrations in fish tissue ranged from 0.14 to 0.73 ppm in Lake Maurepas, <0.0001 to 1.11 ppm in Lake Pontchartrain, and 0.008 to 0.53 ppm in the Mississippi River (Attachment 2).  Fish samples collected in 1995 by USGS at Luling, LA revealed moderately high concentrations of PCBs and TCDD in carp and white bass (USGS, 2002) as well as DDx compounds, chlordane, dieldrin, and nonachlor (USGS, 2002) (Table A-4).

Table A-4.  Contaminant Data (µg/g wet weight) in Fish Tissue Samples from Luling, LA (1995). Source: USGS, 2002.

| Species | Percent Lipid | Percent Moisture | p,p'-DDD | p,p'-DDE | p,p'-DDT | Total DDx | a-Chlordane | g-Chlordane | trans-Nonachlor | Dieldrin | PCB-Total | Nickel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carp | 12.1 | 69.2 | 0.062 | 0.098 | <0.01 | 0.16 | 0.015 | 0.012 | 0.017 | 0.023 | 0.48 | 0.34 |
| Carp | 13.4 | 64.7 | 0.054 | 0.074 | <0.01 | 0.13 | 0.016 | 0.014 | 0.015 | 0.028 | 0.35 | 0.41 |
| Largemouth Bass | 4.69 | 72.4 | 0.021 | 0.041 | <0.01 | 0.06 | <0.01 | <0.01 | <0.01 | 0.021 | 0.22 | 0.53 |
| White Bass | 7.38 | 71.6 | 0.031 | 0.064 | 0.018 | 0.11 | <0.01 | <0.01 | <0.01 | 0.028 | 0.31 | 0.56 |
| White Bass | 7.84 | 71.4 | 0.043 | 0.11 | 0.015 | 0.17 | 0.014 | <0.01 | 0.014 | 0.025 | 0.53 | 0.44 |

A.2.3  Selection of Receptors of Concern (ROCs)

The selection of ROCs takes into account a variety of factors which include federally listed threatened and endangered (T&E) species; species of special concern within the State of Louisiana; the likelihood of the species expected to occur based on existing conditions in the swamp; significance of the species to ecosystem function; availability of toxicity and life history data; and species sensitivity to expected contaminants.

Because it is impractical to assess exposure to all potentially exposed species within a trophic group, representative species were selected as conservative surrogates for exposure to a group of taxonomically related and ecologically similar receptors.

For this risk evaluation, biological information was gathered to determine the species that are most abundant in the Maurepas Swamp study area (Table A-5).  Benthic invertebrates may be exposed to contaminants from the sediment, from surface water, and from food items (i.e, plankton and small fish).  Blue crabs are an example of a large macroinvertebrate that comes into direct contact with potentially contaminated sediments throughout their entire lifespan.  Other invertebrates include shrimp, crayfish, and clams.  These all have the potential to accumulate contaminants in their tissues and transfer them up the food chain when preyed upon by other organisms (Figure A-2).

Available fish species in the Maurepas Swamp study area include the Gulf Sturgeon which is listed on both the State and Federal threatened and endangered species lists.  It is a demersal fish that spends most of its time in close proximity to the benthos, where it may consume contaminated sediments and food items.  Channel catfish are another demersal fish species that are prevalent in the area, foraging on potentially contaminated sediments, invertebrates, and detritus.

A variety of avian species have been observed in and around the Maurepas Swamp study area.  These include wading birds, diving birds, raptors, and song birds.  The bald eagle was selected as a ROC for the Maurepas Swamp freshwater diversion project because it is listed as both a State and Federal threatened and endangered species and is a diurnal opportunistic raptor.  It primarily feeds on carrion and dead and dying fish.  Bald eagles will generally consume whatever food source is most plentiful and easy to scavenge or capture (EPA, 1993).  They will however, actively hunt and prey on fish, waterfowl, and muskrats.  Because bald eagles scavenge for dead or dying prey, they are particularly vulnerable to environmental contaminants and pesticides (e.g., from feeding on birds that died from pesticide toxicity).  To be conservative, the screening-level risk assessment assumes the bald eagle’s diet is comprised completely of contaminated fish from the Maurepas Swamp study area.

Table A-5. Potential Receptors of Concern for the Maurepas Swamp

Ecological Risk Evaluation.

| Receptor | Exposure Media | Rationale for Selection of Receptor and Pathway |
| --- | --- | --- |
| Benthic Invertebrates |  |  |
| Blue crab | Sediment/surface water/biota | Epibenthic omnivorous invertebrate that consumes plankton and small fish and comes into direct contact with contaminated sediments |
| Benthic macroinvertebrate community | Sediment/surface water/biota | Various benthic invertebrate populations representing different trophic levels which are in intimate contact with contaminated sediments |
| Fish |  |  |
| Gulf sturgeon | Sediment/surface water/biota | Listed as a threatened anadromous species on both the State and Federal T&E list and feeds on benthic invertebrates and small fish |
| Largemouth bass | Surface water/biota | Dominant predatory species that has sensitive early-life stages |
| Channel catfish | Sediment/surface water/biota | Freshwater demersal species that comes in direct contact with contaminated sediment as a result of foraging on benthic and epibenthic organisms and detritus |
| Birds |  |  |
| Bald eagle | Sediment/surface water/biota | Listed as a threatened species on the Federal T&E list and endangered on the State list; observed nesting in the area and feeds on fish, waterfowl, and muskrats |
| Great blue heron | Sediment/surface water/biota | Wading bird that potentially consumes large amounts of sediment while feeding on benthic aquatic life |
| Double-crested cormorant | Sediment/surface water/biota | Piscivorous bird that has been observed foraging in the area |

A.2.4  Assessment and Measurement Endpoints

As defined by EPA (1997), assessment endpoints (AEs) are formal expressions of the actual environmental values that are to be protected at a site.  AEs are defined based on technical considerations including the significance of exposure pathways, the presence of receptors, and a chemical’s biotic transfer pathway.  Selection of AEs must consider the ecosystem, communities, and species relevant to a particular site.  The selection of AEs depends on:

- The chemicals present and their concentration;

- Mechanisms of toxicity of the chemicals to different groups of organisms;

- Ecologically relevant receptor groups that are potentially sensitive or highly exposed to the chemicals; and,

- Potentially complete exposure pathways.

The AEs proposed for quantitative evaluation at the Maurepas Swamp are based on protection of the most sensitive environmental resources identified at the site.  For this screening-level ecological the following three AEs and subsequent measurement endpoints (MEs) are proposed to represent the resources to be protected in Maurepas Swamp:

- AE(1): Protection and maintenance (i.e., survival, growth, and reproduction) of benthic invertebrate communities that serve as a forage base for fish and wildlife populations.

ME(1):  This will be evaluated by comparing concentrations of contaminants in sediments to available freshwater sediment benchmarks from the National Oceanic and Atmospheric Administration (NOAA) (e.g., PELs and TELs) which are protective of benthic organisms.

- AE(2): Protection and maintenance (i.e., survival, growth, and reproduction) of benthic and pelagic fish populations that serve as a forage base for other fish and wildlife populations.

ME(2):  This will be evaluated by comparing concentrations of surface water contaminants to the National Recommended Ambient Water Quality Criteria (NRAWQC) developed by EPA (and Canadian standards where U.S. criteria are not available).

- AE(3): Protection and maintenance (i.e., survival, growth, and reproduction) of the bald eagle.

ME(3):  This will be evaluated by modeling the daily dose of chemicals to the bald eagle from the ingestion of contaminated surface water, sediment, and prey items.  Potential risk will be characterized by comparing the modeled dose estimate to toxicity reference values (TRVs).

A.2.5	Exposure Assessment

For the first measurement endpoint, sediment concentrations collected by Macauley and Summers (1998) were compared to available sediment quality guidelines for freshwater.  These guidelines were developed by NOAA and represent a low value, Threshold Effects Level (TEL) and a high value, Probable Effects Level (PEL).  The TEL represents the concentration below which adverse effects are expected to occur only rarely.  The PEL is the level above which adverse effects are frequently expected to occur (Table A-6).

Table A-6.  Comparison of Available Surface Sediment Concentrations from the Mississippi River (1991-1994) to Applicable Benchmarks.

| Chemical Parameters | Sediment Quality Guidelines (TEL) | Sediment Quality Guidelines (PEL) | Minimum Concentration | Maximum Concentration |
| --- | --- | --- | --- | --- |
| Metals (ppm) |  |  |  |  |
| Arsenic | 5.9 | 17 | 2.2 | 15.9 |
| Cadmium | 0.596 | 3.53 | 0.03 | 0.86 |
| Chromium | 37.3 | 90 | 6.7 | 84 |
| Copper | 35.7 | 197 | 1.5 | 33.7 |
| Lead | 35 | 91.3 | 7.4 | 35.7 |
| Mercury | 0.174 | 0.486 | 0.002 | 0.15 |
| Nickel | 18 | 35.9 | 9 | 45.5 |
| Selenium | NVA | NVA | 0.02 | 0.82 |
| Silver | NVA | NVA | 0.04 | 0.46 |
| Zinc | 123.1 | 315 | 12.4 | 165 |
| PAHs (ppb) |  |  |  |  |
| 1-Methylphenanthrene | NVA | NVA | 0.25 | 65.45 |
| 2,3,5-Trimethylnaphthalene | NVA | NVA | NA | NA |
| 2,6-Dimethylnaphthalene | NVA | NVA | NA | NA |
| 2-Methylnaphthalene | NVA | NVA | NA | NA |
| Acenaphthene | NVA | NVA | NA | NA |
| Acenaphthylene | NVA | NVA | NA | NA |
| Anthracene | NVA | NVA | 0.1 | 34.6 |
| Fluorene | NVA | NVA | 0.25 | 76.29 |

Table A-6.  Comparison of Available Surface Sediment Concentrations from the Mississippi River (1991-1994) to Applicable Benchmarks.

| Chemical Parameters | Sediment Quality Guidelines (TEL) | Sediment Quality Guidelines (PEL) | Minimum Concentration | Maximum Concentration |
| --- | --- | --- | --- | --- |
| Naphthalene | NVA | NVA | 1.08 | 218.7 |
| Phenanthrene | NVA | NVA | 0.94 | 164.19 |
| Benzo[a]anthracene | 31.7 | 385 | 0.29 | 224.21 |
| Benzo[a]pyrene | 31.9 | 782 | 0.13 | 307.84 |
| Benzo[b]fluoranthene | NVA | NVA | 0.3 | 89 |
| Benzo[e]pyrene | NVA | NVA | 0.19 | 260.36 |
| Benzo[g,h,i]perylene | NVA | NVA | 0.11 | 252.28 |
| Benzo[k]fluoranthene | NVA | NVA | 0.09 | 94.74 |
| Chrysene | 57.1 | 862 | 0.44 | 294.73 |
| Dibenz[a,h]anthracene | NVA | NVA | NA | NA |
| Dibenzothiophene | NVA | NVA | 0.15 | 54.66 |
| Fluoranthene | 111 | 2355 | 0.73 | 95.9 |
| Indeno[1,2,3-c,d]-pyrene | NVA | NVA | 0.15 | 66.31 |
| Perylene | NVA | NVA | 4.52 | 263.66 |
| Pyrene | 53 | 875 | 1.02 | 185.28 |
| PAHs, Total | 34.1 | 277 | NA | NA |
| PCB 44 | NVA | NVA | 0.03 | 3.89 |
| PCB 209 | NVA | NVA | 0.04 | 0.45 |
| PCBs, Total | NVA | NVA | NA | NA |
| Pesticides (ppb) |  |  |  |  |
| 4,4'-DDD | 3.54 | 8.51 | 0.09 | 5.39 |
| 4,4'-DDE | 1.42 | 6.75 | 0.03 | 2.89 |
| 4,4'-DDT | NVA | NVA | 0.02 | 18.44 |
| Total DDx compounds (sum of 4,4'- and 2,4'-isomers) | 6.98 | 4450 | ND | 23.24 |
| Aldrin | NVA | NVA | 0.028 | 0.09 |
| BHC (alpha) | NVA | NVA | 0.0071 | 1.67 |
| Chlordane | 4.5 | 8.9 | NA | NA |
| Dieldrin | 2.85 | 6.67 | 0.15 | 0.68 |
| Endosulfan | NVA | NVA | NA | NA |
| Endrin | 2.67 | 62.4 | 0.04 | 0.31 |
| Heptachlor | NVA | NVA | NA | NA |
| Heptachlor epoxide | 0.6 | 2.74 | 0.03 | 0.48 |
| Methoxychlor | NVA | NVA | NA | NA |

Table A-6.  Comparison of Available Surface Sediment Concentrations from the Mississippi River (1991-1994) to Applicable Benchmarks.

| Chemical Parameters | Sediment Quality Guidelines (TEL) | Sediment Quality Guidelines (PEL) | Minimum Concentration | Maximum Concentration |
| --- | --- | --- | --- | --- |
| Herbicides (ppb) |  |  |  |  |
| Acetochlor | NVA | NVA | NA | NA |
| Alachlor | NVA | NVA | NA | NA |
| Atrazine | NVA | NVA | NA | NA |
| Carbonate | NVA | NVA | NA | NA |
| Triazine | NVA | NVA | NA | NA |
| 2-4-dichlorophenoxy acetic acid (2,4-D) | NVA | NVA | NA | NA |
| 2, 4, 5-trichlorophenoxy acetic acid (2,4,5-T) | NVA | NVA | NA | NA |
| 4-(2,4-dichlorophenoxy)butyric acid (2,4-DB) | NVA | NVA | NA | NA |
| 2-(2,4,5-Trichlorophenoxy) propionic acid (2,4,5-TP) | NVA | NVA | NA | NA |
| Volatile Organic Compounds (ppb) |  |  |  |  |
| 1,1,1-Trichloroethane | NVA | NVA | NA | NA |
| 1,1,2,2-Tetrachloroethane | NVA | NVA | NA | NA |
| 1,1,2-Trichloroethane | NVA | NVA | NA | NA |
| 1,1-Dichloroethane | NVA | NVA | NA | NA |
| 1,1-Dichloroethene | NVA | NVA | NA | NA |
| 2-Hexanone | NVA | NVA | NA | NA |
| 4-Methyl-2-Pentanone | NVA | NVA | NA | NA |
| Benzene | NVA | NVA | NA | NA |
| Carbon Disulfide | NVA | NVA | NA | NA |
| Carbon Tetrachloride | NVA | NVA | NA | NA |
| Chlorobenzene | NVA | NVA | NA | NA |
| Chloroform | NVA | NVA | NA | NA |
| cis-1,3-Dichloropropene | NVA | NVA | NA | NA |
| Ethylbenzene | NVA | NVA | NA | NA |
| Methylene chloride | NVA | NVA | NA | NA |
| Toluene | NVA | NVA | NA | NA |
| Total Xylenes | NVA | NVA | NA | NA |
| Trans-1,3-dichloropropene | NVA | NVA | NA | NA |

NVA= no (criteria) value available

NA = not available

For the second measurement endpoint, surface water data were compared to chronic aquatic life standards from EPA’s National Ambient Water Quality Criteria (NAWQC) (EPA, 2004) (Table A-7).  Because U.S. guidelines or criteria for aquatic life do not presently exist for most pesticides in current use, the Canadian pesticide water-quality guidelines, where available, are used as a point of reference for pesticide concentrations (Table A-8).

Table A-7. National Ambient Water Quality Criteria (NAWQC) for Metals.

| Metals | NAWQC (ppb) |
| --- | --- |
| Arsenic | 150 |
| Cadmium | 0.25 |
| Chromium | 11 (Cr VI) |
| Copper | 9 |
| Lead | 2.5 |
| Mercury | 0.77 |
| Nickel | 52 |
| Selenium | 5 |
| Silver | 0.12 |
| Zinc | 120 |

For the third measurement endpoint, the exposure assessment quantitatively estimates potential exposure of the bald eagle to chemicals identified at the site. An exposure (dose) model incorporating the receptor’s natural history information and characteristics (including diet composition, ingestion rates, body weights, and foraging ranges) was developed to evaluate its exposure to contaminants. It was assumed that the bald eagle is exposed to site contaminants through consumption of contaminated prey, including fish and benthic invertebrates, as well as from the incidental ingestion of sediment. This screening-level risk evaluation conservatively assumes that the bald eagle will forage only on the most contaminated fish (maximum concentrations) within the Maurepas Swamp study area (i.e., site use factor = 1).

Table A-8.  Aquatic Life Criteria for Selected Major Pesticides in the Mississippi River Basin.  Source:  Meade, 1995.

| Pesticide | Aquatic Life Criteria (µg/L) | Source |
| --- | --- | --- |
| Alachlor | NVA | - |
| Atrazine | 2 | Trotter et al. 1990 (Canada) |
| Bentazon | NVA | - |
| Butylate | NVA | - |
| Cyanazine | 2 | Pauli et al. 1991b (Canada) |
| 2-4-dichlorophenoxy acetic acid (2,4-D) | 4 | Task Force on Water Quality Guidelines (Canada) |
| EPTC | NVA | - |
| Metolachlor | 8 | Kent et al. 1991 (Canada) |
| Metribuzin | 1 | Pauli et al. 1990 (Canada) |
| Molinate | NVA | - |
| Propachlor | NVA | - |
| Propanil | NVA | - |
| Propazine | NVA | - |
| Simazine | 10 | Pauli et al. 1991b (Canada) |
| Triflualin | 0.2 | Kent et al. 1992 (Canada) |
| Carbaryl | NVA | - |
| Carbofuran | 1.75 | Trotter et al. 1989 (Canada) |
| Chlorpyrifos | 0.041 | EPA, 2004 (US) |
| Diazinon | NVA | - |

NVA = no value available

The following dose model was used to assess daily exposures of contaminants to the bald eagle and to characterize exposure:

Dose = (Csed * IRsed)+ (Cfood * IRfood) * SUF 		(Equation 1)

BW

Where:

Dose	=	daily dose resulting from ingestion (mg/kg)

Csed 	= 	concentration of contaminant in surface sediment (mg/kg)

IRsed	=	estimate of receptor’s daily ingestion rate of surface sediment (kg/d)

Cfood	 = 	concentration of contaminant in food tissue (mg/kg)

IRfood	 =	estimate of daily ingestion rate of food tissue (kg/d)

SUF	 = 	site use factor (unitless)

BW	 = 	body weight (kg)

The exposure parameters for the bald eagle that are summarized in Table A-9 and the maximum sediment contaminant data provided in Table A-3 were used to evaluate potential chemical doses to the bald eagle using Equation 1.  Due to the limited amount of available toxicity information on many contaminants, only a select few contaminants (i.e., those that are generally considered the most toxic) were modeled to obtain the most conservative level of risk.  These include mercury, due to its high potential for bioavailability, especially in its methylated form; nickel because it exceeded the screening level benchmark; and total DDx because of its known toxicity to bald eagles (EPA, 1993). Fish tissue data for mercury were acquired by LDEQ from various stations along the Mississippi River (Attachment 2).  Fish tissue data for total DDx and nickel were acquired by USGS from the Mississippi River at Luling, Lousiana (USGS, 2002) (Table A-4).  These tissue concentrations were converted to mg/kg dry weight and used in the dose models to represent the concentration of food (Cfish) for these contaminants.

Table A-9.  Exposure Parameters for the Bald Eagle.

| Exposure Parameter | Abbreviation | Unit | Value | Source |
| --- | --- | --- | --- | --- |
| Body weight | BW | kg | 4.5 | EPA, 1993 |
| Daily ingestion rate of fooda | IRfish | kg/day | 0.451 | EPA, 1993 |
| Daily ingestion rate of sedimentb | IRsed | kg/day | 0.004 | Beyer et al. 1994 |
| Site Use Factor (max of 1)c | SUF | unitless | 1 | Most conservative |

a   Conservatively assumed to be comprised of 100% fish. The food ingestion rate (g/g/day) from EPA (1993) was multiplied by the body weight to obtain a daily ingestion rate (kg/day).

b   Sediment uptake was assumed to be 1% of the daily ingestion rate.

c  A site use factor describes the percentage of the study area the receptor is expected to use for foraging.  In a screening-level evaluation, it is typically set at 1 to be the most conservative (i.e., the eagle spends all of its time foraging within the study area).

# Ecological Effects Assessment

For the purpose of evaluating the ecological effects on benthic organisms, sediment contaminant data were compared to appropriate benchmarks.  It is important to note that the sediment sampling stations (Macauley and Summers, 1998) were all located south of the Maurepas Swamp study area, downstream of New Orleans and a highly industrialized corridor in Louisiana.  Because of the downward flow of Mississippi River water, one might expect the sediments at these stations to contain relatively high levels of contaminants from industrial point sources.  However, concentrations were compared to available benchmarks (NOAA, 1999) revealing that the majority of all maximum surface sediment concentrations were lower than the appropriate PEL, but higher than the TEL.  Only nickel exceeded its PEL with a maximum concentration of 45.5 ppm (compared to a PEL of 35.9) (Table A-6).

To evaluate the potential ecological effects to fish, surface water quality criteria for aquatic life were compared to available data.  In general, metal concentrations in surface water were either not detected or were below water quality criteria.  The majority of concentrations of metals in surface water over a six-year time span either fell below EPA’s standards or were not detected.  Exceedances were noted for cadmium and copper, and one exceedance occurred for nickel (Table A-10). Total herbicide loads in the Mississippi River at Baton Rouge did not exceed 10 µg/L during 1991-1997 (USGS, 1999).  No herbicide or metabolite was detected at a concentration exceeding 5 µg/L. Although concentrations of atrazine, cyanazine, and metolachlor peak simultaneously in late May and early June, none of the average annual concentrations of herbicides exceeded maximum contaminant levels (MCLs) or Health Advisories (HA) for drinking water (USGS, 1999).

Table A-10.  Number of Exceedances of National Ambient Water Quality Criteria (NAWQC) for Metals in the Maurepas Swamp Study Area.

| Metals | NAWQC (ppb) | Number Exceedances |
| --- | --- | --- |
| Arsenic | 150 | 0 |
| Cadmium | 0.25 | 11 |
| Chromium | 11 (Cr VI) | 0 |
| Copper | 9 | 2 |
| Lead | 2.5 | 0 |
| Mercury | 0.77 | 0 |
| Nickel | 52 | 1 |
| Selenium | 5 | NA |
| Silver | 0.12 | NA |
| Zinc | 120 | 0 |

NA = not available

For the purpose of evaluating the potential effects associated with wildlife exposure to contaminants, chemical- and receptor-specific toxicity reference values (TRVs) were compared to the calculated doses.  A TRV is defined as a dose level at which a particular biological effect may be expected to occur in an organism, based on laboratory toxicological investigations.  The calculation of TRVs commonly incorporates both toxicity-based reference doses as well as uncertainty factors to account for a wide range of limitations, including differential interspecies sensitivities.

The EPA Region IX Biological Technical Assistance Group (BTAG), in conjunction with the Navy, has developed effects-based TRVs (DTSC, 2002).  Each of these values represents a critical exposure level from a toxicological study and is supported by a published dataset of toxicological exposures and effects.  Rather than derive a single point estimate associated with specific adverse biological effects, high- and low-TRVs were derived for each receptor and contaminant to reflect the variability of parameters within an ecological risk context.  The low TRV is a conservative value consistent with a chronic, no observed adverse effects level (NOAEL).  It represents a level at which adverse effects are unlikely to occur, and is used to identify areas posing little or no risk.  Conversely, the high TRV is a less conservative estimator of potential adverse effects and is consistent with a low observed adverse effect level (LOAEL), falling approximately mid-range of all of the reported adverse effects.  The high TRV represents a level at which there is a higher potential for adverse effects to occur.  Available TRVs for birds are listed in Table A-11.

Table A-11.  Available TRVs for Birds from EPA Region IX.

| Chemical | Low TRV (mg/kg-day) | High TRV (mg/kg-day) |
| --- | --- | --- |
| Arsenic | 5.5 | 22 |
| Cadmium | 0.08 | 10.4 |
| Copper | 2.3 | 52.3 |
| DDT and metabolites | 0.009 | 1.5 |
| Lead | 0.014 | 8.75 |
| Manganese | 77.6 | 776 |
| Mercury | 0.039 | 0.18 |
| Nickel | 1.38 | 56.3 |
| PCBs | 0.09 | 1.27 |
| Selenium | 0.23 | 0.93 |
| Zinc | 17.2 | 172 |

## A.3.1 Risk Characterization

The risk characterization combines the exposure and effects assessments to provide an estimate of the potential risks to receptors.  Each receptor group is discussed in the following sections.

Protection and Maintenance of Benthic Invertebrate Communities

To assess the potential risks to benthic invertebrate communities in this screening-level risk evaluation, maximum chemical concentrations in sediment were compared to available freshwater sediment benchmarks compiled by NOAA (e.g., PELs and TELs) which are protective of benthic organisms.  With the exception of nickel, maximum concentrations of contaminants in sediment were all below PEL guidelines.  However, some metals (arsenic, cadmium, chromium, lead, and zinc) had maximum concentrations that were above the TEL guidelines.  Similarly, three PAHs (benz[a]pyrene, chrysene, and pyrene), DDT isomers (4,4’-DDD, 4-4’-DDE) and total DDx had maximum concentrations which were also above TEL guidelines.  In general, the maximum concentrations of these compounds are above levels which are considered to be “safe” (i.e., will not likely cause adverse effects); and are well below probable effect levels (PELs) which are consistent with adverse effects.  These results would indicate a low magnitude level of risk to the benthic invertebrate community.  In the absence of any site-specific sediment data, quantitative comparisons to existing conditions are not possible.  However, qualitatively, these compounds, in particular the organic ones, are relatively ubiquitous in industrialized environments.  The concentrations likely reflect regional level of contamination and would be consistent with current levels in Maurepas Swamp and Lake Maurepas.

Protection and Maintenance of Benthic and Pelagic Fish Populations

In the screening-level risk evaluation, fish populations were assessed using EPA’s NAWQC for the protection of aquatic life and Canadian water quality standards where U.S. criteria were not available.  These surface water quality criteria were compared to available data.  The majority of concentrations of metals in surface water over a six-year time span either fall below EPA’s standards or were not detected.  Exceedances were noted for cadmium (eleven exceedances), copper (two exceedances), and one exceedance occurred for nickel (Table A-10).  The low frequency of exceedances for nickel and copper would indicate that these compounds pose a low magnitude level of risk to fish and that cadmium may pose a high magnitude level of risk to fish.

Total herbicide loads in the Mississippi River at Baton Rouge did not exceed 10 µg/L during 1991-1997 (USGS, 1999).  No herbicide or metabolite was detected at a concentration exceeding 5 µg/L.  Although concentrations of atrazine, cyanazine, and metolachlor peak simultaneously in late May and early June, none of the average annual concentrations of herbicides exceeded MCLs or HAs for drinking water.

Protection and Maintenance of Bald Eagles

To assess exposure to bald eagles, a dose assessment model was developed to estimate the daily dose of chemicals to the bald eagle from the ingestion of contaminated surface water, sediment, and prey items.  Potential risk was then characterized by comparing the modeled dose estimate to chemical specific TRVs.

The estimated daily doses were calculated (Equation 1) for potential contaminants, using the available maximum concentrations, which were then compared to the high and low TRVs according to the following equation:

HQ = dose/TRV 	                                                  (Equation 2)

The dose estimates were used to derive two hazard quotients (HQs), an HQlow using the low TRV, based on NOAEL, and an HQhigh using the high TRV, based on a LOAEL, for each contaminant.  When the dose is lower than the low TRV (i.e., HQlow <1), it is likely that no risk is present from the contaminant.  When the dose exceeds the low TRV (i.e., HQlow >1), it indicates that further evaluation may be warranted.  Following EPA’s 8-step Ecological Risk Assessment process, there are three possible outcomes: (1) it is adequate to conclude that the site poses no unacceptable risks to ecological receptors; (2) the conclusions are inconclusive and the ecological risk evaluation should continue; and (3) a potential for adverse ecological effects is indicated and a more thorough analysis is warranted (EPA, 1997).

The screening-level risk assessment evaluated exposures to the bald eagle from mercury, nickel, and DDT.  The results are summarized in Table A-12.  Bald eagle exposures to all three chemicals were below the HQhigh.  However, the HQlow for mercury and total DDx exceeded 1.0, indicating low magnitude risk.  These exposures are primarily driven by the contaminant levels in fish tissue, as the main source of exposure.  Mercury levels in fish collected from Lake Maurepas and Lake Pontchartrain have generally higher tissue concentrations of mercury than fish sampled from the Mississippi River.  Therefore, the potential risk from an increase of mercury-contaminated fish entering Lake Maurepas from the Mississippi River is considered to be equal to or lower than the risk currently at the lake.  Based on available maximum concentrations, there is a low magnitude risk associated with exposure from the river; however, these risks, at least for mercury, are likely lower than present risk levels in the study area.

Table A-12.  Results of the Screening-Level Risk Evaluation to the Bald Eagle.

| Chemical | Csed (mg/kg) | BW (kg) | IRsed (kg/day) | Cfish (mg/kg) | IRfish (kg/day) | SUF | Dose (mg/kg-day) | TRVlow (mg/kg-day) | TRVhigh (mg/kg-day) | HQlow | HQhigh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mercury | 0.15 | 4.5 | 0.004 | 0.53 | 0.451 | 1 | 0.053 | 0.039 | 0.18 | 1.37 | 0.295 |
| Nickel | 45.5 | 4.5 | 0.004 | 2 | 0.451 | 1 | 0.241 | 1.38 | 56.3 | 0.17 | 0.004 |
| Total DDx | 0.023 | 4.5 | 0.004 | 0.59 | 0.451 | 1 | 0.059 | 0.009 | 1.5 | 6.57 | 0.039 |

# Conclusion

The results of this screening-level risk evaluation support the conclusions of the Mississippi River Sediment, Nutrient, and Freshwater Redistribution Study (MRSNR) (as cited in Lee Wilson & Associates, 2001).  In the MRSNR study, only a few compounds, mainly mercury and some organopesticides were found to occasionally exceed water or sediment benchmarks.  In this risk evaluation maximum concentrations in sediment for five metals (arsenic, cadmium, chromium, lead, and zinc), three PAHs (benz[a]pyrene, chrysene, and pyrene), DDT isomers (4,4’-DDD and 4-4’-DDE) and total DDx exceeded the lower sediment quality benchmark (i.e., TEL).  Concentrations of cadmium, copper, and nickel have occasionally exceeded water quality criteria.  Both mercury and DDx compounds were below available ecological benchmarks for surface water, and mercury concentrations were below sediment benchmarks.  However, both mercury and DDx compounds are bioaccumulative and slow to decompose and are, therefore, persistent in the environment.  In addition, the maximum nickel concentration in sediments exceeded the PEL guidance value.  Nickel, mercury, and DDx compounds, therefore, were assessed through a dose assessment model for the bald eagle and were found to present a potential low magnitude risk.

For the benthic invertebrate communities, exposure to the maximum concentration reported for nickel may pose a potential risk.  In addition, the maximum concentration of five metals (arsenic, cadmium, chromium, lead, and zinc), three PAHs (benz[a]pyrene, chrysene, and pyrene), DDT isomers (4,4’-DDD and 4-4’-DDE) and total DDx reported in sediment were slightly elevated and would be classified as presenting a low magnitude risk to benthic invertebrates.

The benthic and pelagic fish populations may be at risk from exposures to cadmium, and to a lesser extent copper and nickel which all exceeded water quality criteria on at least one occasion between 1995 and 2001.

Exposures to maximum concentrations of mercury and total DDx reported in sediment and prey items pose a low magnitude level of risk to bald eagles.

Atrazine

Although atrazine concentrations in the Mississippi River are of public concern due to the widespread use of this pesticide in the river basin, there is limited data for atrazine in sediments or fish tissue in the Maurepas Swamp Study Area.  Furthermore, there are no relevant literature TRVs or screening values for effects of atrazine on wildlife and/or aquatic organisms.  In addition, a study conducted by EPA (2005) concluded that atrazine is not expected to bioaccumulate into fish tissue. Thus, effects of atrazine on wildlife in the study area could not be modeled.  However, atrazine concentrations were monitored in the water column by LDEQ from the Mississippi River in March through August 1999 at the St. Francisville station.  Results ranged from 0.11 µg/L to 1.31 µg/L, which were well below the 3 µg/L Maximum Contaminant Level (MCL) for human health effects from atrazine in drinking water (USGS, 2003). In addition, the concentrations of atrazine from three stations in the Barataria Basin, located just south of Maurepas Swamp, were also relatively low (0.01 to 0.84 µg/L).

LDEQ has identified the Bayou Grosse Tete, a tributary in the Terrebonne Basin, as a stream of concern due to elevated atrazine concentrations.  Because of this stream’s location in the Basin, the waterway is hydrologically isolated from the Mississippi River, and concentrations of atrazine are believed to be the result of local inputs to the stream which is located in a corn and sugarcane-producing area (USGS, 2003).  As discussed above, atrazine is not expected to significantly bioaccumulate into fish tissue (EPA, 2005).  Therefore, it can be concluded that, although concentrations of atrazine may peak at times over the entire Mississippi River, concentrations in the river just north of the Maurepas Swamp study area in Louisiana are not likely to cause adverse effects to wildlife in the Maurepas Swamp. High concentrations of atrazine in the Terrebonne Basin of Louisiana are the result of local atrazine inputs and do not likely originate in the Mississippi River.

In the absence of surface water and sediment data from the study area, it is difficult to quantitatively determine the increase or decrease in potential risks that the diversion project may have on ecological exposures to chemical contaminants.  Qualitatively, the contaminants that exceeded relevant ecological benchmarks are generally consistent with low magnitude levels of risk.  These levels of risk are consistent with an industrialized area, and are likely consistent with regional conditions.   As such, the diversion project may not result in any significant changes from regional conditions and risks to wildlife would not likely change.  Additional investigations would be necessary to validate these assumptions.

# Uncertainty and Recommendations

This screening-level risk evaluation was performed on maximum concentrations of available surface sediment and fish tissue data from the Mississippi River which is assumed to be the source of surface water and suspended sediments for the diversion project.   The existing data set used in this evaluation is not temporally or spatially robust enough to draw any definitive conclusions.  There are six major sources of uncertainty in the data:

- Sediment and fish tissue data were obtained from different areas of the Mississippi River; some were located south of the Maurepas Swamp study area.  Although these data represent the closest sampling location to the proposed project, they may not accurately reflect the contaminant load that may be introduced through the diversion, into Maurepas Swamp.

- Fish tissue data for nickel and DDx concentrations were collected from Luling, LA. This sampling station is located south of the study area and may not adequately reflect contaminant levels in fish tissue in Maurepas Swamp after completion of the diversion project.

- Sediment and fish tissue sampling stations were not co-located, meaning they were not collected from the same station or even during the same time period.  This has significant implications in the dose model for the bald eagle which assumes the sediment and fish tissue concentrations are related.  Using different concentrations from different stations over different years introduces greater uncertainty in the exposure to contaminants.

- Available surface sediment data used in this assessment are from 1998 and are now seven years old.  These data are not likely to represent current environmental conditions.  Depositional and erosional events over the last seven years have likely changed current surface sediment contaminant concentrations.

- Data for only a limited number of potential contaminants of concern were available for surface water, sediment, and fish tissue concentrations.

- There is limited toxicity information for many contaminants.  Sediment quality benchmarks, water quality criteria, and TRVs were not available for some of the PAH compounds and pesticides/herbicides, including atrazine.

The following recommendations are provided to fill in data gaps and reduce areas of uncertainty:

- Additional sediment and surface water samples should be collected in the Mississippi River to validate the chemical exposure data compiled for this screening-level risk evaluation.  These samples should be collected from areas near the proposed diversion such that it represents a source of inputs to the Maurepas Swamp study area.  These data would reduce the temporal and spatial uncertainty in the current data set.

- Additional data collection should be considered within the study area; even a limited number of sediment and surface water samples would allow for comparative risks/benefits to be assessed quantitatively.

- Available criteria for atrazine toxicity are based on values protective of aquatic organisms.  Additional information on the toxicity of atrazine effects to upper-trophic level organisms should be evaluated to address potential public concern and the seasonal spikes of this contaminant.

- This screening-level risk evaluation assumed that the maximum concentration of contaminants in sediment and surface water samples in the Mississippi River will be present in the Maurepas Swamp, following the diversion.  This is a highly conservative assumption and does not take into account any fate and transport processes.   To resolve this uncertainty, a fate and transport model should be considered, in conjunction with the hydrologic model currently being developed for this project.

# References

Beyer, W., Conner, E., and S. Gerould.  1994.  Survey of soil ingestion by wildlife.  Journal of Wildlife Management 58: 375-382.

BirdFest.  2004.  The Great Louisiana BirdFest.  An Event of the Northlake Nature Center.  Available online: http://www.northlakenature.org/BirdFest2005/BirdFest%202004%20sightings.pdf

Day, J.W., Kemp, G.P., Mashriqui, H.S., Lane, R.R., Dartez, D., and R. Cunningham.  2004.  Development Plan for a Diversion into the Maurepas Swamp.  Final Report.  Louisiana State University, Baton Rouge, Louisiana.

DTSC (Department of Toxic Substances Control).  2002. U.S. EPA Region IX Recommended Toxicity Reference Values for Mammals and Birds.  Available online:  http://www.dtsc.ca.gov/ScienceTechnology/ftp/Eco_Btag-mammal-bird-TRV-table.pdf

EPA (US Environmental Protection Agency).  1993.  Wildlife Exposure Factors Handbook.  EPA/600/R-93/187.  December 1993.

EPA. 1997.  Ecological Risk Assessment Guidance for Superfund: Process for Designing and Conducting Ecological Risk Assessments.  Interim Final.  June.  EPA 540-R-97-OCS.

EPA.  2004.  National Recommended Water Quality Criteria.  Office of Water, Office of Science and Technology.

EPA.  2005.  Technical Factsheet on:  Atrazine.  Available at:  http://www.epa.gov/OGWDW/dwh/t-soc/atrazine.html

Handley, L., S. Hartley, J. Johnston, C. O’Neill, D. Braud and J. Snead. 2002. Biological Resources - Estuarine Living Marine Resources (ELMR) in S. Penland, A. Beall and J. Kindinger (editors), Environmental Atlas of the Lake Pontchartrain Basin: Lake Pontchartrain Basin Foundation, New Orleans, LA, U.S. Geological Survey Open File Report 02-206. Available at: http://pubs.usgs.gov/of/2002/of02-206/index.html. Last accessed 7/25/2005

Kent, R.A., B.D. Pauli, D.M. Trotter, and J. Gareau.  1991.  Canadian Water Quality Guidelines for Metolachlor:  Ottawa, Environment Canada Scientific Series No. 184.

Kent, R.A., P-Y. Tache, and B.D. Pauli.  1992.  Canadian Water Quality Guidelines for Trifluralin:  Ottawa Environment Canada Scientific Series No. 192.

LDEQ (Louisiana Department of Environmental Quality).  2001.  Mercury Contaminant Levels in Louisiana Biota, Sediments, and Surface Waters 1994-2000.  Available online:  http://www.deq.state.la.us/surveillance/mercury/2000report/index.htm

LDEQ  (1007-2004), http://www.deq.louisiana.gov/surveillance/mercury/

LDWF (Louisiana Department of Wildlife and Fisheries).  2003.  Maurepas Swamp Wildlife Management Area Description.  Available online:  http://www.wlf.state.la.us/apps/netgear/clientFiles/lawlf/files/Maurepas%20Swamp.doc

Lee Wilson & Associates, Inc.  2001.  Diversion into the Maurepas Swamps.  A Complex Project under the Coastal Wetlands Planning, Protection, and Restoration Act.  Prepared for USEPA, Region 6, Dallas, TX.  Contract No. 68-06-0067 WA #5-02.  June.

Macauley, J.M., and Summers, J.K., 1998, Environmental Monitoring Assessment Program (EMAP) Louisiana Province Database: USEPA Office of Research and Development, online at http://www.epa.gov/emap/html/dataI/estuary/data/.

NOAA (National Oceanic and Atmospheric Administration).  1999.  Screening Quick Reference Tables.  Updated September 1999.  Available online:  http://response.restoration.noaa.gov/cpr/sediment/squirt/squirt.pdf

Pauli, B.D., Kent, R.A., and M.P. Wong.  1990.  Canadian Water Quality Guidelines for Metribuzin:  Ottawa, Environment Canada Scientific Series No. 179.

Pauli, B.D., Kent, R.A., and M.P. Wong.  1991a.  Canadian Water Quality Guidelines for Cyanazine:  Ottawa, Environment Canada Scientific Series No. 180.

Pauli, B.D., Kent, R.A., and M.P. Wong.  1991b.  Canadian Water Quality Guidelines for Simazine:  Ottawa, Environment Canada Scientific Series No. 187.

Piehler, C.M.  2002.  Quantification of Dioxins, Furans, and PCBs in the Lower Mississippi River. LDEQ Office of Environmental Compliance/Surveillance Division.  October 23.

Task Force on Water Quality Guidelines, undated.  Canadian Water Quality Guidelines. Chapter 3, Freshwater Aquatic Life:  Ottawa, Environment Canada.

Trotter, D.M., R.A., Kent, and M.P. Wong.  1989.  Canadian Water Quality Guidelines for Carbofuran:  Ottawa, Environment Canada Scientific Series No. 169.

Trotter, D.M., Baril, A., Wong, M.P., and R.A. Kent.  1990.  Canadian Water Quality Guidelines for Atrazine:  Ottawa, Environment Canada Scientific Series No. 168.

USACE (US Army Corps of Engineers).  1984.  Freshwater Diversion to Barataria and Breton Sound Basins.  Feasibility Study.  Louisiana Coastal Area, Louisiana.  US Army Corps of Engineers, New Orleans District.

USGS (US Geological Survey).  1999.  Discharge of Herbicides from the Mississippi River Basin to the Gulf of Mexico, 1991-1997.  USGS Fact Sheet FS-163-98.  April.

USGS.  2002.  Biomonitoring of Environmental Status and Trends (BEST) Program:  Environmental Contaminants and their Effects on Fish in the Mississippi River Basin.  Station 15.  USGS/BRD/BSR – 2002-0004.  Available at:  http://www.cerc.usgs.gov/data/best/search/statsrch.asp?basin=Mississippi&station=15

USGS.  2003.  Atrazine in Southern Louisiana Streams, 1998-2000.  USGS Fact Sheet FS-011-03.  March 2003