ARSR-4/BI-6 Collimation Instructions

Procedure 1:  Collimation with either rotary joint

- Registration must be performed before this procedure.  This procedure requires downtime.

- At the BI-6, run collimation with beacon 1/2 nautical mile offset enabled to determine search azimuth and range offset.

- Calculate search azimuth offset in iACP (16384 iACP per 360 degrees).

- At the LDC, log in and take control.

- Go to menu 5.1.  Add search azimuth offset to both NORTHMARK OFFSET FOR APG1 and NORTHMARK OFFSET FOR APG2.  Increasing these numbers moves the search targets clockwise.  Change only the highlighted numbers shown in Figure 1.

- Hit CTRL-D and change the offsets.

- In the BEACON/RADAR AZIMUTH OFFSET field, reenter the existing number.  All three parameters should have an asterix to the side indicating they have been changed.

- Hit CTRL-C to get back to the command mode.

- Hit the LOCAL PRINT button to print this menu.

- Select option 1 to load changes.

- Go to menu 1.2.5.

- Swap synchronizers by putting the one that is online into STBY.  This step is necessary to force the ARSR-4 to load the new northmark offsets into the hardware.

- Go to menu 5.2.5.3, RANGE COMPUTATION PARAMETERS.

- For each RANGE BIAS number, add the search collimation range offset in NMI.

- Hit CTRL-D and change the offsets.  Change only the numbers highlighted in Figure 2.

- Hit CTRL-C to get back to the command mode.

- Hit the LOCAL PRINT button to print this menu.

- Hit CTRL-T.  Select option 9 to load the range bias changes.

- The search collimation parameters have now been changed.  These cannot be left in permanently until the ATCBI-6 is commissioned and the ATCBI-5 is de-commissioned.

- A cold start must be performed before the ARSR-4 is restored to service.

Figure 1.  Menu 5.1, Search Azimuth Collimation Parameters

5.1 ANTENNA PARAMETERS

NORTHMARK OFFSET FOR APG1:    -79   (-2048 TO 2047 iACPs)

NORTHMARK OFFSET FOR APG2:    -122  (-2048 TO 2047 iACPs)

BEACON/RADAR AZIMUTH OFFSET:    135 (0 TO 137 iACPs)

TEST TARGET CABLE LOSSES FOR MDS CALCULATION  (0 TO 99.9 dB)

BEAM   ROW      dB LOSS         BEAM   ROW      dB LOSS

F1     F2                       F1     F2

LD     2     59.0   58.1         5    13     58.0   61.2

1     8     59.0   59.6         6    14     57.8   59.9

2     9     57.8   59.2         7    16     58.7   59.9

3    10     57.7   59.4         8    18     58.5   59.2

4    11     58.5   59.4         9    21     58.8   59.2

1. LOAD CHANGES

2. PURGE CHANGES

Figure 2.  Menu 5.2.5.3.  Search Range Collimation Parameters

5.2.5.3 RANGE COMPUTATION PARAMETERS

RANGE RESOLUTION

RANGE BIAS                    THRESHOLD

(-5 TO 5 NMI @ 0.0625)        (0 TO 48 dB @ 0.09375)

LOW STACK:

MODE:     FIXED     BAM      PAM       FIXED      BAM       PAM

VIP  NORMAL  0.0000   0.0000   0.0000     9.00000   9.00000   9.00000

VIP  MTI     0.0000   0.0000              9.00000   9.00000

STAC NORMAL  0.0000   0.0000   0.0000     9.00000   9.00000   9.00000

STAC MTI     0.0000   0.0000              9.00000   9.00000

HIGH STACK:

MODE:     FIXED     BAM      PAM       FIXED      BAM       PAM

VIP  NORMAL  0.0000   0.0000   0.0000     9.00000   9.00000   9.00000

STAC NORMAL  0.0000   0.0000   0.0000     9.00000   9.00000   9.00000

HELP      VIEW   GO TO  ENTER                                  RUNLG

ALARMS MAIN   DATA                                   ENAB