Nd-147

Reviewed by T. Nakagawa, JAERI, March 2004

1. Files

ENDF/B-VI:	Evaluation was made by Schenter et al. in 1974. The data below 35 eV were revised by Wright et al. in 1988.

JENDL-3.3 and JENDL-3.2:

New evaluation was made by JNDC FP Nuclear Data WG for JENDL-3.1 in 1990 [Ka92]. JENDL-3.3 and JENDL-3.2 are the same as JENDL-3.1.

JEFF-3.0 and JEF-2.2:

The data of ENDF/B-V was adopted to JEF-2. The resonance parameters were updated with recommendation of Mughabghab et al. [Mu81]. Charged particle emission cross sections were adopted from REAC-ECN-4. JEFF-3.0 is the same as JEF-2.2.

CENDL-3:	New evaluation was made by Qing-Biao Shen in 2000 [Sh01].

BROND-2:	No data are given.

2. Thermal and resonance region

ENDF/B-VI

Resonance parameters recommended by Mughabghab et al. [Mu81] were adopted.

No unresolved resonance parameters were given.

JENDL-3.3 and JENDL-3.2

Resolved resonance parameters were based on the recommendation of Mughabghab et al. [Mu81]. A negative resonance was added so as to reproduce the thermal capture cross section [Mu81].

The unresolved resonance parameters were determined from optical model and statistical model calculation with CASTHY code.

JEFF-3.0 and JEF-2.2

Resolved resonance parameters were based on the recommendation of Mughabghab et al. [Mu81]. No unresolved resonance parameters were given.

CENDL-3

Resonance parameters were taken from JENDL-3.2, and modified.

Recommendation/conclusion for the thermal and resonance region

Below 35 eV, all the evaluations are in the same quality. Considering connection to the fast neutron region, CENDL-3 is recommended.

3. Fast neutron region

(1) Methodology

ENDF/B-VI

Theoretical calculations were performed using OMP of Moldauer [Mo63], codes of COMNUC-3 for inelastic and NCAP for capture. No threshold reactions were condidered. Angular distributions of inelastic neutrons were assumed to be isotropic.

JENDL-3.3 and JENDL-3.2

Theoretical model calculations of total, elastic and inelastic scattering, and capture were made by using the statistical model code CASTHY. The direct/semi-direct capture was calculated from a simple formula of Benzi and Reffo [Be69].

Other reaction cross sections were calculated with the preequilibrium and multi-step evaporation code PEGASUS. The (n,p) and (n,) reaction cross sections were normalized to the systematics of Forrest [Fo86] at 14.5 MeV. Sum of the threshold reaction cross sections were considered as a competing reaction cross sections in the CASTHY calculation.

JEFF-3.0 and JEF-2.2

The total, inelastic scattering and capture cross sections above 35 eV are the same as ENDF/B-VI. However, the interpolation of the capture cross section below 1 keV specified in JEFF (INT=2) is not correct. Charged-particle production data were taken from REAC-ECN-4 library.

CENDL-3

Theoretical calculation was performed with SUNF and DWUCK4 codes.

(2) Total cross section (MT=1)

See Fig: http://www.nndc.bnl.gov/sg21/fp21/htm/Nd147.htm#mt1, Fig. A1

No experimental data are available for Nd-147. Figure A1 shows the comparison with the experimental data for natural Nd. If the data of Nd-147 are almost the same as those of natural Nd, JENDL-3.3 might be incorrect.

(3) Capture cross section (MT=102)

See Fig: http://www.nndc.bnl.gov/sg21/fp21/htm/Nd147.htm#mt102, Fig. A2

Evaluated data are compared with each other in Fig. A2. Averaged cross sections are shown in the resolved resonance region. No experimental data are available.

Maxwellian-averaged cross section was recommended by Toukan et al. [To95]. Table 3 shows comparison of Maxwellian-averaged cross sections at kT=30 keV. CENDL-3 is the closest to the data of Toukan et al.

(4) Elastic scattering cross section (MT=2)

See Fig: http://www.nndc.bnl.gov/sg21/fp21/htm/Nd147.htm#mt2

(5) Inelastic scattering cross section (MT=4, 51 – 91)

See Fig: http://www.nndc.bnl.gov/sg21/fp21/htm/Nd147.htm#mt4

Discrete levels considered in the evaluations are as follows:

ENDF/B-VI	3 levels up to 310 keV

JENDL-3.3(=JENDL-3.2)	12 levels up to 792.6 keV

JEFF-3.0 (=JEF-2.2)	3 levels up to 310 keV, same as ENDF/B-VI

CENDL-3	10 levels up to 631.5 keV

The direct inelastic was calculated for CENDL-3 with DWUCK4 code.

Cross section shape of ENDF/B-VI and JEFF-3.0 is not reasonable.

(6) (n,2n) reaction cross section (MT=16)

See Fig: http://www.nndc.bnl.gov/sg21/fp21/htm/Nd147.htm#mt16

CENDL-3 and JENDL-3.3 give this reaction cross section.

(7) (n,p) reaction cross section (MT=103)

See Fig: http://www.nndc.bnl.gov/sg21/fp21/htm/Nd147.htm#mt103

(8) (n,) reaction cross section (MT=107)

See Fig: http://www.nndc.bnl.gov/sg21/fp21/htm/Nd147.htm#mt107

(9) Other reactions

The following reaction cross sections are given:

ENDF/B-VI	no other reactions

JENDL-3.3 (JENDL-3.2)	(n,3n), (n,n), (n,np), (n,nd), (n,nt), (n,d), (n,t)

JEFF-3.0 (JEF-2.2)	(n,d), (n,t), (n,3He), (n,2p)

CENDL-3	(n,3n), (n,n), (n,np), (n,d), (n,t)

Recommendation/conclusion for the fast energy region

CENDL-3 is recommended.

References

[An79]	Anufriev V.A. et al.: 1979 Koxville, p.907 (1979).

[Be69]	Benzi V. and Reffo G.: CCDN-NW/10 (1969).

[Fo86]	Forrest R.A.: AERE-R 12419 (1986).

[Ka92]	Kawai M. et al.: J. Nucl. Sci. Technol., 29, 195 (1992).

[He74]	Heck D. et al.: Atomkernenergie, 24, 141 (1974), EXFOR20556.

[Mo63]	Moldauer P.: Nucl. Phys., 47, 65 (1963).

[Mu81]	Mughabghab S.F. et al.: Neutron Cross Sections, Vol.1, Part A, Academic Press (1981).

[Sh01]	Shen Q.: Comm. Nucl. Data Prog., 25, 29 (2001).

[To95]	Toukan K.A. et al.: Phys. Rev., C51, 1540 (1995).

Table 1  Summary for Resolved and Unresolved Resonance Region: Nd-147

-----------------------------------------------------------------------------------------------------------------

Library	RRP	URP	Remarks

-----------------------------------------------------------------------------------------------------------------

ENDF/B-VI	35 eV, SLBW	no urp

JENDL-3.3	36 eV, MLBW	0.036 – 100 keV	same as JENDL-3.2

JEFF-3.0	35.7 eV, MLBW	no urp	same as JEF-2.2

CENDL-3	35.2 eV, MLBW	0.0352 – 100 keV

---------------------------------------------------------------------------------------------------------------

Table 2. Thermal Cross Sections and Resonance Integral: Nd-147

Units: barns

-----------------------------------------------------------------------------------------------------------------

Library/Experiment	Total	Elastic	Capture	Res.Integ.

-----------------------------------------------------------------------------------------------------------------

ENDF/B-VI	524.10	84.21	439.89	574

JENDL-3.3	510.29	79.28	431.02	631

JEFF-3.0	545.58	104.795	439.90	606

CENDL-3	520.31	80.33	439.99	655

Mughabghab et al. [Mu81]			440150	405*1

Heck et al. [He74]			440150*2

Anufriev et al. [An79]				21090*3

-----------------------------------------------------------------------------------------------------------------

*1	Calculated.

*2	Measured in reactor spectrum.

*3	Calculated from their resonance parameters.

Table 3. Maxwellian averaged capture cross section of Nd-147

-----------------------------------------------------------------------------------------------------------------

Library/Experiment	cross section at kT=30 keV(mb)

-----------------------------------------------------------------------------------------------------------------

ENDF/B-VI	642

JENDL-3.3	1002

JEFF-3.0	648

CENDL-3	609

Toukan et al. [To95]	550150*1

-----------------------------------------------------------------------------------------------------------------

*1	calculated.

Fig. A1  Nd-147 and Nd-natural total cross section

Fig. A2  Nd-147 capture cross section