189.1  PURPOSE  To establish requirements and procedures for Bonneville Power Administration’s  Explosives Program as established under Federal regulations and industry standards.  Consistent with Delegation Order No. 0204-161, signed by the Secretary of Energy on October 7, 1996,  exempts BPA from the provisions of any related Department of Energy issuance, including DOE O 420.1, Facility Safety; and DOE O 440.1, Worker Protection.

189.2  SCOPE  The provisions of this chapter apply to all BPA organizations performing work involving explosives.

189.3  POLICY  Requirements for the safe handling, storage and use of explosives and accessories used for detonation, shall be met  to maintain compliance with Federal requirements.  Where conflicts in the application of these codes and standards arise, the more restrictive requirements shall apply.

189.4  DEFINITIONS

A.	Approved Storage Facility:  A facility/magazine used to store explosive materials.

B.	Blasting Machine:  A  handheld instrument used with shock tubing  to initiate a detonator (cap).

- Charge:  An explosive load.

- Detonator:  The device used for initiating the implosive fittings also known as “caps”.

E.	Detonating Cord:  A flexible cord containing a center core of high explosives when detonated, will have enough strength to detonate other cap-sensitive explosives.

- Explosive:  Any chemical compound, mixture, or device that functions by the instantaneous release of gas and heat, unless such compound, mixture or device is otherwise specifically classified.

G. Extraneous Electricity:  The unwanted electrical energy entering blasting circuits.

H.	Flyrock:  Rocks propelled from a blast area by the force of the explosion.

I.	Initiation:  The act of detonating explosives.

- Millisecond Delay Caps:  Detonators (caps) that have a built-in delay element, usually 0.025 second apart consecutively.

- Misfires:  An explosive charge, or part of an explosive charge that fails to detonate as planned.

- Premature:  The charge detonates before intended.

- Spacing:  The distance between charges in a group.

189.5  ELEMENTS

- Implosive fittings and their associated components for detonation shall be the only approved explosive devices for use by BPA personnel.

- Storage locations are designated by the Transmission Business Line’s Transmission Field Services organization.

- Storage of Implosive fittings shall be in approved type 4 outdoor magazines in accordance with NFPA  and ATF storage regulations.

- Storage of detonators shall be in approved type 2 outdoor magazines in accordance with NFPA  and ATF storage regulations.

- Transportation of Implosive fittings to work locations shall be in approved type 3 magazines in accordance with NFPA  and ATF storage regulations.

- Transportation of detonators to work locations shall be in approved IME-22 rated detonator boxes in accordance with ATF storage regulations and Institute of the Makers of Explosives (IME).

- Blasting machines shall be a standardized model with a set number for each storage location.  In addition one set of replacement parts for the blasting machine shall be kept in inventory in each storage location.

- Each detonation that is within one mile of any populated area shall be recorded for vibration and air blast (dB).

- Two qualified employees shall be required for each blasting operation.

- A formal 16-hour training/certification on proper use and storage shall be required on a two-year basis. Upon completion of training each employee shall be required to pass a written examination.

- Proper documentation of inventories of Impo fittings and detonators along with usage, security inspections and blasters shot logs shall be maintained at all storage locations in accordance with ATF storage regulations.

- Inventory levels of all Implosive fittings and detonators for each storage location shall be established.

- Purchasing of explosives and associated equipment for detonation shall be accomplished through  established guidelines under BPA procurement policies.

- The Safety Office shall have oversight responsibilities for the BPA Explosives Program.  BPA shall adhere to the following agencies requirements; Alcohol, Tobacco and Firearms P 5400.7 (ATF), Department of Energy orders 440.1-1 (DOE), National Fire Protection Agency 495 (NFPA), Department of Transportation  (DOT) and Institute of the Makers of Explosives (IME).

189.6  RESPONSIBILITIES

- The Transmission Business Line shall provide proper tools, equipment and training outlined in this policy for all employees involved in explosive work.

- The Safety Office will have oversight of all explosives policies and practices that would affect employee safety and hazards to facilities and contents.  The Safety Office will audit the program to assure compliance with established regulations.

- Qualified BPA employees shall be trained and certified to perform blasting operations using implosive fittings.  These qualified personnel shall ensure proper and timely documentation as outlined in the policy for all storage locations and blasting operations.

  - DOCUMENTATION:

- All handling and movement of implosive sleeves and detonators shall be recorded on BPA form 5480.22e.  Each different manufacture must be listed on it’s own form.  Shock tube does not require documentation.

- A physical inventory of each product shall be preformed and noted on their respective inventory form annually.

- Weekly security inspections of each magazine shall be noted on each inventory form.

- All Inventory/Inspection forms shall be kept in a  file at the location of each magazine.  This file shall be accessible by all qualified employees, Regional Management and the Safety Office.

- All forms shall be kept on file three years after the date of the last entry on the form.

- A Blasters Log, BPA form 5480.21e, shall be filled out with all detonations and kept on file at the District performing the work.  In addition to the Blasters Log, a copy of the vibration/sound blast recording shall be attached to each Log.  This documentation shall be kept for three years from the date of the blasting operation.

  - TRANSPORTATION:

- Implo fittings shall be transported in a type 2 or type 3 magazine to and from the job sites and between storage locations.  Magazines shall be sufficiently secured to prevent movement or loss of cargo while in transit.

- Two 2.8 pound type 1A-10-B,C fire extinguishers shall be required for each transport vehicle during all transportation operations.

- An updated copy of the Inventory/Inspection form 5480.22e  to identify  what explosive components  are  being transported  shall be in the transport vehicle.

- A copy of the Material Safety Data Sheet for Implosive sleeves and detonators shall be in the transport vehicle.

- Vehicles transporting any quantities of explosive shall not be left unattended.

- While crews are in PD status, Implosive fittings shall be stored in a type 2 magazine only.

189.9  REFERENCES:  The following references contain additional criteria and guidelines pertaining to the BPA Explosive Program

- National Fire Protection Association (NFPA) Codes and Standards

- DOE Explosives Safety Manual, DOE/M440.1-1

- BPA Safety and Health Program Handbook

D.	Code of Federal Regulations (CFR) 29, Part 1910.109

E.	CFR 29, Part 1926.900

F.	Local and state fire protection criteria

G.	Alcohol, Tobacco and Firearms P 5400.7 (ATF)

H.	Department of Transportation (DOT)

- Institute of the Makers of Explosives (IME)

# J.  IMPLO Installation Instructions