46-Pd-105 Review

Keiichi SHIBATA, JAERI Nuclear Data Center, December 4, 2002

1.  Files

BROND-2: Year of origin 1984

CENDL-3: Year of origin 1999

ENDF/B-VI: Year of origin 1999

JEF-2.2: Year of origin 1983

JENDL-3.2: Year of origin 1990.

2.  Thermal and Resonance Region

BROND-2:  Resolved resonance region below 2 keV; unresolved resonance region between 2 and 283.2 keV.

CENDL-3:  taken from JENDL-3.2

ENDF/B-VI:  Resolved resonance region below 2.0535 keV; unresolved resonance region.between 2.0535 and 283.207 keV.

JEF-2.2:  Resolved resonance region below 2.06 keV; unresolved resonance region between 2.06 and 20 keV.

JENDL-3.2: Resolved resonance region below 2.0485 keV; unresolved resonance region between 2.0485 keV and 100 keV.

Thermal cross sections and resonance integral for Pd-105 (300K)

|  | Total (b) | Elastic (b) | Capture (b) | Cap. R.I. (b) |
| --- | --- | --- | --- | --- |
| JENDL-3.2 | 2.54045E+01 | 5.15146E+00 | 2.02530E+01 | 9.58012E+01 |
| ENDF/B-VI | 2.82420E+01 | 7.36076E+00 | 2.08813E+01 | 9.52024E+01 |
| JEF-2.2 | 2.70754E+01 | 5.24572E+00 | 2.18297E+01 | 9.30789E+01 |
| CENDL-3 | 2.61769E+01 | 5.15146E+00 | 2.10254E+01 | 9.30789E+01 |
| BROND-2 | 2.62892E+01 | 4.35775E+00 | 2.19315E+01 | 1.08696E+02 |
| BNL-325 |  | 5.00.6 | 20.03.0 | 98 |
| Mughabghab01 |  |  | 21.01.5 |  |

Recommendation

The total cross section of ENDF/B-VI is abnormally enhanced at the upper end of the unresolved resonance region.  This problem would be solved by inserting the 283.207 keV point with ~10-5 b into MF/MT=3/1.  There are discontinuities between the unresolved resonance and fast-energy regions for the total cross sections adopted in CENDL-3 and BROND-2.  The data of JENDL-3.2 and JEF-2.2 are recommended.

3.  Fast Neutron Region

  - Methodology

BROND-2

Taken from ENDF/B-V except for the capture cross section.  As for the capture cross section, the evaluation is based on experimental data in the energy region from 283.2 – 700 keV; the cross sections are smoothly matched to JENDL-1 between 0.7 and 2 MeV; the cross sections were taken from JENDL-1 between 2 and 8 MeV; DSD calculations were performed above 8 MeV.

CENDL-3

Optical and statistical model calculations were performed.  The direct interaction was considered for the inelastic scattering using DWBA.  The library contains the cross sections for the (n,n’), (n,2n), (n,3n), (n,n), (n,np), (n,), (n,p), (n,d), (n,t), (n,3He), and (n,) reactions together with the total and elastic scattering cross sections.  The elastic scattering cross section was obtained by subtracting a sum of partial cross sections from the total cross section.

ENDF/B-VI

Optical and statistical model calculations were performed.  The direct interaction was considered for the inelastic scattering to four levels by using the coupled-channel method.  The library contains the cross sections for the (n,n’), (n,2n), (n,3n), (n,4n), (n,n), (n,2n), (n,np), (n,nd), (n,2np), (n, ), (n,p), (n,d), (n,) reactions as well as the total and elastic scattering cross sections.

JEF-2.2

No detailed information is given on the evaluation method.  The library contains the cross sections for the (n,n’), (n,2n), (n, ), (n,p), (n,d), (n,t), (n,3He), (n,), and (n,2p) reactions as well as the total and elastic scattering cross sections.

JENDL-3.2

Optical and statistical model calculations were performed.  The library contains the cross sections for the (n,n’), (n,2n), (n,3n), (n,n), (n,np), (n,nd), (n, ), (n,p), (n,d), (n,t), (n,3He), and (n,) reactions together with the total and elastic scattering cross sections..  The DSD process was considered for the capture reaction in a phenomenological way.  The elastic scattering cross section was obtained by subtracting a sum of reaction cross sections from the total cross section.

  - MT=1

The cross sections of BROND-2 and JEF-2,2 are too high around 20 MeV as compared with the measured total cross section of elemental Pd.

  - MT=2

I cannot judge which the best evaluation is.

  - MT=4

The excitation function of BROND-2 is very different from those of the other libraries.

  - MT=16

I cannot judge which the best evaluation is.

  - MT=102

All evaluations reproduce experimental data available below several hundred keV.  However, above several hundred keV, all evaluations have different cross section shapes.

  - MT=103

I cannot judge which the best evaluation is.

  - MT=107

I cannot judge which the best evaluation is.

Recommendation

I would recommend JENDL-3.2, although direct interaction was not considered for the inelastic scattering.  If the problem of ENDF/B-VI mentioned in Sec. 2 were solved, the ENDF/B-VI data would be better than JENDL-3.2.