Format Specifications For LVL Data Collection File

Record Sequence:

Each days running should begin with an Instrument record.  Should the rods or 	instrument change during the day, a new Instrument record must be created.

Next, a collimation record must be made.  Should the collimation change, or 	should a new instrument be used during the day, a new collimation record must be 	created.

To begin a section, a Beginning record must be created.

Observations from each instrument set-up during a section running are recorded 	in a set-up record.

When a section is completed, record all ending information using the End 	leveling record.  Should the running be aborted, or later rejected later, place 	an ‘F’ in col 89 for field rejection.

Format Key

a – alpha-numeric

n - numeric

yy - year

mm - month

dd - day of year

Instrument Record – I

Example:

12345678901234567890123456789012345678901234567890123456789012345678901234567890

I 246 103252   100 F 397 12975           397 12976           CM 2.00 C A 2 1

Fields:

Column	Name				Format 		Range			Example

1		Record Type			a			I			I

3-5	Instrument Code		nnn			242,243,244		243

7-14	Inst. Serial Num.		aaaaaaaa		alpha-numeric	103252

16-18	Inst Stadia Factor	nnn			100 to 600		100

20		Inst Stadia Code		F,H			F,H			F

22-24	Rod 1 Code			nnn			300-399		397

26-33	Rod 1 Serial Num.		nnnnnnnn		alpha-numeric	12975

35-40	Rod 1 Constant		nnnnnn		0-999.99		0

42-44	Rod 2 Code			nnn			300-399		397

46-53	Rod 2 Serial Num.		nnnnnnnn		alpha-numeric	12976

55-60	Rod 2 Constant		nnnnnn		0-999.99		0

62-63	Rod Units			aa			CM or MT		CM

65-68	Height of Probe		nn.n			0<>2			2.00

70		Temp Scale			C,F			C,F			C

72		Time Zone			a			see below		A

74		Order				n			1,2,3			2

76		Class				n			0,1,2			1

Instrument Code – NGS defined code for specific instrument.  Choices are limited to codes for three manufacturers, Topcon, Wild/Leica, and Zeiss/Trimble.

The current codes are:

242 - Leica NA2002

243 - Leica NA3000, NA3003, DNA03

244 – Topcon DL-101

245 – Topcon DL102C

246 – Zeiss – All

Instrument Serial Number – Serial number stored digitally within leveling instrument or written on instrument.

Instrument Stadia Factor – Determined by instrument type and defined in INST.DAT

Instrument Stadia Code – H for Half, F for Full

Rod 1 Code – NGS specified code.  Defined in RODS.DAT based on rod serial number.

Rod 1 Serial Number – Manufacturer specified.  Written on rod.

Rod 1 Constant – Zero for digital levels

Rod 2 Code – NGS specified code.  Refer to Bluebook Annex F

Rod 2 Serial Number – Manufacturer specified.  Written on rod.

Rod 2 Constant – Zero for digital levels

Rod Units – Centimeter or Meter

Temperature Scale – Celsius or Fahrenheit

Time Zone – Based on US Navy Time Zone Designations:

Atlantic – Q

Eastern – R

Central – S

Mountain – T

Pacific – U

Yukon – V

AK/HI – W

GMT - Z

Order and Class – Valid combinations: 11,12,21,22,30

Collimation Record – C

Example:

12345678901234567890123456789012345678901234567890123456789012345678901234567890

C 246 103252   0927 050819  0.047   30.5  C

Fields:

Column	Name				Format 		Range			Example

1	Record Type	a	C			C

3-5	Instrument Code		nnn			242,243,244		243

7-14	Inst. Serial Num.		aaaaaaaa		alpha-numeric	103252

16-19	Time				hhmm						0927

21-26	Date				yymmdd					050819

28-33	Collimation			sn.nnn		-0.050-0.050	 0.047

35		Collimation			Y,N <blank>		Y,N, <blank>	<blank>

37-41	Collimation Temp.		nnn.n						030.5

43		Temperature Units		C,F			C,F			C

45-80	Comments			a/n

Collimation - The maximum collimation error for single line of sight is 0.05 mm/m for all order and class except third order which is 0.10 mm/m.  The maximum collimation error for reversible compensator-type instruments, mean of two lines of sight is 0.02 mm/m for all order and class except third order which is 0.04 mm/m.

Beginning Section Record – B

Example:

12345678901234567890123456789012345678901234567890123456789012345678901234567890

B FWM FWM    8 A 197                      30.6 1 1 0927 050819 1  2.0 0.2 0.5 O

Fields:

Column	Name				Format 		Range			Example

1		Record Type			a			B			B

3-5	Observer Initials		aaa			2-3 char		FWM

7-9	Recorder Initials		aaa			2-3 char		GWA

11-14	Survey Point Num		nnnn			1-9999		8

16-41	Mark Designation		a/n						A 197

43-46	Temperature			nnn.n						30.6

48		Wind Code			n			0,1,2			1

50		Sun Code			n			0,1,2			1

52-55	Time				hhmm						0927

57-62	Date				yymmdd					050819

64		Rod On Mark			n			1,2			1

66-69	Height Of Inst		n.nn			1.0<>3.0		2.0

71-73	Lower Probe Ht.		n.n						0.2

75-77	Upper Probe Ht.		n.n						0.5

79		(P)redicted/		P,O			P,O			O

(O)bserved Temps

Survey Point Number – User defined numeric identifier of the starting benchmark.

Designation – alpha-numeric description of the benchmark

Rod On Mark – Rod that is on the mark. Important to ensure that NGS requirement of an even number of setups is followed

Probe Ht – Height of the temperature probe on the tripod

(P)redicted/(O)bserved Temps – Defines whether temperatures were recorded or predicted

Setup Record – S

Example:

12345678901234567890123456789012345678901234567890123456789012345678901234567890

S  1 1.32111  2 0.00 14.13 0.77597  2 0.00 14.05  31.0  30.6

Fields:

Column	Name				Format 		Range		Example

1		Record Type			a			S		S

3-4	Setup Number		nn			1-99		1

6-12	BS Rod Reading		n.nnnnn				1.32111

14-15	Num. Measurements	nn						2

17-20	Setup Imbalance (mm)	n.nnn		    	2.0–10.0	0.00

22-26	BS Distance (m)		nn.nn			<90.0		14.13

28-34	FS Rod Reading		n.nnnnn				0.77597

36-37	Num. Measurements		nn					2

39-42	Total Imbalance (mm)	n.nnn			4.0-10.0	0.00

44-48	FS Distance (m)		nn.nn			<90.0		14.05

50-54	Lower Probe Temp.		nnn.n					31.0

56-60	Upper Probe Temp.		nnn.n					30.6

62-90	Comments			a/n

Setup Imbalance – Computed difference in the foresight and backsight distance of the setup

Total Imbalance – Computed difference in the foresight and backsight distance of the section

Num. Measurements – The number of measurements the instrument takes continuously to determine the rod reading.  Defined within instrument settings

Ending Record – E

Example:

12345678901234567890123456789012345678901234567890123456789012345678901234567890123456

E    9 RENA 2                     32.6 0 2 1011  14   1.0  720.0    9.52686  31.9  31.2

Fields:

Column	Name				Format 		Range		Example

1		Record Type			a			E		E

3-6	Survey Point Num.		nnnn			1-9999	9

8-33	Designation			a/n					Rena 2

35-38	Temperature			nnn.n			-30.0<>110. 32.6

40		Wind Code			n			0,1,2		0

42		Sun Code			n			0,1,2		2

44-47	Time				hhmm					1011

49-51	Number Of Setups		nnn					14

53-57	Stadia Imbalance		nnn.n			-90.0<>90.0	1.0

59-64	Section Length		nnnn.n		4.0<>3100.0	720.0

66-75	Elevation diff.		nnnn.nnnnn		-1000<>1000	9.52686

77-81	Mean Low Probe Tmp	nnn.n					31.9

83-87	Mean Upr Probe Tmp	nnn.n					31.2

Stadia Imbalance – Final difference between backsight and foresight distance at the end of the section

Elevation diff. – Difference in elevation between beginning and ending benchmark

Section length – Length in meters of the section

Error checking during observations

The following errors are flagged based on order and class as defined in:

FGCS Specifications and Procedures to Incorporate Electronic Digital/Bar-Code Leveling Systems

http://www.ngs.noaa.gov/FGCS/tech_pub/Fgcsvert.v41.specs.pdf

Order				1		1		2		2		3

Class				1		2		1		2

Collimation Exceeds Tolerance  X.XX mm/m where X.XX –

0.05		0.05		0.05		0.05		0.10

Setup Imbalance Exceeds X.X meters if foresight and backsight distances differ more than X.X meters where X.X =

Per setup (m)			2.0		5.0		5.0		10.0		10.0

Per section (m)			4.0		10.0		10.0		10.00		10.0

Maximum sight distance (m)	50		60		60		70		90

deltah1 – deltah2 for one setup	0.30		0.30		0.60.		0.70		1.30

not to exceed (mm) for MDS

The following general errors and warnings are flagged during observations:

Temperature Gradient Exceeded if top and bottom probe temperatures > 1.0 and < -3.0 degrees.

Low shot on rod if reading is taken below the 0.5m mark.