| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 13 to Document 41-E |
|  | 22 August 2007 |
|  | Original: English |
| Asia-Pacific Telecommunity Administrations |  |
| common proposals for the work of the conference |  |
| Agenda item 1.13 |  |

1.13	taking into account Resolutions 729 (WRC-97), 351 (WRC-03) and 544 (WRC-03), to review the allocations to all services in the HF bands between 4 MHz and 10 MHz, excluding those allocations to services in the frequency range 7 000-7 200 kHz and those bands whose allotment plans are in Appendices 25, 26 and 27 and whose channelling arrangements are in Appendix 17, taking account of the impact of new modulation techniques, adaptive control techniques and the spectrum requirements for HF broadcasting

Introduction

This is a very broad agenda item, as there are multiple services allocated to the HF bands between 4 MHz and 10 MHz, including aeronautical mobile (OR), aeronautical mobile (R), amateur, broadcasting, fixed, maritime mobile and mobile services. Agenda item 1.13 covers three Resolutions that deal with five separate issues.

Resolution 729 (WRC-97):  This Resolution calls for a review of frequency adaptive techniques for fixed and mobile services in the MF and HF bands. The first part addresses the need to automatically limit simultaneous use of frequencies to the minimum necessary. The second part addresses the need to evaluate the channel occupancy prior to and during operation, with a view to avoiding unintentional harmful interference.

Resolution 351 (WRC-03):  This Resolution calls for a review of the frequency and channel arrangements for the maritime mobile service in the MF and HF bands. Particularly, this review addresses RR Appendix 17 with a view to improving efficiency by considering the use of new digital technology by the maritime mobile service.

Resolution 544 (WRC-03):  This Resolution addresses the identification of additional spectrum for the broadcasting service in the HF bands. It specifically notes a spectrum deficiency of at least 250 kHz, up to 800 kHz, for the broadcasting service in bands below 10 MHz.

Review of allocations:  The task is to review the allocations to all services in the frequency range 4-10 MHz, except those that are excluded, with a view to address those issues outside of Resolutions 729 (WRC-97), 351 (WRC-03) and 544 (WRC-03) and those elements where the Resolutions are interrelated. There is a difference of opinion on the scope of this issue.

# Resolution 729

APT supports no change to the Table of Frequency Allocations to satisfy Resolution 729 (WRC-97) and Resolution 729 (WRC-97) should be updated in accordance with completion of studies.

MOD	ASP/41A13/114

RESOLUTION  729  (Rev.WRC-07)

Use of frequency adaptive systems in the MF and HF bands

The World Radiocommunication Conference (Geneva, 2007),

considering

...

d)	that the Radiocommunication Bureau no longer undertakes examination with respect to the probability of harmful interference caused by new assignments recorded in the Master International Frequency Register (MIFR) in the non-planned bands below 28 MHz;

...

-

Reasons:	Further studies are not necessary.

# Resolution 351

APT supports modification of Appendix 17 to provide frequency bands for data transmission of maritime mobile service and the continued use of Morse telegraphy should be allowed.

APPENDIX  17  (Rev.WRC-03)

Frequencies and channelling arrangements in the
high-frequency bands for the maritime mobile service

(See Article 52)

MOD	ASP/41A13/115

PART  A  –  Table of subdivided bands     (WRC-07)

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service

| Band (MHz) | 4 | 6 | ... |
| --- | --- | --- | --- |
| Limits (kHz) | 4 063 | 6 200 |  |
| Frequencies assignable to ship stations for oceanographic data transmission c) | 4 063.3 to 4 064.8 6 f. 0.3 kHz |  |  |
| Limits (kHz) | 4 065 | 6 200 |  |
| Frequencies assignable to ship stations for telephony, duplex operation a) i) p) | 4 066.4 to 4 144.4 27 f. 3 kHz | 6 201.4 to 6 222.4 8 f. 3 kHz |  |
| Limits (kHz) | 4 146 | 6 224 |  |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 |  | 6 |  | ... |  |
| --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) |  | 4 146 |  | 6 224 |  |  |
| Frequencies assignable to ship stations and coast stations for telephony, simplex operation a) p) |  | 4 147.4 to 4 150.4 2 f. 3 kHz |  | 6 225.4 to 6 231.4 3 f. 3 kHz |  |  |
| Limits (kHz) |  | 4 152 |  | 6 233 |  |  |
| Frequencies assignable to ship stations for wide-band telegraphy, facsimile and special transmission systems p) |  | 4 154 to 4 170 5 f. 4 kHz |  | 6 235 to 6 259 7 f. 4 kHz |  |  |
| Limits (kHz) |  | 4 172 |  | 6 261 |  |  |
| Frequencies assignable to ship stations for oceanographic data transmission c) p) |  |  |  | 6 261.3 to 6 262.5* 5 f.* 0.3 kHz |  |  |
| Limits (kHz) |  | 4 172 |  | 6 262* |  |  |
| Frequencies assignable to ship stations for wideband telegraphy, facsimile and special transmission systems p) |  | 4 174 1 f. 4 kHz |  | 6 264 1 f. 4 kHz |  |  |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | ... |
| --- | --- | --- | --- |
| Limits (kHz) | 4 176 | 6 266 |  |
| Frequencies (paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) j) m) | 4 176.5 to 4 179 5 f. 0.5 kHz | 6 266.5 to 6 270 7 f. 0.5 kHz |  |
| Limits (kHz) | 4 179.25 | 6 270.25 |  |
| Frequencies assignable to ship stations for data transmission p) m) |  |  |  |
| Limits (kHz) | 4 186.75 | 6 284.75 |  |
| f) h) p) Frequencies assignable to ship stations for data transmission m) | 4 187 to 4 202 | 6 285 to 6 300 |  |
| ... |  |  |  |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued )

| Band (MHz) | 4 | 6 | ... |
| --- | --- | --- | --- |
| Limits (kHz) | 4 202.25 | 6 300.25 |  |
| Frequencies (non paired) assignable to ship stations for data transmission systems p) | 4 202.5 to 4 207 | 6 300.5 to 6 311.5 |  |
| Limits (kHz) | 4 207.25 | 6 311.75 |  |
| Frequencies assignable to ship stations for digital selective calling k) l) | 4 207.5 to 4 209 4 f. 0.5 kHz | 6 312 to 6 313.5 4 f. 0.5 kHz |  |
| Limits (kHz) | 4 209.25 | 6 313.75 |  |
| Frequencies assignable to coast stations for data transmission n) o) p) |  |  |  |
| Limits (kHz) | 4 214 | 6 317.5 |  |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) | 4 214.25 to 4 216.25 6 f. 0.5 kHz | 6 317.75 to 6 320.7 8 f. 0.5 kHz |  |
| Limits (kHz) | 4 216.5 | 6 321 |  |
| Frequencies assignable to coast stations for data transmission p) |  |  |  |
| Limits (kHz) | 4 219.25 | 6 330.75 |  |
| Frequencies assignable to coast stations for digital selective calling l) | 4 219.5 to 4 220.5 3 f. 0.5 kHz | 6 331 to 6 332 3 f. 0.5 kHz |  |
| Limits (kHz) | 4 219.25 | 6 330.75 |  |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (end )

| Band (MHz) | 4 | 6 | ... |
| --- | --- | --- | --- |

| Limits (kHz) | 4 221 | 6 332.5 |  |
| --- | --- | --- | --- |

| Frequencies assignable to coast stations for wide-band telegraphy, facsimile, special and data transmission systems and direct-printing telegraphy systems p) |  |  |  |
| --- | --- | --- | --- |

| Limits (kHz) | 4 351 | 6 501 |  |
| --- | --- | --- | --- |

| Frequencies assignable to coast stations for telephony, duplex operation a) p) | 4 352.4 to 4 436.4 29 f. 3 kHz | 6 502.4 to 6 523.4 8 f. 3 kHz |  |
| --- | --- | --- | --- |
| Limits (kHz) | 4 438 | 6 525 |  |

Reasons:	It should be modified to use these frequency bands for data transmission.

NOC	ASP/41A13/116

Notes a), c) to d); whereas the “b)” in the table on page 6 should be suppressed.

SUP	ASP/41A13/117

Notes e) to g)

Reasons:	Notes b) and e) to g) should be suppressed since Section III on NBDP, Section IV on Morse telegraphy (calling) and Morse telegraphy (working) in Part B are proposed to be deleted.

NOC	ASP/41A13/118

Notes h) to l)

MOD	ASP/41A13/119

m)	Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy subject to not claiming protection from other stations in the maritime mobile service using data transmissions.

Reasons:	The introduction of new HF data communication described in Recommendation ITU-R M.1798 is acceptable, however there are still many ships using Morse telegraphy communications. The proposed modification intends to allow continuous use of the Morse telegraph communications by the existing ships.

NOC	ASP/41A13/120

Notes n) to o)

MOD	ASP/41A13/121

p)	These sub-bands, except the frequencies referred to in Notes j), n) and o), could also be used for the initial testing and the introduction within the maritime mobile service of new digital technologies described in Recommendation ITU-R M.1798.

Reasons:	It should be modified to use these frequency bands for data transmission.

MOD	ASP/41A13/122

PART  B  –  Channelling arrangements

Section I  –  Radiotelephony

6	a)	...

ADD	ASP/41A13/123

d)	The channelling arrangement specified in Sub-Sections A and B does not prejudice the rights of administrations to establish, and to notify assignments to stations in the maritime mobile service other than those using radiotelegraphy, provided that:

–	the occupied bandwidth does not exceed 2 800 Hz and is situated wholly within one frequency channel.

Reasons:	This will allow the use of digital technology by the administrations within the maritime radiotelephone for coast and ship stations.

Section II  –  Narrow-band direct-printing telegraphy (paired frequencies)

MOD	ASP/41A13/124

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 4 MHz band 1 |  | 6 MHz band 3 |  | ... |
| --- | --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive |  |
| 1 2 3 4 5 |  |  |  |  |  |
| 6 7 8 9 10 | 4 214 4 214.5 4 215 | 4 176 4 176.5 4 177 | 6 317.5 6 318 6 318.5 6 319 | 6 266 6 266.5 6 267 6 267.5 |  |
| 11 12 13 14 15 | 4 177.5 2 4 215.5 4 216 4 216.5 | 4 177.5 2 4 178 4 178.5 4 179 | 6 268 2 6 319.5 6 320 6 320.5 6 321 | 6 268 2 6 268.5 6 269 6 269.5 6 270 |  |
| 16 17 18 19 20 |  |  |  |  |  |
| 21 22 23 24 25 |  |  |  |  |  |
| 26 27 28 29 30 |  |  |  |  |  |
| 31 32 33 34 35 |  |  |  |  |  |

Reasons:	These frequency bands should be deleted in accordance with the proposed deletion of NBDP frequency bands in Part A.

SUP	ASP/41A13/125

Section III  –  Narrow-band direct-printing telegraphy 
(non-paired frequencies)

Reasons:	The entire section of NBDP should be suppressed for the use of data transmission.

SUP	ASP/41A13/126

Section IV  –  Morse telegraphy (calling)

Reasons:	The entire section of Morse telegraphy (calling) should be suppressed for the use of data transmission.

SUP	ASP/41A13/127

Section V  –  Morse telegraphy (working)

Reasons:	It is proposed to suppress the entire section of Morse telegraphy (working) for the use of data transmission.

# Resolution 544

NOC	ASP/41A13/128

APT supports no additional allocation to the HF broadcasting service.

# Review of allocation in the bands 4-10 MHz

NOC	ASP/41A13/129

APT supports no change in the current Table of Frequency Allocations in the 4-10 MHz band.

Reasons:	The frequency band 4-10 MHz is heavily used by existing services.

____________

* 	Note by the ITU Secretariat: There is inconsistency between these figures. There are additional inconsistencies in the figures included in this Table.