Wafer Probe Test Specifications for the TGL98.V2 Chip (revised 2/22/99 onwards)

Overview:

The purpose of this document is to outline the wafer probe testing of the TGL98.V2 chip.  The TGL98.V2 is a mixed signal chip that has 16 channels of preamplifier-discriminator channels and associated on-chip digital functions for various adjusts and enables.  The main functional units of the chip to be tested are the preamplifier-discriminator block, decoder, serial registers, D/A converter and bias values.  The steps to test the chip are devised to verify and isolate a known good die by eliminating a bad die as soon as it is detected.  In other words, the progressive test steps need take place only if a die passes the previous test.  The testing would do the following in order:

- Determine the quiescent current drawn at the 3 sets of power supply pins.

- Determine the bias values at the bias reference pins which will indicate the DAC output values as well as the resistor bias values.

- Clock in serial data to the 3 sets of serial registers and read the data back which will test the decoder and the serial registers

- Digitally pulse the on-chip charge-pulser for preamplifier/discriminator channels and observe the output current pulse as well as the reference current output.  This tests the preamplifier-discriminator block.

Special instructions for the testing:

The scope of the entire testing includes

- Design and fabrication of all probing fixtures associated with the chip interface at the wafer probe to the tester.

- Programming of the tester in accordance with the test specifications

- Execution of the tests

- Inking out of  “bad dies”

- Standard dicing of the wafer after placing on sticky film

- Visual inspection of the die after dicing and

- Packaging of the good die to Lund (Sweden).

The probe card must be designed and fabricated in accordance to the test circuit shown in FIGURE 1. The separate supplies to power supply sets V1, V2 and V3 must be bypassed by capacitors placed as close to the probes as possible.  The grounds are common.  The bare die is very sensitive to light and care must be taken to eliminate spurious signal pickup at the input pads due to the presence of 60Hz. and other stray fields.

The die will be on 6” diameter wafers from Orbit’s 1.2u analog CMOS process. The price quotation should be for testing 2 batches of wafers.  A first, smaller batch of 5 wafers (4500 chips) and a second batch of 37 wafers (33,300 chips) which will need to be tested in about 6 weeks to 3 months from the first batch.  There will be about 900 dies per wafer.  Tested wafers will be standard diced.  As soon as any die fails it will be identified as defective and any remaining tests for that die can be suspended.  Bad die will also be returned to Lund, Sweden, but has to be inked to distinguish it from a good die.

Details of the testing:

The pad diagram of TGL98.V2 chip is given in Appendix 1 along with a table describing the pin details in Appendix 2.  The numbering of the pins starts at the SW corner and proceeds in a clockwise direction.  The test circuit, which is to be fabricated as the probe card is given in FIGURE 1.  All inputs Vin(x) are left open with no connection for these tests.  Iout(x) outputs are 70uA current pulses through 8.2k load resistor (574mV (peak).  This means a nominal signal of pulse height ~400 mV (+/- 20%) and a pulse width of 150 ns (+/- 20%) occurring about 100 ns after the test pulse has been issued.  All bias pins are to be probed and their voltages measured.  The probe on AO will help to setup the test properly.

The serial decoder/addressing scheme of the TGL98.V2 is described in Appendix 3. The data register bits are defined in Appendix 4.   Logic levels are 5V CMOS.   The pad dimensions of the TGL98.V2 chip (fabricated in 1.2u ORBIT process as a SMALL chip) is given in Appendix 5.

The control pins and their functions are given briefly in Table 1.

# Table 1.  Control signal pins and their function

| Pad | Pin Name | Type | Functional description |
| --- | --- | --- | --- |
| OS(30) | Operate (bar)/Serial mode | input | Mode bit: Used to define the shared function pins. OS = 5V => serial data mode; OS = 0V => operate chip mode. |
| SAD (31) | Serial Address/ Data(bar) mode | input | When OS=5V: SAD = 5V => address register select; and SAD = 0V => control register select; When OS = 0V: SAD = don’t care. |
| RD_SC(33) | Reset Disable / Serial Clock shared function pin | input | RD = (RD_SC).OS(bar); analog reset SC = (RD_SC).OS; serial clock |
| TP_SL (34) | Test pulse / Serial latch shared function pin | input | TP = (TP_SL).OS(bar); test pulser SL = (TP_SL).OS.; serial latch |
| SI(35) | Serial Input | input | Serial data input pin. |
| SO(36) | Serial Output | output | Serial data output pin – active when RW bit =1, SAD=0 Else tristated. |
| SOE (32) | Transceiver direction select | output | SOE = 5V => read-back data from chip. Internally generated chip signal Else, tristated |

Test 1:

- Quiescent Current  Measurement Test

The chip requires 3 independent power supplies of 5V DC and a reference voltage of 2.5V to function.  Bypass capacitors of shown value need to be placed as close as possible to the chip between probe card needle and ground.   Table 2 gives the criteria for the TGL98.V2 power pads and performance.  The values in the table are those after the power-up state.

# Table 2: Power pads and performance

| Pad | Supply | Bypass | Function | Current drawn | Acceptable margin |
| --- | --- | --- | --- | --- | --- |
| 20 and 76 | 5V dc | 4.7uF ; | (V1)Analog1 supply | 4.7mA | 4.0mA – 5.5mA |
| 21 and 75 | 0 |  | Analog1 ground |  |  |
| 24 and 70 | 5V dc | 4.7uF; | (V2)Analog2 supply | 2.8mA | 2.0mA – 3.3mA |
| 25 and 69 | 0 |  | Analog2 ground |  |  |
| 37 and 59 | 5V dc | 4.7uF; | (V3)Digital supply | 0.28mA | 0.2mA – 0.5mA |
| 38 and 58 | 0 |  | Digital ground |  |  |
| 72 | 2.5V dc | 4.7uF | (VM)Midpoint ref. | 4.7 uA | +/- 1uA |

If the current drawn at all pads are within the acceptable margin proceed to the next test

Note 1: All grounds are common.

Note 2: The input signals at the input pins  SI, RD_SC, TP_SL, SAD and  OS should be steady 0v for Test1 and Test2.

TEST 2:

- Testing the Bias pin values:

The voltages at the bias reference pins are to be measured just after the power-up state.  If any one of these values is 0V or 5V the chip is deemed to fail at this point.  The bias pads that need to be probed are given in Table 3.

# Table 3: Bias pads and performance

| Pad | Function | Nominal Value | Acceptable margin Min - Max |
| --- | --- | --- | --- |
| 40 | OS_bias – oneshot bias | 3.2 V | 2.56 V – 3.84 V |
| 41 | OR – Output reference current (~20uA) thru 8.2K | 150mV | 82mV – 240mV |
| 68 | DA12_bias | 1.17 V | 0..94 V – 1.4 V |
| 71 | S2_bias | 3.15 V | 2.5V – 3.8V |
| 74 | S1_bias | 1.8 V | 1.2V – 2.5 V |

If the voltages at all pads are within the acceptable margin proceed to the next test

Note: The input signals at the input pins  SI, RD_SC, TP_SL, SAD and  OS should be steady 0v for Test1 and Test2.

TEST 3: Serial and Address Data Communication Test

Note: The serial register values are all to be loaded lsb (least significant bit) in first as indicated in the timing diagrams when writing to any register in TGL98.V2.

1. Select serial control mode for this test by setting serial mode pin OS = 5V

Note 1: Binary patterns refer ‘1’ to CMOS Logic 5V, ‘0’ to CMOS Logic 0V

Note 2: The clock frequency is 10 MHz and is applied to RD_SC pin.

Note 3: The data is shifted in on the rising edge of the clock when you write to a register

Note 4: The data is shifted out on the rising edge of the clock when you read-back from a register

Note 5: Read-back cannot be made with the broadcast bit A6 of address register set to 1.  So, read-back should always be done with an address match.

TEST 3a: Write to and Read back CR1 even bits (address match test)

Refer to timing diagram in FIGURE 3a for the following test steps.

- DATA WRITE PROCEDURE:

1.  Load address register to select control register address (CR1) and mode (write)

- Select address mode by setting SAD = 5V

- Clock in 0010010101 b pattern to address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after data is clocked in.

- Write  ‘1’s to even bits of the control register CR1

- Change to data mode by setting SAD =0V

- Clock in pattern 0101 0101 0101 0101  010b   to load CR1 even bits with ‘1’. Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |

c. 	Latch the data register by 5V -> 0V transition on TP_SL line after serial data is clocked.

- DATA READ-BACK PROCEDURE:

- Load address register to select control register address (CR1) and mode (read-back)

- Select address mode by setting SAD = 5V

- Clock in 1010 0101 01 b pattern to address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

2.  Read back  ‘1’s from the even bits of control register CR1

- Select data mode by setting SAD =0V

- Issue a pulse 0V -> 5V -> 0V through TP_SL pin, after SAD changes state to 0V.

- Issue one clock pulse when TP_SL is 5V.  This will latch back data to be shifted out.

- Check SOE pin also.  After SAD changes to 0v it should change from a tristated condition to 5V.

- Issue 18 clock pulses after the latch-back operation and after TP_SL has returned to 0V.

- Check the data shifted out on the rising edge of the clock via the SO pin.

It should be 0101 0101 0101 0101  010b

TEST 3b: Write to and Read back CR1 odd bits (address match test)

Refer to timing diagram in FIGURE 3b for the following test steps.

- DATA WRITE PROCEDURE:

1.  Load address register to select control register address (CR1) and mode (write)

- Select address mode by setting SAD = 5V

- Clock in 0010010101 b pattern to address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

- Write  ‘1’s to odd bits of control register CR1

- Change to data mode by setting SAD =0V

- Clock in pattern 1010 1010 1010 1010  101b  to load CR1 odd bits with ‘1’. Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the data register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

- DATA READ-BACK PROCEDURE:

- Load address register to select control register address (CR1) and mode (read-back)

- Select address mode by setting SAD = 5V

- Clock in 1010 0101 01 b pattern to address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after data is clocked in.

2.  Read back  1’s from odd bits of control register CR1

- Select data mode by setting SAD =0V

- Issue a pulse 0V -> 5V -> 0V through TP_SL pin, after SAD changes state to 0V.

- Issue one clock pulse when TP_SL is 5V.  This will latch back data to be shifted out.

- Check SOE pin also.  After SAD changes to 0v it should change from a tristated condition to 5V.

- Issue 18 clock pulses after the latch-back operation and after TP_SL has returned to 0V.

- Check data shifted out on the rising edge of the clock via the SO pin.

It should read 1010 1010 1010 1010  101b

TEST 3c Write to and Read back CR2 even bits; A6=1 (broadcast mode test- no address match)

Refer to timing diagram in FIGURE 3c for the following test steps.

- DATA WRITE PROCEDURE:

1. Load address register to select control register address (CR2) and mode (write)

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

- Write  ‘1’s to even bits of CR2 register

- Change to data mode by setting SAD =0V

- Clock in pattern 0101 0101 0101 010 b to load CR2 odd bits with ‘1’.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data is clocked.

- DATA READ-BACK PROCEDURE:

- Load address register to select control register address (CR2) and mode (read-back)

- a. 	Select address mode by setting SAD = 5V

- Clock in 1100 0101 01b pattern to the address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

2.  Read back  1’s from even bits of control register CR2

- Select data mode by setting SAD =0V

- Issue a pulse 0V -> 5V -> 0V through TP_SL pin, after SAD changes state to 0V.

- Issue a single clock pulse when TP_SL is 5V.  This will latch back data to be shifted out.

- Check SOE pin also.  After SAD changes to 0v it should change from a tristated condition to 5V.

- Issue 14 clock pulses after the latch-back operation and after TP_SL has returned to 0V.

- Check data shifted out on the rising edge of the clock via the SO pin.

It should read 0101 0101 0101 010 b

TEST 3d: Write to and Read back CR2 odd bits; A6=1 (broadcast mode test- no address match)

Refer to timing diagram in FIGURE 3d for the following test steps.

- DATA WRITE PROCEDURE:

1. Load address register to select control register address (CR2) and mode (write)

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b to the address register.

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

- Write  the control word to the control register (CR2)

- Change to data mode by setting SAD =0V

- Clock in pattern 1010 1010 1010 101 b to load CR2 odd bits with ‘1’.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the data register by 5V -> 0V transition on TP_SL line after serial data is clocked.

- DATA READ-BACK PROCEDURE:

1. Load address register to select control register address (CR2) and mode (read-back)

- Select address mode by setting SAD = 5V for serial data transfer to address register through SI pad

- Clock in 1100 0101 01b pattern to the address register.

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after data is clocked in.

2.  Read back  1’s from odd bits of control register CR2

- Select data mode by setting SAD =0V

- Issue a pulse 0V -> 5V -> 0V through TP_SL pin, after SAD changes state to 0V.

- Issue a single clock pulse when TP_SL is 5V.  This will latch back data to be shifted out.

- Check SOE pin also.  After SAD changes to 0v it should change from a tristated condition to 5V.

- Issue 14 clock pulses after the latch-back operation and after TP_SL has returned to 0V.

- Check data shifted out on the rising edge of the clock via the SO pin.

It should read 1010 1010 1010 101 b

TEST 3e: Write to and Read back CR3 even bits; A6=1 (broadcast mode test- no address match)

Refer to timing diagram in FIGURE 3e for the following test steps.

- DATA WRITE PROCEDURE:

1. Load address register to select control register address (CR3) and mode (write)

- Select address mode by setting SAD = 5V

- Clock in 0111 0000 00 b pattern to the address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

- Write  the control word to the control register (CR3)

- Change to data mode by setting SAD =0V

- Clock in pattern 0101 0101 0101 0101 0101 0101 010 b to load 1’s in CR3 even bits. Clock in 27 times.

| D 16 | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | M5 | M4 | M3 | M2 | M1 | F 6 | F 5 | F 4 | F 3 | F 2 | F1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after serial data is clocked.

- DATA READ-BACK PROCEDURE:

1. Load address register to select control register address (CR3) and mode (read-back)

- Select address mode by setting SAD = 5V

- Clock in 1110 0101 01b pattern to the address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

2.  Read 1’s from even bits of control register (CR3)

- Select data mode by setting SAD =0V

- Issue a pulse 0V -> 5V -> 0V through TP_SL pin, after SAD changes state to 0V.

- Issue a single clock when TP_SL is 5V.  This will latch back data to be shifted out.

- Check SOE pin also.  After SAD changes to 0v it should change from a tristated condition to 5V.

- Issue 26 clock pulses after the latch-back operation and after TP_SL has returned to 0V.

- Check data shifted out on the rising edge of the clock via the SO pin.

It should read 0101 0101 0101 0101 0101 0101 010 b

TEST 3f: Write to and Read back CR3 odd bits; A6=1 (broadcast mode test- no address match)

Refer to timing diagram in FIGURE 3f for the following test steps.

- DATA WRITE PROCEDURE:

1. Load address register to select control register address (CR3) and mode (write)

- Select address mode by setting SAD = 5V

- Clock in 0111 0000 00 b pattern to the address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

- Write  the control word to the control register CR3

- Change to data mode by setting SAD =0V

- Clock in pattern 1010 1010 1010 1010 1010 1010 101 b to load 1’s in CR3 odd bits. Clock in 27 times.

| D 16 | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | M5 | M4 | M3 | M2 | M1 | F 6 | F 5 | F 4 | F 3 | F 2 | F1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the data register by 5V -> 0V transition on TP_SL line after serial data is clocked.

- DATA READ-BACK PROCEDURE:

1. Load address register to select control register address (CR3) and mode (read-back)

- Select address mode by setting SAD = 5V

- Clock in 1110 0101 01b  pattern to the address register

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |

- Latch the address register by 5V -> 0V transition on TP_SL line after serial data is clocked in.

2.  Read back  1’s from odd bits of control register CR3

- Select data mode by setting SAD =0V

- Issue a pulse 0V -> 5V -> 0V through TP_SL pin, after SAD changes state to 0V.

- Issue a single clock pulse when TP_SL is 5V.  This will latch back data to be shifted out.

- Check SOE pin also.  After SAD changes to 0v it should change from a tristated condition to 5V.

- Issue 26 clock pulses after the latch-back operation and after TP_SL has returned to 0V.

- Check the data shifted out on the rising edge of the clock via the SO pin.

It should read 1010 1010 1010 1010 1010 1010 101 b

TEST 4: Operating mode Test for Amplitude, Attenuator

Preamplifier-Discriminator testing for 8 conditions:

  - 2fC(DAC=2) input charge, 3 fC threshold (THR=50), Att=1; four channels pulsed at a time; all   channels enabled

4.1.a. channels 1-4 test pulsed

- 4.1.b.channels 5-8 test pulsed

- 4.1.c.channels 9-12 test pulsed

- 4.1.d.channels 13-16 test pulsed

  - 5 fC (DAC=7)input charge, 3 fC threshold(THR=50), Att=1; four channels pulsed at a time; all channels enabled

4.2.a. channels 1-4 test pulsed

4.2.b. channels 5-8 test pulsed

4.2.c. channels 9-12 test pulsed

4.2.d. channels 13-16 test pulsed

  - 5 fC (DAC=6)input charge, 7 fC threshold(THR=10), Att=1; four channels pulsed at a time; all channels enabled

4.3.a. channels 1-4 test pulsed

4.3.b. channels 5-8 test pulsed

4.3.c. channels 9-12 test pulsed

4.3.d. channels 13-16 test pulsed

  - 9fC (DAC=10)input charge, 7 fC threshold (THR=10), Att=1; four channels pulsed at a time; all channels enabled

4.4.a. channels 1-4 test pulsed

4.4.b. channels 5-8 test pulsed

4.4.c. channels 9-12 test pulsed

4.4.d. channels 13-16 test pulsed

  - All channels at 12fC (DAC=15) input charge, 15 fC threshold (THR=31), Att=1/3;all channels pulsed; all channels enabled

  - All channels at 19fC (DAC=24) input charge, 15 fC threshold(THR=31), Att=1/3;all channels pulsed; all channels enabled

  - All channels at 33 fC (DAC=42)input charge, 44 fC threshold(THR=31), Att=1/9;all channels pulsed; all channels enabled

  - All channels at 58 fC (DAC=73) input charge, 44 fC threshold(THR=31), Att=1/9;all channels pulsed; all channels enabled

The tests 4.1 to 4.8 will all have same value in CR3 ( all channels enabled, mux off, Vfb=1.5v(DAC=30) )

The sequence for this will be

- Set serial mode.

- Set SAD and TP_SL pins as needed

- Load address register to write to CR3

- Load serial control registers CR3 – for all channels enabled, mux off, Vfb = 1.5V (DAC=30)

Then for tests 4.1 to 4.4 first load CR2 with appropriate value for that test

- Set serial mode.

- Set SAD and TP_SL pins as needed

- Load address register to write to CR2

- Load serial control registers CR2 with appropriate values of threshold and test pulse values

For each test 4.1 through 4.4, load appropriate values into CR1 to pulse channels 1-4, then 5-8, then 9-12 and 13-16.  After each load of CR1 generate a test pulse and observe the outputs.

- Set serial mode

- Load address register to write to CR1

- load serial control registers CR1 to pulse channels 1-4, steepest slope

- Setup operating mode

- Issue reset

- Issue test pulse

- Observe the output at pads iout1 … iout16.

Repeat steps 1-7 above changing only step 3 for channels 5-8, 9-12, and 13-16 successively.

The sequence for tests 4.5 to 4.8 will be  (CR3 is the same as loaded earlier).  Now CR1 will be the same for these 4 tests (i.e. all channels pulsed and steepest slope)

- Set serial mode.

- Set SAD and TP_SL pins as needed

- Load address register to write to CR1

- Load serial control registers CR1 with all channels pulsed

Then for each test 4.5 through 4.8, load appropriate values into CR2

- Set serial mode

- Load address register to write to CR2

- load serial control registers CR2 with appropriate threshold and test pulse values

- Setup operating mode

- Issue reset

- Issue test pulse

- Observe the output at pads iout1 … iout16.

Note: The outputs will be pulses that are 150 ns (+/- 30 ns) wide, ~570mV(+/- 20%) in amplitude (if flat-top is reached) occurring about 100ns after the test pulse has been issued.  The output at OR pin must be a steady at ~150 mV (+/- 20mV) signal.  The tester should be set at a threshold of about 200mV.  Any signal above that at the output pins should be construed as a “fired channel”.

Test 4 :

For all tests 4.1 to 4.8 (CR3 is same)All channels enabled; mux off; Vfb=1.5v equiv(DAC=30)

(Common for all tests 4.1 through 4.8)

Refer to the timing diagram in FIGURE 4.0 for this part of the test.

- Set up serial mode

Set serial mode pin OS = 5V

- Set SAD pin =5V; set TP_SL pin = 5V

- Load address register to write to CR3

- Select address mode by setting SAD = 5V

- Clock in 0111 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR3 – for all channels enabled, mux disabled; Vfb = 1.5v

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 0000 0000 1000 0011 110 b to load CR3.  Clock in 27 times.

| D 16 | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | M5 | M4 | M3 | M2 | M1 | F 6 | F 5 | F 4 | F 3 | F 2 | F1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 4.1:

All channels at 2 fC (DAC=2) input charge, 3 fC (DAC=50)threshold, Att=1

Refer to the Timing diagram in FIGURE 4.1 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1; threshold = 3fC; pulse amplitude = 2fC

- Change to data mode by setting SAD =0V

- Clock in pattern 0011 0010 0000 010 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 4.1a:

Channels 1-4 pulsed; steepest slope

Refer to the Timing diagram in FIGURE 4.1.a for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 1-4 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 1111 0000 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.1b:

Channels 5-8 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.1.b for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 5-8 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 1111 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.1c:

Channels 9-12 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.1.c for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 9-12 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 1111 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16.

No outputs must fire.

Test 4.1d:

Channels 13-16  pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.1.d for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 13-16 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 0000 1111 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.2:

All channels at 5 fC (DAC=7)input charge, 3 fC (DAC=50)threshold, Att=1

Refer to the Timing diagram in FIGURE 4.2 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1; threshold = 3fC; pulse amplitude = 5fC

- Change to data mode by setting SAD =0V

- Clock in pattern 0011 0010 0000 111 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 4.2a:

Channels 1-4 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.2.a for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 1-4 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 1111 0000 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only outputs 1-4 must fire.

Test 4.2b:

Channels 5-8 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.2.b for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 5-8 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 1111 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only outputs 5-8must fire.

Test 4.2.c:

Channels 9-12 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.2.c for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 9-12 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 1111 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16.

Only outputs 9-12must fire.

Test 4.2.d:

Channels 13-16  pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.2.d for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 13-16 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 0000 1111 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only outputs 13-16 must fire.

Test 4.3:

All channels at 5 fC (DAC=6) input charge, 7 fC (DAC=10)threshold, Att=1

Refer to the  Timing diagram in FIGURE 4.3 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1; threshold = 7fC(DAC=10); pulse amplitude = 5fC(DAC=6)

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 1010 0000 110 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 4.3.a:

Channels 1-4 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.3.a for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 1-4 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 1111 0000 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.3b:

Channels 5-8 pulsed; steepest slope

Refer to the Timing diagram in FIGURE 4.3.b for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 5-8 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 1111 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.3.c:

Channels 9-12 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.3.c for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 9-12 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 1111 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16.

No outputs must fire.

Test 4.3.d:

Channels 13-16 pulsed; steepest slope

Refer to the Timing diagram in FIGURE 4.3.d for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 13-16 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 0000 1111 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.4:

All channels at 9 fC (DAC=10)input charge, 7 fC (DAC=10)threshold, Att=1

Refer to the Timing diagram in FIGURE 4.4 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1; threshold = 7fC(DAC=10); pulse amplitude = 9fC(DAC=10)

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 1010 0001 010 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 4.4a:

Channels 1-4 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.4.a for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 1-4 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 1111 0000 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only outputs 1-4 must fire.

Test 4.4b:

Channels 5-8 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.4.b for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 5-8 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 1111 0000 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only outputs 5-8must fire.

Test 4.4.c:

Channels 9-12 pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.4.c for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 9-12 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 1111 0000 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16.

Only outputs 9-12must fire.

Test 4.4.d:

Channels 13-16  pulsed; steepest slope

Refer to the  Timing diagram in FIGURE 4.4.d for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – channels 13-16 pulsed ; steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 0000 1111 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change OS pin to 0V for operating mode.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only outputs 13-16 must fire.

Test 4. 5to8:

All channels pulsed; steepest leakage slope; all channels enabled, mux off, Vfb=1.5v (DAC=30)

(CR1 values common to tests 4.5 through 4.8 )

Refer to the  Timing diagram in FIGURE 4.5to8 for this test. (CR3 is already loaded with the right value)

- Set up serial mode

Set serial mode pin OS = 5V

- Set SAD pin =5V; set TP_SL pin = 5V

- Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – for all channels pulsed, steepest  leakage slope

- Change to data mode by setting SAD =0V

- Clock in pattern 1111 1111 1111 1111 000 b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 4. 5:

All channels at 12fC input charge (DAC=15), 15 fC threshold (DAC=31), Att=1/3 (01)

Refer to the Timing diagram in FIGURE 4.5 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1/3; threshold = 15fC; pulse amplitude = 12fC

- Change to data mode by setting SAD =0V

- Clock in pattern 0101 1111 0001 111 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.6:

All channels at 19fC (DAC=24) input charge, 15 fC threshold (DAC=31), Att=1/3(01);all channels pulsed; steepest leakage slope; all channels enabled

Refer to the Timing diagram in FIGURE 4.6 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1/3; threshold = 15fC; pulse amplitude = 19fC

- Change to data mode by setting SAD =0V

- Clock in pattern 0101 1111 0011 000 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

All outputs must fire.

Test 4.7:

All channels at 33fC (DAC=42) input charge, 44 fC threshold (DAC=31), Att=1/9 (10b);all channels pulsed; steepest leakage slope; all channels enabled

Refer to the  Timing diagram in FIGURE 4.7 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1/9; threshold = 44fC(DAC=31); pulse amplitude = 33fC(DAC=42)

- Change to data mode by setting SAD =0V

- Clock in pattern 1001 1111 0101 010 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

No outputs must fire.

Test 4.8:

All channels at 58fC (DAC=73)input charge, 44 fC threshold (DAC=31), Att=1/9(10b);all channels pulsed; steepest leakage slope; all channels enabled

Refer to the  Timing diagram in FIGURE 4.8 for this test.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1/9; threshold = 44fC(DAC=31); pulse amplitude = 58fC(DAC=73)

- Change to data mode by setting SAD =0V

- Clock in pattern 1001 1111 1001 001 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

All outputs must fire.

TEST 5: Operating mode Test for Cross Talk/ Feedthrough

- All channels at 90 fC input charge, 2 fC threshold, Att=1;odd channels pulsed; all channels enabled

- All channels at 90 fC input charge, 2 fC threshold, Att=1;even channels pulsed; all channels enabled

The sequence will be

- Setup serial mode

- Set SAD and TP_SL pins

- Load address register to write to CR2

- load serial control registers CR2 – for 90fC charge input, 2fC threshold, att=1

- Load address register to write to CR3

- load serial control registers CR3 – for all channels enabled, mux off, Vfb = 1.5V DAC equivalent

Then for each section, load appropriate values into CR1

- Set serial mode

- Load address register to write to CR1

- load serial control registers CR1 to appropriate value

- Set operating mode

- Issue reset

- Issue test pulse

- Observe the output at pads iout1 … iout16 (42 – 57).

Note: The outputs will be pulses that are 150 ns (+/- 30 ns) wide, ~570mV(+/- 20%) in amplitude (if flat top is reached) occurring about 100ns after the test pulse has been issued.  The output at OR pin must be a steady at ~150 mV (+/- 20mV) signal. The tester should be set at a threshold of about 200mV.  Any signal above that at the output pins should be construed as a “fired channel”.

Test 5: All channels at 90fC (DAC=110); threshold = 2fC (DAC=50), att=1 (00), steepest leakage; all channels enabled; mux off; Vfb = 1.5v equiv.

Refer to the timing diagram in FIGURE 5 for this part of the test.

- Set up serial mode by  OS pin= 0V and TP_SL pin = 5V

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- Load serial control registers CR2 – for 90fC charge input (DAC=110), 2fC threshold (DAC=50), att=1

- Change to data mode by setting SAD =0V

- Clock in pattern 0011 0010 1101 110 b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Load address register to write to CR3

- Select address mode by setting SAD = 5V

- Clock in 0111 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR3 – for all channels enabled, mux disabled; Vfb = 1.5v

- Change to data mode by setting SAD =0V

- Clock in pattern 0000 0000 0000 0000 1000 0011 110 b to load CR3.  Clock in 27 times.

| D 16 | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | M5 | M4 | M3 | M2 | M1 | F 6 | F 5 | F 4 | F 3 | F 2 | F1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 5a:

All channels at 90fC input charge, 2 fC threshold, Att=1;odd channels pulsed; steepest leakage slope; all channels enabled

Refer to the  Timing diagram in FIGURE 5a for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V. Set OS = 5v for serial mode.

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR3 – for odd channels pulsed, steepest slope

- Change to data mode by setting SAD =0V

- Clock in pattern 1010 1010 1010 1010 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change to operating mode by setting OS=0v

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only odd channel outputs must fire.

Test 5b:

All channels at 90fC input charge, 2 fC threshold, Att=1;even channels pulsed; steepest slope; all channels enabled

Refer to the  Timing diagram in FIGURE 5b for this test.

- Load address register to write to CR1

- Select address mode by setting SAD = 5V. Set OS = 5v for serial mode.

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 for even channels pulsed

- Change to data mode by setting SAD =0V

- Clock in pattern 0101 0101 0101 0101 000 b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change to operating mode by setting OS=0v

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only even channel outputs must fire.

TEST 6: Operating mode Test for Cross Talk/ Feedthrough via channel disables

- All channels at 90 fC input charge, 2 fC threshold, Att=1;all channels pulsed; odd channels enabled

- All channels at 90 fC input charge, 2 fC threshold, Att=1;all channels pulsed; even channels enabled

The sequence will be

- Setup OS to serial mode

- Set SAD and TP_SL pins

- Load address register to write to CR1

- load serial control registers CR1 – for all channels pulsed, steepest leakage slope

- Load address register to write to CR2

- load serial control registers CR2 – for 90fC charge input, 2fC threshold, att=1

Then for each section, load appropriate values into CR3

- Set up OS to serial mode

- Load address register to write to CR3

- load serial control registers CR3 to appropriate value

- Change OS to operating mode

- Issue reset

- Issue test pulse

- Observe the output at pads iout1 … iout16 (42 – 57).

Note: The outputs will be pulses that are 150 ns (+/- 30 ns) wide, ~570mV(+/- 20%) in amplitude (if flat-top is reached) occurring about 100ns after the test pulse has been issued.  The output at OR pin must be a steady at ~150 mV (+/- 20mV) signal. The tester should be set at a threshold of about 200mV.  Any signal above that at the output pins should be construed as a “fired channel”.

Test 6: All channels at 90fC; threshold = 2fC, att=1, maximum leakage slope; all channels pulsed

Refer to the  Timing diagram in FIGURE 6 for this test.

- Set up operating mode pin OS = 5V for serial mode and TP_SL pin = 5V

2.    Load address register to write to CR1

- Select address mode by setting SAD = 5V

- Clock in 0011 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR1 – all channels pulsed and maximum leakage slope

- Change to data mode by setting SAD =0V

- Clock in pattern 1111 1111 1111 1111 000b to load CR1.  Clock in 19 times.

| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Load address register to write to CR2

- Select address mode by setting SAD = 5V

- Clock in 0101 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR2 – for att=1; charge = 90fC (DAC=110); threshold = 2fC(DAC=50)

- Change to data mode by setting SAD =0V

- Clock in pattern 0011 0010 1101 110b to load CR2.  Clock in 15 times.

| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

Test 6a: All channels at 90fC input charge, 2 fC threshold, Att=1;all channels pulsed; steepest leakage slope; odd channels enabled

Refer to the  Timing diagram in FIGURE 6a for this test.

- Load address register to write to CR3

- Select address mode by setting SAD = 5V

- Clock in 0111 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- load serial control registers CR3 – for odd channels enabled; mux off; VFb=1.5v equiv.

- Change to data mode by setting SAD =0V

- Clock in pattern 1010 1010 1010 1010 1000 0011 110b to load CR3.  Clock in 27 times.

| D 16 | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | M5 | M4 | M3 | M2 | M1 | F 6 | F 5 | F 4 | F 3 | F 2 | F1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change to operating mode by setting OS=0v

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only odd channel outputs must fire.

Test 6b:

All channels at 90fC input charge, 2 fC threshold, Att=1;all channels pulsed; steepest leakage slope; even channels enabled

Refer to the Timing diagram in FIGURE 6b for this test.

- Load address register to write to CR3

- Select address mode by setting SAD = 5V

- Clock in 0111 0000 00 b pattern to SI pin

| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

- Latch the address register by 5V -> 0V transition on TP_SL line after address register contents are clocked in.

- Load serial control registers CR3 – for odd channels enabled; mux off; VFb=1.4v equiv.

- Change to data mode by setting SAD =0V

- Clock in pattern 0101 0101 0101 0101 1000 0111 110b to load CR3.  Clock in 27 times.

| D 16 | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | M5 | M4 | M3 | M2 | M1 | F 6 | F 5 | F 4 | F 3 | F 2 | F1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |

- Latch the data register by 5V -> 0V transition on TP_SL line after data register contents are clocked.

- Change to operating mode by setting OS=0V

- Issue reset

Issue a 2 sec. 0 -> 5V -> 0V pulse through RD_SC pin about 10 seconds after OS has gone 0V.  This issues the reset to the preamp.

- Issue test pulse

Wait for 7 sec. after the RD_SC reset pulse.  Bring TP_SL pin to 0V.  This issues the internal test pulse.

- Observe the output at pads iout1 … iout16 (42 – 57).

Only even channel outputs must fire.

APPENDIX 1: PINOUT OF THE TGL98.V2 CHIP

## APPENDIX 2: TGL98.V2 PIN LIST

| Pin.no | Name. | Description | I/O type |
| --- | --- | --- | --- |
| 1 | Vin16 | Input 16 | Charge input |
| 2 | Vin15 | Input 15 | Charge input |
| 3 | Vin14 | Input 14 | Charge input |
| 4 | Vin13 | Input 13 | Charge input |
| 5 | Vin12 | Input 12 | Charge input |
| 6 | Vin11 | Input 11 | Charge input |
| 7 | Vin10 | Input 10 | Charge input |
| 8 | Vin9 | Input 9 | Charge input |
| 9 | Vin8 | Input 8 | Charge input |
| 10 | Vin7 | Input 7 | Charge input |
| 11 | Vin6 | Input 6 | Charge input |
| 12 | Vin5 | Input 5 | Charge input |
| 13 | Vin4 | Input 4 | Charge input |
| 14 | Vin3 | Input 3 | Charge input |
| 15 | Vin2 | Input 2 | Charge input |
| 16 | Vin1 | Input 1 | Charge input |
| 17 | t | Power thru spacer pad |  |
| 18 | t | Power thru spacer pad |  |
| 19 | t | Power thru spacer pad |  |
| Corner | NW | Power thru corner pad |  |
| 20 | V1 | Preamp S1 supply | Preamp S1 supply, RC iso from LDO |
| 21 | Gnd1 | Ground | Common ground |
| 22 | X | Ground ring spacer pad |  |
| 23 | THRbias | Threshold bias bypass | Threshold current ref., bypass cap to gnd. |
| 24 | V2 | Discriminator/ S2 supply | Analog supply to S2, mux and discriminator, RC iso from LDO |
| 25 | Gnd2 | Ground | Common ground |
| 26 | AO | Analog output | Analog output voltage |
| 27 | X | Ground ring spacer pad |  |
| 28 | RSTS1 | S1 reset length adjust/ bypass | Reset circuit (1st stage) charging current ref., Bypass cap toV3 |
| 29 | RSTS2 | S2 reset length adjust/ bypass | Reset circuit (2nd stage) charging current ref., Bypass cap toV3 |
| 30 | OS | Operate/serial mode | Digital input, 489 rcvr |
| 31 | SAD | Address/data control | Digital input, 489 rcvr |
| 32 | SOE | Serial output enable | Digital output, open drain, 485 xcvr, tristate |
| 33 | RD/SC | Reset-Disable/Serial clock | Digital input, 489 rcvr |
| 34 | TP/SL | Test pulse/ Serial latch | Digital input, 489 rcvr |
| 35 | SI | Serial input | Digital input, 485 xcvr |
| 36 | SO | Serial output | Digital output, open drain, 485 xcvr,triatate |
| 37 | V3 | Digital/switching supply | Digital/ Reset/ Switching supply, RC isolated from LDO |
| 38 | Gnd3 | Ground | Common ground |
| Corner | NE | Power thru corner pad |  |
| 39 | Vfb | Power thru spacer pad | Feedback adjust thru DAC, bypass cap to gnd. |
| 40 | OSbias | One shot bias adjust/ bypass | Oneshot current bias ref., bypass cap to V3 |
| 41 | OR | Output reference current | Current source output 20uA |
| 42 | Iout1 | Output 1 | Current source output, 70uA |
| 43 | Iout2 | Output 2 | Current source output, 70uA |
| 44 | Iout3 | Output 3 | Current source output, 70uA |
| 45 | Iout4 | Output 4 | Current source output, 70uA |
| 46 | Iout5 | Output 5 | Current source output, 70uA |
| 47 | Iout6 | Output 6 | Current source output, 70uA |
| 48 | Iout7 | Output 7 | Current source output, 70uA |
| 49 | Iout8 | Output 8 | Current source output, 70uA |
| 50 | Iout9 | Output 9 | Current source output 70uA |
| 51 | Iout10 | Output 10 | Current source output 70uA |
| 52 | Iout11 | Output 11 | Current source output 70uA |
| 53 | Iout12 | Output 12 | Current source output 70uA |
| 54 | Iout13 | Output 13 | Current source output 70uA |
| 55 | Iout14 | Output 14 | Current source output 70uA |
| 56 | Iout15 | Output 15 | Current source output 70uA |
| 57 | Iout16 | Output 16 | Current source output 70uA |
| Corner | SE | Power thru corner pad |  |
| 58 | Gnd3a | Ground | Common ground |
| 59 | V3a | Digital/ switching supply | Digital/ Reset/ Switching supply, RC isolated from LDO |
| 60 | V2Ibias | Current switch bias adjust/ bypass | V2I cell bias current ref., bypass cap to V3 |
| 61 | A5 | Address line 5 | CMOS pull-up input, gnd or nc |
| 62 | A4 | Address line 4 | CMOS pull-up input, gnd or nc |
| 63 | A3 | Address line 3 | CMOS pull-up input, gnd or nc |
| 64 | A2 | Address line 2 | CMOS pull-up input, gnd or nc |
| 65 | A1 | Address line 1 | CMOS pull-up input, gnd or nc |
| 66 | A0 | Address line 0 | CMOS pull-up input, gnd or nc |
| 67 | X | Ground ring spacer pad |  |
| 68 | da12_bias | Differential Amp. Bias adjust/ bypass | Diffamp. Stage 1,2 bias current ref. , bypass cap to gnd. |
| 69 | Gnd2a | Ground | Common ground |
| 70 | V2a | Discriminator/ S2 supply | Analog supply to S2, mux and discriminator, RC iso from LDO |
| 71 | S2bias | S2 bias adjust/ bypass | S2 current ref. bypass, cap to V2 |
| 72 | VM | Voltage midpoint input | 2.5V reference input, w/ bypass to gnd. |
| 73 | X | Ground ring spacer pad |  |
| 74 | S1bias | S1 bias adjust/ bypass | S1 current ref. bypass, cap to gnd |
| 75 | Gnd1a | Ground | Common ground |
| 76 | V1a | Preamp S1 supply | Preamp S1 supply, RC iso from LDO |
| Corner | SW | Power thru corner pad |  |

# APPENDIX 3

The TGL98.V2 has a 10 bit address register tgl_adreg that stores command data is present in the chip.  It qualifies the clocks and latches based on value of OS and SAD signals.  It generates the shift register select signal “sel” to either read in data from a serial string or latch back data from the serial register latch based on the value of rw, ro and r1. The address register contents A0..A5 are passed through a decoder that checks if the addresses match with the hardwired chip address.  The chip is selected if addresses match or if bit A6 (broadcast bit is 5V).  It decodes the commands received only if that chip is being addressed. The address decoder first compares the lowest 5 data bits with A0.. A5 pins (66. 61) to see if they match or checks to see if A6=’1’.  Only if one of the above mentioned conditions match is the rest of the decoding enabled.

The TGL98.V2 has three sets of serial data registers CR1, CR2 and CR3 to do the controls of the TGL98.V2 for different modes of operation and configuration, which are controlled, by a serially loaded address register.

CR1 is the test pattern control register.  It selects which channel is to be enabled for test pulsing and has 3 bits for adjusting the leakage slope of the preamp.  The signal to select a channel is active high (5V).

CR2 is the Control register that controls the attenuator, the discriminator threshold and the test amplitude.  The test amplitude is a 7 bit DAC input that selects the value of Vtest which varies in steps of 1fC equivalent.  The discriminator threshold is a 6 bit DAC input that  sets the discriminator level.  The attenuator select helps to select the gross charge range of interest.

CR3 is the register that enables/disables (0/1) a channel and also sets the bits that control the preamp MUX output and sets the 6 bits that adjust the feedback voltage DAC.

The serial data bits of the address register and the 3 data registers is briefly defined in Appendix 4.

The data is shifted via pin (SI) and read out via pin (SO). The test patterns to be applied follow the given sequence.  SOE is an output pin which enables the 489 transceiver board.  It is tristated when data is shifted in and pulled low (0V) when data is being shifted out of the chip.

APPENDIX 4: TGL98.V2 SERIAL DATA BITS

TGL98.V2 SERIAL ADDRESS REGISTER

| msb |  |  |  |  |  |  |  |  | lsb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R/W | R1 | R0 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |

| R0 R1 | Register address MSB; 00=no register, 01=CR1, 10=CR2 , 11= CR3 Register address LSB; 00=no register, 01=CR1, 10=CR2 , 11= CR3 |
| --- | --- |
| A6 A5 A4 A3 A2 A1 A0 | TGL98.V2 address broadcast bit MSB TGL98.V2 address bit 5SB TGL98.V2 address bit 4SB TGL98.V2 address bit 3SB TGL98.V2 address bit 2SB TGL98.V2 address bit LSB TGL98.V2 address bit |

CR1: TEST PATTERN AND DECAY SLOPE COMPENSATION CONTROL REGISTER

| msb |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | lsb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | T11 | T12 | T13 | T14 | T15 | T16 | DS3 | DS2 | DS1 |

| T1 T2 T3 T4 T5 T6 T7 T8 T9 T10 T11 T12 T13 T14 T15 T16 | 0= disable test pulse, 1= enable test pulse, channel 1 0= disable test pulse, 1= enable test pulse, channel 2 0= disable test pulse, 1= enable test pulse, channel 3 0= disable test pulse, 1= enable test pulse, channel 4 0= disable test pulse, 1= enable test pulse, channel 5 0= disable test pulse, 1= enable test pulse, channel 6 0= disable test pulse, 1= enable test pulse, channel 7 0= disable test pulse, 1= enable test pulse, channel 8 0= disable test pulse, 1= enable test pulse, channel 9 0= disable test pulse, 1= enable test pulse, channel 10 0= disable test pulse, 1= enable test pulse, channel 11 0= disable test pulse, 1= enable test pulse, channel 12 0= disable test pulse, 1= enable test pulse, channel 13 0= disable test pulse, 1= enable test pulse, channel 14 0= disable test pulse, 1= enable test pulse, channel 15 0= disable test pulse, 1= enable test pulse, channel 16 |
| --- | --- |
| DS3 DS2 DS1 | MSB, decay slope select MUX, 111 = no decay 2SB, decay slope select MUX LSB, decay slope select MUX, 000 = max. decay |

CR2: ATTENUATOR, DISCRIMINATOR, THRESHOLD, TEST AMPLITUDE CONTROL REGISTER

| msb |  |  |  |  |  |  |  |  |  |  |  |  |  | lsb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| At2 | At1 | Vt6 | Vt5 | Vt4 | Vt3 | Vt2 | Vt1 | Ta7 | Ta6 | Ta5 | Ta4 | Ta3 | Ta2 | Ta1 |

| At2 At1 | MSB, C-R attenuator, 00= no atten, 01=1/3 atten, 10=1/9 atten LSB, C-R attenuator, 00= no atten, 01=1/3 atten, 10=1/9 atten |
| --- | --- |
| Vt6 Vt5 Vt4 Vt3 Vt2 Vt1 | MSB discriminator voltage threshold DAC input, 111111 = min threshold 5SB discriminator voltage threshold DAC input 4SB discriminator voltage threshold DAC input 3SB discriminator voltage threshold DAC input 2SB discriminator voltage threshold DAC input LSB discriminator voltage threshold DAC input |
| Ta7 Ta6 Ta5 Ta4 Ta3 Ta2 Ta1 | MSB Test Pulse Amplitude DAC input, 1111111 = 127 fC 6SB Test Pulse Amplitude DAC input 5SB Test Pulse Amplitude DAC input 4SB Test Pulse Amplitude DAC input 3SB Test Pulse Amplitude DAC input 2SB Test Pulse Amplitude DAC input LSB Test Pulse Amplitude DAC input |

CR3: CHANNEL DISABLE, ANALOG OUTPUT MUX SELECT AND FEEDBACK VOLTAGE CONTROL REGISTER

| msb |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | lsb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D 16 | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | M5 | M4 | M3 | M2 | M1 | F 6 | F 5 | F 4 | F 3 | F 2 | F1 |

| D16 D15 – D2 D1 | MSB, channel 16, 0=enable, 1= disable 15SB – 2SB , channel 15 – channel 2 , 0= enable, 1 = disable LSB, channel 1 |
| --- | --- |
| M5 M4 M3 M2 M1 | Mux enable/disable bit, 0=enable, 1=disable MSB, analog output mux address, 1111 = channel 16 select 3SB analog output mux address 2SB analog output mux address LSB analog output mux address |
| F6 F5 F4 F3 F2 F1 | MSB, Vfb DAC input, 11111 = 2.5v 5SB, Vfb DAC input 4SB, Vfb DAC input 3SB Vfb DAC input 2SB Vfb DAC input LSB Vfb DAC input |

# APPENDIX 5:  PAD DIMENSIONS FOR THE TGL98.V2 CHIP

Pad dimensions for the TGL98 chip

Chip BoundBox	[(0,0), (6224,6224)]	= SW co-ords & NE co-ords

lambda	0.6

Note: All dimensions are with reference to the SW most corner of the chip

x2,y2

INDIVIDUAL PAD REFERENCES

x3,y3

x1,y1

pin name	pin no.	Side	pad SW co-ords		pad NE co-ords	       pad center co-ords

in microns		in microns		in microns

(x1, y1)			(x2, y2)			(x3, y3)

$Vin16		1	W	(6.6, 290.4)		(94.2, 378)		(50.4, 334.2)

$Vin15		2	W	(6.6, 459.6)		(94.2, 547.2)		(50.4, 503.4)

$Vin14		3	W	(6.6, 631.2)		(94.2, 718.8)		(50.4, 675)

$Vin13		4	W	(6.6, 800.4)		(94.2, 888)		(50.4, 844.2)

$Vin12		5	W	(6.6, 972)		(94.2, 1059.6)		(50.4, 1015.8)

$Vin11		6	W	(6.6, 1141.2)		(94.2, 1228.8)		(50.4, 1185)

$Vin10		7	W	(6.6, 1312.8)		(94.2, 1400.4)		(50.4, 1356.6)

$Vin9		8	W	(6.6, 1482)		(94.2, 1569.6)		(50.4, 1525.8)

$Vin8		9	W	(6.6, 1653.6)		(94.2, 1741.2)		(50.4, 1697.4)

$Vin7		10	W	(6.6, 1822.8)		(94.2, 1910.4)		(50.4, 1866.6)

$Vin6		11	W	(6.6, 1994.4)		(94.2, 2082)		(50.4, 2038.2)

$Vin5		12	W	(6.6, 2163.6)		(94.2, 2251.2)		(50.4, 2207.4)

$Vin4		13	W	(6.6, 2335.2)		(94.2, 2422.8)		(50.4, 2379)

$Vin3		14	W	(6.6, 2504.4)		(94.2, 2592)		(50.4, 2548.2)

$Vin2		15	W	(6.6, 2676)		(94.2, 2763.6)		(50.4, 2719.8)

$Vin1		16	W	(6.6, 2845.2)		(94.2, 2932.8)		(50.4, 2889)

$V1		20	N	(290.4, 3640.2)		(378, 3727.8)		(334.2, 3684)

$gnd1		21	N	(460.2, 3640.2)		(547.8, 3727.8)		(504, 3684)

$THR_bias	23	N	(800.4, 3640.2)		(888, 3727.8)		(844.2, 3684)

$V2		24	N	(972, 3640.2)		(1059.6, 3727.8)		(1015.8,	3684)

$gnd2		25	N	(1141.8, 3640.2)		(1239.4, 3727.8)		(1185.6, 3684)

$AO		26	N	(1312.8, 3640.2)		(1400.4, 3727.8)		(1356.6, 3684)

$RSTS1_adj	28	N	(1653.6, 3640.2)		(1741.2, 3727.8)		(1697.4, 3684)

$RSTS2_adj	29	N	(1822.8, 3640.2)		(1910.4, 3727.8)		(1866.6, 3684)

$OS		30	N	(1994.4, 3640.2)		(2082, 3727.8)		(2038.2, 3684)

$SAD		31	N	(2163.6, 3640.2)		(2251.2, 3727.8)		(2207.4, 3684)

$SOE		32	N	(2335.2, 3640.2)		(2422.8, 3727.8)		(2379, 3684)

$RD_SC	33	N	(2504.4, 3640.2)		(2592, 3727.8)		(2548.2, 3684)

$TP_SL		34	N	(2676, 3640.2)		(2763.6, 3727.8)		(2719.8, 3684)

$SI		35	N	(2845.2, 3640.2)		(2932.8, 3727.8)		(2889, 3684)

$SO		36	N	(3016.8, 3640.2)		(3104.4, 3727.8)		(3060.6, 3684)

$V3		37	N	(3186, 3640.2)		(3273.6, 3727.8)		(3229.8, 3684)

$gnd3		38	N	(3357, 3640.2)		(3445.2, 3727.8)		(3400.8, 3684)

$Vfb(PAD)	39	E	(3640.2,3357.6)		(3727.8,3445.2)		(3864, 3401.4)

$OS_bias	40	E	(3640.2, 3186)		(3727.8, 3273.6)		(3684, 3229.8)

$OR		41	E	(3640.2, 3016.8)		(3727.8, 3104.4)		(3684, 3060.6)

$Iout1		42	E	(3640.2, 2845.2)		(3727.8, 2932.8)		(3684, 2889)

$Iout2		43	E	(3640.2, 2676)		(3727.8, 2763.6)		(3684, 2719.8)

$Iout3		44	E	(3640.2, 2504.4)		(3727.8, 2592)		(3684, 2548.2)

$Iout4		45	E	(3640.2, 2335.2)		(3727.8, 2422.8)		(3684, 2379)

$Iout5		46	E	(3640.2, 2163.6)		(3727.8, 2251.2)		(3684, 2207.4)

$Iout6		47	E	(3640.2, 1994.4)		(3727.8, 2082)		(3684, 2038.2)

$Iout7		48	E	(3640.2, 1822.8)		(3727.8, 1910.4)		(3684, 1866.6)

$Iout8		49	E	(3640.2, 1653.6)		(3727.8, 1741.2)		(3684, 1697.4)

$Iout9		50	E	(3640.2, 1482)		(3727.8, 1569.6)		(3684, 1525.8)

$Iout10		51	E	(3640.2, 1312.8)		(3727.8, 1400.4)		(3684, 1356.6)

$Iout11		52	E	(3640.2, 1141.2)		(3727.8, 1228.8)		(3684, 1185)

$Iout12		53	E	(3640.2, 972)		(3727.8, 1059.6)		(3684, 1015.8)

$Iout13		54	E	(3640.2, 800.4)		(3727.8, 888)		(3684, 844.2)

$Iout14		55	E	(3640.2, 631.2)		(3727.8, 718.8)		(3684, 675)

$Iout15		56	E	(3640.2, 459.6)		(3727.8, 547.2)		(3684, 503.4)

$Iout16		57	E	(3640.2, 290.4)		(3727.8, 378)		(3684, 334.2)

$gnd3a		58	S	(3357, 6.6)		(3444.6, 94.2)		(3400.8, 50.4)

$V3a		59	S	(3187.2, 6.6)		(3274.8, 94.2)		(3231, 50.4)

$V2I_bias	60	S	(3015.6, 6.6)		(3103.2, 94.2)		(3059.4, 50.4)

$A5		61	S	(2846.4, 6.6)		(2934, 94.2)		(2890.2, 50.4)

$A4		62	S	(2674.8, 6.6)		(2762.4, 94.2)		(2718.6, 50.4)

$A3		63	S	(2505.6, 6.6)		(2593.2, 94.2)		(2549.4, 50.4)

$A2		64	S	(2334, 6.6)		(2421.6, 94.2)		(2377.8, 50.4)

$A1		65	S	(2164.8, 6.6)		(2252.4, 94.2)		(2208.6, 50.4)

$A0		66	S	(1993.2, 6.6)		(2080.8, 94.2)		(2037, 50.4)

$DA12_bias	68	S	(1652.4, 6.6)		(1740, 94.2)		(1696.2, 50.4)

$gnd2a		69	S	(1482.6, 6.6)		(1570.2, 94.2)		(1526.4, 50.4)

$V2a		70	S	(1311.6, 6.6)		(1399.2, 94.2)		(1355.4, 50.4)

$S2_bias	71	S	(1142.4, 6.6)		(1230, 94.2)		(1186.2, 50.4)

$VM		72	S	(970.8, 6.6)		(1058.4, 94.2)		(1014.6, 50.4)

$S1_bias	74	S	(630, 6.6)		(717.6, 94.2)		(673.8, 50.4)

$gnd1a		75	S	(460.2, 6.6)		(547.8, 94.2)		(504, 50.4)

$V1a		76	S	(289.2, 6.6)		(376.8, 94.2)		(333, 50.4)