# NEW FPD CONTROL CABLES FOR MUX

- Cable 1 (CB1): From W-MUX outputs to IIB.

Qty: 20 (18 + 2 spares)

Type: Control signal cable.

Description: Flat ribbon cable, 37 wires with one D Sub 37 IDC FEMALE connector (1st) at one end and two D Sub 37 IDC MALE connectors(2nd and 3rd) at the other end.

Distance between 1st and 2nd connectors = 55cm.

Distance between 2nd and 3rd connectors = 5cm.

Aproximate cable length, no connectors, strain relief included, = 71cm.

Maintain pin numbers sequence (pin 1 to pin 1 to pin 1).

- Cable 2 (CB): From IIB to R-MUX.

Qty: 20 (18 + 2 spares).

Type: Alarm signal cable.

Description: Round, 9 or more conductors, stranded or solid AWG 20-24 (Stock Room 1170-062000) with one D Sub 9 FEMALE (cup, solder type) at one end and one D Sub 37 MALE (cup, solder type) at the other end.

Distance (face to face)  between  connectors = 88cm.

Connections:

D9F				D37M

Pin 1-------------------------------Pin 1

Pin 2-------------------------------Pin 2

Pin 3-------------------------------Pin 3

Pin 4-------------------------------Pin 4

Pin 5-------------------------------Pin 5

Pin 6-------------------------------Pin 6

Pin 7-------------------------------Pin 7

Pin 8-------------------------------Pin 20 + 21 + 22

Pin 9-------------------------------Pin 23 + 24 + 25 + 26

- Cable 3 (CB3): From MUX to RM.

Qty: 12 (10 +2 spares)

Type: Control signal cable.

Description: Flat ribbon cable, 37 wires with two D Sub 37 IDC MALE connectors.

Distance between connectors = 46cm.

Aproximate cable length, no connectors, strain relief included, = 52cm.

Maintain pin numbers sequence (pin 1 to pin 1).

- Cable 12 (CB12):  From LVDT signal conditioner to RM.

Qty: 5

Type: Analog +/- 10V signal.

Description: This is a set of four small diameter coaxial green cables (RG-58?) running from LVDT signal conditioner screw clamp terminal rule to a D Sub 37 MALE (cup, solder type) connector.

Cable length = 80cm each.

Connections:

D37M				Terminal Rule

Pin 1 -------center wire--------------#4  (UP pot)

Pin20---------mesh-------------------#5

Pin 2 -------center wire--------------#4  (DW pot)

Pin21---------mesh-------------------#5

Pin 3 -------center wire--------------#4  (IN pot)

Pin22---------mesh-------------------#5

Pin 4 -------center wire--------------#4  (OUT pot)

Pin23---------mesh-------------------#5

Note: This cable is already installed. It’s necessary to check the cable length because the RM will be dislocated with the Mux introduction.

- Cable 14 (CB14): From RM to Control circuit box and RMI.

Qty: 6 (5 + 1 spare).

Type: Control signal.

Description: This is a set of 3 coax cables (RG59) running from one D Sub 37 MALE (cup, solder type) connector to two BNCs and one D Sub … MALE (cup, solder type).

Cable length = 70cm each.

Connections:

D37M 				D…M			BNC1		BNC2

Pin 6----center wire-------------- ?

Pin 25-----mesh------------------- ?

Pin 7 -----------------------------center wire--------------center

Pin 26 -------------------------------mesh------------------outer

Pin 8 -----------------------------------------------center wire---------------center

Pin 27 --------------------------------------------------mesh------------------outer

Note: Must determine the number of pins of the RMI reset D Sub connector and the pin numbers.

- Cable 15 (CB15): From resistive devider board to RM.

Qty: 6 (5 + 1 spare).

Type: Analog DC signal.

Description: Round, 5 pairs stranded or solid AWG 24-20 running from a resistive devider board to a D Sub 37 MALE (cup, solder type) connector.

Estimated cable length about 3m. I depends on racks distance and must be mesured in the Tev tunnel.

Connections:

D37M				Resistive devider board

Pin 1 ------------ 1st    ------------------------ +5V

Pin 20-----------  pair  ----------------------- GND

Pin 2 ------------ 2nd    ------------------------ +10V/2

Pin 21-----------  pair  ----------------------- GND

Pin 3 ------------ 3rd    ------------------------ -10V/2

Pin 22-----------  pair  ----------------------- GND

Pin 4 ------------ 4th    ------------------------ +15V/2

Pin 23-----------  pair  ----------------------- GND

Pin 5 ------------ 5th    ------------------------ -15V/2

Pin 24-----------  pair  ----------------------- GND

MATERIAL LIST, CONNECTORS AND CABLES (10% spares included);

# Qty.	Description					Part #

22	D Sub 37 IDC FEMALE			JAMECO 116003

Ribbon cable connector MDC37S

72	D Sub 37 IDC MALE			  	JAMECO 115991

Ribbon cable connector MDC37P

36	D Sub 37 MALE solder-cup			JAMECO 15472

connector

36	Connector Hood DC37H			JAMECO 15456

22	D Sub 9 FEMALE solder-cup			JAMECO 15747

connector

22	Connector Hood MDB9H			JAMECO 25620

7	D Sub ? MALE solder-cup			--------------------

(For RMI reset)

7	Connector Hood ?				--------------------

(For RMI reset conn.)

14	BNC connector				--------------------

23m	IDC 37 wires flat ribbon cable		---------------------

20m	Round 9 wires or more AWG 24-20		Stock Room 1170-062000

cable

20m	Round 5 pairs AWG 24-20 cable		Stock Room 1170-062000

15m	RG 59 green coax cable			----------------------