# OBSERVER FLOW CHECK

# Observer

# Execute Flow Check while at run up area when scanner reads before takeoff checklist to pilot 2. Respond with setting when prompted by scanner

- Comm Panel – Set for Mission Pilot & Observer - GPS – Set for destination or search grid - ATC Radios – Set IAW comm. card - VOR Radios – Set IAW comm. card - ADF – Set - DME – Set - Transponder – Set to altitude w/assigned squawk or 1200 (1277 if real SAR) - DF – Pre-flight checklist complete (set to 121.5 unless DF training mission then 121.775) - CAP Radio – Set (with radio check to mission base)

## Scanner Portion

- Scanner reads checklist to Observer during run up when aircraft checklist calls for radios avionics - Observer will respond with “Set to…” and what the settings are to the crew