About this resource:

EPA is providing this resource solely for informational purposes. The agency can not attest to the accuracy, effectiveness, legality or usefulness of the information provided, nor does the inclusion or reference of such language constitute an endorsement by EPA.

Document Name:

MHD Construction Dust Control – Section 721.561

Organization/Agency Responsible:

Massachusetts Highway Department

Contact:  	Trygve Swift

Environmental Scientist

Massachusetts Turnpike Authority

Central Artery/Tunnel Project

(617) 342-1357

Description:  	Central Artery Project (Big Dig)

This project reconstructs the central artery and extension of the Massachusetts Turnpike and is expected to be completed in the spring of 2007. All pieces of construction equipment must have a diesel oxidation catalyst (DOC) and cannot idle in excess of five minutes.

SECTION 721.561

CONSTRUCTION DUST CONTROL

DESCRIPTION

1.01	GENERAL

A.	This Section specifies requirements for controlling dust generated during Work of this Contract.  It supplements the air pollution control requirements of Division I, Subsection 7.01B.

B.	The Contractor is responsible for control of dust at all times during Work of this Contract, 24 hours per day, 7 days per week, including nonworking hours, weekends, and holidays.

1.02	REGULATORY REQUIREMENTS

A.	The Contractor shall perform all Work specified under this Section in compliance with the Massachusetts Department of Environmental Protection, Code of Massachusetts Regulations (CMR) 310 CMR 7.00, "Air Pollution Control Regulations", specifically 310 CMR 7.09, "Dust, Odor, Construction and Demolition".

B.	Work of this Contract shall be conducted in a manner that will not result in excessive particulate matter emissions, nuisance dust conditions, or PM10 (particulate matter with an aerodynamic diameter less than or equal to 10 microns) concentrations exceeding the Massachusetts and National Ambient Air Quality Standard of 150 μg/m3 on 24-hour average basis.

1.03	SUBMITTALS

A.	Make submittals as required by Division I Subsection 5.02 and 6.01.

B.	Submit product literature and Material Safety Data Sheets for dust suppression wetting agents and stabilizers.

C.	Submit a dust control plan that outlines in detail the measures to be implemented by the Contractor to comply with this Section, including suppression, wind screens and barriers, prevention, cleanup, and other measures.

D.	Submit a plan for seeding to control dust.  The plan shall include University of Massachusetts (Amherst MA, Soil Test Lab) soil test results (including soluble salts and organic content) and recommendations, seed species, and quantity of each seed species to be used.

MATERIALS

2.01	DUST SUPPRESSION AGENTS

A.	Dust suppression wetting agents shall be water soluble, non-toxic, non-reactive, non-volatile, and non-foaming.

B.	Soil stabilizer shall be a sprayable organic or inorganic tackifier.

2.02	BARRIERS, SCREENS, AND COVERS

A.	Wind screens shall be a durable fabric mesh of 50 percent porosity, attached to construction fence.

B.	Wind barriers shall be solid wood fences, solid durable fabric attached to construction fence, or other solid barriers intended to block the passage of wind.

C.	The construction fence itself (chain link or solid) is not part of the Work of this Section.

D.	Covers for stockpiles shall be plastic tarps.  Contaminated soil covers shall comply with Section 120.080.

2.03	SEEDING

A.	Seeding for dust control shall conform to Section 765 and the additional requirements of this Section.  The Contractor shall conduct soil tests to determine the materials and methods necessary for the Contractor to satisfactorily produce a stand of grass that will effectively control dust.  The Contractor shall submit a plan as specified under Submittals.

CONSTRUCTION METHODS

3.01	CONSTRUCTION SITE DUST CONTROL - GENERAL

A.	Wet suppression shall be used to provide temporary control of dust.  Several applications per day may be necessary to control dust depending upon meteorological conditions and work activity.  The Contractor shall apply wet suppression on a routine basis as necessary or directed by the Engineer, to control dust.

1.	Wet suppression consists of the application of water or a wetting agent in solution with water.  Ensure wetting agent is not used on plantable soils.

2.	Wet suppression equipment shall consist of sprinkler pipelines, tanks, tank trucks, or other devices capable of providing regulated flow, uniform spray, and positive shut-off.

B.	Calcium chloride shall be used to control dust instead of wet suppression when freezing conditions exist.  Calcium chloride shall be uniformly applied by a mechanical spreader at 1 1/2 pounds per square yard, unless otherwise directed by the Engineer.  Ensure vegetation, or soil to be used for vegetation, is not treated.

C.	The use of petroleum products for dust suppression is prohibited in this Contract.

D.	Provide wind screens and wind barriers in locations where they would be effective in minimizing wind erosion and spread of dust.  Locations shall be submitted as part of the Contractor's dust control plan.  The Contractor shall keep wind screens and barriers in good repair for the life of the Contract.

E.	Seeding used to prevent wind erosion shall be in accordance with Section 765 and the additional requirements of this Section.  During seeding, furnish to the Engineer all container labels, or empty containers, from all materials used.  Do not seed without direction of the Engineer.

3.02	PUBLIC ROADWAY DUST CONTROL

A.	Vehicles leaving the construction site shall have no mud and dirt on the vehicle body or wheels.  Temporary wheel-wash stations shall be provided and water from wheel-wash stations shall be controlled per Section 140.141.

B.	Haul truck cargo areas shall be securely covered during material transport on public roadways.

C.	Vehicle mud and dirt carryout, material spills, and soil wash-out onto public roadways and walkways and other paved areas shall be cleaned up immediately.

D.	The Contractor is responsible for daily clean-up of public roadways and walkways affected by Work of this Contract.  A wet spray power vacuum street sweeper shall be used on paved roadways.  Dry power sweeping is prohibited.

3.03	CONTROL OF EARTHWORK DUST

A.	During batch drop operations (i.e., earthwork with front-end loader, clamshell bucket, or backhoe) the free drop height of excavated or aggregate material shall be reduced as practical to minimize the generation of dust.

B.	To prevent spills during transport, freeboard space shall be maintained between the material load and the top of the truck cargo bed rail.

3.04	CONTROL OF STOCKPILE DUST

A.	The Contractor shall use the following methods to control dust and wind erosion of active and inactive stockpiles:

1.	Wet suppression without wetting agent during active stockpile load-in, load-out, and maintenance activities.  Salty or brackish water shall not be used on soils to be planted.

2.	Soil stabilizers applied to the surface of inactive stockpiles.

3.	Plastic tarps on stockpiles, secured with sandbags or an equivalent method to prevent the cover from being dislodged by the wind.  The Contractor shall repair or replace covers whenever damaged or dislodged, at no additional cost to the Department.

4.	Seeding of inactive stockpiles.

B.	The methods to be used shall be submitted to the Engineer as part of the dust control plan and plan for seeding specified under Submittals.

3.05	DEMOLITION DUST CONTROL MEASURES

A.	Closed chutes shall be used for the handling of debris as provided by Section 112.  Dropping or throwing of debris is prohibited.

B.	Debris shall not be stockpiled.  Debris shall be removed promptly from the site.

C.	During transport of debris, the truck cargo area shall be securely covered.

D.	Removal of asbestos-containing material shall be in accordance with Section 722.810.

COMPENSATION

4.01	METHOD OF MEASUREMENT

A.	Calcium chloride and soil stabilizer will be measured by the respective weight of each material applied.

B.	Water will be measured by the volume applied, determined by use of tanks of known capacity or by satisfactorily installed meters.  All measuring devices shall be furnished by the Contractor.

C.	Wetting agents will be measured by weight of wetting agent added to the water to form a solution in accordance with the manufacturer's recommendations.

D.	Street sweeping by wet spray power vacuum street sweeper will be measured by time spent sweeping.

E.	Wind screens, barriers, and covers will be measured by area of material installed.

F.	Seeding will be measured as specified in Section 765.

G.	All other dust control measures specified in this Section will be considered incidental to the Work and will not be measured or paid for separately.

4.02	BASIS OF PAYMENT

A.	Calcium chloride and soil stabilizer will be paid for at the respective Contract unit price per pound.

B.	Water will be paid for at the Contract unit price per M gallons (1000 gallons).

(Item 443 will apply to water for stockpile as well as roadway dust control.)

C.	Wetting agents will be paid for at the respective Contract unit prices per pound.

D.	Street sweeping by wet spray power vacuum street sweeper will be paid for at the Contract unit price per hour.

E.	Wind screens, barriers, and covers will be paid for at the respective Contract unit prices per square yard.

1.	Construction fences will not be paid for under this Section, but are part of the Work of Section 644.010, 850.005, or other Section as applicable.

2.	Covers for contaminated stockpiles will not be paid for under this Section but are part of the Work of Section 120.080.

F.	Seeding will be paid for as specified in Section 765.  Seeding includes all necessary related soil testing, seeds, application water, hydrofiber, tackifier, lime, fertilizer, erosion blankets or netting and hay mulch, irrigation, and mowing.

4.03	PAYMENT ITEMS

440.	Calcium Chloride for Dust Control	Pound

442.	Street Sweeping	Hour

443.	Water for Roadway Dust Control	M Gallons

440.101	Wetting Agents for Dust Control	Pound

440.103	Soil Stabilizer for Dust Control	Pound

444.101	Wind Screen	Square Yard

444.102	Wind Barrier	Square Yard

444.111	Temporary Stockpile Cover	Square Yard

765.44	Seeding for Dust Control	Square Yard