MO6 Guide for Determining Ksat Limits

from Traditional Permeability Classes

| Ksat Class | Ksat (m/sec) | Permeability (in/hr) | Permeability Class | Texture1 |
| --- | --- | --- | --- | --- |
|  | 705.00 (max) | 100.00 | Very Rapid2 | S, VFS, FS, COS |
| Very High | 141.14------------- | 20----------------- | ----------------------------- | --------------------------------------- |
|  |  |  | Rapid | LS, LVFS, LFS, LCOS |
| ----------------------- | 100----------------- | 14.17 |  |  |
| High | 42.34--------------- | 6.0----------------- | ----------------------------- | --------------------------------------- |
|  |  |  | Moderately Rapid | SL, VFSL, FSL, COSL, L, SIL, SI (<20% clay) |
|  | 14.11--------------- | 2.0----------------- | ----------------------------- | --------------------------------------- |
| ----------------------- | 10.00--------------- | 1.417 | Moderate | SCL, L, SIL (20-27% clay) |
| Moderately High | 4.23---------------- | 0.6----------------- | ----------------------------- | --------------------------------------- |
|  |  |  | Moderately Slow | CL, SICL, SCL (27-35% clay) |
|  | 1.41---------------- | 0.20--------------- | ----------------------------- | --------------------------------------- |
| ----------------------- | 1.00---------------- | 0.1417 | Slow | C, CL, SC, SICL, SIC (35-60% clay) |
| Moderately Low | 0.42---------------- | 0.06--------------- | ----------------------------- | --------------------------------------- |
| ----------------------- | 0.10---------------- | 0.01417 | Very Slow | C (>60% clay) |
| Low |  |  |  |  |
| ----------------------- | 0.01---------------- | 0.001417--------- | ----------------------------- | --------------------------------------- |
| Very Low | 0.00 | 0.00 | Impermeable | Bedrock, cemented pans |

1Permeability classes for textures are only estimates.  Measured data should be used if possible.

2Red numbers and class names are for permeability.

Example: SCL with 25% clay has a permeability class of Moderate (0.6-2.0 in/hr) which equates to Ksat limits of 4.23-14.11 m/sec and Ksat Classes of Moderately High and High.  NASIS data entries for Ksat would be 4.23 (low) and 14.11 (high).  RV would be midpoint or some other representative number.

| MO6 Guide for Determining Ksat Limits For Bedrock Materials |  |  |  |
| --- | --- | --- | --- |
| Material |  | Permeability in/hr | Ksat m/sec |
| Sandstone | unfractured (lithic) | 0.0 – 0.2 | 0.0 – 1.41 |
|  | fractured (lithic) | 0.2 – 2.0 | 1.41 – 14.11 |
|  | weathered (paralithic) | 0.2 – 0.6 | 1.41 – 4.23 |
| Shales and Mudstones | consolidated | 0.0 – 0.06 | 0.0 – 0.42 |
|  | weathered (paralithic) | 0.0 – 0.2 | 0.0 – 1.41 |
| Igneous & Metamorphic Rocks | unfractured (lithic) | 0.0 – 0.01 | 0.0 – 0.10 |
|  | fractured (lithic) | 0.01 – 20.0 | 0.10 – 141.14 |
|  | weathered (paralithic) | 0.0 – 0.06 | 0.0 – 0.42 |
| Limestone, Dolomite | unfractured (lithic) | 0.0 – 0.06 | 0.0 – 0.42 |
|  | fractured (lithic) | 0.0 – 0.6 | 0.0 – 4.23 |
|  | weathered (paralithic) | 0.0 – 0.2 | 0.0 – 1.41 |
| Limestone Karst |  | 2.0 – 20.0 | 14.11 – 141.14 |