WPEC SG 21: Assessment of Neutron Cross-Section Evaluations for the Bulk of Fission Products

34-Se-80 Review

I. Sirakov, INRNE, January 23, 2004

Files

ENDF/B-VI:	 Year of origin 1974, taken from ENDF/B-V, no revisions since then

JEF-2:	 Year of origin 1982, data taken from ENDF/B-V, last revision 1989

JENDL-3:	 Year of origin 1990, revisions in 1993 (JENDL-3.2) and 2002 (trivial) when transformed to JENDL-3.3

Thermal and resonance region

ENDF/B-VI, JEF-2: Resolved resonance region up to 6.707 keV with resonance parameters from BNL-325 (1973); SLBW representation for five s-wave and two p-wave resonances; artificial value of J=L=1 given to the unknown spins J from the p-wave; a 1/v component added to give the thermal capture cross section of BNL-325 (1973); scattering radius 5.197 fm. No unresolved resonance region (Table 1). Computed resonance integral 1.08 b. JEF-2 changed the flag to the MLBW representation and randomly assigned the spin J for L=1 by the code RNDSPIN.

JENDL-3: Resolved resonance region up to 10 keV with resonance parameters based on BNL-325 (1981); MLBW representation for 12 s-wave resonances (7 of them being background or out of range) and 4 p-wave resonances; a bound resonance level at -3.69 keV added to reproduce the thermal capture and scattering cross sections according to BNL-325 (Table 2); unknown spins J tentatively assigned by the random number method. Scattering radius 7.5 fm. Unresolved region up to 100 keV; S0 and S1 strength functions based on Mughabghab’s compilation and systematics; S2 calculated with CASTHY; radiation width of 230 meV from Mughabghab’s compilation. Level spacing determined to reproduce the capture cross section from CASTHY; the effective scattering radius of 6.99 fm fitted to the calculated MT=1 at 100 keV. Computed resonance integral 0.976 b.

Note on the resonance region (up to 100 keV for JENDL):

ENDF/B-VI evaluation does not show the negative elastic and total cross sections in the derived files (T=0K, T=300K) since it changes the MT=2 negative output to zero through an ‘optionally’ chosen procedure embedded in the preprocessing codes. Without this change, 3 ranges of negative elastic cross sections (up to -1.74 b) come out and result in negative total cross sections throughout the regions 595-1467 eV, 1473-1681 eV and 3796-4037 eV with lowest values of about -1.7 b. The arbitrary procedure of changing the negative output, followed by recalculation of the imbalanced total cross section, is equivalent to and could have rigorously been organized as a background in MF=3, MT=2 (and MT=1) of the basic evaluated file.

The capture cross section from about hundred eV to several hundred keV has a predominant contribution to the one-group capture cross section (1E-5 eV - 20 MeV) over fast-reactor weighting spectrum (http://www.nndc.bnl.gov/sg21/nucl/acs/accs.htm) and accounts for the one-group values of ENDF/B-VI (57.7 mb), JEF-2 (57.7 mb) and JENDL-3 (41.7 mb). The higher value of ENDF/B-VI/(JEF-2) is related to their higher capture cross section from ~ 10 keV to ~1 MeV.

The 10%-larger capture resonance integral for ENDF/B-VI/JEF-2 (1.08 b) compared to JENDL-3 (0.976 b) is mainly due to its higher capture cross section above 6.7 keV, which contributes more than the JENDL-3 p-wave resonances at 6120 eV and  8150 eV.

Disadvantage of ENDF/B-VI(JEF-2) is the lack of unresolved resonance region.

Experimental data – The recommendation for the thermal capture cross section of 0.61(5) b (Mughabghab 2003, INDC(NDS)-440) fully agrees with the identical values of all evaluations (Table 2). Also, the rather scattered capture cross section data of Walter, 1984 (EXFOR22037 006) above 3 keV, generally agree with all evaluations. The capture cross section data of Tolstikov 1968 (EXFOR40007 005) above 10 keV, however, are only in favor of JENDL-3.

The resonance integral data of Nakamura 1998 (EXFOR22632 005) do not contradict the library values. The other RI data, however, like those of Ricabarra 1968, (EXFOR30129 004), and Heft 1978, (EXFOR12866 051, 053), with values of 1.43(16) b and 1.80(11) b respectively as well as the RI recommendation of 1.6(2) b (Mughabghab 2003, INDC(NDS)-440) infer substantial underestimation (~40%) in the evaluated RI values  and are not in favor of the libraries.

As the elastic scattering is concerned, Koester’s 1980 data (EXFOR21653 008) for a bound coherent scattering length of 7.48(3) fm gives a zero-energy scattering cross section, for the free atom, of 6.95(5) b. This value is in full agreement with the JENDL-3 evaluation of 6.95 b for the zero-energy scattering cross section, in contrast to the ENDF/B-VI and JEF-2 values of 0.675 b and 1.064 b, which are too low and lead to rather low elastic and total cross sections in the thermal region and in the region up to ~1 keV. The resolved ENDF/B-VI(JEF-2) elastic cross section between the resonances is also affected by the low effective scattering radius.

Recommendation/Conclusion: JENDL-3 is recommended up to 100 keV for its better agreement with the experimental data for capture and elastic cross sections in the thermal region and in the regions below 1 and above 10 keV.

ENDF/B-VI and JEF-2 have unacceptably low elastic and total cross sections in the region up to 1 keV and do not show agreement with the capture data above 10 keV (Tolstikov 1968, EXFOR40007 005). In addition, ENDF/B-VI presents negative elastic and total cross sections unless an optional procedure for changing the negative output is used in the preprocessing codes.

Some RI data, as well as the RI recommendation of Mughabghab 2003 (INDC(NDS)-440), show a discrepancy of ~40% with the RI values of all libraries and imply a substantial underestimation of the evaluated capture cross sections in the resolved resonance region (1-10 keV).

Fast neutron region

- Methodology

ENDF/B-VI, JEF-2: Spherical OM (Moldauer potential) for MT=1; capture from NCAP code; inelastic cross sections (MF=3, MT=4, 51-58, 91) from COMNUC-3 code, no preeq corrections; 8 discrete levels (from 666 keV to 1.96 MeV) adopted for the inelastic scattering; elastic cross section obtained as (MT1-MT102-MT4); isotropic angular distribution assumed for MF=4, MT=2 in CM system. Account taken of MT=1, 2, 4, 51-58, 91, 102. JEF-2 includes charge particle production data (MT=103, 104, 105, 106, 107, 111) from REAC-ECN-4 Library.

JENDL-3: Spherical OM and statistical model code CASTHY for MT=1, 4, 51-79, 91, 102 by taking into account competing reactions MT=16, 17, 22, 28, 103, 104, 105, 107 which were calculated with the preeq and multi-step evaporation model code PEGASUS. The gamma-ray strength function (4.66E-5) adjusted at 100 keV to the measured capture cross section (Walter 1984). Direct and semi-direct capture estimated; 29 discrete levels (from 666 keV to 3.35 MeV) adopted for the inelastic scattering. MT=103 and MT=107 normalized to recommended by Forrest (1986) values at 14.5 MeV; elastic calculated as (MT1 – sum of partial cross sections); Legendre coefficients from CASTHY to get anisotropic angular distribution for MF=4, MT=2 and MT=51-79 in CM system. MT=1, 2, 4, 16, 17, 22, 28, 51-79, 91, 102, 103, 104, 105, 107.

- MT=1

JENDL-3 presents a worse than ENDF/B-VI(JEF-2) fit to the rather scattered experimental data (Muselyan 1983, 1987) between 250 and 650 keV. JENDL-3 does not merge smoothly into the upper boundary of the URR (100 keV) and does not follow the trend of the URR. It can be noticed that, around 200 keV, ENDF/B-VI(JEF-2) follows the trend of the recommended URR.

JENDL-3, however, shows better agreement with the experimental data (Muselyan 1983, 1987) in the region 750 keV - 2.5 MeV. The largest discrepancies between JENDL-3 and ENDF/B-VI(JEF-2) are 1 b at 300 keV and 0.55 b at 8 MeV.

- MT=2

ENDF/B-VI and JEF-2 show a better agreement, compared to JENDL-3, with the experimental data available from 1.2 MeV to 8 MeV (Lachkar-1976, EXFOR20426 014; Gorlov-1964, EXFOR40221 015; Konobeevskij-1984, EXFOR40845 014; and Korzh-1983, EXFOR40859 004). It should again be noticed that, around 200-300 keV, ENDF/B-VI(JEF-2) follows the trend of the recommended URR.

Following MT=1, JENDL-3 does not merge smoothly into the upper boundary of the URR (100 keV) and does not follow the trend of the URR.

ENDF/B-VI and JEF-2 are, generally, not as smooth as JENDL-3. The biggest discrepancies between JENDL-3 and ENDF/B-VI(JEF-2) are 1.1 b at 300 keV, 0.55 b at 8 MeV and -0.65 b at 20 MeV.

- MT=4

ENDF/B-VI(JEF-2): Unrealistic trend at high energies; (n,2n) reaction neglected.

JENDL-3 is higher than ENDF/B-VI(JEF-2) up to 10.3 MeV. Although closer to the experimental data of Konobeevskij 1984 at 1.19 MeV (EXFOR40845 015), JENDL-3 is 35% lower the measured values. The (n,2n) threshold is 10 MeV

- MT=16

ENDF/B-VI(JEF-2): No (n,2n) in File 3; assumed to be zero.

JENDL-3: Shows a reasonable shape, being systematically higher but close to the experimental data of Frehaut, 1980 (EXFOR20416 009). The maximum difference is 0.22 b at 12 MeV. The (n,3n) threshold is 17.1 MeV.

- MT=102

Compared to ENDF/B-VI(JEF-2), JENDL-3 is in better agreement with the experimental data of Walter, 1984 (EXFOR22037 006) and Tolstikov 1968 (EXFOR40007 005) in the region 100 keV – 3.1 MeV. Above 300 keV, it slightly overestimates the measured values.

ENDF/B-VI and JEF-2 are systematically high compared to the experimental data and JENDL-3. In addition, the descent that starts at 370 keV is not reasonable on the ENDF/B-VI(JEF-2) curve, since it is supposed to start at the inelastic scattering threshold of 674 keV (see JENDL-3 curve).

- MT=103

ENDF/B-VI: No (n,p) in File 3; assumed to be zero.

JENDL-3 and JEF-2 show reasonable shapes and full agreement with experimental data of Minetti, 1967 (EXFOR21107 021) at 14.7 MeV, but differ substantially (9 times) at 20 MeV being 41 mb and 4.56 mb respectively.

- MT=107

ENDF/B-VI: No (n,) in File 3; assumed to be zero.

JENDL-3 and JEF-2 show reasonable shapes among the inconsistent experimental data found between 2.6(3) mb (Hoang 1989, EXFOR32108 007) and 32(4) mb (Casanova 1976, EXFOR20748 032) at around 14 MeV. Differ substantially at 20 MeV being 43 mb and 9.6 mb respectively.

Recommendation/Conclusion in the fast energy region 100 keV - 20 MeV:  A reasonable recommendation that practically retains the cross section balance, could be JENDL-3 for all cross sections, along with JEF-2 for MT=103 and 107.

More precisely, JENDL-3 is preferable for MT=1, 4, 16, and 102, in spite of some shortcomings described above. ENDF/B-VI and JEF-2 perform best for MT=1 in the region 250-650 keV, and for MT=2 in the region 0.25-10 MeV; Both JENDL-3 and JEF-2 should be taken into account for MT=103 and 107.

----------------------------------------

Table 1. Summary for Resolved and Unresolved Resonance Region: Se-80

| Library | Year | First Authors | RRR | URR | Remarks |
| --- | --- | --- | --- | --- | --- |
| ENDF/B-VI | 1974 | Schenter (HEDL), Schmittroth (HEDL) | 6.707 keV SLBW | - | RR parameters from BNL-325 (1973); Five s-wave and two p-wave resonances; artificial value of J=L=1 given to the unknown spins J from the p-wave; a 1/v component added to give the thermal capture cross section of BNL-325 (1973); scattering radius 5.197 fm. Computed resonance integral 1.08 b. |
| JEF-2 | 1982 | Scientific Co-ordination Group | 6.707 keV MLBW | - | RR parameters taken from ENDF/B-V. Flag changed to MLBW. Spin J, for L=1, assigned randomly by the code RNDSPIN. |
| JENDL-3 | 1990 | JNDC FP Nuclear Data Working Group | 10 keV MLBW | 100 keV | RR parameters based on BNL-325 (1981); 12 s-wave resonances (7 of them being background or out of range) and 4 p-wave resonances; a bound resonance level at -3.69 keV added to reproduce the thermal capture and scattering cross sections according to BNL-325 (Table 2). Unknown spins J tentatively assigned by the random number method; Scattering radius 7.5 fm. URR: S0 and S1 strength functions based on Mughabghab’s compilation and systematics; S2 calculated with CASTHY; radiation width of 230 meV from Mughabghab’s compilation. Level spacing determined to reproduce the capture cross section from CASTHY; the effective scattering radius of 6.989 fm fitted to the calculated MT=1 at 100 keV. Computed resonance integral 0.976 b. |

Table 2. Thermal (2200 m/s) Cross Sections for Se-80

| Library / Experiment | Total | Capture | Elastic |
| --- | --- | --- | --- |
| ENDF/B-VI | 1.29 b | 0.61 b | 0.68 b |
| JEF-2 | 1.67 b | 0.61 b | 1.06 b |
| JENDL-3 | 7.56 b | 0.61 b | 6.95 b |
| BNL-325 (1981) |  | 0.61(5) b |  |
| S.F.Mughabghab, INDC(NDS)-424, p.73 (2001) |  | 0.61(5) b |  |
| S.F.Mughabghab, INDC(NDS)-440, p.11 (2003) |  | 0.61(5) b |  |