CABLE, SINGLE FIBER, MULTIMODE, SPACE QUALITY,

SPECIFICATION CUSTODIAN:

THE BOEING COMPANY

SPACE AND COMMUNICATIONS DIVISION

2100 SPACE PARK DRIVE

HOUSTON TEXAS, 77058

CAGE CODE: 2B945

INTERNATIONAL SPACE STATION PROGRAM

SSQ 21654 Revision D

National Aeronautics and Space Administration

International Space Station Program

Johnson Space Center, Houston Texas.

Contract Number: NAS15-10000

REVISION AND HISTORY PAGE

| REV LTR | DESCRIPTION OF CHANGE | Author and Date | Approval Signature |
| --- | --- | --- | --- |
|  | BID ISSUE RELEASE | MDSSC-SSD 03-01-90 | S. J. Stock |
| NEW | Completely revised and redrawn Incorporated PDR RID comments, Supplier comments and all applicable comments from all known sources. | MDSSC-SSD 10-09-91 | S. J. Stock |
| A | See Revision “A” Release Engineering Order for the complete details of the changes incorporated in Revision A. | MDSSC-SSD | S. J. Stock |
| B | Official PCB Release Version (include CR 21654-002, 21654-004 R1, CR 21654-005 R2 and CR 21654-006) | 07-02-96 K Roopwah | P. A. Swartzell |
| C | Revision C authorized by SSCN 000904. | 03-07-00 J. A. White | Curt Tallman |

SSQ 21654 Space Station Program, Cable, Single Fiber, Multimode, Space Quality, General Specification, details the requirements and characteristics for a single fiber cable with a glass optical fiber and the appropriate processes, procedures and tools.

__________________________________                                               ______________

Program Manager (or delegated authority)                                                           Date

Space Station Program

SPACE STATION PROGRAM OFFICE

CABLE, SINGLE FIBER, MULTIMODE, SPACE QUALITY

GENERAL SPECIFICATION

May 03, 1999

PREPARED BY:

PRINT NAME		   			ORGANIZATION

SIGNATURE						DATE

CHECKED BY:

PRINT NAME		   			ORGANIZATION

SIGNATURE						DATE

SUPERVISED BY:

PRINT NAME 				      ORGANIZATION

SIGNATURE						DATE

DQA		 Kinn Roopwah                    	              5-5332

PRINT NAME                                                          	ORGANIZATION

/s/ Kinn Roopwah                	              2-18-00                            				  	            SIGNATURE		DATE

TABLE OF CONTENTS

1.0 	Scope		1-1

1.1 	Purpose		1-1

1.2 	Intent		1-1

1.3	Interconnect System	1-1

1.3.1 	Tools, Processes, And Procedures	1-1

1.4 	Classification	1-2

1.5 	Document Generation	1-2

1.6 	Configuration Management	1-2

1.6.1 	Specification Changes	1-2

1.6.2 	Specification Distribution	1-2

1.6.3 	Procuring Activity	1-2

1.7 	Specification Custodian	1-3

1.8 	Approved Sources	1-3

2.0 	Applicable Documents	2-1

2.1 	Government Documents	2-1

2.1.1 	Specifications	2-1

2.1.1.1 	Federal	2-1

2.1.1.2 	Military	2-1

2.1.2 	Standards	2-1

2.1.2.1 	Federal	2-1

2.1.2.2	Military	2-2

2.2 	Other Publications	2-2

2.3 	Order Of Precedence – Deleted	2-3

2.4 	Interpretation	2-3

2.5 	Communications	2-3

3.0 	Requirements	3-1

3.1 	Item Description	3-1

3.2 	General Requirements	3-1

3.2.1	 Data Required For Design Approval	3-1

3.2.1.1 	Cross-Sectional Drawings	3-1

3.2.1.2	Exceptions And Deviations	3-1

3.2.2	 Data Required For Final Design Approval	3-1

3.2.2.1 	Test Procedures	3-1

3.2.2.1.1	Qualification Test Procedures	3-2

3.2.2.1.1.1 	Qualification Test Lengths	3-2

3.2.2.1.1.2 	Qualification Inspection Routine	3-2

3.2.2.1.1.3 	Qualification Test Discrepancy	3-2

3.2.2.1.1.4 	Qualification Test Failure	3-2

3.2.2.1.1.5 	Qualification Reports	3-2

3.2.2.1.1.5.1	Qualification By Similarity	3-3

3.2.2.1.1.5.2	Qualification By Analysis	3-3

3.2.2.1.1.6 	Qualification Test Sample Disposition	3-3

4

TABLE OF CONTENTS (continued)

3.2.2.1.2 Acceptance Test Procedures                                                                                          3-4

3.2.2.1.3 	Discrepancy/Failure Analysis	3-4

3.2.2.2 	Qualification Test Reports -  Deleted And Moved To 3.2.2.1.1.5	3-4

3.2.2.3 	Top Assembly Drawings	3-4

3.2.2.4 	Materials List	3-4

3.2.3 	Baseline Configuration Control	3-4

3.2.3.1 	Baseline Control Document	3-5

3.2.3.2 	Baseline Control Deviations	3-5

3.3  	Materials - Deleted	3-5

3.3.1 	General - Deleted	3-5

3.3.2 	 Fungus – Deleted And Moved To 3.9.1	3-5

3.3.3	 Offgassing/Toxicity, Odor, Flammability, And Outgassing -  Deleted	3-5

3.3.3.1 	Offgassing/Toxicity – Deleted And Moved To 3.9.2	3-5

3.3.3.2 	Odor – Deleted And Moved To 3.9.3	3-5

3.3.3.3 	Flammability – Deleted And Moved To 3.9.4	3-5

3.3.3.4 	Outgassing (Vacuum Stability) – Deleted And Moved To 3.9.5	3-5

3.3.3.5 	Atomic Oxygen Effects (4.10.6) - Deleted	3-5

3.4 	Design And Construction - Deleted	3-5

3.4.1 	Optical Fiber – Deleted And Moved To 3.11.1	3-5

3.4.1.1 	Optical Fiber Buffer – Deleted And Moved To 3.10.1.2	3-5

3.4.2 	Cable – Deleted And Moved To 3.11.2	3-5

3.4.2.1 	Strength Member – Deleted And Moved To 3.11.2.2	3-5

3.4.2.2 	Cable Jacket – Deleted And Moved To 3.11.2.3	3-5

3.4.2.3 	Color – Deleted And Moved To 3.11.2.4	3-5

3.4.2.4 	Lengths And Splices – Deleted And Moved To 3.11.2.5	3-5

3.4.2.5 	Cable Weight – Deleted And Moved To 3.10.6	3-5

3.5 	Physical – Deleted And Moved To 3.11	3-5

3.5.1 	Fiber – Deleted And Moved To 3.11.1	3-5

3.5.1.1	 Numerical Aperture – Deleted And Moved To 3.8.1	3-5

3.5.1.2	 Optical Fiber Bandwidth – Deleted And Moved To 3.8.2	3-5

3.5.2 	Cable – Deleted And Moved To 3.11.2	3-5

3.5.2.1 	Attenuation Rate – Deleted And Moved To 3.8.3	3-5

3.5.2.2  	Attenuation Measurements – Deleted And Moved To 3.8.3	3-5

3.5.2.2.1 	Induced Attenuation – Deleted And Moved To 3.8.3.2	3-5

3.5.2.3-2.2  Substitution Loss – Deleted And Moved To 3.8.3.1	3-5

3.5.-33 	Radiation Resistance – Deleted And Moved To 3.9.6	3-5

3.3-5.4	 Thermal Shock – Deleted And Moved To 3.9.7	3-5

3-3.5.5 	 Survival Temperature (Cr21654-005 Rev. 1) – Deleted And Moved To 3.9.8	3-5

3.5.6 	Crush (Cr21654-005 Rev 1) – Deleted And Moved To 3.10.4	3-5

3.5.7 	Cable Bend – Deleted And Moved To 3.10.5	3-5

3.5.8 	 Jacket Shrinkage – Deleted And Moved To 3.11.2.3.2	3-5

3.5.9 	Vacuum  - Deleted	3-5

TABLE OF CONTENTS (continued)

3.6	Identification Marking (4.10.7)	3-5

3.6.1	 Durability Of Identification - Deleted	3-5

3.7 	Workmanship (4.10.7)	3-5

3.8	Optical		3-5

3.8.1 	Numerical Aperture	3-5

3.8.2 	Optical Bandwidth	3-6

3.8.3 	Attenuation Rate (4.10.12)	3-6

3.8.3.1	 Substitution Loss (4.10.12.4)	3-7

3.8.3.2  	Induced Attenuation (4.10.12.3)	3-7

3.9 	Environmental	3-7

3.9.1 	 Fungus	3-11

3.9.2 	Toxicity	3-11

3.9.2.1 	By Analysis	3-11

3.9.2.2 	By Test	3-11

3.9.3 	Odor		3-11

3.9.3.1 	By Analysis	3-11

3.9.3.2  	By Test	3-11

3.9.4 	Flammability	3-11

3.9.5 	Outgassed Products/Vacuum Stability	3-11

3.9.5.1 	By Analysis	3-11

3.9.5.2	By Test	3-11

3.9.6 	 Radiation Resistance	3-11

3.9.7 	 Thermal Shock	3-11

3.9.8  	Survival Temperature/Vacuum	3-11

3.10  	Mechanical	3-11

3.10.1 	 Strippability	3-12

3.10.1.1 	Cable Buffer Strippability	3-12

3.10.1.2	Fiber Buffer Strippability (4.10.7.7)	3-12

3.10.1.3	Cable Jacket Strippability (4.10.7.7)	3-12

3.10.2 	 Fiber Proof Test	3-12

3.10.3 	Cycling Flexing (4.10.20)	3-13

3.10.4 	Crush	    	3-13

3.10.5 	Cable Bend	3-14

3.10.6 	Cable Weight	3-14

3.11 	Physical	3-14

3.11.1 	Fiber And Fiber Buffer	3-14

3.11.1.1 	Fiber Core  (4.10.7.4.3)	3-14

3.11.1.2 	Fiber Clad  (4.10.7.4.3)	3-15

3.11.1.3 	Fiber Coating  (4.10.7.4.3)	3-15

3.11.2 	Cable (4.10.7)	3-15

3.11.2.1	Cable Buffer (If Applicable) (4.10.7)	3-15

3.11.2.2 	Cable Strength Member (4.10.7)	3-15

3.11.2.3 	Cable Jacket (4.10.7)	3-15

TABLE OF CONTENTS (continued)

3.11.2.3.1	Cable Jacket Outside Diameter (4.10.7)	3-16

3.11.2.3.2 	Cable Jacket Stability	3-16

3.11.2.4 	Cable Jacket Color  (4.10.7)	3-16

3.11.2.5 	Lengths And Splices	3-16

4.0 	Quality Assurance Provisions	4-1

4.1 	General		4-1

4.2  	Responsibility For Inspection	4-1

4.2.1 	Contractor Responsibilities	4-1

4.2.2 	Test Equipment And Inspection Facilities	4-2

4.2.3 	Test Equipment Error	4-2

4.3 	Product Assurance Program	4-2

4.3.1 	Quality And Reliability Assurance Plan	4-2

4.3.2  	Critical Hardware Notation	4-2

4.3.3  	Flowchart - Deleted	4-3

4.3.4 	 Failure Mode Effects Analysis - Deleted	4-3

4.3.5 	Discrepancy/Failure Analysis - Deleted And Moved To 3.2.2.1.3	4-3

4.3.6	 Disposition And Corrective Action	4-3

4.3.7	 On Orbit Discrepancy/Failure Report Activity - Deleted	4-3

4.3.8 	Product History And Database - Deleted	4-3

4.3.8.1 	History - Deleted	4-3

4.3.8.2 	Database - Deleted	4-3

4.4 	Inspection And Test	4-3

4.4.1 	Qualification Inspection – Deleted And Moved To 3.2.2.1.1.2	4-3

4.5 	Inspection Conditions – Deleted	4-3

4.6 	Preparation Of Optical Test Samples	4-3

4.7 	Materials Inspection	4-3

4.8  	Qualification Inspection – Deleted And Moved To 3.2.2.1.1.2	4-3

4.8.1 	Qualification – Deleted And Moved To 3.2.2.1.1.2	4-3

4.8.1.1 	Sample Unit Length – Deleted And Moved To 3.2.2.1.1.1	4-3

4.8.1.2 	Inspection Routine – Deleted And Moved To 3.2.2.1.1.2	4-3

4.8.2 	 Qualification Test Procedure – Deletedand Moved To 3.2.2.1.1	4-3

4.8.3	 Qualification Test Discrepancy – Deleted And Moved To 3.2.2.1.1.3	4-3

4.8.4 	Qualification Test Failure – Deleted And Moved To 3.2.2.1.1.4	4-3

4.8.5 	Qualification Test Report – Deleted And Moved To 3.2.2.1.1.5	4-3

4.8.6 	Disposition Of Test Samples – Deleted And Moved To 3.2.2.1.1.6	4-3

4.8.7 	Qualification By Similarity -  Deleted And Moved To 3.2.2.1.1.5.1	4-3

4.8.8	 Qualification By Analysis – Deleted And Moved To 3.2.2.1.1.5.2	4-3

4.9 	Product Acceptance Test	4-3

4.9.1 	Acceptance Of Product For Delivery	4-3

4.9.2 	Unit Of Manufacture	4-4

4.9.3 	Sample Unit	4-4

4.9.4 	Product Acceptance Test Procedures - Deleted	4-4

4.9.5 	Destructive Physical Analysis (Dpa) - Deleted	4-4

TABLE OF CONTENTS (continued)

4.9.6  	Receiving Inspection - Deleted	4-4

4.10 	 Methods Of Inspection - Deleted	4-4

4.10.1 	Fungus (3.3.2) – Deleted And Moved To 3.9.1	4-4

4.10.2 	Offgassed Products Test (3.3.2) - Deleted And Moved To 3.9.2	4-4

4.10.3 	Odor Test – Deleted And Moved To 3.9.3	4-4

4.10.4 	Flammability – Deleted And Moved To 3.9.4	4-4

4.10.5 	Outgassed Products Test – Deleted And Moved To 3.9.5	4-4

4.10.6 	 Atomic Oxygen Effects - Deleted	4-4

4.10.7 	Visual And Mechanical Examinations	4-4

4.10.7.1 	General	4-4

4.10.7.2 	Test Equipment	4-6

4.10.7.3 	Test Specimen	4-6

4.10.7.4 	Test Procedure	4-6

4.10.7.4.1	Visual Examination	4-6

4.10.7.4.2  	Mechanical Examination	4-6

4.10.7.4.3  	Fiber And Coating Examination (3.11.1)	4-6

4.10.7.5	Cable Weight – Deleted And Moved To 3.10.6	4-6

4.10.7.6	Durability Of Color Markings - Deleted	4-6

4.10.7.6.1	Durability Testing Apparatus - Deleted	4-6

4.10.7.6.2	Durability Testing Procedure - Deleted	4-6

4.10.7.7 	Fiber Buffer Strippability (3.10.1.2)	4-6

4.10.8	 Fiber Proof Test - Deleted And Moved To 3.10.2	4-7

4.10.9 	 Numerical Aperture – Deleted  And Moved To 3.8.1	4-7

4.10.10 	Optical Bandwidth – Deleted  And Moved To 3.8.2	4-7

4.10.11 	Attenuation Rate – Deleted And  Moved To 3.8.3	4-7

4.10.12	Attenuation Measurements	4-7

4.10.12.1	Optical Test Set	4-7

4.10.12.1.1 	Light Source	4-7

4.10.12.1.2  	Source Monitoring Equipment (Sme)	4-8

4.10.12.1.3 	Detection Equipment	4-8

4.10.12.1.4  	Reference Cable Set	4-8

4.10.12.1.5 	Mtc Cable	4-10

4.10.12.2  	Optical Test Set Calibration	4-10

4.10.12.2.1  	Meter And Source Calibration Using The Mtc Cable	4-11

4.10.12.2.2 	Srd Cable Set Calibration	4-11

4.10.12.3  	Induced Loss (3.8.3.2)	4-12

4.10.12.3.1 	Optical Measurements	4-12

4.10.12.3.2  	Test Set-Up	4-12

4.10.12.3.3  	Test Procedure	4-13

4.10.12.3.4  	Induced Loss Calculations	4-13

4.10.12.4 	Substitution Loss (3.8.3.1)	4-13

4.10.12.4.1 	Optical Measurements	4-13

4.10.12.4.2	 Test Set-Up	4-14

TABLE OF CONTENTS (continued)

4.10.12.4.3 	Test Procedure	4-14

4.10.12.4.4  	Substitution Loss Calculations	4-14

4.10.13 	Radiation Resistance - Deleted And Moved To 3.9.6	4-20

4.10.14 	Thermal Shock – Deleted And Moved To 3.9.7	4-20

4.10.14.1	Test Equipment - Deleted	4-20

4.10.14.2 	Test Sample - Deleted	4-20

4.10.14.3	Test Setup - Deleted	4-20

4.10.14.4	Test Procedure - Deleted	4-20

4.10.14.5	Test Documentation - Deleted	4-20

4.10.15 	Survival Temperature/Vacuum – Deleted And Moved To 3.9.8	4-20

4.10.16 	Crush  (3.10.4) -  Deleted And Moved To 3.10.4	4-20

4.10.16.1	Test Equipment - Deleted	4-20

4.10.16.2 	Test Sample - Deleted	4-20

4.10.16.3	Test Procedure - Deleted	4-20

4.10.16.4	Test Documentation - Deleted	4-20

4.10.17 	Cable Bend- Deleted And Moved To 3.10.5	4-20

4.10.17.1	Test Equipment - Deleted	4-20

4.10.17.2 	Test Specimen -Deleted	4-20

4.10.17.3	 Test Procedure – Deleted	4-20

4.10.17.4  	Test Documentation - Deleted	4-20

4.10.18  	Jacket Shrinkage – Deleted And Moved To 3.11.2.3.2	4-20

4.10.18.1	Test Equipment – Deleted	4-20

4.10.18.2 	Test Sample - Deleted	4-20

4.10.18.3 	Test Setup - Deleted	4-20

4.10.18.4 	Test Procedure - Deleted	4-20

4.10.18.5 	Test Documentation - Deleted	4-20

4.10.19  	Vacuum  - Deleted	4-20

4.10.19.1  	Test Equipment - Deleted	4-20

4.10.19.2  	Test Sample - Deleted	4-20

4.10.19.3  	Test Procedure - Deleted	4-20

4.10.19.4  	Test Documentation - Deleted	4-20

4.10.20 	Cycling Flexing (3.10.3)	4-20

5.0 	Preparation For Delivery	5-20

5.1 	Cleanliness	5-1

5.2	Packaging Requirements	5-1

5.3	Reels And Spools	5-1

5.4	Product And Package Marking	5-1

5.4.1 	 Marking Of Reels And Spools	5-1

5.4.2 	 Space Station Critical Hardware	5-2

5.4.3 	 Packaging Labels	5-2

5.5	Unit Protection And Wrapping	5-2

5.6	Unit Packaging	5-3

5.7	Packing Containers	5-3

TABLE OF CONTENTS (continued)

6.0 	Notes		6-1

6.1 	Part Numbers	6-1

6.2 	Approved Source(S) Of Supply	6-1

7.0 	Requirements Verification Matrix	7-1

AppendiCes

Appendix A 	Item Detail Specification  	A-1

Appendix B 	Cable Preparation For Termination Procedure - Deleted	B-5

Appendix C1	Qualification Test Procedure – Deleted	C1-5

Appendix C2	Acceptance Test Procedures - Deleted	C2-1

Appendix C3	Receiving Inspection Procedure - Deleted	C3-1

Appendix C4	Procedure, Destructive Physical Analysis -Deleted	C4-1

Appendix D  	Supplier Configuration Control Documentation – Deleted	D-1

LIST OF TABLES

Table I  	Classification of Test - Deleted	4-4

Table II 	Qualification Inspection - Deleted 	4-5

Table III 	Acceptable Loss	3-7

Table IV	 Flammability Test Requirement Details	3-10

Table V 	Outgassed Products/Vacuum Stability Test Conditions	3-11

Table VI 	Far Field Output	4-7

Table VII 	Equipment Resolution Requirements	4-8

1

Table VIII 	Requirements Verification Matrix	7-1

Table A-I 	FIBER/CABLE DIMENSIONS	A-3

LIST OF FIGURES

Figure 1	Optical Test Set-up For Vacuum Stability – Deleted                                         4-14

Figure A-1	Cable Configuration                                                                                            A-4

# SCOPE

Purpose

The purpose of this specification is to detail the requirements and characteristics for a single fiber cable with a glass optical fiber and the appropriate processes, procedures and tools.  This cable is suitable for service in a manned, zero gravity, low earth orbit for Space Station with a minimum life expectancy of 10 years.

## Intent

The SSQ 21654 cable specified herein is intended for use with the size 16 optical termini specified in SSQ 21635, SSQ 21636, and SSQ 21637 type connector inserts  and in environments that includes a space vacuum and a normally pressurized habitat. This document is intended to serve as the procurement specification.  This cable will be installed in the pressurized and unpressurized areas of the International Space Station.

## Interconnect System

The fiber optic cable is part of an interconnect system that includes fiber optic termini, connectors, accessories, wire and cable, tools, assembly processes and procedures and, as such, are commonality color coded to aid in their proper match in

The specific color assignments are usually found in the appendix  which  contains the drawings of contacts, or in a table listing color assignments by wire size or listed as a requirement with other detail characteristics (fiber optic cable).  For example, the fiber optic cable defined in this SSQ is color coded violet and the fiber optic termini used with this cable will have a violet stripe on the barrel to indicate it is used with this cable.  The contact will also have strips designating the contact size and connector type.  Anyone familiar with the system can tell at a glance it is a correct combination.

#### Tools, processes, and procedures

Appendices are included within this specification providing tools and fiber/cable stripping processes and procedures.  Users with certified tools and applicable processes and procedures in place need only demonstrate that those tools, processes and procedures meet the requirements applicable to the subject at hand.

Users without adequate tools and applicable processes and procedures are provided with a list of tools and applicable processes and procedures which will assure compliance to the requirements specified herein.

## Classification

## Document generation

This specification, revision, text and graphics, is computer generated. The text software is Microsoft® Word.  This software contains conversion capability for many major word processor programs. See the Microsoft® Word user’s reference.

Configuration management

This NASA specification is under configuration management by the NASA Space Station program, Johnson Space Center, Houston, Texas. Parts defined by and manufactured to this specification are configuration controlled by part number, NOT by specification revision letter.

### Specification changes

Changes to this specification shall be reviewed and approved by the NASA International Space Station Program Office, or their designee, prior to official release of the revision containing those changes.

### Specification distribution

Released copies of this specification are available from the specification custodian and/or the NASA International Space Station Program Office, Parts Control Board, Johnson Space Center, Houston, Texas.

Procuring activity

The procuring activity is responsible for supplier negotiations dealing with their procurement effort. This includes any development associated with product not developed and tooled as part of the initial release of this specification.

The procuring activity is responsible for the coordination of all specification related issues with the specification custodian such that development status and any related changes may be incorporated.

A development status data base shall be maintained by the specification custodian and/or the NASA International Space Station Program Office with the intent of incorporation in a program wide data base (EPIMS) when available.

Specification custodian

The contractor(s) shown on the title page of this specification have been designated by the NASA International Space Station Program Office, Johnson Space Center, Houston Texas as the specification custodian and is tasked to generate this specification and provide related custodial activities on behalf of NASA.

The specification custodian, in conjunction with the NASA Space Station Program Office, Johnson Space Center, Houston Texas, or their designee, is responsible for program coordination, supplier coordination, technical evaluation and incorporation of comments, and final preparation of revisions to this specification.

The specification custodian, with approval of the NASA Space Station Program Office or their designee, may assign any portion, or all, of these duties to any active participant in the Space Station program.

The specification custodian shall act in concert with the joint work package connector board and the procuring activity, or as directed by the NASA International Space Station Program Office or their designee, in all issues regarding this specification.

Approved sources

Approved suppliers shall allow user review of any and all listed materials, processes, procedures, drawings, or any other item of interest related to this specification, at the supplier’s facility or mutually agreed to location.  Should disagreement arise, direction from the NASA International Space Station Program Office via the specification custodian shall be requested.  The decision of the NASA International Space Station Program Office shall be final.

It is intended that data deemed “proprietary” by the supplier shall be protected to the best of the user’s ability.

Approved Sources are documented in 6.0.

APPLICABLE DOCUMENTS

# Government documents

# The following documents, of the exact issue shown, or if no issue is specified, the issue in effect at the date of invitation to bid, form a part of this specification to the extent specified herein.  In the event of conflict between documents referenced here and the contents of 3.0, 4.0, 5.0, 10.0, and the associated Appendices, the requirements detailed by this specification are considered the superseding requirements. See 1.0, scope.

    - Specifications

#### Federal

# Military

- Applicable Military Specifications

| JSC-SPEC-SP-R-0022 | General Specification Vacuum Stability Requirements of Polymeric Material for Spacecraft Applications |
| --- | --- |
| NHB 5300.4 (3G) | Inspection System Provisions for Suppliers of Space Materials, Parts, Components and Services |
| NHB 8060.1B | Flammability, Odor and Offgassing Requirement Test Procedures for Materials in Environments that Support Combustion |
| SSP 30233 | Space Station Requirements for Materials and Processes, General Specification for |
| SSP 30512 | Space Station Ionizing Radiation Design Environment |
| MSFC-HDBK-527/ JSC 09604 | Materials Selection For Space Hardware Systems |

### Standards

#### Federal

- Applicable Federal Standards

| FED-STD-228 | Cable and Wire, Insulated; Methods of Testing. |
| --- | --- |

#### Military

- Applicable Military Standards

| DOD-STD-1678 | Fiber Optic Test Methods and Instrumentation |  |
| --- | --- | --- |
| MIL-C-12000 |  | Cable, Cord, and Wire, Electric, Packaging of |
| MIL-B-81705 |  | Barrier Materials, Flexible, Electrostatic-Free, Heat Sealable |
| MIL-STD-104 | Limits for Electrical Insulation Color |  |
| MIL-STD-129 | Marking for Shipment and Storage. |  |
| MIL-STD-202 | Test Methods for Electronic and Electrical Component Parts |  |
| MIL-STD-454 | Standard General Requirements for Electronic Equipment |  |
| MIL-STD-810 | Environmental Test Methods and Engineering Guidelines |  |
| MIL-STD-45662 | Calibration Systems Requirements. |  |

(Copies of specifications and standards required by contractors in connection with a specific acquisition should be obtained from the contracting activity or as directed by the contracting activity.)

## Other publications

The following documents form a part of this specification to the extent specified herein

#### ELECTRONIC INDUSTRIES ASSOCIATION (EIA)

- Applicable Industry Publications

| EIA/TIA-455-13 | Visual and Mechanical Inspections of Fibers, Cables, Connectors and Other Fiber Optic Devices.(DOD Adopted) |
| --- | --- |
| EIA/TIA-455-20 | Measurement of Change in Optical Transmittance. |
| EIA/TIA-455-30 | Frequency Domain Measurement of Multimode Optical Fiber Information Transmission Capacity |
| EIA/TIA-455-31B | Fiber Tensile Proof Test Method |
| EIA/TIA-455-37A | Low or High Temperature Bend Test for Fiber Optic Cables |
| EIA/TIA-455-41A | Compressive Load Resistance of Fiber Optic |
| EIA/TIA-455-45B | Method for Measuring Optical Fiber Geometry Using a Laboratory Microscope |
| EIA/TIA-455-46 | Spectral Attenuation Measurement for Long-Length, Graded Index Optical Fibers |
| EIA/TIA-455-47B | Output Far-Field Radiation Pattern Measurements. (DOD Adopted) |
| EIA/TIA-455-60 | Measurement of fiber or Cable Length Using an OTDR |
| EIA/TIA-455-61 | Measurement of Fiber or Cable Attenuation Using an OTDR |
| EIA/TIA-455-177A | Numerical Aperture Measurement of Graded-Index Optical Fibers |

Interpretation

Should the requirements of this specification and/or any associated documents become subject to conflicting interpretation, clarification shall be formally requested.

In the event of a conflict between the text of this specification and the references cited herein, the text of this specification shall take precedence.

Communications

All written communications shall be through the NASA International Space Station Program Office, Johnson Space Center, Houston, Texas, the specification custodian or the procuring activity. The procuring activity shall be responsible for assuring program coordination (see 1.7).

REQUIREMENTS

Item description

The design, construction, and physical dimensions of the complete cable and cable elements shall be as specified herein and in appendix A.  The cable shall consist of one optical transmission element surrounded by protective layers to provide the dimensional centering, circularity, and concentricity as specified.  The cable shall be of a circular cross-section and concentric geometry.

## General requirements

### Data required for design approval

Prospective suppliers shall submit one reproducible copy of the following for technical evaluation:

Cross-sectional drawings

The cross-sectional drawing shall show all internal and external dimensions and their tolerances; identification of all materials and coatings (by chemical composition or by original manufacture’s part number) used in the construction of the cable; all methods for processing the materials; the location and size of marking; the maximum guaranteed weight.  The drawing may be exaggerated dimensionally.

Exceptions and deviations

Prospective suppliers shall submit a list of exceptions to or deviations from the conditions or requirements of the specification detailing by paragraph number the specifics of exceptions or deviations and the rationale or reasons thereof. The schedule and/or cost impact of not allowing each exception or deviation shall be identified.

Data required for final design approval

#### Test procedures

Detailed qualification and acceptance test procedures shall be submitted to and approved by the specification custodian prior to the start of testing.

##### Qualification test procedures

Detailed qualification test procedures shall be submitted for approval by the specification custodian four weeks minimum prior to scheduled start of qualification testing. The qualification test limits shall include test margins to guarantee a 15 year service life in the environments specified herein.

###### Qualification test lengths

Unless otherwise specified, the minimum sample unit length shall be 225 meters, and the sample size shall consist of one sample unit: 200 meters shall be retained by the manufacturer for qualification tests and 25 meters shall be supplied to specification custodian to be forwarded to NASA for materials testing.

###### Qualification inspection routine

The sample shall be subjected to the qualification inspection specified in Table IX, however the order may change from that shown.  Fiber dimensional measurements must be performed prior to cabling and sample lengths shall be as required for the individual measurement. In the event of design change(s) in the cable, the qualifying activity reserves the right to require additional inspection of the first unit which reflects the design change. Qualification inspection shall be performed on sample units produced with equipment and procedures normally used in production at a laboratory acceptable to the government and the specification custodian.

###### Qualification test discrepancy

Any discrepancy that occurs during testing shall be reported to the specification custodian within 24 hours.  Testing shall continue unless otherwise advised by the specification custodian.  The supplier shall prepare a description of the discrepancy, an analysis of the cause, corrective action proposed, and the extent of retest if applicable.

###### Qualification test failure

If a failure occurs during qualification tests, testing shall cease, the supplier shall notify the specification custodian at once verbally and shall follow-up with a written notification describing the cause of failure and the proposed corrective action.  Written specification custodian approval of such action must be given prior to resumption of testing.

###### Qualification reports

A detailed qualification test report shall be submitted for approval by the specification custodian within four weeks following completion of testing to provide a record that the articles tested comply with the qualification test requirements of the specification.  The report shall be certified by an authorized representative of the company, and shall contain the following information for which approval is requested:

- Certification of the fact that the test samples were in conformance with and tested to the latest approved revision of the specification with respect tot he design, construction, materials, workmanship, marking, manufacturing processes and controls, test procedures, ect.

- Dated and signed examination and test results verifying that the samples complied with all the qualification test requirements of this drawing including:

1.	Measured or observed test conditions at the time of test or examination as required.

2.	All pertinent qualitative observations and quantitative values obtained by measurement, including oscillographs, recordings, charts, photographs, etc.

3.	Calculated values and sample calculations.

- References to applicable drawings or documents (design drawing, test procedure, Q.C. procedure, etc.) cross-correlated to the specific requirements and test method paragraph of this specification for each of the examination and test parameters itemized in Table VIII.

Qualification by similarity

If a cable, which is similar in materials and construction to what is specified herein, has previously been subjected to, and successfully passed, tests which are the equivalent of those required herein, it may be possible to utilize the previous tests in lieu of part or all of the testing required by this specification.  In such a case, the supplier may submit a statement which identifies the similarities and differences between the previously tested cable and the cable specified herein. The statement should cover design features, construction and materials of the cable to permit a competent evaluation of the similarity of the cables and the validity of the comparison.  The statement, together with certified copies of the earlier tests reports, shall be submitted for the specification custodian approval in advance of the start of qualification testing.  If approved, they shall also be included as part of the qualification test report.

Qualification by analysis

Should any of the requirements be qualifiable by analytical processes the supplier shall submit that analysis to the specification custodian for approval in advance of the start of qualification testing. If approved the analyses shall be included as part of the qualification test report.

Qualification Test Sample Disposition

All test samples, or the remains thereof, shall be clearly identified as qualification test samples, along with their appropriate part number.  The test samples shall then be protectively packaged and, unless otherwise specified in the purchase contract, shipped to the specification custodian in a container marked “Qualification Samples - Do Not Use”. The cable samples shall be conspicuously marked with at least two red “X’s” in a manner which does not obscure other markings.

Acceptance test procedures

Detailed acceptance test procedures shall be submitted for approval by the specification custodian six weeks minimum prior to scheduled start of acceptance testing.

##### Discrepancy/failure analysis

The manufacturer is responsible to provide complete failure analysis of all failures.  The analysis shall consist of an analysis plan, a narrative of the analysis activity and full documentation of any anomalies found, and recommend action.

Discrepancies/failures reported from any source shall be brought to the attention of the specification custodian, or NASA designated technical representative, within 5 working days of receipt by the manufacturer.  The manufacturer shall analyze each report and shall transmit their recommended disposition or corrective action to the specification custodian within 10 working days of receipt of the report.  Review of the proposed recommendations or corrective actions and notification of acceptance status shall be provided to the manufacturer within 5 working days of receipt of the report.

#### Top assembly drawings

The final top assembly drawing identified in the baseline control document shall be submitted to the specification custodian four weeks maximum prior to the start of production.

#### Materials list

A complete list of materials, including material rating in accordance with

MSFC-HDBK-527/JSC-09604, quantity and exposed surface area shall be submitted four weeks prior to start of production.

### Baseline configuration control

Configuration control will be accomplished by baseline control documentation. The specification custodian shall approve the process baseline control documentation prior to initiating manufacturing or testing of products to be furnished under this specification.

#### Baseline control document

The baseline control document shall be submitted to the specification custodian for approval, four weeks maximum prior to the start of production. The baseline control document shall be updated after each requirement change.

#### Baseline control deviations

After final approval of the baseline control documentation, there shall be no deviations without written approval for the specification custodian (see 1.6 and 1.7).  All requests for deviation from the requirements of this specification shall be submitted to the procuring activity procurement department in writing and accompanied by through technical rationale.

After final approval of the

## Induced loss measurement are used to determine the optical performance of the test sample during the exposure to the test environment.  Induced loss measurement require that the test set be operational and connected for the duration of the test.  Induced loss measurements are as accurate as the optical equipment (

## Workmanship (4.7.4)

All details of workmanship shall be in accordance with high grade optical fiber and cable manufacturing practice.  Fiber coatings and cables shall be dimensionally uniform, free of lumps, kinks, splits, scraped or abraded surfaces and inclusions.  The minimum level of visual examination to be performed, unless otherwise specified, shall be as specified in “a.” through “d”.

This requirement is not intended to restrict other pertinent workmanship examinations deemed necessary by the supplier.

a.	Outer jacket: Free of cuts, burnt areas, abrasions, holes, roughened 			areas, bulges, thin spots and discontinuities.

b.	Inner layer: Free of cut, holes, bulges, thin spots and discontinuities

c.	Strength members: Uniform, laid with no discontinuities.

d.	Optical Fiber: The optical fiber and coating shall be continuos and without defect.

## Optical

### Numerical Aperture

The numerical aperture of the fiber shall be in 0.30  0.02.  The fiber optic cable numerical aperture shall be verified in accordance with EIA/TIA RS 455 FOTP 47 Method A.  The cable shall be 2  0.2 meters and the test shall be run at 1300  30 nm.

### Information Transmission Capacity

The –3dB bandwidth of 1 Km of fiber shall be as specified in 600 MHz-Km measured at 1300  30 nm.  The cable shall be measured for information transmission capacity in accordance with EIA/TIA RS 455 FOTP 30.  Report the operator, date, test sample identification, test sample length, source wavelength and results.

### Attenuation rate (4.7.5)

The maximum attenuation rate of cable shall be equal to or less than 6 dB/Km (6 dB/3300 feet)  at room temperature when the cable is new and before and after any single or combination of tests.  The maximum allowable attenuation shall be measured at 1300  30 nm..  The cable length shall be 1 Kilometer.  The source shall be in accordance with 4.10.12.1.1.  The detector shall be in accordance with 4.10.12.3.  The test report shall include the title of test, date and name of operator, measurement data, identification and length of fiber measured, temperature, attenuation and attenuation rate.

### Induced attenuation (4.7.5.3)

Induced loss measurements are used to determine the optical performance of the test sample during the exposure to the test environment.  Therefore, the test equipment must remain connected to the unit under test for the duration of the stress exposure.  Induced loss measurements are as accurate as the optical equipment ( 0.03 dB) and are preferred when testing small lengths of cable.

The maximum allowable increase in attentuation in the specimen during exposure to an environmental condition or mechanical stress shall is as specified in Table III for the individual test requirement.  The cable attentuation shall return to the value specified in Table III when comparing the initial optical power throughput when the environmental condition or mechanical stress is relieved.  (See 4.10.12.3).

- Acceptable Induced Loss

| Requirement Paragraph | Exposure Condition | Sample Length (meters) | Allowable Change During Stress () (dB) | Allowable Change Recovery After Stress () (dB) |  |
| --- | --- | --- | --- | --- | --- |
| 3.8.1 | Numerical Aperture | 2 (Fiber) | N/A | N/A |  |
| 3.8.2 | Optical Bandwidth | 1000 (Fiber) | N/A | N/A |  |
| 3.8.3 | Attentuation | N/A (Cable) | 6 dB/Km | 6 dB/Km |  |
| 3.9.6 | Radiation | 100 (Cable) | 20 | 0.5 |  |
| 3.9.7 | Thermal Shock | 100 (Cable) | 0.3 | 0.1 |  |
| 3.9.8 | Survival Temperature | 100 (Cable) | 0.3 | 0.1 |  |
| 3.10.3 | Cyclic Flexing | 5 (Cable) | 0.3 | 0.1 |  |
| 3.10.4 | Crush | 5 (Cable) | 0.3 | 0.1 |  |
| 3.10.5 | Cable Bend | 10 (Cable) | 0.3 | 0.1 |  |

### Substitution loss (4.7.5.5)

Substitution loss measurements are used to determine the change in optical performance due to the exposure to the test environment.  Since the substitution loss measurements quantify optical performance before and after the exposure the unit under test may be removed f rom the test equipment.  Substitution loss measurement accuracy is limited to the connector remating repeatability (for ST connectors the accuracy is about 0.07 dB).  Unless specified in the test requirement, the change in substitution loss measurements taken before and after the exposure to the test shall not exceed 0.20 dB.

## Environmental

Selected materials shall comply with SSP 30233 and be of a type and quality to assure compliance with the requirements of this specification, and shall be physically and chemically compatible for their intended use and throughout their intended lifetime.

### Fungus (4.8.1)

#### Internal Requirements

- Materials shall be Grade 0 (fungus inert) as defined by MIL-STD-454, Requirement 4.

#### External Requirements

### Toxic Hazard (4.8.2)

#### Internal Requirements

Toxic contamination levels must meet the requirements given in JSC 20584, “Spacecraft Maximum Allowable Concentrations for Airborne Contaminants”, or The Toxic Hazard Index shall be less than 0.5.

#### External Requirements - None

### Odor (4.8.3)

#### Interior

The materials used individually and combined shall have an odor rating of 2.5 or less.

#### Exterior - None

### Flammability (4.8.4)

Internal Requirements

The materials used in the cable shall be non combustible or self-extinguishing and have an “A” flammability rating per MSFC-HDBK-527/JSC09604,

#### External Requirements

The materials used in the cable shall be non combustible or self-extinguishing and have an “A” flammability rating per MSFC-HDBK-527/JSC09604

### Vacuum Stability (4.8.5)

#### Internal Requirements

The polymeric materials used in the cable shall not exceed a maximum total mass loss (TML) of 1.0 percent mass and a maximum collected volatile condensable material (CVCM) content of 0.1 percent.

#### External Requirements

The polymeric materials used in the cable shall not exceed a maximum total mass loss (TML) of 1.0 percent mass and a maximum collected volatile condensable material (CVCM) content of 0.1 percent.

### Atomic Oxygen (4.8.6)

#### Internal Requirements

None because the vehicle shields the cable from the environment.

#### External Requirements

The cable shall not degrade when exposed to 5 X 1021 atoms/cm2year.  The test sample made per protected with beta cloth applied per STP 484 (Jose will confirm)

### Solar UV Radiation (4.10.7)

#### Internal Requirements

None because the vehicle shields the cable from the environment.

#### External Requirements

The cable shall not degrade when exposed TBD

### Plasma (4.8.8)

#### Internal

None because the vehicle will preclude a plasma environment

#### External

Tom Piece to provide input

### Ozone (4.8.9)

The external fiber optic cable shall be able to withstand 15 years of space exposure and 1 hour of atmospheric flight exposure to Ozone.  Two thicknesses of beta cloth applied per SSP 30XXX may be used to protect the cable from the Ozone exposure.

### Radiation (4.8.10)

#### Ionizing Radiation (4.8.10.1)

The external fiber optic cable shall not degrade nor increase more that 20 dB/Km when exposed to a total dose of 118 Krad (Si).

#### Deep Dielectric Charging (4.8.10.2)

The external cable shall be susceptible to discharge damage due to the potential differential charge of various cable components developed by ion and electron flux.

#### Solar Particle Events (4.8.10.3)

The external fiber optic cable shall be able to withstand 15 years of space to solar particles.  Two thicknesses of beta cloth applied per SSP 30XXX may be used to protect the cable from the solar particle exposure.

#### Static-ESD Environment (4.8.10.4)

The fiber optic cable shall not be susceptible to discharging damage in minimum ESD potential of (Tom Pierce will supply environment definition and requirement).

### Vacuum (4.8.11)

The fiber optic cable shall withstand 15 years in orbit without degradation.

### Temperature Survivability (4.8.12)

#### Internal Requirements

The interior fiber optic cable shall withstand thermal excursions throughout its life from -35C to 100C.

#### External Requirements

The exterior fiber optic cable shall withstand thermal excursions throughout its life from -121C to 200C.

### Thermal Shock (4.8.13)

#### Internal Requirements

The interior fiber optic cable shall withstand thermal cycles throughout its life from -35C to 100C.  Each cycle shall have a 90 minute period.

#### External Requirements

The exterior fiber optic cable shall withstand thermal excursions throughout its life from -121C to 200C.  Each cycle shall have a 90 minute period.

### Reactivity in Fluids (4.8.14)

The fiber optic cable and its identification shall be impervious to the fluids in Table 3.9.14.

- Test Fluids

| Fluid | Test Temperature | Interior Duration | Exterior Duration |
| --- | --- | --- | --- |
| Alcohol | Room Temp. | 45 minutes | 45 minutes |
| Hydraulic Fluid MIL-W-81381 | Room Temp 71C | - | 2 hours 48 hours |
| Lubricating Oil | Room Temp71C | - | 2 hours 48 hours |

### Humidity (4.815)

Both the internal and external cables shall be subjected to terrestrial humidity conditions (10% to 100% RH) for five years.  Additionally they shall experience on orbit humidity levels as follows:

#### Internal Cables

Interior fiber optic cable shall not degrade when exposed to 20% to 100% relative humidity for 15 years

#### External Cables

Exterior fiber optic cables shall not degrade when exposed to 0% relative humidity for 15 years.

### Ambient Light Susceptibility (4.8.16)

External cables shall not accept more than –57 dBm when exposed to 1500 W/m2 of white light.

### Ark Tracking Test (4.8.17)

The fiber optic cable jacket shall not contain materials that support ark tracking when the fiber optic cable is harnesses with electrical wires that are prone to ark tracking.

Mechanical

### Strippability

The cable components (the jacket, strength member and cable buffer [if applicable]) shall be hand removable using standard wire mechanical strippers.  The removal/preparation of any cable component shall not affect any other cable component or stress the optical fiber in any way.

#### Cable jacket strippability (4.7.4.4.5)

A five centimeters length of cable jacket shall be removable by hand after it is circumferentialy cut.

#### Cable buffer strippability

The cable buffer shall be cut using Clauss NO-NIK with a cut diameter of 254 microns (0.010 inches).  After the tool is removed the cut end of the buffer tube shall be removable by hand without affecting the remaining buffer tube or the fiber and its coating.  This is verified during the qualification sample preparation and during harness fabrication.

#### Fiber buffer strippability (4.9.1)

The fiber buffer shall be capable of withstanding a pull or push force of 2.5 kilograms (5.5 pounds) over a length of 2.5 mm (0.1 inches) without delaminating or debonding from the glass fiber.

### Fiber life

The optical fiber shall be a minimum 20 year fiber life.  This shall be calculated with a safety factor of 5 at a stress of 0.813 GPa (118 KPSI).

### Cable 	fiber Life

The fiber shall survive 15 years in orbit.  The worst case cable bend is 1.25 cm.  The fiber n value shall be derived by the strength tests per paragraph following in this section and the fiber life shall be calculated using the following equation:

Time to Failure (Tf) = B(a/p)-n p-2

B = 2[AY2(n-2)KIC(n-2)]-1

Where:

a = applied stress

p = fiber proof stress

n = fiber n value which shall be measured for each fiber lot

Y = geometric constant for glass = 1.24

KIC = critical stress-intensity factor for fused silica = 0.789MPa m-2

A = known from static fatigue data and cable be calculated as:

Log A = 3.289n – 10.05 (GPa-s)

(equations above are taken from Optical Fiber Telecommunications II,  Miller and Kaminow, © 1988 AT&T and Bell Research, Inc and Academic Press,  Inc.)

#### Intrinsic Strength of the Fiber

The intrinsic strength zone for fiber shall be a minimum of 450 KPSI (11.0 lbs.) with a hermetic coating and 600 KPSI (14.4 lbs.) without a hermetic coating.  One hundred percent of fiber manufactured to this specification shall be subjected to proof testing during the manufacturing process with a minimum test limit of 200 KPSI (4.9 lbs.).

#### Dynamic strength test (4.9.2)

Each lot of cable fiber will be subjected to an acceptance test procedure (ATP).  ATP will consist of a Dynamic Tensile Strength Test run on a minimum of 20 each 2 meter length samples taken from the beginning and end of the draw (40 samples total).  When tested in accordance with the procedure in paragraph 4.10.X.X, a minimum of 70% of the samples will have breaking strengths in the intrinsic zone of the Weibull Distribution formed from the 40 samples and no sample shall break at less than the minimum proof test limit.

3.10.4 Cycling flexing (4.10.20)

The cable shall withstand without damage 2000 cycles of  90 cyclic bending around a 0.5 inch mandrel in accordance with DOD-STD-1678 METHOD 2010 Procedure 2.  The induced attenuation shall not exceed 0.5 dB during the test (measured when the cable is bent and 90) and shall return to within 0.1 dB of the initial value after the test exposure (measured when the cable is straight).

### Crush resistance

Cable shall withstand a minimum 50 pound compressive loading in accordance with EIA/TIA-455-41 using the following conditions:

a.	The load shall be 22.6 Kilograms (50 lbs.) applied over a 6.25 mm (0.25 in) length (in place of the 102 mm [4 in] length specified in EIA/TIA-455-41).

b.	The load shall be applied for 60 seconds

c. 	The load shall be applied and released in no less than 60 seconds and no greater than 120 	seconds.

d.	Visually inspect for damage at 10X magnification

The induced loss shall not exceed 0.3 dB when measured in accordance with paragraph

3.8.3.2 while load is applied.  The induced loss shall return to within 0.03 dB when the load is removed.  The following exception applies:

- Paragraph 4.3 4.8 and 5.1.6 of EIA/TIA-455-41 standard - The optical power shall be measured in accordance with 4.10.12.3 of this specification.

### Cable bend

The cable shall withstand a 90 degree bend having a radius of 1.25 centimeter (0.5 inches) with less than 0.3 dB attenuation change.  The cable shall also withstand 5 wraps around a 5 cm (2 inch) mandrel in accordance with EIA/TIA 455 FOTP 37 procedure III without affecting the optical power throughput greater than 0.3 dB at room temperature and –121C.

### Cyclic Flexing (4.9.3)

The cable shall withstand 2000 cycles around a 12 mm diameter madrel  90 at temperatures varying from room temperature to 200C

### Cable Weight (4.9.4)

The weight per unit length of the typical cable shall be in accordance with appendix A but shall not exceed 2.7 Kilograms per 300 meters. The weight shall be determined in accordance with FED-STD-228 Method 8311 except the length of the cable sample shall be 2  10 % meters but measured to within 0.5 mm. The weight measurement shall be accurate to 0.20 grams.

## Physical

### Fiber and fiber buffer

The following requirements apply to the bare fiber and the buffer coating and the

finished cable.

### Fiber core  (4.7.4)

The fiber core shall be Germania doped silica or pure silica and have an active diameter of 100  2 microns, and ovality not to exceed 5%.

### Fiber cladding (4.7.4)

The fiber cladding shall be doped silica and the outside diameter of 140  2 microns, with ovality not to exceed 4% and core/cladding offset not to exceed 2%.

### Hermetic coating  (4.7.4)

#### Internal

A hermetic seal coating shall be applied to the fiber as specified in Appendix A.  The coating shall be uniformly applied and defect free when inspected per 4.10.7.

#### External

A hermetic seal coating is not required to be used on an exterior use only cable.

### Fiber coating (4.7.4)

The fiber coating shall be of the materials and construction as specified in Appendix A.  The outside diameter shall be 170  2.0 microns with a wall thickness tolerance of 1 micron.  The fiber core to fiber coating concentricity shall be 4 microns maximum.

### Cable (4.10.7)

#### Cable jacket color

##### Translucent cable jacket

When used with a transparent or translucent cable jacket, one or more of the strands of strength member will be colored violet in accordance with MIL-STD-104.

##### Opaque cable jacket

When an opaque or colored cable jacket is used, the color will be in accordance with MIL-STD-104.  Although the jacket may be somewhat translucent, the coloration of the jacket is not intended to reveal any information contained on the inner layers of cable construction per Appendix A.

When used with a transparent or translucent cable jacket, one or more of the strands of strength member will be colored violet in accordance with MIL-STD-104.

#### Cable jacket (4.7.4)

The cable jacket shall provide environment and physical protection to the cable elements.  The cable jacket shall be applied concentrically to the cable core to maintain circularity in the completed cable.  The cable shall be made of material as specified in Appendix A, with an outside diameter of 2.10  0.05mm and a wall thickness of 0.25mm, minimum.

#### Cable stability (4.10.7.4.5)

The cable jacket and buffer shall be dimensionally stable and shall not shrink more than 0.4 percent after being subjected to thermal shock cycles per section 4.10.13.

#### Identification marking (4.7.4)

The finished cable shall be identified by a printed marking.  This printed marking may be applied either outside of the jacket (if it is opaque or colored) or on an internal cable member (if viewing information is intended to be viewed through the jacket.

##### Legibility

Printed marking shall be sized and oriented on the finished cable to be legible to the unaided eye.

###### Opaque jacket

When finished cable is constructed in accordance with 3.11.2.5.1.a, printing shall be of sufficient contrast against background color to be discernable to the unaided eye at a distance of 0.5 meter.

###### Translucent jacket

When finished cable is constructed in accordance with 3.11.2.5.1.b, printing shall be of sufficient contrast against background color to be discernable to the unaided eye at a distance of 0.5 meter. Colored strands of the shall be visible through the jacket.

##### Marking durability

The marking shall be permanent upon completion of the reactivity to fluids test per 3.5.14 as defined below:

Opaque jacket

When finished cable is constructed in accordance with 3.11.2.5.1.a, printed markings shall be of such a material and installed such that the printing withstands the solvent durability qualification test of Section 4.10.14. Within a marking group of 3.11.2.5.6, no more than three consecutive characters are allowed to be illegible if both adjacent marking groups of 3.11.2.5.6 are intact.

###### Translucent jacket

When finished cable is constructed in accordance with 3.11.2.5.1.b, printing shall be of such a material and installed such that installation of over-layers does not result in damage to the printing in excess of 3.11.2.5.9. b.  Within a marking group of 3.11.2.5.6, 10% of the total characters of that group are allowed to be illegible if both adjacent marking groups of 3.11.2.5.6 are intact and paragraph a of this section is met.

Printing Groups

Printed marking groups of 3.11.2.5.6 shall be installed at .25 meter intervals.

Printed marking groups shall include the following:

a.	Full part number

b.	Manufacturer’s CAGE code

c.	Unique lot/date code

Cable buffer  (4.7.4)

The cable buffer shall be of the materials and construction specified in Appendix A, and has a maximum outside diameter of 785 microns.

#### Cable strength member (4.7.4)

The cable strength member shall be braided material in accordance with Appendix A, with an outside diameter of 1.6mm, maximum.  The braid shall cover a minimum of 90 percent of the cable buffer.

### Lengths and splices

Continuous cable lengths shall be no shorter than 100 meters (328 feet) unless otherwise specified by the purchase order.  The fiber shall not be spliced within a given spool of cable.  The splice free spool shall be verified in accordance with EIA/TIA-455-60 (length) and EIA/TIA-455-61 (splices).

### Cable weight (4.9.4)

The weight per unit length of the typical cable shall be in accordance with appendix A but shall not exceed 8.2 Kilograms per 1000 meters (5.5 lbs per 1000 feet). The weight shall be determined in accordance with FED-STD-228 Method 8311 except the length of the cable sample shall be 2  10 % meters but measured to within 0.5 mm. The weight measurement shall be accurate to 0.20 grams.

QUALITY ASSURANCE PROVISIONS

## General

- This section establishes the requirements for verifying cable performance, operability, and physical characteristics by test, analysis, demonstration or examination. The test program shall assure that the cable and its documentation meet the requirements established in 3.0, 5.0, 6.0 and appendix A.

- The quality assurance provisions shall consist of the following classifications of inspections as follows:

a.	Qualification

Responsibility for inspection

### Contractor responsibilities

- Unless otherwise specified in the contract or purchase order, the supplier is responsible for the performance of all inspection requirements and acceptance tests as specified herein and in Table IX. The specification custodian reserves the right to perform or witness any of the inspections set forth in the specification where such inspections are deemed necessary to assure supplies and services conform to requirements.

- All items must meet all requirements of 3.0 and 5.0 and appendix A. The inspections set forth in this specification shall become part of the supplier’s overall inspection system or quality program.  The absence of any inspection requirements in the specification shall not relieve the contractor of the responsibility of assuring that all products or supplies submitted to the government for acceptance, comply with all requirements of the contract.

### Test equipment and inspection facilities

Provision for test and measuring equipment and inspection facilities of sufficient accuracy, quality, and quantity to permit performance of the required inspections shall be the responsibility of the contractor and approved by the specification custodian.

Unless otherwise specified, all examinations and tests shall be performed within the following ranges of ambient temperature, humidity and pressure.

a.	Temperature: Room ambient ( 200C to 300C)

b.	Relative humidity: Room ambient (10-80 %)

c.	Barometric pressure: 24 to 31 inches of mercury

### Test equipment error

Equipment used to measure parameters shall not introduce errors greater than 10 percent of the tolerance on the parameter, or in the case of a single limit tolerance, the test equipment error shall be applied to the specified parameter in a manner to assure the parameter is being met.  Accuracy of less than one-tenth of the tolerance shall require approval of the specification custodian.  Test equipment calibration shall comply with MIL-STD-45662 or as an alternative, the test equipment calibration shall be current and certified as defined in the domestics contracts paragraph of MIL-STD-45662.

## Product assurance program

### Quality and reliability assurance plan

The manufacturer shall prepare a quality and reliability assurance plan. The plan shall be submitted for review and approval as required by the purchase contract.  Use NHB 5300.4 (3G) as a guideline for the inspection requirements in addition to the inspection required to insure part quality.

### Critical hardware notation

The manufacturer shall establish procedures by which their purchase orders (PO’s), purchase requests (PR’s), request for proposals (RFP’s), contracts, and subcontracts, and those of their subcontractors and suppliers down to the lowest tier, shall include the following information.  Additionally, each sheet of each drawing and the first sheet of each control document (i.e.; process, specification, procedure, etc.) shall also provide this information. The information shall be printed or stamped in bold face type.

FOR USE IN MANNED SPACE FLIGHT.

MATERIALS, MANUFACTURING, AND WORKMANSHIP

OF THE HIGHEST QUALITY STANDARD

ARE ESSENTIAL TO ASTRONAUT SAFETY.

### Disposition and corrective action

The manufacturer shall recommend the disposition or corrective action for all analyzed discrepancies

Implementation of the recommended disposition or corrective action shall be at the sole discretion of the specification custodian and shall not be implemented without written approval from the specification custodian.

## Inspection and Test

## Preparation of optical test samples

All test samples and test support cables shall be tested using a source in accordance with section 4.10.12.1.1. The termination method of the connectors/terminations/splices joining the test cable to the test set shall be proven to be low loss and low variation in their environment throughout the duration of the tests. Physical contact fiber interfaces are acceptable and recommended.

## Materials inspection

Materials inspection shall consist of certification, supported by verifying data, that materials used in fabricating the delivered cable are in accordance with the requirements of 3.3 and appendix A.

## Product acceptance test

### Acceptance of product for delivery

Product Acceptance Tests shall consist of those inspections and tests listed in Table X and shall be performed in accordance with the approved acceptance test plan in accordance with section 3.2.2.1.2.

Unit of manufacture

The unit of manufacture shall be defined by the purchase order.

### Sample unit

Sample length shall be one (1) meter for dimensional and mechanical inspection, as required for numerical aperture (NA), attenuation and bandwidth tests.

### Verification of physical requirements (3.7.2, 3.7.3, 3.7.4, 3.7.5, 3.7.6, 3.7.6.2 3.7.6.3, 3.7.6.5, and 3.7.6.6)

#### General

#### Test equipment

Optical Magnifier, 10X power (maximum) for examination of the product for damage and defects such as cracks, splits in outer jacket, etc.

Optical microscope, 400X minimum, with video processing equipment for verification of fiber core, cladding and coating dimensions.

Dimensional and other measurement equipment (vernier calipers, rules, comparitors, scales, etc.) shall be of sufficient accuracy to verify dimensional and weight parameters specified in 3.0.

Thermal shock chamber capable of operating over a temperature range of –135C to +220C with a worst case temperature error/variation of 2.5C and a maximum time of 2 minutes to transition the test sample between the high and low temperature set points.

#### Test specimen

The test specimen shall be a sample from the reel of the product lot being submitted for inspection.  The length of the specimen for acceptance and receiving inspection shall be sufficient to perform all visual and mechanical examinations.

Dimensional measurements shall be made on at least two (2) samples [minimum one (1) meter in length], one from the outside end of the product and one from the inside end of the reel of product.

#### Test procedures

##### Visual examination

The product shall be visually examined in accordance with the requirements in EIA/TIA RS 455 FOTP 13 to assure that it meets the specified requirements as to:

a.		Materials used (Material certifications to establish conformance to material type requirements) and verification of data submitted relative to lot number, etc.

b.		General design and construction

c.		Workmanship

d.		Finish

e.		Identification of product (marking)

f.		Damage

##### Mechanical examination

The cable jacket, strength member and buffer shall be measured in accordance with EIA/TIA RS 455 FOTP 13 to determine conformance to the dimensions specified in paragraphs 3.11.1 and 3.11.2.  The cable specimens shall be prepared for dimensional inspection by cross-sectioning the cable and viewing it at 50 X. using a video processor with a microscope in accordance with 4.10.7.2.  The OD of each cable component may be measured by stripping off outer jacket, trimming strength member and stripping off buffer as required by the measurement method used.

Data sheets shall be furnished, to the procuring quality organization, documenting the actual measurements of the dimensions specified in Figure A-1 of appendix A for each sample used for dimensional inspection for qualification and quality conformance (acceptance) inspections.

##### Fiber and coating examination (3.11.1)

The fiber core, cladding and coating dimensions shall be verified according to EIA/TIA RS 455 FOTP 45 with the following exceptions:

Para. 2.2	The microscope shall be equipped with video processing equipment allowing measurements to  2 microns at 400 X magnification.

Para. 4.3	The fiber coating shall not be removed from the fiber.

Para. 4.4	The fiber and coating shall be terminated in a connector and polished to a flat surface which is within  2 degrees of perpendicular to the fiber axis.

Para. 4.6	The fiber rotation shall be aligned so that the largest measured diameter is horizontal.  Use the video measurement system to measure the width and height of fiber in this orientation.  Rotate the fiber to align smallest diameter horizontally and measure the width and height using the video measurement system.

Para. 4.7	The video system will impose a measurement image on the monitor and on the photographic image.

The following dimensions are to be verified:

a.		Core diameter

b.		Core ovality

c.		Cladding diameter

d.		Cladding ovality

e.		Core/Cladding offset

f.		Fiber coating diameter

g.		Fiber coating ovality

##### Marking Durability  (3.11.2.5)

This test is to be performed only if markings are applied to the outer jacket of the cable

a.	Immerse a 2 meter length of finished cable into a room temperature solution of isopropyl alcohol, ACS reagent grade.

b.	Remove after 3 minutes of immersion.

c.	Wipe full length of sample with clean cotton cloth

d.	Repeat item c, for a total of three wipes

e.	Inspect for printed marking integrity.  Reject the cable if any symbolization has become

illegible when viewed without magnification from a distance of approximately 0.5 meters.

##### Cable jacket stability verification. (3.6.1.1)

Obtain a test sample that is approximately 1 meters in length.  Measure the actual length of the test sample to the nearest 0.025 millimeters.  Subject the sample to five thermal shock cycles in accordance with the requirements in paragraph 3.9.7, then remove the sample from the chamber.  Flex the sample holding the coiled cable with two hands on opposing sides of the cable and twisting one hand with respect to the other (the rotational axis is the line formed by both hands through the center of the coil).  Then subject the sample to an additional five thermal shock samples per 3.9.7.  Remeasure the length of the cable jacket to the nearest 0.025 millimeters Remove the cable jacket and strength member as required, then measure the buffer length. Calculate the percent of shrinkage.

Hermetic coating verification (3.11.1.3)

TBD – TO BE SUPPLIED BY LUCENT/SPECTRAN

#### Fiber Buffer Strippability (3.10.1.2)

Five fiber samples from random 30 centimeter lengths of cable shall be removed and one end cleaned using a low non-volatile residue wipe and alcohol.  The fibers shall be installed into a connectors/ termini/ferrules measuring 172.7 microns to 175 microns.  A drop of Epotec 353ND epoxy shall be placed on the tip of each ferrule.  The fiber shall be moved a total of 2.5 millimeters in and out of the connector.  The excess epoxy shall be removed and the remaining epoxy shall be cured at 100C for 30 minutes.  The excess fiber shall be cleaved and the remaining materials shall be ground off the tip using 30 micron lapping film.  The termini shall be secured in a tensile test set and the fiber pulled until failure.

Dynamic Tensile Strength Test (3.10.3.2)

Samples without the jacket, strength members, or buffers, shall be conditioned at room 72 5F with a relative humidity of 65  5 % for 24 hours.  After completion of the conditioning, the samples will be pulled to destruction with the strain rates specified in EIA/TIA FOTP 28C.

Attenuation measurements (3.4.3)

The following are the procedures required to make optical loss measurements for the International Space Station Program. The induced and substitution loss measurement procedures as well as the test equipment and SRD fabrication procedures are outlined below.  Although the EIA/TIA Specifications are referenced, the following procedures take precedence in the case of conflict.

#### Optical test set

Below are the functional requirements of the optical test equipment.  The RIFOCS CP-1107 and CP-1148 as well as other RIFOCS equipment built for Space Station meet these requirements.

##### Light source

The optical source shall be an optically and modally stabilized having a center wave length between 1330  30 nanometers.  The far field ½ sine  output of the source shall conform to TableVI.  The source bandwidth shall be within 60 nanometers.  The optical output power shall not be modulated.

- Far Field Output

| NA | Power (Percent) |
| --- | --- |
| 0.093 to 0.118 | 80 |
| 0.139 to 0.167 | 60 |
| 0.182 to 0.208 | 40 |
| 0.224 to 0.250 | 20 |
| 0.267 to 0.290 | 5 |

##### Source monitoring equipment (SME)

The source output power shall be capable of monitoring and recorded without disconnecting the test specimen.   These measurements are used to allow corrections to compensate for source output variations.

Equipment that does not have continuous source monitoring capability, shall include a Certificate of Compliance that the optical source remains optically stable to  0.03 dB and modally stable within 1 percent of the measured optical power at each specified ½ sine  for the duration of the testing sequence.  The validation of the source shall be obtained before and after the testing and shall be submitted with the test data.

##### Detection equipment

The detector and associated electronics shall be capable of measuring all energy exiting from the fiber and have a linear active area at least 10 times larger than the fiber core.  They shall be linear at the wavelengths used, and over the expected range of power.  The resolution of the detection equipment shall be as follows:

- Equipment Resolution Requirements

| Interconnection Device Loss | Resolution |
| --- | --- |
| >.5 dB | .1 dB |
| <.5 dB | .05 dB |

The detector shall be linear within 3 percent over the duration of the test and over a 23  50C temperature range.

##### Reference cable set and fabrication

The reference cable 6  0.6 meters of NFOC-2FFF-1GRP-1 fiber optic cable and terminated with ST connectors polished to attain a physical contact interface.  The substitution loss of the reference cable shall not exceed 0.40 dB and the fiber core shall be free from surface defects when viewed at 100 power magnification.

SRD Cable sets are high quality optical jumpers used for optical testing.  Their performance is continually monitored and logged to insure consistent performance.  Below are the detailed instructions for building the cable sets.

Step 1 - Connect a source (s) cable to a mode conditioned optical source and a detector (d) cable to an optical power meter.  Connect the cables together and measure and record the optical power throughput as Psd.

S

D

s cable	d cable

Measure:	Psd (from previously fabricated SRD cable set)

Step 2 - Terminate a 6 meter length of fiber optic cable with ST-PC connectors.  This cable will be called a PC cable.  The cable can be of a suitable construction to facilitate the testing application however, the fiber must be the same fiber as used in the Units Under Test (UUT) are to be used. Connect the PC cable to the s and d cables and measure and record the optical throughput as PPC. LPC shall not exceed 0.4 dB.  The cables shall not be disconnected from the source until the last step is completed.  Calculate the loss of the PC cable as follows:

s	                                         PC                                        d

Measure:	PPC

Calculate:	LPC = Psd - PPC </= 0.40 dB

Step 3 - Cut the cable approximately two meters from the source end of the fiber the cable and install an ST-PC connector one each side of the cut cable.  Measure and record the OPT as PI1.  Calculate the insertion loss (LI1) as follows:

s cable                  S                                                          d cable

Measure:	PI1

Calculate:	LI1 = PPC-PI1 </= 0.30 dB

Step 4 - Cut the cable approximately two meters from the detector end of the fiber the cable and install an ST-PC connector one each side of the cut cable.  Measure and record the OPT as PI2. Calculate the insertion loss (LI2) as follows:

s cable              	S                  R                    D                    d cable

Measure:	PI2

Calculate:	LI2 = PI1 - PI2 </= 0.30 dB

Step 5 – Calculate LR and identify the cable as show below:

s  cable                    S                  R                    D                   d cable

1      2     3    1    6    4      5   1

NOTES:

Label 1 - ID, month, year and serial number for the SRD cable set

Label 2 - SR

Label 3 - RS

Label 4 - RD

Label 5 - DR

Label 6 - Ref Cable Loss “Lref = X.XX dB” (X.XX = PSD - PSRD measured on the new SRD set)

Step 6 - Remove the new R cable and connect the new S and D cables together.  Measure and record the OPT (PSD).  Calculate the R cable Loss as LR = PSD - PSRD where PSRD = PI2.

s  cable	 	 S                               D                       d cable

##### MTC cable

An MTC cable shall 6.0  0.6 meters of fiber optic cable which uses the fiber specified herein.  It shall be terminated with ST connectors and polished to achieve a physical contact interface.  The substitution loss shall not exceed 0.4 dB and the cable shall be free from surface defects when viewed at 100 power magnification.  This cable shall be protected and stored in a safe and environmentally stable area when not used.

When using a RIFOCS test set, the MTC cable can be RIFOCS’ P/N 333-110-03.

#### Optical test set calibration

The test set calibration consists of two processes; one for the source and detector and one for the S and D cables.  The measurement procedure outlined below is very rigorous and is required to attain 0.10 dB measurement accuracy.

##### Meter and source calibration using the MTC cable

Clean each termini optical surface on the appropriate MTC cable using a clean lint free wipe wetted with N-Propyl alcohol (as required) followed by a clean lint free wipe (dry) (as required).  Using clean, dry compressed air, blow the connector receptacle on the source and detector.  If the mode conditioner is physically separatable from the source, connect the MTC cable to the configuration that is calibrated, usually the source with out the mode conditioner.  Disconnect the source side of the MTC cable from the source and reclean the source receptacle and the mating optical terminus. Repeat the cleaning remating steps until three consecutive OPT measurements agree to within 0.03 dB.  Repeat this process on the detector side of the MTC Cable interfaces.  If this is the first use of the test set, MTC cable or after each calibration, record the final OPT in the equipment log.  Verify that the recorded OPT is within 0.1 dB of the certified source output minus the MTC cable loss (the loss is labeled on the cable).  If this is not the first use or not after calibration, record the final OPT into the equipment log as PMTC(date).  The OPT shall not exceed 0.1 dB change from the other logged readings.

##### SRD cable set calibration

For optical test sets that have a mode conditioner that can be removed from the test set:  Clean the connectors on a small jumper and the optical source port and the mode  conditioner ports.  Clean and connect one end of the jumper to the source and the other to the mode conditioner input.  Clean and connect the MTC cable to the mode conditioner output and the other end of the optical power meter.  Note the OPT.  Disconnect and clean the small jumper to source interface until three readings agree to within 0.03 dB.  Repeat for the small jumper to the mode conditioner interconnect.  Remove the MTC cable.

Clean each connector optical surface on the appropriate S cable and D cable using a clean lint free wipe wetted with N-Propyl alcohol followed by a clean lint free wipe (dry).  Using clean, dry compressed air, blow the connector receptacle on the source (or mode conditioner output port if it is a separate unit which hereafter will be referred to the “source”) and detector.  Install the S and D into the source and the detector and mate the S and D cables.  Disconnect the S from the source and reclean the source receptacle and the mating optical terminus. Repeat the cleaning remating steps until three consecutive OPT measurements agree to within 0.03 dB.  Repeat this process on the D to detector receptacle and then on the S to D interfaces.  Record the final OPT as PSD and the meter monitor power as PMSD (if SEM is used).

S Cable	D Cable						D Cable

Disconnect the S and D cables and install the R cable so that the source side of the R cable connects to the S cable and the D side of the R cable connects to the D cable.  Note the OPT.  Unmate clean and remate the S to R cable junction until three OPTs agree to within 0.03 dB.  Repeat for the R to D cable junction.  Record the final OPT as PSRD.  If  PSRD deviates greater than 0.2 dB for LR list on the Reference cable label remove and discard this SRD cable set.

S Cable	                                 R Cable		D Cable						D Cable

#### Induced loss (3.4.4)

The change in the attenuation of the fiber induced by exposure to environmental or physical test conditions shall be measured in accordance with EIA/TIA RS 455 FOTP 20 with the following parameters:

a.	The source and detector shall comply with 4.10.12.1.

b.	An optical power divider (per paragraph 2.2 of FOTP 20) shall not be used.

c.	The test sample shall be terminated with ST connectors on each end.

d.	A reference sample shall not be used and the reference power shall be the initial optical power through the test specimen.

e.	The optical test setup shall not be disconnected between the time the baseline value is established and measurement of power throughput after completion of the primary test.

##### Optical measurements

The following Optical Power Throughput measurements shall be recorded:

PMSDD	=  Meter Optical Power Throughput (OPT), SD - Cable, Induced Loss

PMDB	=  Meter OPT, Before, Induced Loss

PMDX	=  Meter OPT, After or Subsequent, Induced Loss

PSSD	=  OPT, SD - Cable, Induced Loss

PDB	=  OPT, Before, Induced Loss

PDX	=  OPT, After or Subsequent, Induced Loss

##### Test set-up

##### The test equipment shall be in accordance with 4.10.12.1 and calibrated in accordance with 4.10.12.2.  The test set shall remain connected to the test sample during the environmental exposure.

##### Test procedure

The test setup shall not be separated from source once testing has begun.  Induced attenuation measurements shall be made immediately before, during and/or after tests as specified in the applicable mechanical or environmental test method. Replace the R Cable with the test sample.

S

D

S Cable	 Unit Under Test	 D Cable

Clean the test sample to S Cable to UUT interconnection and record the OPT.  Reclean the interconnection until three consecutive OPT measurements agree to within 0.03 dB.  Repeat for the UUT to D cable interconnection.  Record the final OPT measurement as PDB and the meter monitor power as PMDX.  Run the environmental test.  Measure and record the test sample OPT at the specified intervals as PDX where x indicates the interval .  After the environmental test is complete, measure the final OPT and record it as PDF  and the meter monitor power as PMDF.

#### Induced loss calculations

Induced loss shall be calculated as follows when the power is measured in watts

LD (dB) 	= 10 logPDBPDX⋅PMDXPMDB

or when the power is measured in dB

LD (dB)  	= (PDB-PDX) - (PMDB - PMDX)

#### Substitution loss (3.4.5)

(Reference EIA/TIA 455 FOTP-171, however these requirements supersede FOTP-171 if conflicting.)

##### Optical measurements

The following Optical Power Throughput measurements shall be recorded:

PMSD	=  Meter Optical Power Throughput (OPT), SD - Cable, Substitution Loss

PMSI	=  Meter OPT, Initial, Substitution Loss

PMSF	=  Meter OPT, Final, Substitution Loss

PSD	=  OPT, SD - Cable, Substitution Loss

PSI	=  OPT, Initial, Substitution Loss

PSF	=  OPT, Final, Substitution Loss

##### Test equipment set up

The test equipment shall be per 4.10.12.1 and calibrated per paragraph 4.10.12.2.  The test set may be disconnected from the test sample during the environmental exposure.

##### Test procedure

Replace the R Cable with the test sample.

S Cable		 Unit Under Test	D Cable

D

S

Clean the test sample to S Cable interconnection and record the OPT.  Reclean the interconnection until three consecutive OPT measurement agree to within 0.03 dB.  Repeat this process on the test sample to D Cable interconnection to test sample.  Record the final OPT as Ps  and the meter monitor power as PMS.

##### Substitution loss calculations

Induced loss shall be calculated as follows when the power is measured in watts

LS (dB)	  = 10 logPSDPS⋅PMSPMSD

or when the power is measured in dB

Substitution Loss (dB) = (PS-PSD) - (PMS - PMSD)

## Environmental requirement verification procedures

### Fungus (3.5.1)

Test per MIL-STD-810, Method 508 and MIL-STD-545 Requirement 4.  The test environment shall be per Table IX.  The material shall not support fungus (Grade 0)

- Fungus Environmental Test Conditions

| Test Condition | Test Limit |
| --- | --- |
| Humidity | 90% to 100% |
| Temperature | 23C to 30C |
| Duration | 28 Days |
| Pass/Fail | Grade 0 |

### Toxic Hazard (3.5.2)

#### By Test

The test shall be performed in accordance with NASA-STD-6001 Test 7 with the test conditions specified in Table 4.10.  Ten samples that are at least 10 meters apart in a cable run length shall be tested.

#### By Analysis

The toxic hazard ratings for materials that have been previously tested are documented in MSFC-HDBK-527/JSC09604 or the Material And Process Technical Information System (MAPTIS) electronic data base located at George C. Marshall Space Flight Center, Huntsville, Al.

### Odor (3.5.3)

#### By Test

The test shall be performed in accordance with NASA-STD-6001 Test 7 with the test conditions specified in Table 4.10.

#### By Analysis

The odor ratings for materials that have been previously tested are documented in MSFC-HDBK-527/JSC09604 or the Material And Process Technical Information System (MAPTIS) electronic data base located at George C. Marshall Space Flight Center, Huntsville, Al.

### Flammability (3.5.4)

#### By Test

The flammability test shall be conducted per NASA-STD-6001 Test 1 using the test details listed in Table X.

- Flammability Test Requirement details

| ### Test Requirement | Test Detail |
| --- | --- |
| Environment | 30% O2 and 70% N2 at 68.9 KPa (10psia) |
| Sample Orientation | 60 per Figure 7 of NASA-STD-6001 |
| Sample Length | Fourteen samples 40cm (16 inches) long |
| Ignition/Heat Duration | 1100C and a 6.4 cm high flame, 10 minute maximum burn time |
| Self Extinguishing | Within 30 seconds after the ignition source is removed. |
| Burn Events | No sparking No sputtering No flaming dripping No flame transfer by burning debris |

#### By Analysis

None.  Although the materials that are used on the fiber optic cable are also used as insulators on electrical wire, the heat dissipation of the fiber optic cable sufficiently differs from the wire to warrant testing rather than verification by similarity to a wire product.

### Vacuum Stability – (3.5.5)

#### By Test

The finished cable shall be conditioned at 125oC at a pressure 10-6 torr for 48 hours during the manufacturing or preshipment processing.   The test shall be per ASTM E595 using the test parameters listed in Table XI.

- Outgassing Products/Vacuum Stability Test Conditions

| Condition | Limits |
| --- | --- |
| Pressure | 1.3 X 10-4 Pa max. |
| Temperature | 124C to 126C |
| Condensable Plate Temperature | 24C to 26C |
| Vacuum exposure time | 24 hours |

#### By Analysis

The data for total maximum loss (TML) and maximum volatile condensable material (VCM) content of materials that have been previously tested is documented in MSFC-HDBK-527/JSC09604 or the Materials And Process Technical Information System (MAPTIS) electronic data  base located at George C. Marshall Space Flight Center, Huntsville, Al.

### Atomic oxygen (3.5.6)

TBD

### Solar UV radiation (3.5.7)

TBD

### Plasma (3.5.8)

TBD

### Ozone (3.5.9)

A one meter external fiber optic cable sample shall test in accordance with FOTP 189 with the testing conditions in Table XII.

- Ozone Test Conditions

| Condition | Limits |
| --- | --- |
| Mandrel Diameter | 1.25 cm |
| Temperature | 40C |
| Humidity | 50% minimum |
| Exposure time | 3 days |
| Exposure Concentration | 1000 parts/million |

Jose will confirm dose rate and duration

### Radiation (3.9.10)

#### Ionizing Radiation (3.5.10.1)

Melanie Ott will provide verification procedure

#### Deep Dielectric Charging (3.5.10.2)

Henning Leidecker will provide verification procedure.

#### Solar Particle Events (3.5.10.3)

#### Static-EMI Environment (3.5.10.4)

Tom Pierce will supply the verification.

### Vacuum (3.5.11)

Test per 4.10.12 – Temperature Survivability.

### Temperature Survivability (3.5.12)

100 meters of fiber optic cable shall be subjected to MIL-STD-202F Method 108A Test Condition C using the test conditions in Table XIII.

- Temperature Survivability Test Conditions

|  | Internal | External |
| --- | --- | --- |
| Pressure | 1 X 10-6 max | 1 X 10-6 max |
| Low Temperature | -46C max | -135C max |
| High Temperature | 115C min | 215C min |
| Duration | 500 Hours | 500 Hours |

### Thermal Shock (3.5.13)

100 meters of fiber optic cable shall be exposed to thermal shock per MIL-STD-202F Method 107E using the test conditions in Table 4.10.13.  The temperature transition duration shall be less than 2 minutes.

- Thermal Shock Test Conditions

|  | Internal | External |
| --- | --- | --- |
| Low Temperature | -46C max | -135C max |
| Low Temperature Dwell Time | See Table 107-7 in MIL-STD-202F | See Table 107-7 in MIL-STD-202F |
| High Temperature | 115C min | 215C min |
| High Temperature Duration | See Table 107-7 in MIL-STD-202F | See Table 107-7 in MIL-STD-202F |

### Ractivity in Fluids (3.5.14)

The samples shall be tested in accordance with NASA-STD-6001 Test 15 except the samples shall be 2 meters long and weighed to the nearest 0.01 gram.  A virgin sample shall be used for each fluid.  None of the cable components shall show signs of degradation when inspected per 4.10.7 for damage and dimensions.

### Humidity (3.5.15)

Fiber optic cables samples shall be subjected to humidity per MIL-STD-202 Method 103B Test Condition C.  The specimen shall be weighed before and within 15 minutes from the completion of the test to the nearest 0.01 gram.  The cables shall not increase in mass more than 0.2%.  The samples shall be examined for degradation and the dimensions shall be verified per section 4.10.7 after the sample is allowed to dry at 21C to 25C and 20% to 60% RH.  The dimensions shall meet the requirements in section 3.11 and the materials shall not visibly degrade.

### Ambient Light Susceptibility (3.5.16)

A 4 meter length of fiber optic cable shall be tested per EIA/TIA RS 455 FOTP 22B.  One end of the sample shall be terminated with an ST connector and the other end shall be folded over on itself such that the fiber breaks 15 cm from that end.  The optical power meter shall be capable of reading to –65 dBm.  An integrating sphere shall be used.

### Arc Tracking Test (3.5.17)

A fiber optic cable shall replace the #7 wire in the specified harness and shall be tested per NASA-STD-6001 Test 18.  The test shall be conducted until ark tracking is observed.  The harness shall be disassembled and the fiber optic cable shall inspected for any contributions to ark tracking either by finished cable materials or combusted cable materials.

## Mechanical requirement verification testing

### Fiber Buffer Strippability (3.6.1.3)

Five fiber samples  from random 30 centimeter lengths of cable shall be removed and one end cleaned using a low non-volatile residue wipe and alcohol.  The fibers shall be installed into a connectors/ termini/ferrules measuring 172.7 microns to 175 microns.  A drop of Epotec 353ND epoxy shall be placed on the tip of each ferrule.  The fiber shall be moved a total of 2.5 millimeters in and out of the connector.  The excess epoxy shall be removed and the remaining epoxy shall be cured at 100C for 30 minutes.  The excess fiber shall be cleaved and the remaining materials shall be ground off the tip using 30 micron lapping film.  The termini shall be secured in a tensile test set and the fiber pulled until failure.

### Dynamic Tensile Strength Test  (3.6.3.2)

### Samples without the jacket, strength members, or buffers, shall be conditioned at room 72 +/- 5F with a relative humidity of 65 +/- 5% for 24 hours.  After completion of the conditioning, the samples will be pulled to destruction with the strain rates specified in EIA/TIA FOTP 28(C).

### Cycling flexing (3.6.6)

### Test in accordance with DOD-STD-1678 METHOD 2010, procedure 2 with the following specifications:

a.	The test mandrel shall be 12 mm (0.5 inches) in diameter.

- The test mass shall be 2.25 Kilograms (5 lbs.)

- Sample length shall be five meters.

The following exceptions shall take precedence over DOD-STD-1678 procedure 2:

Step 2	The specimen shall be mounted in the test fixture and the optical power measured in accordance with 4.10.12.3.

Step 4	The test shall be conducted starting at room temperature, then ramp to a temperature of 120C when 900 to 1100 cycles are completed, and then ramp down to room temperature when 1900 to 2000 cycles are completed.

Step 5	The optical power shall be measured at the completion of the 2000 cycles while the specimen is mounted in the fixture in accordance with 4.10.12.3.

Para. 5.4	Optical power shall be measured in accordance with 4.10.12.3.  The number of transmitting fibers will not be counted.

Para. 5.6	The optical loss shall be calculated in accordance with 4.10.12.3.4.

Para. 5.8	Delete

Para. 5.9	Delete.

Para. 5.10	Visual inspection of the outer jacket shall be made under ten-power 				magnification.

Cable weight verification (3.6.7)

Obtain a test sample that is 1 meter  1mm.  Use a scale to determine the weight of the sample in grams.  Measure and report the weight to the nearest 0.1 grams.  That number will be the same as kilograms per kilometer.

# 5.0 	   PREPARATION FFOR DDELIVERY

## Cleanliness

Extreme care shall be exercised during all packaging, packing, and marking procedures such that the cable cleanliness integrity and the packaging requirements of MSFC-SPEC-164 are not compromised.

## Packaging requirements

Unless otherwise specified in the contract, the requirements for packaging of fiber optic cable shall be in accordance with MIL-C-12000.

## Reels and spools

The cable on each reel or spool shall have both ends readily available for testing without unwinding.  A minimum of five meters of cable shall be exposed on the inside end of the cable to facilitate attenuation measurements and removal of the required mechanical and visual samples. The inner end shall be available at the outer surface of the reeled cable.  Cable shall be spooled in such a way to minimize induced fiber losses and special care shall be exercised to prevent excess stress on the cable where it exits the reel on the inside end of spool.  Both ends of the cable shall be secured to a flange.  The reel or spool shall have an inner diameter greater than six (6) times the minimum bend radius of the cable.

## Product and package marking

### Marking of reels and spools

In addition to the marking specified in MIL-STD-129, each reel or spool shall be marked with the length of individual continuous lengths cable wound thereon.  A warning label shall be applied to each reel to advise personnel to exercise caution in the handling of optical fibers.  This label shall alert personnel to avoid skin puncture and contact with the eyes.  It shall warn against staring at the output end of the fiber optic cables without eye protection (infrared viewer or equivalent) during testing when these cables are powered by high radiance light emitting diodes or lasers.

The identification marking label shall contain the following information:

SSQ Part number_______________

Length _____ meters

Manufacture lot number ____________________

Date of manufacture ____________________

Manufacturer’s name _____________

### Space Station critical hardware

Each reel and spool shall be identified with a removable tag.  The tag shall be attached and removed in a manner such that the reel or spool will not be damaged or contaminated or otherwise rendered unsuitable for use.  The tags shall be fluorescent international orange with black letters on both sides and shall read as follows.

#### CRITICAL SPACE STATION HARDWARE

#### DO NOT REMOVE UNTIL INSTALLATION

The tag size shall be commensurate with the unit size and the lettering size shall be legible to a person with normal sight at a distance of 3 feet.

### Packaging labels

Each unit, intermediate, and final package/container shall be affixed with a self adhering label.  The labels shall be fluorescent international orange with black letters and shall read as follows.

#### CRITICAL SPACE STATION HARDWARE

FOR USE IN MANNED SPACE FLIGHT. MATERIALS, MANUFACTURING, AND WORKMANSHIP OF THE HIGHEST QUALITY STANDARDS ARE ESSENTIAL TO ASTRONAUT SAFETY.

The label size shall be commensurate with the package size and the lettering size shall be legible to a person with normal eyesight at a distance 3 feet and shall be logically applied to both sides of bags, top and bottom or top and two sides, depending on box geometry, of small boxes, and the top and at least 2 sides of the final container.

## Unit protection and wrapping

Unless otherwise specified, each deliverable unit shall, as a minimum, be wrapped in an electrostatic free transportable polyethylene bag conforming to MIL-B-81705, Type II.  Bags shall be heat sealed or sealed with a reusable sealing device manufactured from material conforming to MS90376, material B.  The sealing method shall eliminate spillage and exclude particulate contaminates.  Staples, tape or other static generating materials, shall not be used.  The same information required by 5.4.1 for marking of reels and spools plus the date of packaging shall be permanently and legibly marked (1) on a low sulfur content card in and readable through the bag, (2) on the bag, or (3) a combination of marking on a card and the bag.

## Unit packaging

Each bagged reel or spool unit shall be packaged in individual containers (paper product boxes are acceptable) to insure damage free delivery and storage.  The information required by 5.4.1 and the date of packaging shall be permanently and legibly marked on the container.

## Packing containers

Each packaged unit (Reference paragraph 5.2) shall be packed in containers such that damage free delivery is assured.  Containers shall comply with the Consolidated Freight Classification rules or other carrier regulations applicable to the mode of transportation.  Each packed container shall be prominently marked with the same information required by 5.4.1 plus the quantity of reels or spools in the package and the date of packaging.

NOTES

## Part numbers

The specification custodian part number consists of the part numbers shown in 6.2 and

appendix A.

## Part Number Identification Table

NFOC               -2FFF               -1GRP               -1

Number of Fibers

Fiber Code

1GRP = 100/140, Graded Index,

Radiant Resistant, Precision Fiber

Buffer

Cable Code

2FFF = 2.0 mm OD, FEP Teflon

Jacket, Fiberglass with Teflon

Coating, FEP Secondary Buffer

Qualified For Flight

Cable Type – NFOC – NASA

Fiber Optic Cable

## Approved source(s) of supply

Only the item described by this drawing when procured from the supplier or an approved distributor is approved for use.  A substitute item shall not be used without the specification custodian’s approval.  Identification of approved manufacturer is not to be construed as a guarantee of present or continued availability as a source of supply.

| Part Number | Approved Source of Supply | CAGE CODE |
| --- | --- | --- |
| NFOC-2FFF-1GRP-1 | Brand-Rex Company An Affiliate of Cablec Corporation 1600 West Main Street Willimantic, CA 06226-1128 | 71124 |
| NFOC-2FFF-1GRP-1 | Sea Wire and Cable P.O. Box 647 110 Celtic Circle Madison Alabama 35758 | 65959 |

# REQUIREMENTS VERIFICATION MATRIX

- Requirements Verification Matrix

| VERIFICATION METHOD: |  |  | VERIFICATION PHASES: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TBS = TO BE SUBMITTED BY SUPPLIER |  |  |  |  |  |  |
| A = ANALYSIS |  |  | D = DEVELOPMENT |  |  |  |
| S = SIMILARITY |  |  | Q = QUALIFICATION |  |  |  |
| I = INSPECTION |  |  | A = ACCEPTANCE |  |  |  |
| D = DEMONSTRATION |  |  |  |  |  |  |
| T = OPERATIONAL TEST |  |  |  |  |  |  |
| N/A = NOT APPLICABLE |  |  |  |  |  |  |
| TBS = TO BE SUPPLIED BY CABLE SUPPLIER |  |  |  |  |  |  |
| SECTION 3 | PARAGRAPH |  | D | Q | A | SECTION 4 |
| PARAGRAPH | TITLE |  |  |  |  | PARAGRAPH |
| 3.3.3 | FUNGUS - DELETED | N/A |  |  |  |  |
| 3.3.3 | OFFGASSING/TOXICITY - DELETED | N/A |  |  |  |  |
| 3.3.3.1 | OUTGASSING/TOXICITY - DELETED | N/A |  |  |  |  |
| 3.3.3.2 | ODOR - DELETED | N/A |  |  |  |  |
| 3.3.3.3 | FLAMMABILITY - DELETED | N/A |  |  |  |  |
| 3.3.3.4 | OUTGASSING (VACUUM STABILITY) - DELETED | N/A |  |  |  |  |
| 3.3.4 | ATOMIC OXYGEN - DELETED | N/A |  |  |  |  |
| 3.4 | DESIGN AND CONSTRUCTION - DELETED | N/A |  |  |  |  |
| 3.4.1 | OPTICAL FIBER – DELETED | N/A |  |  |  |  |
| 3.4.1.1 | OPTICAL FIBER BUFFER –DELETED | N/A |  |  |  |  |
| 3.4.2 | CABLE – DELETED | N/A |  |  |  |  |
| 3.4.2.1 | STRENGTH MEMBER – DELETED | N/A |  |  |  |  |
| 3.4.2.2 | CABLE JACKET – DELETED | N/A |  |  |  |  |
| 3.4.2.3 | COLOR – DELETED | N/A |  |  |  |  |
| 3.4.2.4 | LENGTHS AND SLICES – DELETED | N/A |  |  |  |  |
| 3.4.2.5 | CABLE WEIGHT – DELETED | N/A |  |  |  |  |
| 3.5 | PHYSICAL – DELETED | N/A |  |  |  |  |
| 3.5.1 | FIBER – DELETED | N/A |  |  |  |  |
| 3.5.1.1 | NUMBERICAL APERTURE | N/A |  |  |  |  |
| 3.5.1.2 | OPTICAL FIBER BANDWIDTH – DELETED | N/A |  |  |  |  |
| 3.5.2 | CABLE | N/A |  |  |  |  |
| 3.5.2.1 | ATTENUATION RATE – DELETED | N/A |  |  |  |  |
| 3.5.2.2 | ATTENUATION MEASUREMENTS – DELETED | N/A |  |  |  |  |
| 3.5.2.2.1 | INDUCED ATTENUATION – DELETED | N/A |  |  |  |  |
| 3.5.2.2.2 | SUBSTITUTION LOSS – DELETED | N/A |  |  |  |  |

Table IX – Requirements Verification Matrix (continued)

| 3.5.3 | RADIATION RESISTANCE – DELETED | N/A |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.5.4 | THERMAL SHOCK – DELETED | N/A |  |  |  |  |
| 3.5.5 | SURVIVAL TEMPERATURE – DELETED | N/A |  |  |  |  |
| 3.5.6 | CRUSH | N/A |  |  |  |  |
| 3.5.7 | CABLE BEND | N/A |  |  |  |  |
| 3.5.8 | JACKET SHRINKAGE | N/A |  |  |  |  |
| 3.5.9 | VACUUM | N/A |  |  |  |  |
| 3.6 | IDENTIFICATION MARKING | I | X | X | X | 4.10.7 |
| 3.6.1 | DURABILITY OF IDENTIFICATION – DELETED | N/A |  |  |  |  |
| 3.7 | WORKMANSHIP | I | X | X | X | 4.10.7 |
| 3.8 | OPTICAL | N/A |  |  |  |  |
| 3.8.1 | NUMERICAL APERTURE | TBS | X | X | X | EIA/TIA 455 FOTP 47 |
| 3.8.2 | OPTICAL FIBER BANDWIDTH | TBS | X | X | X | EIA/TIA 455 FOTP 30 |
| 3.8.3 | ATTENUATION | TBS | X | X | X | 4.10.12 |
| 3.8.3.1 | SUBSTITUTION LOSS | T | X | X |  | 4.10.12.4 |
| 3.8.3.2 | INDUCED LOSS | T | X | X |  | 4.10.12.3 |
| 3.9 | ENVIRONMENTAL |  |  |  |  |  |
| 3.9.1 | FUNGUS | T | X | X |  | MIL-STD-810 Method 508 |
| 3.9.2 | TOXICITY | T | X | X |  | MIL-STD-810 METHOD 508 |
| 3.9.3 | ODOR | T | X | X |  | NHB 8060.1B |
| 3.9.4 | FLAMMABILITY | T | X | X |  | MSFC-HDBK-527 |
| 3.9.5 | OUTGASSED PRODUCTS/ VACUUM STABILITY | T | X | X |  | JSC-SPEC-SP-R-0022 |
| 3.9.6 | RADIATION RESISTANCE |  |  | X |  |  |
| 3.9.7 | THEMAL SHOCK | D | X | X |  | MIL-STD-202 Method 107 |
| 3.9.8 | SURVIVAL TEMPERATURE | D | X | X |  |  |
| 3.10 | MECHANICAL |  |  |  |  |  |
| 3.10.1 | STRIPPABILITY |  | X | X | X | VERIFIED DURING USE |
| 3.10.1.1 | CABLE BUFFER STRIPPABIILITY | D | X | X |  | VERIFIED DURING USE |
| 3.10.1.2 | FIBER BUFFER STRIPPIBILITY | D | X | X |  |  |
| 3.10.1.3 | CABLE JACKET STRIPPABILITY | D | X | X |  | VERIFIED DURING USE |

Table IX – Requirements Verification Matrix (continued)

| 3.10.2 | FIBER PROOF TEST | TBS | X | X | X |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.10.3 | CYCLIC FLEXING | D | X | X |  | 4.10.20 |
| 3.10.4 | CRUSH | D | X | X |  | EIA/TIA-455 FOTP 41 |
| 3.10.5 | CABLE BEND | D | X | X |  |  |
| 3.10.6 | CABLE WEIGHT | TBS | X | X | X |  |
| 3.11 | PHYSICAL |  |  |  |  |  |
| 3.11.1 | FIBER AND FIBER BUFFER |  |  |  |  | 4.10.7 |
| 3.11.1.1 | FIBER CORE | TBS | X | X | X | 4.10.7.4.3 |
| 3.11.1.2 | FIBER CLAD | TBS | X | X | X | 4.10.7.4.3 |
| 3.11.1.3 | FIBER COATING DIAMTER | TBS | X | X | X | 4.10.7.4.3 |
| 3.11.2 | CABLE |  |  |  |  |  |
| 3.11.2.1 | CABLE BUFFER | TBS | X | X | X | 4.10.7 |
| 3.11.2.2 | CABLE STRENGTH MEMBER | TBS | X | X | X | 4.10.7 |
| 3.11.2.3 | CABLE JACKET | TBS | X | X | X | 4.10.7 |
| 3.11.2.3.1 | CABLE JACKET OUTSIDE DIAMETER | TBS | X | X | X | 4.10.7 |
| 3.11.2.3.2 | CABLE JACKET STABILITY | D | X | X | X |  |
| 3.11.2.4 | CABLE JACKET COLOR | D | X |  | X | MIL-STD-104 |
| 3.11.2.5 | LENGTHS AND SPLICES | TBS |  |  | X | EIA/TIA-455 FOTP 60 & 61 |

APPENDIX A 	ITEM DETAIL SPECIFICATION

SPECIFICATION CUSTODIAN:

THE BOEING COMPANY

SPACE AND COMMUNICATIONS DIVISION

2100 SPACE PARK DRIVE

HOUSTON TEXAS, 77058

CAGE CODE: 2B945

INTERNATIONAL SPACE STATION PROGRAM OFFICE SSQ 21654 Revision C May 03, 1999 National Aeronautics and Space Administration International Space Station Program Johnson Space Center, Houston, Texas

10.0  SCOPE

This appendix defines the detail dimensional, material and performance requirements for a single element, multimode, fiber optic cable for use in near earth orbit and internal and external installations.

20.0  Applicable documents

None.

30.0  Requirements

30.1 Fiber characteristics

Attenuation (Cabled) - 4 dB/km @ 1290 + 10 nm Maximum

Numerical Aperture - 0.30 + .02 @ 1290 + 10 nm

Bandwidth - 200 MHz-km @ 1290 + 10 nm Minimum

Proof Strength - 200,000 psi Minimum

Core Ovality - 5%

Cladding Ovality - 4 %

Core/Cladding Offset - 98% Minimum

Cable Weight - 5.5 lbs/1000 ft. Maximum

Color - Violet in accordance with MIL-STD-104

Temperature:       Operating  -120oC to +150oC

Storage     -131oC to +150oC

30.2  Physical dimensions and materials

The physical dimensions and materials for the cable are defined in Table A-I and

Figure A-I. Dimensions are SI metric.

Table A-1, FIBER/CABLE DIMENSIONS

| Find | Item | Dimension | Material | Construction |
| --- | --- | --- | --- | --- |
| A B | Core Cladding | 100+2um 140+2um | Doped Silica | Drawn |
| C | Hermetic Coating | 0.025-0.05 um Thickness | Carbon Based Hermetic Sealer | Chemical Vapor Deposition |
| D | Fiber Coating Buffer | 170+2um | Polyimide | Coat with Heat Cure |
| E | Cable Buffer | 380-760+25um | FEP-Teflon | Extruded |
| F | Strength Member | 1.6mm OD | Teflon Impregnated Fiberglas | Braided |
| G | Cable Jacket | 2.10+0.05mm .25mm Thick | FEP- Teflon | Extruded |

40.0 Quality assurance provisions

The requirements of SSQ 21654 4.0 apply.

50.0 Packaging

Packaging of product produced to the requirements of this appendix shall be packaged in accordance with 5.0 of this specification.

60.0 Notes

The optical cable contains about 0.2% excess fiber when it is removed from the shipping reel.  The excess fiber shall be removed prior to terminating the ends as follow:

Short Lengths (Less than 2 meters)

- Cut the cable to the required length.  Add the length required to terminate the ends.

- After removing the cable jacket, strength member, and buffer tube, lay the length of cable 	so that it is straight and on a flat table.

- Move the fiber in and out of the cable and verify that the fiber is free to move through the 	cable.

- Clean the fiber and replace the fiber in its original position with respect to the optical cable jacket.

G	 F	E	D			A, B and C

Figure A-1.  Cable Configuration

Long Lengths (Greater than 2 meters)

1.		Cut the cable 2 inches longer than required. Add the length required to terminate the ends.

2.		Stretch the cable out on a table so that it lays straight and flat.

3.		Remove one inch of cable jacket, strength member and cable buffer tube from each end.

4.		Grab the fiber from one end and lift the fiber and approximately half the cable off the 	table.

5.		If the fiber moves at the other end, set the cable down,  If the fiber does not move, repeat

the step above on the other cable end.

6.	Cut off the excess fiber at each cable end.