WHOLE RECORD REPORT( + ADDENDUM)

| MSFC Record # A11883 | In-Flight Anomaly Number -- | Contractor Report Number E-121-1 | JSC# -- | KSC# -- |  |
| --- | --- | --- | --- | --- | --- |
| Problem Title LOX PRESSURE TRANSDUCER FAILED DURING ATP |  |  |  |  |  |
| EICN# -- | ELEMENT ET | Contractor MMMSS | FSCM# -- | FCRIT 1R |  |
| HCRIT 1R | Sys_Lvl N | Misc Codes A B C D E F G H I J K L M N O |  |  |  |
| HARDWARE EIM | NOMENCLATURE LOX PRES TRNDUCER | PART# PD7400098-079 | SER/LOT# 2215 | MANUFACTURER GULTON |  |
| HARDWARE LRU | NOMENCLATURE N/A | PART# N/A | SER/LOT# N/A | MANUFACTURER N/A |  |
| HARDWARE NCA | NOMENCLATURE LOX PRES TRNDUCER | PART# PD7400098-079 | SER/LOT# 2215 | MANUFACTURER GULTON |  |
| Test/Operation A - ATP | Prevailing Condtion -- | F / U F | Fail Mode EV - NOT-TO-SPEC | Cause MAE - MFG-ASY-ENVR |  |
| System ELECTRICAL | Defect CN - CONTAM | Material C - EEE | Work Contact J. ADAMS | Fail Date 11/23/1988 |  |
| Received at MSFC 11/28/1988 | Date Isolated -- | FMEA Reference 3.2.1.2.1 | IFA: Mission Phase -- | Mission Elapsed Time -- |  |
| Location GULTON |  | Symptom EE - RANDOM |  | Time Cycle -- |  |
| Effectivity Text -- |  |  |  |  |  |
| Vehicle Effectivity Codes |  |  |  |  |  |
| Vehicle 1 -- | Vehicle 2 -- | Vehicle 3 -- | Vehicle 4 -- | Vehicle 5 -- |  |
| Mission Effectivity Codes |  |  |  |  |  |
| Mssn 1 -- | Mssn 2 -- | Mssn 3 -- | Mssn 4 -- | Mssn 5 -- |  |
| Estimated Completion Dates |  |  |  |  |  |
| MSFC Approved Defer Until Date -- | Contractor Req Defer Until Date -- | LVL 3 Close -- | Remark / Action -- |  |  |
| Investigation / Resolution Summary |  |  |  |  |  |
| Last MSFC Update 02/13/1995 | CN RSLV SBMT -- | Defer Date -- | Add Date 11/28/1988 | R/C Codes 0 - EXPL -- -- |  |
| Assignee |  |  |  |  |  |
| Design A. JACKMAN | Chief Engineer A. JACKMAN | S & MA R. JACKSON | Project M. PESSIN | Project MGR J. SMELSER |  |
| Approval |  |  |  |  |  |
| Design A. JACKMAN | Chief Engineer J. NICHOLS | S & MA R. JACKSON | Project M. PESSIN | Project MGR J. SMELSER |  |
| PAC Assignee J.EL-IBRAHIM | PAC Review Complete JE | MSFC Closure Date 01/18/1989 | Status C - CLOSED | F/A Completion -- |  |
| Problem Type -- | SEV -- | Program Name -- | REVL -- | OPRINC -- |  |
| FUNC MOD -- | Software Effectivity -- -- -- -- -- | Software Fail CD -- |  | SUBTYPE -- | Software Closure CD -- |
| RES PERSON L2 -- | Approval Signature L3 -- |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Related Document Type -- | Related Document ID -- |  |  |  |  |
| Related Document Title -- |  |  |  |  |  |
| Contractor Status Summary |  |  |  |  |  |
| Reliability/Quality Assurance Concerns, Recommendations: |  |  |  |  |  |
| Problem Description THE TRANSDUCER HAD A LOSS OF SIGNAL (DROPOUT) AT APPROXIMATELY 29 PSID ON PRESSURE RISE DURING THE HIGH RESOLUTION CALIBRATION TEST NO DROPOUTS ARE ALLOWED. MARS T-34387, SERIAL NUMBER 2215 |  |  |  |  |  |
| Contractor Investigation/Resolution THIS PROBLEM REPORT HAS BEEN DEFERRED FOR STS-27 BASED ON THE FOLLOWING RATIONALE: THE LOX PRESSURE TRANSDUCER IS CRIT 1R. THERE ARE FOUR UNITS INSTALLED WHILE THREE ARE REQUIRED TO BE FUNCTIONAL IF ONE UNIT FAILS TO OPERATE THE BACK-UP UNIT COULD BE SWITCHED TO UNTIL T-10 SECONDS. ALSO, SHOULD THIS FAILURE MODE OCCUR, THE ORBITER FLOW CONTROL VALVE WOULD OPEN AND CAUSE A CHANGE IN THE ULLAGE PRESSURE AND THE UNIT WILL BE OPERATIONAL AGAIN. IN FLIGHT, IF ONE UNIT FAILS, THE SIGNAL WHICH OPENS CORRESPOND- ING CONTROL VALVE AND TANK PRESSURE WOULD INCREASE. THE OUTER TWO SENSORS COMPENSATE BY CLOSING THE OTHER TWO VALVES THE ABOVE DEFERRAL RATIONALE HAS BEEN APPROVED BY THE ET PROJECT MANAGER, MR. J. SMELSER ___________JERRY SMELSER (11/30/88)________ 12/5/88 GENERAL THE TRANSDUCERS FAILED THE SIGNAL DROPOUT CRITERIA DURING VENDOR ACCEPTANCE TESTING (ATP). OCCASIONAL ATP FAILURES OF THIS TYPE ARE EXPECTED AS PART OF NORMAL PRODUCTION FALLOUT. INVESTIGATION OF PREVIOUS SIMILAR FAILURES HAS FOUND THAT MICROSCOPIC PARTICULATE CONTAMINATION OF THE INTERNAL RESISTIVE ELEMENT AND WIPER IS THE MOST PROBABLE CAUSE. REFERENCE: CAPS E-082 OCCASIONAL ATP FAILURES OF THIS TYPE ARE EXPECTED AS PART OF NORMAL PRODUCTION FALLOUT. INVESTIGATION OF PREVIOUS SIMILAR FAILURES HAS FOUND THAT MICROSCOPIC PARTICULATE CONTAMINATION OF THE INTERNAL RESISTIVE ELEMENT AND WIPER IS THE MOST PROBABLE CAUSE. REFERENCE: CAPS E-082 TASK I FAILURE INVESTIGATION FAILURE ANALYSIS OF THE TRANSDUCER IS NOT WARRANTED. NUMEROUS PREVIOUS FAILURES OF THIS TYPE HAVE OCCURRED AND THE HARDWARE WAS ANALYZED. NO FAILURE CAUSE OTHER THAN CONTAMINATION HAS BEEN FOUND OR POSTULATED. THIS TYPE OF FAILURE MEETS ALL OF THE REQUIREMENTS OF "B80862 SPACE SHUTTLE EXTERNAL TANK CRITICALITY 1, 1R 2 AND 2R PROBLEM REPORTING REQUIREMENTS PLAN: PARAGRAPH 2.3.4 CLOSURE REPORT" AND NEITHER ANALYSIS NOR CORRECTIVE ACTION IS REQUIRED TASK CLOSED TASK III CLEARANCE OF EFFECTIVITIES ALL ET'S CLEARED. THE FAILURE OCCURRED DURING VENDOR ACCEPTANCE TESTING AND ALL INSTALLED TRANSDUCERS HAVE SUCCESSFULLY PASS THE ACCEPTANCE TEST TASK CLOSED TASK IV CAPS CLOSURE SUMMARY THE TRANSDUCER FAILED DURING VENDOR ACCEPTANCE TEST. THE FAILURE WAS OF A TYPE THAT IS CONSIDERED NORMAL PRODUCTION FALLOUT. NO CORRECTIVE ACTION IS REQUIRED TASK CLOSED THIS PROBLEM IS CONSIDERED CLOSED |  |  |  |  |  |
| MSFC Response/Concurrence |  |  |  |  |  |

ASSESSMENT ADDENDUM REPORT

| MSFC Report# A11883 | IFA# -- | Contractor RPT# E-121-1 | JSC# -- | KSC# -- | EICN# -- |
| --- | --- | --- | --- | --- | --- |
| Asmnt Part# PD7400098-079 | Asmnt Part Name GO2 PRES TRANSDUCER | Asmnt Serial/Lot# 2215 |  |  |  |
| HCRIT CD -- | FCRIT CD 1R | CAUSE CD MAE - MFG-ASY-ENVR | FAIL MODE EG - SIG HI OR LO |  |  |
| Asmnt FMEA 3.2.1.1 | Asmnt FM 1 | FMEA CSE E | FMEA SCSE 1 |  |  |
| Asmnt FMEA -- | Asmnt FM -- | FMEA CSE -- | FMEA SCSE -- |  |  |
| Asmnt FMEA -- | Asmnt FM -- | FMEA CSE -- | FMEA SCSE -- |  |  |
| Correlated Part# -- | Correlated Part# -- | Correlated Part# -- |  |  |  |
| Associated LRU# -- | Associated LRU# -- | Associated LRU# -- |  |  |  |
| MAJOR DESIGN CHANGES |  |  |  |  |  |
| APRV DATE -- | DESCRIPTION OF CHANGES -- |  |  |  |  |
| ASSESSMENT TEXT |  |  |  |  |  |