Surface Coating of Wood Building Products NESHAP (subpart QQQQ)

Applicability Flowchart

Does your facility

surface coat wood

building products?

1

Does

your facility

use 4,170 liters (1,100

gal) of total coatings

per year?

2

Does your

facility emit

>

10 tons/yr

of a specific HAP?

3

Does your

facility emit

>

25 tons/

yr of any combination

of HAPs?

3

Are the facility's

coating operations

subject to subpart

DDDD?

4

Are the

facility's

coating operations

subject to subpart

JJ?

Do surface

coating operations occur

during the manufacture

of prefabricated

homes?

Your facility is not

covered by subpart

QQQQ.

Yes

Yes

No

Yes

Yes

Yes

No

No

Yes

No

Yes

No

Does your

facility's

operation overlap with

subpart QQQQ?

Facility is covered by subpart QQQQ; if the facility's

coating operation that is subject to a subpart other than

QQQQ accounts for

>

95% of annual coating usage,

you have the option to comply fully with that subpart.

Yes

No

Do surface

coating operations

occur at research or

laboratory

facilities?

No

Does your

facility use a retort or

other pressure vessel

for wood

treatment?

This part of your facility

is not covered by

subpart QQQQ.

No

Yes

No

1A Wood Building Product is defined as any product that contains more than 50%(by weight) wood or wood fibers, excluding the weight of any glass components, and is used in the construction (interior or exterior) of a residential,  commercial, or institutional building ['63.4681(a)].

commercial,

2Usage limit only applies to surface coating of wood building products.  Do not include coating used on products that are not potentially subject to subpart QQQQ.

3You must determine your affected source's potential to emit HAPs by looking at all stationary emissions within your fenceline, not just those operations covered by the Wood Building Products NESHAP.  If the affected source's potential is greater than or equal to the 10/25 limit, the source is considered a major source of HAP emissions.

Potentially regulated SIC (NAICS) codes include 2421, 2426, 2431, 2435, 2436, 2493, and 2499 (321211, 321212, 321219, 321911, 321918, and 321999).  Applicability of codes ending in 99 is dependent on the final end use of the product.

5Subpart JJ is the Wood Furniture NESHAP.

4Subpart DDDD is the Plywood and Composite Wood Products NESHAP.

No

Yes

Your facility is not

covered by subpart

QQQQ.

Your facility is not

covered by subpart

QQQQ.

Your facility is covered

by subpart QQQQ.

'63.4681(b)

'63.4681(b)

'63.4681(b)

'63.4681(c)(1)

'63.4681(c)(2)

'63.4681(c)(3)

'63.4681(c)(4)

'63.4681(c)(5)

'63.4681(d)